3GPP TSG RAN WG1 Meeting #120                                                                                  R1-2500316

Athens, Greece, February 17th – 21st, 2025

Agenda item:		9.3.3

Source:	MediaTek Inc.

Title:	Discussion on CLI Handling in SBFD System

Document for:		Discussion and Decision

# Introduction

The following is agreed as part of the scope of the Rel-19 WI on Evolution of NR Duplex Operation [1].

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource. - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD.

In this contribution, we provide our views on L1 based UE-to-UE CLI measurement and reporting framework.

# Discussion

## L1 based UE-to-UE CLI Measurement and Reporting

In RAN1#117, RAN1 agreed to specify L1 based UE-to-UE CLI measurement and reporting following existing CSI framework. In this section, we outline our views on measurement resource and reporting configurations for L1 SRS-RSRP and L1 CLI-RSSI.

### Measurement Resource Configuration

#### SRS-RSRP Measurement Resource

To enable faster, up to date L1 SRS-RSRP measurement and reporting at a victim UE, there must be frequent SRS transmissions from aggressor UEs. Based on existing SRS configuration, the frequency of SRS transmission is controlled by the need for SRS at the gNB. SRS is transmitted towards gNB for a specific “usage” – i.e., “beamManagement”, “antennaSwitching”, “codebook”, or “nonCodebook”. A victim UE is configured to perform SRS-RSRP measurement when there is SRS transmitted towards gNB. Therefore, if for any reason there is infrequent SRS transmissions towards the gNB, the UE’s ability to measure and report up to date L1 SRS-RSRP will be impacted.

It is possible to have infrequent SRS transmission towards the gNB if there is no specific need for SRS by the gNB for an extended period of time. For the different “usage” cases, the frequency of SRS transmission is impacted as follows:

- SRS for beamManagement: Less likely to be configured for UEs operating in FR1 since such UEs are unlikely to perform analogue beamforming.

- SRS for antennaSwitching: Less likely to be configured in scenarios where channel reciprocity does not hold. For SBFD operation, channel reciprocity may not hold between UL-SB and DL-SB(s) of SBFD symbols – since each subband occupies a different frequency region in SBFD symbol.

- SRS for “codebook” and “nonCodebook”: If the higher layer parameter txConfig within pusch-Config is not provided, the UE transmits PUSCH on a single antenna port. In this case, UE is not required to transmit SRS to determine a precoder for the PUSCH transmission.

This implies that for a SBFD-aware UE operating in FR1 with the higher layer parameter txConfig within pusch-Config not provided, SRS transmission towards the gNB will be infrequent. For L1 SRS-RSRP measurement, infrequent SRS transmission will be especially problematic since the interference pattern is dynamic and therefore frequent reports are needed for efficient CLI mitigation. To avoid any potential impact of infrequent SRS transmission on L1 SRS-RSRP measurement, configuration of dedicated SRS resource that are transmitted solely for the purpose of L1 SRS-RSRP measurement should be supported, as outlined below.

##### Dedicated SRS Resources for L1 SRS-RSRP Measurement

Agreement RAN1#119 Define a new IE SRS-RSRP-MeasurementResourceSet containing a set of SRS-RSRP measurement resource(s) SRS-RSRP-MeasurementResource for L1 SRS-RSRP measurement. - Configuration of slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured (already agreed in RAN1#118bis) - FFS: Other configurations Define SRS-RSRP-MeasurementResource with the following parameters: - Legacy SRS Resource IE - FFS: Other parameters

In RAN1#119, RAN1 agreed to support configuration of a new IE SRS-RSRP-MeasurementResourceSet containing a set of SRS-RSRP measurement resource(s) SRS-RSRP-MeasurementResource for L1 SRS-RSRP measurement. For such dedicated SRS-RSRP measurement resource sets at a victim UE, there must be a corresponding configuration of SRS transmission resources at an aggressor UE.

As outlined in the above discussion, relying solely on SRS transmission towards gNB may not be sufficient to obtain up to date L1 SRS-RSRP measurement and reporting, due to the possibility of infrequent SRS transmissions. Therefore, in addition to having dedicated SRS-RSRP measurement resource sets at a victim UE, configuring dedicated SRS transmission resources at aggressor UE solely for the purpose of L1 SRS-RSRP measurement is needed support more efficient SBFD operation.

Dedicated SRS transmission resources for L1 SRS-RSRP measurement can be achieved by configuring a new SRS Resource Set (e.g., SRS-CliResourceSet), which provides SRS configuration to be used specifically for SRS transmissions intended for L1 SRS-RSRP measurement. Network can ensure that the new dedicated SRS transmission resources at an aggressor UE corresponds to the new IE SRS-RSRP-MeasurementResourceSet at the victim UE.

The specification impact is minimal since existing SRS Resource Set configuration can be directly reused. The network can provide the aggressor UE with multiple SRS Resource Sets – including the new SRS Resource Set containing a set of SRS resources configured specifically for L1 SRS-RSRP measurement.

An added benefit for having a dedicated SRS Resource Set at the aggressor UE is that the network can optimize the SRS resource configuration for L1 SRS-RSRP measurement. For example, the network can configure UL power control parameters (e.g., alpha, p0) for SRS Resource Sets dedicated to L1 SRS-RSRP measurement considering the fact that the receiving node (victim UE) is within a closer proximity of the aggressor UE.

- It is possible to have scenarios where SRS transmission toward gNB is not frequent enough to allow up to date L1 SRS-RSRP measurement and reporting in SBFD operation.

- Having dedicated SRS transmissions solely for the purpose of L1 SRS-RSRP measurement can support fast, reliable, and more accurate SRS-RSRP measurement and reporting.

- Support configuration of a new IE (e.g., SRS-CliResourceSet) containing a set of SRS resources to be transmitted solely for the purpose of L1 SRS-RSRP measurement.

##### Low Power SRS Transmission for CLI Measurement

As discussed above, existing specification only supports SRS transmission with gNB as intended receiver. Thus, to ensure that gNB receives the transmitted SRS effectively, UE determines SRS transmit power using UE-to-BS pathloss. However, in the scenario proposed above, where dedicated SRS resources are transmitted for L1 SRS-RSRP measurement, the SRS power control can be optimized to reflect transmission towards a nearby UE. This can significantly reduce SRS transmit power, especially for cell-edge UEs.

For example, consider a typical SRS transmission in a cell, as shown in Figure 1, where UE1 transmits SRS towards gNB, while UE2 performs L1 SRS-RSRP measurement from the SRS transmission of UE1. In legacy operation, UE1 determines its SRS transmit power based on UE-to-BS pathloss. When dedicated SRS transmission is supported for L1 SRS-RSRP measurement, the SRS power control can be enhanced to reflect the pathloss between victim UE and aggressor UE, referred to here as low power (LP) SRS transmission.

Figure 1: Illustration of SRS Transmission in a cell

Table 1 shows a comparison of transmit power consumption for the legacy case versus LP SRS transmission. The following are assumed for the results presented in the table:

- pathloss compensation factor α=0.8, p0=-60dBm, Min UE-to-UE distance = 5m, Max UE-to-UE distance = 20m, Min UE-to-gNB distance = 10m, Max UE-to-gNB distance = 100m.

Table 1: SRS Transmit Power Consumption using UE-to-gNB pathloss vs using UE-to-UE pathloss.

|  | Tx Power for Legacy Case (dBm) | Tx Power for Low Power SRS (dBm) | % Power Reduction |
| --- | --- | --- | --- |
| Min Distance | -0.37 | -4.7 | 62% |
| Max Distance | 17.23 | 3.5 | 95.8% |

The results show that, when using LP SRS transmission for L1 SRS-RSRP measurements, cell-edge UEs could spend significantly less power (up to 95% less) compared to using legacy SRS power control. Moreover, in addition to the significant power savings, allowing LP SRS transmissions for L1 SRS-RSRP measurement can reduce inter-cell interference to neighbouring gNBs.

Low transmit power configuration for SRS can be achieved by optimizing the existing SRS power control loop for transmission between nearby UEs. One approach is to modify the existing SRS power control loop to exclude the closed loop power control parameter and UE-to-gNB pathloss compensation. Another approach is to provide a new higher layer SRS power offset to the UE. The UE can be instructed to apply the power offset when transmitting dedicated SRS for L1 based SRS-RSRP measurement. Operations at the transmitting and measurement UEs when supporting LP SRS transmission can be summarized as follows:

- Transmitting UE:

  - Nework configures dedicated SRS Resource Set containing SRS resources to be transmitted solely for L1 SRS-RSRP measurement

    - Dedicated SRS Resources Set includes SRS power control parameters to support LP SRS transmission.

  - When transmiting SRS using dedicated SRS Resource Set for L1 SRS-RSRP measurement, UE applies LP SRS transmission. Low power configuration can be acheived by;

    - Modifying existing SRS power control loop to exclude the closed loop power control parameter and UE-to-gNB pathloss compensation. OR

    - Providing a new SRS power offset (via higher layer) to the UE. UE applies new power offset when transmitting dedicated SRS for L1 SRS-RSRP measurement.

- Measurement UE:

- UE perform L1 SRS-RSRP measurement of the dedicated SRS resource from transmitting UE

- UE applies power scaling to the measurement results and reports L1 SRS-RSRP to the network.

  - Power scaling ensures that the reported L1 SRS-RSRP reflects UE-to-UE CLI that will be experience from UL transmissions with higher transmit power, such PUSCH or PUCCH.

- Allowing LP SRS transmission for L1 SRS-RSRP measurement can significantly reduce power consumption as well as inter-cell interference.

- Support enhanced SRS power control optimized for dedicated SRS transmission for L1 SRS-RSRP measurement.

- Support power scaling of SRS-RSRP measurement results or new SRS-RSRP reporting value range when using enhance SRS power control optimized for dedicated SRS transmission for L1 SRS-RSRP measurement.

#### CLI-RSSI Measurement Resource

Agreement For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported: - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband. - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands. - FFS: Number of resources that can be configured - FFS: UE behavior for measurement Agreement For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs. - A single wideband RSSI measurement report is supported. - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband.

In RAN1#118, RAN1 agreed two measurement resource types for CLI-RSSI measurement resource. It still needs to be determined how many CLI-RSSI resources can be configured for each measurement resource type. Additionally, the UE behaviour for measurement needs to be defined.

In terms of the number of resources to be configured for L1 CLI-RSSI resources, the same total number of resources as in legacy (i.e., 64 resources) can be configured for both measurement resource type#1 and type#2.

For measurement resource type#1, the following three cases may be inferred.

- Case 1: Resources are always confined within one DL-SB only (either upper or lower DL-SB) (Figure 2 (a)).

In this case the UE can follow legacy behaviour, i.e., a total of 64 resources can be configured within a single DL-SB. The UE generates one CLI-RSSI report for each configured CLI-RSSI measurement resource. CLI-RSSI is not measurement on the other DL-SB.

- Case 2: Resources are confined within one DL-SB at one time and within the other DL-SB at another time (Figure 2 (b)). Similar UE behaviour as Case 1. Allows UE to perform CLI-RSSI measurement within both DL-SBs at different times.

- Case 3: Resources are configured within two DL-SB, each single resource is within a DL-SB ((Figure 2 (c)).

In this case the total number of measurement resources can be shared between the two DL-SBs, e.g., maximum of 32 resources per DL-SB. Similar to Case 1, the UE generates one CLI-RSSI report for each configured CLI-RSSI measurement resource.

- (b)                                                 (c)

Figure 2: CLI-RSSI Measurement Resource Type#1 (a) Resource always within the same DL-SB (b) Resource within different DL-SB at different times. (c) Resource within two DL-SBs, each single resource is within a single DL-SB

- For both CLI-RSSI measurement resource types, the number of resources that can be configured is same as legacy (i.e., maximum of 64 resources).

- For measurement resource type#1 the following can be considered.

  - Case 1 and Case 2: The total of 64 resources are confined within a single DL-SB

  - Case 3: The total of 64 resources are shared between the two DL-SBs

- For measurement resource type#2, a non-contiguous resource across two DL-SBs is counted as a single measurement resource.

### Measurement Reporting Configuration

#### Time Domain Behaviour for L1 UE-to-UE CLI Reporting

In the WID [1], RAN1 agreed to support at least aperiodic reporting for L1 based UE-to-UE CLI. It needs to be determined whether periodic and semi-persistent reporting can additionally be supported.

We have the following views on time domain behavior of L1 SRS-RSRP and L1 CLI-RSSI reporting.

- Aperiodic Reporting:

  - Aperiodic Triggering at Aggressor UE: For SRS-RSRP the network does not have any information on which UEs are the aggressors UEs for a particular victim UE. Therefore, network cannot accurately determine the UE for whom aperiodic SRS transmission must be triggered. Thus, network needs to trigger SRS transmission for multiple UEs, some of whom may not be actual aggressor UEs. Such an approach is costly in terms of DL control overhead.

  - Aperiodic Triggering at Victim UE: Unlike CSI reporting, network has no reliable information based on which it will trigger aperiodic UE-to-UE CLI reporting. This is because UE-to-UE CLI is between UEs rather than CSI which represents the channel condition between gNB and UE. One way is to trigger aperiodic CLI reporting after receiving a number of NACKs continuously from a victim UE. However, this is a slow process and cost spectral efficiency loss.

In our view, due to the issues with triggering mechanism for both aggressor and victim UEs, aperiodic CLI reporting is not sufficient for L1 UE-to-UE CLI reporting.

- Periodic Reporting: Periodic L1 UE-to-UE CLI reporting eliminates the issues outlined above regarding the triggering mechanism of aperiodic CLI reporting. In the case of the aggressor UE, the network does not need to trigger SRS transmission by specific UE for SRS-RSRP measurement, which implies no DL control overhead. In the case of the victim UE, periodic reporting enables UE to occasionally update network of the CLI status, without waiting for network instruction for each individual CLI report. Therefore, periodic CLI reporting should be supported in addition to aperiodic CLI reporting. As in the existing CSI framework, PUCCH resources can be used for periodic L1 SRS-RSRP and L1 CLI-RSSI reporting.

- Semi-persistent Reporting: Due to the MAC CE signalling required to activate/deactivate semi-persistent L1 SRS-RSRP and L1 CLI-RSSI reporting, we prefer to deprioritize semi-persistent reporting for L1 UE-to-UE CLI.

- Due to issues with triggering mechanism for both aggressor and victim UEs, aperiodic CLI reporting is not sufficient for L1 UE-to-UE CLI reporting.

- For periodic CLI reporting

- Network does not need to trigger specific UE to transmit SRS resources for SRS-RSRP measurement, which implies no DL control overhead.

- Victim UE can occasionally update network of the CLI status, without waiting for network instruction for each individual CLI report.

- Support periodic L1 SRS-RSRP and L1 CLI-RSSI reporting on PUCCH.

- Do not support semi-persistent L1 SRS-RSRP and L1 CLI-RSSI reporting.

#### Timing for L1 SRS-RSRP Measurement

Agreement For L1 CLI-RSSI measurement, DL timing is used for Method #1 (UE measures RSSI within DL subband). Proposal 2-14 For L1 SRS-RSRP measurement, - Alt 1: Reuse the timing for Rel-16 L3 based SRS-RSRP measurement, i.e., a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset. - Alt 2: Use the UL transmit timing for L1 SRS-RSRP measurement. - FFS: whether to introduce different UL Tx timing for SRS transmission FFS: Impact on CPU occupation

In RAN1#118, RAN1 agreed to use DL timing as the reference timing for L1 CLI RSSI measurement in DL subband. RAN1 proposed two alternatives for the reference timing of L1 SRS-RSRP measurements. In our view, the reference timing for R16 L3 SRS-RSRP measurement, i.e., Alt 1 of above proposal, can be reused as a starting point for L1 SRS-RSRP measurements. Any impact of this alternative on CPU occupation or CLI reporting in general can be further investigated by RAN1.

- For L1 SRS-RSRP measurement, reuse the timing for Rel-16 L3 based SRS-RSRP measurement.

# Conclusion

In this contribution, we discussed considerations for L1 based UE-to-UE measurement and reporting framework.

- It is possible to have scenarios where SRS transmission toward gNB is not frequent enough to allow up to date L1 SRS-RSRP measurement and reporting in SBFD operation.

- Having dedicated SRS transmissions solely for the purpose of L1 SRS-RSRP measurement can support fast, reliable, and more accurate SRS-RSRP measurement and reporting.

- Allowing LP SRS transmission for L1 SRS-RSRP measurement can significantly reduce power consumption as well as inter-cell interference.

- Due to the issues with triggering mechanism for both aggressor and victim UEs, aperiodic CLI reporting is not sufficient for L1 UE-to-UE CLI reporting.

- For periodic CLI reporting

- Network does not need to trigger specific UE to transmit SRS resources for SRS-RSRP measurement, which implies no DL control overhead.

- Victim UE can occasionally update network of the CLI status, without waiting for network instruction for each individual CLI report.

- Support configuration of a new IE (e.g., SRS-CliResourceSet) containing a set of SRS resources to be transmitted solely for the purpose of L1 SRS-RSRP measurement.

- Support enhanced SRS power control optimized for dedicated SRS transmission for L1 SRS-RSRP measurement.

- Support power scaling of SRS-RSRP measurement results or new SRS-RSRP reporting value range when using enhance SRS power control optimized for dedicated SRS transmission for L1 SRS-RSRP measurement.

- For both CLI-RSSI measurement resource types, the number of resources that can be configured is same as legacy (i.e., maximum of 64 resources).

- For measurement resource type#1 the following can be considered.

  - Case 1 and Case 2: The total of 64 resources are confined within a single DL-SB

  - Case 3: The total of 64 resources are shared between the two DL-SBs

- For measurement resource type#2, a non-contiguous resource across two DL-SBs is counted as a single measurement resource.

- Support periodic L1 SRS-RSRP and L1 CLI-RSSI reporting on PUCCH.

- Do not support semi-persistent L1 SRS-RSRP and L1 CLI-RSSI reporting.

- For L1 SRS-RSRP measurement, reuse the timing for Rel-16 L3 based SRS-RSRP measurement.

# References

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, 3GPP TSG RAN Meeting #103, June 2024.