3GPP TSG RAN WG1 #120                                                                  R1-2500360

Athens, Greece, February 17th – 21st, 2025

Source:	vivo

Title:	Views on Rel-19 ISAC deployment scenarios

Agenda Item:	9.7.1

Document for:	Discussion and Decision

Introduction

A new study item of channel modeling for integrated sensing and communications (ISAC) has been approved in RAN #102 [1], in which the objectives of the study for ISAC are as follows:

- The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency. - All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). - Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) - For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using TR38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns, - spatial consistency. - It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

In RAN1 #119, accordingly, the agreements are made as follows:

Agreement For UAV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118 as a baseline: The detailed scenario description in this clause can be used for channel model calibration. ISAC-UAV Details on ISAC-UAV scenarios are listed in Table x. Table x. Evaluation parameters for UAV sensing scenarios Parameters; Value. Applicable communication scenarios; UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV. Sensing transmitters and receivers properties; Rx/Tx Locations; Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenarios. NOTE1: This may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario.. Sensing target; LOS/NLOS; LOS and NLOS. Outdoor/indoor; Outdoor. 3D mobility; Horizontal velocity: uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0. Vertical velocity: 0km/h, optional {20, 40} km/h NOTE2: 3D mobility can be horizontal only or vertical only or a combination for each sensing target FFS: time-varying velocity. NOTE 3: time-varying velocity may be considered for future evaluations.. 3D distribution; Horizontal plane: Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. FFS: Value of N, defined area, and other distributions N = {1, 2, 3, 4, 5} NOTE4: N=0 may be considered for the evaluation of false alarm Vertical plane: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m assuming vertical velocity is equal to 0. FFS Other options are not precluded. NOTE5: target(s) are outside the minimum distance to the Tx/Rx. Orientation; Random in horizontal domain. Physical characteristics (e.g., size); Size: - Option 1: 1.6m x 1.5m x 0.7m - Option 2: 0.3m x 0.4m x 0.2m FFS: Material(s), Structure, Other size(s). Minimum 3D distances between pairs of Tx/Rx and sensing target; Option B: Min distances based on min. TRP/UE distances defined in TR36.777 as a starting point. NOTE5: the sensing target is assumed in the far field of sensing Tx/Rx Option C: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx. Minimum 3D distance between sensing targets; Option 1: At least larger than the physical size of a target Option 2: 10 meters. [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.]; FFS. NOTE: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations. Agreement For Automotive sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline: The detailed scenario description in this clause can be used for channel model calibration. ISAC-Automotive Details on ISAC-Automotive scenarios are listed in Table x. Table x. Evaluation parameters for Automotive sensing scenarios Parameters; Values. Applicable communication scenarios; Highway, Urban Grid. NOTE1. Sensing transmitters and receivers properties; Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenario. NOTE2 FFS: Additional option: ISD between TRPs of Urban Grid is 250m. Sensing target; LOS/NLOS; LOS and NLOS (including NLOSv). Outdoor/indoor; Outdoor. Mobility (horizontal plane only); Based on TR37.885 mobility for urban grid or highway scenario. Distribution (horizontal); Based on dropping in TR37.885 per urban grid or highway communication scenario. Orientation; Lane direction in horizontal plane. Physical characteristics (e.g., size); Type 1/2 (passenger vehicle) Type 3 (truck/bus) Vehicle type distribution per TR 37.885 as a starting point FFS: Other sizes, additional distributions, and vehicle types, e.g. one new type of e-scooter/motorcycle/bike. Minimum 3D distances between pairs of Tx/Rx and sensing target; Option 1: Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point. Option 2: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx NOTE3: the sensing target is assumed in the far field of sensing Tx/Rx. Minimum 3D distance between sensing targets; Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [10] m. value of x is FFS. Environment Objects, e.g., types, characteristics, mobility, distribution, etc.; EO Type 2 for Urban Grid - FFS: details, e.g. up to 4 walls modelled as EO type 2, per building of size [413m x 230m x 20m]. FFS: number of buildings, how many walls are modelled, additional building sizes, etc.. NOTE1: calibration for UMi, Uma, RMa is not performed for the automotive scenario, but UMi, Uma, RMa can be considered for future evaluations of the automotive sensing target scenarios. Calibration for UMi, Uma, RMa is expected to be performed for another sensing scenario. NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations. Agreement For Human (indoor and outdoor) sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline: The detailed scenario description in this clause can be used for channel model calibration. ISAC-Human Details on ISAC-Human scenarios are listed in Table x. Table x. Evaluation parameters for Human (indoor and outdoor) sensing scenarios Parameters; Indoor Values; Outdoor Values. Applicable communication scenarios NOTE1; Indoor office, indoor factory [TR38.901] Indoor room [TR38.808]; UMi, Uma, RMa [TR38.901]. Sensing transmitters and receivers properties; Rx/Tx Locations NOTE 2; Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario; Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario. Rx/Tx Mobility for UEs; Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr; Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr. Sensing target; LOS/NLOS; LOS and NLOS; LOS and NLOS. Outdoor/indoor; Indoor; Outdoor. 3D mobility; Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr (horizontal plane with random direction straight-line trajectory); Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr (horizontal plane with random direction straight-line trajectory). 3D distribution; N targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment FFS: Value of N NOTE1: N=0 may be considered for the evaluation of false alarm; Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. Uniform in horizontal plane NOTE1: N=0 may be considered for the evaluation of false alarm. Orientation; Random over the horizontal area; Random over the horizontal area. Physical characteristics (e.g., size); Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m; Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m. Minimum 3D distances between pairs of Tx/Rx and sensing target; Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx Option 2: Min distances defined in TR 38.901 and TR36.843 and TR38.859as a starting point NOTE2: the sensing target is assumed in the far field of sensing Tx/Rx; Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx Option 2: Min distances defined in TR 38.901 and TR36.843 and TR38.859 as a starting point NOTE3: the sensing target is assumed in the far field of sensing Tx/Rx. Minimum 3D distance between sensing targets; Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS; Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS. Environment Objects, e.g., types, characteristics, mobility, distribution, etc.; FFS, based on outcome for AI 9.7.2; FFS, based on outcome for AI 9.7.2. NOTE1: For the human (indoor and outdoor) sensing targets, additional communication scenarios can be considered for future evaluations. Channel model calibration for Urban Grid with outdoor humans is expected to be performed from Objects creating hazards on the road/railway sensing scenarios. NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations. Agreement For AGV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline: The detailed scenario description in this clause can be used for channel model calibration. ISAC-AGV Details on ISAC-AGV are listed in Table x. Table x. Evaluation parameters for Automated Guided Vehicles Parameters; Value. Applicable communication scenarios NOTE1; InF (TR38.901 including Table 7.8-7). Sensing transmitters and receivers properties NOTE2; Rx/Tx location are selected among the TRPs and UEs location in the corresponding communication scenario Rx/Tx Mobility for UEs - Option 1: 0 km/h - Option 2: 3km/h - Option 3: Uniform distribution between 0km/h and 3km/h. Sensing target; LOS/NLOS; LOS and NLOS. Outdoor/indoor; Indoor. 3D mobility; Horizontal velocity with random straight-line trajectory - Option 1: Uniform distribution in the range of up to 30 km/h - Option 2: Fixed velocities [3, 10] km/h. 3D distribution; Option A: Uniformly distributed in the convex hull of the horizontal BS deployment Option B: Uniformly distributed in horizontal plane. Orientation; Horizontal plane only. Physical characteristics (e.g., size); Size (L x W x H) - Option 1: 0.5m x 1.0m x 0.5m - Option 2: 1.5 m x 3.0m x 1.5 m - FFS: Material, Additional sizes, and AGV size distribution. Minimum 3D distances between pairs of Tx/Rx and sensing target; Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target Option 2: Min distances based on min. TRP/UE distances defined in TR38.901 NOTE: the sensing target is assumed in the far field of sensing Tx/Rx. Minimum 3D distance between sensing targets; Option A: At least larger than the physical size of a target Option B: Fixed value, [x] m. value of x is FFS. Environment objects, e.g., types, characteristics, mobility, distribution, etc.; FFS. NOTE1: For the AGV sensing targets, additional communication scenarios can be considered for future evaluations. NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations. NOTE3: RAN1 can further discuss narrowing down the number of sub-scenarios of InF Agreement For Objects creating hazards sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline: The detailed scenario description in this clause can be used for channel model calibration. ISAC-Hazards Details on ISAC-Hazards are listed in Table x. Table x. Evaluation parameters for objects creating hazards Parameters; Value. Applicable communication scenarios NOTE1; Highway, Urban grid, HST (High Speed Train). Sensing transmitters and receivers properties NOTE2; Rx/Tx Locations; Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenarios. FFS: Option 2 Additional option ISD between TRPs of Urban Grid is 250 m. Sensing target; LOS/NLOS; LOS and NLOS. Outdoor/indoor; Outdoor. 3D mobility; Horizontal velocity: up to [10] km/h for humans and animals FFS: Additional velocities, trajectory. 3D distribution; Uniformly distributed in horizontal plane. Orientation; Random distribution in horizontal plane. Physical characteristics (e.g., size); For human/pedestrians: Child: 0.2m x 0.3m x 1m Adult: 0.5m x 0.5m x 1.75m For animals: Size: 1.5m x 0.5m x 1 m FFS: other types/sizes of targets may be considered for future evaluations. Minimum 3D distances between pairs of Tx/Rx and sensing target; Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target Option 2: based on min TRP/UE distances defined in TR37.885 and TR38.802 and TR36.843 and TR38.859 NOTE: the sensing target is assumed in the far field of sensing Tx/Rx. Minimum 3D distance between sensing targets; Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [10] m. value of x is FFS. Environment objects, e.g., types, characteristics, mobility, distribution, etc.; EO Type 2 for Urban Grid - FFS: details, e.g. up to 4 walls modelled as EO type 2, per building of size [413m x 230m x 20m]. FFS: number of buildings, how many walls are modelled, additional building sizes, etc.. NOTE1: For the objects creating hazards sensing targets, additional communication scenarios can be considered for future evaluations. NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

According to the agreements in RAN1 #119, most evaluation parameters for the UAV, Human, Automotive, AGV and Objects creating hazards have been agreed. To further enable the channel model calibrations, an email discussion was kicked off, and the calibration methodology, assumptions, parameters and metrics were discussed and summarized in [3].

In this contribution, we discuss on the remaining issues for the ISAC channel model calibration.

Discussion

Methodology for channel model calibration.

Regarding the methodology for channel model calibration, the following two options are proposed during the email discussion,

    - OPTION 1: Similar to the stochastic communication channel model calibration approach in TR 38.901 for ISAC channel modelling, which includes parameters and tables for at least the following: large scale parameters, where fast fading is not included and full calibration, which includes fast fading (attached tables for some details).

    - OPTION 2: A step-wise approach for the ISAC channel model calibration simulations which consists of deterministic models for the Tx/Rx-target channels, background channels and combination of the target/background channels, e.g., CDL models limited to channel metrics based on received power and multipath/delay/angular spread.

It is the system level simulation with complex interference link that considers the stochastic channel conditions among sensing transmitters, sensing receivers and sensing targets, such as LOS / NLOS propagation condition and departure/arrival angle spread. Hence, we believe that Option 1 behaves the similarity of the stochastic communication channel model, while Option 2 is more complicated to cover all the possible channel conditions. Therefore, we prefer Option 1 approach for channel model calibration.

- RAN1 uses Option 1 approach for channel model calibration.

Target channel and combined channel

The purpose of channel calibration is to allow the companies to make sure the consistency of the channel model by addressing implementation errors from channel modeling. For the ISAC channel model, due to the additional target channel, the relative RCS, pathloss model and other parameters that may impact the sensing performance have to be newly calibrated. In addition, the ISAC channel is combined by target channel and background channel, the characteristics after the channel combining have to be calibrated as well. The calibration metrics are related to target channel and/or combined channel, and the details channel are discussed in section 2.3.

- RAN1 calibrates the target channel and combined channel (combination of target channel and background channel).

Calibration metrics

According to TR 38.901, the calibration of channel model can be split into large scale channel calibration, full calibration and spatial consistency calibration. The performance of coupling loss, wideband SIR/SINR, delay and angle spread, etc, are almost dependent on the channel modelling and implementation, and can be considered for the typical metrics as in communication channel model calibration.

For the communication channel calibration in TR 38.901, the coupling loss for calibration CDF figure is the coupling loss between UE and its serving cell. Regarding the coupling loss for ISAC channel, however, there is some issue how to determine the serving transmitter-target-receiver link for coupling loss collection.

The following options can be considered:

- All the transmitter-target-receiver links are serving links and sample points for the coupling loss CDF. For TRP-UE and UE-TRP bistatic sensing mode, only the pairs of UE and the serving TRP are considered. For UE-UE and TRP-TRP bistatic sensing mode, it contains the all the pairs of UEs and TRPs in the simulation map. For UE and TRP mono-static sensing mode, it contains the links of all the UEs and TRPs to all the sensing targets.

- Select one transmitter-target-receiver link to be the serving link and the sample point for the coupling loss CDF. For all the sensing modes, select one link with best coupling loss or best RSRP at receiver side as the serving transmitter-target-receiver link, and the best coupling loss or the best RSRP at receiver side as the sample point for the coupling loss CDF.

For TRP-UE/UE-TRP bistatic sensing mode, the transmitter-target-receiver link is selected based on the pairs of UE and the serving TRP.

For UE-UE and TRP-TRP bistatic sensing mode, the transmitter-target-receiver link is selected based on the whole pairs of UEs and TRPs in the simulation map.

For UE and TRP mono-static sensing mode, the transmitter-target-receiver link is selected based the links of all the UEs and TRPs to all the sensing targets.

- Select one or several serving transmitter-target-receiver link(s) to be the serving link(s) and the sample point(s) for the coupling loss CDF.

For monostatic sensing model, only calculate the coupling loss of sensing transmitter/receiver in the same cell area and as the sample point for the coupling loss CDF.

For the TRP-UE/UE-TRP bistatic sensing model, for each target, calculate the coupling loss among the TRP and all the serving UEs in the same cell and all the coupling loss as the sample points for the coupling loss CDF.

For the TRP-TRP/UE-UE bistatic sensing model, calculate the coupling loss of target and TRP-TRP/UE-UE within some range related to the sensing range as the sample points for the coupling loss CDF.

This issue also exists in the calibration for other metrics, such as for the delay spread and angle spread calibration, i.e., the link of transmitter-target-receiver as well as the sample point of the calibration.

- RAN1 study how to determine the serving transmitter-target-receiver link(s) for calibration. The following options can be considered as start point:

- Option 1: The link with best coupling loss or RSRP is considered as the serving link.

- Option 2: All the links in the simulation map are considered as the serving link.

- Option 3: Partial link(s) in some specific area as the serving link(s).

    - For monostatic sensing mode, the specific area can be the cell where the sensing TRP or the sensing UE is.

    - For the TRP-UE and UE-TRP bistatic sensing mode, the specific area can be the serving cell of UE.

    - For the TRP-TRP and UE-UE bistatic sensing mode, the specific area can be the two cells of the two sensing TRP and the cell(s) where the two UEs are attached.

Large scale channel calibration

The calibration metric can be coupling loss of target channel and wideband SINR. Coupling loss is influenced by the LOS/NLOS propagation condition of Tx-target and target-Rx link, pathloss model, RCS, concatenation of Tx-target and target-Rx link, etc. The coupling loss metric can focus on target channel that is calculated based on large scale channel. Regarding the SINR, the received signal power S is defined as received power summation at sensing receiver of the signal across the sensing target from serving sensing transmitter. The interference I can be divided into two parts: the first part is the received power summation at sensing receiver from serving sensing transmitter minus the received signal power S, denoted as I1, and the second part is the received power summation at sensing receiver from the other signal/data transmitters, such as non-serving TRP or UE, denoted as I2. Then the SIR/SINR calculation can be performed by the following options:

SIR=SI1+I2

and

SINR=SI1+I2+N0.

- The calibration metrics for large scale are coupling loss of target channel and wideband SIR/SINR of combined channel.

- The wideband SIR/SINR of combined channel can be defined as following:

- The received signal power S is defined as received power summation at sensing receiver of the signal across the sensing target from serving sensing transmitter;

- The received interference power I is defined as the summation of I1 and I2, where I1  is the received power summation at sensing receiver of the signal not across the sensing target from serving sensing transmitter, and I2, is the received power summation at sensing receiver from the other signal/data transmitters;

- The SIR equals to S/I1+I2 and SINR equals to S/I1+I2+N0.

Full calibration

For full calibration, the calibration metrics including coupling loss of target channel, wideband SINR, which are like the definition as in section 2.2. both large-scale and small-scale channel include delay spread, angle spread (i.e., ASD, ZSD, ASA, ZSA) and the largest PRB singular values, the smallest PRB singular values and the ratio between the largest PRB singular value and the smallest PRB singular value.

The delay spread, angle spread and singular values are influenced by the small scale channel model and the combination method of target and background channels, and hence both target channel and combined channel should be calculated.

- The calibration metrics for full calibration are:

- Coupling loss based on large-scale and small-scale channel for target channel.

- Wideband SIR/SINR.

- Delay spread and Angle Spread (ASD, ZSD, ASA, ZSA) for both target channel and combined channel.

- The largest PRB singular values, the smallest PRB singular values and the ratio between the largest PRB singular value and the smallest PRB singular value for both target channel and combined channel.

Calibration for spatial consistency

For the calibration for spatial consistency, the metric defined in 7.8.3 of TR38.901 can be reused as a starting point with slight modification listed as follows.

- Coupling loss on small-scaling channel of target channel.

- Wideband SINR before receiver of combined channel.

- Cross-correlation coefficient of delay in the target channel for the third cluster between paired links.

- Cross-correlation coefficient of AOA in the target channel for the third cluster between paired links.

- Cross-correlation coefficient of LOS/NLOS status in the target channel between paired links.

- Cross correlation coefficient of the target channel response on the first non-DC subcarrier in an OFDM symbol on antenna port 0 received on the first UE antenna over multiple realizations between paired links.

The paired link can be defined based on different calibration scenarios. According to the spatial consistency model methodology in our companion contribution R1-2500361 [4], two scenarios are introduced; one is single target related to multi-receiver and the other is multi-target related to single receiver. When the scenario of single target related to multi-receiver is used, the paired link is the link group from transmitter to different receiver across the same target, while the paired link is the link group from transmitter to same receiver across the different target if the scenario of multi-target related to single receiver is used.

- The calibration metrics for spatial consistency are:

- Coupling loss on small-scaling channel of target channel;

- Wideband SINR before receiver of combined channel;

- Cross-correlation coefficient of delay in the target channel for the third cluster between paired links;

- Cross-correlation coefficient of AOA in the target channel for the third cluster between paired links;

- Cross-correlation coefficient of LOS/NLOS status in the target channel between paired links;

- Cross correlation coefficient of the target channel response on the first non-DC subcarrier in an OFDM symbol on antenna port 0 received on the first UE antenna over multiple realizations between paired links.

RCS

RCS is a very important parameter in sensing channel model. The impact of RCS on channel model should be considered in the channel model calibration. Up to the discussion of RCS, the parameter models and values are different for different target types, but its relevant discussion is still on going with somewhat uncertainty. Considering the timeline and the impact of RCS on channel model, we suggest determining some fix values for the channel model calibration.

- RAN1 determines fixed RCS values for each sensing target for the channel model calibration.

Parameters for calibration

According to [2], most evaluation parameters for each sensing target are discussed, but there are still some options remained for the parameters, such as mobility, distribution, minimum distance, etc. Each of the options is reasonable in some specific sensing use cases, and thus all the options should be supported in the channel model. For the calibration, some typical assumptions should be selected.

Deployment scenarios

For each sensing target type, several deployment scenarios are agreed and proposed by companies in the email discussion, in our options, considering the diversity of sensing use cases, there is no need to have limitation and priority for each deployment scenario.

- RAN1 does not limit the deployment scenarios for calibration, and it is up to company inputs.

Sensing modes

According to [1], one of the objectives of the study for ISAC channel model is that all six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). In our options, there is no need to have limitation and priority for each sensing mode.

- RAN1 does not limit the sensing mode, and it is up to company inputs.

Target distribution

Regarding the targets 3D distribution, several options are proposed in the previous discussion. Considering the complexity and possible method of combining target(s) channel and background channel, we prefer to restrict the target number to 1~5 per cell.

- For outdoor deployment scenarios, N=1~5 targets uniformly distributed per cell. For indoor deployment scenarios, N=12~60 targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment.

Convolution method

Based on the agreement in the last meeting, two convolution methods are supported in ISAC channel modeling. For the full convolution method, it is clear enough that it can be used as the basic method in channel model calibration. For the one-by-one random coupling convolution method, there are still some implementation details that are unclear, such as whether the selected coupling paths need power normalization before or after one-by-one random coupling, which is not captured in the current agreement. However, this greatly affects the final calibration results since there is a large gap in the final angular expansion distribution of the target channel under the two conditions with and without power normalization, referring to our companion contribution of R1-2500361 [4]. Therefore, RAN1 needs to determine whether or how power normalization is made when using the convolution method of one-by-one random coupling for calibration.

- Full convolution is used as a baseline for ISAC channel calibration.

- Whether or how to do power normalization should be determined for ISAC channel calibration if ray level 1-by-1 random coupling convolution between Tx-target link and target-Rx link is used.

Scenario for spatial consistency calibration

Referring to the spatial consistency model methodology in our companion contribution R1-2500361 [4], two scenarios are introduced; one is the single target related to multi-receiver and the other is the multi-targets related to single receiver. From the calibration perspective, the two scenarios should be all calibrated to ensure the accuracy and feasibility of spatial consistency modeling for ISAC channel model.

- Both two scenarios are selected for spatial consistency calibration of ISAC channel model:

- Single target related to multi-receiver

- Multi-target related to single receiver

Conclusions

In this contribution, we have discussed the views on ISAC channel calibration. The observations and proposals are summarized as follows.

Proposal 1: RAN1 uses Option 1 approach for channel model calibration.

Proposal 2: RAN1 calibrates the target channel and combined channel (combination of target channel and background channel).

Proposal 3: RAN1 study how to determine the serving transmitter-target-receiver link(s) for calibration. The following options can be considered as start point:

- Option 1: The link with best coupling loss or RSRP is considered as the serving link.

- Option 2: All the links in the simulation map are considered as the serving link.

- Option 3: Partial link(s) in some specific area as the serving link(s).

    - For monostatic sensing mode, the specific area can be the cell where the sensing TRP or the sensing UE is.

    - For the TRP-UE and UE-TRP bistatic sensing mode, the specific area can be the serving cell of UE.

    - For the TRP-TRP and UE-UE bistatic sensing mode, the specific area can be the two cells of the two sensing TRP and the cell(s) where the two UEs are attached.

Proposal 4: The calibration metrics for large scale are coupling loss of target channel and wideband SIR/SINR of combined channel.

Proposal 5: The wideband SIR/SINR of combined channel can be defined as following:

- The received signal power S is defined as received power summation at sensing receiver of the signal across the sensing target from serving sensing transmitter;

- The received interference power I is defined as the summation of I1 and I2, where I1  is the received power summation at sensing receiver of the signal not across the sensing target from serving sensing transmitter, and I2, is the received power summation at sensing receiver from the other signal/data transmitters;

- The SIR equals to S/I1+I2 and SINR equals to S/I1+I2+N0.

Proposal 6: The calibration metrics for full calibration are:

- Coupling loss based on large-scale and small-scale channel for target channel.

- Wideband SIR/SINR.

- Delay spread and Angle Spread (ASD, ZSD, ASA, ZSA) for both target channel and combined channel.

- The largest PRB singular values, the smallest PRB singular values and the ratio between the largest PRB singular value and the smallest PRB singular value for both target channel and combined channel.

Proposal 7: The calibration metrics for spatial consistency are:

- Coupling loss on small-scaling channel of target channel;

- Wideband SINR before receiver of combined channel;

- Cross-correlation coefficient of delay in the target channel for the third cluster between paired links;

- Cross-correlation coefficient of AOA in the target channel for the third cluster between paired links;

- Cross-correlation coefficient of LOS/NLOS status in the target channel between paired links;

- Cross correlation coefficient of the target channel response on the first non-DC subcarrier in an OFDM symbol on antenna port 0 received on the first UE antenna over multiple realizations between paired links.

Proposal 8: RAN1 determines fixed RCS values for each sensing target for the channel model calibration.

Proposal 9: RAN1 does not limit the deployment scenarios for calibration, and it is up to company inputs.

Proposal 10: RAN1 does not limit the sensing mode, and it is up to company inputs.

Proposal 11: For outdoor deployment scenarios, N=1~5 targets uniformly distributed per cell. For indoor deployment scenarios, N=12~60 targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment.

Proposal 12: Full convolution is used as a baseline for ISAC channel calibration.

Proposal 13: Whether or how to do power normalization should be determined for ISAC channel calibration if ray level 1-by-1 random coupling convolution between Tx-target link and target-Rx link is used.

Proposal 14: Both two scenarios are selected for spatial consistency calibration of ISAC channel model:

- Single target related to multi-receiver

- Multi-target related to single receiver

References

- RP-234069, “New SID: Study on channel modelling for Integrated Sensing and Communication (ISAC) for NR”, Nokia, Nokia Shanghai Bell.

- 3GPP RAN1 #119, Chair notes RAN1#119, Orlando, US, November 18th – 22nd, 2024.

- R1-2500059, “Email discussion summary on ISAC CM calibration assumptions”, Moderator (AT&T), Athens, Greece, February 17th – 21st, 2025.

- R1-2500361, “Views on Rel-19 ISAC channel modelling”, vivo, Athens, Greece, February 17th – 21st, 2025.