3GPP TSG RAN WG1 #120                                                              R1-2500361

Athens, Greece, February 17th – 21th, 2025

Source:	vivo, BUPT

Title:	Views on Rel-19 ISAC channel modelling

Agenda Item:	9.7.2

Document for:	Discussion and Decision

Introduction

A new study item of channel modeling for integrated sensing and communications (ISAC) has been approved in RAN #102 [1]. Accordingly, in RAN1 #119 [3], It is the good progress on target channel and background channel modelling with the agreements listed as follows:

Agreement-1 Bistatic RCS values for a scattering point of a target are obtained by fixing an incident direction in LCS of target and varying the scattered directions in LCS of target; then changing to other incident direction. Agreement-2 - To generate indirect paths of NLOS ray + NLOS ray in the target channel - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4 - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4 - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2, - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1 - Other methods are up to company choice for complexity reduction - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration - The power threshold for path dropping is X=[-25] dB - X is relative to the strongest indirect path in the target channel - FFS: further power normalization of target channel is performed after path dropping, - Note: power normalization when target channel and background channel are combined can be discussed separately - FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx Agreement-3 The following RCS models are supported when human is modelled with single scattering point for monostatic, where different RCS values and/or models can be supported for human due to different size, shape, frequency, etc. - Model 1 - B1=0 dB - A is mean RCS value - B2 is modelled using log-normal distribution - Model 2 - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A - FFS: conditions for using which model Agreement-4 The following RCS model is supported when vehicle is modelled with single scattering point for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc. - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A Agreement-5 When vehicle is modelled with multiple scattering points for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc. - the recommended five scattering points are located in front, left, back, right and roof side of the vehicle - the following RCS model is supported for each scattering point - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A Agreement-6 EO type-1 (when modelled) is modelled in the same way as a sensing target in the ISAC channel model. Agreement-7 - If blockage/forward scattering between sensing targets is not considered, a propagation path from Tx to Rx interacting with more than one sensing targets is not modelled - FFS whether/how blockage/forward scattering can be modelled in the target channel. Agreement-8 - Doppler for a target including both macro-Doppler and micro-Doppler can be modeled using a unified formula, rrx,n',m'Tt.vrxt+rp,n',m'Tt∙vspt+rtx,n,mTt.vtxt+rp,n,mTt∙vsptλ0+ft Where, - rrx,n',m'T is the spherical unit vector at receiver for the link from Rx to the scattering point - rtx,n,mT is the spherical unit vector at transmitter for the link from Tx to the scattering point - rp,n',m'T is the spherical unit vector at the scattering point for the link from the scattering point to Rx - rp,n,mT is the spherical unit vector at the scattering point for the link from the scattering point to Tx - Dual mobility model in 7.6.10, TR 38.901 is used as start point to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0 - ft is only applicable for indirect path - Support one term of 2αn,mDn,mλ0 for indirect path of LOS ray+NLOS ray, NLOS ray+LOS ray - Support two terms of 2αn,mDn,mλ0 for indirect path of NLOS ray+NLOS ray - Doppler is separately determined for each of the multiple scattering points of a target - vspt can include macro-Doppler and/or micro-Doppler motion, vspt=vmacrot+vmicro,pt - FFS: maximum speed of moving scatterers - FFS: ratio of moving scatterers among all scatterers Agreement-9 - The following options are supported to generate the combined ISAC channel - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by summing the target channel(s) and background channel, i.e., power normalization is not performed. - Option 2: As an additional modelling component, power normalization is performed when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between - Alt 1: Power normalization on both target channel and background channel - Alt 2: Power normalization on background channel only - Alt 3: the target channel of a target will replace one cluster in the background channel - FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2 - FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2 Agreement-10 To model the polarization matrix of a direct/indirect path at a scattering point of an object other than EO type-2, the polarization matrix of the scattering point, i.e., CPMsp,i is modelled by αi,1,αi,2,βi,1,βi,2, and initial random phases Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ, i.e., CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is [uniformly distributed within -π,π] - FFS correlation between αi,1,αi,2,βi,1,βi,2 - FFS specular reflection - FFS: CPM normalization The following options are considered for further study, down select one option from the following - Option 1: αi,1=αi,2=1, βi,1=βi,2=κsp,i-1 is generated for path i, where κsp,i is XPR ratio - κsp,i is randomly generated by log-normal distribution. FFS mean/variance of the distribution - Option 2: αi,1=1, αi,2,βi,1,βi,2 are variables generated for path i - Option 3: αi,1,αi,2,βi,1,βi,2 are variables generated for path i - CPMsp,i defined in LCS - Option 4: αi,1=αi,2=1, βi,1=βi,2=0 is generated for path i Agreement-11 The finite size of the EO type-2 affects identification of specular reflection point. In the target channel, EO type-2 is modelled only if the specular reflection point is in the area of the EO type-2. Agreement-12 Component B2 of RCS is upper bounded by kσ dB for the log-normal distribution, where σ is the standard deviation of B2 in dB. FFS the value of k. Agreement-13 When the EO type-2 is modelled in the target channel, down select between the following options to determine the LOS condition of the Tx-target link and target-Rx link - Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition - FFS changes to the LOS probability defined in existing TRs - FFS details on blockage by EO type-2 - Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model

This contribution is organized as follows:

- The target channel modeling, including the polarization modeling of the target channel, RCS modeling, EO modeling, LOS probability and pathloss calculation.

- Background channel modeling.

- Characteristics of LOS/NLOS for sensing target from the DSP perspective, determining how many LOS/NLOS rays should be potentially contributed to sensing related channel towards different use cases.

- Channel model formation, including two component channel modeling, as well as the enhancements of Doppler and spatial consistency as additional modelling components.

- Link-level based channel model for ISAC.

Common Channel Modeling

Considering the forward compatibility and performance verification in NR and 6G, a common channel model needs to be designed for most of use-cases and sensing modes in various deployment scenarios, which can greatly enhance the effectiveness of ISAC channel modelling study. Based on the SID [1], how to realize a common channel modeling for ISAC should be the focus of the study at a common modelling framework.

In this section, we mainly discuss on target channel modeling, EO modeling, background channel modeling, two component channel modeling, unified Doppler enhancement, spatial consistency, link level channel modeling.

Target Channel Modeling

The discussion on target channel modeling in this section mainly focuses on the aspects of small scale, large scale, multi-point channel model and RCS modeling.

Small Scale Parameter Discussion

      - Polarization in the concatenation channel model

In the #119 meeting, RAN1 made a great progress on how to model the polarization matrix for the target scattering point, i.e., CPMsp,i, mathematically expressed by

CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ

Eq. 1

where αi,1,αi,2,βi,1,βi,2 are the gain factors, and Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ are the initial random phases that may be uniformly distributed in -π,π.

However, regarding the gain factors, it is still controversial with four options, as listed in Agreement 10 in Section 1. Here, we are expressing our views based on the characterises of the target.

In an ellipsometer system [19], the source emits electromagnetic (EM) wave with a known polarization (e.g., linear polarization), which is obliquely incident on the surface of the object, and the detector measures the polarization state (e.g., elliptical polarization) of the EM wave reflected from the surface, as illustrated in Figure 1. The change in polarization state caused by the surface reflection can be defined by the ratio of the sample reflectivity for p-polarized light (or ∥ polarization), over the surface reflectivity for s-polarized light (or ⟘ polarization).

Figure 1: An illustration of an ellipsometer system.

From the ellipsometer system, what we can learn is that regardless of the reflection kinds, such as specular, diffraction and scattering from a surface, at least, the polarization type (i.e., the phase difference between two perpendicular components) will be changed after reflection. As depicted in Figure 2 for NLOS communication channel, if the EM wave travels from the transmitter to the receiver through the complex environment (e.g., multiple objects), where the reflection point may incur either the specular reflection, or diffraction, or scattering, or transmission, the characteristics of the polarization changes could be fully randomized. In such a circumstance, Jones matrices can be an extremely useful way to keep track of all polarization changes.

Figure 2: An illustration of polarization changes during the multiple reflection points.

If the incident electric field Ei interacts with object elements 1, 2,⋯, N in order, then the output field Eo can be found by applying the Jones matrices for the components M1,M2⋯,MN to the incident field

Eo=MN⋯M2M1∙Ei=M∙Ei

where M is called transfer function or CPM in channel modeling.

It should be noting that in case where the number N is large and each interactive object element is complex, then, the CPM for the i-th path can be approximately randomized as the form,

Mi=expjΦiVVXPRl-1∙expjΦiVHXPRi-1∙expjΦiHVexpjΦiHH

In such a case, αi,1=αi,2=1, and βi,1 and βi,2 are randomly modelled by a XPR factor, but with the equal consistency, i.e., βi,1≈βi,2. It should be noted that the more randomly the CPM is modelled, the closer XPR values are equal.

However, unlike the polarization modeling for communication channel in TR38.901, the CPM for a target is not fully randomized. This is because in case where the number N is small and the interactive object element is comparatively simple, e.g., N=1 for single scattering sensing target, its CPM can no longer be modelled by a fully randomized form. For example, a sensing target with a small and smooth metallic surface (e.g., the size of surface is less than 10λ), it appears a partial specular phenomenon but with asymmetric reflection angles as opposed to the incident angle. This phenomenon may be incurred on many types of scattering elements, such as a car with metallic surface. This phenomenon is theoretically approved in [20]. In a general case, αi,1≠αi,2≠1 and βi,1≈βi,2≠0, while in a specular case, αi,1≠αi,2≠1 and βi,1≈βi,2≈0.

- At least, αi,1 and αi,2 in the CPM of target is not equally valued, i.e., αi,1≠αi,2.

Since the polarization does not generate any power gain (i.e., + and - gains), the values of αi,1 and αi,2 should be normalized as

αi,12+αi,22=2.

It is noted that instead of the value 1, the normalized value is 2 to keep the consistency with LOS related CPM.

In the CPM of target, the gain factors of αi,1 and αi,2 are unequally valued (i.e., αi,1≠αi,2) with the normalization of αi,12+αi,22=2, while the gain factors of βi,1 and βi,2 are equally modelled (i.e., βi,1≈βi,2).

      - Power related Issues of Concatenation Methodology in Target Channel Modeling

In the #119 meeting, RAN1 made a down-selection from four options for the concatenation of target channel, but still with a critical FFS for further power normalization of target channel performed after ray dropping. According to our simulation-based investigation below, without the power normalization, the difference of angle spread can be severe, that will seriously impact the sensing detection and tracking.

Issue1: How path dropping will impact target channel

Compared to Option-0, as agreed in the last meeting, Option-3 is the simplified version for the concatenation of target channel, by means of dropping the rays from full-convolutional concatenation to one-by-one random pairing. Since each ray is generated by the procedure in TR38.901, the power gain for each ray before concatenation is the same for both options. This incurs the total power reduction in Option-3 by a factor of MM-1, where M is the number of the rays in each cluster. For instance, in case of M=20, the reduced power could reach 26dB. This is unacceptable in ISAC channel modelling.

To this end, first of all, we are attempting to analyze how the difference between cluster dropping (defined in TR38.901) and ray dropping can occur from the perspective of DSP theory.

In the small scale generation procedure of TR38.901, after generating N clusters each with individual delay τn and power Pn, we drop the clusters if their power levels are less than -25 dB compared to the maximum cluster power (not LOS power). Because the cluster power dropping process meets DSP principles, the scaling factors need not be changed after cluster elimination.

To clarify this dropping process, let’s give an example as follows. It is assumed that six clusters in channel hdτn, with the delay of τn and the power of Pn, where n=1,2,3,4,5,6, as depicted in Figure 3-(a). The power level of each cluster is generated according to its corresponding delay, meaning that the larger the delay, the smaller the power level, and vice versa. Therefore, it is in theory that the use of a time window, wt, as depicted in Figure 3-(b), can directly eliminate the rays which behave small power and large delay. As a result, the discrete time channel, hdτn, is approximated to a new channel, hd'τn, with the almost same characteristics but less dispersive, as depicted in Figure 3-(c). Mathematically, the discrete time channel hd'τn after the direct cluster dropping process can be expressed as

hd'τn=0τmaxhdτn×wtdt.

It should be noted that in this case, the effect of maximum delay of τmax is equivalent to the threshold of Pmax, i.e., fPmin=τmax.

Figure 3: A principle of clusters dropping by a time window.

- A cluster with a lower power gain can be directly dropped from the perspective of DSP theory.

In contrast, directly dropping a ray from cluster in a discrete time channel is violated from the perspective of DSP theory. It should be noted that a discrete time channel is not equivalent to a digitalized time channel; the former belongs to a band-unlimited channel, while the latter belongs to a band-limited channel.

For instance, a couple of rays in the n-th cluster with different power levels form a part of discrete time channel, hdτm, as depicted in Figure 4-(a). If we desire to drop a ray with a small power, only way is to use a wideband filter to convert a discrete time channel into a digitalized time channel, as illustrated in Figure 4-(b) for wideband filter and Figure 4-(c) for resultant digitalized time channel. With this DSP process, then, a ray with a small power can be dropped without the need of the power normalization.

Figure 4: A principle of ray dropping by a wideband filter.

It is worthwhile noting that regardless of how many rays exist as long as the different rays have the similar delays (i.e., within a sampling interval), all the rays are converged to single ray with the same total power after filtering. Therefore, directly dropping rays from a discrete channel violates the principle of DSP processing.

- Directly dropping rays from a discrete channel violates the principle of DSP processing.

Nevertheless, there are two drawbacks incurred with this digital processing; one is to increase the channel generation complexity, and the other is to convert the channel to be a bandlimited channel. Obviously, this is not our expectation. To address such an issue, we may approximately drop the rays based on a power threshold and involve the power normalization over the discrete time channel (not include LOS power). It is the DSP process as if the rays with small power are converted into a ray with large power. Consequently, the discrete time channel after the rays with the weak power dropping, its channel characteristics can be approximately kept being unchanged if the power normalization is involved.

Directly dropping rays with a predetermined power threshold before or after concatenation should be considered with the power normalization over the discrete time channel.

In what follows, we conduct a simulation and investigate the power normalization before or after concatenation is necessary. In simulation aspects, we investigate the necessity of power normalization after using Option3 (i.e., Ray level 1-by-1 random coupling) to generate NLOS+NLOS rays of concatenated target channel. We consider INH scenario with the propagation Case2 (LOS + NLOS). The following options are considered in this sub-part:

- Option0: Ray level full convolution between Tx-target link and target-Rx link.

- Option3: Ray level 1-by-1 random coupling between Tx-target link and target-Rx link.

  - Alt0: No normalization.

  - Alt1: The power only associated with the indirect paths of NLOS+NLOS is normalized to X, where X is the total power of the indirect paths of NLOS+NLOS used for full convolution concatenation. This belongs to the power normalization before concatenation.

Figure 5 shows the CDFs as a function of DS, ASD, ZSD, ASA and ZSA.

Figure 5: The delay and angle spread comparison between Option0 and Option3 for propagation case2 (LOS+NLOS).

It can be observed that, Option3 using power normalization and Option0 have same delay and angle spread distribution, while Option3 without power normalization incurs a bit difference in angle spread distribution especially for ASD and ZSD, as compared with Option0.

- The performance gap is huge for Option3 with and without power normalization, especially for ASD and ZSD.

RAN1 further considers how to align delay/angle spread distribution if companies use different methods (e.g., Option0, Option3) to generate NLOS ray + NLOS ray of target channel in ISAC channel model calibration.

Issue2: Whether power normalization is performed after path dropping of target channel

Previously, we have discussed the necessity of the power normalization before or after the concatenation for Option 3 (i.e., one-by-one random pairing). Nowadays, the question may be raised that whether it is necessary to perform the power normalization for Option 0 (i.e., ray level full convolution pairing) after path dropping of target channel. To address this issue, we conduct a simulation campaign, whereby we check its necessity from the results of delay spread and angle spreads.

In the simulation campaign, we evaluate the performance of whether power normalization for different propagation cases. We consider INH scenario with path drop threshold -25dB based on NLOS cluster and use ray level full convolution between Tx-target link and target-Rx link.

Figure 6, Figure 7, Figure 8 and Figure 9 show the CDFs as a function of DS, ASD, ZSD, ASA and ZSA, in case, 1, 2, 3, and 4, respectively.

Figure 6: The delay and angle spread comparison for Case1.

Figure 7: The delay and angle spread comparison for Case2.

Figure 8: The delay and angle spread comparison for Case3.

Figure 9: The delay and angle spread comparison for Case4.

It can be observed that, the CDFs for all the cases are almost perfectly consistent regardless of performing the power normalization.

- Whether performing the power normalization after path dropping of target channel incurs the same delay and angle spread distribution for all propagation Case1/2/3/4 in Option 0.

No power normalization of target channel after path dropping if Option 0 is involved for target channel concatenation.

Issue3: Method and threshold of path dropping after concatenation

In this section, we further investigate how to operate path dropping after concatenation for TX-Target-RX channel in Option 0 (i.e., ray level full convolution pairing). We conduct the simulations with the detailed simulation assumptions listed in Table 1.

Table 1: Simulation details for different path drop methods of target channel.

| Scenario | Indoor office in TR38.901 |
| --- | --- |
| Propagation conditions of target channel | Case1: LOS condition for both TX-Target and Target-RX paths Case2: LOS condition for TX-Target path, NLOS condition for Target-RX path Case3: NLOS condition for TX-Target path, LOS condition for Target-RX path Case4: NLOS condition for both TX-Target and Target-RX paths |
| Concatenation methods | Ray level full convolution between Tx-target link and target-Rx link. |
| RCS modelling | RCS=A*B1*B2=1 |
| Normalization methods after concatenation | No normalization |
| Method of Path-drop after concatenation | Option A: No drop Option B-1: Drop based cluster pair level, where pairs consist of LOS-LOS pair (if present), LOS-NLOS pairs (if present), NLOS-LOS pairs (if present), and NLOS-NLOS pairs Option B-2: Drop based cluster pair level, where pairs consist of LOS-NLOS pairs (if present), NLOS-LOS pairs (if present), and NLOS-NLOS pairs Option B-3: Drop based cluster pair level, where pairs only consist of NLOS-NLOS pairs Option C-1: Drop based ray pair level, where pairs consist of LOS-LOS ray (if present), LOS-NLOS rays (if present), NLOS-LOS rays (if present), and NLOS-NLOS rays Option C-2: Drop based ray pair level, where pairs consist of LOS-NLOS rays (if present), NLOS-LOS rays (if present), and NLOS-NLOS rays Option C-3: Drop based ray pair level, where pairs only consist of NLOS-NLOS rays |
| Threshold used in path-drop after concatenation | -25dB, -35dB, -45dB, -55dB |
| Normalization methods after path drop | Alt A: No normalization |

In Annex C, we collect the simulation results and show the number of reduced rays in the different cases with the different options, where DS/AS distributions are plotted. Based on the simulation results in Annex C, we compare the DS and AS of different path-drop methods with the case of “No drop”. The availability of different path-drop after concatenation methods compared with “No drop” is summarized in Table 2.

Table 2: The availability of different path-drop method compared with “No drop”.

| Path-drop method | Threshold used in path-drop after concatenation | Propagation condition of Target channel |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Case1 | Case2 | Case3 | Case4 |
| Option B-1 | -25dB | × | × | × | / |
|  | -35dB | √ | √ | √ | / |
|  | -45dB | √ | √ | √ | / |
|  | -55dB | √ | √ | √ | / |
| Option B-2 | -25dB | √ | × | × | / |
|  | -35dB | √ | √ | √ | / |
|  | -45dB | √ | √ | √ | / |
|  | -55dB | √ | √ | √ | / |
| Option B-3 | -25dB | √ | √ | √ | × |
|  | -35dB | √ | √ | √ | √ |
|  | -45dB | √ | √ | √ | √ |
|  | -55dB | √ | √ | √ | √ |
| Option C-1 | -25dB | × | × | × | / |
|  | -35dB | × | × | × | / |
|  | -45dB | × | √ | √ | / |
|  | -55dB | × | √ | √ | / |
| Option C-2 | -25dB | × | × | × | / |
|  | -35dB | √ | × | × | / |
|  | -45dB | √ | √ | √ | / |
|  | -55dB | √ | √ | √ | / |
| Option C-3 | -25dB | √ | √ | √ | × |
|  | -35dB | √ | √ | √ | √ |
|  | -45dB | √ | √ | √ | √ |
|  | -55dB | √ | √ | √ | √ |

From the results above, compared to the “baseline, i.e., No drop”, it can be observed that the following factors may affect the delay and angle distributions:

- Different path-drop options after concatenating TX-Target and Target-RX;

- Cluster-based or ray-based path-drop;

- Different threshold levels used for path-drop.

- When dropping path after concatenation in target channel modelling, the path-drop options, cluster/ray-based path-drop and threshold levels may affect the delay and angle distributions.

RAN1 further considers the following factors to drop path after concatenating TX-Target and Target-RX.

- Cluster-based or ray-based path-drop;

- Threshold levels used for path-drop, e.g., -25dB as baseline.

Issue4: Modification of Power generation for cluster in each component of TX-Target and Target-RX.

For concatenated target channel, some modifications are needed when generating cluster power. Specifically, the shadowing terms per cluster are zn1' ~N0,ξ22, zn2' ~N0,ξ22 for Tx-Target path and Target-Rx path, respectively. Then, the cluster powers are calculated as

Pn1,'=exp-τn1∙rτ-1rτDS∙10-zn1'10,

Pn2'=exp-τn2∙rτ-1rτDS∙10-zn2'10.

In turn, the concatenated variable of the cluster power is calculated as

Pn1,n2'=Pn1,'∙Pn2'=exp-τn1∙rτ-1rτDS∙10-zn1'10∙exp-τn2∙rτ-1rτDS∙10-zn2'10

=exp-τn1+τn2∙rτ-1rτDS∙10-zn1'+zn2'10=exp-τn1,n2∙rτ-1rτDS∙10-zn1'+zn2'10

It is noted that the distribution of Pn1,n2' is the same as that of Pn', including the value of each parameter.

RAN1 studies how to generate the cluster power of target channel.

- As a start point, for cluster power generation for concatenation in sensing channel: Use shadowing term per cluster Zn1~N0, ζ22 for Tx-Target paths, Zn2~N0, ζ22 for Target-Rx paths, respectively.

Large Scale Parameter Discussion

      - General Pathloss Model for Concatenation Channel Model

The pathloss of concatenation target channel model can be obtained by modifying the existing pathloss formula defined in TR 38.901. The defined pathloss formula describes a unidirectional link from a transmitter to a receiver, which includes losses due to propagation distance and the effect of the aperture of the receiver's receiving antenna.

Similarly, for pathloss of concatenation target channel model in ISAC, both losses caused by the propagation from a transmitter to an ISAC target and the propagation from the target to a receiver need to be considered. Besides, the pathloss model needs to be unified, applying to different scenarios and sensing modes. Based on our investigation [5], it can be proved that a common pathloss equation can be expressed as:

PLtargetd1,d2=PLd1+PLd2+10log10λ24πpart-1-10log10σRCSpart-2

Eq. 2

Part-1 contains the losses due to two sensing links and the effect of the aperture of the receive antenna. In part-1, PLd represents the pathloss between two nodes and can reuse the formulas defined in 3GPP TR38.901. In case of mono-static sensing model, d1 is equal to d2.

Part-2 contains the effect of the reflected signal from the ISAC target, denoted as σRCS. It is noted that part-2 is related to the RCS modeling methods. According to the RCS related agreement in the #118b meeting, the σRCS can be modelled by the RCS component of σA.

The pathloss formula of Eq. 2 for the concatenation target channel model in ISAC can be regarded as a starting point for the further pathloss model study.

      - LOS Probability Model and Pathloss Model

The formula in Eq. 2 contains two components that the pathloss from transmitter to target and the pathloss from target to receiver, which can be generated by the pathloss model defined in TR38.901. As the agreement-5 listed in section 1, for bistatic, the LOS condition from Tx to target and from target to Rx is determined separately for a target, while for monostatic, the same LOS condition is determined for Tx to target and target to Rx. Furthermore, the LOS condition from Tx to target and/or from target to Rx is determined with the LOS probability.

However, there still exist certain remaining issues, e.g., the correlation of LOS condition of Tx-target and Rx-target links of a target, and the probability schemes in different scenarios, that need to be discussed.

Correlation of LOS condition

It can be reorganized that the distance from Tx to Rx may be used to judge whether to model the correlation between the two links of Tx-target and target-Rx under the LOS condition. When the Tx and Rx approach gradually, both channel links associated with the target become more identical. Thus, the high correlation of LOS conditions may occur. In a special case where the Tx and Rx are in the same location, i.e., mono-static sensing, the correlation of LOS conditions becomes one. Since both bistatic and monostatic sensing modes are considered, the correlation of LOS condition between the two links of Tx-target and target-Rx should be modelled.

The correlation of LOS condition between TX-target link and target-RX link is modelled.

The methodology based on spatial consistency model for the target channel in ISAC is a potential approach to model the correlation of LOS condition in the Tx-target link and target-Rx link for ISAC channel. The detailed discussion can refer to the description in subsection 2.6.

The spatial consistency mechanism can be used to model the LOS correlation in the ISAC Tx-target-Rx channel.

Multi-point channel modelling

In addition to supporting the global motion detection of the sensing target in ISAC use case, the local motion detection of the sensing target is also supported, such as the wing rotation of the UAV, the tire rotation of the automobile, and the swing arm movement of the human body. Therefore, the multi-point model of target should be introduced to support the local motion detection use case.

Multi-point channel model is supported in the R19 ISAC channel model.

For the different sensing target type, different point number can be defined according to the shape of the target or the type of local motion detection use case. Hence, the channel model of TX/RX to different point in the sensing target should be determined.

Regarding the large scale fading of different point in the sensing target to the TX/RX, the same fading value based on the center point of the sensing target can be applied to the different point since the difference of the large scale fading among different point is small due to the limit 3D distance, which can be ignored in the channel model. Therefore, the different points in the sensing target can share the same LOS/NLOS condition and the same large scale fading gain to TX/RX.

However, the small scale fading is effected by the location of the point in the sensing target and the difference of the small scaling fading among different points cannot be ignored. The related link of each point can be generated independently in the same way as that of single scattering point. The enhanced spatial consistency methodology introduced in section 2.6 for ISAC channel model can be applied to the small scale fading calculation for different multi-points. Consequently, some relevant small scale parameters, such as delay and angle, can be shared for the different points according to the real position of each point, while some relevant small scale parameters, such as phase and Doppler shift, can be individually generated according to the use cases.

Multi-point channel model can be determined based on the following principle:

- The large scale fading of different point to TX/RX share the same value calculated by the center point of the target;

- The spatial consistency methodology is applied to calculate to small scale fading of different point in the sensing target to the TX/RX.

RAN1 further studies whether some relevant small scale parameters, such as delay and angle, can be shared for the different points.

RCS Modeling

      - Mono-static RCS for human

As the agreement-3 listed in the section 1, the main RCS model framework of human has been defined that the RCS related coefficient of a scattering point can be modelled as two options by RCS = A×B1×B2, i.e., option 1 corresponding to that B1 equals to 1 in liner scale, and option 2 corresponding to that B1 is depended on incident/scattered angles. In this section, the mathematical model of RCS for human is proposed based on the experiment campaign cooperated with BUPT and the more detailed results are captured in [9]. The measurement of RCS is carried out in the experiment chamber as shown Figure 10, where the RCS of human is measured as a single point.

Figure 10: Measurement environment for RCS.

The measurement parameters are listed in Table 3.

Table 3: The parameters for the RCS measurement.

| Parameter | Value |
| --- | --- |
| Frequency | 4.9GHz, 10GHz, 20GHz, 28GHz, 36GHz |
| Sensing mode | Mono-static |
| Distance of TX-target | 12m |
| Height of TX antenna | 2m |

Figure 11 and Figure 12 show the RCS patterns measured from the different angles and CDF of the measured RCS at the various operation frequencies, e.g., 4.9GHz, 10GHz, 20GHz, 28GHz, and 36GHz, respectively.

Figure 11: RCS pattern measured in different operation frequencies.

Figure 12: CDF of the measured RCS at the various operation frequencies.

Two observations can be made for the mono-static sensing mode as follows:

- The averaged RCS value can be modeled to be a constant RCS value, independent of the angles, but with certain random variation. The variation can be simply formulated by a stochastic normal distribution with a mean and a standard deviation, where the mean value can be represented by the component A in [m2] and the standard deviation can be represented by the component B in the RCS model framework of RCS=A×B.

- There is a strong relation between the RCS and the operation carrier frequency, i.e., the higher the carrier frequency, the higher the RCS measurement value.

In case of the bi-static sensing mode, it is the evidence in subsection 2.1.4.2 that there is no explicit relationship between the RCS values measured by the bi-static sensing mode and the mono-static sensing mode. This implies that the RCS model for human can be unified between the bi-static sensing mode and the mono-static sensing mode. Therefore, the RCS mathematical models with normal distribution for human at various operation frequencies are summarize in Table 4.

Table 4: The RCS mathematical model with normal distribution for human at various operation frequencies.

| Frequency | Mean | Standard deviation |
| --- | --- | --- |
| 4.9GHz | -5.6785 | 2.2575 |
| 10GHz | -4.4679 | 2.7437 |
| 20GHz | -3.9933 | 2.4178 |
| 28GHz | -1.7976 | 2.6104 |
| 36GHz | -0.1655 | 2.1442 |

According to the above analysis, the RCS model is impacted by the carrier frequency. Based on the outcomes summarized in Table 4, the mean value varies almost linearly dependent on the carrier frequency while the standard deviation varies randomly. Figure 13 illustrates the matching curve of the RCS mean as a function of the carrier frequency for the human as a target.

Figure 13: The matching curve of the RCS mean as a function of the carrier frequency for the human.

Consequently, the RCS for human can be modelled as the log-normal distribution with the mean value corresponding to the component A, mathematically formulated as

Ahuman=0.0029f2+0.0538f-5.7748

Eq. 3

The standard deviation corresponding to the component B2 can be defined by the average value of measured results, where

B2human=2.44

Eq. 4

The Mono-static RCS of human can be unified based on RCS=AB1B2.

- The mean value represented by A in Eq. 3 is considered as a starting point for the human RCS, which is independent of the incident and the scatter angle, but dependent on the frequency band.

- B1 equals to 1.

- The variance corresponding to component B2 in Eq. 4 is considered as a starting point for the human RCS, which is independent on the incident and scatter angle.

      - Bi-static RCS for small UAV and human

We also conducted the experiments cooperated with BUPT to demonstrate the relationship of RCS between the mono-static and the bi-static with the different angles, whereby the applicability of the mathematical model can be validated. The measurement of RCS is carried out in the experiment chamber as shown in Figure 14. The distance from the Tx to the target is 6m and the measurement frequency is 28GHz. The angle of bi-static varies from 0˚ to 20˚ with the angle granularity of 2˚in different mono-static measurement angle.

| Figure 14: The measurement environment with the target of small UAV. |  |
| --- | --- |
| Figure 15: The measurement result of the RCS in mono-static and bi-static. |  |

The experimental results are shown in Figure 15, where the dashed lines parallel to the x-axis are the RCS measurement results of mono-static, and the red dots correspond to the RCS measurement values of the bi-static at different angles. From the measured results of the bi-static RCS, it can be observed that the value of the bi-static RCS does not show the dependency of the angles, only with the up and down random variation between the angles of 0˚~20˚.

In consideration of the various and complex objects in ISAC, therefore, the mono-static and bi-static RCS model of small UAV can share the same mathematical model. Due to the similar RCS characteristics with small UAV, the bi-static RCS model of human can also be applied by the mathematical model of its mono-static in a simple methodology.

The bi-static and mono-static RCS model share the same mathematical model at least for small UAV and human.

      - Unified bi-static RCS with elliptical model

In the #119 meeting, RAN1 made several RCS-related agreements mainly for mono-static sensing mode as listed in Agreement 3, 4 and 5 in Section 1. Although FL is planning to discuss this issue in the next coming meeting, yet RCS for bistatic sensing mode seems still premature and needs somewhat consolidated solution to be realized. We prefer to utilize a unified RCS function for both mono-static and bi-static sensing modes with a set of different parameter values corresponding to different type of target.

In what follows, we are attempting to propose a mechanism unified for both mono-static and bi-static RCSs.

Prior to elaborating the unified RCS solution, let’s briefly discuss the definition of roughness for a regular surface. According to Rayleigh criterion, an object surface is considered smooth if its minimum to maximum protuberance (i.e., bumps) h is less than the critical height hc, i.e., h<hc, where

hc=λ2cosθi.

where θi is the incident angle and λ is the wavelength.

Obviously, the roughness depends on h, λ, and θi. As an illustration, the surface can be seen as smooth one in Figure 16-(a), while the surface can be seen as rough one in Figure 16-(b).

Figure 16: An illustration of Rayleigh criterion-based surface roughness.

Scattering path loss is another important factor that may be related to the RCS definition. Scattering path loss factor ρs is mathematically given by

ρs=exp-8πσhsinθiλ2

Eq. 5

where σh is the standard deviation of surface height about mean surface height. For rough surface, the flat surface reflection coefficient Γ (i.e., specular reflection) is multiplied by scattering loss factor ρs to account for diminished electric field. Thus, the reflected E-fields for h>hc for rough surface can be formulated as Γrough=ρsΓ. It should be noted that in case of θi=0°, the scattering loss factor ρs is equal to 1, meaning no loss at all.

In case of specular reflection on a smooth surface as depicted in Figure 17-(a), the phenomenon of the electric field spread does not occur, namely, the electric field is only reflected towards the reflection angle θr, where θi=θr. If the surface is a bit rough, i.e., h>hc, as depicted in Figure 17-(b), the electric field diffusion occurs, but with a small spread factor. If the surface is rough enough, i.e., h≫hc, as depicted in Figure 17-(c), however, the electric field diffusion highly occurs, with a large spread factor.

Figure 17: An illustration of electric field spread.

It can be envisioned that the shape of the diffusion can be circular or elliptical, dependent on the roughness on the surface. For example, if the surface is irregularly and severely rough, the diffused electric field could be orientated to any direction, in the range of 0,π, with a uniformly distributed amplitude. Thus, a circular shape can be used to model such a diffusion phenomenon. Naturally, once the surface becomes less and less rough, the circular shape gradually changes to the elliptical one. Furthermore, once the surface is smooth enough, the elliptical shape finally converges to a line, which corresponds to the specular reflection.

On the other hand, RAN1 agreed on the RCS model with two components: one for large scale denoted as σA and the other for small scale denoted as σB. Furthermore, the RCS for small scale is divided into two sub-components, denoted as σB1 and σB2, respectively. The former is related to incident and scattering angles and the latter is to the RCS variation with zero mean and a certain standard deviation. It is believed that among the parameters, the biggest issue on RCS is how to model the sub-component of σB1, in consideration of the incident and scattering angles, θi and θr.

To this end, we are trying to model the angle related RCS for small scale, by means of mathematical ellipsoids, according to the principle of Rayleigh criterion-based surface roughness and the behavior of electric field spread.

Mathematically, as depicted in Figure 18, the equation of ellipsoid in Cartesian coordinate system is expressed by

x2a2+y2b2+z2c2=1

where a, b and c are the length of the semi-axes.

Figure 18: An illustration of half-ellipsoid in Cartesian coordinate system over a surface.

In spherical coordinate system for which

x,y,z=rsinθcosϕ,rsinθsinϕ,rcosθ,

the general equation of ellipsoid can be transferred to

rsinθcosϕ2a2+rsinθsinϕ2b2+rcosθ2c2=1.

Eq. 6

It should be noted that once zenith angle θ and the azimuth angle ϕ are given, the distance of r can be derived accordingly. Thus, we believe that this distance of r can be utilized to model the sub-component of σB1.

For mono-static sensing mode, based on the general equation of ellipsoid, the incident and scattering angles can be all assumed to be zenith angle θ and the azimuth angle ϕ, so that the distance of r as mono-static RCS can be derived accordingly. Thus, the remaining task is to determine the values for the parameters of a, b and c associated with the different type of target, based on the experiment measurements.

The benefits to use the equation of ellipsoid is, regardless of the reflection types, the related RCS component can be fulfilled by single equation. In special case where either specular reflection or omni-direction reflection occurs, it can be realized

- by setting a,b≪c; it tents to specular reflection and only in case of θi=θr,ϕi=ϕr, its RCS is σB1θi=θr,ϕi=ϕr≈1, otherwise, σB1≈0.

- by setting a=b=c; it means that the RCS σB1θi,ϕi,θr,ϕr=1 for omni-direction reflection, independent of the incident and scattering angles.

To derive the RCS component of bi-static sensing mode, using the same equation of ellipsoid, we need to rotate the semi-ellipsoid along the direction of specular reflection, i.e., the angles of θr and ϕr, as illustrated in Figure 19. Thus, the procedure of calculating σB1 can be as follows:

- Perpendicularly place the semi-ellipsoid over each surface based on the target shape.

- Set the values of a, b and c according to the same mono-static RCS, based on the type of object, such as size, roughness and material, e.g., a,b≤1 and c=1.

- Rotate the ellipsoid in the spherical coordinate system according to the specular scattering angle as

θrot=-θr      and     ϕrot=π2-ϕr;

θr=θi    and    ϕr=ϕi+π  mod 2π.

- Change the parameter of the semi-ellipsoid, c, to be c→c∙ρs, where ρs is the scattering loss factor defined in Eq. 5, dependent on the incident angle of θi and the standard deviation of surface height σh. Thus, the Eq. 6 can be modified to

rsinθcosϕ2a2+rsinθsinϕ2b2+rcosθ2cρs2=1.

Eq. 7

- Calculate the distance of rr', based on the scattering angle of θr' and ϕr' in any direction from Eq. 7.

- Derive the small scale RCS component σB1, based on

σB1θi,ϕi,θr',ϕr'=rr'α.

where α is the scaling factor to fit the RCS between the experiment measurement and the equation of ellipsoid, e.g., α=1,2,⋯.

Figure 19: An illustration of rotating the half-ellipsoid along the direction of specular reflection in Cartesian coordinate system over a surface.

The benefits to use the rotated equation of ellipsoid is, regardless of the reflection types and mono-static/bi-static modes, the related RCS can be fulfilled by single equation. In special case where either specular reflection or omni-direction reflection occurs, it can be realized

- by setting a,b≪c; it tents to specular reflection, thus, σB1θi,ϕi,θr'=θr,ϕr'=ϕr≈1, otherwise, σB1≈0.

- by setting a=b=c; it means that the RCS σB1θi,ϕi,θr',ϕr'=1 for omni-direction reflection, independent of the incident and scattering angles.

RAN1 considers the equation of ellipsoid(s) unified for both mono-static and bi-static sensing modes, in consideration of the rotation of the ellipsoid along the direction of specular reflection.

In order to investigate whether the equation of ellipsoid can be unified for both mono-static and bi-static sensing modes, we conduct the experiment campaign with BUPT in a large enough experiment chamber, as shown in Figure 20. The experiment assumptions are listed in Table 5.

Figure 20: The measurement environment with the car and UAV targets.

Table 5: Experiment assumptions for the velocity of vibration or swing in case of human.

| Parameters | Descriptions |
| --- | --- |
| Measuring equipment | VNA |
| Frequency | 15 GHz |
| Bandwidth | 3 GHz |
| Tx / Rx Antenna Types | Horn/Horn |
| Polarization | VV / VH / HV / HH |
| Distance btw Tx/Rx d | 6m, 12m |
| Test way | Target on the turntable 360˚ clockwise，5˚ per test |

According to the experiment assumptions listed in Table 5, firstly, we conduct the experiment measurements only in the horizontal dimensional RCS. We try to use four semi-ellipses to fit the σB1 components of vehicle and UAV, in consideration of mono-static sensing. The fitted results in dB scale are shown in Figure 21.

Figure 21: The RCS fitting between the experiment measurement and the semi-ellipse-based calculation.

It can be observed that for both UAV and vehicle, it fits σB1 component well, with three semi-ellipses, due to the symmetrical property from two sides of object.

- The experiment measurements fit semi-ellipse-based calculation well for both UAV and vehicle with three semi-ellipses in mono-static sensing mode.

To realize the experiment campaign of RCS component σB1 for bi-static sensing model, as illustrated in Figure 22, it is assumed that the locations of the Tx and the car as a target are fixed over a large disk, and the disk is rotated at very certain small angle during the measurements. This means that the incident angle is kept unchanged but with three different scenarios, respectively for the incident on the car body, the incident on the car rear, and the incident on the car head, as shown in Figure 22-(a), (b) and (c). The Rx is fixed over quarter-circle frame, moving end to end during the measurements to measure the reflected electric fields from the different scattering angles, both zenith and azimuth angles.

Figure 22: 3D vehicle dual-station sensing measurement setting in bi-static sensing mode.

For the fitting process, we take the measurement data in both horizontal and vertical dimensions, as shown in Figure 23-(a), (b) and (c), respectively, corresponding to three different scenarios.

Figure 23: 3D vehicle dual-station sensing measurements in bi-static sensing mode.

It can be observed that for vehicle, the measurement results fit σB1 component well, with three semi-ellipsoids, in three different car surfaces.

- The experiment measurements fit semi-ellipsoid-based calculation well with three semi-ellipsoids and three different car surfaces in bi-static sensing mode.

In case that multiple semi-ellipsoids are utilized to model single RCS σB1, we need to sum all the RCS components together in linear scale. Assuming the target consists of L surfaces, at most, L semi-ellipsoids are required to model overall RCS of the target. Hence, the overall RCS σB1(all)θi,ϕi,θr,ϕr can be formulated as

σB1(all)θi,ϕi,θr,ϕr=l=0L-1σB1,lθi,ϕi,θr,ϕr

where σB1,lθi,ϕi,θr,ϕr is the RCS of the target for the l-th semi-ellipsoid in the incident angle of θi,ϕi, and the scattering angle of θr,ϕr.

The overall RCS should be summed from the individual RCSs modelled by the corresponding semi-ellipsoid over each surface of the target.

      - Multi-point RCS model

As mentioned in section 2.1.3, the multi-point channel model of sensing target can be adapted to detect the local motion of the sensing target in ISAC. Therefore, the multi-point RCS model is also essential for the ISAC channel model. In the multi-point RCS model, the sensing target can be divided into different part and each part has the different RCS model.

RAN1 support the multi-point RCS model in R19 ISAC channel model.

However, the number of the scatter points in multi-point model should be unified for all the use case of local motion detection. For instance, the UAV can be divided into five scatter points where one point is corresponding to the center of the UAV and the other four points are corresponding to the four airfoils. The human can be divided into six scatter points where four points are referring to the legs and hands, and the others two points are referring to the head and the abdomen of human.

In the multi-point RCS model, the UAV is divided into 5 points, i.e., airframe body and four airfoils.

In the multi-point RCS model, the human is divided into 6 points, i.e., head, abdomen, two legs and two hands.

For multiple scattering points modelling, the RCS of each point can also be realized by the same way as single point scatter. Due to the multiple points belonging to the same target, the RCS of multiple points can either adopt the same value as a special case or generate RCS value independently for each point to differentiate the different reflects from a target.

To verify the relationship between the overall RCS (corresponding to single scattering point) and the partial RCSs of a target (corresponding to multiple scattering points), we conduct an experiment campaign. In the campaign, a human is considered as a target. We consider two instances for our experiment. In the 1st instance, the whole body including the head, abdomen, hands and legs is measured. In the 2nd instance, only the part of the human body (e.g., head or abdomen) is exposed while the other part of human is shielded by a wave-absorbing material. The measurement frequency is 28GHz with the mono-static sensing mode. The results are shown in Figure 24.

Figure 24: RCSs of the head, the abdomen, and the whole body from left to right, respectively.

According to the experiment results, the mean value of the RCS for the whole human is -8.24dBm2. The mean value of the RCS for the head is -16.16dBm2 and the mean value of the RCS for the abdomen is -12.4dBm2. It can be easily observed that the RCS of the whole human is larger than the RCS of each part of body. The RCS for the whole body in linear scale is 0.15 m2, and the summation of RCS value from the head and the abdomen in linear scale is 0.082m2. If additionally considering the RCS of hands and legs, it can be predicted that the RCS of whole body is equivalent to the linearly summed RCS from the segmented parts of the body. Therefore, the following equation holds

RCStarget=l=1LRCSpart,l

where RCStarget is the RCS measured from an entire target, and RCSpart,l is the RCS measured from the l-th part of the target and L is the number of parts segmented from the entire target.

This implies that the RCS conservation law should be ensured in the RCS modeling, between single scatter point and multiple scatter points.

It should be noted that the number of parts segmented from the entire target, L, could be considered as the number of multiple scattering points for RCS.

The RCS value of the entire target is the linear summation of the RCSs from the segmented parts of the entire target.

It is noted that the different parts in a target having the similar behaviors should share the same RCS model. For instance, the four airfoils of UAV should have the same RCS model, and the legs or hands of a human should share the same RCS model.

The two legs or two hands can be modelled by the same RCS.

The four airfoils of UAV can be modelled by the same RCS.

EO Modeling

To investigate the energy gap in the indoor scenario, we conduct an experiment campaign for the EO type-2. To ensure the occurrence of specular phenomenon, we need to very carefully set the Tx and Rx in the room, in consideration of wall, ground, and ceiling. To this end, we carefully manipulate the positions of Tx and Rx in the room, and make sure the specular paths not to be reflected to the Rx at the same time. As the experiment assumption, therefore, the distance between the Tx and Rx is 2.7m, and the distance between the Tx and Rx and the distance from Tx/Rx to each wall is different. The antennas pattern of Tx and Rx are omnidirectional with the frequency band of 4GHz and a transmission bandwidth of 400MHz.

Figure 25 shows the experiment scenario and the measured amplitude in indoor, including the specular paths, respectively.

Figure 25: The experiment scenario in the indoor scenario (left), and the amplitude including LOS, specular path and others (right).

It can be observed that the strongest peak represents LOS path between Tx and Rx, followed by several strong peaks representing the reflected specular paths from the various walls. The gap between the LOS path and the specular paths is about 10dB, while the gap between the specular paths and non-specular paths is about 5dB.

- In case that the specular phenomenon occurs, the amplitude gap between LOS path and specular paths is about 10dB, while the gap between the specular paths and non-specular paths is about 5dB.

Nevertheless, the EO modelling can be done after the target modeling is consolidated. As we discussed in subsection 2.1.1.1, EO type-2 can follow up the characteristics of object type-2.

For type-2 EO, the indoor and the urban grid scenario are prioritized for the type-2 EO modelling.

The detailed EO modelling can be done after the target modelling is consolidated.

According to the agreement-7 in the last meeting [3], when EO type-2 is modelled, specular reflection is considered to model EO type-2. The effect of type-2 EO in the node1-EO-node2 path is modelled as

bR∥specular00-R⊥specular

where b is a scaling factor, e.g., b equals to d3D/dGR relative to LOS ray in section 7.6.8 in TR 38.901.

Regarding the introduction of the scale factor b, the influence of the true path loss of the reflection path has been taken into account, so that the Tx-Rx path loss can be used uniformly for power calculation after the whole small scale channel is generated, which is more convenient. Therefore, the value of b equaling to d3D/dGR does not need to be updated. Regarding the R∥specular and R⊥specular, the electromagnetic properties of several materials have been defined in TR38.901, which is sufficient to support subsequent evaluation work.

Furthermore, the orientation of EO and the size of the EO only effect the location of reflection. No update needs to be introduced. The normal methodology defined in section 7.5 of TR38.901 is used when there is no reflection point in the EO due to the special orientation or the finite size of EO.

- There is no impact by introducing the specular reflection model when the orientation and the finite size of EO is taken into account.

RAN1 reuses the parameters of b, R∥specular, and R⊥specular without any updating.

Background Channel Modeling

Regarding the channel model of background, the basic scenario without EO modelling should be prioritized. As shown in Figure 26, the background channel consists of the LOS ray from Tx to Rx and NLOS ray through B1. The LOS ray and the NLOS ray in the background channel can be generated based on the procedure as defined in TR38.901 since it is the most directly method with minimal modification.

Figure 26: The sensing channel model without EO modelling

When the EO is modelled in the sensing channel model, as shown in Figure 27, on the other hand, the background channel contains the links from Tx to EO and EO to Rx, in addition to the LOS and NLOS link from Tx to Rx. Regarding the link of Tx to EO and EO to Rx, it can be modelled as a deterministic link with the known location of EO, which introduces extra complexity and burden for background channel model. It should be further studied that whether or how much the modelling of a deterministic EO is beneficial to the sensing, and how much the complexity and the burden in channel modelling should be taken to model the EO.

Figure 27: The sensing channel model with EO modelling.

The channel model of background should consider both factors as mentioned above. Nevertheless, the condition considering the EO modelling should be discussed after the basic modelling without EO modelling is consolidated.

The basic case without EO modelling for background channel should be considered with a high priority.

Background Channel for Bi-static Sensing Mode

According to the agreement list in the section 1, the background channel for bi-static sensing mode has been defined based on corresponding TR for different scenario for each target type. One remaining issue is whether/how to resolve the inconsistency between TRP-TRP channel according to TR 38.858 and the TRP-target (UAV) channel according to TR 36.777 when UAV and TRP are set to same height.

It is common understanding that the channel model for UAV scenario defined in TR36.777 is modified based on the channel model defined in TR38.901 by setting a height breakpoint named Hbreak_point of UAV for the large scale fading. The channel model for TRP-UAV link reuses the channel model defined in TR38.901 when Hbreak_point is lower than 22.5m, 22.5m, and 10m for UMa, UMi, and RMa, respectively. Otherwise, the new channel model defined in TR36.777 is used for TRP-UAV link. The modified parameter for TRP-UAV link in small scale fading mainly contains DS, ASA, ZSA, ZSD and K factor. Furthermore, the height of TRP is 25m, 10m, and 35m in UMa, UMi, and RMa scenarios, respectively, which implies that the channel of TRP-UAV is generated based on the updated parameters in large scale and small scale fading while the channel of TRP-TRP link is generated based on the previous defined in TR38.901, resulting in the inconsistency channel coefficient between the two links.

Based on this principle, it seems available that the TRP-TRP link can be generated based on the parameters defined in TR36.777 and modify the hUE = 25m, 10m, and 35m in UMa, UMi, and RMa scenarios, respectively.

The channel of TRP-TPR link is generated based on TR36.777 in UAV scenario.

Channel Combination

Given that two majority method for channel combination have been selected in the last meeting according to the agreement-9 in the section 1. Method 1 is based on the direct summation of target channel and background channel without power normalization, while Method 2 is achieved based on power conservation principle by the summation of target channel and background channel or replacement of cluster in the background channel within the same power. In this section, Method 1 and Method 2 based on replacement of cluster in the background channel are discussed.

Some channel measurement of combined ISAC channel with BUPT has been made before the selection of combination methodology. Referring to the contribution of BUPT, the experiment is made in the indoor scenario under the bi-static sensing mode [9]. The TX and RX is under the LOS condition and the obstacles are in the experiment environment. Such a channel model can be considered as background channel. Once the sensing target of a metal plate is added, the channel should be modelled as combined channel model. The receiver power in all the direction at RX side is recorded under the two different conditions: one without sensing target and the other with sensing target.

The RX power measurement result shows that the receiver power characteristic and the power level in the direction of sensing target location is changed compared to the power distribution of background scenario. The reason is that the sensing target will block the reflection and diffraction path of the electromagnetic wave for some objects in the background channel, and the ability of reflecting and scattering electromagnetic wave between the sensing target and the blocked object is different. This incurs the unignorable changes in the power level and power distribution at the receiver side compared to the background channel. This also implies adding a sensing target is likely to replace one cluster in the background channel, but the power level of replaced cluster and sensing target channel is different.

It can be predicted that the necessity of replacing a cluster in the background channel is highly dependent on how many obstacles in the background channel, i.e., the more the obstacles, the higher the necessity. Therefore, it is desirably that the replacement of cluster should be applied in most of the current scenarios defined in agenda 9.7.1, such as UMa and indoor office scenario. As analyzed above, in this condition, the power normalization is not needed due to the different ability of reflecting and scattering electromagnetic wave between the sensing target and the blocked object. Consequently, the combined channel with each sensing target can be modelled by replacing a cluster in the background channel while the power of the target channel is summed to the background channel without power normalization.

However, in some scenarios with limited obstacles, adding a sensing target may not change the original channel characteristic, but resulting in an increasing receive power level due to the electromagnetic wave reflection and scattering by the sensing target, such as UAV. Therefore, the combined channel can be modelled by the direct summation of the sensing target channel and the background channel without power normalization.

Consequently, it can be concluded that the power normalization is not required either in summation method or in replacement method in the channel combination step.

Power normalization is not required in the channel combination procedure.

Based on the previous analysis, two major methods are possible. One is the direct summation from the target channel and the background channel, which is desirable in case where background is simple, such as UAV related scenario. The other is based on the replacement by the target channel, which is desirable in case where the background is complex with obstacles, such as indoor related scenario. Hence, what exact method should be used for channel combination is dependent on the deployment scenarios. The ISAC scenario has been defined in agenda 9.7.1 for different sensing target type. The channel combination in different scenarios can be classified in Table 6.

Table 6: Channel combination method classification for different scenarios.

| Combination method | Scenario |
| --- | --- |
| Direct summation method | UAV-UMa, UAV-UMi, UAV-RMa, Highway |
| Replacement method | UMa, UMi, RMa, Urban grid, Indoor office, Indoor room, Indoor factory |

Channel combination method selection should be based on scenario and Table 6 can be a starting point.

According to the discussion in RAN1 so far, the number of clusters for each link follows what TR38.901 defines, meaning that that the number of the clusters in either link concatenated with a target channel model is the same as background channel for each scenario. For instance, the number of the clusters for TX-target or target-RX link in target channel is 20 and 12 under NLOS condition and LOS condition, respectively. The total number of the clusters will increase up to several dozen although a path dropping is performed after concatenation. The total number of the clusters for the combined channel may be in the hundreds if multi-target channels are combined to the background channel, which introduces extremely high complexity.

Moreover, the multi-point channel is considered to support the multi-point RCS model. The total number of clusters in the combined channel could be extremely large under such a condition. Therefore, RAN1 should consider reducing the ISAC channel model complexity based on, e.g., reducing the number of clusters for the TX-target or target-RX link in target channel.

RAN1 should consider reducing the ISAC channel model complexity by reducing the number of clusters for TX-target and target-RX link in target channel.

Doppler Enhancement

There are two types of Doppler effects induced by transmitter, receiver (or observer) and/or target. One belongs to macro-Doppler induced by its motion with the velocity, and the other belongs to micro-Doppler induced by mechanical vibration, rotation, coning, swinging, tumbling and others in addition to its bulk translation. In the #119 meeting, RAN1 agreed on the unified Doppler formula for a target including both macro-Doppler and micro-Doppler.

In TR38.901, the effect of macro-Doppler has been well defined, while the effect of micro-Doppler is not yet discussed so far. With respect to the effect of micro-Doppler, if the target or any structure on the target has mechanical vibration or rotation in addition to its bulk translation, it might induce a frequency modulation on the returned or reflected signal that generates sidebands about the target’s Doppler frequency shift. Micro-Doppler signature-based target can be classified into vehicle, helicopters, UAV, human motion, and animal motion for ground moving subjects.

In this section, we investigate the behavior of micro-Doppler frequency, and conduct an experiment campaign, demonstrating that for some use case, e.g., human, micro-Doppler can be formulated by a simple form.

Micro-Doppler Formula for Vibration

In [16], it suggested the different micro-motion formulas for further study on top of the unified Doppler formula. However, the formulas associated with the micro-Doppler induced by mechanical vibration, rotation, coning, swinging, and tumbling are much more complex than what we defined for macro-Doppler. In this section, as a starting point, we focus the discussion only on the case of the vibration which can induce the micro-Doppler and derive the relative micro-Doppler formula based on the mono-static sensing mode.

Figure 28 illustrates the mono-static transmitter/receiver and vibrating scatterer point P, where the mono-static transmitter and receiver are located at a center point O at the origin of the coordinate system x,y,z and a point scatterer vibrating at a center point O' at the coordinate system  x',y',z'. The distance between two center points at different coordinate systems is R0. It is assumed that the center point O' is stationary with respect to the mono-static transmitter/receiver. At the origin of the coordinate system x,y,z, the azimuth and elevation angle of the point O with respect to the mono-static transmitter/receiver are ϕ and θ, respectively, and at the coordinate system x',y',z', the azimuth and elevation angle of the point O' with respect to the mono-static transmitter/receiver are ϕ' and θ', respectively.

Figure 28: Illustration for mono-static transmitter/receiver and vibrating scatterer point P.

According to trigonometric function calculation, the coordinate system of the point O' can be derived as

R0cosϕcosθ,R0sinϕcosθ,R0sinθ

It is assumed that the scatterer point, P, is vibrating at an angular frequency ωv with a maximum amplitude Dv. The vector from the mono-static transmitter/receiver to the scatterer point, P, becomes

R=R0+D.

Thus, the distance R from the mono-static transmitter/receiver to the scatterer point P can be calculated as

R=R=R0cosϕcosθ+Dcosϕ'cosθ'2+R0sinϕcosθ+Dsinϕ'cosθ'2+R0sinθ+Dsinθ'2

Since R0≫D, and in general, the azimuth angle ϕ of the center point O and the elevation angle θ' of the scatterer point P can be set to zero, i.e., ϕ=0, and θ'=0, the distance R can be approximated to

R≈R0+Dcosϕ'cosθ.

Because the amplitude of the vibration varies at time t, the instantaneous amplitude D can be generalized as D=Dvcosωvt, the instantaneous distance from the mono-static transmitter/receiver to the scatterer point can be written as

Rt=R0+Dvcosωvtcosϕ'cosθ.

where ωv is the angular frequency of scatterer point P due to its vibration.

With the mono-static sensing mode, hence, the instantaneous phase of the received sensing signal at time t due to the vibration of the scatterer point becomes

ΦRt=-2×2πRtλ

By using the time derivative, the micro-Doppler frequency shift induced by the vibration of scatterer point is obtained

fDt=12πdΦRtdt=-2λ∙dRtdt=-2λ∙dR0+Dvcosωvtcosϕ'cosθdt.

Furthermore, it yields

fDt=2λωvDvsinωvtcosϕ'cosθ.

Eq. 8

where θ is the elevation angle of the point O at the origin of the coordinate system x,y,z, and ϕ' is the azimuth angle of the point O' at the coordinate system x',y',z'.

It should be noted that in general, the antenna is equipped with a tilted angle, which can be represented by the elevation angle θ, while the target as a vibration scatterer point P can be towards any direction in horizontal, which can be represented by the azimuth angle ϕ'.

- If assuming the azimuth angle ϕ of the center point O and the elevation angle θ' of the scatterer P can be set to zero, i.e., ϕ=0, and θ'=0, the micro-Doppler frequency shift induced by the vibration of a scatterer point can be derived by Eq. 8.

Human arm or leg vibration behaves a minor difference from what we discussed above in the theoretical vibration scenario. As depicted in Figure 29, the centre point O' of vibrating is different from the center point O″ which is derived from two ending vibrating points connected by a straight line.

Figure 29: The vibration scenario for human arm and leg.

However, if the vibration angle ϑ is not that large, it can be assumed that two center points are approximately overlapped with each other. The amplitude yx at any center point can be simply calculated as

yx=xsinϑ 2,

that results in the maximum amplitude Dv as

Dv=Dasinϑ 2.

To precisely model the amplitude for each part of arm/leg (e.g., from shoulder to hand in arm), we can utilize either the instantaneous amplitude yx, or the average amplitude Dv.

The average amplitude Dv, can be derived by taking the integral of yx from the range 0,Da,

Dv=1Da0Daxsinϑ 2dx=Da2sinϑ 2

Therefore, according to Eq. 8, the micro-Doppler frequency shift induced by the human arm/leg vibration can be averaged over the entire arm/leg

fDt=2λωvDvsinωvtcosϕ'cosθ.

Eq. 9

- The micro-Doppler frequency shift induced by the human arm/leg vibration can be calculated by either the instantaneous amplitude or the average amplitude.

From the perspective of the statistical performance evaluation, the formula for calculating the micro-Doppler frequency shift based on the average amplitude Dv is preferable in the human arm/leg vibration scenario.

RAN1 agrees on the formula of micro-Doppler frequency shift in Eq. 8 or Eq. 9 at least for the human arm/leg target with the vibration of a scatterer point and the mono-static transmitter/receiver.

Channel Modeling for Micro-Doppler

In order to investigate whether micro-Doppler is possibly formulated by the form defined in Eq. 8 for the velocity of vibration or swing with a single scatterer point, we conduct the experiment campaign with BUPT in a large enough experiment chamber, as shown in Figure 30. The experiment assumptions are listed in Table 7.

Figure 30: The measurement environment with the human target for micro-Doppler.

Table 7: Experiment assumptions for the velocity of vibration or swing in case of human.

| Parameters | Descriptions |
| --- | --- |
| Sensing target | Human body (Height 175~180cm) |
| Frequency bands | 6.7GHz, 15GHz, 28 GHz, and 36GHz |
| Frequency bandwidth | 600MHz |
| Sensing mode | Mono-static |
| Tx/Rx angles | Elevation angle and azimuth angle at Tx: ϕ=0 and θ=0 |
| Target angles | Elevation angle and azimuth angle at Tx: ϕ'=0 or 90°, and θ'=0 |
| Antenna types | Horn antenna in both Tx and Rx |
| Antenna height | 85~90cm (direct to the centre of human body) |
| Moving direction | The human body is facing or perpendicular to the transmit and receive antennas |
| Measurement types | Case-1: Single arm periodic swing Case-2: Both arm periodic swing Case-3: Single leg periodic swing Case-4: Both leg periodic swing Case-5: Walking on the treadmill Case-6: Walking on the ground |

It is worthwhile noting that the purpose of this experiment measurement is to observe the micro-Doppler phenomenon of different actions in different frequency bands and verify the formula of Eq. 8 or Eq. 9 for the velocity of vibration or swing.

According to the experiment assumptions listed in Table 7, in case where the human body with the moving direction is facing the transmit and receive antennas, the micro-Doppler frequency shift based on the average amplitude Dv can be simplified to

fDt=2λωvDvsinωvt

However, when the human body with the moving direction is perpendicular to the transmit and receive antennas, fDt=0.

In the measurement campaign, as listed in Table 7, two human moving directions and six cases are investigated, and the detailed measurement results for Case-1 ~ Case-6 are consolidated into Annex B which is also captured in R1-2408263 [9]. Accordingly, the important observations can be summarized as follows:

- When the human body with the moving direction is facing the transmit and receive antennas, the sinusoidal function fluctuations due to the micro-Doppler are obvious, for all the six cases.

- When the human body with the moving direction is perpendicular to the transmit and receive antennas, the sinusoidal function fluctuations due to the micro-Doppler are not that obvious, for all the six cases.

- Micro-Doppler behavior for arm periodic swing is more obvious than that for leg periodic swing.

- Different frequency bands influence micro-Doppler differently, i.e., the higher the frequency band, the more influence on micro-Doppler.

- The moving direction of human and frequency bands influence the micro-Doppler:

- When the human body with the moving direction is facing the transmit and receive antennas, the sinusoidal function fluctuations due to the micro-Doppler are obvious, for all the six cases.

- When the human body with the moving direction is perpendicular to the transmit and receive antennas, the sinusoidal function fluctuations due to the micro-Doppler are not that obvious, for all the six cases.

- Micro-Doppler behavior for arm periodic swing is more obvious than that for leg periodic swing.

- Different frequency bands influence micro-Doppler differently, i.e., the higher the frequency band, the more influence on micro-Doppler.

With such an assumption, in what follows, we are trying to validate whether the formula of micro-Doppler frequency fits the experiment results, only in consideration of micro-Doppler induced by single human arm.

It is assumed that the human arm moving frequency is about 0.83Hz, the vibration angle is ϑ=120° and the length of arm is Da=0.7m. It results in the angular frequency of ωv=5π/3 and the average amplitude of Da=0.303m. For different frequency bands, accordingly, the micro-Doppler frequency shift are summarized in Table 8.

Table 8: the micro-Doppler frequency shift fDt calculation with different frequency bands, in consideration of micro-Doppler induced by single human arm.

| Frequency band | Formula of micro-Doppler frequency, fDt |
| --- | --- |
| 6.7GHz | fDt=71×sin5π3t |
| 15GHz | fDt=159×sin5π3t |
| 28GHz | fDt=296×sin5π3t |
| 36GHz | fDt=381×sin5π3t |

Using the formula of micro-Doppler frequency fDt to fit the experiment results, we find that the agreement in between is very well. The curves generated by the relevant formula are plotted in Figure 31.

Figure 31: The agreement between the formula of micro-Doppler frequency fDt and the experiment results under the condition of single vibration human arm in the frequency bands of 6.7GHz, 15GHz, 28GHz and 36GHz.

- The agreement between the formula of micro-Doppler frequency fDt and the experiment results is very well.

RAN1 studies the formula of micro-Doppler at least for human target, and Eq. 8 or Eq. 9 can be a starting point.

Spatial Consistency Enhancement

Spatial consistency is a key concept, involved in both communication and sensing. Although TR38.901 has designed the spatial consistency for communication channel, yet the same mechanism cannot be directly applied for sensing channel. Somewhat enhancement is necessary. In what follows, we discuss the details of how to realize the spatial consistency for sensing channel.

For spatial consistency of ISAC, there are three issues needed to be discussed:

- Issue1: Which sensing mode among six sensing modes should be considered?

- Issue2: Whether single and/or multiple targets channels, and/or background channels need to be considered?

- Issue3: How spatial consistency in sensing channel can be modelled?

- There are at least the following issues related to the spatial consistency of ISAC:

- Issue1: Which sensing mode among six sensing modes should be considered?

- Issue2: Whether single and/or multiple targets channels, and/or background channels need to be considered?

- Issue3: How spatial consistency in sensing channel can be modelled?

In our views, all six sensing modes should be considered when modeling spatial consistency of ISAC. Likewise, the spatial consistency for single target channels, multiple targets channels and background channels should all be modeled.

RAN1 studies the spatial consistency for all six sensing modes.

RAN1 studies the spatial consistency for all single target channels, multiple targets channels and background channels.

Hence, we propose a new unified spatial consistency procedure, named by target-specific correlation method. The relevant parameters and the corresponding correlation types are listed in Table 9.

Table 9: Correlation type of channel parameter set for target-specific correlation method.

| Channel parameters | Correlation type |  |
| --- | --- | --- |
|  | Site-specific correlation method in TR38.901 | Target-specific correlation method |
| Delays | Site-specific | Target-specific |
| Cluster powers | Site-specific | Target-specific |
| AOA/ZOA/AOD/ZOD offset | Site-specific | Target-specific |
| AOA/ZOA/AOD/ZOD sign | Site-specific | Target-specific |
| Random coupling | Site-specific | Target-specific |
| XPR | Site-specific | Target-specific |
| Initial random phase | Site-specific | Target-specific |
| LOS/NLOS states | Site-specific | Target-specific |
| Blockage（Model A） | All-correlated | All-correlated |
| O2I penetration loss | All-correlated | All-correlated |
| Indoor distance | All-correlated | All-correlated |
| Indoor states | All-correlated | All-correlated |

The procedure with three steps to realize the proposed target-specific correlation are summarized in Table 10. It should be noted that the procedure to realize the proposed target-specific correlation method is exemplified based on the generation of random variable of delay. The other random variables, nevertheless, can be generated accordingly.

Table 10: The procedure of the proposed target-specific correlation method.

The procedure of the proposed target-specific correlation method

Taking the random variables used in delay generation as an example: Step1: Generate the correlation coefficient matrix among different targets, similar to step4 in TR38.901 Clause 7.5. - Assuming there are N sensing targets. Generate the cross-correlation matrix of the N targets. The cross-correlation matrix Ω is expressed as Ω=e-∆x1,1dcor⋯e-∆x1,Ndcor⋮⋱⋮e-∆xN,1dcor⋯e-∆xN,Ndcor where each element in matrix R represents the correlation coefficient between any paired two targets, ∆x is the distance between two targets; dcor is the correlation distance, which can be used as provided in Section TR38.901 Clause 7.6.3. Step2: Generate the grid associated with each target, where these grids are correlated. - Step2-1: Assume the grid associated with each target has R×C values, where R and C are the rows and columns number of the grid, respectively. - Step2-2: Randomly generate a grid, named by P, where the rows and columns number are N and R×C, respectively. - Step2-3: Generate another grid Q by Q=chol(Ω)×P where chol() is Cholesky's decomposition, and the rows of Q is also N and each row contains R×C values. - Step2-3: For the i-th target, the values of its associated grid are the values of the i-th row of Q, where i=1,2,...,N. - Step2-4: Until now, the grid associated with each target has been conducted. Step3: Use interpolations method in the grid to generate random value used in channel generation.

- The correlation of grids associated with different targets can be modelled by the proposed target-specific method.

The correlation of grids associated with different targets needs to be modelled.

Besides, in some ISAC scenarios, e.g., UAV, due to the varying heights of UAVs, the spatial consistency needs to be extended to support 3D. Hence, the extended 3D target-specific correlation method is proposed.

Table 11: The procedure of the extended 3D target-specific correlation method.

The procedure of the extended 3D target-specific correlation method

Taking the random variables used in delay generation as an example: Step1: Generate the correlation coefficient matrix among different targets, similar to step4 in TR38.901 Clause 7.5. - Assuming there are N sensing targets. Generate the cross-correlation matrix of the N targets. The cross-correlation matrix Ω is expressed as Ω=e-∆x1,1dcor⋯e-∆x1,Ndcor⋮⋱⋮e-∆xN,1dcor⋯e-∆xN,Ndcor where each element in Ω represents the correlation coefficient between any paired two targets, ∆x is the 3D distance between two targets; dcor is the correlation distance, which can be used as provided in Section TR38.901 Clause 7.6.3. Step2: Generate the 3D grid associated with each target, where these grids are correlated. - Step2-1: Assume the 3D grid associated with each target has L×R×C values, where L, R and C are the layers, rows and columns number of the 3D grid, respectively. Note that, the layers number is related to the height of the 3D space. - Step2-2: Randomly generate a 3D grid, named by P, where the rows and columns number are N and L×R×C, respectively. - Step2-3: Generate another grid Q by Q=chol(Ω)×P where chol() is Cholesky's decomposition, and the rows of Q is also N and each row contains L×R×C values. - Step2-3: For the i-th target, the values of its associated 3D grid is the values of the i-th row of Q, where i=1,2,...,N. - Step2-4: Until now, the 3D grid associated with each target has been conducted. Step3: Use 3D location interpolations method in the grid to generate random value used in channel generation.

- The proposed target-specific method can be extended to 3D to support the 3D spatial consistency for some ISAC scenarios, e.g., UAV.

The 3D spatial consistency needs to be supported for some ISAC scenarios, e.g., UAV.

Evaluation for the proposed Target-specific correlation method

To further validate the mentioned spatial consistency methods, as depicted in Table 10, we conduct the simulations for target-specific method, by considering Cell-to-UE bi-static sensing mode as an example. In the simulations, the correlation of delay and angle are simulated, referring to TR38.901. Furthermore, the correlations of single and multiple target channels are both evaluated, using different spatial consistency modeling methods, as follows:

Evaluation of spatial consistency for single-target channels with Cell-to-UE bi-static sensing mode

Firstly, we evaluate the correlation of sensing channels with the same target and different receivers, e.g., correlation of TX-Target-RX1 channel and TX-Target-RX2 channel, as depicted in Figure 32, using different spatial consistency modeling, as follows:

- Case 1: The correlation of TX-Target-RX1 and TX-Target-RX2, using the proposed target-specific method.

- Case 2: The correlation of TX-Target-RX1 and TX-Target-RX2, using the site-specific method defined in TR38.901.

- Case 3: The correlation of TX-RX1 and TX-RX2, using the site-specific method defined in TR38.901.

- Figure 32: The single target channels with same target and different receivers

Figure 33~Figure 35 show the correlation of delay, AOA and LOS state, in Case1 ~ Case3.

Figure 33: The correlation of delay for different spatial consistency modelling method.

Figure 34: The correlation of AOA for different spatial consistency modelling method.

Figure 35: The correlation of LOS state for different spatial consistency modelling method.

As observed in Figure 33~Figure 35, both the proposed target-specific method, and the site-specific method defined in TR38.901 can guarantee the correlation of delay, AOA and LOS state among single target channels.

- For single target channels, both the proposed target-specific method and the site-specific method defined in TR38.901 can guarantee the spatial consistency among channels.

Evaluation of spatial consistency for multiple-target channels with Cell-to-UE bi-static sensing mode

Secondly, we evaluate the correlation of sensing channels with multiple targets and the same receiver, e.g., correlation of TX-Target1-RX channel and TX-Target2-RX channel, as depicted in Figure 36, using different spatial consistency modeling, as follows:

- Case A: The correlation of TX-Target1-RX and TX-Target2-RX, using the proposed target-specific method.

- Case B: The correlation of TX-Target1-RX and TX-Target2-RX, using the Site-specific method defined in TR38.901.

- Case C: The correlation of TX-RX1 and TX-RX2, using the site-specific method defined in TR38.901.

Figure 36: The multiple targets channels with different targets and same receiver.

Figure 37~Figure 39 show the correlation of delay, AOA and LOS state, in Case A ~ Case C.

Figure 37: The correlation of delay for different spatial consistency modelling method.

Figure 38: The correlation of AOA for different spatial consistency modelling method.

Figure 39: The correlation of LOS state for different spatial consistency modelling method.

As observed in Figure 37~Figure 39, the proposed target-specific method can guarantee the correlation of delay, AOA and LOS state among multiple targets channels. However, site-specific method defined in TR38.901 can only guarantee the correlation of delay and LOS state, but not for AOA.

- For multiple targets channels, the proposed target-specific method can guarantee the spatial consistency among channels. However, the site-specific method defined in TR38.901 cannot guarantee the spatial consistency among channels, e.g., the correlation of AOA angle.

Using the proposed target-specific method to model the spatial consistency of single/multiple targets channels with Cell-to-UE bi-static sensing mode.

Besides, the proposed target-specific spatial consistency modeling methods can be also applied for all the remaining sensing modes, as listed in Table 12. In addition, the proposed method is also applied for single/multiple target channel. It is worthwhile noting that, the same procedures of target-specific spatial consistency modeling can be applied to UE-specific spatial consistency modeling. In such a case, instead of the target, we need to generate the correlated grid associated with each UE.

Table 12: The spatial consistency modelling methods for different sensing modes of target channel.

| Sensing mode of target channel | Spatial consistency modeling method |
| --- | --- |
| Cell-to-UE bi-static / UE-to-Cell bi-static | Target/UE-specific correlation method |
| Cell-to-Cell bi-static | Target/Site-specific correlation method |
| UE-to-UE bi-static | Target/UE-specific correlation method |
| Cell mono-static | Target/Site-specific correlation method |
| UE mono-static | Target/UE-specific correlation method |

Using the proposed target-specific method to model the spatial consistency of target(s) channels with Cell-to-Cell bi-static, Cell mono-static, Cell-to-UE bi static, UE-to-Cell bi-static, UE-to-UE bi-static and UE mono-static sensing mode.

Target-specific method can be replaced by UE-specific method, i.e., use the same procedures to generate correlated grid associated with each UE, instead of the target.

- If using UE-specific method, only the spatial consistency of target(s) channels with Cell-to-UE bi static, UE-to-Cell bi-static, UE-to-UE bi-static and UE mono-static sensing mode can be modelled.

For background channel, if considering EO-type2, there is no need to model spatial consistency due to deterministic channel associated to EO-type2. If considering random clutters, we can use site-specific method defined in TR38.901 to model spatial consistency.

For background channels, RAN1 uses the site-specific method defined in TR38.901 to model the spatial consistency for random clutters, and there is no need to model spatial consistency for EO-type2.

Complexity reduction for the Target-specific correlation method

In the proposed target-specific method, since the grids are generated per target, its complexity could be extremely high as the number of targets increases, especially for the memory overhead. Specifically, the memory overhead is related to the number of channel random parameters and the size of grid.

Hence, the reducing the complexity of the proposed method becomes more important. To this end, a potentially feasible way is to reduce the size of grids, e.g., generate the grid based on one site or one cell topology rather than the whole scenario topology, as shown in Figure 40, where instead of whole scenario topology, one site, one cell or small topology can be employed.

Figure 40: The different size of grid associated with different topology

To leverage the complexity reduction and the accuracy of the channel model, we analyse and evaluate the proposed complexity reduction method, named by simplified target-specific method. The corresponding parameters of evaluation are listed in Table 13.

Table 13: Evaluation details for the proposed complexity reduction method.

| Scenario | UMA with ISD 500m in TR38.901 |  |
| --- | --- | --- |
| Comparison Method | Target-specific method | The size of grid covers whole scenario topology. The interpolation target-based grid is considered. |
|  | Simplified target-specific method | - Option1: The size of grid covers one site/cell topology - Option2: The size of grid covers smaller topology, e.g., 150m×150m. If the Rx is in the grid, the interpolation with target-based grid is considered. In case that Rx is outside of the grid: - Alt1: Not model correlation - Alt2: Interpolation with a single site-based grid - Alt3: Extrapolation with target-based grid, as shown in Figure 41. |
| Note: Since targets and UEs are dropped only in a single cell when considering spatial consistency calibration in TR38.901, the correlation evaluation results are the same for Target-specific method and Simplified target-specific method with Option1. Therefore, the correlation evaluation results of simplified target-specific method with Option1 are not performed. |  |  |

Figure 41: The proposed simplified target-specific method, using

the combination of interpolation and extrapolation.

Firstly, the complexity of the proposed simplified target-specific method is analyzed, as listed in Table 14.

Table 14: The complexity of the proposed simplified target-specific method

| Considering UMA scenario with 7 sites, 3 sector per site, 10 UEs per cell, 5 target per cell, ISD 500m |  |  |
| --- | --- | --- |
| Simulation case | Time overhead | Memory overhead |
| Site-specific method in TR38.901 | 9s | 3381MB |
| Target-specific method | 1503s | 50241MB |
| Simplified target-specific method, with Option1 | 53s | 9436MB |
| Simplified target-specific method, with Option2, Alt1/Alt3 | 25s | 4425MB |
| Simplified target-specific method, with Option2, Alt2 | 28s | 4876MB |

- Reducing the size of grid for target-specific method can significantly reduce the memory overhead and time overhead.

Then, to validate the correlation performance of the proposed simplified target-specific method, we also evaluate the correlation of sensing channels with single or multiple targets.

Evaluation of spatial consistency for single target channels with Cell-to-UE bi-static sensing mode

The following cases are considered in the simulation evaluation:

- Case1：Communication channel using site-specific modelling in TR38.901

- Case2：Target channel using site-specific modelling in TR38.901

- Case3：Target channel using target-specific modelling

- Case4：Target channel using simplified target-specific modelling, option2, Alt1

- Case5：Target channel using simplified target-specific modelling, option2, Alt2

- Case6：Target channel using simplified target-specific modelling, option2, Alt3

Figure 42~Figure 44 show the correlation of delay, AOA and LOS state, in Case 1 ~ Case 6.

Figure 42: The correlation of delay.

Figure 43: The correlation of AOA.

Figure 44: The correlation of LOS probability.

As observed in Figure 42~Figure 44, the proposed simplified target-specific method with Option2 and Alt2/Alt3 can guarantee the correlation of delay, AOA and LOS state among single target channels.

- For single target channels, the proposed simplified target-specific method with Option2 and Alt2/Alt3 can guarantee the spatial consistency among channels.

Evaluation of spatial consistency for multiple targets channels with Cell-to-UE bi static sensing mode

The following cases are considering in the sub-section:

- Case A: Communication channel using site-specific modelling in TR38.901

- Case B: Target channel using site-specific modelling in TR38.901

- Case C: Target channel using target-specific modelling

- Case D: Target channel using simplified target-specific modelling, option2, Alt1

- Case E: Target channel using simplified target-specific modelling, option2, Alt2

- Case F: Target channel using simplified target-specific modelling, option2, Alt3

Figure 45~Figure 47 show the correlation of delay, AOA and LOS state, in Case A ~ Case F.

Figure 45: The correlation of delay.

Figure 46: The correlation of AOA.

Figure 47: The correlation of LOS probability.

As observed in Figure 45~Figure 47, the proposed simplified target-specific method with option2 and Alt3 can guarantee the correlation of delay, AOA and LOS state among multiple targets channels. However, the proposed simplified target-specific method with option2 and Alt2 cannot guarantee the correlation of AOA.

- For multiple targets channels, the proposed simplified target-specific method with Option2 and Alt3 can guarantee the spatial consistency among channels. While, Alt2 cannot guarantee the spatial consistency among channels, e.g., the correlation of AOA angle.

The complexity reduction of target-specific method can be considered, e.g., by reducing the size of grid and using the combination of interpolation and extrapolation.

Link level channel model

The link-level simulation is typically used for comparative evaluation of waveform or signal design, receiver processing algorithms, and other technical solutions. It focuses on the performance of single-link target detection and parameter estimation. The existing channel model for link level evaluation is based on stochastic model without detail location of TX and RX.

For the link-level channel model in ISAC, considering the enhanced principle based on TR38.901, it is desirable that the stochastic channel model should also be applied to ISAC, resulting in less workload.

The stochastic link level channel model based on TR38.901 should be considered as a starting point in the ISAC.

As an implementation method, the link-level channel model for ISAC can be enhanced based on the clustered delay line (CDL) channel model in TR 38.901 by adding the cluster modeling of the sensing target. A specific approach is to model the sensing target as one or more clusters, as shown in Figure 48, consisting of sensing clusters and environment clusters. The generation procedure defined in TR 38.901, including sensing cluster, can be reused, with certain modification, e.g., the delay, angle, velocity or doppler parameter. Nevertheless, this is not the only approach, and other scheme is not precluded. RAN1 should further study the detailed generation methodology of sensing cluster in the next meetings.

Figure 48 Sensing target in the LLS channel model as cluster

RAN1 further studies the stochastic link level channel model for ISAC by adding sensing target cluster based on CDL channel model defined in TR38.901.

Conclusions

In this contribution, we have studied the target channel modeling, including the realization of concatenation in target channel modeling, RCS modeling, EO modeling, and background modeling, the characteristics of LOS/NLOS for sensing target, and channel model formation, including two component channel modeling, as well as the enhancements of Doppler and spatial consistency. The observations and proposals are summarized as follows.

Observation 1: At least, αi,1 and αi,2 in the CPM of target is not equally valued, i.e., αi,1≠αi,2.

Observation 2: A cluster with a lower power gain can be directly dropped from the perspective of DSP theory.

Observation 3: Directly dropping rays from a discrete channel violates the principle of DSP processing.

Observation 4: The performance gap is huge for Option3 with and without power normalization, especially for ASD and ZSD.

Observation 5: Whether performing the power normalization after path dropping of target channel incurs the same delay and angle spread distribution for all propagation Case1/2/3/4 in Option 0.

Observation 6: When dropping path after concatenation in target channel modelling, the path-drop options, cluster/ray-based path-drop and threshold levels may affect the delay and angle distributions.

Observation 7: The experiment measurements fit semi-ellipse-based calculation well for both UAV and vehicle with three semi-ellipses in mono-static sensing mode.

Observation 8: The experiment measurements fit semi-ellipsoid-based calculation well with three semi-ellipsoids and three different car surfaces in bi-static sensing mode.

Observation 9: In case that the specular phenomenon occurs, the amplitude gap between LOS path and specular paths is about 10dB, while the gap between the specular paths and non-specular paths is about 5dB.

Observation 10: There is no impact by introducing the specular reflection model when the orientation and the finite size of EO is taken into account.

Observation 11: If assuming the azimuth angle ϕ of the center point O and the elevation angle θ' of the scatterer P can be set to zero, i.e., ϕ=0, and θ'=0, the micro-Doppler frequency shift induced by the vibration of a scatterer point can be derived by Eq. 8.

Observation 12: The micro-Doppler frequency shift induced by the human arm/leg vibration can be calculated by either the instantaneous amplitude or the average amplitude.

Observation 13: The moving direction of human and frequency bands influence the micro-Doppler:

- When the human body with the moving direction is facing the transmit and receive antennas, the sinusoidal function fluctuations due to the micro-Doppler are obvious, for all the six cases.

- When the human body with the moving direction is perpendicular to the transmit and receive antennas, the sinusoidal function fluctuations due to the micro-Doppler are not that obvious, for all the six cases.

- Micro-Doppler behavior for arm periodic swing is more obvious than that for leg periodic swing.

- Different frequency bands influence micro-Doppler differently, i.e., the higher the frequency band, the more influence on micro-Doppler.

Observation 14: The agreement between the formula of micro-Doppler frequency fDt and the experiment results is very well.

Observation 15: There are at least the following issues related to the spatial consistency of ISAC:

- Issue1: Which sensing mode among six sensing modes should be considered?

- Issue2: Whether single and/or multiple targets channels, and/or background channels need to be considered?

- Issue3: How spatial consistency in sensing channel can be modelled?

Observation 16: The correlation of grids associated with different targets can be modelled by the proposed target-specific method.

Observation 17: The proposed target-specific method can be extended to 3D to support the 3D spatial consistency for some ISAC scenarios, e.g., UAV.

Observation 18: For single target channels, both the proposed target-specific method and the site-specific method defined in TR38.901 can guarantee the spatial consistency among channels.

Observation 19: For multiple targets channels, the proposed target-specific method can guarantee the spatial consistency among channels. However, the site-specific method defined in TR38.901 cannot guarantee the spatial consistency among channels, e.g., the correlation of AOA angle.

Observation 20: Reducing the size of grid for target-specific method can significantly reduce the memory overhead and time overhead.

Observation 21: For single target channels, the proposed simplified target-specific method with Option2 and Alt2/Alt3 can guarantee the spatial consistency among channels.

Observation 22: For multiple targets channels, the proposed simplified target-specific method with Option2 and Alt3 can guarantee the spatial consistency among channels. While, Alt2 cannot guarantee the spatial consistency among channels, e.g., the correlation of AOA angle.

Proposal 1: In the CPM of target, the gain factors of αi,1 and αi,2 are unequally valued (i.e., αi,1≠αi,2) with the normalization of αi,12+αi,22=2, while the gain factors of βi,1 and βi,2 are equally modelled (i.e., βi,1≈βi,2).

Proposal 2: Directly dropping rays with a predetermined power threshold before or after concatenation should be considered with the power normalization over the discrete time channel.

Proposal 3: RAN1 further considers how to align delay/angle spread distribution if companies use different methods (e.g., Option0, Option3) to generate NLOS ray + NLOS ray of target channel in ISAC channel model calibration.

Proposal 4: No power normalization of target channel after path dropping if Option 0 is involved for target channel concatenation.

Proposal 5: RAN1 further considers the following factors to drop path after concatenating TX-Target and Target-RX.

- Cluster-based or ray-based path-drop;

- Threshold levels used for path-drop, e.g., -25dB as baseline.

Proposal 6: RAN1 studies how to generate the cluster power of target channel.

- As a start point, for cluster power generation for concatenation in sensing channel: Use shadowing term per cluster Zn1~N0, ζ22 for Tx-Target paths, Zn2~N0, ζ22 for Target-Rx paths, respectively.

Proposal 7: The pathloss formula of Eq. 2 for the concatenation target channel model in ISAC can be regarded as a starting point for the further pathloss model study.

Proposal 8: The correlation of LOS condition between TX-target link and target-RX link is modelled.

Proposal 9: The spatial consistency mechanism can be used to model the LOS correlation in the ISAC Tx-target-Rx channel.

Proposal 10: Multi-point channel model is supported in the R19 ISAC channel model.

Proposal 11: Multi-point channel model can be determined based on the following principle:

- The large scale fading of different point to TX/RX share the same value calculated by the center point of the target;

- The spatial consistency methodology is applied to calculate to small scale fading of different point in the sensing target to the TX/RX.

Proposal 12: RAN1 further studies whether some relevant small scale parameters, such as delay and angle, can be shared for the different points.

Proposal 13: The Mono-static RCS of human can be unified based on RCS=AB1B2.

- The mean value represented by A in Eq. 3 is considered as a starting point for the human RCS, which is independent of the incident and the scatter angle, but dependent on the frequency band.

- B1 equals to 1.

- The variance corresponding to component B2 in Eq. 4 is considered as a starting point for the human RCS, which is independent on the incident and scatter angle.

Proposal 14: The bi-static and mono-static RCS model share the same mathematical model at least for small UAV and human.

Proposal 15: RAN1 considers the equation of ellipsoid(s) unified for both mono-static and bi-static sensing modes, in consideration of the rotation of the ellipsoid along the direction of specular reflection.

Proposal 16: The overall RCS should be summed from the individual RCSs modelled by the corresponding semi-ellipsoid over each surface of the target.

Proposal 17: RAN1 support the multi-point RCS model in R19 ISAC channel model.

Proposal 18: In the multi-point RCS model, the UAV is divided into 5 points, i.e., airframe body and four airfoils.

- If using UE-specific method, only the spatial consistency of target(s) channels with Cell-to-UE bi static, UE-to-Cell bi-static, UE-to-UE bi-static and UE mono-static sensing mode can be modelled.

Proposal 19: In the multi-point RCS model, the human is divided into 6 points, i.e., head, abdomen, two legs and two hands.

Proposal 20: The RCS value of the entire target is the linear summation of the RCSs from the segmented parts of the entire target.

Proposal 21: The two legs or two hands can be modelled by the same RCS.

Proposal 22: The four airfoils of UAV can be modelled by the same RCS.

Proposal 23: For type-2 EO, the indoor and the urban grid scenario are prioritized for the type-2 EO modelling.

Proposal 24: The detailed EO modelling can be done after the target modelling is consolidated.

Proposal 25: RAN1 reuses the parameters of b, R∥specular, and R⊥specular without any updating.

Proposal 26: The basic case without EO modelling for background channel should be considered with a high priority.

Proposal 27: The channel of TRP-TPR link is generated based on TR36.777 in UAV scenario.

Proposal 28: Power normalization is not required in the channel combination procedure.

Proposal 29: Channel combination method selection should be based on scenario and Table 6 can be a starting point.

Proposal 30: RAN1 should consider reducing the ISAC channel model complexity by reducing the number of clusters for TX-target and target-RX link in target channel.

Proposal 31: RAN1 agrees on the formula of micro-Doppler frequency shift in Eq. 8 or Eq. 9 at least for the human arm/leg target with the vibration of a scatterer point and the mono-static transmitter/receiver.

Proposal 32: RAN1 studies the formula of micro-Doppler at least for human target, and Eq. 8 or Eq. 9 can be a starting point.

Proposal 33: RAN1 studies the spatial consistency for all six sensing modes.

Proposal 34: RAN1 studies the spatial consistency for all single target channels, multiple targets channels and background channels.

Proposal 35: The correlation of grids associated with different targets needs to be modelled.

Proposal 36: The 3D spatial consistency needs to be supported for some ISAC scenarios, e.g., UAV.

Proposal 37: Using the proposed target-specific method to model the spatial consistency of single/multiple targets channels with Cell-to-UE bi-static sensing mode.

Proposal 38: Using the proposed target-specific method to model the spatial consistency of target(s) channels with Cell-to-Cell bi-static, Cell mono-static, Cell-to-UE bi static, UE-to-Cell bi-static, UE-to-UE bi-static and UE mono-static sensing mode.

Proposal 39: Target-specific method can be replaced by UE-specific method, i.e., use the same procedures to generate correlated grid associated with each UE, instead of the target.

- If using UE-specific method, only the spatial consistency of target(s) channels with Cell-to-UE bi static, UE-to-Cell bi-static, UE-to-UE bi-static and UE mono-static sensing mode can be modelled.

Proposal 40: For background channels, RAN1 uses the site-specific method defined in TR38.901 to model the spatial consistency for random clutters, and there is no need to model spatial consistency for EO-type2.

Proposal 41: The complexity reduction of target-specific method can be considered, e.g., by reducing the size of grid and using the combination of interpolation and extrapolation.

Proposal 42: The stochastic link level channel model based on TR38.901 should be considered as a starting point in the ISAC.

Proposal 43: RAN1 further studies the stochastic link level channel model for ISAC by adding sensing target cluster based on CDL channel model defined in TR38.901.

References

- RP-234069, “New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, Nokia, Nokia Shanghai Bell.

- 3GPP RAN1 #116bis, Chair notes RAN1#116bis, Changsha, China, April 15th – 19th, 2024.

- 3GPP RAN1 #118b, Chair notes RAN1#118b, Hefei, October 14th – 18th, 2024.

- R1-2406302, “Summary #4 on ISAC channel modelling”, RAN1#118, Moderator (Xiaomi),

- RP-231798, “Rel-19 Channel Modeling for ISAC and New Spectrum (7-24GHz) (RAN1-led)”, vivo.

- M. I. Skolnik, Radar Handbook, Second Edition, McGraw-Hill, 1990.

- R. L. Wittschen, “Statistical Description of the Bistatic Radar Cross Section Distributions for a Right Circular Cylinder,” Rome Laboratory Air Force Systems Command Griffiss Air Force Base, June 1992.

- Kell, R.E., On the derivation of bistatic RCS from monostatic measurements., Proc. IEEE, Vol.53, pp983-988, 1965.

- R1-2408263, “ISAC channel measurements and modeling”, RAN1#118bis, BUPT.

- J. W. Crispin and K. M. Siegel, Methods of Radar Cross Section Analysis. New York: Academic, 1968.

- Robert L. Eigel, et al., “Bistatic Scattering Characterization of Complex Objects,” IEEE Transactions on Geoscience and Remote Sensing, Vol. 38, No. 5, September 2000.

- 3GPP TR 22.837, Feasibility Study on Integrated Sensing and Communication, V19.0.0, 2023-06.

- Stephan Jaeckel, et al. “A Geometric Polarization Rotation Model for the 3-D Spatial Channel Model”, IEEE Transactions on Antennas and Propagation, Vol. 60, Issue 12, December 2012.

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”, V17.0.0, 2022-03.

- R1-2403715, “Summary #4 on ISAC channel modelling”, RAN1#116bis, Moderator (Xiaomi).

- ISC(24)000103, Views and measurements on micro-Doppler frequency, ZTE, June 11, 2024.

- R1-2402255, “Views on Rel-19 ISAC channel modelling”, RAN1#116b, vivo.

- R1-2406197, “Views on Rel-19 ISAC channel modelling”, RAN1#118, vivo.

- Fujiwara, H., Spectroscopic ellipsometry: principles and applications. 2007: John Wiley & Sons.

- Constantine A. Balanis, Advanced Engineering Electromagnetics, 2nd Edition, 2012.

- Annex A. Channel coefficient generation procedure of target channel with concatenation method

For the concatenation method in target channel modeling, the propagation parameters of direct path can be generated based on the geometry location of Tx, target and Rx, e.g., delay, AoA/ZoA at Rx, AoD/ZoD at Tx and AoA/ZoA/AoD/ZoD at target. The indirect paths in the target channel of a target can be modelled by stochastic cluster generation with modification of procedure in TR38.901 subsection 7.5.  In this section, we provide a step-by-step procedure to generate channel coefficient of indirect path in target channel. The detailed analysis, formulas and generation procedure of some steps can be found in the main sections.

For the indirect path of target channel, the propagation parameters of sensing Tx-target link, the target-sensing Rx link can be generated respectively by following Step1~ Step 8 of TR38.901, without much modification. Besides, the characteristics of sensing target should be generated additionally, e.g., the RCS of target, the cross polarization matrix associated with the scattering of target. To model different propagation conditions of target channel (e.g., LOS/NLOS condition of Tx-target link and target-Rx link, specular reflection, diffraction or scattering by target or EO type-2), channel coefficients generation in Step 11 should be specially modified for target channel, including coupling paths in Tx-target link and target-Rx link. Then pathloss and shadowing are applied in Step 12.

Without loss of generality, we take the BS-UE bi-static sensing mode as an example to illustrate how to model target channel by concatenating path(s) from Tx to target and from target to Rx, as illustrated in Figure 49. The generation formulas of channel coefficients and channel impulse response of Tx-target-Rx link are summarized in Table 15, Table 16 and Table 17, respectively. The mono-static sensing mode can be regarded as a special case of bi-static sensing mode, with the same sensing Tx and Rx location.

Figure 49: The illustration of target channel coefficient generation procedure

Note: In the following generation steps, the description in bule is to highlight the modification of the 38.901, the description in gray shading is to refer to the previous chapter for detailed theoretical analysis. The detailed procedure in each step can reuse TR38.901 if without special descriptions.

Step 1: Set environment, network layout, antenna array parameters, and target properties (e.g., target RCS, target type). Including giving number and 3D locations of BS, UT and target, giving speed and direction of motion of UT and target in the global coordinate system, and determine LOS AOD、ZOD、AOA、ZOA of each BS-target, target-UT link in the global coordinate system.

Step 2: Assign propagation condition (LOS/NLOS) for each BS-target, target-UT link according to corresponding LOS probability. The LOS probability of different scenarios is analyzed in section 2.1.2.2.

Step 3: Calculate pathloss for each BS-target-UT link. For different cases, the pathloss is calculated as following:

PLTx-target-Rx=PLTx-targetdTx-target,f0+PLtarget-Rxdtarget-Rx,f0+10log10λ24π-10log10σ

where σ is the RCS of the target in large scale fading. The RCS of a target can be represented by RCS=σ∙ρD∙  ρR, which is introduced in section 2.1.3.4. The pathloss formulas of PLTx-target  and PLtarget-Rx in different scenarios is analyzed in section 2.1.2.2.

Step 4: Generate large scale parameters, e.g.DS, angular spreads (ASA, ASD, ZSA, ZSD), K and SF for each BS-target, target-UT link respectively.

Step 5: Generate cluster delays τn in each BS-target, target-UT link, where

τn'=12∙-rτ∙DS∙lnXn

τn=sortτn'-minτn'

is performed respectively in BS-target, target-UT link.

In the case of LOS condition for BS-target or target-UT link, additional scaling of delays is required following the method in TR38.901 Step5:

τnLOS=τnCτ

The scaled delays are not to be used in cluster power generation.

Step 6: Generate cluster powers Pn' in each BS-target, target-UT link with exponential delay distribution:

Pn'=exp-τnrτ-1rτDS∙10-Zn10

Where Zn~N0, ζ22 is the per cluster shadowing term in [dB]. The cluster power normalization is not carried out until the power concatenation between BS-target and target-UT link in Step 11. So Pn=Pn' is used in arrival angles and departure angles generation.

In the case of LOS condition for BS-target or target-UT link, power of the single LOS ray in BS-target or target-UT link is:

P1,LOS=KR∙n=1NPn'

The cluster powers are updated as:

Pn=Pn'+δn-1P1,LOS

which is only used in arrival angles and departure angles generation.

The modification of Zn and power normalization is analyzed and simulated in section 2.1.1.2.

Step 7: Generate arrival angles and departure angles for both azimuth and elevation for each BS-target, target-UT link cluster respectively, following the same procedure in TR38.901 Step7.

Step 8: Coupling of rays within a cluster for both azimuth and elevation for each BS-target, target-UT link cluster respectively.

Step 9: Generate the cross polarization power ratios (XPR) for direct path scattered by sensing target and each indirect path.

Step 10: Draw initial random phases for direct path scattered by sensing target and each indirect path.

Step11: Coupling propagation paths of BS-target, target-UT link and generate channel coefficients for direct path and indirect paths of target channel for each receiver and transmitter element pair u, s. The LOS ray and all the NLOS paths of Tx-target link are fully coupled to the LOS ray and all the NLOS paths of target-Rx link here, some reasonable simplification method with simulation analysis is provided in section 2.1.1.2.

The channel coefficients of Tx-target-Rx link can be formulated by the concatenation between the receiver vector and the transmitter vector for different case. After concatenation, the channel coefficients of each ray in target channel can be formulated as following, which are summarized in Table 15.

- If target-Rx link is LOS condition and Tx-target link is LOS condition:

Hu,tar,s,1,1LOS,LOSt=Frx,u,θθLOS,ZOAtarget-Rx,ϕLOS,AOAtarget-RxFrx,u,ϕθLOS,ZOAtarget-Rx,ϕLOS,AOAtarget-RxT∙CPM1,1tx,sp,rx∙Ftx,s,θθLOS,ZODTx-target,ϕLOS,AODTx-targetFtx,s,ϕθLOS,ZODTx-target,ϕLOS,AODTx-target∙expj2πrrx,LOST∙drx,uλ0expj2πrtx,LOST∙dtx,sλ0exp-j2πt0tfD1,1tdtρ1,1Dρ1,1R

- If target-Rx link is LOS condition and Tx-target link is NLOS condition:

Hu,tar,s,1,n,mLOS,NLOSt=PnMFrx,u,θθLOS,ZOAtarget-Rx,ϕLOS,AOAtarget-RxFrx,u,ϕθLOS,ZOAtarget-Rx,ϕLOS,AOAtarget-RxT∙CPM1,n,mtx,sp,rx∙Ftx,s,θθn,m,ZODTx-target,ϕn,m,AODTx-targetFtx,s,ϕθn,m,ZODTx-target,ϕn,m,AODTx-target∙expj2πrrx,LOST∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0exp-j2πt0tfD1,n,mtdtρ1,n,mDρ1,n,mR

- If target-Rx link is NLOS condition and Tx-target link is LOS condition:

Hu,tar,s,n',m',1NLOS,LOSt=Pn'MFrx,u,θθn',m',ZOAtarget-Rx,ϕn',m',AOAtarget-RxFrx,u,ϕθn',m',ZOAtarget-Rx,ϕn',m',AOAtarget-RxT∙CPMn',m',1tx,sp,rx∙Ftx,s,θθLOS,ZODTx-target,ϕLOS,AODTx-targetFtx,s,ϕθLOS,ZODTx-target,ϕLOS,AODTx-target∙expj2πrrx,n',m'T∙drx,uλ0expj2πrtx,LOST∙dtx,sλ0exp-j2πt0tfDn',m',1tdtρn',m',1Dρn',m',1R

- If target-Rx link is NLOS condition and Tx-target link is NLOS condition:

Hu,tar,s,n',m',n,mNLOS,NLOSt=Pn'MPnMFrx,u,θθn',m',ZOAtarget-Rx,ϕn',m',AOAtarget-RxFrx,u,ϕθn',m',ZOAtarget-Rx,ϕn',m',AOAtarget-RxT∙CPMn',m',n,mtx,sp,rx∙Ftx,s,θθn,m,ZODTx-target,ϕn,m,AODTx-targetFtx,s,ϕθn,m,ZODTx-target,ϕn,m,AODTx-targetexpj2πrrx,n',m'T∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0

exp-j2πt0tfDn',m',n,mtdtρn',m',n,mDρn',m',n,mR

Where CPMtx,sp,rx is the total cross polarization matrix, which can be calculated as described in section 2.1.1.1 for different cases. The RCS of a target can be represented by RCS=σ∙ρD∙  ρR, which is introduced in section 2.1.3.4. σ is the RCS of target applied in the pathloss of large scale. ρD  is determined according to the angle (or bistatic angle) formed with the impinging direction to the target and the reflecting direction from the target, and ρR is generated by a pre-determined distribution of the target. fD is the unified doppler formula, as described in section 2.5.

Then, generate the channel impulse response of each channel component in target channel, according to the propagation condition (LOS/NLOS) of BS-target, target-UT link, which are summarized in Table 16. Specially, the cluster powers should be normalized when generating the channel impulse response, so that the sum of all cluster powers in the channel impulse response is equal to one, as following:

Hu,tar,sLOS,NLOSτ,t = 1n=1NPn'n=1NmMHu,tar,s,1,n,mNLOS,LOStδτ-τ1target-Rx-τn,m

Hu,tar,sNLOS,LOSτ,t = 1n'=1N'Pn'n'=1N'm'=1M'Hu,tar,s,n',m',1NLOS,LOStδτ-τn',m'-τ1Tx-target

Hu,tar,sNLOS,NLOSτ,t = 1n'=1N'n=1NPn'Pnn'=1N'm'=1M'n=1Nm=1MHu,tar,s,n',m',n,mNLOS,NLOStδτ-τn',m'-τn,m

Where τn,m and τn',m' is equal to the delay of corresponding cluster or sub-cluster.

In the LOS condition both for BS-target and target-UT link, the channel impulse response is given by adding the channel coefficient of direct path to the channel impulse response of indirect path and scaling both terms according to the desired K-factor as

Hu,tar,starget channelτ, t=KRtarget-RxKRtarget-Rx+11KRTx-target+1Hu,tar,sLOS, NLOSτ-τTx-target-Rx,t

+1KRtarget-Rx+1KRTx-targetKRTx-target+1Hu,tar,sNLOS, LOSτ-τTx-target-Rx,t

+1KRtarget-Rx+11KRTx-target+1Hu,tar,sNLOS,NLOSτ-τTx-target-Rx,t

+KRtarget-RxKRtarget-Rx+1KRTx-targetKRTx-target+1Hu,tar,s,1,1LOS,LOStδτ-τTx-target-Rx

The channel impulse responses of other propagation conditions are summarized in Table 17.

Step12:  Apply pathloss and shadowing for the target channel coefficients.

Table 15. The channel coefficients of each ray in target channel.

| Case [1] | The channel coefficients [2][3] |
| --- | --- |
| LOS ray + LOS ray | Hu,tar,s,1,1LOS,LOSt=Frx,u,θθLOS,ZOAtarget-Rx,ϕLOS,AOAtarget-RxFrx,u,ϕθLOS,ZOAtarget-Rx,ϕLOS,AOAtarget-RxT∙CPM1,1tx,sp,rx∙Ftx,s,θθLOS,ZODTx-target,ϕLOS,AODTx-targetFtx,s,ϕθLOS,ZODTx-target,ϕLOS,AODTx-target∙expj2πrrx,LOST∙drx,uλ0expj2πrtx,LOST∙dtx,sλ0exp-j2πt0tfD1,1tdtρ1,1Dρ1,1R |
| LOS ray + NLOS ray | Hu,tar,s,1,n,mLOS,NLOSt=PnMFrx,u,θθLOS,ZOAtarget-Rx,ϕLOS,AOAtarget-RxFrx,u,ϕθLOS,ZOAtarget-Rx,ϕLOS,AOAtarget-RxT∙CPM1,n,mtx,sp,rx∙Ftx,s,θθn,m,ZODTx-target,ϕn,m,AODTx-targetFtx,s,ϕθn,m,ZODTx-target,ϕn,m,AODTx-target∙expj2πrrx,LOST∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0exp-j2πt0tfD1,n,mtdtρ1,n,mDρ1,n,mR |
| NLOS ray + LOS ray | Hu,tar,s,n',m',1NLOS,LOSt=Pn'MFrx,u,θθn',m',ZOAtarget-Rx,ϕn',m',AOAtarget-RxFrx,u,ϕθn',m',ZOAtarget-Rx,ϕn',m',AOAtarget-RxT∙CPMn',m',1tx,sp,rx∙Ftx,s,θθLOS,ZODTx-target,ϕLOS,AODTx-targetFtx,s,ϕθLOS,ZODTx-target,ϕLOS,AODTx-target∙expj2πrrx,n',m'T∙drx,uλ0expj2πrtx,LOST∙dtx,sλ0exp-j2πt0tfDn',m',1tdtρn',m',1Dρn',m',1R |
| NLOS ray + NLOS ray | Hu,tar,s,n',m',n,mNLOS,NLOSt=Pn'MPnMFrx,u,θθn',m',ZOAtarget-Rx,ϕn',m',AOAtarget-RxFrx,u,ϕθn',m',ZOAtarget-Rx,ϕn',m',AOAtarget-RxT∙CPMn',m',n,mtx,sp,rx∙Ftx,s,θθn,m,ZODTx-target,ϕn,m,AODTx-targetFtx,s,ϕθn,m,ZODTx-target,ϕn,m,AODTx-target∙expj2πrrx,n',m'T∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0exp-j2πt0tfDn',m',n,mtdtρn',m',n,mDρn',m',n,mR |
| Note1: Ray coupling by combining ray in target-Rx link and ray in Tx-target. Note2: the channel coefficients are generated by concatenating the m-th ray in the n-th cluster or LOS ray of Tx-target link, and the m'-th ray in the n'-th cluster or LOS ray of Tx-target link. Note3:CPMtx,sp,rx is the total cross polarization matrix, which can be calculated as described in section 2.1.1.1 for different cases. |  |

Table 16. The channel impulse response of each channel component.

| Channel component (target-Rx+ Tx-target) | Channel impulse response [1][2][3] |
| --- | --- |
| LOS ray + NLOS ray part | Hu,tar,sLOS,NLOSτ,t = 1n=1NPn'n=1NmMHu,tar,s,1,n,mNLOS,LOStδτ-τ1target-Rx-τn,m |
| NLOS ray + LOS ray part | Hu,tar,sNLOS,LOSτ,t = 1n'=1N'Pn'n'=1N'm'=1M'Hu,tar,s,n',m',1NLOS,LOStδτ-τn',m'-τ1Tx-target |
| NLOS ray + NLOS ray part | Hu,tar,sNLOS,NLOSτ,t = 1n'=1N'n=1NPn'Pnn'=1N'm'=1M'n=1Nm=1MHu,tar,s,n',m',n,mNLOS,NLOStδτ-τn',m'-τn,m |
| Note1: m, n, represents the m-th ray in the n-th cluster of Tx-target link. m', and n' represents the m'-th ray in the n'-th cluster of Tx-target link. Note2: the cluster powers should be normalized when generate the channel impulse response, so that the sum of all cluster powers in the channel impulse response is equal to one. Note3: τn,m and τn',m' is equal to the delay of corresponding cluster or sub-cluster. |  |

Table 17. The channel impulse response of target channel

| Propagation condition (target-Rx+ Tx-target) | Channel impulse response |
| --- | --- |
| LOS+LOS condition | Hu,tar,starget channelτ, t =KRtarget-RxKRtarget-Rx+11KRTx-target+1Hu,tar,sLOS, NLOSτ-τTx-target-Rx,t +1KRtarget-Rx+1KRTx-targetKRTx-target+1Hu,tar,sNLOS, LOSτ-τTx-target-Rx,t +1KRtarget-Rx+11KRTx-target+1Hu,tar,sNLOS,NLOSτ-τTx-target-Rx,t +KRtarget-RxKRtarget-Rx+1KRTx-targetKRTx-target+1Hu,tar,s,1,1LOS,LOStδτ-τTx-target-Rx |
| LOS+NLOS condition | Hu,tar,starget channelτ, t =KRtarget-RxKRtarget-Rx+1Hu,tar,sLOS, NLOSτ-τTx-target-Rx,t +1KRtarget-Rx+1Hu,tar,sNLOS,NLOSτ-τTx-target-Rx,t |
| NLOS+LOS condition | Hu,tar,starget channelτ, t =KRTx-targetKRTx-target+1Hu,tar,sNLOS, LOSτ-τTx-target-Rx,t +1KRTx-target+1Hu,tar,sNLOS,NLOSτ-τTx-target-Rx,t |
| NLOS+NLOS condition | Hu,tar,starget channelτ, t=Hu,tar,sNLOS,NLOSτ-τTx-target-Rx,t |

- Annex B. Measurements for Micro-Doppler

To investigate the formula for micro-Doppler, the experiment measurement campaign is conducted in a large enough experiment chamber, as shown in Figure 30, with the experiment assumptions as listed in Table 7.

In case that the human body is facing the transmit and receive antennas:

As listed in Table 7, six cases are measured with different human walking behaviors. Figure 50, Figure 51 and Figure 52 show the micro-Doppler frequency in Hz as a function of time, t, in case that the human body is facing the transmit and receive antennas, respectively. In this case, it can be observed that the sinusoidal function fluctuations due to the micro-Doppler are obvious, for all the six cases. Moreover, micro-Doppler behavior for arm periodic swing is more obvious than that for leg periodic swing.

Figure 50: Single arm and leg periodic swing at frequency band of 15GHz.

Figure 51: Both arm and leg periodic swing at frequency band of 15GHz.

Figure 52: Walking on the treadmill and the ground at frequency band of 15GHz.

In case that the human body is perpendicular to the transmit and receive antennas:

As listed in Table 7, six cases are measured with different human walking behaviors. Figure 53, Figure 54 and Figure 55 show the micro-Doppler frequency in Hz as a function of time, t, in case that the human body is perpendicular to the transmit and receive antennas, respectively. In this case, it can be observed that the sinusoidal function fluctuations due to the micro-Doppler are not that obvious as compared to the case that the human body is facing the transmit and receive antennas, for all the six cases.

Figure 53: Single arm and leg periodic swing at frequency band of 15GHz.

Figure 54: Both arm and leg periodic swing at frequency band of 15GHz.

Figure 55: Walking on the treadmill and the ground at frequency band of 15GHz.

Micro-Doppler influence of frequency band:

As listed in Table 7, four frequency bands (e.g., 6.7GHz, 15GHz, 28GHz, and 36GHz) are employed to measure the micro-Doppler. In this experiment campaign, it only considers that human body is facing the transmit and receive antennas.

Figure 56 and Figure 57 show the micro-Doppler frequency in Hz as a function of time, t, in case that the sensing is operating at different frequency bands of 6.7GHz, 15GHz, 28GHz, and 36GHz, respectively. In this case, it can be observed that different frequency bands influence micro-Doppler differently, i.e., the higher the frequency band, the more influence on micro-Doppler.

Figure 56: Micro-Doppler influence at the frequency bands of 6.7GHz and 15GHz.

Figure 57: Micro-Doppler influence at the frequency bands of 28GHz and 36GHz.

- Annex C. Simulation results of different path-drop after concatenation methods in Target channel modelling

| Simulation assumption |  |
| --- | --- |
| Scenario | Indoor office in TR38.901 |
| Propagation conditions of target channel | Case1: LOS condition for both TX-Target and Target-RX paths Case2: LOS condition for TX-Target path, NLOS condition for Target-RX path Case3: NLOS condition for TX-Target path, LOS condition for Target-RX path Case4: NLOS condition for both TX-Target and Target-RX paths |
| Concatenation methods | Ray level full convolution between Tx-target link and target-Rx. |
| RCS modelling | RCS=A*B1*B2=1 |
| Path drop for segmented TX/RX-Target paths | No drop |
| Normalization methods after concatenation | No normalization |
| Path-drop after concatenation | Option A: No drop Option B-1: Drop based cluster pair level, where pairs consist of LOS-LOS pair (if present), LOS-NLOS pairs (if present), NLOS-LOS pairs (if present), and NLOS-NLOS pairs Option B-2: Drop based cluster pair level, where pairs consist of LOS-NLOS pairs (if present), NLOS-LOS pairs (if present), and NLOS-NLOS pairs Option B-3: Drop based cluster pair level, where pairs only consist of NLOS-NLOS pairs Option C-1: Drop based ray pair level, where pairs consist of LOS-LOS ray (if present), LOS-NLOS rays (if present), NLOS-LOS rays (if present), and NLOS-NLOS rays Option C-2: Drop based ray pair level, where pairs consist of LOS-NLOS rays (if present), NLOS-LOS rays (if present), and NLOS-NLOS rays Option C-3: Drop based ray pair level, where pairs only consist of NLOS-NLOS rays |
| Threshold used in path-drop after concatenation | -25dB, -35dB, -45dB, -55dB |
| Normalization methods after path drop | Alt A: No normalization |

Evaluation results for the propagation condition Case1 of target channel

Results for Option B-1

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 90601 | 1946 | 2.15% |
| -35dB | 90601 | 8341 | 9.21% |
| -45dB | 90601 | 22141 | 24.44% |
| -55dB | 90601 | 42566 | 46.98% |

Results for Option B-2

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 90601 | 5241 | 5.78% |
| -35dB | 90601 | 18131 | 20.01% |
| -45dB | 90601 | 42366 | 46.76% |
| -55dB | 90601 | 62486 | 68.97% |

Results for Option B-3

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 90601 | 33901 | 37.42% |
| -35dB | 90601 | 54801 | 60.49% |
| -45dB | 90601 | 70201 | 77.48% |
| -55dB | 90601 | 79701 | 87.97% |

Results for Option C-1

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 90601 | 36 | 0.04% |
| -35dB | 90601 | 191 | 0.21% |
| -45dB | 90601 | 586 | 0.65% |
| -55dB | 90601 | 4121 | 4.55% |

Results for Option C-2

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 90601 | 541 | 0.60% |
| -35dB | 90601 | 3331 | 3.68% |
| -45dB | 90601 | 13466 | 14.86% |
| -55dB | 90601 | 34686 | 38.28% |

Results for Option C-3

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 90601 | 33881 | 37.40% |
| -35dB | 90601 | 54791 | 60.48% |
| -45dB | 90601 | 70196 | 77.48% |
| -55dB | 90601 | 79701 | 87.97% |

Evaluation results for the propagation condition Case2 of target channel

Results for Option B-1

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 114380 | 19465 | 17.02% |
| -35dB | 114380 | 46775 | 40.89% |
| -45dB | 114380 | 76780 | 67.13% |
| -55dB | 114380 | 95680 | 83.65% |

Results for Option B-2

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 114380 | 19465 | 17.02% |
| -35dB | 114380 | 46775 | 40.89% |
| -45dB | 114380 | 76780 | 67.13% |
| -55dB | 114380 | 95680 | 83.65% |

Results for Option B-3

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 114380 | 56880 | 49.73% |
| -35dB | 114380 | 84180 | 73.60% |
| -45dB | 114380 | 98980 | 86.54% |
| -55dB | 114380 | 107180 | 93.71% |

Results for Option C-1

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 114380 | 2965 | 2.59% |
| -35dB | 114380 | 13575 | 11.87% |
| -45dB | 114380 | 37680 | 32.94% |
| -55dB | 114380 | 68780 | 60.13% |

Results for Option C-2

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 114380 | 2965 | 2.59% |
| -35dB | 114380 | 13575 | 11.87% |
| -45dB | 114380 | 37680 | 32.94% |
| -55dB | 114380 | 68780 | 60.13% |

Results for Option C-3

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 114380 | 56875 | 49.72% |
| -35dB | 114380 | 84180 | 73.60% |
| -45dB | 114380 | 98980 | 86.54% |
| -55dB | 114380 | 107180 | 93.71% |

Evaluation results for the propagation condition Case3 of target channel

Results for Option B-1

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 114380 | 12045 | 10.53% |
| -35dB | 114380 | 42470 | 37.13% |
| -45dB | 114380 | 76275 | 66.69% |
| -55dB | 114380 | 92975 | 81.29% |

Results for Option B-2

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 114380 | 12045 | 10.53% |
| -35dB | 114380 | 42470 | 37.13% |
| -45dB | 114380 | 76275 | 66.69% |
| -55dB | 114380 | 92975 | 81.29% |

Results for Option B-3

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 114380 | 57480 | 50.25% |
| -35dB | 114380 | 84380 | 73.77% |
| -45dB | 114380 | 99980 | 87.41% |
| -55dB | 114380 | 109180 | 95.45% |

Results for Option C-1

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 114380 | 745 | 0.65% |
| -35dB | 114380 | 6370 | 5.57% |
| -45dB | 114380 | 29975 | 26.21% |
| -55dB | 114380 | 67575 | 59.08% |

Results for Option C-2

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 114380 | 745 | 0.65% |
| -35dB | 114380 | 6370 | 5.57% |
| -45dB | 114380 | 29975 | 26.21% |
| -55dB | 114380 | 67575 | 59.08% |

Results for Option C-3

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 114380 | 57475 | 50.25% |
| -35dB | 114380 | 84380 | 73.77% |
| -45dB | 114380 | 99980 | 87.41% |
| -55dB | 114380 | 109180 | 95.45% |

Evaluation results for the propagation condition Case4 of target channel

Results for Option B-3

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 144400 | 92900 | 64.34% |
| -35dB | 144400 | 123600 | 85.60% |
| -45dB | 144400 | 135200 | 93.63% |
| -55dB | 144400 | 138600 | 95.98% |

Results for Option C-3

| Threshold used in path-drop | Ray number X before path-drop | Ray number Y after path-drop | Y/X |
| --- | --- | --- | --- |
| -25dB | 144400 | 92900 | 64.34% |
| -35dB | 144400 | 123600 | 85.60% |
| -45dB | 144400 | 135200 | 93.63% |
| -55dB | 144400 | 138600 | 95.98% |