3GPP TSG RAN WG1 #120          						R1-2500480

Athens, Greece, Feb 17th – 21st, 2025

Source:	Tejas Networks Ltd.

Title:	Discussion on timing acquisition and synchronization aspects for A-IoT

Agenda item:	9.4.3

Document for:     Discussion and Decision

# Introduction

In Ambient IoT (A-IoT) communication, the timing acquisition and synchronization signals are carried by the preamble, midambles, and postamble associated with the PRDCH/PDRCH signals. Each physical layer signal starts with preamble and ends with postamble. The midambles are added if needed based on the length of the frame and clock accuracy.  The functionality of postamble is to indicate the end of the PRDCH or PDRCH frame. Similar to the start indicator part of preamble, the postamble also may contain a known sequence to indicate the end of the frame.

# Functionalities of Preamble for Timing Acquisition and Synchronization

Agreement

At least the following time domain frame structure is studied for A-IoT R2D and D2R transmission.

- For R2D transmission,

  - A R2D timing acquisition signal (e.g. R2D preamble) is included at least for timing acquisition and for indicating the start of the R2D transmission in time domain.

- For D2R transmission,

  - A D2R timing acquisition signal (e.g. D2R preamble) is included at least for timing acquisition and for indicating the start of the D2R transmission in time domain.

- FFS other necessary component(s), e.g. midamble, postamble, periodic sync signal, control fields, guard period

The following agreements were made in RAN1 #117 on the R2D and D2R timing acquisition signal [1]:

The preamble sequence consists of two parts: 1) start indicator and 2) clock-acquisition. In R2D signal, by detecting the length of clock-acquisition signal, the device estimates the M-value of the received PRDCH signal. Since CP length is dependent on the M-value, the device can estimate the CP length of the received PRDCH signal form the M-value. In D2R signal, by detecting the length of clock acquisition signal, the reader can estimate the chip-length of the device.

The functionality of preamble includes:

- Indicating the start time of PRDCH/PDRCH frame

- Clock synchronization

- Indicating the M-value for R2D transmission or the chip length for D2R transmission.

Observation 1: In R2D signal, by detecting the length of clock-acquisition signal, the device estimates the M-value of the received PRDCH signal. Since CP length is dependent on the M-value, the device can estimate the CP length of the received PRDCH signal form the M-value. In D2R signal, by detecting the length of clock acquisition signal, the reader can estimate the chip-length of the device.

Proposal 1: The functionality of preamble includes:

- Indicating the start time of PRDCH/PDRCH frame

- Clock synchronization

- Indicating the M-value for R2D transmission or the chip length for D2R transmission.

# Functionalities of Midambles for Timing Acquisition and Synchronization

If the frame length is large, midambles are used to maintain the clock synchronization as the sampling frequency offset of the clock of A-IoT device is very high; in the order of 104-105 PPM. Moreover, if the length of PRDCH/PDRCH frame is higher than the maximum allowed message size, the frame is divided into multiple segments. The ending and staring of each segment can be indicated using midambles.

The functionalities of midambles include:

- Clock synchronization

- Indicate the continuation of a single message signal.

Proposal 2: The functionalities of midambles include:

- Clock synchronization

- Indication of the continuation of a single message signal.

# Conclusion

This work includes the functionalities of preamble, midambles, and postamble for timing acquisition and synchronization of A-IoT. We have made the following observations and proposals related to the above-mentioned aspects:

Observation 1: In R2D signal, by detecting the length of clock-acquisition signal, the device estimates the M-value of the received PRDCH signal. Since CP length is dependent on the M-value, the device can estimate the CP length of the received PRDCH signal form the M-value. In D2R signal, by detecting the length of clock acquisition signal, the reader can estimate the chip-length of the device.

Proposal 1: The functionality of preamble includes:

- Indicating the start time of PRDCH/PDRCH frame

- Clock synchronization

- Indicating the M-value for R2D transmission or the chip length for D2R transmission.

Proposal 2: The functionalities of midambles include:

- Clock synchronization

- Indication of the continuation of a single message signal.

# 4. References

[1] 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #117, Fukuoka, Japan, May 20th – May 24th, 2024.