3GPP TSG RAN WG1#120		R1-2500486

Athens, Greece, February 18th – 22nd, 2024

Source:	Panasonic

Title: 	Discussion on enabling TX/RX for XR during RRM measurements

Agenda Item:		9.10.1

Document for:	Discussion

# Introduction

A new work item for XR enhancements was approved for Rel. 19 [1]. The objectives include enabling the transmission (TX)/reception (RX) in gaps/restrictions, due to RRM measurement, which can reduce the latency and improve the capacity [2]. In this contribution, we present our views on enabling the TX/RX during the RRM measurement gaps.

# Discussions

A measurement gap (MG) is used by a UE to perform the RRM measurement if it is not capable of measuring a target carrier while transmitting or receiving on the serving cell. The MG could be configured for the intra-frequency, inter-frequency, and inter-RAT measurements. The MG length could be 1.5, 3, 3.5, 4, 5.5, and 6 ms and with the periodicity of 20, 40, 80, and 160 ms [3]. The integer periodicities defined for the MG configuration does not allow aligning the MG instances with the XR video frame arrival, which would result in the collision of MG instances and video frame arrivals in time domain. To address this issue, it was agreed to use a dynamic skipping indication carried by a DCI. A DCI can carry a single bit indicating whether to skip the next RRM gap, considering the minimum processing time.

Here we discuss some other remaining issues related to the MG skipping indication.

Multiple MG configurations with different priorities

A UE could be configured with different MG configurations with different priorities. When the MG occasions with different priorities collide in time domain, the UE is supposed to drop the MG with the lower priority and apply the MG occasion with the higher priority.

When the MG skipping indication is introduced, the indication could be interpreted differently as following when the MG occasions collide.

- Interpretation A: The MG skipping is applied before handling the MG collisions.

- Interpretation B: The MG skipping is applied after handling the MG collisions.

The result could be different for these two interpretations. As shown in the Figure 2, the UE is configured with two MG configurations, the first MG configuration is assigned with a high priority and the second MG configuration is assigned with low priority. The UE receives a DCI indicating to skip the next MG occasion. The two following MG occasions collide in time. Figure 1(a) shows the interpretation A. The UE first applies the MG skipping indication, which results in skipping MG1. Figure 1(b) shows the interpretation B. The UE first applies the MG collision handling, which results in skipping MG2 that has a lower priority. The UE then applies the DCI indication for skipping the MG occasion, which results in skipping MG1. To avoid the misalignment between the gNB and the UE, the order of applying MG skipping indication and handling the MG collision should be defined.

Proposal 1: The order of applying MG skipping indication and handling the MG collision should be defined.

Figure 1. MG skipping indication with MG collision handling.

Applicable MG configurations

Using a single bit as a MG skipping indication would limit the flexibility to select which MG occasion to be skipped, especially when more than one MG configuration is configured for the UE. In case there are two MG occasions after each other overlap in time domain, the gNB may not be able to indicate to skip the one that begins later. In addition, some of MG occasions might have high priority and should not be skipped. For these reasons, the gNB should be able to indicate MG occasions from certain configuration(s) to be skipped. This can be achieved by defining applicable MG configuration(s) for the MG skipping feature or each DCI format, as part of RRC configuration.

Proposal 2: The applicable MG configuration(s) to be skipped should be defined for the MG skipping feature or each DCI format separately.

# Conclusions

In this contribution, we provided our view on MG enhancements. We made the following proposals.

Proposal 1: The order of applying MG skipping indication and handling the MG collision should be defined.

Proposal 2: The applicable MG configuration(s) to be skipped should be defined for the MG skipping feature or each DCI format separately.

# Reference

[1] 	RP-234057, “New WID: XR (eXtended Reality) for NR Phase 3,” RAN#102.

[2] 	TR 38.835, “Study on XR enhancements for NR,” V18.0.1.

[3] 	TS 38.133, “Requirements for support of radio resource management,” V18.4.0

[4]	R1-2410742, “Moderator summary #2 - Enabling TX/RX for XR during RRM measurements”