3GPP TSG-RAN WG1 #120	R1-2500534

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.1.4.1

Source:	InterDigital, Inc.

Title:	On AI/ML-based CSI compression

Document for:	Discussion and Decision

# Introduction

In RAN #106, the study part of the AI/ML for NR Air Interface work (NR_AIML_air, [12]) was captured in a separate study item [16].

For the CSI enhancement use case, the objective of the study is to address some of the outstanding issues identified during the Rel-18 Study Item, as summarized in TR 38.843 [3].

The study objectives for CSI compression are shown below[16].

Study objectives: - CSI feedback enhancement [RAN1]: - For CSI temporal prediction: further update TR 38.843 with additional evaluations. - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

The study of spatial/temporal/frequency compression started in RAN1 #116 [4], where the following categorization was adopted for study.

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following categorization for study: Case; Target CSI slot(s); Whether the UE uses past CSI information; Whether the network uses past CSI information. 0; Present slot; No; No. 1; Present slot; Yes; No. 2; Present slot; Yes; Yes. 3; Future slot(s); Yes; No. 4; Future slot(s); Yes; Yes. 5; Present slot; No; Yes.

During RAN1 #118 [13] it was agreed to prioritize further discussion on Case 2 and Case 3.

Agreement For temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, among Cases 1, 2, 3, 4, and 5, prioritize further discussion on Case 2 and Case 3.

Regarding the inter-vendor training collaboration, RAN1 #118b agreements [15] include the following.

Agreement For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression, - For Direction A: - Further study the following options with potential down-selection - 4-1: dataset is {target CSI, CSI feedback}, - 3a-1: encoder parameter exchange, with {target CSI} - Note: Additional information beyond what’s listed above may need to be exchanged if necessary.

Moreover, in RAN1 #119 [17], for studying the standardized model structure, the following agreement was made.

Agreement For studying the standardized model structure, - For temporal domain Case 0, - In case of spatial-frequency domain input, adopt Transformer as the backbone structure. - Further study angular-frequency and angular-delay domain input - For temporal domain Case 2, - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation with additional layers - Conv-LSTM - LSTM - Option 2: Latent adaptation on top of Case 0 model structure - Reusing Case 0 structure and changing quantization operation and/or feedback payload over different slots - Note: other options are not precluded - Further study angular-frequency and angular-delay domain input - For temporal domain Case 3, - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation - Option 2: Latent adaptation - Further study angular-frequency, angular-delay, and angular-delay-Doppler domain input. - Take precoding matrix in the spatial-frequency domain in a subband granularity as the input, and the reconstructed precoding matrix in the spatial-frequency as the output, for the study of standardized model structure. - Note: Processing steps at the UE to derive angular, delay, and/or Doppler domain basis, and corresponding reconstruction steps at the gNB are considered. - Note: scalar/vector quantization and dequantization are considered. Quantization-aware training with jointly updated quantization method/parameters (Case 2-2) is used.

This contribution discusses aspects of CSI compression performance, including the impact of pre-processing methods such as beam-domain processing on the overall compression performance and complexity. We discuss options on UE-sided and NW-sided model monitoring, and present considerations regarding Direction A for addressing inter-vendor training collaboration.  Additionally, we provide simulation results for Option 3a-1 of inter-vendor training collaboration, as well as for beam domain pre-processing and evaluation results of temporal domain aspects of AI/ML-based CSI compression for Case2 (with and without missing UCI, and mitigation of missing UCI), and for Case 3 scenarios.

# Considerations on AI/ML-based CSI compression

## CSI compression performance improvements

During the Rel-18 SI, the performance benefit and potential specification impact were studied for the AI/ML-based CSI compression sub-use case. With respect to the performance benefit of AI/ML-based CSI compression, the conclusion section of TR 38.843 [3] identified the trade-off between performance and complexity/overhead as one area that requires further study.

Aspects that impact complexity include ML model complexity, model size, as well as the number of ML models that a UE may need to support, to name a few. The evaluation of model scalability over Tx port numbers, summarized in Section 6.2.2.3 of TR 38.843 [3], shows significant performance degradation for generalization Case2. While this degradation may be partially mitigated using a mixed training dataset, along with a zero-padding approach to ensure same input dimensionality, the results for generalization Case3 show that moderate (and sometimes severe) performance loss may still be experienced.  In such cases, a potential solution may consist of UE-side model switch. However, the UE-side model switch incurs signalling overhead and has an associated latency.

### Beam domain processing for further compression performance improvement

Pre-processing in the beam domain (discussed in [8]) may be used effectively to reduce the need for model switching in this scenario. Beam domain processing (BDP) is a dimensionality reduction step that transforms the high-dimensional antenna domain channel to a low-dimensional beam domain channel, possibly with a minor loss of information. This can be considered as an efficient feature extraction step that captures most of the spatial information in a few beams that need to be carefully selected from a predefined codebook. The beam-frequency channel can then be compressed and decompressed using an AE model that operates on low-dimensional channel samples. Then, at the output of the decoder, a beam domain postprocessor may be used to transform the channel back to the spatial domain using the selected beams.

The BDP alleviates the need for using complex/large-size AE models with large input dimensionality, which in turn results in a reduction in the memory and computational requirements relative to the spatial-domain AE. In addition, BDP results in AE models that are agnostic to the number of antennas, so one model operating in the beam domain can handle different configurations with variable antenna sizes. This provides more flexibility from a system perspective and may reduce the need for model switching.

Observation 1: Beam domain processing results in low-dimensional channel samples that may be compressed using low complexity Autoencoder models, resulting in reduction in both memory and computational requirements relative to spatial domain Autoencoders.

Observation 2: Beam domain processing exhibits good generalization capabilities as a single beam domain AE model can deal with multiple antenna configurations, potentially reducing the need for model switching.

### TSF compression for further compression performance improvement

Another way to further improve the performance of the spatial-frequency (SF) compression is to leverage the temporal correlation properties of the channel in the compression process, namely temporal-spatial-frequency (TSF) compression. The past CSI may be used as a prior information along with the current CSI to further improve the performance of the SF compression. The improvement may be in terms of overhead reduction, namely how much reduction in the overhead can the temporal prior information provide, while achieving the same reconstruction performance (e.g., NMSE or SGCS) as SF. The improvement can also occur in the reconstruction performance at a given overhead, namely how much improvement in the reconstruction performance can the temporal prior information yield at the same overhead as SF. The SF compression can be viewed as a special case of TSF where the prior information is neglected. From a complexity perspective, TSF models may be more complex, depending on how the prior information is utilized. For example, one way to incorporate the prior information in the compression process is to use AI/ML models with recurrent neural network architecture, e.g., LSTM, where the prior information may be used at the encoder side, or decoder side, or both.  Moreover, the length of the observation window (to capture the temporal correlation of the channel) impacts both model performance and complexity (see section 3.1.4 for the complexity evaluation of a 2-frame and a 4-frame TSF CSI compression model).  Therefore, the complexity-performance study should also consider the length of the observation window.

Observation 3: While TSF compression may potentially result in further improvement in the compression performance relative to SF compression, it may require more complex AE models with higher computational and memory requirements relative to SF.

As captured in the observations endorsed in RAN1 #118 [14], CSI Compression Case 2 provides a small to moderate performance gain (in terms of mean UPT) with respect to CSI Compression Case 0.  As mentioned above, the gain comes from leveraging the temporal correlation properties of the channel, where longer observation window (longer CSI historical/temporal information) may further improve the performance for slow to moderate fading channels. However, when the channel changes fast (e.g., due to higher UE speeds), the correlation time of the channel decreases, so maintaining a long history buffer may not be beneficial. To accommodate to a wider range of UE speeds, it may be necessary to study the benefits and specification impact of adapting the observation window length to the channel conditions, such as UE speed.

Proposal 1: Study the specification impact of adapting the length of the historical CSI buffer to channel conditions, such as UE speed.

Using historical CSI information at the NW and the UE requires good synchronization of the historical buffers between the UE-side CSI generation model and the NW-side CSI reconstruction model. This makes the CSI Compression Case 2 performance sensitive to missing UCI (which may occur either due to dropped CSI reports or to incorrect reception).

To mitigate the missing UCI errors, mechanisms are needed to align the UE-side and the NW-side historical CSI information; the mechanisms may include the full or partial reset of the UE-side and NW-side buffers.  The partial buffer reset (where the buffers are reset to the most recent point where the NW-side and the UE-side are in-synch) has the potential to outperform the full buffer reset approach, as it restores the historical CSI information quicker than in case of the full buffer reset.  For both mitigation approaches, the specification impact includes signaling the buffer reset, including timing information on when to perform the reset.

Observation 4: Specification impacts of mitigation of missing UCI for CSI Compression Case 2 include signaling and timing information to maintain synchronization of the historical CSI information between the UE-side and NW-side.

## Model monitoring for AI/ML-based CSI compression

With reference to a two-sided AI/ML model for the CSI compression sub-use case, the model may be trained jointly or separately. Once the trained model is deployed at the UE, the UE-part AI/ML encoder performs inference based on received over-the-air transmissions that may have different statistics compared to the statistics of the training datasets. While the models may use mixed datasets for training and validation, it is still possible that the AI/ML encoder does not generalize well across all realistic channel conditions.

The generalization performance for CSI performance was evaluated during the Rel-18 SI, and the results summarized in Section 6.2.2.2 of TR 38.843 show that even with generalization Case3, there are scenarios where moderate performance loss is observed.  This suggests model monitoring is needed to enable the mitigation of potential performance degradation due to poor model generalization.

Furthermore, the Rel-18 SI investigated UE-side performance monitoring, whereby the UE monitors the model performance and reports, e.g., intermediate KPI to the NW, while the NW makes the LCM decisions (of model/functionality activation/deactivation/updating/switching).

One of the options for UE-side model monitoring (using intermediate KPI) that was discussed is based on the output of the CSI reconstruction model (proxy model) at the UE-side.  In this approach, the UE could use the proxy decoder to reconstruct the CSI (based on the compressed CSI which is reported back to the NW), and compare the reconstructed CSI to the CSI measured by the UE. However, the availability of proxy decoders at the UE side may not always be possible.  Moreover, the need for the UE to support different gNB Tx antenna port configurations, or variable BW size may result in the need for the UE to support multiple UE-side ML encoders, and potentially multiple proxy decoders. This may excessively increase the UE complexity and the memory storage; it may also increase the UE power consumption, since the UE may need to run the proxy decoder solely for monitoring the performance of the UE-side ML encoder.

Therefore, lower complexity solutions are needed for the UE to determine the compression performance for ML encoder performance monitoring.

For UE-side monitoring, the UE may need to report the monitoring metric, for example if the selected metric is an intermediate KPI such as the SGCS, NMSE or other; the reporting may be periodic or aperiodic. For this example, triggers for monitoring metric reporting may also need to be considered. The triggers may be time-based and/or event-based. Examples of event-based triggers include: measurements of intermediate KPIs meeting configured thresholds, PDSCH ACK/NACK performance, change in scenario (including channel conditions such as Doppler), change in beams, TRP change, cell change, etc. Lastly, signaling the UE-side monitoring metrics may need to be specified.

Observation 5:	For UE-side monitoring, both time-based trigger and event-based trigger needs to be studied for reporting the monitoring metrics.

The monitoring metric needs to be carefully studied to avoid unnecessary overhead due to model updating or switching. For example, in some scenarios, SCGS used as an intermediate KPI may not be a good indicator of the eventual KPI performance. For example, in the simulation results in our contribution [9], a 25% gap in the SGCS performance of two different models only translated to a throughput gap of 5% between the models, which may not justify model switching.

Observation 6:	For UE-side monitoring, appropriate monitoring metrics need to be identified (e.g., other than SGCS) to avoid unnecessary model updating or switching considering that SGCS couldn’t reflect AI/ML model performance in a reasonable manner.

As discussed above, model monitoring based on intermediate KPIs may not be indicative of the system performance. At the same time, system KPIs may not be sufficient to identify the cause of performance degradation.  UE-side performance monitoring based on data distribution may be considered, where out-of-distribution metrics UE measured by the UE may increase the accuracy of the model monitoring.

Observation 7:	Out-of-distribution metrics for UE-side monitoring can be another candidate as monitoring metric but further study is necessary.

While model monitoring is important for the detection of potential model performance degradation, reporting the UE-side metrics (e.g. intermediate KPIs such as SGCS and/or NMSE) may introduce additional feedback overhead. Further study of the monitoring metrics report size, quantization of the metrics, frequency of reporting, and thresholds for triggering reports is needed to avoid increasing the feedback overhead.

Observation 8: 	For UE-side monitoring, monitoring metrics in terms of report size, metrics quantization, and report frequency also needs to be studied to investigate feedback overhead.

For UE-side monitoring based on the output of the CSI reconstruction model, the reconstructed CSI needs to be indicated by the NW or obtained from the network side. The format of this indication needs to be specified; in addition to the reconstructed CSI, the indication may include information such as the type of CSI (full channel matrix or eigenvector), as well as means to identify the CSI report associated to the reconstructed CSI. While this monitoring method has the advantage of not requiring a CSI reconstruction model at the UE-side, it may result in additional downlink signaling overhead.

Observation 9: 	UE-side monitoring based on the output of the NW-side CSI reconstruction model may increase the downlink signaling overhead, because the output CSI reconstructed at the NW needs to be indicated by the NW to the UE.

The following agreement was reached in RAN1 #117 [11] regarding monitoring options:

Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook

The difference between actual CSI at the UE and the reconstructed CSI at the NW could cause performance degradation if the difference is above a threshold. The level of difference could be measured at UE side and used as another end-to-end performance monitoring metric for CSI compression. In case the UE does not have access to the reconstructed CSI at the NW, the UE may compare the expected precoding gain with the actual precoding gain using precoded RS (e.g., CSI-RS, DMRS). If the difference between actual CSI and the reconstructed CSI is low, then the difference between actual precoding gain measured at UE using received precoded RS and expected precoding gain computed at UE should also be low. Otherwise, the UE may detect mismatch between actual CSI at the UE and the reconstructed CSI at the NW if the difference between actual precoding gain and expected precoding gain is high.

Observation 10: Precoded RS (CSI-RS, DM-RS) can be used for UE-side model monitoring, by comparing the expected precoding gain to actual precoding gain and determining the mismatch between actual CSI at UE and reconstructed CSI at NW.

The Rel-18 SI also investigated NW-side performance monitoring, whereby the NW monitors the performance and make decisions of model/functionality activation/ deactivation/updating/switching.  As indicated in Section 7.1.2 of TR38.843, the metric for monitoring and comparison includes intermediate KPI and eventual KPI. To calculate the intermediate KPI at the NW-side, it may require an existing CSI feedback scheme as the reference, which may result in higher than desired overhead.

To enable NW-side monitoring with lower feedback overhead, another option may be to use a set of reference vectors (predefined and available at both NW-side and UE-side), such that metrics relative to the reference vectors and output CSI, as well as reference vectors and input CSI may be measured both at the NW-side and UE-side, respectively.

To enable NW-side monitoring with minimal feedback overhead, another option may be to insert a small number of redundancy elements to the input CSI vector of the encoder at the UE, so that the NW can use the reconstructed redundancy elements to estimate the quality of the reconstructed CSI after decompression. This option may allow the NW to monitor the quality of the reconstructed CSI for each CSI feedback.

Observation 11:  NW-side monitoring with lower signaling overhead options can be studied further in Rel-19 as alternative to the UE-side monitoring.

Based on discussions and observations above, we propose following for UE-part model monitoring:

Proposal 2:	Study further the following aspects for model monitoring in Rel-19:

- Details of reporting mechanism for the monitoring metrics with both time/event-trigger based

- Appropriate UE-side monitoring metric which reflects AI/ML model performance accurately

- UE-side monitoring based on precoded RS (CSI-RS, DM-RS)

- Reporting contents/structure of UE-side monitoring metric and its associated feedback overhead

- NW-side monitoring with lower signaling overhead

In the context of the inter-vendor training collaboration, specifically for Options 1/3/4/5, it was identified that UE/UE vendor-side off-line engineering of the UE-side CSI generation may lead to interoperability issues when the UE-side CSI generation is deployed in the field. To address this issue, RAN1 #117 reached the following agreement.

Agreement For option 1 / 3 / 4 / 5 and their sub-options, study mechanisms (e.g., post-deployment performance monitoring) for identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation to guarantee good performance in the field.

To identify the error cause, the UE may use UE-side model monitoring measurements such as the intermediate KPIs (SGCS, NMSE) or other statistics, such as HARQ ACK/NACK. Additionally, the UE may need to provide out-of-distribution (OOD) metrics with respect to the reference dataset. The reference dataset is exchanged from the NW-side to the UE-side in Option4, and may be shared as additional information (also from the NW-side to the UE-side) for Options 3a/5a. For these cases, the UE can perform OOD measurements for the inference data relative to the reference dataset, for example when configured to perform measurements. The UE may use the determined monitoring metrics and the OOD metrics to assess the error cause and may report the UE-assessed error cause (e.g., data drift, UE-side, or undetermined). Mechanisms for fallback to legacy CSI feedback need to be studied. Additionally, mitigation mechanisms and potential specification impacts need to be studied, for example for the case where the UE has more than one applicable model corresponding to the current conditions and may be able to perform model switch to an alternate model, including the standard reference model, if applicable.

Proposal 3: Mechanisms for identifying the cause of performance degradation include:

- UE assistance information such as UE-side model monitoring metrics and UE-side out-of-distribution metrics

- Reporting the UE-assessed error cause (e.g., data drift, UE-side, or undetermined)

- Mitigation mechanisms, including fallback to legacy CSI reporting, model switching

# Performance evaluation for AI/ML-based CSI compression with different pre-processing

## AI/ML Model architecture

For the CSI compression use case, we utilize multiple neural network models with varying configurations to accomplish the compression task. The discussed neural network models can be broadly classified as CNN based or transformer based.

### CNN based models

For CNN based models, we utilize two different architectures. The first is based on the CSI-Net model proposed in [5] and the second is based on the EVCsiNet model proposed in [6]. Next, we provide architectural details of the two models.

- Input CSI Type:

  - raw channel matrix estimated by UE

  - eigenvectors derived from the raw channel matrix

- Output CSI type: compressed channel matrix and compressed eigenvectors

- Pre-processing – for the full channel compression

To effectively compare the performance of the existing CSI feedback and precoding methods with the deep learning-based methods, we utilize the following pre-processing strategy:

  - we average the channel matrix across 2 resource blocks and across 1 time slot. Therefore, if the size of the raw channel is 624x2x16, representing 52 RBs, Nt = 16 and Nr = 2, the pre-processed channel will have a dimension of 26x2x16, respectively.

  - Additionally, we normalize the channels to zero mean and unit variance.

- Pre-processing – for the eigenvector based compression

  - Similar to the full channel case, first, we average the channel across 2 resource blocks and across 1 time slot. Thus, getting a channel of dimensions 26x2x16.

  - Next, we evaluate the SVD of the channel for each of the 26 sub-bands to evaluate the Nt ×1 = 16×1 dimensional eigenvectors. The Nr = 2, eigenvectors are then ordered based on their eigenvalues, from the largest to the smallest. Depending on the training configuration, the eigenvectors corresponding to a specific ordering, or a group of ordering may be selected for training.

- Quantization

The output of the encoder is quantized using a uniform quantizer. The encoder output is passed through a tanh layer to restrict the range of the encoder output for uniform quantization. The quantization operation is included during the training so that the encoder and decoder can learn appropriate weights while taking into account the quantization impact.

- Model Information:

- CSI-Net based Model

  - Base Model: For our evaluation we use the CSI-Net [5] autoencoder model. The model has 3 main functional blocks. Encoder block, Quantization, Decoder block

    - Encoder architecture: CNN->BN->FC->FC

    - Quantizer: Linear quantization

    - Decoder architecture: FC-> FC -> RN->RN->CNN, where:

      - FC: Fully connected layer,

      - CNN: Convolutional Neural Network

      - BN: Batch Normalization

      - RN: CNN->BN->CNN->BN->CNN->BN with a skip connection from the input to the RN block

  - Feedback size: 64, 96, 128, 192 or 256 bottleneck elements with 2 bits per element

  - Loss function

    - We utilize the mean squared error loss function for training. The mean squared error is calculated between the input to the encoder and the output of the decoder.

  - Additional information:

    - Optimizer: ADAM with adaptive learning rate starting from a rate of = 0.0001 and scale it down by a factor of 0.87 every 5 epochs.

- EVCsiNet based Model

  - Base Model: For our evaluation we use a modified version of the EVCsiNet [6] autoencoder model. The model has 3 main functional blocks. Encoder block, Quantization, Decoder block

    - Encoder architecture: FC->BN->FC

    - Quantizer: Linear quantization

    - Decoder architecture: FC-> followed by CNN layers and skip connections,

      - FC: Fully connected layer,

      - CNN: Convolutional Neural Network

      - BN: Batch Normalization

### Transformer based models

  - Base Model: For our evaluation we use a modified version of the Transformer architecture proposed in [7]. The utilized model has 3 main functional blocks. Encoder block, Quantization, Decoder block

    - Encoder architecture: CNN->Tf-block-> Tf-block -> Tf-block -> Tf-block ->FC

    - Decoder architecture: FC->Tf-block-> Tf-block -> Tf-block -> Tf-block ->CNN

    - Quantizer: Linear quantization

      - FC: Fully connected layer,

      - CNN: Convolutional Neural Network

      - Tf-block: Multiheaded-attention block->FC->FC->LN with skip connections.

### TSF model

TSF CSI Compression Case 2 model

The model for temporal-spatial-frequency compression comprises of transformer blocks, as described earlier for the spatial-frequency compression setup. For a setup with N frame TSF compression, N encoder-decoder blocks are trained. For the first CSI frame, a model with multiple transformer blocks is utilized to compress the first CSI frame. For compressing the i-th CSI frame, priors associated with each of the [1 to i-1] CSI frames as well as the i-th CSI frame are provided as input to the encoder block for compression. A similar strategy is followed for the decoder side models too, where the decoder associated with first CSI frame decodes the data independently, whereas for decoding the i-th CSI frame, priors associated with each of the [1 to i-1] reconstructed frames as well as the i-th compressed frame are utilized for the decoding.

TSF CSI Compression Case 3 model

For evaluation of case 3, we employ a prediction block followed by a compression block and train each block independently. The prediction block uses K channel matrices as input to predict N4 = 1 channels at the output, where the prediction block may be an AI/ML model or the Kalman Filter. The AI/ML prediction model utilizes the same pre-processing, architecture and training scheme as we have detailed in our CSI prediction contribution. We predict each resource block independently and then combine the results in a post-processing step.

In the case 3 framework, the N4 predicted channels are compressed and quantized by the encoder at the UE, transmitted to the NW in a CSI report, then dequantized and decompressed by the NW hosted decoder. The compression scheme may be an AI/ML model or Release 18 Codebook. Our AI/ML compression model comprises of transformer blocks, as described earlier for the spatial-frequency compression setup. We train our AI/ML compression model using the ideal channel eigenvectors as the inputs and ground truth, as opposed to the predicted channel eigenvectors as the inputs. Within the case 3 framework this can be thought of as training the model using perfect predictions. Note that the compression model may be finetuned using the predicted channels.

### Model Complexity

TSF CSI Compression Case2

The model complexity of the CSI generation part and CSI reconstruction part for the TSF CSI Compression Case2 are shown in Table 1 and Table 2 for the 2-CSI frame and the 4-CSI frame architecture, respectively.

Table 1 Model complexity for the 2-Frame TSF CSI Compression Case2

|  | FLOPs/M | Parameters/M | Storage/Mbytes |
| --- | --- | --- | --- |
| CSI generation | 115 | 2.36 | 30 |
| CSI reconstruction | 115 | 2.36 | 30 |

Table 2 Model complexity for the 4-Frame TSF CSI Compression Case2

|  | FLOPs/M | Parameters/M | Storage/Mbytes |
| --- | --- | --- | --- |
| CSI generation | 157 | 3.16 | 39 |
| CSI reconstruction | 157 | 3.16 | 39 |

TSF CSI Compression Case3

The model complexity for TSF CSI Compression Case 3 is shown in Table 3, where the CSI generation part comprises the prediction block plus the encoder and the CSI reconstruction part consists of the decoder. The latent dimension is 30.

Table 3 Model complexity for TSF CSI Compression Case3

|  | FLOPs/M | Parameters/M | Storage/Mbytes |
| --- | --- | --- | --- |
| CSI generation(Prediction + Encoder) | 816 | 1.16 | 13MB |
| CSI reconstruction | 25M | 1.00 | 11 |

### Datasets for model training

To evaluate the performance of the AI/ML models, we consider the dataset per the simulation assumptions in [3].  More details on the dataset and the configurations used can be found in the Appendix.

For beam-domain AE and spatial-domain AE training, a dataset containing 25k channel samples, using UMi channel scenario, has been utilized; 80% of the dataset is used for model training, while 20% is used for validation.  For BDP, a DFT codebook is used with oversampling factors O1 and O2 are set to 2, N1= 4, N2= 2 with double polarization. The number of selected beams (L) is varied from 2 to 6.

For the TSF model training, a dataset with 210k channel samples, generated based on the configurations listed in the Appendix; 80% of the dataset is used for model training, while 20% is used for validation.

## CSI compression baseline: Rel-16 Type II

Rel-16 type II codebook will be used as the performance baseline to show the gain of AI/ML based CSI compression. Table 6.3.2.1.2-1 of TS 38.212 shows the details about how to calculate PMI payload of each rank for Rel-16 Type II codebook. Given N1 = 4, N2 = 2, O1 = O2 = 4, and the number of subbands = 13, the PMI overhead of paramCombination from 1 to 6 is shown in Table 4.

Table 4 PMI overhead for Rel-16 Type II codebook

| paramCombination | Rank 1 | Rank 2 | Rank 3 | Rank 4 |
| --- | --- | --- | --- | --- |
| 1 | 61 | 111 | 98 | 109 |
| 2 | 89 | 167 | 154 | 165 |
| 3 | 107 | 203 | 183 | 203 |
| 4 | 164 | 315 | 295 | 315 |
| 5 | 221 | 427 | 467 | 507 |
| 6 | 275 | 535 | 523 | 563 |

For the rank > 1 transmission cases in system-level simulation, we use the weighted overhead based on the rank distribution during the simulation to find the final overhead.

## CSI compression simulation configuration

The common parameters for dataset generation and for performance evaluation are as shown in [3]; for convenience, the common parameters table is also included in the Appendix.

## Evaluation on CSI compression performance improvement

### BDP-based CSI compression simulation results – Case 0

In this section, we evaluate the performance of the AE with beam domain processing. First, it is important to highlight that the performance of this two-step compression approach (BDP + AE) is dependent on the reconstruction quality of both BDP and AE together. For BDP, the key driver of the performance is how to find the set of beams that results in a minimal loss of information. While increasing the number of beams may improve the reconstruction performance, it incurs additional overhead as each beam needs to be signalled for transforming the beam-domain channel back to the antenna domain. The overhead associated with the BDP is given by L log2N1N2O1O2, where L is the number of selected beams, N1 and N2 is the number of antennas in the horizontal and vertical domain, respectively,  O1 and O2 are the associated oversampling factors. For example, by using the following configuration for BDP N1=8, N2=2, O1= O2=4, then each beam introduces 8 bits of overhead. It is then important to find the minimum number of beams that results in an acceptable reconstruction performance from a BDP perspective.

The beams are selected based on the wideband channel which is obtained through averaging out the spatial-frequency channels across all RBs. In addition, it is assumed that the beams are selected across one of the polarizations and reused across the other polarization. This reduces the signalling overhead associated with the selected beams. Once the beams are selected, the beam-frequency channel may be obtained by projecting the channel associated with each subband on the selected beams. The size of the transformed beam-frequency channel is 2L × Nr× NSB, where NSB is the number of sub-bands in the frequency domain and each sub-band represents the averaged channel over a number of RBs, e.g., 2.

Figure 1 shows the impact of the beam selection algorithm on the BDP reconstruction performance. Three different algorithms are evaluated and can be briefly described as follows. Algorithm A projects the covariance matrix of the channel on the oversampled codebook and picks the L beams that preserve most of the information through minimizing the BDP reconstruction error. Algorithm B is a more efficient low-complexity greedy method that picks the beams that preserve most of the energy while minimizing the redundancy between the selected beams. Algorithm C employs an exhaustive search procedure to select the best L beams but it has high computational requirements. From Figure 1, it can be seen that different algorithm can result in different BDP reconstruction performance. We observe that selecting the beams that preserve most of the spatial information while eliminating the redundancy (overlapping information) between the selected beams is a reasonable criterion that results in a minimal loss of information using a few beams.

Figure 1 BDP NMSE reconstruction performance as a function of the number of selected beams for three different beam selection algorithms

Algorithm B is adopted to generate the evaluation results in the remainder of this section. Since the criteria used in Algorithm B is to perform the selection in a way that eliminates redundancy across the selected beams, it makes the feature extraction step using BDP more efficient and in line with the goal of the compression task in general. This, however, will leave the AE to handle the frequency domain compression but not too much room for compressing the beam domain information.  While increasing the number of beams will improve the reconstruction performance of the BDP, it negatively impacts the AE reconstruction performance as the number of AE inputs scales up with the number of beams.

Figure 2 shows the reconstruction performance as a function of the number of beams, for the BDP only (blue curve), beam domain AE only (green curve), and the end-to-end/combined performance of BDP and AE.  The simulation results show that aggressively reducing the number of beams will result in a high combined NMSE, that is dominated by the poor performance of the BDP.  Moreover, the results show that significantly increasing the number of beams will also result in a high combined NMSE, that is dominated by the poor performance of the beam domain AE. It can be seen that choosing two beams serves as a sweet-spot for the end-to-end performance.

Figure 2 NMSE reconstruction performance as a function of the number of selected beams for BDP only, beam domain AE only, and combined performance

Figure 3 shows the reconstruction performance of the AE using beam domain processing using two beams, compared to a spatial domain AE. The results show that the AE with beam domain processing outperforms the AE with spatial domain processing (it has lower reconstruction error/NMSE at the same feedback overhead, and lower overhead for the same reconstruction error). For the two-step approach (BDP+AE), red curve in Figure 3, the number of bits is calculated based on the sum of the BDP overhead and the beam-domain AE overhead.

Figure 3 Reconstruction performance of the spatial-domain AE compared to the end-to-end performance of AE with beam domain processing

Observation 12: AE with beam domain processing outperforms the AE with spatial domain processing in terms of intermediate KPI.

Lastly, the complexity of the AE with beam domain processing for different number of beams is compared to the complexity of the spatial-domain AE, in terms of number of model parameters and number of floating-point operations (FLOPs). The data is summarized in Table 5, and it can be seen that the beam-domain AE has lower complexity than the spatial-domain AE, in terms of number of model parameters and FLOPs, even when a relatively large number of beams is used. In Table 5, the number of flops for the two-step approach include the sum of the number of FLOPs of the beam domain AE in addition to the number of FLOPs associated with the beam selection algorithm (Algorithm B).

Table 5 Complexity and memory comparison of spatial domain AE and BDP + AE

| Approach | Number of selected beams | Number of parameters (x1e5) | Number of FLOPs (x1e6) |
| --- | --- | --- | --- |
| BDP + AE | 1 2 3 4 5 6 | 2.35 | 1.89 |
|  |  | 4.49 | 3.74 |
|  |  | 6.62 | 5.59 |
|  |  | 8.75 | 7.45 |
|  |  | 10.88 | 9.3 |
|  |  | 13.01 | 11.15 |
| Spatial-domain AE | N/A | 68.44 | 46.6 |

Observation 13: AE with beam domain processing has lower complexity and memory requirements compared to the AE with spatial domain processing.

### TSF-based CSI compression simulation results – Case 2

#### CSI compression Case 2 with spatial domain processing

##### Intermediate KPI results

In this section, we present simulation results for the TSF-based CSI compression Case 2 and compare it to the SF-based CSI compression, in terms of intermediate KPIs (specifically the NMSE reconstruction performance.). The simulations assume error free CSI reporting, while the impact of missing UCI is evaluated in Section 3.4.2.1.3.

We first focus on the model architecture using 2 CSI frames, i.e., using 2 time-domain CSI samples.

Figure 4 shows the reconstruction performance (NMSE) of the TSF-based compression (orange curve) as a function of the average CSI report size bits, where the average is calculated over two consecutive CSI reports. The TSF performance is compared to the SF, where the SF CSI compression uses a constant report size of 128 bits; the NMSE performance of the SF-based CSI compression is represented by the blue dotted line.  The results show that as long as the average CSI report size of the TSF exceeds a threshold, the TSF outperforms the SF in terms of the reconstruction performance (NMSE), while having lower feedback overhead.     Specifically, for the example in Figure 4, the average size of the TSF CSI report can be reduced up to about 85 bits while still outperforming the 128 bit SF in terms of the reconstruction performance.

Figure 4 TSF performance vs SF using 128 latent bits

Observation 14: For the scenarios investigated using 2 TD samples, TSF-based CSI compression can reduce the CSI feedback overhead by up to 33% compared to SF-based CSI compression, with a smaller reconstruction error.

Next, we evaluate the model architecture using 4 CSI frames, i.e., using 4 time-domain CSI samples.

The NMSE performance of the 4-frame TSF architecture described in Section 3.1.3 is shown in Figure 5 below.

Figure 5 NMSE performance of multi-frame TSF compared to SF

Figure 5 shows the reconstruction error for the CSI compression task with different TSF configurations. A rate-distortion styled analysis has been presented where the X-axis represents the average number of bits used per frame for the CSI feedback reporting and the Y-axis represents the NMSE associated with the CSI reconstruction. The blue curve shows the performance of the baseline spatial frequency compression scheme.  The performance of the multi-frame TSF for 2, 3 and 4 CSI frames is represented with the orange, yellow and purple lines, respectively. From the figure it can be observed that as the number of CSI frames used for TSF compression is increased, the TSF performance improves, as the same reconstruction error can be achieved with fewer bits. Furthermore, as the number of CSI frames used for TSF is increased, the gain in performance eventually starts to saturate, thus minimal performance improvement is observed going from 3 frame TSF to 4 frame TSF.

Observation 15: For the scenarios investigated for multi-frame TSF architectures, increasing the number of frames for TSF compression improves the performance, as the same reconstruction error is achieved with a lower average number of CSI feedback bits.

Observation 16: The gain of the multi-frame TSF performance appears to saturate as the number of TD CSI samples increases above a threshold. For example, for the scenarios investigated, the performance starts saturating for 4 frame TSF architectures.

Based on the intermediate KPI performance improvement of the multi-frame TSF architecture shown in this section, it would be beneficial to further evaluate the TSF compression performance for multiple TD CSI samples (multiple observation window lengths).

Proposal 4: TSF compression performance should be evaluated under multiple observation window lengths.

##### System level results

The simulation parameters following the agreed evaluation assumption in [2] are summarized in Table 3 in the Appendix.

To evaluate the TSF-based CSI compression performance based on system-level results, two baselines are adopted, namely the Rel-16 Type II CSI codebook and the SF-based CSI compression.

The SLS evaluation results presented in this section correspond to the TSF architecture using 2 CSI frames.

In each case, we selected the configurations to result in the same overhead, as follows. For the SF model, the latent dimension is fixed to 40 latents and 2 bits/latent is used for quantization. This results in an overhead of 80 bits. For TSF model, the observation window length is set to two, with the first sample compressed using an SF model with 64 latents while the second sample is compressed using a TSF model with 16 latents, and 2 bits/latent is used for both samples. This results in average overhead of 80 bits similar to the standalone SF compression. For both SF and TSF models, the full CSI is used as the model input. For Rel-16 Type II, para-comb1 configuration is used. The rank-1 transmission percentage is roughly 0.6, which results in an average overhead of 80 bits for Rel-16 Type II.

We evaluated the performance of TSF based compression (i.e., Case 2) for FTP traffic with low, medium and high RU. The simulation results are presented in Figure 6 and Figure 7, respectively.

Figure 6 shows the relative mean throughput gain of TSF-based compression over Rel-16 Type II and SF-based compression, under FTP traffic with low, medium and high RU. It can be seen that TSF-based compression provides marginal mean throughput gain, roughly 1-2%, over SF compression, and considerable mean throughput gain over Rel-16 Type II.

Figure 6 Mean throughput gain of TSF compression over SF compression and Rel-16 Type II under FTP traffic with low, medium and high RU

Observation 17: Under FTP traffic with low, medium and high RU, TSF based compression achieves minor mean throughput gain over SF-based compression, and considerable mean throughput gain over Rel-16 Type II, at the same overhead of 80 bits. Specifically, for the 2 CSI frame scenario investigated, TSF achieves the following throughput gains,

- 1-2% mean throughput gain over SF compression under low, medium and high RU traffic

- 12-16% cell-edge throughput gain over Rel-16 Type II under low, medium and high RU traffic

Figure 7 shows the cell-edge throughput gain of TSF-based compression over SF compression and Rel-16 Type II, under FTP traffic with low, medium and high RU. It can be seen that TSF compression becomes more effective for cell-edge users, where TSF achieves cell-edge gain between 12% and 17% over SF compression under FTP traffic. In addition, compared to Rel-16 Type II, TSF achieves cell-edge gain around 25% for medium and high RU, and 45% for low RU.

Figure 7 Cell-edge throughput gain of TSF compression over SF compression and Rel-16 Type II under FTP traffic with low, medium and high RU

Observation 18: Under FTP traffic with low, medium and high RU, TSF based compression outperforms SF compression and Rel-16 Type II in terms of cell-edge throughput, at the same overhead of 80 bits. Specifically, for the 2 CSI frame scenario investigated, TSF achieves the following cell-edge throughput gains,

- 12-17% cell-edge throughput gain over SF compression under low, medium and high RU traffic

- 25-45% cell-edge throughput gain over Rel-16 Type II under low, medium and high RU traffic

##### Impact of missing UCI and mitigation of missing UCI

The SLS evaluation results presented in this section correspond to the TSF CSI compression Case 2 architecture using 4 CSI frames.

During RAN1 #118 [13] it was agreed to study the impact of non-ideal UCI feedback on the performance of CSI compression Case 2, as shown below.

Agreement For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for Case 2, study the performance impact resulting from non-ideal UCI feedback. - Scenario A: no UCI loss - Note: Corresponds to an upper bound or re-aligning missing historical CSI information - Scenario B: UCI loss, known at NW and unknown at UE, with mitigation at NW - Note: Corresponds to implementation-based mitigation at NW but no signaling to UE. - Scenario C: UCI loss, known at NW and UE, with mitigation at NW and UE - Note: Corresponds to reset of historical CSI information at both UE and NW or any other mitigation approach enabled by signaling. - UCI loss modeling - 10% UCI loss probability on all UCI reports - Other options are not precluded, e.g., No UCI loss for the first UCI report of each observation window, and 10% UCI loss probability for the subsequent reports of each observation window, as a shortcut to simplify the evaluation work. - Other values for UCI loss probability are not precluded. - Note: The same UCI loss modeling shall be applied to the benchmark for fair comparison. - FFS: partial PMI-related UCI loss

In this section, we evaluate the impact of missing UCI on the TSF CSI compression Case 2 performance for scenarios A, B and C defined above, where we used the TSF model architecture described in Section 3.1.3 with 4 CSI frames.

Table 6 shows the mean throughput gain of AI-SF compression without missing UCI, TSF compression Case2 without missing UCI (Scenario A), TSF compression Case2 with 10% missing UCI (Scenario B), and TSF compression Case 2 with 10% missing UCI and mitigation at both NW-side and UE-side using full buffer reset (Scenario C), relative to Rel-16 codebook with missing UCI. It can be seen that AI-TSF with missing UCI results in performance loss compared with AI-TSF without missing UCI and it also becomes worse than AI-SF. On the other hand, AI-TSF with missing UCI and mitigation, using full buffer reset, approaches the performance of AI-TSF without missing UCI and it outperforms AI-SF compression without missing UCI.

Table 6 Mean throughput gain over the Rel-16 Type II benchmark for FTP traffic with low, medium and high RU, and payload sized of 80 bits

| Methods (Mean Throughput Gain over Rel-16 with missing UCI) / RU | Low | Med | High |
| --- | --- | --- | --- |
| AI-SF (Case 0) | 11.9% | 17.1% | 12.1% |
| Scenario A: AI-TSF, w/o missing UCI | 12.4% | 18.8% | 13.2% |
| Scenario B: AI-TSF, w/ missing UCI (10% rate) | 9.6% | 12.5% | 6.8% |
| Scenario C: AI-TSF, w/ missing UCI (10% rate) and mitigation at NW and UE | 12% | 17.9% | 12.2% |

Observation 19: Under FTP traffic with low, medium and high RU, AI-TSF CSI Compression Case 2 with 10% missing UCI results in performance loss in terms of mean throughput relative to AI-TSF Compression Case 2 and AI-SF compression Case 0 without missing UCI.

Observation 20: For CSI Compression Case 2, mitigation of missing UCI by using full buffer reset at both NW-side and UE-side approaches the performance of CSI Compression Case 2 without missing UCI, and outperforms CSI Compression Case 0 (AI-SF) without missing UCI.

Table 6 shows the NMSE and SGCS for both layers of AI-SF Case 0 compression without missing UCI, AI-TSF compression Case2 without missing UCI (Scenario A), AI-TSF compression Case2 with 10% missing UCI (Scenario B), and AI-TSF compression Case 2 with 10% missing UCI and mitigation using full buffer reset (Scenario C). It can be seen that AI-TSF Compression Case 2 with missing UCI and mitigation using full-buffer reset on both NW- and UE-side approaches the performance of AI-TSF Case 2 without missing UCI and outperforms the AI-SF Case 0 in terms of NMSE and SGCS.

Table 7 Intermediate KPI results for AI-TSF compression Case 2 with missing UCI

| Methods / Intermediate KPI | NMSE | SGCS L1 | SGCS L2 |
| --- | --- | --- | --- |
| AI-SF (Case 0) | 0.3 | 0.54 | 0.466 |
| Scenario A: AI-TSF, w/o missing UCI | 0.22 | 0.663 | 0.585 |
| Scenario B: AI-TSF, w/ missing UCI (10% rate) | 0.258 | 0.647 | 0.568 |
| Scenario C: AI-TSF, w/ missing UCI (10% rate) and mitigation at NW and UE | 0.235 | 0.655 | 0.577 |

Observation 21: In terms of NMSE and SGCS, AI-TSF Case 2 with 10% missing UCI (Scenario B) results in performance loss relative to AI-TSF Case 2 without missing UCI (Scenario A) while AI-TSF Case 2 with missing UCI and mitigation using a full buffer reset (Scenario C) achieves close performance to AI-TSF without missing UCI (Scenario A).

Proposal 5: For further performance improvement of AI-TSF compression Case2 with missing UCI, study different missing UCI mitigation solutions e.g., partial buffer reset, or retransmission of the missed UCI.

#### CSI compression Case 2 with beam domain processing

Based on the BDP results provided in Section 3.4.1, it was observed that BDP is more beneficial in scenarios where the number of selected beams is small, i.e., to reduce the associated overhead resulting from signalling the beam indices and also to reduce the beam domain AE complexity.. In RAN1 meeting #119, it was agreed to study CSI compression case 2 with beam domain processing. One way to reduce the overhead associated with the beam domain approach is to leverage the channel correlation properties in the temporal domain. The beams selected by BDP are expected to change less frequently over time resulting in the same set of beams being selected across multiple CSI reporting instances. This will reduce the overhead associated with signaling the selected beams since they will need to get reported every N CSI reports, where N depends on some temporal channel parameters, e.g., UE speed. In addition, the relatively slow variation of the selected beams over time provides margin to relax the constraint of selecting a small number of beams and also gives more room to increase the payload size of the AE since the BDP overhead can be significantly reduced. Overall, this results in a more efficient beam domain framework and increases the applicability of the beam domain AE..

Figure 8 shows the relation between the average validity time of the beams (measured in number of CSI reporting instances) and the UE speed. It can be seen that the higher the UE speed, the beam changes are more frequent (the number of consecutive CSI reporting instances will be without beam change is lower). It may be argued that for moderate UE speeds, e.g., 10, 30, km/hr, the feedback associated with the BDP can be neglected when compared to the beam domain AE overhead. For instance, the results in Figure 8 show that for the realistic mobility model with spatial consistency with a speed of 30 km/hr, the average number of consecutive CSI reporting instances with the same selected beam is 6; for a 128 beams codebook where each beam incurs an overhead of 7 bits, using the same beam over 6 consecutive CSI reporting instances results in an average overhead of roughly 1 bit/beam.

Observation 22: For low to moderate UE speeds, the overhead for reporting the selected beams (BDP overhead) is low, even under complex mobility models.

Proposal 6: For CSI compression Case 2 with beam domain processing, study long-term reporting of the selected beams associated with the BDP, e.g., as a function of the UE speed.

Figure 8 The average validity time (number of CSI reporting instances) of the beams versus the UE speed for two channel models; the Doppler model and the mobility/spatial consistency model

Figure 9 shows the reconstruction performance of the TSF (BDP and AE) approach assuming long term reporting of the beams against the SF (BDP+AE) with beam reporting every CSI feedback reporting instance and the spatial domain AE approach. It can be seen that the TSF (BDP + AE) provides reduction in the overhead at the same reconstruction error compared to the other two baselines.

Observation 23: CSI compression Case 2 with beam domain processing achieves better performance, e.g., overhead reduction, relative to Case 0 with spatial domain processing and beam domain processing.

Figure 9 Reconstruction performance of the TSF BDP approach (green line) versus the BDP SF approach (red line) and the standalone AE approach with spatial domain compression (black line)

### TSF-based CSI compression simulation results – Case 3

The performance of AI/ML based CSI prediction and AI/ML based CSI compression is evaluated for Case 3 (the NW doesn’t use past CSI information) for a UE speed of 30 km/h.  Figure 10 shows the SGCS performance for AI/ML based CSI prediction and AI/ML based CSI compression (black dashed line), relative to the non-AI/ML prediction (Kalman) with Rel-18 CB (green dashed line) and AI/ML based CSI prediction with Rel-18 CB (blue line), for N4=1.

Figure 10 SGCS performance of Case 3, for 30 km/h UE speed and N4=1

From Figure 10 it can be seen that Case 3 shows only a small SGCS improvement over the AI/ML based CSI prediction with Rel-18 CB, and a small SGCS degradation as compared to the non-AI/ML prediction (Kalman) with Rel-18 CB.

Observation 24: TSF Case 3 shows only a small SGCS improvement over the AI/ML based CSI prediction with Rel-18 CB, and a small SGCS degradation as compared to the non-AI/ML prediction (Kalman) with Rel-18 CB.

### Evaluation results for inter-vendor collaboration Option 3a-1

In RAN1 #118b [15] it was agreed to further study Options 3a-1 and 4-1 of inter-vendor training collaboration. For evaluation purposes, the following agreements were made ([15]).

Agreement For the evaluation studies of Direction A, - For encoder parameter exchange from NW-side to UE-side of 3a-1, - Alt. 1: UE-side first trains a nominal decoder - Step 1: UE-side develops a nominal decoder against the exchanged encoder. - Step 2: UE-side develops actual encoder against the nominal decoder - Alt. 2: UE-side directly develops and trains the actual encoder. - Note: Target CSI is available at UE side - For dataset exchange from NW-side to UE-side {target CSI, CSI feedback} of 4-1, - Alt. 1: UE-side first trains a nominal decoder - Step 1: UE-side develops a nominal decoder using the exchanged dataset. - Step 2: UE-side develops an actual encoder against the nominal decoder. - Alt. 2: UE-side directly develops and trains the actual encoder. Agreement For discussion on performance degradation due to UE-side / NW-side data distribution mismatch with respect to UE side additional condition (issue 4 and 6), consider - NW-side data (dataset A) and UE-side data (dataset B) are mismatched in terms of UE-side additional condition (e.g., SVD phase normalization method, UE antenna virtualization, antenna imbalance, antenna spacing / layout, etc) - Note: dataset A may or may not contain dataset B - Note: if dataset A includes dataset B, this means NW timely collect data from the UE - Note: Dataset A and dataset B are aligned in terms of NW-side additional conditions. - Case 1: An encoder-decoder pair is trained on dataset B. This serves as an upper-bound. - Case 2 (for direction A): - For direction A: NW-side trains a decoder on dataset A, and UE-side trains an encoder based on either dataset A, dataset B, or both (where the choice depends on the inter-vendor collaboration Options (4-1 or 3a-1), the training alternatives (Alt 1 or Alt 2) within the Options, and the choice of target CSI for training) - Case 3 (for direction B): NW-side train an encoder-decoder on dataset A and transfers the encoder parameters to UE. - Study issue 4 and 6 by evaluating the inference performance for Case 2 / 3 on test dataset B, comparing to the inference performance for case 1 on test dataset B. - Note: Evaluations are assumed to performed without common reference model structure among companies.

For our evaluation, we generated one dataset (Dataset A) for training the NW-side actual decoder and two datasets to simulate different UE-side additional conditions (Dataset B1 and Dataset B2).

The common parameters used to generate datasets A, B1 and B2 are listed in the Appendix, with specific updates as shown below.

- UMa

- 100% outdoor, UE speed 10 km/h

- The NW-side additional conditions are aligned across Datasets A, B1 and B2.

- Training set size: 362880 samples

- Testing set size: 90720 samples

The configurations used to simulate different UE-side additional conditions are as follows.

- Dataset A: UE antenna setup: 1 polarized AE

- Dataset B1: UE antenna setup: 2 non-polarized AE, spaced at 0.5 λ

- Dataset B2: UE antenna setup: 2 non-polarized AE, spaced at 0.8 λ

Additionally, we generated a mixed dataset A+B1 by combining 50% of samples of Dataset A with 50% samples of Dataset B1 (and similarly, a mixed dataset A+B2).

Direction A – Issue 1

To investigate Issue 1 for Direction A, we trained a reference encoder and the NW-side actual decoder pair using Dataset A where the reference encoder and the NW-side actual decoder use a transformer backbone.  The NW-side exchanged the following information with the UE-side: parameters of the reference encoder, Dataset A (containing the target CSI), model backbone information (in this example, transformers), as well as minimum KPI (SGCS, NMSE) for model training.  In Step 1 of 3a-1, the UE-side first develops a reduced complexity nominal decoder using the exchanged reference encoder and the exchanged dataset (Dataset A), and using the same backbone as indicated by the NW-side.  In Step 2 of 3a-1, off-line engineering at UE-side develops an actual encoder against the nominal decoder, where the actual encoder has lower complexity compared to the reference encoder.

For the considered example, the number of parameters of the models is shown in Table 8 below.

Table 8 Example of Number of parameters for UE-side model complexity reduction

|  | NW-side | UE-side |
| --- | --- | --- |
| Encoder | Reference encoder: 1.46 M | Actual encoder: 0.758 M |
| Decoder | Actual decoder: 1.46 M | Nominal decoder: 1.02 M |

The intermediate KPI performance of the UE-side off-line engineered actual encoder operating with the NW-side actual decoder is presented in Table 9.

Table 9 Intermediate KPI for Option 3a-1

| Side | Test | Training/Testing | Encoder | Decoder | NMSE | SGCS |
| --- | --- | --- | --- | --- | --- | --- |
| NW | Train E2E AE (reference encoder) | Dataset A | Reference encoder | Actual decoder | 0.19551 | 0.81993 |
| UE | Step1: develop nominal decoder | Dataset A | Reference encoder | Nominal decoder | 0.22572 | 0.79268 |
| UE | Step2: develop actual encoder | Dataset A | Actual encoder | Nominal decoder | 0.23631 | 0.78359 |
|  | Inference | Dataset A | Actual encoder | Actual decoder | 0.22648 | 0.79282 |
|  | Inference performance relative to reference encoder/actual decoder |  |  |  | 0.6 dB | -3.3% |

From Table 9 above, it can be seen that for Option 3a-1, the inference using the actual encoder and actual decoder pair exhibits only a small performance degradation compared to the reference encoder-actual decoder pair developed at the NW-side (specifically, 0.6 dB NMSE degradation, and 3.3% SGCS degradation).  This suggests that it is feasible to perform UE-side off-line engineering to develop a UE-side encoder (including a lower complexity encoder model) that is compatible with a proprietary NW-side actual decoder, at a price of modest performance degradation.

Observation 25:  For Option 3a-1, it is feasible to perform UE-side off-line engineering to develop a UE-side encoder (including a lower complexity encoder) that is compatible with a proprietary NW-side actual decoder. There is a small performance impact (up to 0.6 dB NMSE degradation, and 3.3% SGCS degradation) relative to the reference encoder-actual decoder pair developed at the NW-side.

Observation 26:  For Option 3a-1, the additional information that needs to be exchanged from the NW-side to the UE-side to enable UE-side off-line engineering of an actual encoder includes: information on the exchanged dataset A (containing the target CSI), model backbone information (e.g., transformers, CNN, etc.), as well as minimum KPI (SGCS, NMSE).

Direction A – Issue 4

To investigate Issue 4 for Alt 1 of Option 3a-1, we trained a reference encoder and the NW-side actual decoder pair using Dataset A, where the reference encoder and the NW-side actual decoder use a transformer backbone.  In Step 1 of 3a-1, the UE-side first develops a nominal decoder using the exchanged reference encoder and the mixed dataset A+B1. For this step, the UE-side assumes knowledge of the backbone of the nominal decoder, but not of the architecture of the NW-side actual decoder.  In Step 2 of 3a-1, the UE-side develops the actual encoder using the nominal decoder developed in Step 1, and the mixed dataset A+B1.  Lastly, for the Case 1 benchmark (upper bound), the pair reference encoder/actual decoder pair was trained on Dataset B1, and the Case2 inference for the actual UE-side encoder/actual NW-side decoder pair was run on Dataset B1.

The intermediate KPI results are presented in Table 10 for Dataset B1.

Table 10 Intermediate KPI for Option 3a-1, Issue 4 (Dataset B1)

| Side | Test | Training | Testing | Encoder | Decoder | NMSE | SGCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NW | Train E2E AE (reference encoder) | Dataset A | Dataset A | Reference encoder | Actual decoder | 0.19551 | 0.81993 |
| UE | Step1: develop nominal decoder | Dataset A + B1 | Dataset A + B1 | Reference encoder | Nominal decoder | 0.20109 | 0.81475 |
| UE | Step2: develop actual encoder | Dataset A + B1 | Dataset A + B1 | Actual encoder | Nominal decoder | 0.20359 | 0.81263 |
|  | Case 1 (benchmark) | Dataset B1 | Dataset B1 | Reference encoder | Actual decoder | 0.18834 | 0.82765 |
|  | Case 2 Inference | - | Dataset B1 | Actual encoder | Actual decoder | 0.21069 | 0.80607 |
|  | Case 2 relative to Case1 | - | - | - | - | 0.49 dB | -2.6% |

The results for a similar experiment using Dataset B2 are shown in Table 11.

Table 11 Intermediate KPI for Option 3a-1, Issue 4 (Dataset B2)

| Side | Test | Training | Testing | Encoder | Decoder | NMSE | SGCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NW | Train E2E AE (reference encoder) | Dataset A | Dataset A | Reference encoder | Actual decoder | 0.19551 | 0.81993 |
| UE | Step1: develop nominal decoder | Dataset A + B2 | Dataset A + B2 | Reference encoder | Nominal decoder | 0.20317 | 0.81273 |
| UE | Step2: develop actual encoder | Dataset A + B2 | Dataset A + B2 | Actual encoder | Nominal decoder | 0.20550 | 0.81076 |
|  | Case 1 (benchmark) | Dataset B2 | Dataset B2 | Reference encoder | Actual decoder | 0.18964 | 0.82661 |
|  | Case 2 Inference | - | Dataset B2 | Actual encoder | Actual decoder | 0.21470 | 0.80224 |
|  | Case 2 relative to Case1 | - | - | - | - | 0.54 dB | -2.95% |

The results in Table 10 and Table 11, show a modest degradation in Case 2 relative to Case 1, specifically up to 2.95% SGCS degradation and up to 0.54 NMSE degradation. This suggests that for Issue 4, for the considered configuration, the performance degradation due to UE-side / NW-side data distribution mismatch with respect to UE side additional condition is small.

Observation 27:  For Issue 4, for the considered configuration, there is a small performance degradation due to UE-side / NW-side data distribution mismatch with respect to UE side additional conditions, specifically of up to 2.95% SGCS degradation and up to 0.54 dB NMSE degradation.

# Conclusion

In this contribution, we discussed aspects of CSI compression performance, complexity, pre-processing, as well as two-sided model monitoring. We also provided simulation results for beam domain CSI compression and TSF-based CSI compression.

We provide the following observations and proposals.

Observation 1: Beam domain processing results in low-dimensional channel samples that may be compressed using low complexity Autoencoder models, resulting in reduction in both memory and computational requirements relative to spatial domain Autoencoders.

Observation 2: Beam domain processing exhibits good generalization capabilities as a single beam domain AE model can deal with multiple antenna configurations, potentially reducing the need for model switching.

Observation 3: While TSF compression may potentially result in further improvement in the compression performance relative to SF compression, it may require more complex AE models with higher computational and memory requirements relative to SF.

Observation 4: Specification impacts of mitigation of missing UCI for CSI Compression Case 2 include signaling and timing information to maintain synchronization of the historical CSI information between the UE-side and NW-side.

Observation 5:	For UE-side monitoring, both time-based trigger and event-based trigger needs to be studied for reporting the monitoring metrics.

Observation 6:	For UE-side monitoring, appropriate monitoring metrics need to be identified (e.g., other than SGCS) to avoid unnecessary model updating or switching considering that SGCS couldn’t reflect AI/ML model performance in a reasonable manner.

Observation 7:	Out-of-distribution metrics for UE-side monitoring can be another candidate as monitoring metric but further study is necessary.

Observation 8: 	For UE-side monitoring, monitoring metrics in terms of report size, metrics quantization, and report frequency also needs to be studied to investigate feedback overhead.

Observation 9: 	UE-side monitoring based on the output of the NW-side CSI reconstruction model may increase the downlink signaling overhead, because the output CSI reconstructed at the NW needs to be indicated by the NW to the UE.

Observation 10: Precoded RS (CSI-RS, DM-RS) can be used for UE-side model monitoring, by comparing the expected precoding gain to actual precoding gain and determining the mismatch between actual CSI at UE and reconstructed CSI at NW.

Observation 11:  NW-side monitoring with lower signaling overhead options can be studied further in Rel-19 as alternative to the UE-side monitoring.

Observation 12: AE with beam domain processing outperforms the AE with spatial domain processing in terms of intermediate KPI.

Observation 13: AE with beam domain processing has lower complexity and memory requirements compared to the AE with spatial domain processing.

Observation 14: For the scenarios investigated using 2 TD samples, TSF-based CSI compression can reduce the CSI feedback overhead by up to 33% compared to SF-based CSI compression, with a smaller reconstruction error.

Observation 15: For the scenarios investigated for multi-frame TSF architectures, increasing the number of frames for TSF compression improves the performance, as the same reconstruction error is achieved with a lower average number of CSI feedback bits.

Observation 16: The gain of the multi-frame TSF performance appears to saturate as the number of TD CSI samples increases above a threshold. For example, for the scenarios investigated, the performance starts saturating for 4 frame TSF architectures.

Observation 17: Under FTP traffic with low, medium and high RU, TSF based compression achieves minor mean throughput gain over SF-based compression, and considerable mean throughput gain over Rel-16 Type II, at the same overhead of 80 bits. Specifically, for the 2 CSI frame scenario investigated, TSF achieves the following throughput gains,

- 1-2% mean throughput gain over SF compression under low, medium and high RU traffic

- 12-16% cell-edge throughput gain over Rel-16 Type II under low, medium and high RU traffic

Observation 18: Under FTP traffic with low, medium and high RU, TSF based compression outperforms SF compression and Rel-16 Type II in terms of cell-edge throughput, at the same overhead of 80 bits. Specifically, for the 2 CSI frame scenario investigated, TSF achieves the following cell-edge throughput gains,

- 12-17% cell-edge throughput gain over SF compression under low, medium and high RU traffic

- 25-45% cell-edge throughput gain over Rel-16 Type II under low, medium and high RU traffic

Observation 19: Under FTP traffic with low, medium and high RU, AI-TSF CSI Compression Case 2 with 10% missing UCI results in performance loss in terms of mean throughput relative to AI-TSF Compression Case 2 and AI-SF compression Case 0 without missing UCI.

Observation 20: For CSI Compression Case 2, mitigation of missing UCI by using full buffer reset at both NW-side and UE-side approaches the performance of CSI Compression Case 2 without missing UCI, and outperforms CSI Compression Case 0 (AI-SF) without missing UCI.

Observation 21: In terms of NMSE and SGCS, AI-TSF Case 2 with 10% missing UCI (Scenario B) results in performance loss relative to AI-TSF Case 2 without missing UCI (Scenario A) while AI-TSF Case 2 with missing UCI and mitigation using a full buffer reset (Scenario C) achieves close performance to AI-TSF without missing UCI (Scenario A).

Observation 22: For low to moderate UE speeds, the overhead for reporting the selected beams (BDP overhead) is low, even under complex mobility models.

Observation 23: CSI compression Case 2 with beam domain processing achieves better performance, e.g., overhead reduction, relative to Case 0 with spatial domain processing and beam domain processing.

Observation 24: TSF Case 3 shows only a small SGCS improvement over the AI/ML based CSI prediction with Rel-18 CB, and a small SGCS degradation as compared to the non-AI/ML prediction (Kalman) with Rel-18 CB.

Observation 25:  For Option 3a-1, it is feasible to perform UE-side off-line engineering to develop a UE-side encoder (including a lower complexity encoder) that is compatible with a proprietary NW-side actual decoder. There is a small performance impact (up to 0.6 dB NMSE degradation, and 3.3% SGCS degradation) relative to the reference encoder-actual decoder pair developed at the NW-side.

Observation 26:  For Option 3a-1, the additional information that needs to be exchanged from the NW-side to the UE-side to enable UE-side off-line engineering of an actual encoder includes: information on the exchanged dataset A (containing the target CSI), model backbone information (e.g., transformers, CNN, etc.), as well as minimum KPI (SGCS, NMSE).

Observation 27:  For Issue 4, for the considered configuration, there is a small performance degradation due to UE-side / NW-side data distribution mismatch with respect to UE side additional conditions, specifically of up to 2.95% SGCS degradation and up to 0.54 dB NMSE degradation.

Proposal 1: Study the specification impact of adapting the length of the historical CSI buffer to channel conditions, such as UE speed.

Proposal 2:	Study further the following aspects for model monitoring in Rel-19:

- Details of reporting mechanism for the monitoring metrics with both time/event-trigger based

- Appropriate UE-side monitoring metric which reflects AI/ML model performance accurately

- UE-side monitoring based on precoded RS (CSI-RS, DM-RS)

- Reporting contents/structure of UE-side monitoring metric and its associated feedback overhead

- NW-side monitoring with lower signaling overhead

Proposal 3: Mechanisms for identifying the cause of performance degradation include:

- UE assistance information such as UE-side model monitoring metrics and UE-side out-of-distribution metrics

- Reporting the UE-assessed error cause (e.g., data drift, UE-side, or undetermined)

- Mitigation mechanisms, including fallback to legacy CSI reporting, model switching

Proposal 4: TSF compression performance should be evaluated under multiple observation window lengths.

Proposal 5: For further performance improvement of AI-TSF compression Case2 with missing UCI, study different missing UCI mitigation solutions e.g., partial buffer reset, or retransmission of the missed UCI.

Proposal 6: For CSI compression Case 2 with beam domain processing, study long-term reporting of the selected beams associated with the BDP, e.g., as a function of the UE speed.

# References

RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description.

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description.

3GPP TR38.843 V18.0.0 (2023-12) Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)

RAN1 #116 Chairman’s note

C. Wen, W. Shih and S. Jin, "Deep Learning for Massive MIMO CSI Feedback," in IEEE Wireless Communications Letters, vol. 7, no. 5, pp. 748-751, Oct. 2018, doi: 10.1109/LWC.2018.2818160.

W. Liu, W. Tian, H. Xiao, S. Jin, X. Liu and J. Shen, "EVCsiNet: Eigenvector-Based CSI Feedback Under 3GPP Link-Level Channels," in IEEE Wireless Communications Letters, vol. 10, no. 12, pp. 2688-2692, Dec. 2021, doi: 10.1109/LWC.2021.3112747.

A. Vaswani, N. Shazeer, N. Parmar, J. Uszkoreit, L. Jones, A. N. Gomez, Ł. Kaiser, and I. Polosukhin. "Attention is all you need." Advances in neural information processing systems 30 (2017).

R1-2208728, “Discussions on AI-CSI”, Ericsson

R1-2301156, “Evaluation on AI/ML for CSI feedback enhancement”, InterDigital

RAN1 #116b Chairman’s note

RAN1 #117 Chairman’s note

RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description.

RAN1 #118 Chairman’s note

R1-2407499, “Summary of Evaluation Results for Additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm)

RAN1 #118b Chairman’s note

RP-243245, “New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Work Item Description.

RAN1 #119 Chairman’s note

# Appendix

The common parameters used for the considered suite of simulations are based on the assumptions agreed in [3] and is shown below for convenience.

Table 12 Common parameters used in all Scenarios/Configurations

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Uma, Umi |
| Frequency Range |  | FR1 only, 2GHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 16 ports ; configuration specific |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15 kHz |
| Simulation bandwidth |  | 10MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | MU-MIMO with rank adaptation |
| Number of users |  | 6 UE per BS |
| Max number of MU layers |  | 12 |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Traffic model |  | Full buffer, Non full buffer (FTP Model 1), packet size 0.5 Mbytes |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | 4ms delay, ideal |
| Channel estimation |  | ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. |
| Baseline for performance evaluation |  | Rel-16 Type II CSI |