3GPP TSG RAN WG1 #120		R1-2500545

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.1.1

Source:	Google

Title:	ML based Beam Management

Document for:	Discussion/Decision

# Introduction

In RAN1 #119 meeting, the following on ML based beam management has been agreed.

Agreement - For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A. Conclusion For BM-Case 2 of UE-side model, only fixed Set B across different time instance is supported for single CSI report. Agreement For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration, - Two CSI-ResourceConfigId s are configured for Set A and Set B separately Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion Agreement Send LS to RAN2 with below information. RAN1 thanks RAN2 for the LS on applicable functionality reporting for beam management UE-sided model. In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs - The concept/terminology “ functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same. RAN 1 made the following agreements related to the Questions from RAN 2: Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion. RAN1 would like to provide replies on the following questions from RAN2 in R2-2407848: Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others? Answer to Q1: In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-enabled Features/FGs (including components and corresponding value ranges). These UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI. Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID? Answer to Q2: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list. Q3: Is NW-side additional condition functionality specific? Answer to Q3: Please also refer to the answer to Q2 to understand the ongoing discussion about the associated ID for NW-side additional condition. And please refer to the agreements related to the Questions from RAN 2. Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5): Answer to Q4: And please refer to the agreements related to the Questions from RAN 2. Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition? Answer to Q4-1: There is no consensus yet on whether it is mandatory or optional. There is no conclusion yet on whether it is feasible or not for UE to decide the applicability without NW-side additional condition, and RAN 1 is discussing the related issues. Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional condition in Step 3 for UE to determine applicable functionalities? Answer to Q4-2: Please refer to the agreements related to the Questions from RAN 2. Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc? Answer to Q4-3: Please refer to the agreements related to the Questions from RAN 2. Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities? Answer to Q4-4: Please refer to the agreements related to the Questions from RAN 2. Q5: What is the content of applicable functionality reporting in Step 4? Answer to Q5: Please refer to the agreements related to the Questions from RAN 2. Q6: What is the content of inference configuration in Step 5? Answer to Q6: Please refer to the agreements related to the Questions from RAN 2. The content of inference configuration as CSI-ReportConfig is to be designed later in RAN1. Q7: If inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3? Answer to Q7: Please refer to the agreements related to the Questions from RAN 2. Q8: If inference configuration is not provided in Step 3, does configuration in Step 5 activate the functionality immediately upon receiving Step 5? Answer to Q8: Please refer to the agreements/conclusion related to the Questions from RAN 2. Q9: If more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated? Answer to Q9: Please refer to the agreements related to the Questions from RAN 2. Q10: Is L1/L2 signaling for functionality activation/deactivation needed? Answer to Q10: Please refer to the agreements related to the Questions from RAN 2. With that, RAN1 understands that L1 and MAC signalling can be used for aperiodic CSI Report and semi-persistent CSI report.

In this contribution, we provide some discussion on AI/ML based BM.

# Beam measurement

In RAN1 #118b meeting, the following on BM RS for set B has been agreed.

Agreement For UE-side AI/ML model, for BM-Case1, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR: - Periodic (P) CSI-RS - Semi-persistent (SP) CSI-RS - Aperiodic (AP) CSI-RS For UE-side AI/ML model, for BM-Case 2, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR: - Periodic (P) CSI-RS - Semi-persistent (SP) CSI-RS - FFS: Aperiodic (AP) CSI-RS Note: above CSI-RS resource refers to that used for beam management.

The open issue is whether to support AP-CSI-RS for BM for Set B measurement for BM-Case 2. Compared to P/SP CSI-RS for BM, AP-CSI-RS is widely used in current deployment. Further, AP-CSI-RS based CSI prediction has already been supported in Rel-18 MIMO. Therefore, there is no problem to support AP-CSI-RS as Set B RS for BM-Case 2. In addition, current agreement discusses CSI-RS only. Compared to CSI-RS, SSB is used more widely in current field. Therefore, configuring SSB as the Set B RS should be supported. In addition, Rel-19 NES introduces on-demand SSB, which can be configured for L1 and L3 measurement. Since the beam prediction is based on L1-RSRP, the on-demand SSB should also be supported for BM Case 1 and Case 2.

Proposal 1: Support to configure AP-CSI-RS for BM as RS for Set B for BM-Case 2.

Proposal 2: Support to configure SSB as RS for Set B for both BM-Case 1 and BM-Case 2.

- Support both legacy SSB and on-demand SSB introduced in Rel-19 NES

# Beam report

## Beam report for NW-side model

In RAN1 #117, the following on beam report for NW model has been agreed.

Agreement At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support - Support differential L1-RSRP reporting with legacy quantization step and range - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information - Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

One open issue is the measurement error impact on the NW model. The UE is unable to measure all the beams accurately, since the coverage for different beams can be quite different. Therefore, it is necessary to introduce a confidence level indicator for the UE to report the maximum L1-RSRP measurement error for each reported beam.

Proposal 3: Support to report a confidence level indicator for L1-RSRP report to indicate the maximum L1-RSRP measurement error for each beam.

Another open issue is whether to consider some constraints for the selected beams for a beam report for NW side model. Different NW side model may require different model inputs. Then such kind of beam subset can be useful for the UE to report the L1-RSRP for the essential beams. Therefore, it is necessary to introduce a beam subset configuration, which indicates a set of beams that the UE should always report in an L1-RSRP report. However, as discussed before, UE may not be able to measure all the beams for the model input accurately. Then for such beams, one L1-RSRP state should be defined to indicate that the reported beam is in an invalid state.

Proposal 4: Support to introduce a beam subset configuration indicating a subset of beams that the UE should always report in an L1-RSRP report

- Support to define one L1-RSRP state to indicate the reported beam is in an invalid state (UE is unable to measure the L1-RSRP that can meet the L1-RSRP measurement requirement)

Further, for BM case-2, the NW requires the beam report for historical status to predict the beam in future. However, as shown in Figure 1, the NW may fail to receive some beam report instances, which could cause performance degradation for NW-side beam prediction. Therefore, some mechanisms to support the L1-RSRP retransmission should be supported. Then if the NW fails to receive some bean report instances, it can schedule the UE to retransmit such beam report.

Figure 1: Impact of L1-RSRP report lost on NW-side beam prediction

Proposal 5: Support L1-RSRP report retransmission to facilitate the NW-side beam prediction for BM case 2

## Beam report for inference results for UE-side model

For UE side model inference, in RAN1 #116 and RAN1 #118, the following has been agreed.

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction. Agreement For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances - FFS details Agreement For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A

One open issue is the definition of the predicted top K beams. Similar to legacy beam report, the predicted K beams do not have to be the beams with the highest probability to become the best beam. The UE may report the top K beams based on the probability and the spatial correlation with regard to the diversity as shown in Figure 2. Therefore, similar to the legacy beam report, which beams should be reported is up to UE implementations. The spec should only define the number of reported beams.

Figure 2: Top-K beam selection based on the probability and spatial correlation for the beams

The second issue is the definition of the RSRP. The RSRP should be defined as a predicted RSRP based on a reference transmission power. It should be noted that the transmission power for the DL RS for the beam measurement could be different. Therefore, a reference transmission power should be considered. One possible way is to configure the SSB transmission power as the reference transmission power.

The third issue is whether to support option 3 and option 4 additionally. The probability information and the confidence information are useful for the network to determine whether to perform TCI switching based on the beam prediction result or not. Therefore, option 3 and option 4 should be supported.

Another open issue is how to calculate the different RSRP for BM case 2. One simple way is to calculate the differential RSRP based on the absolute RSRP, which is similar to current L1-RSRP report.

Proposal 6: For beam report based on UE model inference for SD beam prediction, support the followings on the remaining open issues:

- The selection of the “top-K” beams are up to UE implementation

  - Spec only defines the number of reported beams

- The reported RSRP should be defined as the predicted RSRP based on a reference transmission power

- Support both option 3 (probability information report) and option 4 (confidence information report)

- For BM case 2, the differential RSRP is calculated based on the reported absolute RSRP

In addition, the following related to the CPU rule is agreed in RAN1 #118b.

Agreement For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing. - FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities. - FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2

UE may perform inference based on dedicated hardware/resource. Therefore, a different type of CPU should be considered for inference. Thus, one inference report should take 1 enhanced CPU (eCPU). Further, the inference also requires UE to perform some L1-RSRP measurement. Therefore, the inference report should also take 1 legacy CPU. For different use cases, UE may use one model to perform both spatial and temporal beam prediction, or use separate models for the spatial and temporal beam predictions. Then for different use cases, it is necessary to introduce a UE capability to report whether some use cases can share eCPU or not. Further, since performance monitoring also requires measurement and inference, the same number of CPUs for performance monitoring should be required.

Proposal 7: An inference report for BM-Case 1 should take 1 eCPU and 1 legacy CPU

- eCPU is occupied for inference and legacy CPU is occupied for Set B RS measurement

- Support the UE to report whether BM-Case 1 and BM-Case 2 can share common eCPUs or not

- Performance monitoring for BM-Case 1 should also take 1 eCPU and 1 legacy CPU

# Beam indication

## TCI activation/indication for BM case 1

Currently, the QCL rule is defined as Figure 3. The TRS (CSI-RS for tracking) is a central RS for QCL indication. The TRS is a periodic RS. Currently, the NW can only use the RRC to update the beam. Usually, the NW configures N TRSs for a cell with N SSBs. The NW provides common configuration for the N TRSs for all the UEs for overhead reduction. For BM case 1, for overhead reduction, the number of SSBs can be smaller than N, which can be, for example, 0.25*N. Then similarly, the TRS overhead reduction should be considered. However, for potential beam switching to a predicted beam, the NW needs to transmit a periodic TRS based on the predicted beam. Therefore, it is better to consider dynamic activation/deactivation for TRS.

Figure 3: Current QCL rule

Proposal 8: Support dynamic activation/deactivation of periodic TRS with regard to TCI activation/indication based on the predicted beam.

In addition, since the beam indication is always based on unknown TCI state. It is necessary to speed up to TCI switching delay. Otherwise, the activated/indicated TCI state may not be the best one at its action time. To speed up the TCI switching delay for unknown TCI state, one possible way is to trigger aperiodic CSI-RS resources that are QCLed with the SSB/CSI-RS configured as the QCL source in the TCI state and the UE can measure the time/frequency offset and Rx beam based on the aperiodic CSI-RS resources. For joint TCI state, UE can also measure the pathloss based on the aperiodic CSI-RS resources.

Proposal 9: Since the activated/indicated TCI based on SD beam prediction is usually an unknown TCI state, to reduce the latency for TCI activation/indication based on SD beam prediction, support the NW to trigger aperiodic CSI-RS resources QCLed with the SSB/CSI-RS configured as the QCL source in the TCI state.

- UE measures time/frequency offset and Rx beam based on the aperiodic CSI-RS resources

- UE can also measure the pathloss based on the aperiodic CSI-RS resources

## TCI activation/indication for BM case 2

For temporal beam prediction, the NW can indicate the action time or action delay for predicted TCI state. The NW can also indicate whether the TCI state is based on beam prediction or not. One possible way is to configure different TCI state pools. One TCI state pool is used for legacy beam indication and the other TCI state pool is used for beam prediction.

Proposal 10: To differentiate the TCI state for legacy beam indication and TCI state for beam prediction, support to configure separate TCI state pools for legacy beam indication and TCI state for beam prediction.

Proposal 11: Support to configure the action delay for the TCI state for beam prediction.

For temporal beam prediction, performance validation is necessary. Current beam from an indicated TCI can be a reference for performance validation. If none of the predicted beam(s) can provide better beam quality than current beam plus an offset, it can be assumed the beam prediction cannot pass the performance validation. Thus, although gNB may provide a TCI based on the predicted beam for beam prediction, in addition to the ACK/NACK for the TCI update signaling, some additional UE feedback before the beam action time can be studied for performance validation for predicted beams.

Figure 4 shows the procedure for UE feedback on performance validation for time domain beam prediction. The UE’s movement/rotation and blockage may cause some abrupt beam change. Then the previously indicated predicted beam could get invalid. Therefore, in addition to the ACK for the TCI update signaling, before the action time for the predicted beam, UE can report some information on performance validation for the predicted beam. If the predicted beam passes the performance validation, the gNB and UE can switch to the predicted beam at the action time; otherwise, gNB can send an updated TCI indication to the UE to overwrite or withdraw the previously indicated predicted beam.

Figure 4: UE feedback on performance validation for time domain beam prediction

Proposal 12: For temporal beam prediction, the beam quality for current beam from an indicated TCI can be used for performance validation, and if none of the predicted beam(s) can provide better beam quality than current beam, the predicted beam(s) are assumed to fall to pass the performance validation.

Proposal 13:  Support UE feedback before the beam action time for performance validation for predicted beam in addition to the ACK/NACK for the TCI update signaling for temporal beam prediction.

# Beam failure recovery

For BFR based on NW side model for BM case 1, after UE declares beam failure, if the UE cannot identify a candidate beam, it can report L1-RSRP for a set of SSBs/CSI-RSs configured by the NW. Then the NW can predict the best beam based on the reported L1-RSRP for the set of SSBs/CSI-RSs. Therefore, if the UE cannot identify a candidate beam, it can trigger a UE initiated beam report to report the L1-RSRP for a set of SSBs/CSI-RSs configured by the NW.

For BFR based on UE side model for BM case 1, after UE declares beam failure, if the UE cannot identify a candidate beam, it can report N beam index(es) based on a beam codebook. UE can predict the best N beam index(es) based on the measurement of a set of SSBs/CSI-RSs. The UE can report the N beam index(es) by a BFRQ.

Proposal 14: For BFR based on NW side model for SD beam prediction, after UE declares beam failure, if the UE cannot identify a candidate beam, it can trigger a UE initiated beam report

- UE reports L1-RSRP for a set of SSBs/CSI-RSs configured by the NW in the UE initiated beam report

- NW performs beam prediction based on the received L1-RSRP

Proposal 15: For BFR based on UE side model for SD beam prediction, after UE declares beam failure, if the UE cannot identify a candidate beam, it can report N predicted beam index(es) based on a beam codebook by the BFRQ.

For BFR based on NW side model for BM case 2, after UE declares beam failure, if the UE cannot identify a candidate beam, it can report L1-RSRP for a set of SSBs/CSI-RSs configured by the NW for multiple measured slots. Then the NW can predict the beam for further slots, and the NW can indicate the beam for future slots by BFRR.

For BFR based on UE side model for BM case 2, after UE declares beam failure, if the UE cannot identify a candidate beam, it can report the predicted beam in one or multiple future slots. The UE can report the beam index(es) for future slot(s) by a BFRQ.

Proposal 16: For BFR based on NW side model for temporal beam prediction, after UE declares beam failure, if the UE cannot identify a candidate beam, it can trigger a UE initiated beam report

- UE reports L1-RSRP for a set of SSBs/CSI-RSs configured by the NW for multiple measured slots in the UE initiated beam report

- NW performs beam prediction based on the received L1-RSRP

Proposal 17: For BFR based on UE side model for SD beam prediction, after UE declares beam failure, if the UE cannot identify a candidate beam, it can report predicted beam index(es) in one or multiple future slots by the BFRQ.

# Set A RS configuration

In RAN1 #118, the following has been agreed for the set A configuration.

Agreement For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A

Thus, the NW may configure a list of reference signals as set A RS. The set A RS can be used for data collection or performance monitoring. But with regard to the overhead of the set A RS, the NW does not need to transmit the set A RS frequently. Therefore, one simple way is to configure the set A RS in aperiodic manner. Then, with regard to the data collection and performance monitoring for multiple UEs, the NW can trigger the aperiodic CSI-RS resources as set A by a group-cast DCI.

Proposal 18: Support to configure aperiodic CSI-RS as set A RS, which is triggered by a group-cast DCI.

Further, for Set A configuration and transmission, with regard to the UE beam refinement, it is better to consider some enhancement for the fast UE beam tracking. For example, the NW configures 1-port CSI-RS as the set A RS and does not multiplex any other DL signal in the same symbol with the 1-port CSI-RS. Then the CSI-RS can be transmitted based on IFDMA structure, which can create multiple repetitions within a symbol. UE can receive different intra-symbol repetition by different beams for fast UE beam tracking.

Proposal 19: For Set A RS, support the NW configures 1-port CSI-RS and UE shall not expect the NW multiplex any other DL RS in the same symbol with the set A RS.

- The NW transmits the set A RS based on IFDMA structure for fast UE beam refinement

# Performance monitoring

For performance monitoring, the following has been agreed in RAN1 #117 and RAN1 #119 meetings.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

For performance monitoring option 2, it is agreed to use the beam prediction accuracy as the performance metric. Then one possible way is that the UE can perform the beam prediction accuracy evaluation periodically, and count the number of incorrect beam predictions within a time window, and then perform the report based on the counter, which is similar to UE initiated beam report and BFR. Such report can be in an event triggered manner. Compared to the Rel-19 UE initiated beam report (SR/PN+UCI), legacy SCell BFR based mechanism (SR+MAC CE) is simpler for such event triggered report.

Proposal 20: For UE-side model monitoring, support the following mechanisms:

- UE performs the beam prediction accuracy evaluation periodically

- UE counts the number of incorrect beam prediction within a time window

- UE transmits a report to the NW to notify the counter is above a threshold

  - Reuse the SCell BFR based mechanism (SR+MAC CE) to transmit the report

Another open issue is the connection between the DL-RS for UE-side monitoring and Set A RS. The DL-RS does not have to be the same as the Set A RS or a subset of Set A, but the NW can transmit such DL-RS based on wide beams and configure the association between the DL-RS and Set A RS as shown in Figure 5 for the UE to determine whether a predicted beam is correct or not.

Figure 5: Beam prediction accuracy determination based on measurement from a set of DL-RS based on wide beam

Proposal 21: Support the network to configure the association between the DL-RS for monitoring and the Set A RS

- If the predicted beam from inference is associated with one of the top K beams for DL-RS for monitoring, UE determines that it is a correct prediction

- Otherwise, UE determines that it is an incorrect prediction

Further, similar to RRM, with regard to UE power saving, the performance monitoring relaxation should be considered for UE in low mobility and high beam quality state. In addition, since the NW can dynamically activate or deactivate an inference configuration. For performance monitoring, it is necessary to support dynamic activate or deactivate the performance monitoring configurations.

Proposal 22: Similar to RRM, support performance monitoring relaxation for UE in low mobility and high beam quality state

- UE does not need to perform performance monitoring when it is in low mobility and high beam quality state

Proposal 23: With regard to dynamic activate/deactivate an inference configuration, support to dynamically activate/deactivate the performance monitoring configuration(s).

# Consistency between training and inference

In RAN1 #118, the associated ID has been agreed regarding the consistency between training and inference.

Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

One open issue is how to configure the associated ID. Since the inference is based on the CSI report configuration, the associated ID can be configured per CSI report configuration. Then with regard to different NW states, e.g., different beam grids as a result of NW energy saving, the NW can configure multiple CSI report configurations based on different associated IDs and activate a subset of CSI report configurations.

Proposal 24: Support to configure the associated ID per CSI report configuration

- Support dynamic activation/deactivation of CSI report configuration

# Event triggered beam prediction report

The event driven beam report is a good complimentary to network triggered beam report. Usually, the UE knows better on when a beam report is necessary or not. Especially with the help of ML based beam prediction, the UE can predict when the beam status may change and the beam report could be necessary. Therefore, it is necessary to study the ML based beam prediction to facilitate the event triggered beam report.

Proposal 25: Support the ML based beam prediction to facilitate the event triggered beam report.

# Inference configuration management procedure

In RAN1 #119, two options for the inference configuration procedure have been agreed, where the option A and option B are shown in Figure 6.

Figure 6: Procedure for option A and option B for inference configuration procedure

The remaining issue could be the details for the inference related parameters for Option B. Since the inference related parameters are a subset of parameters for CSI report configuration, it can be decided later after all the parameters for CSI report configuration for beam prediction are finalized.

Proposal 26: Defer the discussion on Option B based inference configuration procedure until all the parameters for CSI report configuration for beam prediction are finalized.

# Conclusion

In this contribution, we provided discussion on AI/ML based BM. Based on the discussion, the following proposals are provided.

Proposal 1: Support to configure AP-CSI-RS for BM as RS for Set B for BM-Case 2.

Proposal 2: Support to configure SSB as RS for Set B for both BM-Case 1 and BM-Case 2.

- Support both legacy SSB and on-demand SSB introduced in Rel-19 NES

Proposal 3: Support to report a confidence level indicator for L1-RSRP report to indicate the maximum L1-RSRP measurement error for each beam.

Proposal 4: Support to introduce a beam subset configuration indicating a subset of beams that the UE should always report in an L1-RSRP report

- Support to define one L1-RSRP state to indicate the reported beam is in an invalid state (UE is unable to measure the L1-RSRP that can meet the L1-RSRP measurement requirement)

Proposal 5: Support L1-RSRP report retransmission to facilitate the NW-side beam prediction for BM case 2

Proposal 6: For beam report based on UE model inference for SD beam prediction, support the followings on the remaining open issues:

- The selection of the “top-K” beams are up to UE implementation

  - Spec only defines the number of reported beams

- The reported RSRP should be defined as the predicted RSRP based on a reference transmission power

- Support both option 3 (probability information report) and option 4 (confidence information report)

- For BM case 2, the differential RSRP is calculated based on the reported absolute RSRP

Proposal 7: An inference report for BM-Case 1 should take 1 eCPU and 1 legacy CPU

- eCPU is occupied for inference and legacy CPU is occupied for Set B RS measurement

- Support the UE to report whether BM-Case 1 and BM-Case 2 can share common eCPUs or not

- Performance monitoring for BM-Case 1 should also take 1 eCPU and 1 legacy CPU

Proposal 8: Support dynamic activation/deactivation of periodic TRS with regard to TCI activation/indication based on the predicted beam.

Proposal 9: Since the activated/indicated TCI based on SD beam prediction is usually an unknown TCI state, to reduce the latency for TCI activation/indication based on SD beam prediction, support the NW to trigger aperiodic CSI-RS resources QCLed with the SSB/CSI-RS configured as the QCL source in the TCI state.

- UE measures time/frequency offset and Rx beam based on the aperiodic CSI-RS resources

- UE can also measure the pathloss based on the aperiodic CSI-RS resources

Proposal 10: To differentiate the TCI state for legacy beam indication and TCI state for beam prediction, support to configure separate TCI state pools for legacy beam indication and TCI state for beam prediction.

Proposal 11: Support to configure the action delay for the TCI state for beam prediction.

Proposal 12: For temporal beam prediction, the beam quality for current beam from an indicated TCI can be used for performance validation, and if none of the predicted beam(s) can provide better beam quality than current beam, the predicted beam(s) are assumed to fall to pass the performance validation.

Proposal 13:  Support UE feedback before the beam action time for performance validation for predicted beam in addition to the ACK/NACK for the TCI update signaling for temporal beam prediction.

Proposal 14: For BFR based on NW side model for SD beam prediction, after UE declares beam failure, if the UE cannot identify a candidate beam, it can trigger a UE initiated beam report

- UE reports L1-RSRP for a set of SSBs/CSI-RSs configured by the NW in the UE initiated beam report

- NW performs beam prediction based on the received L1-RSRP

Proposal 15: For BFR based on UE side model for SD beam prediction, after UE declares beam failure, if the UE cannot identify a candidate beam, it can report N predicted beam index(es) based on a beam codebook by the BFRQ.

Proposal 16: For BFR based on NW side model for temporal beam prediction, after UE declares beam failure, if the UE cannot identify a candidate beam, it can trigger a UE initiated beam report

- UE reports L1-RSRP for a set of SSBs/CSI-RSs configured by the NW for multiple measured slots in the UE initiated beam report

- NW performs beam prediction based on the received L1-RSRP

Proposal 17: For BFR based on UE side model for SD beam prediction, after UE declares beam failure, if the UE cannot identify a candidate beam, it can report predicted beam index(es) in one or multiple future slots by the BFRQ.

Proposal 18: Support to configure aperiodic CSI-RS as set A RS, which is triggered by a group-cast DCI.

Proposal 19: For Set A RS, support the NW configures 1-port CSI-RS and UE shall not expect the NW multiplex any other DL RS in the same symbol with the set A RS.

- The NW transmits the set A RS based on IFDMA structure for fast UE beam refinement

Proposal 20: For UE-side model monitoring, support the following mechanisms:

- UE performs the beam prediction accuracy evaluation periodically

- UE counts the number of incorrect beam prediction within a time window

- UE transmits a report to the NW to notify the counter is above a threshold

  - Reuse the SCell BFR based mechanism (SR+MAC CE) to transmit the report

Proposal 21: Support the network to configure the association between the DL-RS for monitoring and the Set A RS

- If the predicted beam from inference is associated with one of the top K beams for DL-RS for monitoring, UE determines that it is a correct prediction

- Otherwise, UE determines that it is an incorrect prediction

Proposal 22: Similar to RRM, support performance monitoring relaxation for UE in low mobility and high beam quality state

- UE does not need to perform performance monitoring when it is in low mobility and high beam quality state

Proposal 23: With regard to dynamic activate/deactivate an inference configuration, support to dynamically activate/deactivate the performance monitoring configuration(s).

Proposal 24: Support to configure the associated ID per CSI report configuration

- Support dynamic activation/deactivation of CSI report configuration

Proposal 25: Support the ML based beam prediction to facilitate the event triggered beam report.

Proposal 26: Defer the discussion on Option B based inference configuration procedure until all the parameters for CSI report configuration for beam prediction are finalized.