3GPP TSG RAN WG1 #120			R1-2500564

Athens, Greece, February 17th – 21st, 2025

Title:	Draft Reply LS on UL 8Tx

Response to:	R1-2500012 (R2-2411220)

Release:	Release 18

Work Item:	NR_MIMO_evo_DL_UL-Core

Source:	Google, [RAN1]

To:	RAN2

Cc:

Contact Person:

Name:	Yushu Zhang

E-mail Address:	yushuzhang@google.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN1 would like to thank RAN2 for the questions on UL 8Tx operation and would like to provide the following answers to the questions.

1. To support UL 8Tx in the MAC specification, RAN2 made the following assumption regarding how to model UL 8Tx operation from MAC point of view to minimize specification impacts.

For UL 8Tx 2TB transmission scheduled by one DCI, MAC entity considers two uplink grants for two TBs in two HARQ processes with one HARQ process ID and two HARQ buffers.

A1: In current RAN1 spec (38.214) as follows, the UL 8Tx 2TB transmission is scheduled by one uplink grant in one DCI.

## 6.1 UE procedure for transmitting the physical uplink shared channel PUSCH transmission(s) can be dynamically scheduled by an UL grant in a DCI, or the transmission can correspond to a configured grant Type 1 or Type 2. The configured grant Type 1 PUSCH transmission is semi-statically configured to operate upon the reception of higher layer parameter of configuredGrantConfig including rrc-ConfiguredUplinkGrant without the detection of an UL grant in a DCI. The configured grant Type 2 PUSCH transmission is semi-persistently scheduled by an UL grant in a valid activation DCI according to clause 10.2 of [6, TS 38.213] after the reception of higher layer parameter configuredGrantConfig not including rrc-ConfiguredUplinkGrant. If configuredGrantConfigToAddModList is configured, more than one configured grant configuration of configured grant Type 1 and/or configured grant Type 2 may be active at the same time on an active BWP of a serving cell.

2. For UL 8Tx 2TB transmission scheduled by one DCI, if MAC entity only generates a single TB due to the uplink skipping, would there be any issue for PHY layer?

A2: According to current RAN1 spec, RAN1 has not identified any issue for PHY layer.

# 2	Actions

To RAN2

ACTION:   RAN1 respectfully asks RAN2 to take above information for further work.

# 3	Dates of next TSG RAN WG1 meetings

TSG-RAN1 #120b	7 April. – 11 April. 2025			China

TSG-RAN1 #121	19 May. – 23 May. 2025			Malta