3GPP TSG RAN WG1 Meeting #120		R1-2500631

Athens, Greece, February 17th – 21st, 2025

Source:	Fujitsu

Title:	Discussion on measurement related enhancements for LTM

Agenda Item:	9.9.1

Document for:	Discussion

# Introduction

In this contribution, we discuss remaining main issues for CSI-RS measurement for LTM, event triggered L1 measurement reporting and CSI acquisition.

# SP CSI-RS for gNB scheduled reporting

Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability.

In reply LS from RAN3 [1], RAN3 confirmed it is feasible to specify the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s), and RAN3 will continue to work on the details. Based on this reply LS, we propose to confirm the working assumption. We think that no further task in RAN1 is needed but the legacy scheme can be reused for supporting SP CSI-RS.

      - Proposal 1: Confirm the working assumption to support SP CSI-RS in the gNB scheduled reporting.

# Event triggered reporting

## Semi-persistent CSI-RS resource

##### [FL Proposal 1-3-v1] Companies are encouraged to provide their view on the following FL suggestion - For the support of semi-persistent CSI-RS transmission for event triggered reporting, - Approach 1: - The necessity depends on the presence of activation/deactivation mechanism of event triggered reporting - If the LTM events and evaluation are always active after RRC configuration, the UE expects that CSI-RS is always transmitted by periodic resource. There would be no chance for candidate cells to turn off the CSI-RS transmission - If the LTM events and evaluation can be activated/deactivated by e.g. MAC CE, use of semi-persistent CSI-RS resource is more relevant - RAN1 to ask RAN2 the presence of activation/deactivation mechanism of evaluation for LTM events. If their answer is yes, the same approach as gNB scheduled report can be taken, i.e. set working assumption - Approach 2: - semi-persistent CSI-RS is not considered for event triggered reporting

In previous meeting, the periodic CSI-RS was agreed for the event triggered reporting. In turn, the semi-persistent CSI-RS had been discussed in the last meeting, but the consensus was not made. Instead, FL summary provided two approaches as shown in above. As we proposed in the gNB scheduled reporting, only periodic CSI-RS may suffer from the redundant interference, and disturb the network energy saving operation.

      - Proposal 2: Support SP CSI-RS in the event-triggered reporting in order to achieve the network energy saving and interference reduction.

One thing we need to care about is to avoid UE monitoring when SP CSI-RS is not transmitted. Generally, the UE keeps monitoring the event as soon as it is configured even when the SP CSI-RS is not transmitted as shown in Fig.1.

Figure 1. Idle period during the event monitoring

The “IDLE” period in Fig.1 affects to the measurement burden to the UE where the measurement results are not useful. In order to avoid this issue, RAN1 should ask to RAN2 whether the event monitoring can be activated / deactivated in accordance with that of SP CSI-RS.

      - Proposal 3: Send an LS to RAN2 to ask whether the event monitoring can be activated / deactivated in accordance with that of SP CSI-RS.

For the aperiodic CSI-RS, it may not fit into the event triggered monitoring since the serving cell should trigger every single CSI-RS transmission for the event. This makes inefficient behaviours for both the serving/candidate cells and UE. Hence, we propose that the aperiodic CSI-RS transmission should not be supported for the event triggered reporting.

      - Proposal 4: The aperiodic CSI-RS transmission is not supported for the event triggered reporting.

## Filtering for the event evaluation and measurement

[FL Proposal 3-5-v1] - Approach 1: L1-specified filtering for L1-RSRP based on SSB and CSI-RS is introduced for event evaluation and reporting - The network configurable filter is a first order IIR filter, which gives more flexibility for the network - Approach 2: No L1-specified filtering for L1-RSRP based on SSB and CSI-RS is introduced for event evaluation and reporting

In previous meeting, the filtering for event triggered reporting had been discussed, but no consensus was made. The goals for the filtering are to avoid ping-pong issue and to reduce the reporting overhead. In the discussion, some companies proposed that the Time-to-Trigger (TTT) can solve the issue. We think, however, TTT may not be enough to cope with the large fluctuation of the L1-RSRP. As an example, the L1-RSRP has large fluctuation. Then, the serving cell can set the long duration of TTT to avoid the fluctuation. In this case, it might be difficult to satisfy the TTT condition, accordingly, there will be large delay the event. On the other hand, if the duration of TTT is short, the TTT is too sensitive against the measured L1-RSRP, therefore the ping pong issue may occur frequently. Besides, if the filtering is applied to the RSRP, the fluctuation will be suppressed, and hence it makes easier for the serving cell to set the TTT duration.

There was another discussion in previous meeting about the filtering method with the network configuration and the UE implementation filter. Even though the filtering by either the network configuration or the UE implementation can mitigate such large fluctuation, however, the fluctuation level of the filtered L1-RSRP significantly varies depending on the UE implementation, consequently, it will also be an obstacle for the serving cell to appropriately set the threshold of the event condition.

Based on our analysis, we propose to introduce the network configurable filter for the event triggered reporting.

      - Proposal 5: Introduce the network configurable filter for event evaluation and reporting

# CSI acquisition

## Triggering mechanism for CSI acquisition

Working Assumption As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE - Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified)

The working assumption as shown in above was made in last meeting for CSI acquisition timing. The remaining issue is whether to introduce the one triggering mechanism for early CSI acquisition before the reception of the cell switch command (CSC). The working assumption provides the baseline that the CSI-RS measurement for CSI is performed after the reception of CSC. This behaviour impacts on the interruption time because of CSI measurement time as shown in the Fig.2. Therefore, it is important to have the feature of CSI measurement before the reception of CSC.

Figure 2. Duration of the CSI measurement after CSC for RACH-based LTM.

      - Observation 1: The CSI measurement before the reception of CSC is also needed in order to mitigate impact to the interruption time.

In the discussion of the previous meeting, some companies proposed that the UE can measure the CSI before the reception of CSC without trigger. This method may increase the measurement burden in UE side since the UE does not have any information which CSI-RS resource to measure, and thus, UE has to measure multiple candidate cells. Alternatively, the serving cell can indicate the cell to be measured to UE via the existing signalling such as PDCCH order or TCI state activation/deactivation MAC CE. However, those existing signallings are optional features. So, those signals cannot be the complete solution. Hence, the new signalling for the measurement trigger is naturally required.

      - Observation 2: The UE does not have any information which CSI-RS resource to measure if triggering indication is not introduced before CSC resulting in higher UE complexity.

The solution is to adopt the CSI measurement before the reception of CSC by introducing the unified triggering mechanism. We believe that the unified triggering mechanism before the reception of CSC can play a role to indicate whether the CSI acquisition is performed for both UEs with/without capability. For example, it is straightforward for UE with capability that the triggering mechanism can indicate to perform CSI acquisition before CSC. On the other hand, for UE without capability, the triggering mechanism can indicate to perform CSI acquisition after reception of the CSC. If the UE does not receive the triggering indication, the UE does not have to measure the CSI after CSC. Based on our analysis, we propose to confirm the working assumption by introducing the unified triggering mechanism for CSI acquisition for the UE with and without UE capability.

      - Proposal 6: Confirm working assumption by introducing the unified trigger of CSI acquisition with and without UE capability.

- Triggering indication can be provided before the reception of CSC.

- Measurement timing is up to UE capability according to the triggering indication

  - For UE with capability, the triggering indication can indicate to perform CSI acquisition, immediately.

  - For UE without capability, the triggering mechanism can indicate to perform CSI acquisition after reception of the cell switch command.

## Time domain property of CSI-RS resource

Similar to other discussions for the time domain property of CSI-RS resource, we have three options as periodic, semi-persistent and aperiodic CSI-RS transmission. For the periodic CSI-RS transmission, it might be reasonable option because of its simplicity. Furthermore, for the network energy saving and interference perspective, the gNB-controlled CSI-RS transmission is needed such as semi-persistent or aperiodic CSI-RS. As discussed before, those two schemes require RAN3 support for inter-CU case. Based on our analysis, we propose to support periodic CSI-RS transmission for CSI acquisition as baseline.

      - Proposal 7: Support periodic CSI-RS transmission for CSI acquisition as baseline.

- FFS: semi-persistent and aperiodic CSI-RS

## CSI reporting

Time domain property

We also have to consider about the time domain property of the CSI reporting. For the periodic CSI reporting, the UE starts reporting after reception of CSC regardless of the measurement timing according to the working assumption made in the last meeting. However, the reporting after cell switching completion may be meaningless since the precision of the measured CSI is not high due to the limited measurement configuration (e.g., less number of ports, wideband). Accordingly, we need to discuss how to treat the periodic reporting after cell switch completion. Similarly, the semi-persistent reporting requires that the target cell should transmit deactivation command to stop reporting since the reporting may not be useful after cell switching completion similar to periodic reporting. On the other hand, the aperiodic reporting might be reasonable option to apply CSI to first several DL schedules in target cell before the nominal CSI acquisition in the target cell as there is no need to continue reporting after cell switch completion.

      - Proposal 8: At least aperiodic reporting should be supported for CSI acquisition

Container

In RAN1#119 meeting, it has been agreed to support CSI acquisition for LTM, where CSI is reported after the cell switch command. As a simple but efficient way for determination of the report container, CSI can be multiplexed on the first PUSCH transmission scheduled by configured or dynamic grant PUSCH that carries the RRCRecofigurationComplete message for RACH-less LTM, or the first PUSCH after the random access procedure for RACH-based LTM. As a further study, for RACH-based LTM, the PUSCH scheduled by RAR UL grant can be considered if earlier reporting is beneficial and feasible.

      - Proposal 9:

- For RACH-less LTM, support the first PUSCH transmission scheduled by CG or DG as a report container of CSI acquisition.

- For RACH-based LTM, support at least the first PUSCH scheduled by RAR as a report container of CSI acquisition.

  - FFS: Msg3 for RACH-based LTM

# Conclusion

In this contribution, we provide our views as below.

      - Proposal 1: Confirm the working assumption to support SP CSI-RS in the gNB scheduled reporting.

      - Proposal 2: Support SP CSI-RS in the event-triggered reporting in order to achieve the network energy saving and interference reduction.

      - Proposal 3: Send an LS to RAN2 to ask whether the event monitoring can be activated / deactivated in accordance with that of SP CSI-RS.

      - Proposal 4: The aperiodic CSI-RS transmission is not supported for the event triggered reporting.

      - Proposal 5: Introduce the network configurable filter for event evaluation and reporting

      - Observation 1: The CSI measurement before the reception of CSC is also needed in order to mitigate impact to the interruption time.

      - Observation 2: The UE does not have any information which CSI-RS resource to measure if triggering indication is not introduced before CSC resulting in higher UE complexity.

      - Proposal 6: Confirm working assumption by introducing the unified trigger of CSI acquisition with and without UE capability.

- Triggering indication can be provided before the reception of CSC.

- Measurement timing is up to UE capability according to the triggering indication

  - For UE with capability, the triggering indication can indicate to perform CSI acquisition, immediately.

  - For UE without capability, the triggering mechanism can indicate to perform CSI acquisition after reception of the cell switch command.

      - Proposal 7: Support periodic CSI-RS transmission for CSI acquisition as baseline.

- FFS: semi-persistent and aperiodic CSI-RS

      - Proposal 8: At least aperiodic reporting should be supported for CSI acquisition

      - Proposal 9:

- For RACH-less LTM, support the first PUSCH transmission scheduled by CG or DG as a report container of CSI acquisition.

- For RACH-based LTM, support at least the first PUSCH scheduled by RAR as a report container of CSI acquisition.

  - FFS: Msg3 for RACH-based LTM

# References

[1] R1-2500027	Reply LS for LS on the support of semi-persistent CSI-RS resource for LTM candidate cells	RAN3, CATT

[2] Chair’s note RAN1#119