3GPP TSG RAN WG1 #120	R1-2500678

Athens, Greece, Feb. 17th – 21st, 2025

Source:	Lenovo

Title:	Channel model extension for 7-24 GHz

Agenda Item:	9.8.2

Document for:	Discussion and Decision

# Introduction

In RAN1#119 [1], the following has been agreed as part of the channel model extension study for 7-24 GHz, including near-field channel modeling and spatial non-stationarity modeling:

Agreement For near-field channel, the following formula is adopted to model the angular domain parameters of direct path between TRP and UE as antenna element-wise channel parameter: Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100-1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s Where θLOS,ZOA,u,s, ϕLOS,AOA,u,s, θLOS,ZOD,u,s, ϕLOS,AOD,u,s are the respective antenna element-wise elevation arrival angles, azimuth arrival angles, elevation departure angles and azimuth departure angles of LoS path between the transmit antenna element s and receive antenna element u. Working Assumption - For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the nearest K blockers from BS are selected. - For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, at least for blockage model B, the following parameters to define the building edge are introduced in the Table 7.6.4.2-5 in TR 38.901: Typical set of blockers; Blocker dimensions; Mobility pattern. Outdoor; Building edge; Cartesian: w=50 m; h=20 m; Stationary. - In the blockage model B, the following equation is used to calculate the attenuation caused by the building edge: - LdB=20log100.5-F, where F represents one of Fh1, Fh2, Fv1, and Fv2 defined in equation 7.6-29 in section 7.6.4.2 of TR 38.901. Agreement For the modelling of spatial non-stationarity at BS side, if stochastic based approach, at least for unified visible probability (VP) and visibility region (VR), is adopted, the following steps are considered: - Step 1: Determine whether a cluster is impacted by SNS - Step 2: Generate the visibility region - Step 3: Calculate the power attenuation factor FFS: the details of each step Note: Merge of some of the steps is not precluded Working Assumption For the modelling of spatial non-stationarity at UE side mainly due to close proximity, if blocker-based approach is adopted, the Option-3 is supported with following details: - Three typical cases are introduced: one hand grip, dual hand grip, head and one hand grip. - FFS: probability of each case in the simulation. - For each case, to model the attenuation for each of antenna in the candidate antenna location defined in the UE antenna model for handheld device with one of following alternatives: - Alt 1: Introduce a fixed value for each candidate antenna position per case. - Alt 2: Introduce a distribution for candidate antenna position. - FFS: distribution per antenna position(s) and per case. Agreement For near-field channel, to generate the channel parameter, the reference point is defined as: - The physical center of the antenna array/center of the device. Conclusion For near-field, no changes are expected on following parameter for direct path between TRP and UE: - Doppler shift Conclusion For near-field, no changes are expected on following parameter for non-direct path between TRP and UE: - Doppler shift Agreement For near-field channel, the following formula is adopted to model the angular domain parameters of non-direct path between TRP and UE as antenna element-wise channel parameter: Frx,u,θθn,m,ZOA,u,ϕn,m,AOA,uFrx,u,ϕθn,m,ZOA,u,ϕn,m,AOA,uTexpjΦn,mθθkn,m-1expjΦn,mθϕkn,m-1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,s,ϕn,m,AOD,sFtx,s,ϕθn,m,ZOD,s,ϕn,m,AOD,s where θn,m,ZOA,u,ϕn,m,AOA,u, θn,m,ZOD,s,ϕn,m,AOD,s are the ray-wise angular domain parameters of ray m cluster n between the transmit antenna element s and receive antenna element u. Agreement For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the rotation and power variation calculation are conducted in ray level. Agreement For near-field channel, for the non-direct paths, the distance d1 between the BS and the 1st point associated with cluster is generated with one of following options that to be selected in RAN1#120: - Option-1: The distance d1 is generated directly following a specific distribution. - For the k1 clusters, d1 = speed of light times the absolute delay of the cluster - For other k2 clusters, the distance d1 is equal or less than the speed of light times the absolute delay of the cluster and generated following a specific distribution - FFS: The specific distribution, e.g., log-normal distribution for UMi - FFS: The values of k1 and k2 - The generation of absolute delay of the cluster is according to the procedure defined in Section 7.6.9 in TR 38.901 - FFS: The value for UMi, UMa, InH-Office. - Option-2: The distance d1 is generated by a scaling factor multiplied by the absolute distance corresponding to the absolute delay of the cluster, where the scaling factor is generated from a specific distribution. - For the k1 clusters, d1 = speed of light times the absolute delay of the cluster (i.e., the scaling factor is 1) - For other k2 clusters, the distance d1 is equal or less than the speed of light times the absolute delay of the cluster and the scaling factor generated following a specific distribution (i.e., the scaling factor is equal or less than 1) - FFS: Distribution of scaling factor, e.g., Beta distribution for UMa - FFS: The values of k1 and k2 - The generation of absolute delay of the cluster is according to the procedure defined in Section 7.6.9 in TR 38.901 - FFS: The value for UMi, UMa, InH-Office. Note 1: If d2 is agreed to be considered, the generation of d2 can follow the same principle with the generation of d1. Working Assumption For near-field channel, for non-direct paths, impact of spherical wavefront is optionally considered at UE side. Agreement For near-field channel, in principle, one of following options is considered to calculate the antenna element-wise delay of direct path between TRP and UE: - Option-1: The following formula is used to calculate the antenna element-wise delay of direct path: τ1,u,s=τ1-d3D-¦ru,s¦c where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side. - The above formula is applied with the same principle as existing model in Clause 7.6.2 of TR 38.901, and the channel response of LOS ray between Rx antenna u and Tx antenna s at delay τ at time t is given by: Hu,s,1LOSt;τ=Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100-1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s exp-j2πd3Dλfexp-j2πru,s-d3Dλfexpj2πrrx,LOST∙vλ0tδτ-τ1,u,s - Option-2: The antenna element-wise delay of direct path is directly modeled based on the distance of direct path between antenna elements at TRP and UE side. - Following formula is adopted to model the antenna element-wise delay in the channel coefficient as: δτ-τ1,u,s, τ1,u,s=τ1-d3D-¦ru,s¦c where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side. - Option-3: Confirm the following working assumption made in RAN1#118BIS: - “The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay.” - Note: τ1 is same as that is defined in 7.5-30 in TR38.901

In this document we discuss further remaining aspects on channel extension for 7-24 GHz that are expected to be captured in TR 38.901 [2].

# Channel model extension for 7-24 GHz

In RAN1#118bis [3], modeling of near-field channels for both the direct path between the TRP and the UE as well as the corresponding non-direct path(s) has been discussed, where it has been agreed to model the angular domain parameters in an antenna element-wise fashion for the direct path and non-direct path(s), as well as supporting antenna element-wise delay modeling as an optional component. No conclusion was reached on Doppler shift handling. Furthermore, for delay modeling, a working assumption has been made corresponding to adopting the same model in Clause 7.6.2 of TR 38.901 for large bandwidth and large antenna array. Under this model, the delay values are modeled to vary across different rays per cluster, and additionally, the number of rays per cluster would depend on the antenna aperture size. The additional modeling component does not however model delay values to be per antenna element; it models the delay values to vary based on the overall antenna array dimension. In our opinion, further clarity is needed on the applicability of the additional modeling component in Clause 7.6.2 in TR 38.901 to antenna element-wise modeling for the direct and non-direct path(s) of the near field channel model.

For antenna element-wise modeling of delay for the near-field channel for the direct and non-direct path(s), further clarification is needed on the working assumption regarding the applicability of the additional modeling component in Clause 7.6.2 of TR 38.901 to the near-field channel with antenna element-wise delay modeling.

In general, the applicability of the near-field region depends on two factors: the BS-UE distance as well as the BS antenna aperture size. Large BS antenna arrays are expected to be packed with a massive number of antenna elements, e.g., >1000 antenna elements, and hence element-wise modeling of non-angular parameters may incur significant complexity in simulations requiring to model channels for multiple UEs, each with different TRPs, as well as modeling each BS-UE channel with multiple clusters and multiple rays within, with further modeling of channel parameters to be different across antenna elements. While accurate channel modeling is important, the support of reasonably complex models is instrumental to enable more engaged simulation activities and ensure that simulation runtime is not exponentially larger, leading to limiting the simulation activity. Therefore, antenna common parameters or at least antenna-group common parameter design is preferred, e.g., an antenna array with K antenna elements is decomposed to G groups of M antenna elements each, wherein the M antenna elements within the same antenna group share the same parameter values, i.e., K= G.M antenna elements per BS antenna array with G groups of non-angular parameter values are supported.

For non-angular parameters in near-field channels, the parameters are modeled to have antenna group-wise parameters, i.e., antenna elements of a sub-array of the BS antenna array comprising multiple antenna elements are modeled to have the same parameter values.

It was also agreed in RAN1#118bis to model non-direct path(s) between a UE and a gNB for near-field channels to be based on one or two reference distance positions d1,m and d2,m associated with the distance from a cluster m to the BS side and the UE side, respectively. Furthermore, two options were discussed in RAN1#119 as summarized below in Table 1.

| Feature | Option-1 | Option-2 |
| --- | --- | --- |
| The distance d1 generation | Following a specific distribution | Via a scaling factor multiplied by the absolute distance corresponding to the absolute cluster delay; the scaling factor generated via a specific distribution |
| Distance d1 calculation w.r.t. the k1 specular reflection clusters | d1=c × the absolute cluster delay | d1=c × the absolute cluster delay (i.e., the scaling factor is 1) |
| Distance d1 calculation w.r.t. the k2 remaining non-specular reflection clusters | d1 ≤ c × the absolute cluster delay; generated via a specific distribution | d1 ≤ c × the absolute cluster delay; the scaling factor is generated via a specific distribution (i.e., the scaling factor is ≤ 1) |
| The distance d2 generation | If supported. Follows the same principles of the generation of d1 |  |

Table 1. Option 1 vs. Option 2 for reference distance positions of non-direct clusters

Note that c above is the speed of light. In our opinion, the two options are correlated, with the main discrepancy related to whether the cluster delay is based on a specific distribution (Option 1), vs. being based on the absolute cluster delay multiplied by a scaling factor, according to a specific distribution (Option 2). Option 2 appears to be simpler to implement and model, compared with Option 1.

In a near-field environment, for modeling the reference distance positions d1,m and d2,m associated with the the non-direct path(s) from a cluster to the BS side and the UE side, respectively, Option 2 is preferred.

Furthermore, for the near-field channel, it was agreed in RAN1#119 to support one of following options to calculate the antenna element-wise delay of direct path between the BS and the UE:

- Option-1: The following formula is used to calculate the antenna element-wise delay of direct path:

τ1,u,s=τ1-d3D-|ru,s|c

where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side, where the above formula is applied with the same principle as existing model in Clause 7.6.2 of TR 38.901, with the following highlighted modifications:

Hu,s,1LOSt;τ=Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100-1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s

exp-j2πd3Dλfexp-j2πru,s-d3Dλfexpj2πrrx,LOST∙vλ0tδτ-τ1,u,s

- Option-2: The antenna element-wise delay of direct path is directly modelled based on the distance of direct path between antenna elements at TRP and UE side, with the antenna element-wise delay in the channel coefficient is adopted via the following formula:

δτ-τ1,u,s,  τ1,u,s=τ1-d3D-|ru,s|c

- Option-3: Reuse the existing model in Clause 7.6.2 of TR 38.901, as per RAN1#118bis working assumption.

In our view, it is not clear whether the discrepancy between Option 1 and Option 2 is the use of λf to replace λ0 in the highlighted terms. It is also not clear to us whether for a direct path the delay discrepancy would be notable or impactful for near-field channels. Once again, unless there is clear evidence obtained via field measurements that different resolvable paths can be identified in the near-field channel direct path, our preference would be confirming the RAN1#118bis working assumption, which reuses the existing model in Clause 7.6.2 of TR 38.901.

In a near-field environment, for the antenna element-wise delay of direct path between the BS and the UE, reuse the existing model in Clause 7.6.2 of TR 38.901 is used to model the delay.

Furthermore, non-stationarity modeling of the attenuation per antenna element due to blockage at the UE side has been discussed, and was agreed in RAN1#119 to support Option-3, where the attenuation factor is either generated via a distribution or kept at a fixed value based on three typical cases: (i) one hand grip, (ii) dual hand grip, or (iii) head and one hand grip. For each case, two alternatives can be considered to model the attenuation for each antenna in the candidate antenna location defined for the handheld UE, as follows:

- Alt-1: Introduce a fixed value for each candidate antenna position per case.

- Alt-2: Introduce a distribution for candidate antenna position.

Based on the three cases above, the antennas to be blocked would be different based on the different geometry of the blockage cases, which also depends on the UE antenna layout per UE, e.g., number and placement of UE antennas. Hence, Alt-1 requires providing fixed values for each antenna layout and for each case, which can be a tedious process to do. It is not clear how the correlation between different antennas can be modelled in this approach. Corelation can be introduced in Alt-2 more easily if correlation between different antennas needs to be included in the model. To simplify this process, our preference is to use a distribution for candidate antenna positions, so as to have a unified methodology for all use cases.

For non-stationarity modeling of the attenuation per antenna element due to blockage at the UE side in the candidate antenna location, Alt-2 is supported: Introduce a distribution for candidate antenna position.

# Conclusion

This contribution studied aspects on channel model extension for 7-24 GHz. We have the following proposals:

- For antenna element-wise modeling of delay for the near-field channel for the direct and non-direct path(s), further clarification is needed on the working assumption regarding the applicability of the additional modeling component in Clause 7.6.2 of TR 38.901 to the near-field channel with antenna element-wise delay modeling.

- For non-angular parameters in near-field channels, the parameters are modeled to have antenna group-wise parameters, i.e., antenna elements of a sub-array of the BS antenna array comprising multiple antenna elements are modeled to have the same parameter values.

- In a near-field environment, for modeling the reference distance positions d1,m and d2,m associated with the the non-direct path(s) from a cluster to the BS side and the UE side, respectively, Option 2 is preferred.

- In a near-field environment, for the antenna element-wise delay of direct path between the BS and the UE, reuse the existing model in Clause 7.6.2 of TR 38.901 is used to model the delay

- For non-stationarity modeling of the attenuation per antenna element due to blockage at the UE side, further discuss Option-2 and Option-3, whereas Option-1 is down prioritized.

# References

| [1] | 3GPP TSG RAN WG1#119, "Draft Meeting Report," Orlando, USA, Nov. 18-22, 2024. |
| --- | --- |
| [2] | 3GPP TR 38.901, "Study on channel model for frequencies from 0.5 to 100 GHz," Mar. 2022. |
| [3] | 3GPP TSG RAN WG1#118bis, "Draft Meeting Report," Hefei, Anhui, China, Oct. 14-18, 2024. |