3GPP TSG RAN WG1 #120                                                                                                  R1-2500713

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.1.4.1

Source:	            		Xiaomi

Title:	Views on two-side AI/ML model based CSI compression

Document for:	Discussion and Decision

Introduction

In the RAN#105 meeting, the following SID on two-sided AI/ML model based CSI compression feedback were achieved [1]. In this contribution, our views on two-sided AI/ML model based CSI compression feedback are provided.

Study objectives: - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

Performance evaluation of two-sided AI/ML based CSI compression feedback

Performance evaluation on use case 2

The following cases were agreed for the evaluation of temporal domain aspects of AI/ML-based CSI compression, and Case 2 and Case 3 are agreed to study with high priority. For Case 2, it has agreed that performance impact resulting from non-ideal UCI feedback needs to study. The following agreement on UCI loss modelling and three scenarios were provided to evaluate the performance impact of UCI loss.

Agreement For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following categorization for study: Case; Target CSI slot(s); Whether the UE uses past CSI information; Whether the network uses past CSI information. 0; Present slot; No; No. 1; Present slot; Yes; No. 2; Present slot; Yes; Yes. 3; Future slot(s); Yes; No. 4; Future slot(s); Yes; Yes. 5; Present slot; No; Yes. Note 1: For the UE, the past CSI information may include past model inputs and/or any information derived from them. For the network, the past CSI information may include past CSI feedback instances and/or any information derived from them. Note 2: For case 3 and case 4, the UE may perform prediction as a separate step or jointly with compression. Similarly, the network may perform prediction as a separate step or jointly with reconstruction. Companies to report which option is selected, the number of future slots, and whether the prediction is AI/ML-based or not. Note 3: “Target CSI slot(s)” refers to the slot(s) to which the CSI feedback in the report corresponds. “Present slot” refers to the slot of the most recent CSI-RS measurement used to generate the CSI report. “Future slot(s)” includes at least one slot after the present slot and may include the present slot as well. Note 4: Down-selection is not precluded. Agreement For temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, among Cases 1, 2, 3, 4, and 5, prioritize further discussion on Case 2 and Case 3. Agreement For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for Case 2, study the performance impact resulting from non-ideal UCI feedback. - Scenario A: no UCI loss - Note: Corresponds to an upper bound or re-aligning missing historical CSI information - Scenario B: UCI loss, known at NW and unknown at UE, with mitigation at NW - Note: Corresponds to implementation-based mitigation at NW but no signaling to UE. - Scenario C: UCI loss, known at NW and UE, with mitigation at NW and UE - Note: Corresponds to reset of historical CSI information at both UE and NW or any other mitigation approach enabled by signaling. - UCI loss modeling - 10% UCI loss probability on all UCI reports - Other options are not precluded, e.g., No UCI loss for the first UCI report of each observation window, and 10% UCI loss probability for the subsequent reports of each observation window, as a shortcut to simplify the evaluation work. - Other values for UCI loss probability are not precluded. - Note: The same UCI loss modeling shall be applied to the benchmark for fair comparison. - FFS: partial PMI-related UCI loss

In this section, performance evaluations on UCI loss for Case 2 are provided. Before proving the simulation results, the two-side AI/ML model design for Case 2 is firstly illustrated.

Two-side AI/ML model design for Case 2

As Figure 1 shown, the AI model structure is consisted with LSTM and Transformer. Different from AI model for spatial/frequency compression, the input of AI model for spatial/temporal/frequency compression includes both the CSI of current slot and historical CSI information from historic slot. The output of encoder is the CSI feedback and historic CSI information which would be used as input information of encoder for the next slot. Similar as the encoder part, the decoder needs to recovery the CSI based on both received CSI feedback and the historic CSI information.

Figure 1: Model structure of AI model for spatial/temporal/frequency domain CSI compression

Table 1 shows the complexity and parameter numbers of AI/ML model for spatial/frequency domain CSI compression and AI/ML model for spatial/temporal/frequency domain CSI compression. In the table, AI-SF and AI-TSF respectively denote the model of spatial/frequency domain CSI compression and spatial/temporal/frequency domain CSI compression. It is observed that the complexity of TSF compression is higher than SF compression, while the number of parameters of TSF compression is less than SF compression. The simulation assumption and parameters of AI model training are respectively given in Table A1 and A2 in the appendix.

Table 1: Complexity and memory storage of AI model

| Cases | FLOPs |  | Number of AI/ML model parameters |  |
| --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder | Decoder |
| AI-SF | 279 M | 371 M | 10.79 M | 14.29 M |
| AI-TSF | 792 M | 738 M | 7.66 M | 7.46M |

Evaluation results on UCI loss for Case 2

In the simulation, 10% UCI loss probability is adopted, and 20 slots historic CSI information is utilized as input of AI/ML for the current slot. For simplicity, we only consider one layer data transmission. For Scenario C, to align the missing historical CSI information, both UE and NW adopt latest output of historic CSI information before missing UCI. Two benchmarks, i.e., Rel-16 eType II codebook and Case 0 are evaluated as well as a reference. The evaluation results are respectively given in Table 2~Table 4 for Scenario A, Scenario B and Scenario C.

Table 2: Evaluation results for Scenario A, i.e., no UCI loss

| Temporal setting |  | Case 2, AI-TSF |
| --- | --- | --- |
| Benchmark 1 for compression (non-AI/ML) |  | Rel-16 eType Ⅱ codebook |
| Benchmark 2 for compression (Rel-18 AI/ML) |  | Case 0, AI-SF |
| SGCS of benchmark | X:<=80/a bits, benchmark 1 /benchmark 2 | 0.7049(62 bits)/0.7361(60 bits) |
|  | Y:100/a bits-140/a bits, benchmark1/benchmark2 | 0.7838(111 bits)/0.8028(120 bits) |
|  | Z:>=230/a bits, benchmark 1 /benchmark2 | 0.8689(279 bits)/0.8751(280 bits) |
| SGCS-Absolute value(gain% over benchmark) | X:<=80/a bits, over benchmark 1/benchmark 2 | 0.0967(13.71%)/0.0655(8.90%) |
|  | Y:100/a bits-140/a bits, over benchmark1/ benchmark 2 | 0.0918(11.71%)/0.0728(9.07%) |
|  | Z:>=230/a bits, over benchmark1/benchmark 2 | 0.0655(7.54%)/0.0593(6.78%) |

Table 3: Evaluation results for Scenario B, i.e., UCI loss, known at NW and unknown at UE, with mitigation at NW

| Temporal setting |  | Case 2, AI-TSF |
| --- | --- | --- |
| Benchmark 1 for compression (non-AI/ML) |  | Rel-16 eType Ⅱ |
| Benchmark 2 for compression(Rel-18 AI/ML) |  | Case 0, AI-SF |
| SGCS of benchmark | X:<=80/a bits, benchmark 1 /benchmark 2 | 0.7004(62 bits)/0.7352(60 bits) |
|  | Y:100/a bits-140/a bits, benchmark1/benchmark2 | 0.7773(111 bits)/0.8012(120 bits) |
|  | Z:>=230/a bits, benchmark 1 /benchmark2 | 0.8601(279 bits)/0.8727(280 bits) |
| SGCS-Absolute value(gain% over benchmark) | X:<=80/a bits, over benchmark 1/benchmark 2 | 0.0636(9.08%)/0.0288(3.92%) |
|  | Y:100/a bits-140/a bits, over benchmark1/ benchmark 2 | 0.0622(8.00%)/0.0383(4.78%) |
|  | Z:>=230/a bits, over benchmark1/benchmark 2 | 0.035(4.07%)/0.0224(2.57%) |

Table 4: Evaluation results for Scenario C, i.e., UCI loss, known at NW and UE, with mitigation at NW and UE

| Temporal setting |  | Case 2, AI-TSF |
| --- | --- | --- |
| Benchmark 1 for compression (non-AI/ML) |  | Rel-16 eType Ⅱ |
| Benchmark 2 for compression(Rel-18 AI/ML) |  | Case 0, AI-SF |
| SGCS of benchmark | X:<=80/a bits, benchmark 1 /benchmark 2 | 0.7004(62 bits)/0.7352(60 bits) |
|  | Y:100/a bits-140/a bits, benchmark1/benchmark2 | 0.7773(111 bits)/0.8012(120 bits) |
|  | Z:>=230/a bits, benchmark 1 /benchmark2 | 0.8601(279 bits)/0.8727(280 bits) |
| SGCS-Absolute value(gain% over benchmark) | X:<=80/a bits, over benchmark 1/benchmark 2 | 0.0701(10.00%)/0.0353(4.80%) |
|  | Y:100/a bits-140/a bits, over benchmark1/ benchmark 2 | 0.0682(8.77%)/0.0443(5.53%) |
|  | Z:>=230/a bits, over benchmark1/benchmark 2 | 0.0408(4.74%)/0.0282(3.23%) |

Based on the simulation results, we have the following observations:

Observation 1: For Scenario A, the SGCS AF-TSF performance gain over eType II codebook and AI-SF for different feedback payload is shown as follows:

- When the feedback payload is lower than 80 bits, the AI-TSF performance gain over eTypeII codebook is around 13.71% while the gain over AI-SF compression is around 8.9%.

- When the feedback payload is between 100 bits and 140 bits, the AI-TSF performance gain over eTypeII codebook is around 11.71% while the gain over AI-SF compression is around 9.07%.

- When the feedback payload is larger than 230 bits, the AI-TSF performance gain over eTypeII codebook is around 7.54% while the gain over AI-SF compression is around 6.78%.

Observation 2: For Scenario B, the AF-TSF performance gain over eType II codebook and AI-SF for different feedback payload is shown as follows:

- When the feedback payload is lower than 80 bits, the AI-TSF performance gain over eTypeII codebook is around 9.08% while the gain over AI-SF compression is around 3.92%.

- When the feedback payload is between 100 bits and 140 bits, the AI-TSF performance gain over eTypeII codebook is around 8.00% while the gain over AI-SF compression is around 4.78%.

- When the feedback payload is larger than 230 bits, the AI-TSF performance gain over eTypeII codebook is around 4.07% while the gain over AI-SF compression is around 2.57%.

Observation 3: For Scenario C, the AF-TSF performance gain over eType II codebook and AI-SF for different feedback payload is shown as follows:

- When the feedback payload is lower than 80 bits, the AI-TSF performance gain over eTypeII codebook is around 10.00% while the gain over AI-SF compression is around 4.80%.

- When the feedback payload is between 100 bits and 140 bits, the AI-TSF performance gain over eTypeII codebook is around 8.77% while the gain over AI-SF compression is around 5.53%.

- When the feedback payload is larger than 230 bits, the AI-TSF performance gain over eTypeII codebook is around 4.74% while the gain over AI-SF compression is around 3.23%.

Since eType II codebook and AI-SF compression do not utilize historic time information, the performance of them keep constant for Scenario B and Scenario C. However, the performance gains of AI-TSF compression over eType II codebook and AI-SF compression are different for Scenario B and Scenario C. Compared with performance of AI-TSF compression in Scenario B, the SGCS performance gain of AI-TSF compression Scenario C could further improved, as shown in Table 5. We can observe that there are additional around 1% performance gain obtained for Scenario C compared with Scenario B.

Table 5: The SGCS performance gain of AI-TSF compression Scenario C over Scenario B

| Payload | SGCS performance gain |
| --- | --- |
| 60 bits | 0.85% |
| 120 bits | 0.71% |
| 280 bits | 0.65% |

Observation 4: Compared with Scenario B, additional around 1% performance gain could be obtained for Scenario C.

According to above observation, we can see that aligning the missing historical CSI information between UE and NW could bring a little performance gain. To align the missed historical CSI information, both UE and NW adopting latest output of historic CSI before missing UCI is straightforward and simple way. There is no additional signalling or feedback overhead if such aligning approach is predefined.

Proposal 1: To align the missed historical CSI information when UCI loss occurs, the latest output historic CSI of encoder and decoder before missing UCI could be respectively adopted as the input of encoder and decoder in the current inference instance.

Performance evaluation on use case 3

AI model description

For case 3, one CSI prediction model and one CSI encoder model is used in the UE side while one CSI decoder model is used in the NW side in our evaluation. The input and output of CSI prediction model is raw channel information while the input of CSI encoder is the eigenvector information of the output of CSI prediction model.

Evaluation assumption and results

The SGCS results for R18 Doppler eTypeII and AI-TSF (case 3) CSI compression are respectively shown is Table 6 and Table 7 for different UE distribution. The performance of R18 Doppler TypeII codebook is used as benchmark.

Table 6：SGCS results for R18 Doppler Type II and AI-TSF(case 3) CSI compression on UE distribution  100% outdoor (30 km/h)

| Temporal setting |  | Case 3, AI-TSF |
| --- | --- | --- |
| Benchmark 1 for compression (non-AI/ML) |  | Rel-18 Doppler eType II |
| SGCS of benchmark, [layer 1] | X: <=80/α bits | 0.5762 (60 bits) |
|  | Y: 100/α bits-140/α bits | 0.6873 (120 bits) |
|  | Z: >=230/α bits | 0.7154 (280 bits) |
| SGCS-Absolute value (gain% over benchmark), layer 1 | X: <=80/α bits | 0.8053 (39.76%) |
|  | Y: 100/α bits-140/α bits | 0.8305 (20.84%) |
|  | Z: >=230/α bits | 0.8877 (24.08%) |

Regarding to UE distribution 100% outdoor (30km/h), we have the following observations:

Observation 5: Regarding to UE distribution 100% outdoor (30km/h), the SGCS gain of case 3 over Rel-18 Type II Doppler codebook for different feedback payload is shown as follows：

- When the feedback payload is lower than 80 bits, the SGCS gain of case 3 over benchmark is around 39.76% for layer 1.

- When the feedback payload is between 100 bits and 140 bits, the SGCS gain of case 3 over benchmark is around 20.84% for layer 1.

- When the feedback payload is larger than 230 bits, the SGCS gain of case 3 over benchmark is around 24.08% for layer 1.

Table 7：SGCS results for R18 Doppler eTypeII and AI-TSF(case 3) CSI compression on UE distribution 80% indoor (3 km/h), 20% outdoor (30 km/h)

| Temporal setting |  | Case 3, AI-TSF |
| --- | --- | --- |
| Benchmark 1 for compression (non-AI/ML) |  | Rel-18 Doppler eType II |
| SGCS of benchmark, [layer 1] | X: <=80/α bits | 0.5923 (60 bits) |
|  | Y: 100/α bits-140/α bits | 0.6221 (120 bits) |
|  | Z: >=230/α bits | 0.7845 (280 bits) |
| SGCS-Absolute value (gain% over benchmark), layer 1 | X: <=80/α bits | 0.7419(25.26%) |
|  | Y: 100/α bits-140/α bits | 0.8000(28.60%) |
|  | Z: >=230/α bits | 0.8799(12.16%) |

Regarding to UE distribution 80% indoor (3km/h),20% outdoor(30km/h), we have the following observations:

Observation 6: Regarding to UE distribution 80% indoor (3km/h),20% outdoor(30km/h), the SGCS gain of case 3 over Rel-18 Type II Doppler codebook for different feedback payload is shown as follows：

- When the feedback payload is lower than 80 bits, the SGCS gain of case 3 over benchmark is around 25.26% for layer 1.

- When the feedback payload is between 100 bits and 140 bits, the SGCS gain of case 3 over benchmark is around 28.60% for layer 1.

- When the feedback payload is larger than 230 bits, the SGCS gain of case 3 over benchmark is around 12.26% for layer 1.

Inter-vendor training collaboration for two-sided model

Remained issues on inter-vendor collaboration options for Direction A, Direction B and Direction C

In RAN1#118 and RAN1#118 bis meeting, the following agreements on inter-vendor collaboration for Direction A, Direction B and Direction C were achieved.

Agreement Continue the study of following directions - on-device operation and UE side offline engineering - Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4) - Potential down-selection into one or more among sub-options 3a/5a-1, 3a/5a-2, 3a/5a-3, 4-1, 4-2, and 4-3 considering their feasibility and performance, including at least the following issues - [Issue 1] What additional information should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing? - [Issue 2] Is there concern for NW’s proprietary information disclosure, and if so, how to address it? - [Issue 3] Is there an overhead concern, and if so, how to address it? - [Issue 4] Is there performance impact due to mismatch between NW side data distribution and UE side data distribution, and if so, how to address it? - Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b), including at least the following issues - [Issue 3] Is there an overhead concern, and if so, how to address it? - [Issue 5] Whether it is feasible to use a common encoder across UEs, and whether it is feasible for NW-side to train multiple encoders for different UEs? - [Issue 6] Is there performance impact due to mismatch between NW side data distribution and UE side inference data distribution, and if so, how to address it? - [Issue 7] Is there concern for NW’s and UE’s proprietary information disclosure, and if so, how to address it? - Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1), including at least the following issues - [Issue 8] Whether to consider 3GPP’s statistical channel model or field data for reference model(s) training. In case of the latter, how does RAN1 collect field data and agree on them? - [Issue 9] Is there performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution, and if so, how to address it? - [Issue 10] What additional information should be specified to enable UE-side encoder training, validation, and testing, and NW-side decoder training, validation, and testing? - Note: - 1-1: Only reference encoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference encoder. - 1-2: Only reference decoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference decoder. - 1-3: Both reference encoder and reference decoder are specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately that are compatible to the reference decoder/encoder. - Note: UE-side data and NW-side data in “UE-side data distribution” and “NW-side data distribution” are field data. - Note: Some issues identified in one direction may/may not be applicable for other Direction. - Note: potential down selection among the 3 directions is not precluded - Study of data distribution mismatch to consider the use of synthetic data and/or field data. Agreement For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression, - For Direction A: - Further study the following options with potential down-selection - 4-1: dataset is {target CSI, CSI feedback}, - 3a-1: encoder parameter exchange, with {target CSI} - Note: Additional information beyond what’s listed above may need to be exchanged if necessary. Agreement - For Direction C: - It is clarified that, the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side. - Study whether the fully specified reference models or their compatible models may provide reasonable performance for deployment at NW and UE - Including performance evaluation of training/finetuning the models by modeling data distribution mismatch between synthetic and field data, and/or by using actual field data - Including performance evaluation of training/finetuning the models by modelling data distribution mismatch between NW-side field data and UE-side field data, and/or by using actual field data from both NW-side and UE-side - Note: Training/finetuning is done at either or both sides without inter-vendor collaboration. - FFS whether the reference model is the encoder (1-1) or decoder (1-2) or both (1-3) - FFS whether the reference model is the same as RAN4 specified model - FFS whether a single or multiple reference (and how many) models need to be specified Agreement For the evaluation studies of Direction A, - For encoder parameter exchange from NW-side to UE-side of 3a-1, - Alt. 1: UE-side first trains a nominal decoder - Step 1: UE-side develops a nominal decoder against the exchanged encoder. - Step 2: UE-side develops actual encoder against the nominal decoder - Alt. 2: UE-side directly develops and trains the actual encoder. - Note: Target CSI is available at UE side - For dataset exchange from NW-side to UE-side {target CSI, CSI feedback} of 4-1, - Alt. 1: UE-side first trains a nominal decoder - Step 1: UE-side develops a nominal decoder using the exchanged dataset. - Step 2: UE-side develops an actual encoder against the nominal decoder. - Alt. 2: UE-side directly develops and trains the actual encoder. Agreement For discussion on performance degradation due to UE-side / NW-side data distribution mismatch with respect to UE side additional condition (issue 4 and 6), consider - NW-side data (dataset A) and UE-side data (dataset B) are mismatched in terms of UE-side additional condition (e.g., SVD phase normalization method, UE antenna virtualization, antenna imbalance, antenna spacing / layout, etc) - Note: dataset A may or may not contain dataset B - Note: if dataset A includes dataset B, this means NW timely collect data from the UE - Note: Dataset A and dataset B are aligned in terms of NW-side additional conditions. - Case 1: An encoder-decoder pair is trained on dataset B. This serves as an upper-bound. - Case 2 (for direction A): - For direction A: NW-side trains a decoder on dataset A, and UE-side trains an encoder based on either dataset A, dataset B, or both (where the choice depends on the inter-vendor collaboration Options (4-1 or 3a-1), the training alternatives (Alt 1 or Alt 2) within the Options, and the choice of target CSI for training) - Case 3 (for direction B): NW-side train an encoder-decoder on dataset A and transfers the encoder parameters to UE. - Study issue 4 and 6 by evaluating the inference performance for Case 2 / 3 on test dataset B, comparing to the inference performance for case 1 on test dataset B. - Note: Evaluations are assumed to performed without common reference model structure among companies. Agreement - For the evaluation study of offline engineering for Direction C for Issue 9: - Case 1: The AI/ML model is trained based on training Dataset B, and then the AI/ML model performs inference/test on a dataset from test Dataset B. - Note: This serves as the upper-bound performance. - Case 2: The AI/ML model is trained based on training Dataset S, and then the AI/ML model performs inference/test on test Dataset B. - Note: This represents the performance of Direction C when not using field data. - Case 2A: - Step 1: The AI/ML model is trained based on training Dataset S - Step 2: - NW-side and/or UE-side may separately train new CSI reconstruction part and/or CSI generation part compatible to the AI/ML model from Step 1 based on training Dataset B, as agreed in Note 1-1, 1-2, and 1-3 as agreed in RAN1#118. - Dataset from Step 1 may be additionally used. - Step3: The CSI generation part and the CSI reconstruction part from Step 2 performs inference/test on test Dataset B - Note: This represents the performance of Direction C when the AI/ML models are updated based on field data. - Note: Dataset S is meant to represent the synthetic data used for fully specified model development, and Dataset B is meant to represent the field data. - Examples for Dataset S and B: - Modeling via TxRU mapping, UMi vs. UMa, UMa vs. InH, etc. - It is acknowledged that the scenario/configuration modeling between S and B may not be complete representative of the difference between synthetic and field data distribution mismatch. - Synthetic vs. actual field data (if available) - Companies are encouraged to provide details on the scenarios/configurations underlying the field data - Note: Case 1 and Case 2 have been studied during Rel-18 generalization study and conclusions have been captured in the TR. - Note: For Case 2A, companies may additionally evaluate UE-side / NW-side data distribution mismatch with respect to UE side additional condition. This can be done, in Step 2, by treating the training Dataset B as representing UE-side data distribution and considering a training Dataset A representing NW-side data distribution. - Note: Evaluations are assumed to performed without common reference model among companies.

According to above agreements, some remained issues for Direction A, Direction B and Direction C have not been identified. Our views are provided for these issues in this sub-section.

Direction A: Sharing parameters/ dataset that enables UE-side offline engineering (I.e., inter-vendor collaboration option 3a-1/4-1)

- Issue 1: What additional information should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing?

According to discussion on other aspects of AI/ML model and data, the NW-sided additional condition is associated with an ID in order to avoid disclosing NW’s property information. For training encoder at UE side, the associated ID needs to share to UE as well for denoting the NW’s additional condition. For Option 3a-1 and Option 4-1, encoder will be updated or retrained at UE side.  It is necessary to share performance target to UE for training a reasonable encoder. Hence, at least associated ID and performance target should be shared to UE.

Proposal 2: At least the following additional information is shared to enable UE-side encoder training, validation, and testing for Direction A:

- Associated ID

- Performance target

- Issue 3: Is there an overhead concern, and if so, how to address it?

For Option 3a-1 and Option 4-1, the parameter or dataset needs to be exchanged between NW side and UE for model training. For the CSI generation part and CSI reconstruction part model of CSI compression including time-spatial-frequency domain, the size of model parameter is no more than 30 MB, as given in Table 1. The size of exchanged training dataset is usually very larger. The size of dataset could be up to several hundred MB or even serval GB. When parameter/dataset is exchanged via over-the-air, the overhead of model parameter exchanged from NW to UE is affordable for Option 3a-1, while the overhead of training dataset is too large for Option 4-1.

In order to address the issue, Rel-16 eType II codebook or high-resolution eType II codebook can be used to quantize the target CSI. It has shown that the performance loss is supportable according to the discussion during Rel-18.

Observation 7：When parameter/dataset is exchanged via over-the-air, the overhead of model parameter exchanged from NW to UE is affordable for Option 3a-1, while the overhead of training dataset is too large for Option 4-1.

Proposal 3: For Option 4-1, Rel-16 eType II codebook or high-resolution eType II codebook could be considered to transfer target CSI for addressing the overhead issue.

- Issue 4: Is there performance impact due to mismatch between NW side data distribution and UE side data distribution, and if so, how to address it?

In the last meeting, the evaluation method on mismatch between NW side data distribution and side data distribution were provided. In order to denote the data mismatch, the following two different phase normalization approach are adopted.

- Option1: The first element of each vector is real, i.e., each element is multiplied by e1-jp, where p is the phase of the first element.

- Option2: The n-th subband eigenvector en is multiplied by  enen-jp, where p is the phase of enHen-1. For the first subband eigenvector, e1=en.

Let Dataset A and Dataset B respectively denote the NW side data distribution and UE side data distribution. Dataset A and Dataset B are respectively generated by using Option 1 and Option2. The simulation parameters are given in Table A1 in the appendix. For simplicity, one layer transmission is assumed in the simulation. For Option3-1, the following four options are considered to train actual encoder at UE side:

- Option 3a-1 (Alt 1a): Both encoder and nominal decoder are trained by using UE measured dataset.

- Option 3a-1 (Alt 1b): Both encoder and nominal decoder are trained by using NW’s dataset

- Option 3a-1 (Alt 1c): Nominal decoder is trained by using NW’s dataset while encoder is trained by using UE measured dataset.

For Option 4-1, the following four options are considered to train actual encoder at UE side:

- Option 4-1 (Alt 1a): both nominal decoder and encoder are trained by using the exchanged dataset from NW side

- Option 4-1 (Alt 1b): Nominal decoder is trained by using NW’s dataset while encoder is trained by using UE measured dataset.

- Option 4-1 (Alt 2a): UE-side directly trains the actual encoder by using the exchanged dataset from NW side.

- Option 4-1 (Alt 2b): UE-side directly trains the actual encoder by using UE measured dataset.

The simulation results are provided in Table 8. In the table, the case ‘Upper bound’ denotes that the training data distribution is consistence with the test data distribution.

Table 8： Simulation results on mismatch between NW side data distribution and UE side data distribution for Option 3a-1 and Option 4-1

| Overhead Options | 280 bits | 120 bits | 60 bits |
| --- | --- | --- | --- |
| Upper bound | 0.8912 | 0.8103 | 0.7515 |
| Option 3a-1 (Alt 1a) | 0.8874(-0.4%) | 0.8014(-1.10%) | 0.7449(-0.88%) |
| Option 3a-1 (Alt 1b) | 0.8864(-0.54%) | 0.7901(-2.50%) | 0.7384(-1.74%) |
| Option 3a-1 (Alt 1c) | 0.8893(-0.21%) | 0.8021(-1.01%) | 0.7435(-1.06%) |
| Option 4-1 (Alt 1a) | 0.8851(-0.68%) | 0.7905(-2.44%) | 0.7409(-1.41%) |
| Option 4-1 (Alt 1b) | 0.8870(-0.47%) | 0.8011(-1.14%) | 0.7450(-0.86%) |
| Option 4-1 (Alt 2a) | 0.8785(-1.43%) | 0.7903(-2.47%) | 0.7409(-1.41%) |
| Option 4-1 (Alt 2b) | 0.8709(-2.28%) | 0.7835(-3.31%) | 0.7327(-2.50%) |

For Option 3a-1, we can observe that the performance of Alt 1a and Alt 1c is larger than that of Alt 1b. The performance of Alt 1a and Alt 1c is similar, and the Alt 1a is close to the upper bound. This implies that there is no much performance loss even without using NW’s dataset for training.

For Option 4-1, it can be observed that the model performance of Option 4-1 with normal decoder training is better than that of Option 4-1 without normal decoder training. Compared with Alt 1b, Alt1a could achieve better performance. It means that UE’s dataset for model training could help to improve the performance of model.

Observation 8: For Option 3a-1, the training dataset mismatch between UE side and NW side does not bring obvious performance loss.

Observation 9: For Option 4-1, model performance could be further improved if UE measurement dataset is used to train model.

Proposal 4: Sharing NW side training dataset to UE is not needed because mismatch data between UE side and NW side does not bring obvious performance loss for Option 3a-1.

Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (I.e., inter-vendor collaboration Option 3b)

- Issue 3: Is there an overhead concern, and if so, how to address it?

According to above agreement, the overhead is based on size of encoder, the number of encoder and the frequency of transferring parameters. As discussed for Option 3a-1, the size of model parameter is no more than 30 MB. The overhead of sharing encoder parameter is limited. If there are multiple different encoders and the parameters of encoders are frequently transferred, the overhead will be significantly increased. How to reduce the overhead needs to study. In order to make one model be appliable to a new scenario, the model could be updated or retrained with the dataset generated in the new scenario through fine-tuning. During updating or retraining the model, some parameters of partial network layers could be frozen, and the model performance is still acceptable. This implies that these parameters could be shared by multiple models. Such observation of model training could be used to reduce the overhead of model parameter exchanged. I.e., the parameters of partial neural network layer are fixed or specified, such that NW only delivers the parameters of remained neural network layers.

Proposal 5：For saving overhead of transferring model parameters, the parameters of partial neural network layers of encoder are transferred from NW-side to UE-side, while the parameters of remained neural network layers are standardized.

- Issue 5: Whether it is feasible to use a common encoder across UEs, and whether it is feasible for NW-side to train multiple encoders for different UEs?

In our view, whether a common encoder used across UEs depends on the generality or scalability of the encoder.  If the common encoder could achieve performance requirement for different users with different configurations or channel quality, it is feasible that the same encoder is used for these users. It has showed that localized model could further improve CSI compression performance due to similar channel character for different users. Regarding to such case, it is feasible to use the same encoder for them.

Multiple encoders for different users could be trained by NW if their datasets are obtained at the network. However, the motivation of training multiple encoders for different UEs needs to clarify. If the one encoder could achieve better generalized and/or scalable performance, it is not necessary to train multiple encoders.

Observation 10：It is feasible to use a common encoder across different UEs which located in the same area.

Observation 11：It is feasible to train multiple encoders for different UEs.

- Issue 7: Is there concern for NW’s and UE’s proprietary information disclosure, and if so, how to address it?

It was agreed that there is no NW’s proprietary information concern of disclosing such encoder parameters from NW side to UE side for Option 3a-1. For Option 3b, the encoder parameters exchanged to UE side from NW side is directly used for model inference. Similarly, there is also no NW’s proprietary information concern of disclosing encoder parameters for Option 3b.

Observation 12：For Option 3b, there is no NW’s proprietary information concern of disclosing encoder parameters from NW side to UE side.

Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (I.e., inter-vendor collaboration Option 1), including at least the following issues

- Issue 8: Whether to consider 3GPP’s statistical channel model or field data for reference model(s) training. In case of the latter, how does RAN1 collect field data and agree on them?

RAN4 has discussed the reference model training. At present, the synthetic data based on 3GPP’s statistical channel model is adopted to train the reference model training in RAN4. We think it is feasible and reasonable to use synthetic data to train reference model. If field data is used as training data, it will cost a lot of engineering efforts to collect data. In addition, the field scenario is complexity, which makes the collected dataset by different companies be much more diverging. The performance of model may be quite divergent as well, such that it is difficult to achieve common understanding on reference model. However, the synthetic data could avoid such obstacle since the divergence of dataset based on 3GPP’s statistical channel model is not too large. If the synthetic data is regarded as training dataset, it is possible that the trained model is regarded as reference model.

Observation 13：The field data may be quite divergent due to complexity field scenario, and it will cost a lot of engineering efforts to collect field data.

Proposal 6：Suggest the collected dataset based on 3GPP’s statistical channel model for reference model training.

- Issue 10: What additional information should be specified to enable UE-side encoder training, validation, and testing, and NW-side decoder training, validation, and testing?

For Direction C, although model is specified, UE may still retrain or update the specified model based on collected dataset at UE side. Similar to Direction A, associated ID is required to capture the NW side additional condition for the retrained AI model. In order to ensure the retained model to satisfy the required performance, performance target should be shared to UE as well.

Proposal 7: At least the following additional information is shared to enable UE-side encoder training, validation, and testing at UE side for Direction C:

- Associated ID

- Performance target

Specification impact for inter-vendor collaboration options

In the RAN1#116 bis, RAN1#117 and RAN1#119 meeting, the following agreements or conclusions on inter-vendor training collaboration for two-sided model were identified.

Agreement - For option 3a/3b/4/5a and their sub-options, at least the following potential specification impacts have been identified. Further study the necessity, feasibility, their specification impact. - Exchange - Parameter / model exchange methods, format/contents, and related spec impacts (3a/3b/5a) - Dataset exchange methods, format/type/contents of data/dataset, and related spec impacts (4) - Additional information, if necessary, that may be shared from the NW-side to help UE-side offline engineering and provide performance guidance (3a/5a/4) - Performance target (3a/5a/4) - Dataset or information related to collecting dataset (3a/5a) - Any other additional information - Model pairing (3a/3b/4/5a) - UE capability (3a/3b/4/5a) - Model related aspects, such as scalability (e.g., payload sizes, antenna ports, bandwidth), rank and layer handling (3a/3b/4/5a) - Quantization of feedback (3a/3b/4/5a) - Model structure details (3a/3b) Note: Option 3a/4/5a and option 3b serve two different deployment time scales, UE capabilities, device-side optimizations, and training methods, and therefore may be complementary to each other, with potential specification of both. - Specification of option 1, if needed from RAN1, can reuse specification of option 3a/3b, with the additional specification of parameters. Agreement - For Direction A Option 3a-1 and Direction C, study the feasibility of scalable model structure specification over numbers of Tx ports, CSI feedback payload sizes, and bandwidths, number of slots. - Feasibility of Direction A Option 3a-1 is contingent on the feasibility of the scalable model structure specification. - Feasibility of Direction C is contingent on the feasibility of the scalable model structure specification and model parameters specification. - Note: Angular/Delay/Doppler domain conversion and quantization are considered to be part of feasibility study Agreement For studying the standardized model structure, - For temporal domain Case 0, - In case of spatial-frequency domain input, adopt Transformer as the backbone structure. - Further study angular-frequency and angular-delay domain input - For temporal domain Case 2, - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation with additional layers - Conv-LSTM - LSTM - Option 2: Latent adaptation on top of Case 0 model structure - Reusing Case 0 structure and changing quantization operation and/or feedback payload over different slots - Note: other options are not precluded - Further study angular-frequency and angular-delay domain input - For temporal domain Case 3, - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation - Option 2: Latent adaptation - Further study angular-frequency, angular-delay, and angular-delay-Doppler domain input. - Take precoding matrix in the spatial-frequency domain in a subband granularity as the input, and the reconstructed precoding matrix in the spatial-frequency as the output, for the study of standardized model structure. - Note: Processing steps at the UE to derive angular, delay, and/or Doppler domain basis, and corresponding reconstruction steps at the gNB are considered. - Note: scalar/vector quantization and dequantization are considered. Quantization-aware training with jointly updated quantization method/parameters (Case 2-2) is used.

According to above agreements, the necessity, feasibility and specification impact for parameter/dataset exchange, model pairing, UE capability, quantization of feedback and scalable model structure need to further study.

Parameter/dataset exchange

For Option 3a-1/3b, the exchanged content is parameters of encoder. Float 32 used as the format of parameter is studied as a starting point. For Option 4-1, the exchanged content includes target CSI and CSI feedback. The type of target CSI could precoding matrix in the spatial-frequency domain in a subband granularity as a starting point. Rel-16 eType II codebook or high resolution eType II codebook could be used as the format of target CSI. Float 32 could be as the format of CSI feedback. In RAN1#119 meeting, RAN1 has seen LS on the study of feasibility of exchanged parameter/dataset from NW side to UE side via over-the-air or other approaches.

Proposal 8: At least the following format/type could be studied as a starting point:

- Precoding matrix in the spatial-frequency domain for target CSI

- Float 32 for CSI feedback and parameter

Proposal 9: The feasibility of exchanged parameter/dataset via over-the-air or other approaches depends on the progress of RAN2 or other WGs.

Model pairing

In RAN1#112 meeting, six options on pairing information were provided to enable UE to select a CSI generation model that is compatible with the CSI reconstruction models used by gNB. The pairing information could be CSI generation model ID, CSI reconstruction model ID, paired ID, dataset ID or training session ID. For Option 3a-1 or 4-1, a different model may be retrained based on the transferred model, parameter or dataset at UE side. In order to make UE select a compatible CSI generation model with the CSI reconstruction model used at NW side, it is necessary to study model pairing. Although Option 3b does not need to retrain a model, the model pairing is also needed to ensure UE selecting a compatible CSI generation model. The pairing information could be assigned by UE or gNB. If UE assigns the pairing information to gNB, different UE vendors may assign different pairing information. This will increase the model management complexity of gNB. Hence, it is reasonable that the pairing information is assigned by gNB. The pairing information could be exchanged together with model/parameter/dataset to reduce signaling overhead.

Proposal 10: In order to make the two-sided model be compatible, the pairing information for model pairing is assigned by gNB with parameter/dataset exchange for Option 3a-1/4-1.

UE capability

For Options 3a-1, 3b or 4-1, if the model could be retrained on-device. UE capability is different for the three options. Option 3a-1 and Option 4-1 require higher UE capability due to model training at UE side. While Option 3b does not have the requirement on UE capability since it does not need to retain a model.  Even the model is retrained on-device, UE capability may be different as well for Option 3a-1 and Option 4-1. Considering the different inter-vendor training collaboration options require different UE capabilities, it is necessary to report UE capability to NW side for determining one collaboration training option if multiple inter-vendor collaboration options are supported.

Proposal 11: The supported inter-vendor collaboration options are indicated via UE capability reporting if multiple inter-vendor collaboration options are supported.

Quantization of CSI feedback

There are different quantization approaches for CSI feedback, such as scalar quantization, vector quantization or non-uniform quantization and so on. Different UEs may adopt different quantization approaches to report CSI feedback. If quantization approach is not specified. UE needs to report quantization approach so that gNB adopts accurate dequantization. Different quantization approaches require gNB supports different dequantization. This will increase the complexity of dequantization processing at gNB side. The standardized quantization approach could address these issues.

Proposal 12: Quantization approach of CSI feedback needs to be specified for Option 3a-1/3b/4-1.

Model structure

For Option 3a and Option 1, model structure needs to specify. According to above agreements, Transformer is adopted as the backbone structure for Case 0. The model structures of Case 2 and Case 3 take into account the model structure of Case 0 with an adaptation. Hence, it is necessary to firstly study the model structure of Case 0.

In our view, the hyper parameters of model structure need to defined. Then, the values of those hyper parameters are provided by companies based on performance evaluation. At last, the hyper parameter and values of their of model structure are specified.

Proposal 13: The hyper-parameter of Transformer model structure for Case 0 should be firstly defined.

Model related aspects, such as scalability (e.g., payload sizes, antenna ports, bandwidth), rank and layer handling

For Option 3a-1 or 4-1, UE may retain a CSI generation part model based on the received parameter or dataset. Different vendors may train different models even receiving the same parameters or dataset. For example, some vendors may train a scalable model for different payload sizes or bandwidths. While other vendors may only train one model for a payload size or bandwidth. If NW does not obtain the new model related information, NW could not flexibly configurate the suitable parameters. Hence, it is necessary to report the model related aspects for Option 3a-1 or 4-1 if the scalable model is retrained at UE side.  For Option 3b, NW has known the model information. Model related information is not necessary to report.

Proposal 14: For Option 3a-1 or 4-1, the supported input/output model parameters, such as number of antenna ports, payload size, and so on, are reported to NW for indicating model scalability so that NW can flexibly configure parameters.

Performance monitoring

Performance monitoring methods

In RAN1#117 meeting, the following agreements on NW-side or UE-side performance monitoring were identified. The necessity and feasibility for NW-side and UE-side monitoring considering overhead, latency, complexity, monitoring accuracy and UE capability need to further study.

Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Option1: Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Option2: Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Option3: Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Option4: Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Option5: Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Regarding monitoring metrics: - Monitoring accuracy also includes generalization considerations, if applicable. - Complexity also includes LCM complexity, if applicable. - Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions. - Testability of UE reported metrics Discussion may include the following aspects: - Consideration of Options 1-5 and their sub-options for alleviating / resolving the issues related to inter-vendor training collaboration - Temporal domain aspects of CSI compression - How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches. Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches.

NW-side monitoring

For Case 1, NW monitors the performance of model based on the reported target CSI, where the format of target CSI is eType II codebook or high resolution eType II codebook. Such format could save a lot of feedback overhead.  The accuracy of monitoring is associated with the number of target CSI. This implies multiple target CSI reporting is needed. However, more reporting of target CSI will increase the latency of performance monitoring. The trade-off between monitoring accuracy and latency should be considered. Since eType II codebook is adopted to report target CSI, UE should have the capability of supporting eType II codebook. gNB could adopt the received target CSI to calculate intermediated KPI for performance monitoring. The accuracy of monitoring performance could be further evaluated.

For SRS-based monitoring, it does not consume large feedback overhead and require stronger calculation capability of UE. The monitoring accuracy may be associated with number of SRS transmission. More SRS transmissions or larger interval of adjacent SRS transmission incurs larger latency. The accuracy of monitoring performance needs to be evaluated as well.

UE-side monitoring

There are five options for UE-side monitoring. For Option1, UE monitors the model performance based on the output of the CSI reconstruction model at UE side. It does not need to report target CSI, which could significantly save feedback overhead. If the CSI reconstructed model used by UE is same as model of gNB, the monitoring accuracy could be ensured. Otherwise, the monitoring accuracy is not guaranteed due to generality/scalability of reference model or proxy model. The computation complexity is associated with the two-sided model used by UE. In addition. The introduced reference model or proxy model also results in increase of LCM complexity. The latency is dependent on the inference time of the CSI reconstruction model, reference model or proxy model. In addition to CSI generation part model deployed at UE side, CSI reconstruction part model or proxy model are deployed at UE side as well. This will increase the model storage memory of UE.

For Option2, model performance is monitored via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI. It does not need feedback overhead of reporting target CSI. The monitoring accuracy needs to evaluate. The complexity depends on the AI/ML model of intermediate KPI. The latency depends on the number of estimations of KPI and inference time of AI/ML model. This option requires UE have the capability of estimation intermediate KPI. Option 3 is similar with Option2. The difference of them is that the estimation contents are different.

For Option4, it does not require too much downlink signalling overhead by using precoded RS. The monitoring accuracy needs to evaluate. In generally, UE has the ability of estimation of precoded RS. Additional UE capability is not needed. The channel estimation complexity is also affordable to UE. In order to obtain accurate performance monitoring, multiple precoded RSs are needed to transfer from NW side. Hence, the monitoring latency depends on the number of precoded RS.

For Option5, it will consume more downlink overhead by transferring the output of the CSI reconstruction model compared with Option 4. As above discussion, since the output of the CSI reconstruction model could be represented by eT2 codebook or eT2-like high-resolution codebook, such feedback overhead could be further reduced. The monitoring accuracy needs to evaluate. This option also requires that UE has the capability of supporting eType II-like codebook. Similar with other options, the latency depends on the number of outputs of CSI reconstruction model. Based on above discussion, we have the following observation.

Observation 14: For different options of model performance monitoring, our views on the feasibility of them are provided in the following table considering the overhead, latency, complexity, monitoring accuracy and UE capability.

| Monitoring option |  | Overhead | Latency | Complexity | Monitoring accuracy | UE capability |
| --- | --- | --- | --- | --- | --- | --- |
| NW-side monitoring | Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook | More | Depends on the number of reported target CSI | Higher | Needs to evaluate | Require to support eT2 codebook |
|  | SRS-based monitoring | Less | Depends on the number and interval of SRS transmission | Lower |  | Do not require additional capability |
| UE-side monitoring | Option1: Based on the output of the CSI reconstruction model at the UE | Less | Depends on the number of output | Depends on the used model and increase LCM complexity |  | Require to support multiple models |
|  | Option2: Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI | Less | Depends on the number of estimation of KPI | Depends on the estimation algorithm |  | Require to support the estimation algorithm of KPI |
|  | Option3: Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI | Less | Depends on the number of estimation of monitoring output | Depends on the estimation algorithm |  | Require to support the estimation algorithm of monitoring output |
|  | Option4: Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model | Less | Depends on the number of precoded RS and interval of adjacent precoded RS. | Lower |  | Do not require additional capability |
|  | Option5: Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook | More | Depends on the number of output of CSI reconstruction model | Higher |  | Require to support eT2 codebook |

Considering the pros and cons of different performance monitoring methods, the following proposal is given.

Proposal 15: NW-side monitoring model performance based on target CSI reported by UE and UE-side monitoring model performance based on precoded RS are studied with high priority.

Identifying root cause of performance degradation

In RAN1#117 meeting, the study mechanisms for identifying the cause of the performance degradation needs to further study.

Agreement For option 1 / 3 / 4 / 5 and their sub-options, study mechanisms (e.g., post-deployment performance monitoring) for identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation to guarantee good performance in the field.

In the above agreement, the performance degradation may be caused by NW side, UE side, data drift or others. The root cause of performance degradation could be identified by using UE-side or NW-side monitoring.

- Identify the cause of performance degradation based on UE-side performance monitoring

Assume that encoder and decoder have been trained at NW-side. If performance monitoring based on output of the decoder trained at NW-side, the decoder will be transferred to UE, which discloses the NW-side model property. For proxy model, direct estimation of intermediate KPI or estimation of monitoring output to monitor performance, the performance degradation may be caused by the proxy model or AI/ML model estimation of KPI or monitoring output. The root causes of performance degradation could not found. For precoded RS to monitor performance, the performance degradation may be caused by encoder, decoder or data drift, it is also difficult to tell the causes of performance degradation. For performance monitoring based on the output of the decoder at NW-side, two CSI feedback inferred by encoder trained at NW-side and retained at UE-side are respectively to reported. Then, NW-side transfers the double output of decoder to UE based on the twice CSI feedback. This incurs larger downlink and uplink signaling overhead.

Proposal 16: Considering the accuracy of monitoring and signalling overhead, identifying the cause of performance degradation based on UE-side performance monitoring is studied with low priority.

- Identify the cause of performance degradation based on NW-side performance monitoring:

Assume encoder and decoder trained at NW-side are respectively denoted as E1 and D1.  Let UE report the measured target CSI and CSI feedback to NW side. NW could calculate the first intermediate KPI (denoted as K1) based on received CSI feedback and target CSI. Then, NW also calculate the second intermediate KPI (denoted as K2) based the target CSI and CSI feedback generated at NW-side based on E1 at NW side. Let δ denote performance threshold. If K1< δ and K2 ≥δ, it implies that UE-side issue cause performance degradation. If both K1 and K2 are smaller than the threshold, this means the performance degradation may be caused by data drift or the encoder/decoder trained by NW-side (i.e., NW-side issue).

Assume that UE retrains the encoder as E2. Similarly, NW will calculate the third intermediate KPI (denoted as K3) based on CSI feedback inferred by E2 and target CSI reported by UE-side. If K3< δ and K2 ≥δ, it implies that the performance degradation is caused by the retrained at UE-side. If both K3 and K2 are smaller than the threshold, it also means that performance degradation is caused by data drift or encoder/decoder trained at NW-side. Since NW side has the training dataset, the reported target CSI and training dataset are compared to determine whether the issue is data drift or the trained encoder/decoder at NW-side. Hence, the root cause of performance degradation could be found. Based on above discussion, we think it is feasible that NW-side identify the issues of performance degradation according to the target CSI and CSI feedback reported by UE.

Proposal 17:  In order to identify the cause of the performance degradation, both target CSI and CSI feedback measured by UE are reported for NW-side performance monitoring.

Model inference

Data collection

The following agreements on data collection were achieved in RAN1#112 and RAN1#119 meeting. Dataset for training or performance monitoring could be collected at UE or NW side. The remained issues on data collection are further discussed.

Agreement UE side data collection: In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least - Enhancement of CSI-RS configuration to enable higher accuracy measurement. - Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc. - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. - Signaling for triggering the data collection NW side data collection: In CSI compression using two-sided model use case, further discuss the necessity, feasibility, and potential specification impact for NW side data collection including at least: - Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement. - Contents of the ground-truth CSI including: - Data sample type, e.g., precoding matrix, channel matrix etc. - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like). - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator) - Latency requirement for data collection - Signaling for triggering the data collection Agreement In CSI compression using two-sided model use case, further study the necessity, complexity, overhead, latency and potential specification impact on ground truth CSI report for NW side data collection for model performance monitoring, including: - Scalar quantization for ground-truth CSI - FFS: any processing applied to the ground-truth CSI before scalar quantization - Codebook-based quantization for ground-truth CSI - FFS: Parameter set enhancement of existing eType II codebook, based on evaluation results in 9.2.2.1 - RRC signaling and/or L1 signaling procedure to enable fast identification of AI/ML model performance - Aperiodic/semi-persistent or periodic ground-truth CSI report. Agreement For NW to collect data for training, study following spec impacts - Data format: codebook-based Rel-16 eType2 or Rel-18 eType2 for PMI prediction. - FFS if enhancement is needed - FFS number of samples in the report. - FFS whether channel or precoder is needed for temporal Cases 3 - Configuration of rank/layer, number of subbands - Mechanism for ground-truth reporting - FFS: Report additional information regarding the samples, e.g., data quality, FFS the definition of data quality and corresponding parameters. - FFS if enhancements in CSI-RS and SRS configuration is needed. - FFS: Report associated information that captures UE side additional condition - FFS: Configuration / reporting of temporal aspects for temporal Case 2 and Case 3, e.g., association between input and output CSI - FFS: details of CSI measurement For UE to collect data for training, study following spec impacts - NW configuration or UE request, e.g., RS configuration/transmission for data collection - Whether enhancements in CSI-RS configuration is needed. - Configuration of temporal aspects for temporal case 2/3, e.g., association between input and output CSI - FFS: Need of configuration of ID, and configuration of ID

For NW side data collection, the ground truth CSI should be reported to NW from UE. Both scalar quantization and codebook-based quantization could be used for ground-truth CSI. Compared with scalar quantization, codebook-based quantization could save a lot of overhead. At the same time, there is no much performance loss by using enhanced eType II codebook parameters, as given in TR 38.843 [2].

In current specification, CSI-RS resource is UE-specific reference signal. This means that CSI-RS resource is configured for each user. For network side data collection, this will consume a lot of signalling overhead. In addition, the configured CSI-RS resource per UE may be different. In order to avoid additional pre-processing of training data, dataset dimension of ground truth CSI should be same as far as possible. E.g., the number of CSI-RS resource ports is same for all users. The straightforward way is that CSI-RS resource is configured per cell. It can not only reduce signalling overhead, but also make the dataset dimension of ground truth CSI be same.

For temporal Case 3, the collected dataset should be associated with UE condition, since the configuration for collecting dataset depends on the channel variation which is caused by UE’s speed. Hence, the information of channel variation should be reported to NW as an associated information. In order to obtain the accurate information of channel variation, the TDCP (time domain channel property) introduced in Re-18 could be reported by UE. NW could configure suitable temporal parameters to collect training dataset according to the received TDCP.

The rank/layer or number of subbands could be configured to UE for collecting dataset. However, these configuration parameters have been supported in current specification. We think there is no spec impact since legacy configuration could be reused.

Observation 15: Current specification has supported the configuration of rank and number of subbands.

Regarding to the method of reporting ground truth CSI, we think that aperiodically/semi-persistently or periodically CSI reporting via L1 or RRC signalling is sufficien.

Proposal 18: For NW side data collection for training, the following aspects could be considered:

- Cell-specific CSI-RS resource is configurated by NW-side to collect training dataset.

- Associated information, such as TDCP, is reported to capture the UE side condition at least for Case 3.

- Ground truth CSI could be aperiodically/semi-persistently or periodically reported by using L1 or RRC signalling.

For UE-side data collection, NW configuration or UE request could be considered to collect dataset at UE side. Which way adopted depends on the two-sided model trained at which side. If two-sided model is trained at UE side, it is preferred to collect dataset triggered by UE. Otherwise, it is suitable to collect dataset via NW configuration. Since we have agreed that Option 3a-1 and Option 4-1 are studied with high priority, Considering some NW-side conditions belongs to the NW’s side property information, this information explicitly indicated to UE is not feasible. Associated ID could be introduced to implicitly indicate the NW-side additional condition.

Proposal 19: For UE side data collection for training, the associated ID needs to be configured for indicating NW side additional condition.

CSI reporting

For CQI calculation, different options on CQI calculation were agreed and given in the following [7].

Agreement In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

For Option 1a, the calculated CQI is over-optimization if CQI is calculated based on target CSI with realistic channel measurement. The reason is that the inference CSI at gNB is not same target CSI. There is some channel information loss due to CSI compression loss.  For Option 1b, if CQI is adjusted based on assistance of NW indication, some indication overhead will be required. In addition, the accuracy of CQI cannot be guaranteed since NW does not obtain raw channel information. For Option 1c, if CQI is calculated by using traditional codebook, e.g., eType II codebook, the computation complexity is significantly increased compared with other alternatives. The calculated CQI based on traditional codebook may be underrated, since channel information loss incurred by traditional codebook is much more than inference CSI at NW side.

For Option 2a, CSI reconstruction model need to be deployed at UE side. Model transfer or deliver will be involved in. If model transfer or deliver is not supported. This alternative is not feasible. For Option 2b, the gNB transmits beamformed CSI-RS at first stage, where the beam is inferred PMI via AI/ML-based CSI reconstruction part, i.e., decoder, at gNB side. At the second stage, RI and/or CQI is calculated by UE through the received beamformed CSI-RS. This procedure is similar to non-PMI feedback in current specification. In Figure 2, the processing procedure for such use case is given to illustrate RI, PMI and CQI calculation for Option 2b. The calculated RI/CQI can match the PMI, which avoids performance loss incurred by the mismatch between RI/CQI and PMI. Therefore, Option 2b, i.e., CQI is calculated using two stage approach should be supported. I.e., UE derives CQI using precoded CSI-RS transmitted with a reconstructed precoder.

Figure 2: The processing procedure of sub-use case on improved accuracy for separately calculating RI, PMI and CQI at UE side

Proposal 20: Support CQI calculated using two stage approach.

Conclusions

In this contribution, the proposals and observations on two-side AI model based CSI compression feedback are summarised as follows:

Performance evaluation of two-sided AI/ML model

Observation 1: For Scenario A, the SGCS AF-TSF performance gain over eType II codebook and AI-SF for different feedback payload is shown as follows:

- When the feedback payload is lower than 80 bits, the AI-TSF performance gain over eTypeII codebook is around 13.71% while the gain over AI-SF compression is around 8.9%.

- When the feedback payload is between 100 bits and 140 bits, the AI-TSF performance gain over eTypeII codebook is around 11.71% while the gain over AI-SF compression is around 9.07%.

- When the feedback payload is larger than 230 bits, the AI-TSF performance gain over eTypeII codebook is around 7.54% while the gain over AI-SF compression is around 6.78%.

Observation 2: For Scenario B, the AF-TSF performance gain over eType II codebook and AI-SF for different feedback payload is shown as follows:

- When the feedback payload is lower than 80 bits, the AI-TSF performance gain over eTypeII codebook is around 9.08% while the gain over AI-SF compression is around 3.92%.

- When the feedback payload is between 100 bits and 140 bits, the AI-TSF performance gain over eTypeII codebook is around 8.00% while the gain over AI-SF compression is around 4.78%.

- When the feedback payload is larger than 230 bits, the AI-TSF performance gain over eTypeII codebook is around 4.07% while the gain over AI-SF compression is around 2.57%.

Observation 3: For Scenario C, the AF-TSF performance gain over eType II codebook and AI-SF for different feedback payload is shown as follows:

- When the feedback payload is lower than 80 bits, the AI-TSF performance gain over eTypeII codebook is around 10.00% while the gain over AI-SF compression is around 4.80%.

- When the feedback payload is between 100 bits and 140 bits, the AI-TSF performance gain over eTypeII codebook is around 8.77% while the gain over AI-SF compression is around 5.53%.

- When the feedback payload is larger than 230 bits, the AI-TSF performance gain over eTypeII codebook is around 4.74% while the gain over AI-SF compression is around 3.23%.

Observation 4: Compared with Scenario B, additional around 1% performance gain could be obtained for Scenario C.

Observation 5: Regarding to UE distribution 100% outdoor (30km/h), the SGCS gain of case 3 over Rel-18 Type II Doppler codebook for different feedback payload is shown as follows：

- When the feedback payload is lower than 80 bits, the SGCS gain of case 3 over benchmark is around 39.76% for layer 1.

- When the feedback payload is between 100 bits and 140 bits, the SGCS gain of case 3 over benchmark is around 20.84% for layer 1.

- When the feedback payload is larger than 230 bits, the SGCS gain of case 3 over benchmark is around 24.08% for layer 1.

Observation 6: Regarding to UE distribution 80% indoor (3km/h),20% outdoor(30km/h), the SGCS gain of case 3 over Rel-18 Type II Doppler codebook for different feedback payload is shown as follows：

- When the feedback payload is lower than 80 bits, the SGCS gain of case 3 over benchmark is around 25.26% for layer 1.

- When the feedback payload is between 100 bits and 140 bits, the SGCS gain of case 3 over benchmark is around 28.60% for layer 1.

- When the feedback payload is larger than 230 bits, the SGCS gain of case 3 over benchmark is around 12.26% for layer 1.

Proposal 1: To align the missed historical CSI information when UCI loss occurs, the latest output historic CSI of encoder and decoder before missing UCI could be respectively adopted as the input of encoder and decoder in the current inference instance.

Inter-vendor training collaboration

Observation 7：When parameter/dataset is exchanged via over-the-air, the overhead of model parameter exchanged from NW to UE is affordable for Option 3a-1, while the overhead of training dataset is too large for Option 4-1.

Observation 8: For Option 3a-1, the training dataset mismatch between UE side and NW side does not bring obvious performance loss.

Observation 9: For Option 4-1, model performance could be further improved if UE measurement dataset is used to train model.

Observation 10：It is feasible to use a common encoder across different UEs which located in the same area.

Observation 11：It is feasible to train multiple encoders for different UEs.

Observation 12：For Option 3b, there is no NW’s proprietary information concern of disclosing encoder parameters from NW side to UE side.

Observation 13：The field data may be quite divergent due to complexity field scenario, and it will cost a lot of engineering efforts to collect field data.

Proposal 2: At least the following additional information is shared to enable UE-side encoder training, validation, and testing for Direction A:

- Associated ID

- Performance target

Proposal 3: For Option 4-1, Rel-16 eType II codebook or high-resolution eType II codebook could be considered to transfer target CSI for addressing the overhead issue.

Proposal 4: Sharing NW side training dataset to UE is not needed because mismatch data between UE side and NW side does not bring obvious performance loss for Option 3a-1.

Proposal 5：For saving overhead of transferring model parameters, the parameters of partial neural network layers of encoder are transferred from NW-side to UE-side, while the parameters of remained neural network layers are standardized.

Proposal 6：Suggest the collected dataset based on 3GPP’s statistical channel model for reference model training.

Proposal 7: At least the following additional information is shared to enable UE-side encoder training, validation, and testing at UE side for Direction C:

- Associated ID

- Performance target

Proposal 8: At least the following format/type could be studied as a starting point:

- Precoding matrix in the spatial-frequency domain for target CSI

- Float 32 for CSI feedback and parameter

Proposal 9: The feasibility of exchanged parameter/dataset via over-the-air or other approaches depends on the progress of RAN2 or other WGs.

Proposal 10: In order to make the two-sided model be compatible, the pairing information for model pairing is assigned by gNB with parameter/dataset exchange for Option 3a-1/4-1.

Proposal 11: The supported inter-vendor collaboration options are indicated via UE capability reporting if multiple inter-vendor collaboration options are supported.

Proposal 12: Quantization approach of CSI feedback needs to be specified for Option 3a-1/3b/4-1.

Proposal 13: The hyper-parameter of Transformer model structure for Case 0 should be firstly defined.

Proposal 14: For Option 3a-1 or 4-1, the supported input/output model parameters, such as number of antenna ports, payload size, and so on, are reported to NW for indicating model scalability so that NW can flexibly configure parameters.

Performance monitoring

Observation 14: For different options of model performance monitoring, our views on the feasibility of them are provided in the following table considering the overhead, latency, complexity, monitoring accuracy and UE capability.

| Monitoring option |  | Overhead | Latency | Complexity | Monitoring accuracy | UE capability |
| --- | --- | --- | --- | --- | --- | --- |
| NW-side monitoring | Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook | More | Depends on the number of reported target CSI | Higher | Needs to evaluate | Require to support eT2 codebook |
|  | SRS-based monitoring | Less | Depends on the number and interval of SRS transmission | Lower |  | Do not require additional capability |
| UE-side monitoring | Option1: Based on the output of the CSI reconstruction model at the UE | Less | Depends on the number of output | Depends on the used model and increase LCM complexity |  | Require to support multiple models |
|  | Option2: Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI | Less | Depends on the number of estimation of KPI | Depends on the estimation algorithm |  | Require to support the estimation algorithm of KPI |
|  | Option3: Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI | Less | Depends on the number of estimation of monitoring output | Depends on the estimation algorithm |  | Require to support the estimation algorithm of monitoring output |
|  | Option4: Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model | Less | Depends on the number of precoded RS and interval of adjacent precoded RS. | Lower |  | Do not require additional capability |
|  | Option5: Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook | More | Depends on the number of output of CSI reconstruction model | Higher |  | Require to support eT2 codebook |

Proposal 15: NW-side monitoring model performance based on target CSI reported by UE and UE-side monitoring model performance based on precoded RS are studied with high priority.

Proposal 16: Considering the accuracy of monitoring and signalling overhead, identifying the cause of performance degradation based on UE-side performance monitoring is studied with low priority.

Proposal 17:  In order to identify the cause of the performance degradation, both target CSI and CSI feedback measured by UE are reported for NW-side performance monitoring.

Model inference

Observation 15: Current specification has supported the configuration of rank and number of subbands.

Proposal 18: For NW side data collection for training, the following aspects could be considered:

- Cell-specific CSI-RS resource is configurated by NW-side to collect training dataset.

- Associated information, such as TDCP, is reported to capture the UE side condition at least for Case 3.

- Ground truth CSI could be aperiodically/semi-persistently or periodically reported by using L1 or RRC signalling.

Proposal 19: For UE side data collection for training, the associated ID needs to be configured for indicating NW side additional condition.

Proposal 20: Support CQI calculated using two stage approach.

References

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, Sep. 9-12, 2024.

- TR 38.843, V1.3.0 (2023.11).

Appendix

Table A1： Simulation assumption

| Parameter | Value |
| --- | --- |
| Scenario | UMa |
| Frequency Range | 2GHz |
| Bandwidth | 10MHz |
| Subcarrier spacing | 15KHz |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| Dataset type | Eigenvector (layer1, layer2) |
| Channel estimation | Ideal |
| UE distribution | 80% indoor (3km/h), 20% outdoor (30km/h) |

Table A2：Parameter of AI model training

| AI training parameter | Value |
| --- | --- |
| Quantization | Scalar quantization |
| Loss function | SGCS |
| Learning rate | 0.0001 |
| Optimizer | Adam |
| Epoch | 200 |
| Batch size | 128 |