3GPP TSG RAN WG1 #120			R1-2500798

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.8.1

Source:	Apple

Title:	On Validation of Channel Model

Document for:	Discussion/Decision

# Introduction

In RAN #102 meeting, the SID of Release 19 NR channel modelling enhancements for 7-24 GHz was agreed [1]. One objective of the SID is validating the channel model of TR 38.901 at least for 7-24 GHz using measurements. In RAN1 #119 meeting [2], the following observation, conclusions and agreements were made.

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- Building type ratio: 90% residential, 10% commercial

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- UT Vertical Distribution Option 2: uniform distribution across all floors for a building type

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- UT mobility in horizontal plane only: Indoor UTs: 3 km/h, outdoor UTs: 40 km/h

Agreement

For handheld devices adopt the following device dimensions for UE antenna modeling:

- 15 cm x 7 cm x 0 cm

Conclusion

For any new channel modeling parameter updates, replace the existing parameters values (whenever applicable). For parameters that were replaced TR should add information that the parameter value/model equation/procedure was changed in Rel-19.

- Note: If deemed important, it should be still possible to discuss how specific channel model parameters need to be updated.

Conclusion

Based on measurement data provided, RAN1 concludes pathloss models in TR38.901 for the following scenarios are validated and no updates to TR are made.

- InH-Office LOS

- InH-Office NLOS

- RMa LOS

- RMa NLOS

- InF LOS

- InF NLOS

Observation

For the following scenarios, some sources have observed some differences in delay spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed delay spread consistent with the model in the TR.

- InF LOS

- InF NLOS

Conclusion

For the following scenarios, there is no consensus to update delay models due to lack of consistent and significant observed difference between model and measurements.

- InF LOS

- InF NLOS

Agreement

Study further on the need for introduction of intra-cluster power profile.

- Potential options for introduction of intra-cluster power profile:

  - Introduce unequal power per ray within a cluster and change the unequal angle offset to equal angle offset among rays within a cluster to maintain the same PAS characteristic as in TR 38.901 (example in R1-2409724)

  - Power scaling within a cluster based on DS associated with the cluster (example in R1-2410213)

Agreement

Further study penetration loss modeling, including the following aspects:

- Need for change and details of the updates to the material penetration loss, for example, if any

|  | Concrete | Wood |
| --- | --- | --- |
| Existing TR | Lconcrete=5+4f | Lwood=4.85+0.12f |
| New Updates | Lconcrete=α∙(5+4f) α=Tconcrete23cm | Lwood=α∙(4.85+0.12f) α=Twood6cm |
| Tconcrete and Twood represent reference (default) thickness of the material. For concrete and wood, the reference thickness are 23 cm and 6 cm, respectively. |  |  |

  - Whether penetration loss can be scaled linearly with thickness for each material

  - For standard multi-pane glass, FFS whether and how to reflect the penetration loss resonation effect, which may relate to thickness of the glass.

  - FFS the impact of updates to material penetration loss to total build penetration loss

- Need for change and details of total building penetration loss, e.g. low-loss and high-loss model, if any

- Impact of material composition on material penetration loss

Agreement

For the following scenarios, further study path loss model parameters for at least 6 – 24 GHz frequency range.

- UMi LOS

- UMi NLOS

- UMa LOS

- UMa NLOS

Agreement

For the following scenarios, further study delay spread parameters for at least 6 – 24 GHz frequency range.

- UMi LOS

- UMi NLOS

- InH-Office LOS

- InH-Office NLOS

- UMa LOS

- UMa NLOS

Agreement

Based on measurement data provided, RAN1 concludes angular spread parameters in TR38.901 that following scenarios are validated and no updates to TR are made.

- UMa LOS: ZSD

- UMa NLOS: ZSD

Observation

For the following scenarios, some sources have observed some differences in angular spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed angular spread consistent with the model in the TR.

- InH LOS and NLOS: ZSA, ZSD

- UMi LOS: ZSA, ZSD

- UMi NLOS: ZSD

- UMa LOS: ZSA

Conclusion

For the following scenarios, there is no consensus to update angular spread models due to lack of consistent and significant observed difference between model and measurements.

- InH LOS and NLOS: ZSA, ZSD

- UMi LOS: ZSA, ZSD

- UMi NLOS: ZSD

- UMa LOS: ZSA

Agreement

For the following scenarios, further study angular spread parameters for at least 6 – 24 GHz frequency range.

- InH LOS/NLOS: ASA, ASD

- UMi LOS: ASA, ASD

- UMi NLOS: ASA, ASD, ZSA

- UMa LOS: ASD, ASA

- UMa NLOS: ASA, ASD, ZSA

- InF LOS and NLOS: ASD, ASA, ZSA, ZSD

Agreement

Further study number of clusters for existing deployment scenarios (InH, InF, UMi, UMa, RMa) taking into consideration the following:

- Consistency/continuity across frequencies below 6GHz and above 24 GHz, including whether to update values for other frequencies based on measurements

- Whether changes to number of clusters result in meaningful impact to system performances and evaluation of features

Conclusion

Based on measurement data provided, RAN1 concludes shadow fading parameters in TR38.901for the following scenarios are validated and no updates to TR are made.

- InH LOS

- InH NLOS

- UMi NLOS

Observation

For the following scenarios, some sources have observed some differences in shadow fading in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed shadow fading with the model in the TR.

- UMi LOS

Conclusion

For the following scenarios, there is no consensus to update shadow fading due to lack of consistent and significant observed difference between model and measurements.

- UMi LOS

Agreement

Further study the need for updating the K-factor parameters for existing deployment scenarios (InH, InF, UMi, UMa, RMa) taking into consideration the following:

- Consistency/continuity across frequencies below 6 GHz and above 24 GHz, including whether to update values for other frequencies based on measurements.

- Whether changes to K-Factor result in meaningful impact to system performances and evaluation of features.

Working Assumption

- Adopt the following absolute delay for UMi and UMa scenarios.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | UMi | UMa |
| --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.4 |
|  | σlgΔτ | 0.5 | 0.2 |
| Correlation distance in the horizontal plane [m] |  | 15 | 50 |

Agreement

- Introduce absolute delay for InH scenarios.

  - Absolute delay is enabled by re-using the absolute time of arrival for InF scenarios.

  - FFS details of the parameter for InH. Consider the following for further study.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InH |
| --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | Option 1) -7.9 Option 2) -8.6 |
|  | σlgΔτ | Option 1) 0.3 Option 2) 0.1 |
| Correlation distance in the horizontal plane [m] |  | 10 |

Agreement

- To address the angle-scaling issue when generating CDL channels for link-level evaluations, adopt the following two-step approach to compute the scaled angles.

  - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired,

  - μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A.

  - WrapTo180(⋅) is a function which wraps an azimuth angle to the half-open interval (-180, 180].

- Down-select among options.

  - Option 1)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - where s is a scale factor chosen to change the distribution of the angles (up to each company to provide information, e.g. s=ASdesiredASmodel).

  - Option 2a)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s=minx≥0{x: ASx=ASdesired}.

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 2b)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s is chosen such that AS(ϕn,intermediate) = AS_desired

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 3)

    - ϕn,intermediate=ASdesiredASmodel* WrapTo180ϕn,model-μϕ,model,

    - Note: The resulting scaled ray angle may or may not achieve the desired angle mean and the desired rms angle spread.

- FFS whether and how to enable specular rays of CDL-D and CDL-E to be in the direction of the desired vector.

Agreement

For suburban scenario, further consider following options and down-select a value for BS height assumption for calibration purposes:

- Option 1) BS height = 25m

- Option 2) BS height = 35m

Note: It should be understood that BS height selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios may be applicable for a range of BS heights [25 – 35] m.

Agreement

For suburban scenario, further consider following options and down-select a value for ISD assumption for calibration purposes:

- Option 1) ISD = 1299 m

- Option 2) ISD = 1732 m

Note: It should be understood that ISD selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios is applicable for a range of ISD [1200 – 1800]m.

Agreement

For suburban scenario, further study the LOS probability equation.

- Example of LOS probability equation for study:

  - PrLOS=exp-d2D-out-dmexpα∙hBSβ+γhUEδ, where dm, α, β, γ, δ are modeling parameters

- Study includes whether distinction of LOS probability should be made for UEs in residential and commercial buildings, which have different building heights.

- Study includes whether or not vegetation should be explicitly considered and modeled for determining LOS probability, if determined to be explicitly modeled, how to model vegetation impact, e.g. reuse of InF clutter to change the LOS probability.

Agreement

Further study the need of additional low loss outdoor-to-indoor (O2I) building penetration loss model for suburban scenario.

- Potential example of the new low loss O2I building penetration loss:

PLtw=5-10log100.3⋅10-Lglass10+0.7⋅10-Lwood10

Agreement

- UMa can also be used for other ISDs between 200m and 500m, including the ISD proposed in ITU-R M.2412 -0

- One ISD value will be selected for calibration with ISD = 500m

- No change to any other parameters due to other ISD values

- FFS: How to reflect it in TR38.901

Agreement

Capture channel modeling updates regarding suburban macrocell scenario as follows:

- Define a new scenario, e.g., SMa

- The key differentiating aspects of SMa (compared to UMa and RMa) in high level description will be highlighted in Section “6.2 Scenario of Interest” of the TR.

Agreement

Further study device dimension and antenna placement applicability for customer premise equipment (CPE) devices, include the need for defining CPE device type dimensions.

Agreement

Further refine previous agreement (made in RAN1#118bis) on UE antenna radiation pattern modeling as follows:

- For UE antenna radiation pattern modeling, consider the following options for further study:

  - Option 1) Adapt the Table 7.3-1: Radiation power pattern of a single antenna element

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=FFS°,SLAV=FFS dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hϕ″=-min12ϕ″ϕ3dB2,Am,ϕ3dB=FFS°,Am=FFS dB |
| Combining method for 3D antenna element pattern (dB) | A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | FFS dBi |
| Note: max direction gain and half power beam widths should be selected such that total antenna efficiency should be equal or less than 0dBi. |  |

  - Option 2) half-wavelength dipole.

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: E(θ) = cos(cos(θ)*π /2) / sin(θ), where θ is the angle relative to the +Z-axis

      - FFS: range of θ

  - Option 2b) half-wavelength dipole

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: sin(π/2*cos(θ))

    - range of θ = -90° to +90°

  - Option 3) omni-directional antenna pattern

Agreement

For UE antenna modeling of handheld devices,

- Further study on default antenna placement locations for calibration purpose for 2, 3, 4, 6, and 8 antenna elements at least for FR1 and 6-24 GHz, including the need to define default antenna placement locations, and applicability of frequency ranges, e.g. FR1, FR2.

  - FFS on handling of antenna array panels

Agreement

For handheld UE device type antenna radiation pattern, further study details of antenna field pattern orientation with respect to the direction of antenna for each antenna candidate locations.

- For example, maximal gain directivity of the antenna radiation pattern to be aligned with direction of the antenna candidate location.

In this contribution, we provide our views on SMa scenario and provide measurement results on various parameters for channel model validation of TR 38.901 for 7-24 GHz.

# Discussion

The channel model for frequencies from 0.5 to 100 GHz was provided in TR 38.901 [3] at the beginning of 5G era. Since the spectrum used in 5G includes FR1 and FR2, the channel model in TR 38.901 was generated, mainly based on measured data with carrier frequency in sub-6 GHz and above 24 GHz. There were not many measurements performed in 7-24 GHz carrier frequency. Hence, the existing channel model defined in TR 38.901 needs to be validated for 7-24 GHz carrier frequency, using measurement data. We provide our views on SMa scenario and measurement results for the purpose of validating the existing channel model defined in TR 38.901.

## Suburban scenario

In RAN1 #118 meeting, it was agreed to enable necessary model to support sub-urban macro deployments scenario. The parameters for SMa scenario were discussed in RAN1 #118, RAN1 #118bis and RAN1 #119 meetings. Some parameters’ values are defined, while other parameter values have some options to down-select.

For the BS height, we think 25 m can be used for SMa scenario. This is because the building heights

in suburban area should be similar as those in urban area, and hence, the BS mounted above rooftop levels of surrounding buildings in SMa scenario should have the similar heights (i.e., 25 m) as those for UMa scenario. Following the BS height, we think the ISD of 1299 m should be used for SMa scenario. Correspondingly, the minimum BS – UT distance could be 25 m (i.e., Option 2).

Proposal 1: Consider the following parameters for suburban scenario for calibration purpose:

- Layout with ISD: 1299 m (Option 1)

- BS height: 25 m (Option 1)

- Min BS - UT distance (2D): 25 m (Option 2)

The LOS probability for SMa scenario has been discussed in RAN1 #118bis and RAN1 #119 meetings. It was agreed to further study LOS probability equation of PrLOS=exp-d2D-out-dmexpα∙hBSβ+γhUEδ. It is open on whether distinction of LOS probability should be made for UEs in residential and commercial buildings with different building heights. In our view, the LOS probability should be independent of building types for UEs in a building. The LOS probability formula already incorporates UE height, which is related to building types. It is open on whether vegetation should be explicitly considered in determining LOS probability. Note that vegetation is not explicitly considered in LOS probability in other scenarios. Hence, we do not think it is necessarily to be considered for SMa scenario.

Proposal 2: For SMa, the LOS probability is given by PrLOS=exp-d2D-out-dmexpα∙hBSβ+γhUEδ, which applies to UE in different building types and vegetation is not explicitly considered.

The pathloss formula for SMa scenario has been discussed in RAN1 #118bis and RAN1 #119 meetings. It was working assumption to have the pathloss formula for SMa (based on ITU M.2135-1). In our view, this working assumption can be confirmed. As per UE heights for the applicability of the pathloss formula, we think it should be extended to up to 13.5 meters, which is for a UE at the fifth floor of a commercial building. Also, the BS’s height of the applicability of the pathloss formula should be adjusted to the definition of SMa scenario.

Proposal 3: Confirm the pathloss formula for SMa (based on ITU M.2135-1), where it is applicable to UE height up to 13.5 meters.

It was agreed to study the need of additional low loss O2I building penetration loss for SMa, where a potential example is PLtw=5-10log100.3⋅10-Lglass10+0.7⋅10-Lwood10. A typical residential building in SMa scenario could be made of wood and glass, and the O2I penetration loss should reflect this material combination. Hence, we support to introduce additional low loss O2I building penetration loss for SMa.

Proposal 4: For SMa, the O2I penetration loss for residential buildings is given by PLtw=5-10log100.3⋅10-Lglass10+0.7⋅10-Lwood10.

## Measurement results

### Material penetration loss

The material penetration loss has been measured by companies. The spot-focused antennas are used for material penetration loss measurement for 7.5 GHz to 18 GHz, while lens horn antennas are used for material penetration loss measurement for 18 GHz to 30 GHz. This is shown in Figure 1.

Figure 1: Illustration of measurement environment

Wood

The wood in the measurement is with thickness of 1.75 cm. The measured penetration over different FR3 frequencies are shown in Figure 2. For comparison, we plot the wood penetration loss of TR 38.901 in the same figure.

It is observed from the figure that the measured penetration loss of wood with thickness of 1.75 cm is less than 4 dB below TR 38.901 at frequencies of 7.5 GH to 30 GHz. We think this gap of up to 3.5 dB is on the borderline, and RAN1 may decide whether or not to reduce wood penetration loss from TR 38.901.

Observation 1: The measured penetration loss of wood with thickness of 1.75 cm is less than 3.5 dB below TR 38.901 at frequencies of 7.5 GHz to 30 GHz.

Figure 2: Penetration loss of wood at frequencies of 7.5 GHz to 30 GHz

Concrete

The concrete in the measurement is with thickness of 4.7 cm. The measured penetration over different FR3 frequencies are shown in Figure 3. For comparison, we plot the concrete penetration loss of TR 38.901 in the same figure.

It is observed from the figure that the measured penetration loss of concrete with thickness of 4.7 cm is between 30 dB and 55 dB below TR 38.901 at frequencies of 7.5 GHz to 30 GHz.

Observation 2: The measured penetration loss of concrete with thickness of 4.7 cm is between 30 dB and 100 dB below TR 38.901 at frequencies of 7.5 GH to 30 GHz.

Figure 3: Penetration loss of concrete at frequencies of 7.5 GHz to 30 GHz

Based on this large gap, we think the material thickness should be taken into the account in material penetration loss. To facilitate this effort, another measurement is conducted with two layer concrete, where the two layer concrete is of thickness of 9.4 cm.

Figure 4 shows the measurement results for both one layer concrete with a thickness of 4.7 cm and two layers concrete with 9.4 cm. It can be seen in the figure that at 14 GHz as an example, the penetration loss of two layer concrete is almost double the penetration loss of one layer concrete. Subsequently, we could use the penetration loss per unit length model as Lconcrete(dB/cm) = 0.043+0.187 f. In an alternative way, as proposed in [4], the concrete penetration loss is given Lconcrete=α 5+4f, where α=Tconcrete23 cm and Tconcrete is the concrete thickness in cm.

Figure 4. Penetration loss of one and two layers concrete of 7.5 GHz to 18 GHz

IRR glass

The IRR glass in the measurement is of one layer with thickness of 0.55 cm. The measured penetration over different FR3 frequencies are shown in Figure 5. For comparison, we plot the IRR glass penetration loss of TR 38.901 in the same figure.

It is observed from the figure that the measured penetration loss of IRR glass of two-panel with thickness of 0.55 cm is between 4 dB and 9 dB above TR 38.901 at frequencies of 7.5 GH to 30 GHz.

Based on this large gap, we think it is necessary to increase IRR glass penetration loss from TR 38.901 of LIRR glass=23+0.3f. The increase could be on the offset of 23 dB. For example, we can consider LIRR glass=30+0.3f.

Observation 3: The measured penetration loss of IRR glass of one layer with thickness of 0.55 cm is between 4 dB and 9 dB above TR 38.901 at frequencies of 7.5 GH to 30 GHz.

Figure 5: Penetration loss of IRR glass at frequencies of 7.5 GHz to 30 GHz

Glass

The tempered glass in the measurement is with thickness of 0.55 cm. The measured penetration over different FR3 frequencies are shown in Figure 6. For comparison, we plot the glass penetration loss of TR 38.901 in the same figure.

It is observed from the figure that the measured penetration loss of glass with thickness of 0.55 cm is less than 5 dB below TR 38.901 at frequencies of 7.5 GH to 24 GHz. Furthermore, the measured glass penetration loss does not always linearly increase with frequency.

We think this gap may not be enough to trigger the modification of glass penetration loss from TR 38.901. However, it may be worth to further check whether the glass penetration loss is linearly increasing with frequency.

Observation 4: The measured penetration loss of glass with thickness of 0.55 cm is less than 5 dB below TR 38.901 at frequencies of 7.5 GH to 24 GHz. The measured glass penetration loss does not linearly increase with frequency.

Figure 6: Penetration loss of glass at frequencies of 7.5 GHz to 30 GHz

Based on the above arguments, we have the following proposal.

Proposal 5: RAN1 to consider

- expressing concrete penetration loss in terms of unit thickness (e.g., L(dB/cm) = 0.043+0.187 f or L=α⋅5+4f with α=Tconcrete23 and Tconcrete being concrete thickness in cm)

- increasing IRR glass penetration loss from TR 38.901 (e.g., L=30+0.3f)

- whether or not to reduce wood penetration loss from TR 38.901

- whether the glass penetration loss is linearly increasing with frequency.

### Delay spread

It was concluded in RAN1 #119 meeting to further study the delay spread for InH-Office LOS/NLOS, UMi LOS/NLOS and UMa LOS/NLOS.

#### Indoor office

The measurement layout is shown in Figure 7. This is within an Apple’s office building. The measurement settings are summarized in Table 1.

The BS antenna is fixed at 3 meters height and there are two BS positions in the measurement: 1). BS position is at middle top of Figure 7; 2). BS position is at right top of Figure 7. A list of measurements are performed at various positions inside the building. Depending on BS location and actual environments, we mark different measurement positions as LOS or NLOS, respectively.

Table 1: List of parameters used in measurements for indoor office

| Measurement parameter names | Measurement parameter values |
| --- | --- |
| Base station height | 3 meters |
| UE height | 1 meter |
| VNA configuration | Start frequency: 12.7 GHz; Stop frequency: 13.25 GHz; Step: 1 MHz; RBW: 500 Hz |
| Tx / Rx antenna | Directional / Omni-directional |

Figure 7: Measurement environment and equipment for indoor office

The distribution of delay spread in the measurements is shown in Figure 8. Note that this result is based on 25 dB dynamic range from the strongest path. It is observed that the mean RMS delay spread in the measurements is about 28 ns for LOS and 42 ns for NLOS, while the mean RMS delay spread from TR 38.901 is 10-0.01⋅log101+fc-7.692 (or 20 ns at 13 GHz) or 10-0.28⋅log101+fc-7.173 (or 32 ns at 13 GHz), for LOS and NLOS, respectively. In other words, with 25 dB dynamic range from the strongest path, our measurements are approximately aligned with TR 38.901.

We also check the mean RMS delay spread with different dynamic ranges from 10 dB to 60 dB. This is shown in Figure 9. It is observed that the mean RMS delay spread increases with dynamic range and it tends to be stable after 40 dB of dynamic range. Note that up to 60 dB dynamic range is possible, since the strongest path is much higher than noise floor in the measurements.

Observation 5: For indoor office scenario at 13 GHz, the measured delay spread is approximately aligned with TR 38.901 with 25 dB dynamic range.

Figure 8: Distributions of delay spread in indoor office scenario at 13 GHz

Figure 9: Mean RMS delay spread vs. dynamic range in indoor office scenario at 13 GHz

#### UMa

It was observed in our contribution [6] that for UMa NLOS scenario at 8 GHz, the measured delay spread in logarithm has mean of -7 and standard deviation is 0.5. Table 2 summarizes the comparison of the simulated delay spread and that in TR 38.901 at 8 GHz. Overall, we think the delay spread mean value should be reduced from TR 38.901 for FR3.

Table 2: Comparison of delay spread for UMa NLOS at 8 GHz

|  | Ray tracing simulations | UMa NLOS in TR 38.901 |
| --- | --- | --- |
| Delay spread mean | -7 | -6.46 |
| Delay spread standards deviation | 0.5 | 0.39 |

Proposal 6: RAN1 to consider reducing the delay spread mean value for UMa NLOS scenario in FR3.

#### UMi

The measurement layout is shown in Figure 10. This is around an Apple’s office building. The measurement settings are summarized in Table 1, where the base station is located at the top floor of parking garage.

Figure 10: Measurement layout

Our measurement results are shown in Figure 11. Table 3 summarizes the comparison of the simulated delay spread and that in TR 38.901 at 13 GHz.

Figure 11: Measured RMS delay spread for UMi

Table 3: Comparison of delay spread for UMi LOS/NLOS at 13 GHz

|  | Measurement for LOS | UMi LOS in TR 38.901 | Measurement for NLOS | UMi NLOS in TR 38.901 |
| --- | --- | --- | --- | --- |
| Delay spread mean | -7.85 | -7.42 | -7.27 | -7.11 |
| Delay spread standards deviation | 0.54 | 0.38 | 0.09 | 0.46 |

Proposal 7: RAN1 to consider reducing the delay spread mean value for UMi LOS/NLOS scenario in FR3.

#### SMa

It was observed in our contribution [6] that for SMa NLOS scenario at 8 GHz, the delay spread in logarithm has mean of -6.98 and standard deviation is 0.74.

Table 4 summarizes the comparison of the simulated delay spread for SMa NLOS and that for UMa NLOS in TR 38.901 at 8 GHz. Overall, we think the delay spread mean value for SMa NLOS should be smaller than that for UMa NLOS in TR 38.901 for FR3.

Table 4: Comparison of delay spread for SMa NLOS at 8 GHz

|  | Ray tracing simulations | UMa NLOS in TR 38.901 |
| --- | --- | --- |
| Delay spread mean | -6.98 | -6.46 |
| Delay spread standards deviation | 0.74 | 0.39 |

Proposal 8: RAN1 to consider the delay spread mean value for SMa NLOS is smaller than that for UMa NLOS scenario in TR 38.901 in FR3.

### Pathloss for UMa

It was concluded in RAN1 #119 meeting that the pathloss updates are not needed for InH-Office LOS/NLOS, InF LOS/NLOS and RMa LOS/NLOS. It was agreed to continue studying pathloss for UMi LOS/NLOS and UMa LOS/NLOS.

It was observed in our contribution [4] that the pathloss of UMa LOS scenario in TR 38.901 is aligned with the measurement results at frequency of 13 GHz; the pathloss of UMa NLOS scenario in TR 38.901 is aligned with the measurement results without building clutter at frequency of 13 GHz; the pathloss of UMa NLOS scenario in TR 38.901 is not well aligned with the measurement results with building clutter at frequency of 13 GHz, with the difference about 5 dB.

Additionally, it was observed in our contribution [6] that the pathloss of UMa scenario in TR 38.901 is aligned with the simulation results at frequency of 8 GHz.

Overall, we think the difference of 5 dB for the case of NLOS with building clutter can be considered as approximately aligned. Hence, we propose to conclude that pathloss updates is not needed for UMa LOS/NLOS scenario.

Proposal 9: The pathloss updates for UMa LOS/NLOS scenarios are not needed.

### Angular spread

It was agreed that ZSD for UMa LOS/NLOS should not be updated. Furthermore, It was concluded that there is no consensus to update angular spread models for

- InH LOS/NLOS: ZSA, ZSD

- UMi LOS: ZSA, ZSD

- UMi NLOS: ZSD

- UMa LOS: ZSA

On the other hand, it was agreed to further study the following angular spread parameters for at least 6-24 GHz frequency range

- InH LOS/NLOS: ASA, ASD

- UMi LOS: ASA, ASD

- UMi NLOS: ASA, ASD, ZSA

- UMa LOS: ASD, ASA

- UMa NLOS: ASA, ASD, ZSA

- InF LOS and NLOS: ASD, ASA, ZSA, ZSD

#### UMa

It was observed in our contribution [6] that for UMa NLOS scenario at 8 GHz the AoA spread in logarithm has mean of 1.72 and standard deviation of 0.6; the AoD spread in logarithm has mean of 0.6 and standard deviation of 0.765; the ZoA spread in logarithm has mean of 0.54 and standard deviation of 0.6.

Table 5 summarizes the comparison of the simulated angular spread and that in TR 38.901 at 8 GHz. Overall, we think at least AoD and ZoA angular spread mean values should be reduced from TR 38.901 for FR3.

Table 5: Comparison of angular spread for UMa NLOS at 8 GHz

|  | Ray tracing simulations | UMa NLOS in TR 38.901 |
| --- | --- | --- |
| AoA spread mean | 1.72 | 1.84 |
| AoD spread standard deviation | 0.6 | 2 |
| AoD spread mean | 0.6 | 1.4 |
| AoD spread standard deviation | 0.765 | 2 |
| ZoA spread mean | 0.54 | 1.22 |
| ZoA spread standard deviation | 0.6 | 1.4 |

Proposal 10: RAN1 to consider reducing the AoD/ZoA spread mean values for UMa NLOS scenario in FR3.

#### UMi

It was observed in our contribution [6] that for UMi NLOS scenario at 8 GHz, the AoA spread in logarithm has mean of 1.41 and standard deviation of 0.8; the AoD spread in logarithm has mean of 0.88 and standard deviation of 1; the ZoA spread in logarithm has mean of 0.18 and standard deviation of 1.1.

Table 6 summarizes the comparison of the simulated angular spread and that in TR 38.901 at 8 GHz. Overall, we think the angular spread mean values should be reduced from TR 38.901 for FR3.

Table 6: Comparison of angular spread for UMi NLOS at 8 GHz

|  | Ray tracing simulations | UMi NLOS in TR 38.901 |
| --- | --- | --- |
| AoA spread mean | 1.41 | 1.73 |
| AoD spread standard deviation | 0.8 | 0.35 |
| AoD spread mean | 0.88 | 1.31 |
| AoD spread standard deviation | 1 | 0.44 |
| ZoA spread mean | 0.18 | 0.88 |
| ZoA spread standard deviation | 1.1 | 0.34 |

Proposal 11: RAN1 to consider reducing the AoA/AoD/ZoA spread mean values for UMi NLOS scenario in FR3.

#### SMa

It was observed in our contribution [6] that for SMa NLOS scenario at 8 GHz, the AoA spread in logarithm has mean of 1.27 and standard deviation of 0.86; the AoD spread in logarithm has mean of 0.86 and standard deviation of 1; the ZoA spread in logarithm has mean of -0.388 and standard deviation of 1.17.

Table 7 summarizes the comparison of the simulated angular spread for SMa NLOS and that for UMa NLOS in TR 38.901 at 8 GHz. Overall, we think the angular spread mean values for SMa NLOS should be smaller than those for UMa NLOS from TR 38.901 for FR3.

Table 7: Comparison of angular spread for SMa NLOS at 8 GHz

|  | Ray tracing simulations | UMa NLOS in TR 38.901 |
| --- | --- | --- |
| AoA spread mean | 1.27 | 1.84 |
| AoD spread standard deviation | 0.86 | 2 |
| AoD spread mean | 0.86 | 1.4 |
| AoD spread standard deviation | 1 | 2 |
| ZoA spread mean | -0.388 | 1.22 |
| ZoA spread standard deviation | 1.17 | 1.4 |

Proposal 12: RAN1 to consider the AoA/AoD/ZoA spread mean values for SMa NLOS scenario are smaller than those mean values for UMa NLOS scenario in TR 38.901 in FR3.

### Number of clusters

#### UMi

As in [6], we simulate the number of clusters for UMi NLOS scenario, where a cluster is counted if its power is within 30 dB of that of the strongest cluster. Figure 12 shows the CDF of the number of clusters. It is shown that at 95 percentile, the number of clusters is 18, which is aligned with the value of 19 in TR 38.901.

Observation 6: For UMi NLOS scenario at 8 GHz, the number of clusters is 18 at 95% percentile.

Figure 12: CDF of the number of clusters for UMi NLOS

## UE antenna model

Several options on UE antenna radiation pattern modeling have been identified. Among the existing options, we support Option 1 which reuse Table 7.3-1 of TR 38.901. Within Option 1, we support θ3dB=90°, SLAV=25 dB, ϕ3dB=90°, Am=25 dB, and maximum directional gain of an antenna element of 5 dBi, as in Table 5.2.3.3-1 of [7].

Proposal 13: For UE antenna radiation pattern modeling, support Option 1 (reuse Table 7.3-1 of TR 38.901) with θ3dB=90°, SLAV=25 dB, ϕ3dB=90°, Am=25 dB, and maximum directional gain of an antenna element of 5 dBi.

# Conclusion

In this contribution, we provided our simulation results for channel model validation of TR 38.901 for 7-24 GHz and our views on suburban scenario definition. Our observations and proposals are as follows:

Proposal 1: Consider the following parameters for suburban scenario for calibration purpose:

- Layout with ISD: 1299 m (Option 1)

- BS height: 25 m (Option 1)

- Min BS - UT distance (2D): 25 m (Option 2)

Proposal 2: For SMa, the LOS probability is given by PrLOS=exp-d2D-out-dmexpα∙hBSβ+γhUEδ, which applies to UE in different building types and vegetation is not explicitly considered.

Proposal 3: Confirm the pathloss formula for SMa (based on ITU M.2135-1), where it is applicable to UE height up to 13.5 meters.

Proposal 4: For SMa, the O2I penetration loss for residential buildings is given by PLtw=5-10log100.3⋅10-Lglass10+0.7⋅10-Lwood10.

Observation 1: The measured penetration loss of wood with thickness of 1.75 cm is less than 3.5 dB below TR 38.901 at frequencies of 7.5 GHz to 30 GHz.

Observation 2: The measured penetration loss of concrete with thickness of 4.7 cm is between 30 dB and 100 dB below TR 38.901 at frequencies of 7.5 GH to 30 GHz.

Observation 3: The measured penetration loss of IRR glass of one layer with thickness of 0.55 cm is between 4 dB and 9 dB above TR 38.901 at frequencies of 7.5 GH to 30 GHz.

Observation 4: The measured penetration loss of glass with thickness of 0.55 cm is less than 5 dB below TR 38.901 at frequencies of 7.5 GH to 24 GHz. The measured glass penetration loss does not linearly increase with frequency.

Proposal 5: RAN1 to consider

- expressing concrete penetration loss in terms of unit thickness (e.g., L(dB/cm) = 0.043+0.187 f or L=α⋅5+4f with α=Tconcrete23 and Tconcrete being concrete thickness in cm)

- increasing IRR glass penetration loss from TR 38.901 (e.g., L=30+0.3f)

- whether or not to reduce wood penetration loss from TR 38.901

- whether the glass penetration loss is linearly increasing with frequency.

Observation 5: For indoor office scenario at 13 GHz, the measured delay spread is approximately aligned with TR 38.901 with 25 dB dynamic range.

Proposal 6: RAN1 to consider reducing the delay spread mean value for UMa NLOS scenario in FR3.

Proposal 7: RAN1 to consider reducing the delay spread mean value for UMi LOS/NLOS scenario in FR3.

Proposal 8: RAN1 to consider the delay spread mean value for SMa NLOS is smaller than that for UMa NLOS scenario in TR 38.901 in FR3.

Proposal 9: The pathloss updates for UMa LOS/NLOS scenarios are not needed.

Proposal 10: RAN1 to consider reducing the AoD/ZoA spread mean values for UMa NLOS scenario in FR3.

Proposal 11: RAN1 to consider reducing the AoA/AoD/ZoA spread mean values for UMi NLOS scenario in FR3.

Proposal 12: RAN1 to consider the AoA/AoD/ZoA spread mean values for SMa NLOS scenario are smaller than those mean values for UMa NLOS scenario in TR 38.901 in FR3.

Observation 6: For UMi NLOS scenario at 8 GHz, the number of clusters is 18 at 95% percentile.

Proposal 13: For UE antenna radiation pattern modeling, support Option 1 (reuse Table 7.3-1 of TR 38.901) with θ3dB=90°, SLAV=25 dB, ϕ3dB=90°, Am=25 dB, and maximum directional gain of an antenna element of 5 dBi.

# References

- RP-234018, “New SID: Study on channel modelling enhancements for 7-24 GHz for NR,” Dec. 2023.

- Chairman’s Notes, 3GPP TSG RAN WG1 #119 Meeting, Nov. 2024.

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, v17.1.0, Jan. 2024.

- R1-2410910, “Summary #5 of discussions for Rel-19 7-24 GHz Channel Modeling Validation” Nov. 2024.

- R1-2404304, “Initial measurement results for channel model validation,” May 2024.

- R1-2408484, “On validation of channel model,” Oct. 2024.

- 3GPP TR 38.803, Study on new radio access technology: Radio Frequency (RF) and co-existence aspects, v14.4.0, Jun. 2024.