3GPP TSG RAN WG1 #120	R1-2500883

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.3.2

Source: 	Lenovo

Title: 	SBFD random access operation

Document for:	Discussion and Decision

Introduction

In RAN#102, a new work item “Evolution of NR duplex operation: Sub-band full duplex (SBFD)” was agreed [1]. The objectives of the work item for random access procedure are the following:

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC CONNECTED mode [RAN1, RAN2] - Study and specify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2] - RAN#104 to check whether to proceed normative work

In previous RAN1 meetings, many agreements have been reached for SBFD random access operation.  In this contribution, we provide further analysis and proposals on the SBFD operation to support random access in SBFD symbols by UEs in RRC CONNECTED mode and RRC_IDLE/INACTIVE mode.

Discussion

In 5G NR system, random access procedure is used by both RRC CONNECTED UEs and RRC IDLE/INACTIVE UEs for various purposes. ROs are configured in the initial UL BWP and can be configured in an active UL BWP for the UEs to perform random access procedure. The UE determines valid ROs from the configured ROs and associates the valid ROs with the transmitted SSBs. When a RACH procedure starts, the UE transmits the preambles in an RO that is associated with an SSB, which corresponds to good channel quality.

Based on the measured channel quality and RACH resource configurations, the UE may decide whether to use PRACH transmission with repetitions. PRACH repetition was introduced in Rel-18 coverage enhancement. The repetitions are transmitted in a group of ROs, and ROs in the group should use same frequency domain resource, be continuous in time domain, and are associated with a same SSB.

For random access for SBFD aware UEs, it has been agreed to make the progress for enabling Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols in accordance with the discussion in AI 9.3.1. So, in this section, we will focus on issues of Msg 1 transmission in the UL subband.

## 2.1 Discussion on RO configuration and PRACH resource determination

The SBFD aware UEs should be provided with configurations to determine valid ROs in UL subband in the SBFD symbols. The configurations may include e.g., PRACH slot configuration, time domain resource and frequency domain resource of the ROs. It has been agreed that both RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2 are supported, and also agreed that ‘Enabling both options at the same time for a UE is not supported’. And for gNB side, the following conclusion and agreement are agreed in previous meetings.

Conclusion There is no consensus in RAN1 to support the following in Rel-19: - Enabling both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side. Agreement Introduce explicit indications (1-bit SIB1 indication and/or dedicated signalling to convey cell-specific configuration) to indicate whether RACH configuration Option 1 for SBFD random access operation is enabled or not from network side.

From gNB side, it has been agreed that RACH configuration Option 1 with Alt 1-1 could be enabled or disabled by an explicit indication in SIB1. For PRACH configuration Option 2, it could be enabled or disabled by the existence of the additional RACH configuration.

Proposal 1: RACH configuration Option 2 could be enabled or disabled by the existence of the additional RACH configuration.

Considering only one of two options is enabled from gNB side, a UE supporting the other options should fallback to legacy UE’s behavior, i.e., use legacy ROs to transmit PRACH.

Proposal 2: If one of two options between Option 1 with Alt 1-1 and RACH configuration Option 2 is enabled from gNB side, a UE supporting the other option should fallback to legacy UE’s behaviour.

RO configuration option 1

Regarding Option 1 with Alt 1-1, following agreement is reached in last meeting.

Agreement For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, Alt 2-1 is supported. - Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs. - ROs that are outside of UL usable PRBs are invalid

The intention of the reinterpretation is to guarantee more configured ROs to be in UL usable RBs for SBFD symbols, However, it has been already agreed/concluded in RAN1#117 meeting (see below) that if legacy ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, network should ensure the ROs are within the UL usable PRBs, so in this case, there is no need to reinterpret msg1-FrequencyStart. So, we propose this interpretation should be used only for SBFD symbols configured as DL by tdd-UL-DL-ConfigurationCommon.

Conclusion For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1: - The legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. - It is up to the network to configure ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. If legacy ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, network ensures the ROs are within the UL usable PRBs

Proposal 3: For option 1 with Alt 1-1, Alt 2-1 should be used for SBFD symbols configured as DL by tdd-UL-DL-ConfigurationCommon.

- Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

- ROs that are outside of UL usable PRBs are invalid

RO configuration option 2

Regarding Option 2, the remaining issues and related agreements could be seen in the following.

Agreement Confirm the following working assumption. Agreement For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, - For FR2, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211) - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols. - Working Assumption: For FR1, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211) - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols. Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options. Agreement Confirm the following Working Assumption: Working Assumption For RACH configuration Option 2, use legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration. - FFS: Whether/How to handle the case where the legacy ROs overlap with additional ROs Agreement For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, down-select from the following options: - Alt-1: introduce a subframe/slot offset (similar as prach-ConfigurationSOffset-IAB) to determine the subframe/slot number for ROs in SBFD symbols for FR1/FR2 - FFS: details of the subframe/slot offset - Alt-2: introduce a new parameter indicating from an entry from a pre-defined/configured table to derive additional subframe/slot numbers. - Details are FFS. - Alt-3: introduce a bitmap indicating or puncturing subframe(s)/slot(s) for additional ROs in SBFD symbols. - Details are FFS. - Alt-4: no enhancement Agreement For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, down-select from the following alternatives: - Alt-1: do not optimize this case - Alt-2: the additional-ROs are considered invalid when overlapping with legacy-ROs

For interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, it has been agreed that for both FR2 and FR1, existing random access configurations table for unpaired spectrum should be used. Considering the simplicity and less overhead, we think there is no need to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols. Thus, Alt-4 should be used.

Proposal 4: For option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), for both FR1 and FR2, Alt -4 should be supported, thus there is no need to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols.

For the parameters in additional RACH configurations, for simplicity, we think all the parameters currently in rach-ConfigCommon should be kept.

Proposal 5: For option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), all the parameters currently in rach-ConfigCommon should be kept.

In SBFD symbol configured with flexible configured by tdd-UL-DL-ConfigurationCommon, there would be a case that the legacy valid ROs are overlapped with additional valid ROs in SBFD symbol configured with flexible, then how to handle the collision should be discussed. We think Alt-2 should be supported, considering it might not be supported by gNB to receive the PRACH if the two ROs are overlapping but associated with different SSBs and it’s also difficult for gNB to make sure the overlapping ROs are associated with same SSB.

Proposal 6: For option 2(i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), if the legacy ROs overlap with additional ROs, the additional-ROs are considered invalid. (Alt-2).

## 2.2  PRACH resource selection

Both legacy-ROs and additional-ROs can be used for SBFD-aware UEs in RACH configuration Option 1 and Option 2. Regarding how to select the RO for PRACH initial attempt, there are two main options as listed in following agreement.

Agreement It is up to RAN2 to determine the detailed specified/configured conditions/prioritizations for RO selection for initial PRACH transmission attempt in one random access procedure.

For how to select the RO for PRACH re-attempt, three options are listed as following.

Initial proposal 1-4-3-1: For SBFD aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure Initial proposal 1-4-3-2: For SBFD aware UEs and RACH configuration Option 2, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

For fast initial access, we think option 3 independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure should be supported. Besides, option 1 and option 2 can be achieved by UE implementation of option 3.

Proposal 7: For SBFD-aware UEs and both RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2, for initial PRACH transmission re-attempt in one random access procedure, consider the following option 3 for PRACH transmission re-attempt in one random access procedure:

- Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

## 2.2 Others

It has been discussed in last meeting on whether type 2 random access procedure should be supported in SBFD symbols. We think this discussion could be postponed until the discussion on Type-1 random access procedure and sub-band full duplex operations in 9.3.1 are finished. We are open to support type 2 random access in SBFD symbols if clear benefits can be obtained.

Proposal 8: The discussion on whether/how to support type 2 random access procedure in SBFD symbol could be postponed.

It has been agreed that random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode is supported.

One of the main challenges of supporting random access (RA) in SBFD UL subband for SBFD-aware UEs is the potential increase in the inter-UE CLI. This comes mainly due to the random nature of Msg.1 preamble transmission, i.e., the UE decides, based on its own need, when and how to transmit the Msg.1 preamble. Nonetheless, the selection space of when and how the UE transmits the Msg.1 preamble is under the control of network. For example, the network controls the time and the frequency locations of the valid ROs, the uplink power limit of Msg.1 preamble, etc. These aspects can still be used by network to limit the inter-UE CLI impact, although only to a certain degree. Note that, since the inter-UE CLI strength depends on the distance between UEs, it can be expected that the inter-UE CLI strength and impact is larger in hotspot areas than otherwise. The location of hotspot areas, in some scenarios, are known to the network, e.g., a shopping mall, while in some other scenarios, it can be predicted by the network, e.g., from location of scheduled UEs. Such information can be utilized by the network to limit the impact of inter-UE CLI. For example, if the network knows that SSB#0 is providing coverage to a hotspot area, it can impose restrictive conditions/thresholds on SBFD-aware UEs performing RA procedure via SSB#0. Differently, the network can impose relaxed conditions/thresholds on SBFD-aware UEs performing RA procedure via SSB#1, if the network knows that SSB#1 is providing coverage to a non-hotspot area.

Observation 1: Some side-information at network, like location of hotspot areas can be utilized to obtain more efficient RA configurations in SBFD UL subband and limit impact of inter-UE CLI.

Proposal 9: RAN1 to discuss solutions to limit impact of inter-UE CLI that may occur due to RA operations in SBFD UL subband.

Conclusion

In summary, we have following observations and proposals for SBFD random access operation:

Proposal 1: RACH configuration Option 2 could be enabled or disabled by the existence of the additional RACH configuration.

Proposal 2: If one of two options between Option 1 with Alt 1-1 and RACH configuration Option 2 is enabled from gNB side, a UE supporting the other option should fallback to legacy UE’s behaviour.

Proposal 3: For option 1 with Alt 1-1, Alt 2-1 should be used for SBFD symbols configured as DL by tdd-UL-DL-ConfigurationCommon.

- Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

- ROs that are outside of UL usable PRBs are invalid

Proposal 4: For option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), for both FR1 and FR2, Alt -4 should be supported, thus there is no need to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols.

Proposal 5: For option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), all the parameters currently in rach-ConfigCommon should be kept.

Proposal 6: For option 2(i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), if the legacy ROs overlap with additional ROs, the additional-ROs are considered invalid. (Alt-2).

Proposal 7: For SBFD-aware UEs and both RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2, for initial PRACH transmission re-attempt in one random access procedure, consider the following option 3 for PRACH transmission re-attempt in one random access procedure:

- Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

Proposal 8: The discussion on whether/how to support type 2 random access procedure in SBFD symbol could be postponed.

Observation 1: Some side-information at network, like location of hotspot areas can be utilized to obtain more efficient RA configurations in SBFD UL subband and limit impact of inter-UE CLI.

Proposal 9: RAN1 to discuss solutions to limit impact of inter-UE CLI that may occur due to RA operations in SBFD UL subband.

References

- RP-234035, “New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, RAN#102.