3GPP TSG RAN WG1 #120		R1-2501027

Athens, Greece, February 17th – 21st, 2025

Source: 	MediaTek Inc.

Title:	Discussion on ISAC channel modelling

Agenda item:	9.7.2

Document for:	Discussion

## Introduction

Integrated sensing and communication (ISAC) have attracted much attention due to the increasing demand, e.g., the UAV detection/tracking in some area, human detection in the outdoor/indoor, etc. The channel modelling is critical for evaluating the technical performance. However, the current NR 3GPP channel modelling only focus on the communication side, and not considering the sensing capability. So, a new study item on channel modelling for ISAC has been approved in RAN#112 [1]:

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

In last RAN1 meeting [2], it discussed how to obtain the bistatic RCS, the generation of indirect path of NLOS rays + NLOS rays, etc, and achieved some consensus. However, there are still some open issues for the ISAC channel modelling needed to be further discussed. In this contribution, we will further discuss the ISAC channel modelling related issues, e.g., the details design of target modelling, background environment modelling, etc.

## Discussion

### Target object modelling

From our view, how to model the sensing target is the major difference between communication link and sensing link. In communication link, it does not care how the signal reflection via a specific object, and the channel characteristic is modelled by mathematical statistical distribution. However, according to the requirement of sensing service, it needs to detect the target’s shape, whether the target exists (e.g., intrusion), the target’s velocity/range/angle, etc, that is, it needs to know the details/exact sensing signal reflection route if we want to obtain the details information of the sensing target.

Regarding how to model the sensing target, e.g., modelled by single scattering point or multiple scattering points, the following agreement was achieved in RAN1#116bis meeting [3]:

Agreement - In the target channel between Tx and Rx, scattering of a sensing target can be modelled as single scattering point or multiple scattering points - FFS one or multiple incoming/output rays corresponding to a scattering point - FFS how to select single or multiple scattering points for the target, e.g. depending on the distance between target and Tx/Rx, size/shape of target, etc. - Note: the sensing target can be assumed in far field of sensing Tx/Rx. - FFS details to model the single or multiple scattering points

Based on the above agreement, the sensing target can be modelled by single scattering point or multiple scattering points. Regarding how to select single scattering point or multiple scattering points for the sensing target and how to model a sensing target using a multiple scattering points, the following proposal was discussed in last meeting [4], however, it didn’t achieve consensus due to controversial views and limited meeting time.

[H][FL1] Proposal 4.4-1 If a target is modelled with multiple scattering points, - The number of scattering points of a target is generated in the beginning of the simulation and keeps unchanged in the whole simulation - The number of scattering points may be dependent on multiple factors, including type/size of the target, the scenario/use case. FFS dependence on the distance between sensing Tx/Rx and the target - (Working assumption) The number and locations of the scattering points are common to each pair of sensing Tx/Rx - FFS A scattering point that is invisible from an incident direction or a scattered direction is modelled by angular dependent RCS component B1 or self-blockage at the target - The changes of relative locations (i.e., micro motion) of the multiple scattering points are NOT considered in the modelling of delay and angle of a direct/indirect path - FFS micro-Doppler - RAN1 assumes no ray is scattered from one scattering point to another scattering point of the same target - RCS values of the multiple scattering points are individually determined

Regarding the multiple scattering points modelling, we suggest the following factors need to be considered:

- Sensing resolution: if the sensing resolution is lower, it cannot differentiate multiple key components by sensing receiver, which also is relevant with the target size, so, in this situation, it seems that the target can be modelled by lesser components or even one component. The detailed sensing resolution calculation of both monostatic mode and bistatic mode for sensing target can refer to the Table 1.

- The demand of the ISAC service: if the requirement of the sensing service only detect whether there is an intruder, it seems that multiple key components modelling is not needed.

Table 1: sensing resolution

Proposal 1: At least the sensing resolution (also relevant with target size, simulation configuration parameter, etc) and the demand of the sensing service can be as two factors on selecting single or multiple scattering points for the sensing target modelling.

For multiple scattering points modelling mechanism of one sensing target, there are some potential open questions, e.g., whether to consider the ray’s reflection interaction of the multiple scattering points, and/or whether to consider the relative position change of multiple scattering points. From our understanding, the interaction and relative position change are not needed at this current stage, especially the signal power will be decreased for multiple bounce reflection, and this also will make the channel modelling complexity.

Proposal 2: It is suggested not to consider the interaction and relative position change of multiple scattering points belonging to the same sensing target.

If there are multiple sensing targets, whether to consider the sensing path interaction was discussed in last meeting. From our understanding, for sensing target detection, the direct path among sensing Tx, sensing Rx and sensing target is very crucial, and the sensing signal received power reflected by multiple sensing target will significantly downgrade, which will have less impact on the sensing performance evaluation. Instead, if consider the behaviour, it will make the sensing channel modelling behaviour more complicated but without any clear benefit/motivation. Thus, we suggest not to model a propagation path from Tx to Rx interacting with more than one sensing target.

Proposal 3: It is suggested not to model a propagation path from Tx to Rx interacting with more than one sensing target in the sensing channel modelling.

### Target channel modelling

The target channel modelling can be divided into two parts, the first part is direct path modelling, and the second part is indirect path modelling. For the direct path modelling, we have agreed that only a single direct path modelled for a scattering point of target. And TR 38.901 stochastic cluster modelling mechanism is used to model an indirect path in the target channel for Tx-target and target-Rx link. How to concatenate the two links were discussed in last several meeting and the following agreements were achieved for direct path and indirect path of NLOS ray + NLOS ray.

Agreement For radio propagation Case 1, for modelling the target channel of a target with single scattering point, - To model a direct path, a single LOS ray from Tx to target and a single LOS ray from target to Rx are generated - AoA/ZoA of the direct path at Rx, AoD/ZoD of the direct path at target are generated at least based on the 3D location of target and Rx in the global coordinate system - AoD/ZoD of the direct path at Tx, AoA/ZoA of the direct path at target are generated at least based on the 3D location of Tx and target in the global coordinate system - The Delay of the direct path = (d3D_tx_target + d3D_target_rx)/c - The Doppler of the direct path is generated by spherical unit vectors by AoD/ZoD at Tx, by spherical unit vectors by AoA/ZoA at Rx, and velocity of Tx, target and Rx - The power of the direct path is generated as the product of the power of the LOS ray from Tx to target, the power of the LOS ray from target to Rx, and the effect of RCS - FFS initial phase - FFS how to model RCS, polarization of target - FFS number of direct paths - FFS on detailed modelling of indirect path(s) - FFS applicability of direct path generation to each scattering point when the target is modelled as multiple scattering points Agreement - To generate indirect paths of NLOS ray + NLOS ray in the target channel - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4 - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4 - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2, - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1 - Other methods are up to company choice for complexity reduction - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration - The power threshold for path dropping is X=[-25] dB - X is relative to the strongest indirect path in the target channel - FFS: further power normalization of target channel is performed after path dropping, - Note: power normalization when target channel and background channel are combined can be discussed separately - FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx

Based on the above agreements, it can be seen that both how to generate the sensing direct path channel of LoS ray + LoS ray and the indirect paths of NLoS + NLoS have had been agreed in previous several meeting, however, how to generate the sensing indirect path of LoS + NLOS, NLoS+LoS doesn't have the corresponding clear agreement. Per our understanding, the same mechanism of indirect paths of NLOS ray + NLOS ray can be directly reused for the indirect paths of NLOS ray + LOS ray or LOS ray + NLOS ray for simplicity. Thus, the following proposal is suggested for clarification:

Proposal 4: Clarify that the agreements achieved in RAN1#119 on generating indirect paths of NLOS ray + NLOS ray in the target channel can be reused for indirect paths of NLOS ray + LOS ray or LOS ray + NLOS ray.

For sensing channel modelling, the direct path with LoS+LoS ray is critical for sensing service since the sensing target detection and tracking mainly rely on the sensing LoS ray. For NLoS cluster/ray using stochastic cluster modelling mechanism, the current TR 38.901 communication cluster are as a starting point for each link modelling. In current communication cluster modelling, it defined so many clusters/rays per scenario based on the mathematical statistic distribution. For NLoS cluster modelling in target channel, we only care about the cluster/ray reflected by the sensing target. However, not all the communication clusters/rays will be reflected by the sensing target, in other words, modelling so many clusters/rays for each sensing link, e.g., Tx-Tg and/or Tg-Rx, is not needed, what’s worse, it will make the sensing channel generation procedure more complicated. Thus, the following proposal is suggested.

Proposal 5: For indirect path modelling in sensing target modelling, reducing the number of rays per cluster and/or reducing the number of clusters are supported.

EO modelling

Regarding how to generate the indirect paths in target channel and whether EO is needed for the indirect path, it was extensively discussed in the several previous meetings, and the following agreement was achieved.

Agreement in RAN1#117bis EO is a non-target object with known location. - FFS other known parameters of the EO - FFS details on EO modeling The following options for EO modeling are considered for further study - Option 1: EO is modelled different from a sensing target - Applicable at least for an EO having extremely large size (referred as EO type-2 for discussion purpose) - FFS modeled similar to section 7.6.8 ground reflection in TR 38.901 - FFS EO modeling impacts the target channel and/or the background channel - Option 2: EO is modeled same/similar as a sensing target - Applicable for an EO having comparable physical characteristics as a sensing target, (referred as EO type-1 for discussion purpose) - FFS Applicable for EO type-2 - FFS EO modeling impacts the target channel and/or the background channel - Option 3: EO is modeled and its location is determined from a stochastic clutter generated following the cluster generation in TR 38.901 - FFS details - Option 4: EO is not modelled - Other options are not precluded Note: it is not precluded that multiple options can be supported in the channel modelling Agreement in RAN1#118: When the stochastic cluster is used to generate the indirect paths in the target channel of a target - The stochastic cluster generation in section 7, TR 38.901 is used as starting point. - FFS a stochastic cluster is generated between Tx and Rx satisfying Tx-target-Rx geometry, or between Tx/Rx and target - FFS modification to stochastic cluster generation in section 7, TR 38.901 - FFS use of sub-cluster to model the indirect paths Note: RAN1 continues studying using EO to generate the indirect paths in the target channel of a target Agreement in RAN1#118: When EO type-2 is modelled, specular reflection is considered to model EO type-2 using section 7.6.8 of TR 38.901 as reference - As starting point, the effect of type-2 EO (i.e., in the path node1-EO-node2) is modelled as bR∥specular00-R⊥specular, b is a scaling factor (e.g., c equals to d3DdGR relative to LOS ray in section 7.6.8 in TR 38.901) - FFS any update to b,R∥specular,R⊥specular - FFS any update taking EO orientation into account - FFS any changes based on section 7.6.8 of TR 38.901 if EO type-2 has finite size - FFS whether diffraction and scattering can be considered in addition to specular reflection - EO type-2 is an optional modelling component if supported in a sensing scenario - FFS which deployment scenario(s) EO type-2 will apply

For most of ISAC use cases, the demand of velocity/range/angle detection is increasing. For these detection metrics, the LoS path between sensing transmitter and sensing target and between sensing target and sensing receiver are needed. Otherwise, the NLoS path will lower the detection performance if the reflection position is not known between sensing transmitter and sensing target or between sensing target and sensing receiver.

Observation 1: For ISAC, NLoS path will lower the detection performance if the reflection position is not known.

From our understanding, whether there is NLOS path between sensing transmitter and sensing target or sensing target, it also depends on the sensing scenario. E.g., for UAV scenario, the NLOS path can be neglected since it is a LOS dominant scenario. However, it seems that the NLoS path is unavoidable in some real deployment scenario, e.g., urban scenario. But we need to consider the purpose of ISAC channel modelling, if the scenario is NLOS dominant and the reflection position is unknow, it seems that the sensing performance is bound by the channel condition, which is not preferred for evaluating the sensing technic itself, especially for ISAC LLS.

Considering the real deployment scenario and the purpose of defining channel modelling, we suggest, at least, the one hop {between sensing transmitter and sensing target} or {between sensing target and sensing receiver}is LOS propagation path hop as illustrated in Figure 1.

Figure 1: Example of ISAC propagation path

Proposal 6: If NLOS path is modelled for target channel in some scenario, it is suggested that the NLOS can be modelled by EO besides using the stochastic mechanism, including EO type-1 and EO type-2.

From our understanding, whether the EO is finally modelled in specific implementation, it can up to the sensing scenario/use case, e.g., for UAV scenario, the EO modelling can be disabled in the scenario, and for the dense urban scenario, the EO modelling is needed since the channel condition is complexity. Thus, whether EO can be modelled for ISAC channel modelling is up to which sensing scenario will be considered.

Proposal 7: EO modelling can be enabled/disabled for ISAC channel modelling, which is up to the sensing evaluation scenario.

### Sensing pathloss

For the sensing pathloss calculation, the position of the sensing target needs to be considered since the sensing signal needs to be reflected by the sensing target. Assuming the distance between sensing transmitter and sensing target is d1, the distance between the sensing receiver and sensing target is d2, the receiver antenna aperture is A=λ24π, the sensing transmitter power is P1, so, the sensing receiver power reflected by the sensing target is denoted as P2 ∝P14πd12*4πd22*RCS*λ24π. Thus, the sensing pathloss can be denoted as following equation:

PLdBd1,d2=PLd1+PLd2-10log10RCS+10log10λ24π

Where, the pathloss PLd1 and PLd2 can refer to the pathloss model defined in corresponding 3GPP TRs, e.g., UMa,UMi,InH scenario of TR 38.901, UAV scenario of TR 36.777, Vehicle scenario of TR 37.885. Since the pathloss is separately calculated for each link and legacy communicaiton pathloss only is applicable for some range of BS/UE height, so, it needs to consider the impact of the sensing target height for the pathloss model. E.g., for UMa LOS scenario, the tranmitter height for each pathloss link is assumed with fixed 25m height.

Proposal 8: For sensing pathloss, it can be denoted asPLdBd1,d2=PLd1+PLd2-10log10RCS+10log10λ24π, where the d1 is the distance between sensing transmitter and sensing target, and d2 is the distance between the sensing receiver and sensing target, PL equation use the current pathloss model in 3GPP TRs as a starting point. FFS: how to consider the impact of the sensing target height on pathloss.

### Background modelling

For ISAC system, besides the sensing target object, there are also some clutters as interference to affect the sensing performance, e.g., the building, trees, streetlamp, unwanted moving car/humans, etc. For these background environment clutter, we propose the clutter interference can be classified following two types:

- Static clutter interference (i.e., environment object): e.g., building, trees, streetlamp, etc.

- Random clutter interference (i.e., stochastic clutter): e.g., unwanted moving cars/humans, etc.

For the static clutter, its position is known/fixed in the coordinator system, the corresponding parameters can be obtained using the similar mechanism with sensing target modelling. For the random clutter interference, it can be modelled by the stochastic mechanism using the existing 3GPP TR procedure, e.g., TR 38.901, TR 37.885, TR 36.777, etc.

Proposal 9: For ISAC background channel modelling, two channel types are defined: environment object and random clutter.

- environment object: object(s) with known location, and modelling method is similar with the sensing target modelling, the difference is environment object is static in the whole simulation.

- random clutter: object(s) with known location, and it can be modelled using the stochastic mechanism of the existing 3GPP TR procedure e.g., TR 38.901, TR 37.885, TR 36.777.

Regarding the random clutter for background channel modelling, the following agreements, including bistatic sensing mode and monostatic sensing mode, were achieved in the previous meeting:

Agreement For modeling stochastic cluster in background channel, in order to define the background channel for TRP-UE and UE-TRP bistatic sensing mode, - The large scale and small scale parameters defined in TR 38.901, TR 37.885, TR 36.777 are used as start point In order to define the background channel for TRP-TRP and UE-UE bistatic sensing mode, - RAN1 to study how to model background channel - Option 1: The large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as start point - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results FFS whether/how to do power normalization between target channel and background channel Agreement In order to define the background channel for TRP and UE mono-static sensing mode, - RAN1 to study how to model the background channel - Option 1: randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario - FFS EO is modelled in the background channel - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results or radar literatures - FFS EO is modelled in the background channel - Option 3: the locations of clusters in the target channel are deterministically generated, then the background channel is generated using the clusters with the determined locations. - Other options are not precluded

As we discussed in previous proposal, the principle for random clutter modelling is to maximally reuse the current 3GPP TRs. So, for the TPR(UE)-UE(TRP) bistatic sensing mode, and TRP-TRP/UE-UE bistatic sensing mode, the existing TR 38.901, TR 37.885, TR 36.777, TR 38.859 can be reused as a baseline.

For TRP and UE mono-static sensing mode, there are no reference 3GPP TRs for background channel since these two sensing modes are first introduced. Three options were agreed for further study in previous meeting. For the three options, we prefer using the option 1 as a reference since it also can directly reuse the mechanism of TR 38.901 for simplicity.

Proposal 10: For ISAC random clutter for background channel modelling in TRP and UE mono-static sensing mode, randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario.

### Sensing spatial consistency

Regarding the spatial consistency, it was discussed in last meeting, and the following agreement was achieved:

Agreement - Spatial consistency should be supported for ISAC channel - Spatial consistency should be supported based on movement of sensing Tx, sensing target and/or sensing Rx - FFS EO handling

For communication link, the spatial consistency issue is discussed due to the UE mobility. However, for ISAC sensing service, we assume the position of sensing transmitter and sensing receiver is known in advance for better sensing performance, which means if one of them or both of them is/are moving, their trajectory is known in advance. As discussed in above, the parameters of the sensing target are calculated based on the geometry of the coordinate system, thus, it naturally meets the spatial consistency for direct path, which also can apply to the static background environment clutter. For indirect path, for each link, i.e., Tx-target and target-Rx, it can directly reuse the TR 38.901 spatial consistency procedure, then combining the two links to generate the target indirect path. For random background environment clutter, the TR 38.901 spatial consistency procedure can be as a starting point since the clutter’s moving is random.

Proposal 11: For ISAC sensing target with direct path and static background environment (i.e., environment object), both naturally meet the spatial consistency since the sensing parameters are calculated based on the deterministic/known information.

Proposal 12: For ISAC sensing target with indirect path, each link directly reuse the TR 38.901 spatial consistency procedure, then combining the two links to generate the target indirect path.

Proposal 13: For the spatial consistency of ISAC random background clutter, taking TR 38.901 spatial consistency procedure as a starting point.

### Sensing LLS

Regarding the sensing LLS, the following agreement was achieved in previous meeting.

Agreement ISAC channel model for link level simulation is to be discussed after the system level channel model is sufficiently stable with basic functionalities.

Considering the system level channel model discussion progress and limited meeting time for R19, it is better to discuss the sensing LLS at this meeting. For sensing LLS, there are two potential solutions to implement it.

- Option 1: Using current TR 38.901 communication LLS procedure as a baseline.

  - Option 1-1: Select one or more clusters of the existing CDL channel models as the sensing clusters corresponding to the sensing target.

  - Option 1-2: Newly generate sensing target related sensing cluster parameters, e.g., time delay, power, angle, and velocity.

- Option 2: simplify the current ISAC SLS procedure, e.g., only consider one BS-UE link

  - still using the SLS to generate the channel coefficient, then fed into the LLS simulator.

Since the communication LLS procedure is typical and mostly used for LLS performance evaluation, option 1 is straightforward as starting point for sensing LLS. However, per our understanding, the sensing related cluster in option 1 only reflect the sensing LOS ray + LOS ray and the remaining clusters corresponds to the background channel, so, it needs more discussion on how to reflect the indirect path related clusters, also more discussions on how to replace/add the sensing cluster. For option 2, there is not these issues, and can directly reuse the system level discussion progress. Thus, the following proposal is suggested for sensing LLS:

Proposal 14: For sensing LLS channel generation, using the simplified SLS to generate the channel coefficient, then fed into the LLS simulator. FFS: how to simplify the SLS procedure for sensing LLS.

### Sensing mode

Regarding the sensing mode, the SID states that all six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). One potential question is whether the sensing mode needs to be prioritized, especially when considering the workload. From our understanding, at least from the channel modelling perspective, the prioritization of sensing mode is not needed since the channel modelling mechanism is same/similar for both monostatic mode and bistatic mode, which means the HISAC shall include both monostatic mode channel and bistatic mode ISAC channel.

Proposal 15: From ISAC channel modelling perspective, a common channel modelling framework is defined for all six sensing modes, i.e., HISAC shall include both monostatic mode ISAC channel and bistatic mode ISAC channel.

## Conclusion

In this contribution, it discusses the ISAC channel modelling issues with following proposals:

Proposal 1: At least the sensing resolution (also relevant with target size, simulation configuration parameter, etc) and the demand of the sensing service can be as two factors on selecting single or multiple scattering points for the sensing target modelling.

Proposal 2: It is suggested not to consider the interaction and relative position change of multiple scattering points belonging to the same sensing target.

Proposal 3: It is suggested not to model a propagation path from Tx to Rx interacting with more than one sensing target in the sensing channel modelling.

Proposal 4: Clarify that the agreements achieved in RAN1#119 on generating indirect paths of NLOS ray + NLOS ray in the target channel can be reused for indirect paths of NLOS ray + LOS ray or LOS ray + NLOS ray.

Proposal 5: For indirect path modelling in sensing target modelling, reducing the number of rays per cluster and/or reducing the number of clusters are supported.

Proposal 6: If NLOS path is modelled for target channel in some scenario, it is suggested that the NLOS can be modelled by EO besides using the stochastic mechanism, including EO type-1 and EO type-2.

Proposal 7: EO modelling can be enabled/disabled for ISAC channel modelling, which is up to the sensing evaluation scenario.

Proposal 8: For sensing pathloss, it can be denoted as𝑷LdBd1,d2=PLd1+PLd2-10log10RCS+10log10λ24π, where the d1 is the distance between sensing transmitter and sensing target, and d2 is the distance between the sensing receiver and sensing target, PL equation use the current pathloss model in 3GPP TRs as a starting point. FFS: how to consider the impact of the sensing target height on pathloss.

Proposal 9: For ISAC background channel modelling, two channel types are defined: environment object and random clutter.

- environment object: object(s) with known location, and modelling method is similar with the sensing target modelling, the difference is environment object is static in the whole simulation.

- random clutter: object(s) with known location, and it can be modelled using the stochastic mechanism of the existing 3GPP TR procedure e.g., TR 38.901, TR 37.885, TR 36.777.

Proposal 10: For ISAC random clutter for background channel modelling in TRP and UE mono-static sensing mode, randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario.

Proposal 11: For ISAC sensing target with direct path and static background environment (i.e., environment object), both naturally meet the spatial consistency since the sensing parameters are calculated based on the deterministic/known information.

Proposal 12: For ISAC sensing target with indirect path, each link directly reuse the TR 38.901 spatial consistency procedure, then combining the two links to generate the target indirect path.

Proposal 13: For the spatial consistency of ISAC random background clutter, taking TR 38.901 spatial consistency procedure as a starting point.

Proposal 14: For sensing LLS channel generation, using the simplified SLS to generate the channel coefficient, then fed into the LLS simulator. FFS: how to simplify the SLS procedure for sensing LLS.

Proposal 15: From ISAC channel modelling perspective, a common channel modelling framework is defined for all six sensing modes, i.e., HISAC shall include both monostatic mode ISAC channel and bistatic mode ISAC channel.

## Reference

- RP-234069, “New SID: Study on channel modelling for Integrated Sensing and Communication (ISAC) for NR”, Nokia, Nokia Shanghai Bell.

- RAN1 Chair’s Notes, RAN1#119 meeting, November 2024.

- RAN1 Chair’s Notes, RAN1#116bis meeting, April 2024.

- R1-2410014, Summary #4 on ISAC channel modelling, November 2024.

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”, V17.1.0 (2024-01)