3GPP TSG-RAN WG1 Meeting #120	R1-2501323

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.14

Source:	Huawei, HiSilicon, China Unicom, ZTE, Sanechips, CATT

Title:	Enhancement of SRS beamforming for FR2 positioning

Document for:	Discussion and Decision

# Introduction

In the RAN1#118bis meeting, RAN1 discussed the TEI proposal of SRS beamforming enhancement in  [1] and [2]. The discussion has been going on with the comments received considered.

The proposal in the contribution is the updated version with the comments addressed. Details will be elaborated in the paper.

# Discussion

For mmWave, the SRS transmission is directional with beamforming, resulting in less spatial abundance to enable positioning. Therefore, the SRS beam sweeping transmission similar to the BM-SRS is desired.

However, directly reusing BM-SRS does not work for positioning, given that BM-SRS, as part of the MIMO SRS, does not support additional positioning related features, such as bandwidth aggregation and transmission in RRC_INACTIVE state.

Fig. 1 shows the observed CIR under different beamformers assuming 4x1 antenna array at UE side. It can be seen that with more Tx beam attempts, more useful information can be extracted.

Additional Path Search Window

Additional Path Beamforming

LoS Path Beamforming

Fig. 1 Observed CIR using different beamformers

With that, single-BS positioning with high accuracy is possible where only two SRS resources (each corresponding to a Tx beam) are used.

Tab. 1 shows the evaluation results of single-BS RTT+AoA positioning, where the RTT group delay error is assumed with Gaussian distribution with the std of 2ns. It is worth noting that 2ns is already a very stringent requirement from the perspective of BS/UE group delay calibration.

It can been that 0.2m and 0.1m accuracy can be achieved under 400MHz and 800MHz, respectively, using two selected SRS beams, while the group delay error dominates the error if only single SRS beam is used.

Tab. 1 Evaluation results for single-BS RTT+AoA positioning

| Location evaluation InF-DH: (Central frequency=30GHz, ISD=20m) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Single-BS, RTT+AOA | Bandwidth | Group delay error STD | 50% | 67% | 80% | 90% |
| Horizontal accuracy | Single SRS beam (LoS) | 400MHz | 2ns | 0.326 | 0.525 | 0.695 | 0.875 |
|  |  | 800MHz | 2ns | 0.317 | 0.513 | 0.682 | 0.857 |
|  | Two SRS beams (LoS+MP) | 400MHz | 2ns | 0.029 | 0.051 | 0.085 | 0.157 |
|  |  | 800MHz | 2ns | 0.016 | 0.023 | 0.036 | 0.063 |

Observation 1: Positioning SRS transmission using beam sweeping is desired for positioning feature.

In the current specification, for positioning SRS not configured with spatial relation, it is up to UE to choose the SRS spatial relation as in [TS 38.214] as follows:

If the UE is not configured with the higher layer parameter spatialRelationInfoPos the UE may use a fixed spatial domain transmission filter for transmissions of the SRS configured by the higher layer parameter SRS-PosResource across multiple SRS resources or it may use a different spatial domain transmission filter across multiple SRS resources.

Nevertheless, this UE implementation is not friendly to network to perform positioning. For example, for some UE implementations, when the spatial relation RS is not configured, UE may be prone to transmit the SRS resources using the spatial domain transmission filters based on the Rx beamformed RSRP from serving cell downlink RS, resulting in a fixed SRS Tx beam. However, the network expectation would be rather requesting UE to transmitting SRS in a way that explores the spatial diversity, e.g. based on blind beam sweeping, or path-power based beam correspondence.

In addition, a fixed spatial domain transmission filter could already be realized by configuring multiple positioning SRS resources with the same SRS resource as the spatial relation RS as exemplified in Fig. 2.

SRS resource #0

SRS resource #1

SRS resource #2

spatialRelationInfoPos not configured

spatialRelationInfoPos = SRS resource #0

spatialRelationInfoPos = SRS resource #0

Fig. 2: Realizing the same spatial transmission filer

Observation 2: There exists a configuration method to allow multiple SRS resources transmitted using the fixed beam, e.g. by configuring one SRS resource as the spatial relation RS of other SRS resources in the resource set.

Overall, towards boosting the mmWave positioning performance, the issue would like to be addressed is the case when UE is “not configured with the spatial relation RS” but UE is expected to use different beams for SRS transmissions.

The solution is defining a new UE capability based on which network can indicate UE to transmit SRS using a different spatial domain transmission filter across multiple SRS resources.

Proposal 1: Introduce a new RRC parameter for a positioning SRS resource set, indicating that the UE is expected to use different spatial transmission filters for the positioning SRS resources not configured with spatialRelationInfoPos.

  - A per-band UE capability is introduced.

During RAN1#119, there were comments on the benefit of such a proposal and also the comment on potential impact on the power control. We would like to provide the following responses.

  - Benefit: It should be well acknowledged that beam coverage across spatial domain is important for mmWave positioning, for both single-BS positioning and multi-BS positioning. With the TEI, we aim to fix the ambiguity of the existing beam sweeping configuration. It can be jointly used with beam repetition by, e.g., configuration of a first subset of resources without spatial relation for beam sweeping and a second subset of resources with spatial relation each pointing to a SRS resource in the first set as repetition, which cannot be achieved with the existing specification, as shown in Fig. 3.

  - Power control: In general, there is no impact on the power control, as the pathloss RS and other open loop PC parameters are still resource set level, and are based on RSRP of the DL RS. In fact, keeping the same transmission power for the SRS resources in an SRS resource set is important, so that network could benefit from the receive power difference to infer the pathloss difference.

SRS resource #0

SRS resource #1

SRS resource #2

spatialRelationInfoPos not configured

spatialRelationInfoPos = SRS resource #0

spatialRelationInfoPos = SRS resource #0

SRS resource #3

SRS resource #4

SRS resource #5

spatialRelationInfoPos not configured

spatialRelationInfoPos = SRS resource #3

spatialRelationInfoPos = SRS resource #3

Beam 1

Beam 2

Fig. 3: Configuration of SRS with beam sweeping and repetition

Proposal 2: Endorse the TP in Annex.

# Conclusion

The TEI proposal is updated as follows:

Proposal 1: Introduce a new RRC parameter for a positioning SRS resource set, indicating that the UE is expected to use different spatial transmission filters for the positioning SRS resources not configured with spatialRelationInfoPos.

  - A per-band UE capability is introduced.

Proposal 2: Endorse the TP in Annex.

# Reference

- R1-2408177, Enhancement of positioning SRS beam configuration, Huawei, HiSilicon, China Unicom, ZTE, Sanechips, RAN1#118bis, Hefei, China, October 14-18, 2024.

- R1-2409094, FL Summary #1 on Rel-19 TEIs, Moderator (NTT DOCOMO, INC.), RAN1#118bis, Hefei, China, October 14-18, 2024.

# Annex

========================= Unchanged parts =========================

#### 6.2.1.4	UE sounding procedure for positioning purposes

When the SRS is configured by the higher layer parameter SRS-PosResource and if the higher layer parameter spatialRelationInfoPos is configured, it contains the ID of the configuration fields of a reference RS according to Clause 6.3.2 of [TS 38.331]. The reference RS can be an SRS configured by the higher layer parameter SRS-Resource or SRS-PosResource, CSI-RS, SS/PBCH block, or a DL PRS configured on a serving cell or a SS/PBCH block or a DL PRS configured on a non-serving cell. If the UE is configured for transmission of SRS-PosResource in RRC_INACTIVE mode, the configured spatialRelationInfoPos is also applicable.

The UE is not expected to transmit multiple SRS resources with different spatial relations in the same OFDM symbol.

If the UE is not configured with the higher layer parameter spatialRelationInfoPos the UE may use a fixed spatial domain transmission filter for transmissions of the SRS configured by the higher layer parameter SRS-PosResource across multiple SRS resources or it may use a different spatial domain transmission filter across multiple SRS resources.

Subject to UE capability, if the UE is configured with the high layer parameter [DifSplFiler] for a SRS resource set configured by SRS-PosResourceSet, the UE is expected to use different spatial domain transmission filters across the SRS resources that are not configured with the higher layer parameter spatialRelationInfoPos in the SRS resource set.

In RRC_CONNECTED mode, the UE is only expected to transmit an SRS configured by the higher layer parameter SRS-PosResource within the active UL BWP of the UE.

When the configuration of SRS is done by the higher layer parameter SRS-PosResource, the UE can only be provided with a single RS source in spatialRelationInfoPos per SRS resource for positioning.

For operation on the same carrier, if an SRS configured by the higher parameter SRS-PosResource collides with a scheduled PUSCH, the SRS is dropped in the symbols where the collision occurs.

Unless specified otherwise, the UE does not expect to be configured with SRS-PosResource on a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission.

========================= Unchanged parts =========================