3GPP TSG RAN WG1 #120		R1- 2501514

Athens, Greece, 17 – 21 February, 2025

Source:	Moderator (Nokia)

Title:	[120-Pre-R18-NR] Slot aggregation configuration with multi-PUSCH

Agenda Item:	7

Release:	Release 17

WI code:	NR_ext_to_71GHz-Core

Document for:	Discussion and Decision

# Introduction

RAN1#120  discussed the specification conflict related to the configuration of pusch-AggregationFactor and pusch-TimeDomainAllocationListforMultiPUSCH [1]. Contribution suggested two alternatives, either

- Make the Rel-16 restriction conditional to the UE not being configured with extendedK2, so that the additional text introduced in Rel-17 would be applicable when extendedK2 is provided, or.

- Make the Rel-16 restriction also applicable in Rel-17 and eliminate the conflicting definition where the behaviour for the simultaneous configuration is defined for the case where extededK2 is provided.

The discussion in the RAN1#120 Monday pointed to the direction where it is clarified that the Rel-16 restriction is applicable also in later releases.

# Moderator proposals

Proposal 1:

Adopt the following clarification in Rel-17 TS38.214 subclause 6.1.2.1 Resource allocationin time domain:

If a UE is configured with extendedK2 in pusch-TimeDomainAllocationListForMultiPUSCH in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPUSCH.

Please provide you views on the proposal above

| Company | Comments |
| --- | --- |
| ASUSTeK | It’s our opinion that this change (Alt2 in R1-2501268) will revert previous RAN1 #107e agreement. Agreement (RAN1#107-e) - If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PDSCH for DCI format 1_1, the UE does not expect to be configured with repetitionNumber for the TDRA table, and if pdsch-AggregationFactor is configued in PDSCH-config, it does not apply to DCI format 1_1. - Note: repetitionNumber cannot be configured with pdsch-TimeDomainAllocationListDCI-1-2 as in Rel-16. - Note: Under agenda item 8.2.4, in RAN1#106-bis, it was already agreed that within the TDRA table for multi-PDSCH scheduling, the UE does not expect to be configured with the higher layer parameter repetitionNumber. - Note: These does not preclude pdsch-AggregationFactor can be configured and applies to DCI format 1_2 - If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PUSCH for DCI format 0_1, the UE does not expect to be configured with numberOfRepetitions for the TDRA table, and if pusch-AggregationFactor is configued in PUSCH-config, it does not apply to DCI format 0_1. - Note: These does not preclude numberOfRepetitions is configured for TDRA table corresponding to DCI format 0_2 Note: These does not preclude pusch-AggregationFactor can be configured and applies to DCI format 0_2 We agree what Nokia proposed is a valid issue, however, it’s more reasonable to make change without reverting what we have in Rel- 17. It’s more reasonable to support Alt1 in R1-2501268. |
| Ericsson | We can live with Proposal 1. What it means is that if a TDRA table is configured for scheduling multiple PUSCHs with DCI 0_1, then it will not be possible to simultaneously configure aggregationFactor to apply to (single) PUSCH scheduled with DCI 0_2. However, this does not seem to preclude use cases of practical interest. |
| LG Electronics | Proposal 1 is okay to us considering less specification impact and practical interest. |
| Samsung | We slightly prefer not to revert RAN1’s agreement. From our perspective, we support the same principle to both multi-PDSCH scheduling and multi-PUSCH scheduling. To be clear, pdsch-AggregationFactor can be configured and applied to DCI format 1_2 as agreed. |
| Xiaomi | We prefer to not revert RAN1 agreement. But we can also live with the proposal if supported by majority companies. |
| MTK | We can be fine with Proposal 1. Prefer to also leave a note in Chairman’s note that the corresponding RAN1 #107-e agreement is reverted with this change. |

Proposal 2:

Agree to the following for the CR cover page:

| Title: | Correction of simultaneous configuration of pusch-AggregationFactor and pusch-TimeDomainAllocationListforMultiPUSCH |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Nokia), xxxxxxx |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  | Date: | 2024-02-21 |
| Category: | F |  |  |  | Release: | Rel-17 |

…

| Reason for change: | The current specification in 6.1.2.1 prevents the simulatenous configuration of the two parameters: If a UE is configured with higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, the UE does not expect to be configured with pusch-AggregationFactor. And in the following paragraphin 6.1.2.1 states the UE is not to apply the aggregation factor when configured simultaneoustly with pushTimeDomainAllocationListforMultiPUSH and with extendedK2. If a UE is configured with extendedK2 in pusch-TimeDomainAllocationListForMultiPUSCH in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPUSCH. The two statements are in conflict as the first one prevents the second one. |
| --- | --- |
| Summary of change: | Removing the conflicting “the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and” to make the “the UE does not expect to be configured with push-AggregationFactor” statement apply also in the second paragraph. |
| Consequences if not approved: | Conflicting definitions for the simultaneous configuration of pusch-AggregationFactor and pusch-TimeDomainAllocationListforMultiPUSCH remains. |

Please provide you views on the proposal above

| Company | Comments |
| --- | --- |
| MTK | Fine with the proposal. |

## For Tuesday online

No unanimous support for reverting the RAN1#107 agreement and make the simpler cmodification as proposed.

Agree wither Alt1 or Alt2 of the original proposal in [1]:

Alternative 1: Fix the specification to enable the operation according to the RAN1#107-e agreement by clarifying that the pusch-AggregationFactor is not configured with pusch-TimeDomainAllocationListForMultiPUSCH (i.e. Rel-16 behavior), if extendedK2 (i.e. Rel-17 operation) is not configured. This would need to be captured in the first statement in TS 38.214:

If a UE is configured with higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH but not configured with extendedK2 in pusch-TimeDomainAllocationListForMultiPUSCH, the UE does not expect to be configured with pusch-AggregationFactor.

And propagate the same change also to the TS 38.331 RRC parameter field description:

pusch-TimeDomainAllocationListForMultiPUSCH

Configuration of the time domain resource allocation (TDRA) table for multiple PUSCH (see TS 38.214 [19], clause 6.1.2). The network configures at most 64 rows in this TDRA table in PUSCH-TimeDomainResourceAllocationList-r16 configured by this field. This field is not configured simultaneously with pusch-AggregationFactor if extendedK2 is not configured. The network does not configure the pusch-TimeDomainAllocationListForMultiPUSCH-r16 simultaneously with the pusch-TimeDomainAllocationListDCI-0-1-r16. The network does not configure the pusch-TimeDomainAllocationListForMultiPUSCH-r16 simultaneously with the numberOfSlotsTBoMS-r17.

Alternative 2: Keep the Rel-16 behavior of not allowing to configure the UE simultaneously with pusch-AggregationFactor and pusch-TimeDomainAllocationListForMultiPUSCH currently reflected in 38.214 and 38.331 / RRC and therefore override the RAN1#107-e agreement. Just remove the conflicting parts from the second statement in 38.214:

If a UE is configured with extendedK2 in pusch-TimeDomainAllocationListForMultiPUSCH in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPUSCH.

# References

| [1] | R1-2501268 | Correction to slot aggregation configuration with multi-PUSCH | Nokia |
| --- | --- | --- | --- |