3GPP TSG RAN WG1 #120b			        R1-2501748

Wuhan, China, April 7th – 11th, 2025

Agenda Item:		9.2.3

Source:				Moderator (InterDigital, Inc.)

Title:						Summary of Discussion on 3TX CB-based Uplink in RAN1#120bis

Document for:		Discussion and Decision

# Background

RANP #102 approved the WID for NR MIMO Phase 5 [1]. The WID covers five main objectives, where one of the described objectives is to specify 3-antenna-port codebook-based transmissions. In RANP #105, the WID was revised to including some additional related enhancements, as highlighted below [2],

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource - Note: UL full power transmission mode 1 and 2 are not supported. - Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

Per revision, support of the following enhancements for a 3TX UE are to be discussed and specified in RAN1,

- SRS antenna switching operation for 3T6R,

- SRS antenna switching operation for 3T3R,

- Non-codebook PUSCH transmission.

# RAN1 Agreements for Sub-agenda 9.2.3

RAN1 #116

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for single-layer transmission.

13100,13010,13001

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for two-layer transmission.

13100100, 13100001, 13 001001

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for three-layer transmission.

13100010001

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, down-select one of

- Alt1 – Support configuration of X 4-port SRS resources in a resource set where one the ports is muted

- Alt2 – Support configuration of X SRS resources with equal/unequal number of ports (e.g. 2 + 1 or 1 + 1 + 1) in a resource set,

The value for X is FFS, and it will be determined according to the selected alternative.

Agreement

For a 3TX UE, down-select one of the following options for the number of PTRS ports,

- Option-1: A single PTRS port is supported.

- Option- 2: Up to 2 PTRS port may be configured.

Agreement

For a 3-antenna-port codebook-based UL transmission, study PTRS-DMRS association.

Agreement

For a 3-antenna-port codebook-based UL transmission, study power split for each port of SRS and PUSCH.

Agreement

For codebook-based uplink transmission by a 3TX UE, support full-power Mode 0, subject to UE capability.

Conclusion

There is no consensus in RAN1 to support antenna switching for 3TX UE in Rel-19

Agreement

For performance evaluation of 3TX UE, adopt the following Table as the reference EVM for LLS evaluation

- Companies may provide additional evaluation results per their case of interest

- LLS is optionally used for 3Tx UL evaluation, if needed

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Waveform | CP-OFDM |
| SCS | 30 KHz |
| System bandwidth | 20 MHz, 100 MHz |
| Scheduled PRBs | 5, 25, 50, 260 PRBs |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ |
| UE speed | 3 Km/h |
| Number of Layers | Adaptive, Fixed (reported by company) |
| AMC | Adaptive, Fixed (reported by company) |
| DMRS configuration | Type 1; 1 front loaded + 1 additional symbol |
| Channel estimation | Real |
| Channel Model | CDL-A (30ns), CDL-B (100ns), CDL-C (300ns) |

Agreement

For performance evaluation of 3TX UE, adopt the following Table as the reference EVM for SLS evaluation.

- Companies may provide additional evaluation results per their case of interest.

Note: The considered EVM is to be used as a baseline set of assumption for future potential studies related to 3TX.

| Parameter | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | 14 CP-OFDM symbol slot SCS , 30 KHz |
| Scenario | eMBB: Dense Urban (200m), 3.5GHz Outdoor FWA: UMa (500m), 3.5GHz |
| UE Outdoor/Indoor (%) | eMBB: 80%, 20% Outdoor FWA: 100%, 0% |
| System bandwidth | 20 MHz, 100 MHz |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Optional: Classical: two 8x1 xpols, 4λ apart; 4 TXRUs tilt=[104°] |
| gNB antenna radiation pattern parameters | - Outdoor/Indoor Per 38.901, Table 7.3-1 |
| gNB receiver noise figure | 5dB |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | - Up to 64 QAM - Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation |
| UE speed | 3 Km/h |
| UE antenna config | eMBB: - Xpol+1pol; isotropic ULA - Xpol+1pol; 110°, 4 dBi Outdoor FWA: - Xpol+1pol; isotropic ULA - 3 directional 1pol: 110°, 4 dBi |
| Traffic model | - FTP model 1: Packet size 500KB, RU= 50% and suggested low/high RU of values of 20% and 70% - Full buffer (optional) |
| Suggested benchmarking | Rel-15 2Tx non-coherent |
| Precoder granularity | Wideband |
| Power control | Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario |
| UE power rating | eMBB: 23 dBm, UL FPTx mode 0 or Rel-15 power scaling Outdooe FWA: 31 dBm, UL FPTx mode 0 |
| Metric | UL mean-user throughput, 5%-ile and 95%-ile UPT |

Agreement

For performance evaluation of 3TX UE, consider following reference configurations,

- A linear array (1D) of single-polarized antenna configuration with a spacing of 0.5λ,

  - For example:		 |←0.5λ→|←0.5λ→|

- A configuration of a cross-polarized and a single-polarized antennas,

  - For example:		 ×←0.5λ → ⁄

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, one SRS resource set is configured for single TRP operation.

Agreement

For codebook-based transmission by a 3TX UE,

- Only PUSCH antenna ports 1000, 1001, 1002 are used

- Option- 2: Subject to UE capability, up to 2 PTRS ports may be configured in PTRS-UplinkConfig,

  - FFS whether a single bit or 2 bits are used for PTRS-DMRS association indication.

Above is only for single panel transmission.

RAN1 #116-bis

Agreement

To support codebook-based UL transmission by a 3TX UE, the agreed rank1 precoders in RAN1#116 can also be used when transform precoding is enabled (DFT-s-OFDM ).

Agreement

To indicate precoding information for codebook-based UL transmission by a 3TX UE,

- Reuse legacy TPMI indication framework where TPMI and TRI are jointly indicated

- TPMI field is 2 or 3bits for 3-antenna-port transmission

  - For maxRank equals to 1, TPMI field is 2 bits for DFT-s-OFDM and CP-OFDM

  - For maxRank equals to 2 or 3, TPMI field is 3 bits for CP-OFDM

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, support Alt1,

- Alt1: Support configuration of X 4-port SRS resources in a resource set where one the ports is muted

  - FFS muting mechanism

where X can be up to 2, subject to UE capability.

Agreement

- For codebook-based UL transmission by a 3TX UE, when 2 PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, PTRS-DMRS association indication is as follows:

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |

- Note: PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT_RS port 0, and PUSCH antenna port 1001 is associated with PT_RS port 1

- Number of bits used for the indication

  - 1 bit

Agreement

For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource,

- Option 3: Always a same port is muted, e.g., the 4th port

Agreement

For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, UE splits a linear SRS power equally across the 3 unmuted antenna ports of the 4-port SRS resource.

Agreement

For 3-port codebook-based PUSCH transmission for a 3TX UE, scale factor s should be the ratio of the number of antenna ports with a non-zero PUSCH transmission power to 3 (except for full-power Mode 0).

- FFS: Whether specification needs to be updated to reflect the above

Agreement

For codebook-based UL transmission by a 3TX UE, when 1 PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, PTRS-DMRS association indication is as follows:

- Alt2: 2-bit indication

PTRS-DMRS association when 1 PT-RS port is configured

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port |
| 1 | 2nd scheduled DMRS port |
| 2 | 3rd scheduled DMRS port |
| 3 | 4th scheduled DMRS port Reserved |

Agreement

For a 3TX UE, support Rel-17 M-TRP PUSCH repetition where,

- Two SRS resource sets, each with up to 2 of 4-port SRS resources are configured,

Note: The configured 4 port SRS resources are used to enable 3-port SRS transmission

RAN1 #117

Agreement

Update the agreement made in RAN1 #116bis as the following.

Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, Option 3: Always a same port is muted, e.g., i.e., the 4th port

Agreement

For codebook-based M-TRP PUSCH repetition by a 3TX UE, scheduled by DCI format 0_1/0_2,

- Reuse Rel-17 M-TRP PUSCH repetition design, where the second precoding information field only indicates TPMI index, and applies same rank as indicated by the first precoding information field.

Agreement

For codebook-based M-TRP PUSCH repetition by a 3TX UE, scheduled by DCI format 0_1/0_2,

- Introduce new tables as Table I, II, III for the second precoding information field, for maxRank=1 or 2 or 3, respectively.

  - Table I: Second precoding information for 3 antenna ports if maxRank=1

| Bit field | codebookSubset=NonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | Reserved |

  - Table II: Second precoding information for 3 antenna ports if maxRank=2

| Bit field | codebookSubset=NonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | 1 layer: Reserved |
| 0 | 2 layer: TPMI=0 |
| 1 | 2 layer: TPMI=1 |
| 2 | 2 layer: TPMI=2 |
| 3 | 2 layer: Reserved |

  - Table III: Second precoding information for 3 antenna ports if maxRank=3

| Bit field | codebookSubset=NonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | 1 layer: Reserved |
| 0 | 2 layer: TPMI=0 |
| 1 | 2 layer: TPMI=1 |
| 2 | 2 layer: TPMI=2 |
| 3 | 2 layer: Reserved |
| 0 | 3 layer: TPMI=0 |
| 1~3 | 3 layer: reserved |

Agreement

For codebook-based M-TRP PUSCH repetition by a 3TX UE, for indication of PTRS-DMRS association,

- When 1 PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, reuse Rel-17 multi-TRP TDM repetition,

- When 2 PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, and maxRank = 2 or 3,

  - A second PTRS-DMRS association field (1 bit) is used to indicate the association between PTRS ports and DMRS ports for the 2nd SRS resource set (i.e.,2nd TRP).

Agreement

For codebook-based UL transmission by a 3TX UE, subject to its capability,

- A 3TX UE may report a maximum number of 3 layers

- A 3TX UE may report a maximum number of SRS ports of up to 3

Note: SRS resource definition is not changed nor the number of SRS ports in the SRS resource.

RAN1 #118

Conclusion

Support to reuse SRS features (i.e., cyclic shift hopping, and comb offset hopping) in Rel-18 for SRS resource set with usage of codebook based 3TX PUSCH transmission, without introducing new UE capabilities (i.e., reusing Rel-18 UE capabilities).

- Note: The conclusion does not include the Rel-18 8-port-specific SRS features.

Agreement

For a UE reporting UE capability of 3TX operation, support introduction of a new RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled.

RAN1 #118-bis

Agreement

To support 3T6R antenna switching for a 3TX UE,

- The resourceType for a configured SRS resource set can be one of ‘periodic’, ‘semi-persistent’ and ‘aperiodic’,

- Reuse the principles used for 4T8R, i.e.,

  - For 4T8R 3T6R, zero or one or two SRS resource sets configured with a different value of resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with 'semi-persistent' and up to one SRS resource set configured with 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has two SRS resources, transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports with port 1003 disabled, and the SRS ports of the resource in the set are associated with a different UE antenna ports, or

  - For 4T8R 3T6R, zero or one or two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of one resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports with port 1003 disabled, and the SRS ports of the resource in the set are associated with a different UE antenna ports. In the case of two resource sets a total of two SRS resources are transmitted in different symbols of two different slots, and where the SRS ports of each SRS resource in the given two sets are associated with a different UE antenna ports.

Note: This is not meant to be a text proposal to be captured for the specifications.

Agreement

To support non-codebook-based PUSCH transmission by a 3TX UE,

- UE reports a capability of up to a maximum of 3 layers.

Conclusion

To support non-codebook-based precoding by a 3TX UE, for SRI indication, re-use the legacy-based solution according to NSRS ≤ maxNumberSRS-ResourcePerSet.

Conclusion

To support non-codebook-based PUSCH transmission by a 3TX UE,

- For sTRP, a single SRS resource set, with up to NSRS ≤ maxNumberSRS-ResourcePerSet single-port SRS resources, can be configured,

- For Rel-17 M-TRP PUSCH repetition, two SRS resource sets, each with up to NSRS ≤ maxNumberSRS-ResourcePerSet single-port SRS resources, can be configured.

RAN1 #119

Conclusion

To support 3T6R antenna switching for a 3TX UE,

- Reuse the power control parameters configuration principles of 4T8R for 3T6R, i.e., when two aperiodic SRS resource sets are configured,

  - if a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State, the UE shall expect that the two SRS resource sets are associated with the same values of the higher layer parameters p0AlphaSetforSRS and pathlossReferenceRS-Id

  - Otherwise, the UE shall expect that the two SRS resource sets are configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet.

Agreement

The following capabilities are introduced for Rel-19

- 't3r3' for 3T3R.

- 't3r6' for 3T6R.

Agreement

Support the following replies to Questions 2 and 3 in RAN4 LS,

Question 2: RAN4 would like to check with RAN1 whether a 3Tx UE supporting UL full power transmission mode 0 needs to achieve the same maximum output power with both 1Layer and 2Layers. Question 3: RAN4 would like to check with RAN1 whether 3Tx UE can support UL full power transmission mode 0 with 2Layer 3-port codebooks but not support 1Layer 3-port codebooks.

- Reply 1: Yes, a 3Tx UE supporting UL full power transmission mode 0, delivers the same maximum output power with both 1- and 2-layer precoders.

- Reply 2: No, a 3Tx UE supporting UL full power transmission mode 0, shall support full power with both 1- and 2-layer 3-port codebook.

Agreement

Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement in endorsed in R1-2410870

Agreement

For a UE operating with 3TX,

- the RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled, is applied for all SRS resource set(s) for SRS usage set as codebook.

- the RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled, is applied for all SRS resource set(s) for SRS usage set as antenna switching.

- Note: The RRC parameter mentioned in the above two bullets is configured per set

A UE does not expect an SRS resource to be configured in both codebook and antenna switching resource sets with conflicting (disabled state) configuration for the port 1003.

RAN1 #120

Agreement

For 3T3R antenna switching, reuse the same mechanism as xTxR for the SRS configuration as follows:

- Up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 4 with port 1003 disabled if the UE is not indicating srs-AntennaSwitching2SP-1Periodic.

- Up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 4 with port 1003 disabled.

- For 3T=3R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same symbol.

Note: This is not meant to be a text proposal to be captured for the specifications.

Conclusion

To support 3T3R antenna switching for a 3TX UE,

- Reuse the power control parameters configuration principles, i.e., 3T6R, when two aperiodic SRS resource sets are configured,

  - If a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State, the UE shall expect that the two SRS resource sets are associated with the same values of the higher layer parameters p0AlphaSetforSRS and pathlossReferenceRS-Id

  - Otherwise, the UE shall expect that the two SRS resource sets are configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet.

Conclusion

For a 3TX UE, following agreements for codebook-based transmission can also be extended for non-codebook-based transmission,

Agreement For codebook-based transmission by a 3TX UE, - Option- 2: Subject to UE capability, up to 2 PTRS ports may be configured in PTRS-UplinkConfig, - FFS whether a single bit or 2 bits are used for PTRS-DMRS association indication. Above is only for single panel transmission. Agreement For codebook-based UL transmission by a 3TX UE, when 1 PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, PTRS-DMRS association indication is as follows: - Alt2: 2-bit indication PTRS-DMRS association when 1 PT-RS port is configured Value; DMRS port. 0; 1st scheduled DMRS port. 1; 2nd scheduled DMRS port. 2; 3rd scheduled DMRS port. 3; 4th scheduled DMRS port Reserved.

Agreement

For non-codebook-based UL transmission for a 3TX UE, when 2 PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, PTRS-DMRS association indication for the two DMRS ports that share a PTRS port is as follows:

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares the PTRS port | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares the PTRS port | 1 | 2nd DMRS port which shares PTRS port 1 |

- Number of bits used for the indication

  - 1 bit

- Above is applicable when a 3TX UE is configured with 3 SRS resources for non-codebook-based UL transmission

Conclusion

For a 3TX UE, following agreements for codebook-based transmission can also be extended for non-codebook-based transmission,

Agreement For codebook-based M-TRP PUSCH repetition by a 3TX UE, for indication of PTRS-DMRS association, - When 1 PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, reuse Rel-17 multi-TRP TDM repetition, - When 2 PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, and maxRank = 2 or 3, - A second PTRS-DMRS association field (1 bit) is used to indicate the association between PTRS ports and DMRS ports for the 2nd SRS resource set (i.e.,2nd TRP).

Agreement

For 3T3R and 3T6R antenna switching, support the following separate UE capabilities for SRS configuration,

- “srs-AntennaSwitching3T6R2SP-1Periodic” to extend srs-AntennaSwitching2SP-1Periodic to the case of 3T6R.

- “srs-AntennaSwitching3T3R2SP-1Periodic” to extend srs-AntennaSwitching2SP-1Periodic to the case of 3T3R.

RAN1 #120-bis

Agreement

Adopt the following changes in Clause 6.3.1.5 in TS38.211.

#### 6.3.1.5 Precoding The block of vectors y0i…yυ-1iT shall be precoded according to z(p0)i⋮z(pρ-1)i=Wy(0)i⋮yυ-1i where i=0,1,…,Msymbap-1, Msymbap=Msymblayer. The set of antenna ports p0,…,pρ-1 shall be determined according to the procedure in [6, TS 38.214]. For non-codebook-based transmission, the precoding matrix W equals the identity matrix. For codebook-based transmission, the precoding matrix W depends on the number of antenna ports used for the transmission: - for single-layer transmission on a single antenna port, W=1; - for transmissions using 2, or 4 antenna ports, W is given by Tables 6.3.1.5-1 to 6.3.1.5-7; - for transmissions using 3 antenna ports when 4portSRS_3TX is configured, the UE does not transmit on antenna port 1003 and W is given by Tables 6.53.1.5-48 to 6.3.1.5-50;

Agreement

Adopt the following changes in Clause 6.2.1.2 in TS38.214.

#### 6.2.1.2 UE sounding procedure for DL CSI acquisition -------------------------------------------Unchanged parts are omitted------------------------------------------- - For 1T=1R, 2T=2R, 3T=3R, 4T=4R or 8T=8R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2, 4 with port 1003 disabled by higher layer parameter [RRC_parameter] is configured, 4 or 8 if the UE is not indicating srs-AntennaSwitching2SP-1Periodic or [srs-AntennaSwitching3T3R2SP-1Periodic]. Up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating srs-AntennaSwitching2SP-1Periodic, srs-AntennaSwitching8T8R2SP-1Periodic, or [srs-AntennaSwitching3T3R2SP-1Periodic], where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, 4 with port 1003 disabled by higher layer parameter [RRC_parameter] is configured, 4, or 8 or -------------------------------------------Unchanged parts are omitted------------------------------------------- - For 3T6R, zero or one or two SRS resource sets configured with a different value of resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' and with higher layer parameter [RRC_parameter] is configured if the UE is not indicating [srs-AntennaSwitching2SP-1Periodic] or [srs-AntennaSwitching3T6R2SP-1Periodic], or up to two SRS resource sets configured with 'semi-persistent' and up to one SRS resource set configured with 'periodic' if the UE is indicating [srs-AntennaSwitching2SP-1Periodic] or [srs-AntennaSwitching3T6R2SP-1Periodic], where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports with port 1003 disabled, and the SRS ports of the resource in the set are associated with a different UE antenna ports, or - For 3T6R, zero or one or two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic' and with higher layer parameter [RRC_parameter] is configured, where in the case of one resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports with port 1003 disabled, and the SRS ports of the resource in the set are associated with a different UE antenna ports. In the case of two resource sets a total of two SRS resources are transmitted in different symbols of two different slots, and where the SRS ports of each SRS resource in the given two sets are associated with a different UE antenna ports. -------------------------------------------Unchanged parts are omitted-------------------------------------------

Agreement

Adopt the following changes in Clause 6.2.1 in TS38.214.

### 6.2.1 UE sounding procedure -------------------------------------------Unchanged parts are omitted------------------------------------------- [The UE does not transmit SRS on antenna port 1003 if [RRC_parameter] is configured.] where [RRC_parameter] is only applicable when SRS resource(s) are configured with 4 ports with usage as codebook or antenna switching. -------------------------------------------Unchanged parts are omitted-------------------------------------------

Agreement

Adopt the following changes in Clause 7.1 in TS38.213.

## 7.1 Physical uplink shared channel For a PUSCH transmission on active UL BWP b, as described in clause 12, of carrier f of serving cell c, a UE first calculates a linear value PPUSCH,b,f,c(i,j,qd,l) of the transmit power PPUSCH,b,f,c(i,j,qd,l), with parameters as defined in clause 7.1.1. For a PUSCH transmission scheduled by a DCI format other than DCI format 0_0, or configured by ConfiguredGrantConfig or semiPersistentOnPUSCH, if txConfig in PUSCH-Config is set to 'codebook', - if ul-FullPowerTransmission in PUSCH-Config is provided, the UE scales PPUSCH,b,f,c(i,j,qd,l) by s where: -------------------------------------------Unchanged parts are omitted------------------------------------------- - else, if each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of SRS ports supported by the UE in one SRS resource [if 4portSRS_3Tx is not provided, or to 3 if 4portSRS_3Tx is provided]. -------------------------------------------Unchanged parts are omitted-------------------------------------------

Agreement

For a 3TX UE, when configured to ‘nonCodebook’, to distinguish UE behavior for PTRS-DMRS association between the Rel-19 and Rel-18 behavior,

- Do not introduce a new RRC parameter, but enhance the definition of the RRC parameter 4portSRS_3Tx, for example,

  - ‘1-bit RRC parameter to indicate that port 1003 is disabled for all the 4-port resources in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, or to indicate 3-port transmission is enabled for a configured SRS resource set with usage ‘nonCodebook’.

# References

- RP-234007, “New WID: NR MIMO Phase 5”, Samsung, 3GPP RAN Meeting #112, December 11-15, 2023

- RP-242394, “WID revision: NR MIMO Phase 5”, RAN1 vice-chair (CMCC), September 2024

- R1-2501744, On Specifications of Rel-19 CB-based UL for 3TX UE	InterDigital, Inc.

- R1-2501782, Discussion on 3-antenna-port codebook-based transmissions	, ZTE Corporation, Sanechips

- R1-2501801, Remaining issues on 3-antenna-port codebook-based uplink transmissions, vivo

- R1-2501852, 3-antenna-port UL transmissions, MediaTek Inc.

- R1-2501941, Remaining issues for 3 Tx UL transmissions, Ericsson Japan K.K.

- R1-2501951, Uplink 3 Port Codebook based Transmission, Google

- R1-2501987, Remaining issues on 3-antenna-port uplink transmission, CATT

- R1-2502109	Support for 3-antenna-port codebook-based transmissions, Lenovo

- R1-2502155	Discussion on support for 3-antenna-port codebook-based transmissions, CMCC

- R1-2502226	Discussion on 3-antenna-port UL transmission, Huawei, HiSilicon

- R1-2502295	Discussion on 3-antenna-port uplink transmissions, OPPO

- R1-2502359	Views on Rel-19 3-antenna-port codebook-based transmissions, Samsung

- R1-2502436	Discussion on the support of 3-antenna-port CB based transmissions	, Xiaomi

- R1-2502526	On the support for 3-antenna-port codebook-based transmissions, Nokia

- R1-2502600	Remaining issues on R19 MIMO 3Tx transmission, Apple

- R1-2502667	Support for 3-antenna-port transmission, Sharp

- R1-2502761	Discussion on support for 3-antenna-port codebook-based transmissions, NTT DOCOMO, INC.