3GPP TSG RAN WG1 #120b  	          	              R1-2501767

Wuhan, China, April 7th–11th, 2025

Source: 	ZTE Corporation, Sanechips

Title:	Discussion on LP-WUS design

Agenda item: 	9.6.1

Document for:	Discussion and decision

Introduction

In RAN #105 meeting, a revised WID on low-power Wake-up Signal and Receiver for NR was approved in [1]. In RAN1 #120 meeting, the following agreements were made in [2].

Agreement For LP-WUS information carried by OOK, select alt 1 for codepoint to meet FAR and MDR performance. - Alt 1: Coding with rate matching if any - For more than 2 information bits: RM coding as in section 5.3.3.3 of 38.212 (FFS: whether scrambling in the same section is applied or not) - 2 information bits (if supported): coding as in section 5.3.3.2 of 38.212 (FFS: whether scrambling in the same section is applied or not) - 1 information bit (if supported): repetition as in section 5.3.3.1 of 38.212 (FFS: whether scrambling in the same section is applied or not) - FFS: Code block length - FFS: Further repetition before or after Manchester coding Conclusion For RRC idle/inactive, LP-WUS is not supported for the case where associated CD-SSB and initial DL BWP have different SCS. Agreement At least for M>1, for the overlaid OFDM sequence length, support - Alt2: - Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included) Agreement Update the agreements in RAN1 #118bis as below Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Applicable to both OOK-1 and OOK-4 - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 only and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences Agreement For RRC connected, for LP-WUS SCS: - Alt 1: LP-WUS SCS is same as the active DL BWP Working Assumption Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, - codepoint based - The maximum number of codepoints checked per MO by a UE is up to 8 - Depending on UE capability, a UE may support less than 8 Agreement For M=2, 4, is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) , - Note it doesn’t preclude any pulse shaping scheme if any. Agreement For RRC connected, LP-WUS frequency resource can be outside of active DL BWP but has to be within the same carrier as the active DL BWP - Basic capability is LP-WUS, if present, frequency resource within active DL BWP. LP-WUS frequency resource outside of active DL BWP is subject to separate UE capability - No RAN1 optimization specific to the case where LP-WUS frequency resource is outside of active DL BWP Agreement For M=1, L=4 (if supported), the set of LP-SS binary sequences is: [0 1 0 1] [0 1 1 0] [1 0 0 1] [1 0 1 0] Agreement For M=1, L=6 (if supported), the set of LP-SS sequence is: [1 0 1 0 1 0] [0 1 0 1 0 1] [1 0 0 1 0 1] [1 0 1 0 0 1] Agreement For M=1, L=8 (if supported), the set of LP-SS sequence is: [1 0 1 0 0 1 0 1] [1 0 1 0 1 0 0 1] [1 0 0 1 0 1 0 1] [0 1 0 1 0 1 0 1] Agreement For M=2, L=8 (if supported), the set of LP-SS sequence is: [0 1 0 1 1 0 0 1] [0 1 1 0 0 1 0 1] [0 1 1 0 1 0 0 1] [1 0 0 1 0 1 1 0] Agreement For M=2, L=12 (if supported), the set of LP-SS sequence is: [1 0 0 1 1 0 0 1 1 0 0 1] [0 1 1 0 1 0 0 1 1 0 0 1] [0 1 1 0 0 1 1 0 1 0 0 1] [0 1 1 0 0 1 0 1 1 0 0 1] Agreement For M=2, L=16 (if supported), the set of LP-SS sequence is: [1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1] [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1] [1 0 0 1 1 0 1 0 0 1 0 1 1 0 0 1] [1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1] Agreement For M=4, L=16 (if supported), the set of LP-SS sequence is down-selected between: - Set 1 [0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0] [0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0] [1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1] [1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0] - Set 2 [1 0 0 0 1 0 0 0 0 0 0 1 0 0 1 0] [1 0 0 0 0 1 0 0 1 0 0 0 0 0 1 0] [1 0 0 0 0 1 0 0 0 1 0 0 1 0 0 0] [1 0 0 0 0 1 0 0 0 0 0 1 0 0 0 1] Agreement For M=4, L=32(if supported), the set of LP-SS sequence is down-selected between: - Set 1: [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1] [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1] [0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0] [0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1] - Set 2 [0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 0 1 0 0] [0 0 0 1 0 1 0 0 1 0 0 0 1 0 0 0 0 1 0 0 0 1 0 0 0 0 0 1 0 1 0 0] [0 0 1 0 0 0 1 0 1 0 0 0 0 1 0 0 1 0 0 0 0 0 0 1 0 0 1 0 0 0 1 0] [0 0 1 0 0 1 0 0 0 0 0 1 1 0 0 0 0 0 1 0 1 0 0 0 0 1 0 0 0 0 1 0] Agreement For idle mode, regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip. Agreement For WUS information carried by the overlaid OFDM sequence(s), consider at least the following alternatives: - Alt 1: Raw information bits are mapped to sequence(s) - Alt 2: Raw information bits are mapped to sequence(s) after channel coding - Same channel coding scheme(s) as OOK is applied. - FFS same or different rate matching and/or repetition factor as OOK - Alt 3: Codepoint/Subgroup is mapped to sequence(s)

In this contribution, issues on waveform design and signal structure for LP-WUS/LP-SS are discussed.

Waveform generation

M values for OOK-4

In RAN1#118 meeting, RAN1 made following agreement.

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4

It is recommended that the same solution is supported for both 15kHz SCS and 30kHz SCS. With the same allocated resources and smaller residual time error, such as time error <=1us, LP-WUS with OOK-4 with M=4 can achieve the same or better detection performance than OOK-4 with M=1,2.

For the case of larger residual time error, LP-SS or additional sync signal can be used to ensure the residual time error before LP-WUS detection smaller than 1us. Moreover, as description in section 5.2, inserting Gaps at the beginning and the ending of each OOK symbol of OOK-4, can also be used to solve the issue of LP-WUS detection performance degradation caused by larger residual time error.

Therefore, multiple solutions can be used to solve the issue of time error for OOK-4. It is recommended to support M =4 for both 15kHz SCS and 30kHz SCS.

Proposal 1: Confirm the working assumption in below agreement:

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) FFS M=1 for OOK-4

In RAN1 #119 meeting, RAN1 made following agreement on the M value for LP-WUS and LP-SS.

Agreement For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120: - Alt1: The M values for LP-WUS and LP-SS are always same. - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

For Alt1, it restricts the gNB configuration which may cause resource wasting. For example, for LP-SS and M=1, using LP-WUS and M=1 would occupy more resources than M=2.

For Alt2, M value for LP-WUS is not larger than that of LP-SS which may benefit to the LP-WUS reception due to the timing drifting. However, when the M value is the same, it is still possible that the LP-WUS is out of sync after LP-SS for a time duration. Moreover, LP-WUS using smaller M values bring more resources occupation.

For Alt3, WUR has the stronger capability than Ambient-IoT UE, and there is no issue to support multiple M values. Moreover, different M value does not impact implementation much. The WUR side still can use similar or same decoding algorithm. And Alt3 is more beneficial for the NW to configure the M value and allocate the resources.

Therefore, it is not necessary to make a relationship between the M values of LP-WUS and LP-SS. Thus, Alt3 is supported.

Proposal 2: For the M value for LP-WUS and LP-SS, it should be configured by gNB respectively.

OOK1 and OOK4 with M=1 waveform generation

In RAN1#118b meeting, the following working assumption on overlaid OFDM sequence(s) of LP-WUS was made.

Working Assumption For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n Note1: OOK-1 is considered as specific case of OOK-4 with M=1.

Considering that ZC sequence after DFT operation is still a ZC sequence, in order to generate sequence in frequency domain for OOK-1 and OOK-4 with M=1, ZC sequence with DFT operation can be considered.

Additionally, using 2^n DFT size is beneficial for the NW since it is easiser to be implemented based on 2^n DFT size. Moverover, if the DFT size is not 2^n, e.g., 132 or 132, the simples received in time domain is not matched with the local oscillator, 7.68MHz, which may bring additional processing complexity due to the clock counting. Therefore, the working assumption could be confirmed.

Proposal 3: Confirm the working assumption on overlaid OFDM sequence(s) of LP-WUS.

Working Assumption For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n Note1: OOK-1 is considered as specific case of OOK-4 with M=1.

LP-SS design

Principle for binary sequence selection

- In order to maintain same powering boosting level of multiple OFDM symbols allocated for LP-SS, same number of OOK-ON symbols within one OFDM symbol should be ensured. Otherwise, gNB is required to do separate power control implementation for each OFDM symbol and it is too complicated for gNB.

For OOK based LR, LP-RSRP/LP-RSRQ should be calculated based on LP-SS sequence. In the corresponding calculation process, OOK-OFF symbols should be used to evaluate the received power of Interference+Noise, OOK-ON symbols should be used to evaluate the received power of the received signal including LP-SS and Interference + Noise. In this calculation process, it is preferred to configure equal number of OOK-ON symbols and OOK-OFF symbols to ensure the balanced power and obtain better LP-RSRP/LP-RSRQ performance. Moreover, to configure equal number of OOK-ON symbols and OOK-OFF symbols also has benefits of achieving ideal AGC gain.

- Observation 1: For LP-SS, to configure equal number of OOK-ON symbols (carrying bit “1”) and OOK-OFF symbols (carrying bit “0”) within M OOK symbols for one OFDM symbol has benefits of

- Ensuring the power control implementation at gNB side unchanged;

- Improving LP-RSRP/LP-RSRQ calculation accuracy;

- Ensuring the AGC performance;

- Proposal 4:Candidate LP-SS binary sequences should satisfy that

- For OOK-4 with M>1, equal number of OOK-ON symbols (carrying bit “1”) and OOK-OFF symbols (carrying bit “0”) within M OOK symbols of one OFDM symbol for OOK-4 with M>1.

- For OOK-4 with M=1, equal number of OOK-ON symbols (carrying bit “1”) and OOK-OFF symbols (carrying bit “0”) within the binary sequence.

Evaluation results for TO correction

For OOK-4 with M=1

- 4-length binary sequence

For OOK-4 with M=1, TO correction cross-checking results of the agreed four 4-length binary sequence under the principle of balanced OOK-ON and OOK-OFF symbols are listed in Table 1.

Table 1. Summary of TO correction results for OOK-4 with M=1 and 4-length binary sequences

| Sequence index | Candidate sequences | Average sync accuracy(us) (Remove the max and min values) |
| --- | --- | --- |
| 0 | 0 1 0 1 | 4.25 |
| 1 | 0 1 1 0 | 8.98 |
| 2 | 1 0 0 1 | 3.43 |
| 3 | 1 0 1 0 | 4.88 |

From LP-SS cross-checking results, it can be seen that only three binary sequences can meet the requirement of TO correction accuracy <=5us.

Observation 2: For OOK-4 with M=1, only three of the agreed 4-length binary sequences can meet the requirement of TO correction accuracy <=5us.

- 6-length binary sequence

For OOK-4 with M=1, TO correction cross-checking results of the agreed four 6-length binary sequence under the principle of balanced OOK-ON and OOK-OFF symbols are listed in Table 2.

Table 2. Summary of TO correction results for OOK-4 with M=1 and 6-length binary sequences

| Sequence index | Candidate sequences | Average sync accuracy(us) (Remove the max and min values) |
| --- | --- | --- |
| 0 | 1 0 1 0 1 0 | 3.309 |
| 1 | 0 1 0 1 0 1 | 3.369 |
| 2 | 1 0 0 1 0 1 | 2.84 |
| 3 | 1 0 1 0 01 | 2.645 |

From LP-SS cross-checking results, it can be seen that four 6-length binary sequences can meet the requirement of TO correction accuracy <=5us.

Observation 3: For OOK-4 with M=1, the four agreed 6-length binary sequences can meet the requirement of TO correction accuracy <=5us.

- 8-length binary sequence

For OOK-4 with M=1, TO correction cross-checking results of the agreed four 8-length binary sequence under the principle of balanced OOK-ON and OOK-OFF symbols are listed in Table 3.

Table 3. Summary of TO correction results for OOK-4 with M=1 and 8-length binary sequences

| Sequence index | Candidate sequences | Average sync accuracy(us) (Remove the max and min values) |
| --- | --- | --- |
| 0 | 1 0 1 0 0 1 0 1 | 2.835 |
| 1 | 1 0 1 0 1 0 0 1 | 2.914 |
| 2 | 1 0 0 1 0 1 0 1 | 3.035 |
| 3 | 0 1 0 1 0 1 0 1 | 3.176 |

From LP-SS cross-checking results, it can be seen that four 8-length binary sequences can meet the requirement of TO correction accuracy <=5us.

Observation 4: For OOK-4 with M=1, the four agreed 8-length binary sequences can meet the requirement of TO correction accuracy <=5us.

For OOK-4 with M=1 both 6-length and 8-length binary sequences can meet the requirement of TO correction accuracy <=5us. The summary of binary sequence resource overhead Vs TO accuracy gain is shown in Table 4. From Table 4 it can be seen that compared with 6-length binary sequence, additional resource overhead of 33.3% for 8-length binary sequence can only bring additional TO estimation gain of 5.73%. Therefore, from the perspective of LP-SS overhead reduction and additional TO accuracy gain, the agreed four 6-length binary sequences are supported for LP-SS for OOK-4 with M=1.

Table 4. Summary of binary sequence resource overhead Vs TO accuracy gain for OOK-4 with M=1

| Cases | Additional resource overhead compared with 6-length binary sequence | Average sync accuracy for 6-length,(us) | Additional TO accuracy gain compared with 6-length binary sequence,(us) |
| --- | --- | --- | --- |
| 6-length binary sequence | / | [2.645, 3.369] | / |
| 8-length binary sequence | 33.3% | [2.835, 3.176] | 5.73% |
| Note 1: Additional TO accuracy gain is calculated based on the boundary value of the sync accuracy results. |  |  |  |

Observation 5: For OOK-4 with M=1, compared with 6-length binary sequence, additional resource overhead of 33.3% for 8-length binary sequence can only bring additional TO estimation gain of 5.73%.

Proposal 5: For OOK-4 with M=1, the following four 6-length binary sequences are supported for LP-SS.

[1 0 1 0 1 0]

[0 1 0 1 0 1]

[1 0 0 1 0 1]

[1 0 1 0 0 1]

For OOK-4 with M=2

- 8-length binary sequence

For OOK-4 with M=2, TO correction cross-checking results of the agreed four 8-length binary sequence under the principle of balanced OOK-ON and OOK-OFF symbols are listed in Table 5.

Table 5. Summary of TO correction results for OOK-4 with M=2 and 8-length binary sequences

| Sequence index | Candidate sequences | Average sync accuracy(us) (Remove the max and min values) |
| --- | --- | --- |
| 0 | 0 1 0 1 1 0 0 1 | 1.730 |
| 1 | 0 1 1 0 0 1 0 1 | 1.651 |
| 2 | 0 1 1 0 1 0 0 1 | 1.582 |
| 3 | 1 0 0 1 0 1 1 0 | 1.741 |

From LP-SS cross-checking results, it can be seen that 8-lenght binary sequences can meet the requirement of TO correction accuracy <=2us.

Observation 6: For OOK-4 with M=2, the four agreed 8-length binary sequences can meet the requirement of TO correction accuracy <=2us.

- 12-length binary sequence

For OOK-4 with M=2, TO correction cross-checking results of the agreed four 12-length binary sequence under the principle of balanced OOK-ON and OOK-OFF symbols are listed in Table 6.

Table 6. Summary of TO correction results for OOK-4 with M=2 and 12-length binary sequences

| Sequence index | Candidate sequences | Average sync accuracy(us) (Remove the max and min values) |
| --- | --- | --- |
| 0 | 1 0 0 1 1 0 0 1 1 0 0 1 | 1.054 |
| 1 | 0 1 1 0 1 0 0 1 1 0 0 1 | 1.103 |
| 2 | 0 1 1 0 0 1 1 0 1 0 0 1 | 1.098 |
| 3 | 0 1 1 0 0 1 0 1 1 0 0 1 | 1.271 |

From LP-SS cross-checking results, it can be seen that 12-length binary sequences can meet the requirement of TO correction accuracy <=2us.12-length binary sequences balances overhead and performance better compared to 8 binary sequences and 16 binary sequences, and has the same overhead as a 6-length sequence with M=1.

Observation 7: For OOK-4 with M=2, the four agreed 12-length binary sequences can meet the requirement of TO correction accuracy <=2us.

Observation 8: For OOK-4 with M=2,12-length binary sequences balances overhead and performance better compared to 8 binary sequences and 16 binary sequences, and has the same overhead as a 6-length sequence with M=1.

- 16-length binary sequence

For OOK-4 with M=2, TO correction cross-checking results of the agreed four 16-length binary sequence under the principle of balanced OOK-ON and OOK-OFF symbols are listed in Table 7.

Table 7. Summary of TO correction results for OOK-4 with M=2 and 16-length binary sequences

| Sequence index | Candidate sequences | Average sync accuracy(us) (Remove the max and min values) |
| --- | --- | --- |
| 0 | 1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1 | 0.991 |
| 1 | 1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1 | 1.006 |
| 2 | 1 0 0 1 1 0 1 0 0 1 0 1 1 0 0 1 | 0.971 |
| 3 | 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1 | 0.969 |

From LP-SS cross-checking results, it can be seen that 16 binary sequences can meet the requirement of TO correction accuracy <=2us.

Observation 9: For OOK-4 with M=2, the four agreed 16-length binary sequences can meet the requirement of TO correction accuracy <=2us.

For OOK-4 with M=2, 8-length,12-length and 16-length binary sequences can meet the requirement of TO correction accuracy <=2us. The summary of binary sequence resource overhead Vs TO accuracy gain is shown in Table 8. From Table 8 it can be seen that compared with 8-length binary sequence, additional resource overhead of 50% for 12-length binary sequence can bring additional TO estimation gain of 27.00%, additional resource overhead of 100% for 16-length binary sequence can bring additional TO estimation gain of 42.22% which is less than two times of 27.00%.

Therefore, from the perspective of LP-SS overhead reduction and additional TO accuracy gain , the agreed four 12-length binary sequences are supported for LP-SS for OOK-4 with M=2.

Table 8. Summary of binary sequence resource overhead Vs TO accuracy gain for OOK-4 with M=2

| Cases | Additional resource overhead compared with 6-length binary sequence | Average sync accuracy for 6-length,(us) | Additional TO accuracy gain compared with 6-length binary sequence,(us) |
| --- | --- | --- | --- |
| 8-length binary sequence | / | [1.582, 1.741] | / |
| 12-length binary sequence | 50% | [1.054, 1.271] | 27.00% |
| 16-length binary sequence | 100% | [0.969, 1.006] | 42.22% |
| Note 1: Additional TO accuracy gain is calculated based on the boundary value of the sync accuracy results. |  |  |  |

Observation 10: For OOK-4 with M=2, compared with 8-length binary sequence, additional resource overhead of 50% for 12-length binary sequence can bring additional TO estimation gain of 27.00%, additional resource overhead of 100% for 16-length binary sequence can bring additional TO estimation gain of 42.22% which is less than two times of 27.00%.

Proposal 6: For OOK-4 with M=2, the following four 12-length binary sequences are supported for LP-SS.

[1 0 0 1 1 0 0 1 1 0 0 1]

[0 1 1 0 1 0 0 1 1 0 0 1]

[0 1 1 0 0 1 1 0 1 0 0 1]

[0 1 1 0 0 1 0 1 1 0 0 1]

For OOK-4 with M=4

- 16-length binary sequence

For OOK-4 with M=4, TO correction cross-checking results of the agreed four 16-length binary sequence under the principle of balanced OOK-ON and OOK-OFF symbols are listed in Table 9.

Table 9. Summary of TO correction results for OOK-4 with M=4 and 16-length binary sequences

| Sequence index | Candidate sequences | Average sync accuracy(us) (Remove the max and min values) |
| --- | --- | --- |
| 0 | 0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0 | 1.011 |
| 1 | 0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0 | 1.048 |
| 2 | 1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1 | 1.013 |
| 3 | 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 | 0.985 |

From LP-SS cross-checking results, it can be seen that 4 binary sequences can meet the requirement of TO correction accuracy <=1us.

Observation 11: For OOK-4 with M=4, the four agreed 16-length binary sequences can meet the requirement of TO correction accuracy <=1us.

- 32-length binary sequence

For OOK-4 with M=4, TO correction cross-checking results of the agreed four 32-length binary sequence under the principle of balanced OOK-ON and OOK-OFF symbols are listed in Table 10.

Table 10. Summary of TO correction results for OOK-4 with M=4 and 32-length binary sequences

| Sequence index | Candidate sequences | Average sync accuracy(us) (Remove the max and min values) |
| --- | --- | --- |
| 0 | 0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1 | 0.708 |
| 1 | 0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1 | 0.621 |
| 2 | 0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 | 0.708 |
| 3 | 0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1 | 0.670 |

From LP-SS cross-checking results, it can be seen that 4 binary sequences can meet the requirement of TO correction accuracy <=1us.

Observation 12: For OOK-4 with M=4, the four agreed 32-length binary sequences can meet the requirement of TO correction accuracy <=1us.

For OOK-4 with M=4, both 16-length and 32-length binary sequences can meet the requirement of TO correction accuracy <=1us. The summary of binary sequence resource overhead Vs TO accuracy gain is shown in Table 11. From Table 11 it can be seen that compared with 16-length binary sequence, additional resource overhead of 100% for 32-length binary sequence can bring additional TO estimation gain of 32.44%. Therefore, from the perspective of LP-SS overhead reduction and additional TO accuracy gain , the agreed four 16-length binary sequences are supported for LP-SS for OOK-4 with M=4.

Table 11. Summary of binary sequence resource overhead Vs TO accuracy gain for OOK-4 with M=4

| Cases | Additional resource overhead compared with 6-length binary sequence | Average sync accuracy for 6-length,(us) | Additional TO accuracy gain compared with 6-length binary sequence,(us) |
| --- | --- | --- | --- |
| 16-length binary sequence | / | [0.985, 1.048] | / |
| 32-length binary sequence | 100% | [0.621, 0.708] | 32.44% |
| Note 1: Additional TO accuracy gain is calculated based on the boundary value of the sync accuracy results. |  |  |  |

Observation 13: For OOK-4 with M=4, compared with 16-length binary sequence, additional resource overhead of 100% for 32-length binary sequence can bring additional TO estimation gain of 32.44%.

Proposal 7: For OOK-4 with M=4, the following four 16-length binary sequences are supported for LP-SS.

[0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0]

[0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0]

[1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1]

[1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0]

Additional sync signal

Time error

In RAN1 #119 meeting, RAN1 made following agreement.

Agreement Regarding whether to support additional sync signal to LP-SS, down-selection from the following: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support at least one of [80ms,160ms] - FFS: whether different values can be applicable for different M values - Option 1B: No additional sync signal, LP-SS periodicity =320ms - No additional support of other periodicities - Option 2: With additional sync signal, LP-SS periodicity =320ms - FFS additionally support other LP-SS periodicities - FFS details on additional sync signal - FFS additional sync signal is configurable and/or conditionally present

In this section, a rough calculation of TO value before LP-WUS detection is made in Table 12.

Table 12. A rough calculation of TO value before LP-WUS detection assuming residual frequency error 10ppm

|  | Residual frequency error=10ppm |  |  |
| --- | --- | --- | --- |
| LP-SS periodicity (ms) | Worst time drift before LP-WUS detection, T,(us) | LP-SS TO estimation accuracy, ∆T,(us) | Total TO before LP-WUS detection,(us) |
| 80 | 0.8 | 5/3.04/2.99 | 5.8/3.84/3.79 |
|  |  | 2/1.676/0.98 | 2.8/2.476/1.78 |
|  |  | 1/1.01/0.68 | 1.8/1.81/1.48 |
| 160 | 1.6 | 5/3.04/2.99 | 6.6/4.94/4.59 |
|  |  | 2/1.676/0.98 | 3.6/3.276/2.58 |
|  |  | 1/1.01/0.68 | 2.6/2.61/2.28 |
| 320 | 3.2 | 5/3.04/2.99 | 8.2/6.24/6.19 |
|  |  | 2/1.676/0.98 | 5.2/4.876/4.18 |
|  |  | 1/1.01/0.68 | 4.2/4.21/3.88 |

Assuming 320ms periodicity for LP-SS is configured and M value for LP-SS is 1, then even LP-WUS uses M=1 the time error is beyond the LP-WUS reception tolerance. If M value for LP-SS is 2, it is possible to configure LP-WUS using M=1. If M value for LP-SS is 4, still the LP-WUS should use M=1.

Observation 14: Without additional sync signal, based on residual frequency error 10ppm

- if LP-SS periodicity is 320ms and M value >1, the LP-WUS shall use M=1 .

- if LP-SS periodicity is 320ms and M value =1, the drifted TO is out of LP-WUS tolerance.

If we consider to configure a smaller periodicity to guarantee the LP-WUS performance, that is to say for 320ms LP-SS periodicity, still it can not work appropriately and the NW should avoid to use that, which is kind of conflicting with our agreement.

Table 13. A rough calculation of TO value before LP-WUS detection assuming residual frequency error 5ppm

|  | Residual frequency error=5ppm |  |  |
| --- | --- | --- | --- |
| LP-SS periodicity (ms) | Worst time drift before LP-WUS detection, T,(us) | LP-SS TO estimation accuracy, ∆T,(us) | Total TO before LP-WUS detection,(us) |
| 80 | 0.4 | 5/3.04/2.99 | 5.4/3.44/3.39 |
|  |  | 2/1.676/0.98 | 2.4/2.076/1.38 |
|  |  | 1/1.01/0.68 | 1.4/1.41/1.08 |
| 160 | 0.8 | 5/3.04/2.99 | 5.8/4.14/3.79 |
|  |  | 2/1.676/0.98 | 2.8/2.476/1.78 |
|  |  | 1/1.01/0.68 | 1.8/1.81/1.48 |
| 320 | 1.6 | 5/3.04/2.99 | 6.6/4.64/4.59 |
|  |  | 2/1.676/0.98 | 5.2/3.276/2.58 |
|  |  | 1/1.01/0.68 | 4.2/2.61/2.28 |

Observation 15: Without additional sync signal, based on residual frequency error 5ppm

- if LP-SS periodicity is 320ms and M value =1,2,4, the LP-WUS shall use M=1 to make sure the timer error in within 2us.

Given above analysis, it is seen that the the LP-WUS reception performance can not be guaranteed without additional sync signal. Some contributions mentioned that smaller periodicity can mitigate this issue. However, it can not address the issue when 320ms periodicity is configured. At least, option 1B can be precluded.

Proposal 8: Option 1B can be precluded and down-select between option 1A and option 2.

Overhead comparison

Considering LP-SS resource overhead, the following options and evaluated cases are assumed.

- Option 1A: smaller periodicity and longer length for LP-SS

- Option 2-1: additional sync signal be located before LP-WUS

- Option 2-2: additional sync signal be located before LO

Evaluated case 1: Target 4000 UEs in a cell, number of subgroups is 31, per UE paging rate 0.001, per group paging rate=3.17%

Table 14. Overhead comparison for the evaluated case 1

|  | LP-SS periodicity and length | additional sync signal M and Length | overhead for sync | NOTE |
| --- | --- | --- | --- | --- |
| M=1 baseline | 320ms, L=6 | / | 0.135% |  |
| M=1 | Option 1A: 160ms, L=8 | / | 0.36% |  |
| M=1 | Option 2-1: 320ms, L=6 (one preamble for one LP-WUS) | M=1,L=6 (6 symbols) M=2,L=8 (4symbols) M=4,L=16(4symbols) | 0.268% 0.223% 0.223% | If additional sync sync use larger M value, it can provide better sync performance and less overhead. |
| M=1 | Option 2-2: 320ms, L=6 (one preamble for one LO assuming LO periodicity 320ms) | M=1,L=6 (6 symbols) M=2,L=8 (4symbols) M=4,L=16(4symbols) | 0.169%~0.27% 0.158%~0.225% 0.158%~0.225% (it depends on whether all the LP-WUS is paged in one LO or not) |  |
| M=2 | 320ms, L=8 | / | 0.09% |  |
| M=2 | Option 1A: 160ms, L=16 | / | 0.36% |  |
| M=2 | Option 2-1: 320ms, L=8 (one preamble for one LP-WUS) | M=1,L=6(6symbols) M=2,L=8 (4symbols) M=4,L=16(4symbols) | 0.222% 0.179% 0.179% | If additional sync sync use larger M value, it can provide better sync performance and less overhead. |
| M=2 | Option 2-2: 320ms, L=8 (one preamble for one LO assuming LO periodicity 320ms) | M=1,L=6(6symbols) M=2,L=8(4symbols) M=4,L=16(4symbols) | 0.124%~0.225% 0.113%~0.18% 0.113%~0.18% |  |
| M=4 | 320ms, L=16 | / | 0.09% | same as M=2 |
| M=4 | Option 1A: 160ms, L=32 | / | 0.36% |  |
| M=4 | Option 2-1: 320ms, L=16 (one preamble for one LP-WUS) | M=1,L=6(6symbols) M=2,L=8(4symbols) M=4,L=16(4symbols) | 0.222% 0.179% 0.179% |  |
| M=4 | Option 2-2: 320ms, L=16 (one preamble for one LO assuming LO periodicity 320ms) | M=1,L=6(6symbols) M=2,L=8(4symbols) M=4,L=16(4symbols) | 0.124%~0.225% 0.113%~0.18% 0.113%~0.18% |  |

The above table reveal a fact that if the paged subgroups is not very large, additional sync signal can have less overhead than reducing the LP-SS periodicity. And also, additional sync sync can provide more flexible M value configuration for LP-SS and LP-WUS.

Additionally, if

- per group paging rate is no more than 3%

- at maximum 31 subgroups is configured per PO

- number of POs in a cycle is X=1~64

the expected paged number of subgroups is 3%*31*X and the corresponding range is 0.93~59.52. Typically, the NW may configure number of POs as 4 or 8, then the typical expected paged number of subgroups is 3.72 and 7.44. In this typical case, the number of LP-WUS would be no more than 8 and the required number of additional preamble is no more than 8.

Evaluated case 2: Number of subgroups is 31, per group paging rate=3%, number of POs is 8

Table 15. Overhead comparison for case 2

|  | LP-SS periodicity and length | additional sync signal M and Length | overhead for sync | NOTE |
| --- | --- | --- | --- | --- |
| M=1 baseline | 320ms, L=6 | / | 0.135% |  |
| M=1 | Option 1A: 160ms, L=8 | / | 0.36% |  |
| M=1 | Option 2-1: 320ms, L=6 (one preamble for one LP-WUS) | M=1,L=6 (6 symbols) M=2,L=8 (4symbols) M=4,L=16(4symbols) | 0.3861% 0.3024% 0.3024% | If additional sync sync use larger M value, it can provide better sync performance and less overhead. |
| M=1 | Option 2-2: 320ms, L=6 (one preamble for one LO assuming LO periodicity 320ms) | M=1,L=6 (6 symbols) M=2,L=8 (4symbols) M=4,L=16(4symbols) | 0.169%~0.27% 0.158%~0.225% 0.158%~0.225% (it depends on whether all the LP-WUS is paged in one LO or not) |  |
| M=2 | 320ms, L=8 | / | 0.09% |  |
| M=2 | Option 1A: 160ms, L=16 | / | 0.36% |  |
| M=2 | Option 2-1: 320ms, L=8 (one preamble for one LP-WUS) | M=1,L=6(6symbols) M=2,L=8 (4symbols) M=4,L=16(4symbols) | 0.3411% 0.2574% 0.2574% | If additional sync sync use larger M value, it can provide better sync performance and less overhead. |
| M=2 | Option 2-2: 320ms, L=8 (one preamble for one LO assuming LO periodicity 320ms) | M=1,L=6(6symbols) M=2,L=8(4symbols) M=4,L=16(4symbols) | 0.124%~0.225% 0.113%~0.18% 0.113%~0.18% |  |
| M=4 | 320ms, L=16 | / | 0.09% | same as M=2 |
| M=4 | Option 1A: 160ms, L=32 | / | 0.36% |  |
| M=4 | Option 2-1: 320ms, L=16 (one preamble for one LP-WUS) | M=1,L=6(6symbols) M=2,L=8(4symbols) M=4,L=16(4symbols) | 0.3411% 0.2574% 0.2574% |  |
| M=4 | Option 2-2: 320ms, L=16 (one preamble for one LO assuming LO periodicity 320ms) | M=1,L=6(6symbols) M=2,L=8(4symbols) M=4,L=16(4symbols) | 0.124%~0.225% 0.113%~0.18% 0.113%~0.18% |  |

If the number of POs is increased, the number of LP-WUS transmitted would be increased, and the overhead for additional sync signal for option 2-1 would be increased. In this case, if the preamble is configured before each LO, the overhead still can be under control. Given that the LP-WUS should be deployed at low paging rate and strive to avoid the overhead impact on the NW, the low paging rate scenario should be promised at least. For example, if the number of POs is 64, assuming 31subgroups per PO and subgroup paging rate 3%, the overhead occupied only by LP-WUS would be up to 9.3%, which is unacceptable for the NW. Therefore, this kind of scenarios are not practical for LP-WUS deployment.

Observation 16: In low paging rate scenarios which is more practical for LP-WUS deployment, additionally sync signal can provide less overhead than smaller LP-SS periodicity.

Observation 17: Support additional sync signal has more flexibility for gNB and provide better sync performance for LP-WUS reception.

Last, if the additional sync signal can be configured before the LO and assume that the LO periodicity is 320ms, it can realize the same purpose by reducing the LP-SS periodicity. Moreover, it can help reduce the system overhead since the preamble is one-demand signal, which is only present when the LO carries the LP-WUS as following illustration shown.

Figure 1. Comparison between option 1A and option 2-2

By configuration, option 2-2 can realize option 1A. Moreover, it can provide more flexible gNB transmission, which could help save system overhead.

Observation 18: Option 2-2 can realize option 1A by configuration and provide more gNB flexibility, less overhead, and better sync performance for LP-WUS reception

Proposal 9: Support additional sync signal

- reuse LP-SS design

- it is configurable

- support larger and smaller LP-SS periodicity

LP-WUS

Information bits carried by OOK

In RAN1#120 meeting, RAN1 made following agreement.

Agreement For LP-WUS information carried by OOK, select alt 1 for codepoint to meet FAR and MDR performance. - Alt 1: Coding with rate matching if any - For more than 2 information bits: RM coding as in section 5.3.3.3 of 38.212 (FFS: whether scrambling in the same section is applied or not) - 2 information bits (if supported): coding as in section 5.3.3.2 of 38.212 (FFS: whether scrambling in the same section is applied or not) - 1 information bit (if supported): repetition as in section 5.3.3.1 of 38.212 (FFS: whether scrambling in the same section is applied or not) - FFS: Code block length - FFS: Further repetition before or after Manchester coding

For the above mentioned three coding cases with different information bit size (Case 1 for More than 2 bits, Case 2 for 2bits, Case 3 for 1bit), if Case 2 and Case 3 are also supported, 3 cases are encoded in accordance with the channel coding of the TS38.212 and support rate matching to a unified coded sequence length. The Time-Frequency resources occupied by the three cases are of the same size, but the channel coding schemes are different. In receiver, the UE should try multiple channel decoding schemes to decode the LP-WUS signal, resulting in high complexity of LP-WUS detection by the UE.

Observation 19: If cases of 2 information bits and 1 information bit with independent channel coding solution are supported, the UE will attempt multiple channel decoding schemes when receiving LP-WUS, resulting in high complexity of LP-WUS detection.

It is recommended that cases of 2 information bits and 1 information bit (if supported) can be indicated by the reserved states of the case of more than 2 information bits. In this way, the UE only needs to do LP-WUS detection with one channel coding scheme, which is easy for UE implementation.

Proposal 10: For LP-WUS information carried by OOK, channel coding schemes in section 5.3.3.2 of 38.212 for 2 information bits or 1 information is not supported.

Based on RM coding as in section 5.3.3.3 of TS38.212, LP-WUS information bits,a0,a1,⋯,aK−1, are encoded as follows:

a0,a1,⋯,aK−1KRM coding  d0,d1,⋯,dN−1,N  Rate matching f0,f1,f2,⋯,fE−1E2−length Manchester code  f00,f012,  f10,f112,f2,0,f2,12 ,⋯,  fE−1,0,fE−1,12

In this section, a preliminary evaluation for the performance of Repetition is made. The following five cases are evaluated.

Case 1: 5 information bits+RM(5,32)+ rate matching to 16 + Manchester code 1/2

Case 2: 5 information bits+RM(5,32)+ Manchester code 1/2

Case 3: 5 information bits+RM(5,32)+ rate matching to 16+ Manchester code 1/2+Repetition=2

Case 4: 5 information bits+RM(5,32)+ rate matching to 16+ Manchester code 1/2+Repetition=4

Case 5: 5 information bits+RM(5,32)+ Manchester code 1/2+Repetition=2

The evaluation results are shown in Figure 2.

Figure 2: LP-WUS with 5 information bits detection performance with different transmitting schemes

From Figure 2, for the case of RM coding with rate matching to a shorter coded binary sequence, such as 16-length for Case 3,4, repetition can be used to improve LP-WUS detection performance. But under the situation of same resource overhead, the case of RM coding with rate matching to a longer coded binary sequence,such as 32-length for Case 2,5, it has better LP-WUS detection performance. Output sequence length after rate matching is preferred to integer multiples of 7 or 14 in order for LP-WUS to occupy the whole resource of slots.

Figure 3: LP-WUS detection performance with 5 or 6 bits information

From the evaluation, under the situation of same length of output coded sequence by RM, LP-WUS with smaller information bits(5 bits) has better detection performance than that of larger information bits(such as 6 bits).8,14,28,42,56

Proposal 11: For LP-WUS information carried by OOK, RM coding as in section 5.3.3.3 of 38.212 with rate matching is supported before Manchester coding

- Rate matching to a longer output binary sequence lager than 32 can be used as repetition.

- Output sequence length after rate matching is preferred to integer multiples of 7 or 14.

- Additional repetition after rate matching is not supported.

- LP-WUS information bits size is no larger than 5 and detailed value can be configured through signaling.

Information bits carried by OFDM sequence

In RAN1#120 meeting, RAN1 made following agreement.

Agreement For WUS information carried by the overlaid OFDM sequence(s), consider at least the following alternatives: - Alt 1: Raw information bits are mapped to sequence(s) - Alt 2: Raw information bits are mapped to sequence(s) after channel coding - Same channel coding scheme(s) as OOK is applied. - FFS same or different rate matching and/or repetition factor as OOK - Alt 3: Codepoint/Subgroup is mapped to sequence(s)

Considering that ZC sequences set corresponding to different M values can carry different size of LP-WUS information bit, take “4bits@M=1, 3bits@M=2, 2bits@M=4” for example, it is recommended that the information bit size should be divided by 2 or 3 or 4 in accordance with the value of M. To achieve this, Alt2 is the most preferred solution since its bit size cab be flexibly adjusted as the M value changes.

For Alt 2, after RM coding and rate matching, the LP-WUS information bits is converted to an integer multiple of a bit size that can be carried by the ZC sequences corresponding to the current M value, and then each part of the coded information with the bit size is carried by ZC sequences on a OOK-On symbol.

Proposal 12: For LP-WUS information carried by the overlaid OFDM sequence(s), Alt2 is supported.

- Rate matching after RM code is supported and the output sequence length should be an integer multiple of a bit size that can be carried by the ZC sequences in accordance with M value.

Overlaid ZC Sequence Ncs

Bzc and Lzc for ZC sequence

Regarding overlaid ZC sequence, it was agreed that for M=1, 2, 4,is given by the largest prime number such that,is the overlaid OFDM sequence length andLZC=12∗11/M=132/M. Values of LZC and BZC for OOK-4 with M=1,2,4 are shown in Table 16.

Table 16. Value of LZC and BZC for OOK-4 with M=1,2,4

| M value | LZC | BZC |
| --- | --- | --- |
| 1 | 132 | 131 |
| 2 | 66 | 61 |
| 4 | 33 | 31 |

Proposal 13: For the overlaid ZC sequences, values of LZC and BZC for OOK-4 with M=1,2,4 are adopted as shown in the following Table.

| M value | LZC | BZC |
| --- | --- | --- |
| 1 | 132 | 131 |
| 2 | 66 | 61 |
| 4 | 33 | 31 |

Root and CS for overlaid ZC sequence generation

Based on the analysis in section 4.1.2, take residual frequency error[ppm] =5ppm for OFDM based LR without FFT block for example, at 2.6GHz frequency offset at receiver side is about 2.6*10^9*5*10^-6 = 13kHz, it is almost one subcarrier@SCS=15KHz and half of one subcarrier@SCS=30KHz. Frequency offset has impact on overlaid ZC sequence detection performance. Therefore, overlaid ZC sequence generation should solve the issue of frequency offset.

Observation 20: Frequency offset caused by residual frequency error has impact on overlaid ZC sequence detection performance.

Considering that NR preamble has corresponding solution to solve the problem of frequency offset and almost the same ZC generation method for both LP-WUS overlaid ZC sequence and NR preamble, it is recommended that NR preamble generation method of restricted type A can be reused for LP-WUS overlaid ZC sequence.

Based on the analysis in Appendix B, for overlaid ZC sequence generation, Ncs requirements corresponding to different SCS is summarized in Table 17.

Table 17. Ncs requirements corresponding to different SCS

| SCS(KHz) | For timing drift = 0.4us | For timing drift = 0.8us | For timing drift = 1.6us | For timing drift = 4.0us |
| --- | --- | --- | --- | --- |
| 15 | 1*2=2 | 2*2=4 | 4*2=8 | 8*2=16 |
| 30 | 2*2=4 | 4*2=8 | 7*2=14 | 16*2=32 |
| 60 | 4*2=8 | 7*2=14 | 13*2=26 | 32*2=64 |
| 120 | 7*2=14 | 13*2=26 | 26*2=52 | 64*2=128 |

In case of considering larger frequency offset, maximum number of supported ZC sequences with CS in a root index can be different for different root index. Additional, The maximum number is also related with the configured Ncs. For example, in Table 18-Table 20 for different ZC sequence length the optimized ZC sequence root index are listed which can support at least 2 ZC sequences with CS for a root index even for larger value of Ncs.

Table 18. Optimized ZC sequence root index, Bzc=31, Ncs=4

| RootIndex | 3 | 4 | 6 | 7 | 8 | 12 | 14 | 17 | 19 | 23 | 24 | 25 | 27 | 28 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 19. Optimized ZC sequence root index, Bzc=61, Ncs=8

| RootIndex | 3 | 6 | 7 | 11 | 16 | 17 | 18 | 19 | 23 | 25 | 27 | 29 | 32 | 34 | 36 | 38 | 42 | 43 | 44 | 45 | 50 | 54 | 55 | 58 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 20. Optimized ZC sequence root index, Bzc=131, Ncs=16

| RootIndex | 3 | 4 | 6 | 7 | 8 | 15 | 16 | 23 | 25 | 27 | 30 | 31 | 32 | 36 | 37 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 39 | 40 | 41 | 45 | 46 | 47 | 50 | 51 | 53 | 54 | 57 | 59 | 60 | 62 | 64 |
|  | 67 | 69 | 71 | 72 | 74 | 77 | 78 | 80 | 81 | 84 | 85 | 86 | 90 | 91 | 92 |
|  | 94 | 95 | 99 | 100 | 101 | 104 | 106 | 108 | 115 | 116 | 123 | 124 | 125 | 127 | 128 |

Therefore, it is recommended to select the ZC sequence root index supporting larger number of ZC sequences with CS in the root index, and thus the orthogonality of selected overlaid ZC sequences can be kept. It can improve the overlaid ZC sequence detection performance.

Proposal 14: For LP-WUS overlaid ZC sequence generation, it is recommended to reuse NR preamble generation method of restricted type A.

- ZC root index is prioritized to the root index with supporting larger number of ZC sequences with CS in the root index

Conclusions

In this contribution, we have discussed issues on waveform design and signal structure for LP-WUS. We make the following observations and proposals:

- Observation 1: For LP-SS, to configure equal number of OOK-ON symbols (carrying bit “1”) and OOK-OFF symbols (carrying bit “0”) within M OOK symbols for one OFDM symbol has benefits of

- Ensuring the power control implementation at gNB side unchanged;

- Improving LP-RSRP/LP-RSRQ calculation accuracy;

- Ensuring the AGC performance;

Observation 2: For OOK-4 with M=1, only three of the agreed 4-length binary sequences can meet the requirement of TO correction accuracy <=5us.

Observation 3: For OOK-4 with M=1, the four agreed 6-length binary sequences can meet the requirement of TO correction accuracy <=5us.

Observation 4: For OOK-4 with M=1, the four agreed 8-length binary sequences can meet the requirement of TO correction accuracy <=5us.

Observation 5: For OOK-4 with M=1, compared with 6-length binary sequence, additional resource overhead of 33.3% for 8-length binary sequence can only bring additional TO estimation gain of 5.73%.

Observation 6: For OOK-4 with M=2, the four agreed 8-length binary sequences can meet the requirement of TO correction accuracy <=2us.

Observation 7: For OOK-4 with M=2, the four agreed 12-length binary sequences can meet the requirement of TO correction accuracy <=2us.

Observation 8: For OOK-4 with M=2,12-length binary sequences balances overhead and performance better compared to 8 binary sequences and 16 binary sequences, and has the same overhead as a 6-length sequence with M=1.

Observation 9: For OOK-4 with M=2, the four agreed 16-length binary sequences can meet the requirement of TO correction accuracy <=2us.

Observation 10: For OOK-4 with M=2, compared with 8-length binary sequence, additional resource overhead of 50% for 12-length binary sequence can bring additional TO estimation gain of 27.00%, additional resource overhead of 100% for 16-length binary sequence can bring additional TO estimation gain of 42.22% which is less than two times of 27.00%.

Observation 11: For OOK-4 with M=4, the four agreed 16-length binary sequences can meet the requirement of TO correction accuracy <=1us.

Observation 12: For OOK-4 with M=4, the four agreed 32-length binary sequences can meet the requirement of TO correction accuracy <=1us.

Observation 13: For OOK-4 with M=4, compared with 16-length binary sequence, additional resource overhead of 100% for 32-length binary sequence can bring additional TO estimation gain of 32.44%.

Observation 14: Without additional sync signal, based on residual frequency error 10ppm

- if LP-SS periodicity is 320ms and M value >1, the LP-WUS shall use M=1 .

- if LP-SS periodicity is 320ms and M value =1, the drifted TO is out of LP-WUS tolerance.

Observation 15: Without additional sync signal, based on residual frequency error 5 ppm

- if LP-SS periodicity is 320ms and M value =1,2,4, the LP-WUS shall use M=1 to make sure the timer error in within 2us.

Observation 16: In low paging rate scenarios which is more practical for LP-WUS deployment, additionally sync signal can provide less overhead than smaller LP-SS periodicity.

Observation 17: Support additional sync signal has more flexibility for gNB and provide better sync performance for LP-WUS reception.

Observation 18: Option 2-2 can realize option 1A by configuration and provide more gNB flexibility, less overhead, and better sync performance for LP-WUS reception

Observation 19: If cases of 2 information bits and 1 information bit with independent channel coding solution are supported, the UE will attempt multiple channel decoding schemes when receiving LP-WUS, resulting in high complexity of LP-WUS detection.

Observation 20: Frequency offset caused by residual frequency error has impact on overlaid ZC sequence detection performance.

Proposal 1: Confirm the working assumption in below agreement:

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4

Proposal 2: For the M value for LP-WUS and LP-SS, it should be configured by gNB respectively.

Proposal 3: Confirm the working assumption on overlaid OFDM sequence(s) of LP-WUS.

Working Assumption For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n Note1: OOK-1 is considered as specific case of OOK-4 with M=1.

- Proposal 4:Candidate LP-SS binary sequences should satisfy that

- For OOK-4 with M>1, equal number of OOK-ON symbols (carrying bit “1”) and OOK-OFF symbols (carrying bit “0”) within M OOK symbols of one OFDM symbol for OOK-4 with M>1.

- For OOK-4 with M=1, equal number of OOK-ON symbols (carrying bit “1”) and OOK-OFF symbols (carrying bit “0”) within the binary sequence.

Proposal 5: For OOK-4 with M=1, the following four 6-length binary sequences are supported for LP-SS.

[1 0 1 0 1 0]

[0 1 0 1 0 1]

[1 0 0 1 0 1]

[1 0 1 0 0 1]

Proposal 6: For OOK-4 with M=2, the following four 12-length binary sequences are supported for LP-SS.

[1 0 0 1 1 0 0 1 1 0 0 1]

[0 1 1 0 1 0 0 1 1 0 0 1]

[0 1 1 0 0 1 1 0 1 0 0 1]

[0 1 1 0 0 1 0 1 1 0 0 1]

Proposal 7: For OOK-4 with M=4, the following four 16-length binary sequences are supported for LP-SS.

[0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0]

[0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0]

[1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1]

[1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0]

Proposal 8: Option 1B can be precluded and down-select between option 1A and option 2.

Proposal 9: Support additional sync signal

- reuse LP-SS design

- it is configurable

- support larger and smaller LP-SS periodicity

Proposal 10: For LP-WUS information carried by OOK, channel coding schemes in section 5.3.3.2 of 38.212 for 2 information bits or 1 information is not supported.

Proposal 11: For LP-WUS information carried by OOK, RM coding as in section 5.3.3.3 of 38.212 with rate matching is supported before Manchester coding

- Rate matching to a longer output binary sequence lager than 32 can be used as repetition.

- Output sequence length after rate matching is preferred to integer multiples of 7 or 14.

- Additional repetition after rate matching is not supported.

- LP-WUS information bits size is no larger than 5 and detailed value can be configured through signaling.

Proposal 12: For LP-WUS information carried by the overlaid OFDM sequence(s), Alt2 is supported.

- Rate matching after RM code is supported and the output sequence length should be an integer multiple of a bit size that can be carried by the ZC sequences in accordance with M value.

Proposal 13: For the overlaid ZC sequences, values of LZC and BZC for OOK-4 with M=1,2,4 are adopted as shown in the following Table.

| M value | LZC | BZC |
| --- | --- | --- |
| 1 | 132 | 131 |
| 2 | 66 | 61 |
| 4 | 33 | 31 |

Proposal 14: For LP-WUS overlaid ZC sequence generation, it is recommended to reuse NR preamble generation method of restricted type A.

- ZC root index is prioritized to the root index with supporting larger number of ZC sequences with CS in the root index

References

RP-241824, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, RAN#105.

Draft_Minutes_report_RAN1#120_v030

R1-2410939, “Collection of LP-SS binary sequences”, vivo, RAN1#120

- Appendix A: Simulation assumptions for LP-WUS

Table A1. Simulation assumptions for LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz |
| SCS of OFDM generator for NR signal | 30kHz |
| gNB Channel BW | 20MHz and with Normal CP=2.344us。 |
| Filter | 5rd order Butterworth LPF |
| Sampling Rate | 7.68 MHz |
| ADC bit width | 4bits ADC |
| Channel Model | TDL-C 300ns |
| Number of Rx for LP-WUS | 1 Rx |
| UE speed | 3 km/h |
| Inter-cell interference | No |
| Phase noise modeling | No |

- Appendix B: Candidate Ncs for the Overlaid ZC Sequence

Based on the analysis in section 4.1.2, 4 candidate time drift values are considered for OFDM sequence based LR corresponding different LP-SS periodicity, such as [0.4us,0.8us,1.6us, 4us].

Since time drift is considered, at receiver the received ZC sequence is a cyclic shift version of the transmitted ZC sequence. It is related to the configuration of SCS and time drift value. A preliminary summary of the relationship between CS value, SCS and time drift value are shown in Table B1-Table B4.

Table B1. Relationship between CS value and SCS for Timing drift = 0.4us

| SCS(KHz) | FFT Num | Number of time samples | Corresponding max CS value | Ceil(Corresponding max CS value) |
| --- | --- | --- | --- | --- |
| 15 | 512 | 3.072 | 0.792 | 1 |
| 15 | 1024 | 6.144 | 0.792 | 1 |
| 15 | 2048 | 12.288 | 0.792 | 1 |
| 15 | 4096 | 24.576 | 0.792 | 1 |
| 30 | 512 | 6.144 | 1.584 | 2 |
| 30 | 1024 | 12.288 | 1.584 | 2 |
| 30 | 2048 | 24.576 | 1.584 | 2 |
| 30 | 4096 | 49.152 | 1.584 | 2 |
| 60 | 512 | 12.288 | 3.168 | 4 |
| 60 | 1024 | 24.576 | 3.168 | 4 |
| 60 | 2048 | 49.152 | 3.168 | 4 |
| 60 | 4096 | 98.304 | 3.168 | 4 |
| 120 | 512 | 24.576 | 6.336 | 7 |
| 120 | 1024 | 49.152 | 6.336 | 7 |
| 120 | 2048 | 98.304 | 6.336 | 7 |
| 120 | 4096 | 196.608 | 6.336 | 7 |

Table B2.Relationship between CS value and SCS for Timing drift = 0.8us

| SCS(KHz) | FFT Num | Number of time samples | Corresponding max CS value | Ceil(Corresponding max CS value) |
| --- | --- | --- | --- | --- |
| 15 | 512 | 6.144 | 1.584 | 2 |
| 15 | 1024 | 12.288 | 1.584 | 2 |
| 15 | 2048 | 24.576 | 1.584 | 2 |
| 15 | 4096 | 49.152 | 1.584 | 2 |
| 30 | 512 | 12.288 | 3.168 | 4 |
| 30 | 1024 | 24.576 | 3.168 | 4 |
| 30 | 2048 | 49.152 | 3.168 | 4 |
| 30 | 4096 | 98.304 | 3.168 | 4 |
| 60 | 512 | 24.576 | 6.336 | 7 |
| 60 | 1024 | 49.152 | 6.336 | 7 |
| 60 | 2048 | 98.304 | 6.336 | 7 |
| 60 | 4096 | 196.608 | 6.336 | 7 |
| 120 | 512 | 49.152 | 12.672 | 13 |
| 120 | 1024 | 98.304 | 12.672 | 13 |
| 120 | 2048 | 196.608 | 12.672 | 13 |
| 120 | 4096 | 393.216 | 12.672 | 13 |

Table B3.Relationship between CS value and SCS for Timing drift = 1.6us

| SCS(KHz) | FFT Num | Number of time samples | Corresponding max CS value | Ceil(Corresponding max CS value) |
| --- | --- | --- | --- | --- |
| 15 | 512 | 12.288 | 3.168 | 4 |
| 15 | 1024 | 24.576 | 3.168 | 4 |
| 15 | 2048 | 49.152 | 3.168 | 4 |
| 15 | 4096 | 98.304 | 3.168 | 4 |
| 30 | 512 | 24.576 | 6.336 | 7 |
| 30 | 1024 | 49.152 | 6.336 | 7 |
| 30 | 2048 | 98.304 | 6.336 | 7 |
| 30 | 4096 | 196.608 | 6.336 | 7 |
| 60 | 512 | 49.152 | 12.672 | 13 |
| 60 | 1024 | 98.304 | 12.672 | 13 |
| 60 | 2048 | 196.608 | 12.672 | 13 |
| 60 | 4096 | 393.216 | 12.672 | 13 |
| 120 | 512 | 98.304 | 25.344 | 26 |
| 120 | 1024 | 196.608 | 25.344 | 26 |
| 120 | 2048 | 393.216 | 25.344 | 26 |
| 120 | 4096 | 786.432 | 25.344 | 26 |

Table B4. Relationship between CS value and SCS for Timing drift = 4us

| SCS(KHz) | FFT Num | Number of time samples | Corresponding max CS value | Ceil(Corresponding max CS value) |
| --- | --- | --- | --- | --- |
| 15 | 512 | 30.72 | 7.92 | 8 |
| 15 | 1024 | 61.44 | 7.92 | 8 |
| 15 | 2048 | 122.88 | 7.92 | 8 |
| 15 | 4096 | 245.76 | 7.92 | 8 |
| 30 | 512 | 61.44 | 15.84 | 16 |
| 30 | 1024 | 122.88 | 15.84 | 16 |
| 30 | 2048 | 245.76 | 15.84 | 16 |
| 30 | 4096 | 491.52 | 15.84 | 16 |
| 60 | 512 | 122.88 | 31.68 | 32 |
| 60 | 1024 | 245.76 | 31.68 | 32 |
| 60 | 2048 | 491.52 | 31.68 | 32 |
| 60 | 4096 | 983.04 | 31.68 | 32 |
| 120 | 512 | 245.76 | 63.36 | 64 |
| 120 | 1024 | 491.52 | 63.36 | 64 |
| 120 | 2048 | 983.04 | 63.36 | 64 |
| 120 | 4096 | 1966.08 | 63.36 | 64 |

Considering value of time drift can be positive or negative, the necessary Gap(Ncs) between two ZC sequence of the same Root should be at least twice of "Corresponding max CS value" in the above table.
Therefore, the value of Ncs is summarized in Table B5. Moreover, FFT points are also considered but for the same SCS FFT points have no impact on the value of Ncs.

Table B5. Ncs requirements corresponding to different SCS

| Ncs/ SCS(KHz) | For timing drift = 0.4us | For timing drift = 0.8us | For timing drift = 1.6us | For timing drift = 4.0us |
| --- | --- | --- | --- | --- |
| 15 | 1*2=2 | 2*2=4 | 4*2=8 | 8*2=16 |
| 30 | 2*2=4 | 4*2=8 | 7*2=14 | 16*2=32 |
| 60 | 4*2=8 | 7*2=14 | 13*2=26 | 32*2=64 |
| 120 | 7*2=14 | 13*2=26 | 26*2=52 | 64*2=128 |