3GPP TSG RAN WG1 #120	bis                                            R1-2501873

Wuhan, China, 07th – 11th April 2025

Agenda Item:	9.5.2

Source:	Spreadtrum, UNISOC

Title: 	Discussion on on-demand SIB1 for idle/inactive mode UEs

Document for:	Discussion and decision

Introduction

It was agreed in WID [1] to specify OD-SIB1 for case 2 for UEs in idle/inactive state.

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

We will share our view on OD-SIB1 for UEs in idle/inactive state.

Indication the status of OD-SIB1

In RAN1#120 [2], companies discussed how to indicate the status of OD-SIB1 and the following agreement had been achieved.

Agreement If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2, down select from the following: - Alt. 1: Only if K_SSB=30 for FR1 and K_SSB=14 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission; Otherwise, UE doesn’t monitor Type 0 PDCCH - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified. - Alt. 2: UE monitors Type 0 PDCCH for SIB1 transmission - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified. - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission - Alt. 4: UE doesn’t monitor Type 0 PDCCH for SIB1 transmission. - Alt. 5: UE monitors Type 0 PDCCH for SIB1 based on some repurposed parameters in MIB or PBCH Note: If the UE detects a SSB where K_SSB<=23 for FR1 or K_SSB<=12 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission as legacy. Note: From Ericsson point view, Alt 3 and Alt 4 may be inconsistent with existing RAN2 agreements Note: The above cases are for SSB on the sync raster.

For Alt 1, when OD-SIB1 is broadcasting, K_SSB is configured as 30/14 and UE should monitor PDCCH of OD-SIB1. In this case, UE not needed to transmit UL WUS if K_SSB is configured as 30/14. If K_SSB is configured as other values, it means OD-SIB1 is not broadcasting, UE need to transmit UL WUS for triggering SIB1 transmission. The benefit of Alt 1 is avoiding the UE from sending unnecessary UL WUS, which can help UE energy saving.

For Alt 2 and Alt 3, UE don’t know whether OD-SIB1 is broadcasting and UE always need to monitors Type 0 PDCCH for SIB1 transmission which may waste UE energy.

For Alt 4, it means UE always needs to transmit UL WUS before monitoring Type 0 PDCCH for SIB1 transmission. It has negative impact on UE energy saving.

For Alt 5, which parameters in MIB or PBCH can be repurposed, which is not clear for us.

- For SSB on sync raster case, support Alt 1 to indicate UE whether OD-SIB1 is broadcasting or not:

Alt. 1: Only if K_SSB=30 for FR1 and K_SSB=14 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission; Otherwise, UE doesn’t monitor Type 0 PDCCH.

OD-SIB1 transmission

In RAN1#120 [2], it was agreed that PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding to at least the SSB associated with the PRACH for UL-WUS if this is indicated via UL WUS configuration. For the first FFS, it is up to network implementation whether to transmit OD-SIB1 in one or more beams. For the second FFS, it is unnecessary for gNB to indicate the SSB the UE can expect PDCCH transmission on in UL-WUS configuration.

Agreement The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding to at least the SSB associated with the PRACH for UL-WUS if this is indicated via UL WUS configuration - FFS: Whether UE expect PDCCH on PDCCH monitoring occasion corresponding to SSB other than the one mentioned above - FFS: Whether gNB can indicate (in UL-WUS configuration) the SSB the UE can expect PDCCH transmission on

- It is up to network implementation whether to transmit OD-SIB1 in one or more beams.

- It is unnecessary for gNB to indicate the SSB the UE can expect PDCCH transmission on in UL-WUS configuration.

Feedback from gNB

In RAN1 #119 [2], it was agreed that the contents of RAR in response to SI request are included in RAR in response to UL WUS, e.g., RAPID in RAR. We think only RAPID in RAR is enough, there is no need to provide other information in RAR.

Agreement At least the contents of RAR in response to SI request are included in RAR in response to UL WUS

- Don’t support additional contents in RAR for OD-SIB1 other than contents of RAR for SI request.

Conclusion

We have the following observations and proposals.

- For SSB on sync raster case, support Alt 1 to indicate UE whether OD-SIB1 is broadcasting or not:

Alt. 1: Only if K_SSB=30 for FR1 and K_SSB=14 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission; Otherwise, UE doesn’t monitor Type 0 PDCCH.

- It is up to network implementation whether to transmit OD-SIB1 in one or more beams.

- It is unnecessary for gNB to indicate the SSB the UE can expect PDCCH transmission on in UL-WUS configuration.

- Don’t support additional contents in RAR for OD-SIB1 other than contents of RAR for SI request.

Reference

- RP-242354, “Revised WID: Enhancements of network energy savings for NR”, Ericsson, RAN#105, Sep 9-12, 2024.

- 3GPP RAN1#120 Chair’s Notes.

- 3GPP RAN1#118bis Chair’s Notes.