3GPP TSG RAN WG1 #120bis		                                                     R1- 2502001

Wuhan, China, April 7th – 11th, 2025

Source:	CATT

Title:	System design and procedure of LP-WUS operation for UE in CONNECTED Modes

Agenda Item:	9.6.3

Document for:	Discussion and Decision

Introduction

In RAN1#120, the following agreements were achieved for studying low-power wake-up signal and receiver (LP-WUS/WUR) for NR [1]:

Agreement For RRC connected mode, UE does not monitor LP-WUS during C-DRX active time - FFS FAR and MDR impact Agreement LP-WUS can be configured with legacy PDCCH skipping. - Note: There is no intention to modify legacy PDCCH skipping Agreement For RRC CONNECTED mode, LP-WUS can be configured with Rel-17 SSSG switching Agreement - For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode, - LP-WUS can be configured on a serving cell - LP-WUS indication is applicable to all serving cells - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell or can be RRC configured cell other than PCell Agreement - For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, - LP-WUS can be configured on a serving cell per cell-group - This does not imply that LP-WUS has to be configured on both cell groups - LP-WUS indication is applicable to all serving cells in the cell-group - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - FFS whether to support this case as separate UE capability from CA case - FFS UE capability for monitoring LP-WUS on one cell group or both cell groups - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell/PSCell or can be RRC configured cell other than Pcell/PSCell Agreement - For RRC CONNECTED mode, LP-WUS can be configured with Cell DTX - FFS During Cell DTX inactive time, whether the UE is expected to monitor LP-WUS or not Conclusion From RAN1 perspective, when LP-WUS monitoring is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers MR PDCCH monitoring according to the legacy UE behaviour - Above applies at least for Option 1-1 Agreement For LP-WUS monitoring in RRC CONNECTED mode, select one of the options on how to define/configure/indicate QCL source of LP-WUS - Opt1: Separate indication/configuration for QCL source of LP-WUS - Opt2: Association of QCL source between LP-WUS and PDCCH/CORESET Agreement For LP-WUS MOs in connected mode for Option 1-1, support Approach 1: - LP-WUS MOs, including periodicity and time offset1, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - Time offset2 indicates a time prior to a slot where the drx-onDurationTimer would start - UE monitors the first Z LP-WUS MO(s) at or after Time offset2. - FFS: How to determine the value of Z - Time offset2 is RRC configured - UE is not required to monitor LP-WUS during the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer. FFS: How X is determined. Agreement For LP-WUS MOs in connected mode for Option 1-2, support Approach 1: - LP-WUS MOs, including periodicity and time offset3, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - FFS one or multiple MOs per periodicity - UE monitors LP-WUS in the LP-WUS MOs and, if triggered to wake up, starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset4. - The time offset4 configured by the network indicating a time, after which the UE starts PDCCH monitoring via starting the new timer. - FFS: Definition of time offset4 Agreement - For the X = [2, 3] candidate values for the UE capability report on the minimum time gap for each SCS, consider {0.25, 0.5, 1, 2, 3, 6, 10, 20, 30, 40}ms - Note: Other values are not precluded - Only for reported value of [20ms,] 30ms and 40ms (if supported), the reported values assume SSB periodicity of 20ms and ensures SSB(s) before PDCCH reception and during time gap - FFS: translation of minimum time gap for different SSB periodicities including different candidates for different SSB periodicities - FFS whether the same set of candidates are used for different SCSs - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR - FFS whether/how to consider the case of CA - FFS: candidate values that can be indicated by UAI

This paper discusses the aspects of the timeline between LP-WUS reception/detection and PDCCH monitoring, LP-WUS monitoring occasion (LP-WUS MO), QCL/TCI-state, and CA/DC with LP-WUS for RRC_CONNECTED mode.

Discussion

The timeline between LP-WUS reception/detection and PDCCH monitoring

It was agreed in RAN1#118bis that UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. The value of X and the definition of reported candidate value are FFS. In RAN1#120, following agreement was made:

Agreement - For the X = [2, 3] candidate values for the UE capability report on the minimum time gap for each SCS, consider {0.25, 0.5, 1, 2, 3, 6, 10, 20, 30, 40}ms - Note: Other values are not precluded - Only for reported value of [20ms,] 30ms and 40ms (if supported), the reported values assume SSB periodicity of 20ms and ensures SSB(s) before PDCCH reception and during time gap - FFS: translation of minimum time gap for different SSB periodicities including different candidates for different SSB periodicities - FFS whether the same set of candidates are used for different SCSs - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR - FFS whether/how to consider the case of CA - FFS: candidate values that can be indicated by UAI

The values of minimum time gap considered for LP-WUS is different to those minimum time gap in Rel-16 DCP since the UE in Rel-16 DRX adaptation already wake up early to prepare for the coherent demodulation of DCP. In other words, the minimum time gap of LP-WUS needs to take into account of the time MR in preparation of channel tracking for PDCCH coherent demodulation when LP-WUS triggers UE wakeup. Even UE is in-sync (the C-DRX cycle has the periodicity less than 160ms), UE still needs at least 1 SSB to get the latest channel estimation. Thus, 20ms would be the delay for MR ramping up time in preparation of PDCCH demodulation. Regarding the value of X, for RRC CONNECTED mode, support two candidate value for the UE capability report on the minimum time gap with considering at least {20ms}.

Proposal 1: The minimum time gap of LP-WUS needs to take into account the time of MR in preparation of channel tracking for PDCCH coherent demodulation when LP-WUS triggers UE wakeup.

Proposal 2: For RRC CONNECTED mode, support 2 candidate value for the UE capability report on the minimum time gap with considering at least {20ms}.

LP-WUS monitoring occasions

Agreement For LP-WUS MOs in connected mode for Option 1-1, support Approach 1: - LP-WUS MOs, including periodicity and time offset1, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - Time offset2 indicates a time prior to a slot where the drx-onDurationTimer would start - UE monitors the first Z LP-WUS MO(s) at or after Time offset2. - FFS: How to determine the value of Z - Time offset2 is RRC configured - UE is not required to monitor LP-WUS during the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer. FFS: How X is determined. Agreement For LP-WUS MOs in connected mode for Option 1-2, support Approach 1: - LP-WUS MOs, including periodicity and time offset3, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - FFS one or multiple MOs per periodicity - UE monitors LP-WUS in the LP-WUS MOs and, if triggered to wake up, starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset4. - The time offset4 configured by the network indicating a time, after which the UE starts PDCCH monitoring via starting the new timer. - FFS: Definition of time offset4

In RAN1#120 the above agreements were achieved for LP-WUS MO configuration with following remaining issues.

- The periodicity and offset for LP-WUS MOs

It was agreed that the periodicity and offset are configured independently from the C-DRX periodicity/offset by new RRC parameter(s) for LP-WUS MOs. In Rel-16 DCP, the periodicity and offset for DCI format 2_6 monitoring is configured by an RRC parameter i.e. monitoringSlotPeriodicityAndOffset for indicating the slots for PDCCH monitoring. Also, the first symbol(s) for PDCCH monitoring in the configured slots is configured by an RRC parameter i.e.  monitoringSymbolsWithinSlot. Similar to the configuration of PDCCH monitoring, the slots for LP-WUS MOs can be configured with an RRC parameter e.g. monitoringSlotPeriodicityAndOffset-lpwus in slot-level. The first symbol for LP-WUS MO in a slot can be determined by an RRC parameter e.g. monitoringSymbolsWithinSlot-lpwus. It is noted that no more than one LP-WUS MOs for one given UE is configured in a slot.

Proposal 3: For LP-WUS MO in for UE in CONNECTED mode, the periodicity and offset is configured in slot-level and the first symbol for LP-WUS MO in a slot can be determined by an RRC parameter.

- How to determine the value of Z and  X  in Option 1-1

It was agreed in RAN1#120 that UE monitors the first Z LP-WUS MO(s) at or after Time offset2, and the value of Z is FFS in Option1-1. In order to provide gNB with flexibility, multiple MOs can be configured for LP-WUS, so that gNB can choose one or more preferable MOs to transmit LP-WUS. Thus, the value of Z can be larger than 1, which is configured by the RRC parameter and would depend on the number of LP-WUS MOs in the time window, as shown in Figure 1.  Figure 1 illustrates the time window for LP-WUS monitoring.

Regarding the value of X, it was agreed that UE is not required to monitor LP-WUS during the period of X [slots/symbols] prior to the beginning of the slot where the UE would start the drx-onDurationTimer. How X is determined is FFS. In Rel-17 DCP, the minimum time gap value that UE reports corresponds to the SCS of the active DL BWP. Unlike Rel-17 DCP, the X can be decouple with SCS and corresponds to UE capability of minimum time gap e.g. {0.25, 0.5, 1, 2, 3, 6, 10, 20, 30, 40}ms, as discussion in section 2.1.

Figure 1: The configured time window in Option 1-1

Proposal 4: For Option 1-1, 1, Z can be larger than 1 and configured by RRC.

Proposal 5: X is the value of UE capability report with ‘ms’ unit.

- Whether to support multiple LP-WUS MOs per periodicity and the definition of time offset4 in Option1-2

In Rel-17 DCP, the duration of PDCCH monitoring was part of the SearchSpace configuration for indicating the number of consecutive slots of PDCCH monitoring in every cycle, i.e. upon every period as given in the periodicityAndOffset. Similar, multiple MOs can be configured in every LP-WUS monitoring cycle, as shown in Figure 2, in which a time window is introduced as the duration of LP-WUS MO. Thus, the definition of time offset is related with the LP-WUS MO window.

Figure 2：The configured time window in Option 1-2

Proposal 6: In Option1-2, support multiple LP-WUS MOs per periodicity and the definition of time offset4 is related with the LP-WUS MO window.

QCL/TCI-state for LP-WUS

Agreement For LP-WUS monitoring in RRC CONNECTED mode, select one of the options on how to define/configure/indicate QCL source of LP-WUS - Opt1: Separate indication/configuration for QCL source of LP-WUS - Opt2: Association of QCL source between LP-WUS and PDCCH/CORESET

In RAN1#120, two options were discussed on how to define/configure/indicate TCI-state of LP-WUS. Since the LP-WUS is used to trigger PDCCH monitoring and the TCI-state is configured per CORESET for PDCCH [2], the LP-WUS can be QCLed with the RS (reference signal) associated with the TCI state of the CORESET. One LP-WUS would be configured to be QCLed with the RS associated with one of the TCI states of the CORESETs when the UE is configured with more than one CORESET for PDCCH monitoring.

Proposal 7: One LP-WUS is configured to be QCLed with the RS associated with each of the TCI states of the CORESETs in order to trigger the PDCCH monitoring at the associated CORESETs when LP-WUS is detected.

CA/DC with LP-WUS

Agreement - For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode, - LP-WUS can be configured on a serving cell - LP-WUS indication is applicable to all serving cells - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell or can be RRC configured cell other than PCell Agreement - For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, - LP-WUS can be configured on a serving cell per cell-group - This does not imply that LP-WUS has to be configured on both cell groups - LP-WUS indication is applicable to all serving cells in the cell-group - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - FFS whether to support this case as separate UE capability from CA case - FFS UE capability for monitoring LP-WUS on one cell group or both cell groups - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell/PSCell or can be RRC configured cell other than Pcell/PSCell

It was agreed in RAN1#120, for the case when UE is configured with NR-DC and/or CA without secondary DRX group, LP-WUS  can be configured on a serving cell per cell-group and LP-WUS indication is applicable to all serving cell in the cell-group. Whether the serving cell on which the LP-WUS is configured is only PCell/PSCell or can be RRC configured cell other than Pcell/PSCell is FFS. In Rel-16 DCP, a UE configured with DRX mode operation can monitor the DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell when UE is configured with NR-DC and/or CA without secondary DRX group. Similar, LP-WUS can be monitored on a serving cell e.g. the PCell or on the SpCell per cell-group.

Proposal 8: For the case when UE is configured with NR-DC and/or CA without dual group in RRC CONNECTED mode, LP-WUS can be configured in the PCell or on the SpCell per cell-group.

For the case when UE is configured with CA in RRC CONNECTED mode, the dual DRX group can be configured with common parameters e.g.  drx-SlotOffset and different parameters i.e. drx-onDurationTimer, drx-InactivityTimer [2]. In other words, the drx-onDurationTimer in both DRX group starts in same time but may last in different durations. For Rel-16 DCP, the secondary DRX group and DCP cannot be configured simultaneously [2] due to the longer duration of drx-onDurationTimer or drx-InactivityTimer may overlap with the next DCP occasion. Thus, for LP-WUS, if dual DRX configured, same issue may be arisen. Thus, the network should not configure the dual DRX group with LP-WUS simultaneously for Rel-19 LP-WUS.

Proposal 9: For Rel-19 LP-WUS, the dual DRX group and LP-WUS cannot be configured simultaneously.

FAR and MDR impact

Agreement For RRC connected mode, UE does not monitor LP-WUS during C-DRX active time - FFS FAR and MDR impact

In RAN1#120, it was agreed that UE does not monitor LP-WUS during C-DRX active time. The impact of FAR and MDR to not monitoring the LP-WUS during C-DRX active time should be addressed. Since the periodicity of LP-WUS MO is configured independently to that of C-DRX, the resource configured for LP-WUS in the MOs may overlaps with the C-DRX active time. If there is a false alarm of LP-WUS detection, the UE would stop monitoring LP-WUS and start PDCCH monitoring. The LP-WUS wake-up indication cannot be detected by UE in the C-DRX active time, as shown in Figure 3. If UE would not detect any wakeup indication from LP-WUS during the C-DRX active time, the MR will not be waken-up. This will lead to excessive delay in the PDCCH monitoring and subsequent data transmission. In addition, the MR has to be waken-up to monitor the PDCCH at the MO in the active time even there is no PDCCH in the active time, which will increase the UE power consumption. If there is miss detection on LP-WUS, the MR may not be waken-up for a long time due to the LP-WUR continuously monitors LP-WUS. This will lead to excessive delay of the PDCCH decoding triggered by LP-WUS. Thus, the FAR and MDR impact should be considered.

Figure 3: The analysis of FAR and MDR impact

The FAR impact can be solved based on the gNB implementation i.e. the duration of drx-onDurationTimer can be configured shorter to avoid the overlapping between LP-WUS MO and the active time of C-DRX. To solve the MDR impact, a fallback mechanism to the legacy behavior can be considered. For example, when MR does not wake-up for a long time (e.g. a duration), MR wakes up and fall back to legacy mode.

Proposal 10: A fallback mechanism to the legacy behavior of PDCCH monitoring can be specified to battle the MDR impact to the PDCCH decoding for UEs in CONNECTED mode.

Conclusion

In this contribution, we discuss the aspects of the timeline between LP-WUS reception/detection and PDCCH monitoring, LP-WUS monitoring occasion (LP-WUS MO), QCL/TCI-state and CA/DC with LP-WUS for RRC_CONNECTED mode and give the following proposals.

Proposal 1: The minimum time gap of LP-WUS needs to take into account the time of MR in preparation of channel tracking for PDCCH coherent demodulation when LP-WUS triggers UE wakeup.

Proposal 2: For RRC CONNECTED mode, support 2 candidate value for the UE capability report on the minimum time gap with considering at least {20ms}.

Proposal 3: For LP-WUS MO in for UE in CONNECTED mode, the periodicity and offset is configured in slot-level and the first symbol for LP-WUS MO in a slot can be determined by an RRC parameter.

Proposal 4: For Option 1-1, 1, Z can be larger than 1 and configured by RRC.

Proposal 5: X is the value of UE capability report with ‘ms’ unit.

Proposal 6: In Option1-2, support multiple LP-WUS MOs per periodicity and the definition of time offset4 is related with the LP-WUS MO window.

Proposal 7: One LP-WUS is configured to be QCLed with the RS associated with each of the TCI states of the CORESETs in order to trigger the PDCCH monitoring at the associated CORESETs when LP-WUS is detected.

Proposal 8: For the case when UE is configured with NR-DC and/or CA without dual group in RRC CONNECTED mode, LP-WUS can be configured in the PCell or on the SpCell per cell-group.

Proposal 9: For Rel-19 LP-WUS, the dual DRX group and LP-WUS cannot be configured simultaneously.

Proposal 10: A fallback mechanism to the legacy behavior of PDCCH monitoring can be specified to battle the MDR impact to the PDCCH decoding for UEs in CONNECTED mode.

References

- Chairman notes, RAN1#120 meeting, Athens, Greece, February 17th – 21st, 2025.

- TR 38.331, NR Radio Resource Control (RRC) protocol specification.