3GPP TSG RAN WG1 #120-bis			R1-2502168

Wuhan, China, April 7th–April 11th, 2025

Source: 	CMCC

Title:	Discussion on LP-WUS operation in IDLE/INACTIVE mode

Agenda item:	9.6.2

Document for:	Discussion & Decision

- Introduction

In the RAN#103 meeting, the WI of low-power wake-up signal and receiver for NR (LP-WUS/WUR) was revised as [1], one of the objectives is to study LP-WUS operation in IDLE/INACTIVE mode. The related objective is as following.

For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In this contribution, we provide views on LP-WUS MO design to support LP-WUS operation in IDLE/INACTIVE mode.

- Discussion on LP-WUS MO design

For LP-WUS MO design, the following agreements are agreed during RAN1#118bis-120 meeting [2-4].

Conclusion in RAN1#119: There is no RAN1 consensus to support additional codepoints indicating wake-up at least for 1:1 LO to PO mapping. Working Assumption in RAN1#119: If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration. Agreement in RAN1#118bis: When K (K>1) LP-WUS MOs are configured for each beam in an LO, down select between - Option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs. A UE monitors all or some of the MO(s) within the K LP-WUS MOs. - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted. - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs. - M = 1 and M > 1 is supported. - FFS: detailed UE monitoring behavior - FFS R=1 or R>= 1 - Option B: K LP-WUS MOs for a beam are divided into G (G >= 1) groups of R*M (M >= 1) LP-WUS MOs. A UE monitors all or some of the MO(s) within one group of R*M LP-WUS MOs based on its subgroup ID. - Each group of R*M LP-WUS MOs is further divided into M groups of R LP-WUS MOs. - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted. - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs. - FFS: detailed UE monitoring behavior - M = 1 and M > 1 is supported. - FFS R=1 or R>=1 - Note: this achieves the same purpose as “Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.” Agreement in RAN1#120: The EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB. - FFS whether there is a common configuration or separate configuration for LP-WUS and LP-SS - Above is not applicable for the case when all the subcarriers for LP-WUS/LP-SS are transmitted with zero power (from baseband perspective) - Above does not mandate any UE implementation for different receiver types - FFS: whether/how to support for the case when there are different number of OOK ON symbols in different OFDM symbols (if supported) Agreement in RAN1#120: For the offset value(s) between an LO and a reference PO/PF, at least a frame-level offset is provided. - The reference point (reference PO/PF) for the frame-level offset is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is supported), associated with the LO. - FFS other offset value(s) to determine the MOs of the LO Agreement in RAN1#120: The previous agreement in RAN1#119 is updated as follows: - Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB. Agreement in RAN1#120: Confirm the following working assumption with the additional text in red: Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications Existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications. - The metrics SS-RSRP and SS-RSRQ are applicable for OFDM-based LP-WUR for RRC_IDLE and RRC_INACTIVE serving cell measurement. - FFS: whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement. Agreement in RAN1#120: For the LO to PO mapping from network perspective, support Option 2 (UEs corresponding to different POs monitor the same LO). - This should not increase the maximum number of codepoints per LO/LP-WUS compared to Option 1. - FFS conditions/restrictions for mapping multiple POs to one LO - Down-select between 2 and 4 for max number of POs per LO. Agreement in RAN1#120: For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities - Send an LS to RAN4 after the down-selection to confirm the number of SSBs. Final LS in R1-2501624. Agreement in RAN1#120: For the offset value(s) between an LO and a reference PO/PF, adopt Option 2B-1. - gNB can configure 1 or 2 offset values. - FFS whether gNB can configure 3 offset values - If multiple offset values are configured and if the gap between the LO associated with the largest offset value and the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the smallest offset value that has a gap between the LO and the PO no less than the wake-up delay. - Note: if a single offset value is configured, UE behaviour is according to Option 1-1. - All the UEs supporting LP-WUS for idle/inactive mode supports the configuration of 2 offset values (FFS: 3 values). Agreement in RAN1#120: Confirm the following working assumption with the modification: Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 31 32.

In this section, we discuss the general structure of LP-WUS MO, LP-SS/LP-WUS MO determination and SMTC for LP-SSS-RSRP/RSSI measurement.

2.1. General structure of LP-WUS MO

Regarding the discussion on general structure of LP-WUS MO, the first issue is whether K LP-WUS MOs for a beam are divided into M groups of R LP-WUS MOs, or G groups of R*M LP-WUS MOs. From our understanding, dividing the LP-WUS MOs according to different subgroup(s) can indeed reduce the overhead on LP-WUS monitoring from UE side, but will simultaneously increase the LP-WUS transmission overhead from gNB side. In addition, a per-subgroup(s) LP-WUS MO scheme means a design for short code need to be introduced, so that LP-WUS in different RRC modes may require a different coding. Therefore, consider the overhead on transmission and spec impact, it is recommended to have a simple, unify design for LP-WUS MO.

Proposal 1. For K (K>1) LP-WUS MOs configured for each beam in an LO, support Option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs.

Furthermore, the issues on detailed UE monitoring behavior and value of R are also worth noting, to be specific:

- Regarding the issue on detailed UE monitoring behavior, since LP-WUS MO in different groups may indicate any subgroup(s) if needed, it is recommended for the UE to monitor all M LP-WUS MO group(s) to avoid miss detection; In addition, if a UE detects the LP-WUS that corresponds to its subgroup, it may not need to detect the remaining LP-WUS MO(s) in the LO, therefore can reduce the unnecessary waste for energy.

- Regarding the issue on value of R, based on the evaluation in AI 9.6.1, the LP-WUS design for both OOK-receiver and OFDM-receiver has a certain gap in coverage requirements (i.e. align with Msg3 coverage performance). Thus, R>1 shall be supported to enhance the performance on LP-WUS detection.

Proposal 2. Regarding Option A on LP-WUS MO design:

- For detailed UE monitoring behaviour, support:

  - The UE monitors all M LP-WUS MO group(s) in an LO.

  - If the UE detects the LP-WUS that corresponds to its subgroup, it may not need to detect the remaining LP-WUS MO(s) in the LO.

- For the exact value of R, support R>= 1.

2.2. LP-SS/LP-WUS MO determination

According to the evaluation results in AI 9.6.1, a single LP-WUS or LP-SS transmission may need several OFDM symbols (e.g. 8 symbols or 16 symbols). Meanwhile, LP-WUS or LP-SS transmission also need repetition and beam sweeping to ensure the coverage performance, which will extend a single LO/LP-SS burst to frame or half-frame level duration. Given consideration that:

- LP-WUR does not need to be aware of the existing NR configuration (e.g. frame structure, slot/frame boundary).

- It is relatively hard and complex to avoid all existing transmission occasion in NR system (e.g. UL slot/symbol, common signal configuration, configuration related to L1/L3 measurement) while determine LP-SS/LP-WUS MO.

It is recommended that a LP-SS occasion/LP-WUS MO can go across multiple slots and contiguous in time to achieve better transmission efficiency. Also it should be noted that a LO/LP-SS burst can be contiguous in time.

Proposal 3. Support a LP-SS occasion/LP-WUS MO can go across multiple slots and contiguous in time.

- Also support that a LO/LP-SS burst can be contiguous in time.

Furthermore, since existing transmission occasion in NR system may overlap with the LP-SS/LP-WUS transmission, or the time interval between LP-SS/LP-WUS transmission and existing transmission occasion may smaller than RF retuning time (e.g. when LR/MR is deployed in FR2, or when SCS of PRACH is equal or larger than 15kHz). UE behaviour under such scenario needs to be considered and clarified.

Proposal 4. RAN1 to discuss the UE behaviour when LP-SS/LP-WUS transmission overlap with existing pattern (e.g. UL slot/symbol, common signal transmission, configuration related to L1/L3 measurement), or when the time interval between LP-SS/LP-WUS transmission and existing pattern is smaller than the RF retuning time (e.g. when LR/MR is deployed in FR2, or when SCS of PRACH is equal or larger than 15kHz).

2.3. SMTC for LP-SSS-RSRP/RSSI measurement

In current NR system, SMTC window is used for UE to perform cell re-selection/L3 measurement procedure. That is, the UE monitors SSB for serving cell, camped cell or neighbour cell per frequency according to the configuration of SMTC window. For LP-WUR, given consideration that;

- LR measurement cannot used for cell re-selection/L3 measurement based on previous discussion and agreement.

- RAN4 has already agreed that UE shall performs SSB-based LR measurement according to LO periodicity [5].

There is no obvious motivation to introduce the SMTC window for LP-SSS-RSRP/RSSI measurement.

Proposal 5. Do not introduce the SMTC window for LP-SSS-RSRP/RSSI measurement.

- Conclusions

In this contribution, we discussed the LP-WUS operation in IDLE/INACTIVE mode, and the following proposal are made.

Proposal 1. For K (K>1) LP-WUS MOs configured for each beam in an LO, support Option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs.

Proposal 2. Regarding Option A on LP-WUS MO design:

- For detailed UE monitoring behaviour, support:

  - The UE monitors all M LP-WUS MO group(s) in an LO.

  - If the UE detects the LP-WUS that corresponds to its subgroup, it may not need to detect the remaining LP-WUS MO(s) in the LO.

- For the exact value of R, support R>= 1.

Proposal 3. Support a LP-SS occasion/LP-WUS MO can go across multiple slots and contiguous in time.

- Also support that a LO/LP-SS burst can be contiguous in time.

Proposal 4. RAN1 to discuss the UE behaviour when LP-SS/LP-WUS transmission overlap with existing pattern (e.g. UL slot/symbol, common signal transmission, configuration related to L1/L3 measurement), or when the time interval between LP-SS/LP-WUS transmission and existing pattern is smaller than the RF retuning time (e.g. when LR/MR is deployed in FR2, or when SCS of PRACH is equal or larger than 15kHz).

Proposal 5. Do not introduce the SMTC window for LP-SSS-RSRP/RSSI measurement.

- References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo

- R1-2410863, Moderator (Apple Inc.), Summary #6 (Final) on LP-WUS operation in IDLE/INACTIVE mode

- R1-2409241, Moderator (Apple Inc.), Summary #6 (Final) on LP-WUS operation in IDLE/INACTIVE mode

- R1-2500795, Moderator (Apple Inc.), Summary #5 (Final) on LP-WUS operation in IDLE/INACTIVE mode

- R4-2502600, Moderator (vivo), WF on RRM requirements for NR_LPWUS