3GPP TSG RAN WG1 Meeting #120bis	R1-2502367

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.3.3

Source:	Samsung

Title:	Cross-link interference handling for SBFD

Document for:	Discussion

# Introduction

The Rel-19 WID Evolution of Duplex operation introduces NR specification support for gNB-side SBFD within a TDD carrier, enhancements to inter-gNB and inter-UE CLI handling, and BS RF and RRM requirements [1].

In the Rel-19 WID, it is assumed that SBFD is deployed at the gNB side and that the TDD UE operates in half-duplex. Subband full-duplex (SBFD) operation on a time-domain resource is limited to FDM in non-overlapping DL and UL subbands.

SBFD enhancements for SBFD-aware UEs assume SBFD operation for NR single carrier in FR1 or FR2-1. From the perspective of the SBFD-aware UE, SBFD operation is assumed within a single configured DL and UL BWP pair with aligned center frequencies. For the semi-static indication of frequency/time locations of the SBFD subbands to SBFD-aware UEs on a legacy DL or F symbol/slot, it is assumed that one UL subband is configured for SBFD operation in an SBFD symbol.

The revised Rel-19 WID objectives include introduction of specification support for cross-link interference handling [1].

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

  - Information exchange among gNBs, including [RAN3]

    - (…)

  - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

    - (…)

  - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4]

    - (…)

  - Note: Without dedicated optimization for dynamic/flexible TDD.

In Section 2 we discuss UL resource muting for PUSCH. In Section 3 we discuss L1-based UE-to-UE CLI measurements and reporting.

# UL resource muting for PUSCH

During down-selection of inter-gNB CLI handling proposals in RAN1#117, support for non-transparent UL resource muting for PUSCH was agreed. In RAN#104, the Rel-19 Duplex WID was accordingly revised [1].

- UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

  - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

  - PUSCH resource mapping, i.e., rate-matching around the muted REs

  - UCI resource determination in symbols with muted REs

- Notes: Following are assumed for UL resource muting

  - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting

  - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212

  - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH

  - UL resource muting applies only for UEs in RRC_CONNECTED mode

  - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol

  - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols

  - No changes on TBS determination for PUSCH

  - Check impact on transmit signal quality/MPR requirement, if any, with RAN4

  - This feature is subject to UE capability

## Configuration and indication

With respect to time- or frequency-domain configuration and indication of muted UL symbol positions, the following RAN1 agreements were made.

Agreement (RAN1#118)

For the reference point of time location of UL resource muting for PUSCH, Option 1 is supported

- Option 1: Starting symbol of a slot for both PUSCH mapping type A and type B

Agreement (RAN1#119)

To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH

- Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI.

Agreement (RAN1#119)

For PUSCH repetition Type A in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH repetitions.

For PUSCH repetition Type B in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions.

Agreement (RAN1#120)

Confirm the following working assumption

Working assumption

For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM

- A comb offset {0, 1} is configured for the up to two UL muting symbols

Agreement (RAN1#120)

For the semi-statically configured time location and frequency location of UL muting symbol(s) for PUSCH,

- Separate configurations of up to two UL muting symbols can be provided per UE for DG PUSCH/Type 2 CG PUSCH and Type 1 CG PUSCH

  - Separate configuration for each of multiple Type 1 CG PUSCH configurations is supported

Agreement (RAN1#120)

For PUSCH that is scheduled by DCI format 0_0 or Type 2 CG PUSCH activated by DCI format 0_0, UL resource muting is not applied.

Agreement (RAN1#120)

For the configuration of UL resource muting

- Alt.1: A new IE PUSCH-MutingResources is introduced to define the time location and frequency location of UL muting resources. The UL muting pattern configures up to two symbols within a slot and a configurable comb offset.

Agreement (RAN1#120)

A new parameter ul-MutingIndicator is introduced to PUSCH-Allocation-r16 to indicate whether the semi-statically configured UL muting symbol(s) is enabled.

Agreement (RAN1#120)

For PUSCH repetition Type A in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH repetitions in SBFD symbols.

For PUSCH repetition Type B in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions in SBFD symbols.

Note: whether UL resource muting is also applied in non-SBFD symbols is discussed separately

The following FL proposal was discussed in RAN1#120.

#### Proposal 1-4

UL resource muting can be applied in non-SBFD symbols.

Note: No additional specification impact is expected to support above.

In our view, there is no need to restrict the possible configuration of UL muting for PUSCH to SBFD symbols in the UL subband. UL muting as CLI mitigation feature is equally beneficial for d/f-TDD operation where an UL reception in an UL slot can be interfered by the DL transmissions from the neighbour cells.

We do not see any additional specification impact to support this case. If UL muting for PUSCH in non-SBFD/normal UL slot was excluded, this would result in additional specification effort. For example, the corresponding DCI reception behavior would then need to be described, i.e., excluding applicability of any indication of presence of the 1 or UL muted symbols in the non-SBFD symbols/slots, or adding additional provisions for Rel-19 SBFD parameter ul-MutingIndicator.

Proposal 1: UL resource muting can be applied in non-SBFD symbols.

One issue discussed in RAN1#120 was how to handle the case of UL TBoMS. Option 1 and Option 2 were agreed for down-selection.

Agreement (RAN1#120)

For UL muting applied for UL TBoMS, the index of the starting coded bit in a slot, except the first slot, within the 𝑁𝑠 slots allocated for the transmission of TB processing over multiple slots is determined according to Section 6.2.5, TS38.212 as follows

k0=k0'+H+τmodNcb

and H is down-selected from one of the following options

  - Option 1: 𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and assuming no muted REs.

  - Option 2: 𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and considering the muted REs

Note: No impact on TBS determination in Option 1 or Option 2.

The issue is how to interpret “the total number of coded bits available for transmission of the transport block”. This issue is not limited to TBoMS and relevant to rate-matching determination with UL muting for PUSCH in general.

For example, we determine the rate matching output sequence when “the total number of coded bits available for transmission of the transport block” is used. Similarly, although the wording is not the same, “the set of resource elements available for transmission of data” affects the rate-matching in the Data and Control multiplexing. Relevant text from TS38.212 is copied below.

Denoting by the rate matching output sequence length for the -th coded block, where the value of is determined as follows: Set for to if the -th coded block is not scheduled for transmission as indicated by CBGTI according to Clause 5.1.7.2 for DL-SCH and 6.1.5.2 for UL-SCH in [6, TS 38.214]: ; else if ; else ; end if ; end if end for where: - is the number of transmission layers that the transport block is mapped onto; - is the modulation order; - is the total number of coded bits available for transmission of the transport block; - if CBGTI is not present in the DCI scheduling the transport block and is the number of scheduled code blocks of the transport block if CBGTI is present in the DCI scheduling the transport block.

Denote as the set of resource elements, in ascending order of indices , available for transmission of data in OFDM symbol , for . Denote as the number of elements in set . Denote as the -th element in . Denote as the set of resource elements, in ascending order of indices , available for transmission of UCI in OFDM symbol , for . Denote as the number of elements in set .

In our view, the definition of “the total number of coded bits available for transmission of the transport block” should be consistent in TS38.212.

When determining the rate matching output sequence, “the total number of coded bits available for transmission of the transport block” should consider the available REs for data transmission. This means that the muted REs should be excluded. Also, “the set of resource elements available for transmission of data” should then only includes non-muted REs.

Moreover, it has been agreed in the WID and separately again in the RAN1#119 agreement to not allow specification changes for the data and control multiplexing.

Agreement (RAN1#119)

Confirm the following working assumption

Working Assumption

UL resource muting is allowed on PUSCH symbols that carry UCI, i.e., UCI is not mapped on muted REs

Note: As mentioned in the WID, no specification changes are allowed on data and control multiplexing in section 6.2.7, TS 38.212.

The same principle should therefore be applied. Otherwise, a functional/spec change would be needed for the definition of “the set of resource elements available for transmission of data” for Data and control multiplexing. This is not aligned with the WID objectives.

- Proposal 2: When determining “the total number of coded bits available for transmission of the transport block” used in TS38.212, the muted REs are not considered.

- Note: No specification impacts are expected.

## DMRS or PTRS

The revised WID already assumes that a muted UL symbol in the PUSCH allocation cannot overlap with DMRS or PTRS in the same symbol. With respect to DMRS and PTRS collision handling, the following agreements were made in RAN1#119 and RAN1#120.

Agreement (RAN1#119)

If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol.

Agreement (RAN1#119)

If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS.

Agreement (RAN1#119)

3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting.

- FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

Include the above in the LS to RAN4. In addition, mention that RAN1 is still discussing the boosting in symbols containing PTRS.

Agreement (RAN1#120)

For the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled, 3 dB power boosting for both PUSCH and PTRS REs if present, regardless of the PTRS density K and number of PUSCH layers L.

PTRS, if present, are separately RRC configured for the UE by the network. A potential collision between a muted RE in an RB of a muted UL symbol and a PTRS RE can occur depending on the PTRS time-domain and frequency domain configuration.

In the CP-OFDM case, such a potential frequency-domain collision case can be considered from the RE-level multiplexing perspective. In the DFTS-OFDM case, presence of the transform precoder requires a different consideration, i.e., because the PTRS are “spread” across frequency-domain.

For the CP-OFDM case, the revised WID already assumed that the PTRS RE is transmitted and the UE doesn’t apply the UL resource muting on such an RE. When transform precoding is not present, i.e., for CP-OFDM, a collision of PTRS REs with the muted REs of a muted UL symbol can be avoided for many (but not all) PTRS configurations by the gNB. For example, the gNB can set the time-domain density of the PTRS configuration for the UE such that muted UL symbols and PTRS symbols don’t overlap. Also, the RRC configurable comb offset {0, 1} for the muted UL symbol can be used to set the even/odd subcarrier muting of muted RE such that depending on the PUSCH DMRS port assignment which determines the PTRS RE offset value, no collision occurs. Such an approach is however burdened with a certain complexity due to its impact of RE level mapping the PUSCH processing chain. The RAN1#119 agreement was made that “the UE does not expect the muted REs overlap the REs occupied by PT-RS”.

For the DFTS-OFDM case, difficulties result to apply this principle. When transform precoding is present, all subcarriers after the DFT “carry” PTRS information and accordingly, all subcarriers should be transmitted “as is”. If some muted RE subcarriers then need to be dropped, PTRS (de-)mapping is then necessarily affected. For example, MscPUSCH in table 6.4.1.2.2.2-1 of TS38.211 would need to be changed to MscPUSCH/2.

For handling of the DFT-size when transform precoding is enabled, two options were provided in RAN1#120 for the purpose of specification drafting. How the UE implementation is done is left to each company.

Agreement (RAN1#120)

The following agreement replaces the earlier agreement from RAN1#119

If transform precoding is enabled for a PUSCH, the following options are provided for the purpose of specification drafting (how UE implementation is done is left to each company).

- Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting

  - The MscPUSCH2 modulated symbols are transformed to the frequency domain with a DFT of size MscPUSCH2, and the result is mapped to the MscPUSCH2 unmuted scheduled subcarriers.

- Option 2: The DFT size is MscPUSCH for symbol(s) with UL resource muting

  - Option 2A: The block of MscPUSCH2 modulated symbols is repeated before DFT. After DFT, the MscPUSCH2 non-zero even-indexed subcarriers are extracted and then mapped to the unmuted scheduled subcarriers.

  - Option 2B: The block of MscPUSCH2 modulated symbols is extended to  MscPUSCH modulated symbols by inserting ‘0’ after every modulated symbol. After DFT, the first MscPUSCH2 subcarriers are extracted and then mapped to the unmuted scheduled subcarriers.

Note: MscPUSCH is the total number of subcarriers of the PUSCH.

Note: Above options are mathematically equivalent.

Note: It is up to the spec editor how to capture the above as long as the specification captures the above in a mathematically identical manner.

On symbols with PTRS, for example, using ½ DFT size would then also impact the PTRS sample positions and different PUSCH symbols, i.e., normal symbols and muted UL symbol of the PUSCH allocation can then have different PTRS sample positions. In our view, such an approach would be associated with significant implementation complexity and specification effort. Reducing the DFT size on some symbols where PTRS occurs together with UL resource muting in the PUSCH would have very large implementation impacts and should be avoided.

It also needs to be considered that for the case where PTRS is most relevant, i.e., FR2, gNB-gNB CLI is not significant based on the Rel-18 evaluation results. It is then not meaningful to design for a complex solution with HW impact for PTRS handling with DFT-S-OFDM and muted UL symbol collisions when an alternative is simply to rely on other CLI handling schemes. For FR2, sharing strongest beam information is a very viable and effective CLI mitigation solution. The focus for CLI equalization design should therefore be on FR1.

Another consideration for FR2 is that a 1-symbol gap is usually available between PUSCH transmissions with the different (analog) beams to provide sufficient beam switching time. If CLI equalization needs to be implemented by the network, gNB-gNB CLI can still be estimated on transparent muted UL symbol positions and using 13-symbol PUSCH allocations, or using the REs corresponding to REs of a CDM group without data in the DMRS symbols.

Therefore, we do not agree with the assumption that UL muting on a PTRS symbol with transform precoding necessarily needs to be supported, in particular when considering UE implementation challenges for the Alt-1 to Alt-3.

- Proposal 3: If an UL resource muting symbol overlaps with a symbol containing PTRS for a PUSCH and when transform precoding is enabled for PUSCH, PTRS is prioritized, i.e., UE does not apply UL resource muting in the symbol.

The following FL proposal was discussed in RAN1#120.

#### Proposal 1-11

When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, the following alternatives are further studied for PT-RS insertion in a block of MscPUSCH2 samples prior to transform precoding:

- Alt-1

  - NgroupPT-RS and Nsampgroup are determined as legacy from 38.214 Table 6.2.3.2-1

  - PT-RS sample positions determined as legacy, but with MscPUSCH replaced by MscPUSCH2  in 38.211 Table 6.4.1.2.2.2-1

- Alt-2

  - NgroupPT-RS and Nsampgroup are determined as legacy, but with NRB replaced by NRB2 in 38.214 Table 6.2.3.2-1

  - PT-RS sample positions determined as legacy, but with MscPUSCH replaced by MscPUSCH2  in 38.211 Table 6.4.1.2.2.2-1

- Alt-3

  - NgroupPT-RS and Nsampgroup are determined as legacy from 38.214 Table 6.2.3.2-1

  - PT-RS sample positions are determined as legacy, but only for the first NgroupPT-RS2 groups

Note: After PT-RS insertion, all of the mathematically equivalent DFT Options 1, 2A, and 2B from the RAN1#120 agreement are applicable, where the choice is up to UE implementation

Among the three alternatives, Alt-3 introduces a whole new PTRS pattern (Figure 1) where (NgroupPT-RS,NsampPT-RS)=2,2,2,4, 4,2, 4,4,8,4 and NscPUSCH=120  (10RBs). As agreed, the signal with comb-2 muting can be represented as the block of MscPUSCH2 modulated symbols repeated before DFT. In Alt-3 as by the example in Figure 1, the PT-RS pattern within the first 60 samples is repeated and thus a PT-RS pattern within 120 samples results.

Figure 1: PT-RS pattern in Alt-3

We can observe that

- If (NgroupPT-RS,NsampPT-RS)=2,2 or 4,2, the PTRS pattern of Alt-3 is exactly same as the legacy PTRS pattern.

- If (NgroupPT-RS,NsampPT-RS)=2,4 or 4,4 or 8,4, the PTRS pattern of Alt-3 is different from the legacy PTRS pattern. The PTRS pattern of Alt-3 is new and never used before. Actually, the legacy PTRS pattern is mirrored at the middle of samples and the first samples and the last samples are used for PTRS and the remaining PTRS groups are equally spread over all samples. However, the PTRS pattern of Alt-3, these characteristics do not hold any more, i.e., no longer equally spread over all samples

- Proposal 4: Do not introduce any new PT-RS patterns in Rel-19.

## Additional restrictions and UE features

In order to keep the expected specification effort in Rel-19 SBFD commensurate when introducing UL resource muting for PUSCH, a number of restrictions and conditions were already captured by the revised WID [1].

In our view, additional restrictions or conditions of UL resource muting for PUSCH are motivated by operational assumptions. The following FL proposal was made in RAN1#118bis.

FL proposal 1-7 (RAN1#118bis)

Study the necessity of following conditions to apply UL resource muting for a PUSCH

- Condition 1: When the number of PUSCH symbols is less than threshold, UL resource muting is not applied

- Condition 2: If the scheduled PUSCH uses a MCS lower than certain reference MCS index, UL resource muting is not applied

- Condition 3: When the number of scheduled RBs is less than threshold, UL resource muting is not applied

- Condition 4: When the number of DMRS symbols is larger than threshold, UL resource muting is not applied

- Condition 5: UL resource muting is not applied according to the PHY priority of PUSCH

- Condition 6: For PUSCH that is a retransmission, UL resource muting is not applied

- Condition 7: For PUSCH that is scheduled by DCI format 0_0, UL resource muting is not applied

- Condition 8: for PUSCH with repetition type B, UL resource muting is not applied

- Condition 9: For PUSCH carrying certain UCI type with high PHY priority, UL resource muting is not applied.

- Other conditions are not precluded.

Condition 1: UL resource muting is not motivated for short PUSCH time-domain allocations, e.g., <7 symbols.

The gNB can create a scheduling gap, i.e., insert an un-scheduled symbol between back-to-back PUSCH allocations in the slot (Figure 1). The DMRS symbol and the un-scheduled/transparent muting symbol are then separated only by at most 4 symbols using PUSCH mapping type A. For PUSCH mapping type B, even more flexibility exists and the un-scheduled/transparent muting symbol of the short PUSCH allocation can occur before the first symbol of the PUSCH TDRA in the slot.

The resulting computational delay between CLI estimation and PUSCH equalization is deemed acceptable. It is not necessary to defer CLI equalization during PUSCH Rx processing until the end of the NR slot compared to the case when the un-scheduled/transparent symbol is allocated as last symbol of the slot.

Figure 2: No support for UL resource muting for PUSCH <7 symbols

Another reason to only support UL resource muting for long PUSCH time-domain allocations is that more flexibility exists to avoid impacts to the UCI mapping and in particular the A/N carrying REs following the FL DMRS symbol(s). It becomes more easily possible to configure the comb-2 muted symbol after the UCI carrying symbols in the PUSCH allocation. MCS selection during gNB scheduling and the TBS determination for the PUSCH can more easily absorb the presence of comb-2 muted REs on a symbol in the long PUSCH allocation, i.e., the number of muted REs is small when compared to the total number of REs allocated for PUSCH NRE.

- Proposal 5: UL resource muting is not applied to PUSCH allocations <7 symbols.

(Modified) Condition 9: For PUSCH carrying certain UCI type with high PHY priority, UL resource muting is not applied.

With respect to UCI handling and UL muting, the following latest RAN1 agreements were made.

Agreement (RAN1#118bis)

For UCI rate matching, to determine the number of coded modulation symbols per layer for UCI transmission on PUSCH, the following option is adopted

- Option 2: The number of coded modulation symbols per layer for UCI on PUSCH is calculated based on MscUCIl after excluding the UL muting resource.

Agreement (RAN1#119)

Confirm the following working assumption

Working Assumption

UL resource muting is allowed on PUSCH symbols that carry UCI, i.e., UCI is not mapped on muted REs

Note: As mentioned in the WID, no specification changes are allowed on data and control multiplexing in section 6.2.7, TS 38.212.

It is assumed by the revised WID that no impact on data and control multiplexing as defined in section 6.2.7 “Data and control multiplexing” in TS 38.212 occurs. One reason is to keep the Rel-19 specification effort commensurate.

As by the RAN1#119 agreement, UL resource muting is allowed on PUSCH symbols that carry UCI, i.e., UCI is not mapped on muted REs.

However, while a direct specification impact and corresponding change to implementation for the UCI coding in 38.212 Section 6.2.7 can be avoided, there are indirect impacts on existing specifications.

Any configuration of a comb-2 muted PUSCH symbol on a symbol containing the CSI or A/N still requires to address the UCI resource determination. This is because several loop variables such as the set of REs available for transmission of UCI in the OFDM symbol or the computation of the MscUCI(l) still need to account for absence of every 2nd RE. Some impact, i.e., new adjustment factor, therefore result in section 6.2.7 of 38.212. For example, in the Rel-15 NR specifications, loop variables are set to 0 for symbols carrying the DMRS.

It should also be considered that for the mapping of the UL muting symbol after the FL DMRS (for PUSCH mapping type A and B), configuration of the muted symbol to A/N-carrying symbols can be avoided in principle. The size of the A/N payload in a PUSCH is typically small even if A/N is frequently present.

Depending on the semi-static rate-matching factor settings, i.e., beta value, it is not typical that the channel coded A/N bits map to more than one data symbol after the FL DMRS in the PUSCH allocation. Configuration and indication of the UL muting symbol on the next following data symbol where no A/N UCI is present is then a viable option.

If the UL muting symbol occurs before the FL DMRS (for PUSCH mapping type A), its symbol location would necessarily overlap with a CSI Part 1 or 2, if present. However, it is not a reasonable assumption that CSI is present in every PUSCH allocation. Aperiodic CSI reporting is usually only triggered at reasonably large reporting intervals, e.g., every several tens of msec’s. Periodic CSI reporting due to overhead is often configured at larger intervals. Therefore, even if the CSI payload can potentially become large, i.e. up to 2000 bits or more, depending on the configured CSI reporting mode, the CSI reporting in the PUSCH allocation is infrequent.

The need to design for UL muting before the FL DMRS on CSI carrying symbols in the PUSCH allocation for mapping type A can simply be avoided by not applying the UL muting in the occasional PUSCH where a CSI is present. Note that the muted symbol after FL DMRS can still be used, i.e., UL muting is always available to equalize for PDSCH-PUSCH CLI even if the additional ability to equalize for PDCCH-PUSCH CLI is then lost for a few intermittent PUSCH transmissions where CSI reporting needs to occur.

In summary, we think that mandatory support of UL muting on PUSCH symbols that can carry UCI is not required. For Rel-19 UEs not supporting such a functionality, the gNB can configure time-domain locations of UL muting symbols based on the max. UCI payload assumption and accounting for the beta offsets. When the UE not supporting UL muting on PUSCH symbols that can carry UCI determines presence of UCI on such a symbol resulting from the control/data multiplexing for PUSCH, the UE does not apply the UL muting the in the symbol.

- Proposal 6: Support of UL muting on PUSCH symbols that can carry UCI is an optional UE capability in Rel-19 SBFD.

- Proposal 7: When the UE does not indicate support for UL muting on PUSCH symbols that can carry UCI, and the UE determines presence of UCI on a configured/indicated UL muting symbol, the UE does not apply the UL resource muting in the symbol.

(New) Condition 10: UL resource muting should not be supported for PUSCH with UCI-only payload.

Even with a combined CSI and A/N payload size of ~2000 bits, the resulting PUSCH allocation would typically not exceed a few symbols over the PUSCH allocation BW. In absence of data payload in the PUSCH, the resulting specification effort to support a comb-2 muted UL symbol increases. For UCI-only PUSCH, a transparent muting symbol can be created by the gNB as scheduling gap following the UCI-only PUSCH similar to the case of short PUSCH allocations.

- Proposal 8: UL resource muting is not applied to PUSCH carrying UCI-only payload.

# L1 based UE-to-UE measurement and reporting

In RAN#104, the WID was revised to include support for L1 based CLI measurement and reporting based on the existing CSI framework.

- L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4]

  - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource

  - At least aperiodic reporting

  - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

  - UCI bits generation

  - Priority rules for multiple CSI reporting

  - CLI measurement accuracy requirement

- Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting

  - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs

  - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting

  - For the new reporting quantities, at least wideband reporting is supported

  - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

## L1 CLI measurement resources

The following RAN1 agreements were made with respect to the configuration of the L1 CLI measurement resources.

Agreement (RAN1#118bis)

Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement.

- The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic

- The number of periodic/semi-persistent CLI measurement resource sets is restricted to 1 in a CSI-ResourceConfig as in current specifications.

- Note: No new usage needs to be defined for SRS resource sets

Agreement (RAN1#119)

Define a new IE SRS-RSRP-MeasurementResourceSet containing a set of SRS-RSRP measurement resource(s) SRS-RSRP-MeasurementResource for L1 SRS-RSRP measurement

- Configuration of slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured (already agreed in RAN1#118bis)

- FFS: Other configurations

Define SRS-RSRP-MeasurementResource with the following parameters:

- Legacy SRS Resource IE

- FFS: Other parameters

Agreement (RAN1#119)

Define a new IE CLI-RSSI-MeasurementResourceSet containing a set of CLI-RSSI measurement resource CLI-RSSI-MeasurementResource for L1 CLI-RSSI measurement.

- Configuration of the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured (already agreed in RAN1#118bis)

Define CLI-RSSI-MeasurementResource with the following parameters:

- CLI-RSSI measurement resource ID

- Starting PRB index

- Number of PRBs

- Starting symbol of the CLI-RSSI resource within a slot

- Number of symbols of the CLI-RSSI resource within a slot

- Periodicity and slot offset for the CLI-RSSI resource

- FFS: Other parameters

Agreement (RAN1#120)

- UE determines the subcarrier spacing of the CLI-RSSI measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig.

- UE determines the subcarrier spacing of the SRS-RSRP measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig.

- Note: No RAN1 spec impact is expected.

## L1 CLI reporting

The following RAN1 agreements and conclusions were reached with respect to L1 CLI reporting.

Agreement (RAN1#118)

For L1 UE-to-UE CLI measurement and reporting, support two additional report quantities {‘cli-RSSI’, ‘cli-SRS-RSRP’} to the higher layer parameter reportQuantity.

- FFS: configuration of number of reported CLI resources in the reportConfig

- FFS: reporting criteria

Agreement (RAN1#118)

For L1 UE-to-UE CLI measurement and reporting, the following are supported

- Wideband CLI-RSRP reporting

- Wideband CLI-RSSI reporting

- FFS: Subband CLI-RSSI reporting

Agreement (RAN1#118bis)

In CSI-ReportConfig, reuse resourcesForChannelMeasurement providing the ID of a CSI-ResourceConfig that contains configuration of measurement resources for L1-SRS-RSRP and/or L1-CLI-RSSI.

Agreement (RAN1#118bis)

For aperiodic CLI reporting on PUSCH

- Reuse CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig

- Reuse the existing DCI field 'CSI request' to trigger an aperiodic CLI report

- Introduce an indication in CSI-AssociatedReportConfigInfo that selects a set from the list of CLI-RSSI measurement resource sets or SRS-RSRP measurement resource sets configured in CSI-ResourceConfig

- Note: As in current specifications, the parameter for indication (e.g., resourceSet) is always present, but only used in the case that the CSI-ResourceConfig is configured with multiple sets of aperiodic CLI measurement resources.

Agreement (RAN1#118bis)

For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH, support the following in the IE CSI-AperiodicTriggerStateList:

- In CSI-AssociatedReportConfigInfo a list of TCI state(s) with qcl-Type set to 'typeD' is optionally configured where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo

  - The list of TCI states can be configured (present) only if unifiedTCI-StateType is configured and the CLI measurement resource(s) are aperiodic

- If the list of TCI states is not configured (absent), the UE assumes the following for each CLI measurement resource:

  - If unifiedTCI-StateType is not configured

    - Same assumption as for L3-based CLI measurement as in 38.133: “For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET”

  - If unifiedTCI-StateType is configured

    - “Indicated” DL only/joint TCI state as specified in TS 38.214

  - Note: Above default rules apply to aperiodic reporting based on aperiodic, semi-persistent, and periodic CLI measurement resources

FFS: details of configuration of TCI state(s) for aperiodic reporting based on periodic and semi-persistent CLI measurement resources

Agreement (RAN1#118bis)

For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs.

- A single wideband RSSI measurement report is supported

  - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband.

Conclusion (RAN1#119)

There is no RAN1 consensus on the following:

- Subband CLI-RSSI reporting for L1 CLI-RSSI measurement in Rel-19.

Conclusion (RAN1#119)

For Method#1 (RSSI measurement within DL subband) with the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands), two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband is not supported.

Agreement (RAN1#119)

For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of periodic CLI measurement resources, the TCI state(s) with QCL typeD for the CLI measurement resources in the periodic CLI measurement resource set are configured per CLI measurement resource (either for all resources in the set or none) by higher-layer parameter

- For each periodic CLI measurement resource, this higher-layer parameter provides a reference to one TCI-State in TCI-States for providing the QCL source and QCL typeD.

- If the TCI state is not configured, the UE QCL assumptions are the default rules agreed in RAN1#118bis

For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE

- TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none.

- If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis

Agreement (RAN1#120)

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’

- carrier in CSI-ReportConfig is used to indicate in which serving cell the CLI-RSSI measurement resources or SRS-RSRP measurement resources in CSI-ResourceConfig are to be found. If the field is absent, the resources are on the same serving cell as this report configuration.

- bwp-Id in the associated CSI-ResourceConfig is used to indicate the DL BWP where the CLI-RSSI measurement resources or SRS-RSRP measurement resources are located in.

Note: No RAN1 specification impact

## Discussion

### SBFD specific UE-to-UE CLI measurement issue

The following latest RAN1 agreements were made.

Agreement (RAN1#118)

For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported

- Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband

- Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands

- FFS: Number of resources that can be configured

- FFS: UE behaviour for measurement

Agreement (RAN1#118)

For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- FFS: Method#3: UE measures RSSI within UL subband

Agreement (RAN1#119)

For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported

- Method #3: UE measures RSSI within UL subband

- A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig.

Agreement (RAN1#120)

Update the following agreement in RAN1#118 (in red)

For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported

- Measurement resource type#1: OneThe CLI-RSSI measurement resource is configured within a DL subband or a UL subband

- Measurement resource type#2: OneThe CLI-RSSI measurement resource is configured across two DL subbands

- FFS: Number of resources that can be configured

FFS: UE behavior for measurement

One open issue is how to account for the number of supported L1 CLI-RSSI measurement resources by the UE in terms of resource counting on the SRS-RSRP and CLI-RSSI measurement resources.

In particular, for the case of measurement resource type#2 where the CLI-RSSI measurement resource is configured across two DL subbands, it should be avoided to increase the UE complexity when compared to Rel-16 CLI-RSSI reporting.

Another open issue is the maximum number of L1-CLI-RSSI or the L1-SRS-RRSP measurement resources that can be configured.

If a UE supports Rel-16 CLI-RSSI reporting, the UE indicates the maximum number of measurement resources configurable for CLI-RSSI measurement using maxNumberCLI-RSSI-r16. If a UE supports Rel-16 SRS-RSRP reporting, the UE indicates a maximum number of measurement resources across all CCs configurable for the SRS-RSRP measurement using maxNumberCLI-SRS-RSRP-r16, and a maximum number of measurement resources across all CCs configured for SRS-RSRP measurement within a slot using maxNumberPerSlotCLI-SRS-RSRP-r16.

Above were discussed in RAN1#119 and RAN1#120 but without reaching a conclusion/agreement.

FL proposal 2-5 Round 3 (RAN1#119)

The maximum configured number of L1 CLI-RSSI measurement resources in CLI-RSSI-MeasurementResourceSet is same as maxNrofCLI-RSSI-Resources-r16.

- The supported number of CLI-RSSI measurement resources is up to UE capability

The maximum configured number of L1 SRS-RSRP measurement resources in SRS-RSRP-MeasurementResourceSet is same as value of maxNrofSRS-RSRP-Resources-r16.

- The supported number of SRS-RSRP measurement resources is up to UE capability

#### FL proposal 2-3 Round 1 (RAN1#120)

For L1-CLI-RSSI measurement and reporting, a UE reports the maximum number of supported CLI-RSSI measurement resources of all three measurement types across all CCs (sum of measurement resource type#1, measurement resource type#2 and measurement resource type#3)

From the signaling perspective, the same maxNrofCLI-RSSI-Resources can be supported as in Rel-16, i.e., 64.

For Method#1 (RSSI in DL subband) and measurement resource type#1 where the CLI-RSSI measurement resource is configured within a DL subband, the existing Rel-16 CLI-RSSI measurement resource is reused, i.e., configured as a number of contiguous RBs with start PRB and a number of PRBs as integer multiple of 4.The UE can measure and report L1-CLI-RSSI for as long as the CLI-RSSI measurement resource is configured by the gNB within the SBFD DL subband and within the active DL BWP of the UE.

For Method#1 (RSSI in DL subband) and measurement resource type#2 where the CLI-RSSI measurement resource can be configured across two DL subbands, the Rel-16 CLI-RSSI measurement resource is configured as a number of contiguous RBs with start PRB and a number of PRBs as integer multiple of 4. The UE discards, i.e., does not consider the RBs falling into the SBFD UL subband and/or guard band(s) if present for the L1-CLI-RSSI measurement. The SBFD-aware UE should then follow the same approach in terms of resource counting as for the CSI-RS resources and CSI reporting in the general case. The UE implementation can implement a measurement resource type #2 as two separate CLI-RSSI measurement processes, i.e., one for each DL subband, therefore separate counting is required.

For Method#2 (RSRP in UL subband) or Method#2 (RSSI in UL subband), the configured SRS or RSSI measurement resource is contiguous, and the existing Rel-16 behavior can be followed.

- Proposal 9: The CLI-RSSI measurement resource type#2 is counted as two L1 CLI-RSSI measurement resource type#1.

- Proposal 10: The maximum configurable number of L1 CLI-RSSI measurement resources in CLI-RSSI-MeasurementResourceSet is same as maxNrofCLI-RSSI-Resources-r16.

- Proposal 11: The supported number of CLI-RSSI measurement resources is subject to UE capability.

- Proposal 12: The maximum configurable number of L1 SRS-RSRP measurement resources in SRS-RSRP-MeasurementResourceSet is same as value of maxNrofSRS-RSRP-Resources-r16.

- Proposal 13: The supported number of SRS-RSRP measurement resources is subject to UE capability.

Another aspect to consider is measurement resource configuration for the possible case that Rel-16 L3 CLI reporting and the Rel-19 SBFD L1 CLI reporting are both/concurrently configured for the UE. For example, from the L1 perspective, a configured CLI-RSSI measurement resource may then be used to generate a sample for the periodic L3 CLI report or for aperiodic L1 reporting. A limitation to one/same CLI-RSSI resource instead of two concurrent ones is needed.

- Proposal 14: Down-select from the following 2 options

- Option 1: the UE doesn’t expect to be configured for Rel-16 L3 CLI reporting and Rel-19 L1 reporting.

- Option 2: when the UE is configured for both Rel-16 L3 CLI reporting and Rel-19 L1 reporting, the UE expects to be configured with a maximum number of CLI-RSSI or SRS-RSRP measurement resources according to the reported Rel-19 UE capability.

### Reporting

According to the revised WID, at least aperiodic CLI reporting is supported for L1 UE-to-UE CLI reporting. The following agreements and conclusions were then made in RAN1#119 and RAN1#120.

Conclusion (RAN1#119)

There is no RAN1 consensus to support aperiodic CLI reporting on PUCCH in Rel-19

Agreement (RAN1#120)

- Periodic L1 CLI-RSSI reporting for L1 based UE-to-UE CLI measurement and reporting is supported

- Semi-persistent L1 CLI-RSSI reporting for L1 based UE-to-UE CLI measurement and reporting is NOT supported

- Periodic/Semi-persistent L1 SRS-RSRP reporting for L1 based UE-to-UE CLI measurement and reporting is NOT supported

For the aperiodic SRS-RSRP and CLI-RSSI reporting, the followingRAN1 agreements were made.

Agreement (RAN1#118bis)

For aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured is configured by higher layer parameter.

- FFS: Whether to reuse the legacy SRS resource set

- FFS: Whether available slot offset list is also reused

For aperiodic CLI-RSSP reporting using aperiodic CLI-RSSI measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured is configured by higher layer parameter.

Agreement (RAN1#120)

If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for aperiodic CLI reporting and the first symbol of the aperiodic SRS-RSRP measurement resource or CLI-RSSI measurement resources is smaller than the UE reported threshold beamSwitchTiming, down-select from the following alternatives:

- Alt.1: UE does not expect to be configured with a scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming.

- Alt.2: Same default beam rules as AP CSI-RS resource defined in 38.214 can be applied to AP CLI SRS-RSRP or CLI-RSSI resource.

FFS: whether Alt.1 or Alt.2 has any specification impact.

In our view, the default rule according to Alt.2 can be applied.

- Proposal 15: If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for aperiodic CLI reporting and the first symbol of the aperiodic SRS-RSRP measurement resource or CLI-RSSI measurement resources is smaller than the UE reported threshold beamSwitchTiming, support Alt.2.

### Report mapping

The following agreements and conclusions were made in RAN1#119 and RAN1#120.

Agreement (RAN1#119)

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, the following are supported

- The number of reported CLI measurement resources M can be configured as {1, 2, 3, 4}

- The M CLI measurement resource ID(s) are reported

- The absolute (not differential) value of the most interfered (largest) measured L1-CLI-RSSI or L1-SRS-RSRP is reported

- When the number of reported CLI measurement resources is larger than one, a differential reporting method is used taking the most interfered measured L1-CLI-RSSI or L1-SRS-RSRP as the reference

- FFS: other reporting criteria, e.g., the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP

- FFS: whether/how to report differential value in case the largest measurement is out of range

Conclusion (RAN1#120)

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, new reporting criteria considering the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP are not supported.

Working assumption (RAN1#120)

For L1 based CLI measurement and reporting, the following is adopted

- For L1 SRS-RSRP, a 7-bit value in the quantization range [-140, -44] dBm with 1dB step size, and the differential L1 SRS-RSRP is quantized to a 4-bit value. The differential L1 SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1 SRS-RSRP value.

- For L1 CLI-RSSI, a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size, and the differential L1 CLI RSSI is quantized to a 4-bit value. The differential L1 CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1 CLI-RSSI value.

Send an LS to RAN4 about the above working assumption. LS on L1 based UE-to-UE CLI measurement and reporting to RAN4 is agreed. Final LS in R1-2501620.

Agreement (RAN1#120)

For discussion purpose, SRS-RSRP measurement resource index is referred to as SRS-RSRP-MRI and CLI-RSSI measurement resource index is referred to as SRS-RSRP-MRI.

The mapping order of CSI fields of one report for SRS-RSRP-MRI/SRS-RSRP or CLI-RSSI-MRI/CLI-RSSI reporting is provided in Table 1 with following modifications based on Table 6.3.1.1.2-8 defined in TS38.212

- CRI and SSBRI is replaced with SRS-RSRP-MRI or CLI-RSSI-MRI.

- Capability index is crossed out in the table.

- RSRP is replaced with L1-SRS-RSRP

- L1-CLI-RSSI is added in the table if the CSI report is associated with CLI-RSSI measurement resource.

Table 1: Mapping order of CSI fields of one report for SRS-RSRP-MRI/SRS-RSRP or CLI-RSSI-MRI/CLI-RSSI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | SRS-RSRP-MRI or CLI-RSSI-MRI #1 as in Table 2, if reported |
|  | SRS-RSRP-MRI or CLI-RSSI-MRI #2 as in Table 2, if reported |
|  | … |
|  | SRS-RSRP-MRI or CLI-RSSI-MRI #4 as in Table 2, if reported |
|  | L1-SRS-RSRP or L1-CLI-RSSI #1 as in Table 2, if reported |
|  | Differential L1-SRS-RSRP or L1-CLI-RSSI #2 as in Table 2, if reported |
|  | … |
|  | Differential L1-SRS-RSRP or L1-CLI-RSSI #4 as in Table 2, if reported |

Table 2. Bitwidth for SRS-RSRP-MRI, SRS-RSRP, CLI-RSSI-MRI and CLI-RSSI

| Field | Bitwidth |
| --- | --- |
| SRS-RSRP-MRI | ⌈log2KsSRS-RSRP⌉ |
| CLI-RSSI-MRI | ⌈log2KsCLI-RSSI⌉ |
| L1-SRS-RSRP | 7 |
| L1-CLI-RSSI | 7 |
| Differential RSRP | 4 |
| Differential RSSI | 4 |

KsSRS-RSRP: Number of SRS-RSRP measurement resources in the corresponding resource set

KsCLI-RSSI: Number of CLI-RSSI measurement resources in the corresponding resource set

In our view, we think that the above agreements are in principle sufficient. We do not think that it is justified to support new/additional report quantities such as ‘cli-RSSI-measurement-resource-bitmap. When a bitmap corresponding to a set of CLI-RSSI measurement resource indices indicating whether L1-CLI-RSSI is above a given threshold is reported by the UE, the gNB can at best schedule into the set of UEs which were identified below the threshold setting. In practice, we expect little (if any) usefulness) for such as feature. The ability of the gNB to co-schedule UEs in the DL and UL subbands depends on availability of DL/UL traffic in the TTI and proximity. Proximity, i.e., cases of severe UE-to-UE interference can be identified by the gNB using proprietary techniques such as location estimation.

### CPU occupation and CSI computation time

The following agreements was made in RAN1#120.

Agreement (RAN1#120

For L1 CLI measurement and reporting, the number of occupied CPU(s) is determined as follows

- OCPU=1 for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’, ‘cli-SRS-RSRP’.

In our view, the CPU timeline for L1 beam reporting can be reused for the Rel-19 L1 CLI measurement and reporting.

A concern which was discussed in previous RAN1 meetings is that for L1 based UE-to-UE CLI measurement and reporting, a constant offset relative to the DL reference timing in the serving cell or UL reference timing is applied. This in turn may have some impact on the starting symbol of CPU occupation. We think however that under the intra-cell CLI measurement assumption as by the RAN1#119 LS to RAN4, the offset is small and can be absorbed in the CPU processing timeline.

- Proposal 16: The existing CSI timeline for L1 beam reporting is reused for the aperiodic CLI measurement and reporting in Rel-19 SBFD.

# Conclusion

In this contribution, we made the following proposals for UL resource muting for PUSCH:

Proposal 1: UL resource muting can be applied in non-SBFD symbols.

- Proposal 2: When determining “the total number of coded bits available for transmission of the transport block” used in TS38.212, the muted REs are not considered.

- Note: No specification impacts are expected.

- Proposal 3: If an UL resource muting symbol overlaps with a symbol containing PTRS for a PUSCH and when transform precoding is enabled for PUSCH, PTRS is prioritized, i.e., UE does not apply UL resource muting in the symbol.

- Proposal 4: Do not introduce any new PT-RS patterns in Rel-19.

- Proposal 5: UL resource muting is not applied to PUSCH allocations <7 symbols.

- Proposal 6: Support of UL muting on PUSCH symbols that can carry UCI is an optional UE capability in Rel-19 SBFD.

- Proposal 7: When the UE does not indicate support for UL muting on PUSCH symbols that can carry UCI, and the UE determines presence of UCI on a configured/indicated UL muting symbol, the UE does not apply the UL resource muting in the symbol.

- Proposal 8: UL resource muting is not applied to PUSCH carrying UCI-only payload.

In this contribution, we made the following proposals for L1 based UE CLI measurement/reporting:

- Proposal 9: The CLI-RSSI measurement resource type#2 is counted as two L1 CLI-RSSI measurement resource type#1.

- Proposal 10: The maximum configurable number of L1 CLI-RSSI measurement resources in CLI-RSSI-MeasurementResourceSet is same as maxNrofCLI-RSSI-Resources-r16.

- Proposal 11: The supported number of CLI-RSSI measurement resources is subject to UE capability.

- Proposal 12: The maximum configurable number of L1 SRS-RSRP measurement resources in SRS-RSRP-MeasurementResourceSet is same as value of maxNrofSRS-RSRP-Resources-r16.

- Proposal 13: The supported number of SRS-RSRP measurement resources is subject to UE capability.

- Proposal 14: Down-select from the following 2 options

- Option 1: the UE doesn’t expect to be configured for Rel-16 L3 CLI reporting and Rel-19 L1 reporting.

- Option 2: when the UE is configured for both Rel-16 L3 CLI reporting and Rel-19 L1 reporting, the UE expects to be configured with a maximum number of CLI-RSSI or SRS-RSRP measurement resources according to the reported Rel-19 UE capability.

- Proposal 15: If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for aperiodic CLI reporting and the first symbol of the aperiodic SRS-RSRP measurement resource or CLI-RSSI measurement resources is smaller than the UE reported threshold beamSwitchTiming, support Alt.2.

- Proposal 16: The existing CSI timeline for L1 beam reporting is reused for the aperiodic CLI measurement and reporting in Rel-19 SBFD.

# References

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”; RAN#104

- TR38.858 v2.0.0 “Study on Evolution of NR Duplex Operation (Rel-18)”; RAN#102

- R1-2502351 “Discussion on LS reply to RAN4 on L1 CLI measurement”; Samsung

- R1-2502365 “SBFD operation and procedures”; Samsung

- R1-2502366 “Random access on SBFD resources”; Samsung

- R1-2500849 “Cross-link interference handling for SBFD”; Samsung

- R1-2502393 “UE features for Rel-19 Duplex Evolution for NR”; Samsung