3GPP TSG RAN WG1 #120bis			R1-2502425

Wuhan, P. R. China, April 7th – 11th, 2025

Agenda Item:	9.15.3

Source:	Ericsson

Title:	UE-features for SBFD

Document for:	Discussion, Decision

# Introduction

In this contribution we present our views on the UE feature list for the SBFD WI [1]. In doing so, we have tried to adhere to the guidelines in [2] and the updated guidelines in [3].

# Discussion

The SBFD WI consists of three major objectives,

1.	Time/frequency configuration and signal/channel-specific behavior,

2.	Random access, and

3.	CLI,

and we think it makes sense to also address UE features along the same divide.

A general proposal to the SBFD UE-feature list from a network vendor’s perspective is that the number of UE-features should be limited, in order to reduce the complexity in the network. Furthermore, much of the functionality should be collected in baseline features, e.g., for time and frequency configuration and signal/channel scheduling and RACH.

RAN1 to strive to keep the number of UE-features small for SBFD.

## T/F configuration and signal/channel-specific behavior

Include support for Configuration 1, including determination of valid symbol type, in the basic FG (Alt. 1).

Include support for separate UL configurations for SBFD and non-SBFD symbols in the basic FG (Alt. 1).

Include support for DL reception in two non-contiguous DL subbands in basic FG (Alt. 1).

Include support for separate CSI reports for SBFD and non-SBFD symbols, including the determination of subbands for CSI reporting, in basic FG (Alt. 1).

Include support for DL partial PRG in basic FG (Alt. 1).

Include support for transmissions and receptions across SBFD and non-SBFD symbols in a single FG for both UL transmission and DL reception (Alt. 1).

Include support for determination of transport block size of PDSCH/PUSCH subject to valid symbol type, FDRA in DCI and SBFD configuration, in basic FG.

In RAN1 #120, Configuration 1 and Configuration 2 were introduced as UE-features with components as shown in Table 1. In our view, it is a poor choice of naming to use the same component naming for opposing features, as is shown by the magenta highlighting. The same applies to the column describing the consequences for not supporting the feature. The only way for a reader to be able to differentiate between Component 1 of FG X-1a and FG X-2, respectively, as well as the consequences for not supporting it, is to disregard the main clause of the sentence and focus on the following subordinate clause for Configuration 1, alternatively resort to a definition of ‘Configuration 1’ and ‘Configuration 2’ which are names that may not even be included in the specifications.

Table 1: FGs and components for Configuration 1 and Configuration 2, respectively.

| Index | Feature group | Components | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- |
| [X-1a] | [Transmission and reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 1)] | [1. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1)] | [Transmission and reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 1) are not supported |
| X-2 | [Transmission and reception] across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | [1. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] [2. Support of DL reception across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: whether each of components is defined as separate FG | [Transmission and reception] across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) are not supported |

Instead, we propose to reformulate the names and descriptions such that they are unmistakenly unique, as presented in Table 2. Here, there is a clear distinction between supporting multi-slot transmissions and receptions in either SBFD or non-SBFD symbols in different slots and supporting multi-slot transmissions and receptions in both SBFD and non-SBFD symbols in different slots.

Table 2: Proposed component naming and descriptions for FGs X-1a and X-2.

| Index | Feature group | Components | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- |
| [X-1a] | [Transmission and reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 1)] | [1. Support of multi-slot UL transmissions or DL receptions in either SBFD symbols or non-SBFD symbols in different slots | [Multi-slot transmission and reception in either SBFD symbols or non-SBFD symbols in different slots is not supported] |
| X-2 | [Transmission and reception] across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | [1. Support of multi-slot UL transmissions in both SBFD symbols and non-SBFD symbols in different slots [2. Support of multi-slot DL receptions in both SBFD symbols and non-SBFD symbols in different slots] | [Multi-slot transmission and reception in both SBFD symbols and non-SBFD symbols in different slots is not supported] |

Using the same component names or descriptions for contradictory capabilities in different FGs opens for ambiguous interpretations and is bad practice. If Configuration 2 is not specified (which there is no need for), there is no difference.

For FG X-1a, change Component name to ‘1. Support of multi-slot UL transmissions or DL receptions in either SBFD symbols or non-SBFD symbols in different slots’.

For FG X-2, change Component name to ‘1. Support of multi-slot UL transmissions in both SBFD symbols and non-SBFD symbols in different slots’ and ‘2. Support of multi-slot DL receptions in both SBFD symbols and non-SBFD symbols in different slots’.

For both FGs X-1a and X-2, change consequence description to ’Multi-slot transmission and reception in both SBFD symbols and non-SBFD symbols in different slots is not supported’.

## Random access

RAN1 #119 agreed to support Configuration 1 for Msg3 repetitions [5]:

Agreement For an SBFD aware UE, if addition RO is selected for Msg1 transmission, only Configuration 1 is supported for Msg3 repetition.

I our view, the above agreement leaves no room for reinterpretation and should be adhered to. Hence, a UE supporting Msg3 repetitions must support at least Configuration 1 and only supporting Configuration 2 is not an alternative for SBFD RACH.

Apart from Msg3 repetitions, in our view there is no additional need to support either Configuration 1 or Configuration 2 with respect to the PRACH preamble or RAR. RAN1 has also not discussed these configurations in any other context in AI 9.3.2. Both PRACH preamble transmission and RAR reception are unique operations during which time the UE has very little else to do. Hence, it should not be a problem for a UE to either transmit PRACH or receive RAR independently of whether it supports Configuration 1 or Configuration 2. Considering that RAN1 has already agreed to support Configuration 1 for Msg3 repetitions, and that Configuration 1 is the default configuration for all other signals and channels, it is reasonable that it is also the configuration that is assumed during the remainder of the RACH procedure.

RAN1 agreements clearly states that Configuration 1 is used for Msg3 repetitions.

Configurations 1 and 2 does not apply to PRACH or RAR.

A UE supporting the single RACH configuration needs to support at least Configuration 1 (Alt. 1).

RAN1 also discussed whether Option 1 or Option 2, if any, should be the default feature. In our view, the only thing Option 1 and Option 2 have in common is that they are both specified withing the SBFD WI. Apart from that, they address completely different use cases and even different UE types. Whereas Option 2 can be found in eMBB UEs, Option 1 addresses IoT and MTC UEs. This view is also supported by the agreement on the two options where it is clearly stated that a “UE is not required to support both options.” Based on that, it is not possible to claim that one option should have precedence over the other.

Option 1 and Option 2 target different UE types and therefore, per agreement, a UE is not expected to support both options.

Support separate FGs for the single and additional RACH configurations.

Another point up for discussion is how to handle the network configuration where a valid RO spans both SBFD and non-SBFD symbols. For that particular use case, to perform initial access with an extended range, it makes little sense to have this component as a separate UE-feature. In that case, the SBFD-aware UE supporting Option 2 would still not be able to connect to the network due to the network being configured with ROs across both SBFD and non-SBFD symbols. That kind of functionality mismatch should surely be avoided, and since RAN1 agreed that this feature is configurable on the network side, it should be mandatory on the UE side, for UEs supporting Option 2.

Support for ROs across both SBFD and non-SBFD symbols is a network configuration but should not be a separate UE-capability to avoid capability mismatch.

Support of a valid RO across SBFD and non-SBFD symbols is a component of the FG for SBFD random access based on the additional RACH configurations (Option 2) (Alt. 1).

A common design aspect of the single RACH configuration (Option 1) is that it should not introduce any additional parameters. In our view, that is also the case for UE-features. Hence, for Option 1, PRACH preamble repetitions and Msg3 repetitions can be determined by the corresponding legacy feature. Additionally, there is in our view no technical merit to why an SBFD-aware UE also supporting preamble repetitions or Msg3 repetitions in legacy RACH should not be able to also support it for Option 1.

In contrast, for the additional RACH configuration (Option 2), a separate RACH configuration will be provided. In that case, although the same technical argument can be made as for Option 1, it would simplify higher layer work if a complete additional RACH configuration is provided, and not only increments to the legacy RACH configuration. However, it is also advantageous to limit the number of FGs to allow for coherent UE behavior in the network. Furthermore, since coverage and range are the main scenarios for Option 2, it is sensible to include these coverage enhancing features in the basic Option 2 FG and not include them as separate FGs. In any case, since RACH in initial access is performed prior to the UE signaling its capabilities to the network, without support of repetitions, the range and coverage performance of UEs not supporting repetitions will be negatively impacted.

PRACH preamble repetitions and Msg3 repetitions for the single RACH configuration are determined by the corresponding legacy UE-features for PRACH preamble and Msg3 repetitions.

A separate FG is introduced for PRACH repetitions related to the additional RACH configuration.

Include support for Msg3 repetitions as a component in FG X-4.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Using the same component names or descriptions for contradictory capabilities in different FGs opens for ambiguous interpretations and is bad practice. If Configuration 2 is not specified (which there is no need for), there is no difference.

Observation 2	RAN1 agreements clearly states that Configuration 1 is used for Msg3 repetitions.

Observation 3	Option 1 and Option 2 target different UE types and therefore, per agreement, a UE is not expected to support both options.

Observation 4	Support for ROs across both SBFD and non-SBFD symbols is a network configuration but should not be a separate UE-capability to avoid capability mismatch.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 to strive to keep the number of UE-features small for SBFD.

Proposal 2	Include support for Configuration 1, including determination of valid symbol type, in the basic FG (Alt. 1).

Proposal 3	Include support for separate UL configurations for SBFD and non-SBFD symbols in the basic FG (Alt. 1).

Proposal 4	Include support for DL reception in two non-contiguous DL subbands in basic FG (Alt. 1).

Proposal 5	Include support for separate CSI reports for SBFD and non-SBFD symbols, including the determination of subbands for CSI reporting, in basic FG (Alt. 1).

Proposal 6	Include support for DL partial PRG in basic FG (Alt. 1).

Proposal 7	Include support for transmissions and receptions across SBFD and non-SBFD symbols in a single FG for both UL transmission and DL reception (Alt. 1).

Proposal 8	Include support for determination of transport block size of PDSCH/PUSCH subject to valid symbol type, FDRA in DCI and SBFD configuration, in basic FG.

Proposal 9	For FG X-1a, change Component name to ‘1. Support of multi-slot UL transmissions or DL receptions in either SBFD symbols or non-SBFD symbols in different slots’.

Proposal 10	For FG X-2, change Component name to ‘1. Support of multi-slot UL transmissions in both SBFD symbols and non-SBFD symbols in different slots’ and ‘2. Support of multi-slot DL receptions in both SBFD symbols and non-SBFD symbols in different slots’.

Proposal 11	For both FGs X-1a and X-2, change consequence description to ’Multi-slot transmission and reception in both SBFD symbols and non-SBFD symbols in different slots is not supported’.

Proposal 12	Configurations 1 and 2 does not apply to PRACH or RAR.

Proposal 13	A UE supporting the single RACH configuration needs to support at least Configuration 1 (Alt. 1).

Proposal 14	Support separate FGs for the single and additional RACH configurations.

Proposal 15	Support of a valid RO across SBFD and non-SBFD symbols is a component of the FG for SBFD random access based on the additional RACH configurations (Option 2) (Alt. 1).

Proposal 16	PRACH preamble repetitions and Msg3 repetitions for the single RACH configuration are determined by the corresponding legacy UE-features for PRACH preamble and Msg3 repetitions.

Proposal 17	A separate FG is introduced for PRACH repetitions related to the additional RACH configuration.

Proposal 18	Include support for Msg3 repetitions as a component in FG X-4.

# References

RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD),” Huawei, RAN #104, June 2024.

R1-2001513, “Guidelines for UE capability definitions,” RAN2, Ericsson, Intel, RAN1 #100, April 2020.

R1-2306381, “Further Guidelines on UE capability definitions,” RAN2, Ericsson, RAN1 #114, August 2023.

R1-2501588, “Summary#3 of discussion on Rel-19 NR UE features for SBFD,” NTT DOCOMO, RAN1 #120, February 2025.

R1-2500003, “Report of RAN1#119 meeting,” ETSI MCC, RAN1 #120, February 2025.