3GPP TSG RAN WG1 #120bis   		                                                                       R1-2502845 Wuhan, China, April 7th – 11th, 2025

Agenda item:	9.5.3

Source: 	Qualcomm Incorporated

Title: 	Adaptation of common channel transmissions

Document for:	Discussion/Decision

# Introduction

The contribution provides our views on remaining aspects to support adaptation of common signal/channel transmissions that is one of the objectives in [1] for NR network energy savings enhancements:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

# Adaptation of SSB in Time Domain

## Enhancements to DCI 2_9

RAN1#120 made the following agreements to support SSB burst periodicity adaptation for NCD-SSB in Scell:

Agreement For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

For the value of X, it is our view that one additional periodicity should be sufficient since the SSB with SSB burst periodicity configured by legacy signalling for the SCell should be transmitted with a long periodicity e.g., 160ms for NES while one shorter additional SSB burst periodicity e.g., 20ms can be provided. The DCI based switching between the SSB burst periodicity configured by legacy signaling and the additional SSB burst periodicity is expected to achieve a good tradeoff between targeted network energy saving gain and performance of SSB based operation at UE. Furtheremore, having a configuration of multiple additional SSB burst periodicity values for the SCell while DCI is to switch among these values creates high UE complexity while the benefits to both NES and UE’s performance of SSB based operation is not well quantified or justified.

Proposal 1: Support configuration of one additional SSB burst periodicity for an Scell.

For the DCI content, the current DCI 2_9 format consists of blocks for each serving cell. To support the SSB burst periodicity adaptation, a new RRC parameter (e.g., ssbPeriodicityAdaptation) can be introduced to indicate whether SSB periodicity adaptation information will be included in the DCI or not. For example,  if ssbPeriodicityAdaptation is set to enabled, k bits (e.g., k = 1) are included in the DCI for indicating the SSB burst periodicity to be applied.

For the position of the SSB periodicity adaptation indication, the block of information for SSB periodicity adaptation could be separate or together with the cell DTX/DRX indication block. For a separate block of information for SSB periodicity adaptation, a new RRC parameter e.g., positionInDCI-ssbAdaptation will need to be provided by higher layers.

Proposal 2: Introduce a new RRC parameter (e.g., ssbPeriodicityAdaptation) to indicate whether information of SSB periodicity adaptation will be included in DCI2_9 for a serving cell or not while the position in DCI is determined by one of the following options:

- Option 1: SSB adaptation indication is included at the end of the cell DTX/DRX indication block.

- Option 2: Information block of SSB periodicity adaptation is separate from cell DTX/DRX indication block and is identified by a new parameter positionInDCI-ssbAdapatation provided by higher layers.

Proposal 2.1:  If Option 1 is adopted, RAN1 adopts the following update to TS 38.212.

##### 7.3.1.3.10 Format 2_9 DCI format 2_9 is used for activating or de-activating the cell DTX and/or DRX configuration of one or multiple serving cells for one or more UEs, and/or for providing NES-mode indication of the primary cell for one or more UEs. The following information is transmitted by means of the DCI format 2_9 with CRC scrambled by cellDTRX-RNTI: - block number 1, block number 2,…, block number N where the starting position of a block associated with a serving cell is determined by the parameter positionInDCI-cellDTRX provided by higher layers for the UE. If the UE is configured to monitor DCI 2_9 with CRC scrambled by cellDTRX-RNTI, one or more blocks are configured for the UE by higher layers, with the following fields defined for each block: - Cell DTX/DRX indication - number of bits determined by the following: - If higher layer parameter cellDTX-DRX-L1activation is configured - 2 bits as defined in Clause 11.5 of [5, TS38.213] if cellDTX-DRX-ConfigType is configured to dtxdrx for the associated serving cell of the block, with the MSB corresponding to cell DTX configuration and the LSB corresponding to cell DRX configuration; - 1 bit as defined in Clause 11.5 of [5, TS38.213] if cellDTX-DRX-ConfigType is configured to either dtx or drx for the associated serving cell of the block; - 0 bit otherwise. - NES-mode indication – 1 bit indicating NES-specific CHO execution condition as defined in Clause 11.5 of [5, TS38.213], if the higher layer parameter nesEvent is configured and the associated serving cell of the block is primary cell; 0 bit otherwise. - SSB burst periodicity indication – 1 bit as defined in [Table Y] if higher layer parameter [ssbPeriodicityAdaptation] is configured; 0 bit otherwise. [Table Y]: SSB burst periodicity indication for X = 1 SSB burst periodicity indication; SSB burst periodicity. 0; The SSB burst periodicity provided by ssb-periodicityServingCell in ServingCellConfigCommon.. 1; The SSB burst periodicity provided by [ssb-periodicitySetServingCell]. The size of DCI format 2_9 is indicated by the higher layer parameter sizeDCI-2-9.

Proposal 2.2:  If Option 2 is adopted, RAN1 adopts the following update to TS 38.212.

##### 7.3.1.3.10 Format 2_9 DCI format 2_9 is used for activating or de-activating the cell DTX and/or DRX configuration of one or multiple serving cells for one or more UEs, and/or for providing NES-mode indication of the primary cell for one or more UEs. The following information is transmitted by means of the DCI format 2_9 with CRC scrambled by cellDTRX-RNTI for Cell DTX/DRX operation: - block number 1, block number 2,…, block number N where the starting position of a block associated with a serving cell is determined by the parameter positionInDCI-cellDTRX provided by higher layers for the UE. <omitted text> The following information is transmitted by means of the DCI format 2_9 with CRC scrambled by cellDTRX-RNTI for SSB burst periodicity adaptation: - block number N+1, block number N+2, …, block number N + M where the starting position of a block associated with a serving cell is determined by the parameter [positionInDCI-ssbAdaptation] provided by higher layers for the UE and the associated serving cell of the block is secondary cell. If the UE is configured to monitor DCI 2_9 with CRC scrambled by cellDTRX-RNTI, one or more blocks are configured for the UE by higher layers, with the following fields defined for each block: - SSB burst periodicity indication – 1 bit as defined in [Table Y] if higher layer parameter [ssbPeriodicityAdaptation] is configured; 0 bit otherwise. [Table Y]: SSB burst periodicity indication SSB burst periodicity indication; SSB burst periodicity. 0; The SSB burst periodicity provided by ssb-periodicityServingCell in ServingCellConfigCommon.. 1; The SSB burst periodicity provided by [ssb-periodicitySetServingCell]. The size of DCI format 2_9 is indicated by the higher layer parameter sizeDCI-2-9.

For the applicable scenario, the current TS38.321 specifies that if the Scell is deactivated, the UE doesn’t monitor PDCCH on the Scell nor for the Scell. Therefore, DCI indication of adapted SSB periodicity can only occur on activated Scell for the same cell and/or other activated scells. From our perspective, no specification change is necessary.

Observation 1: The current specifications mean that DCI adaptation of SSB periodicity is only limited to scenario 3B defined in agenda item 9.5.1. No specification change on application scenarios is necessary.

Application timeline of SSB burst periodicity adaptation using DCI 2_9

When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SS/PBCH burst periodicity of the SSB transmission on a second serving cell, the UE assumes SS/PBCH is transmitted on the second serving cell according to the indicated SS/PBCH burst periodicity from the beginning of the first slot containing the first actually transmitted SSB index within the first SSB burst that does not begin before the beginning of the slot m+d+∆ of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1, ∆ is a number of slots for the SCS of the active DL BWP of the first serving cell corresponding to [4ms]. This type of timeline is very similar to the timeline we have as working assumption for MAC-CE based indication of OD-SSB in agenda 9.5.1. The reason of having additional time on top of the value of d is that the UE would need to have more additional times e.g., 4ms for internal processing in order to take the change in SSB burst periodicity into account. If there is no consensus on the value of ∆, we propose to send an LS to RAN4 on checking it since RAN4 is discussing the similar aspects in MAC-CE based indication of on-demand SSB in agenda 9.5.1.

Proposal 3: Adopt the following update to Clause 11.5 of TS 38.213 for timeline of SSB burst periodicity adaptation using DCI 2_9.

- If no consensus on a value of ∆ is reached in RAN1, we suggest to send an LS to RAN4 on determining the value.

## 11.5 Adaptation of cell operation <omitted text> When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SS/PBCH burst periodicity of the SSB transmission on a second serving cell, the UE assumes SS/PBCH is transmitted on the second serving cell according to the indicated SS/PBCH burst periodicity from the beginning of the first slot containing the first actually transmitted SSB index within the first SSB burst that does not begin before the beginning of the slot m+d+∆ of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1, ∆ is a number of slots for the SCS of the active DL BWP of the first serving cell corresponding to [4ms]. Table 11.5-1: Minimum time gap value d SCS (kHz); Number of slots. 15; 3. 30; 6. 60; 12. 120; 24. 480; 96. 960; 192.

## Impact of SSB burst periodicity adaptation on other channels

PRACH transmission

The transmission of OD-SSB in response to SSB burst periodicity indication in DCI 2_9 may impact whether the UE can transmit PRACH in a valid PRACH occasion.  In particular, the UE is not expected to transmit PRACH in a valid PRACH occasion if the occasion precedes a SS/PBCH block in the PRACH slot and does not start at least Ngap symbols after a last SS/PBCH block reception symbol, where Ngap is provided in Table 8.1-2 of TS 38.213. Here, SS/PBCH block is the SS/PBCH block that is transmitted in response to SSB burst periodicity indication in DCI 2_9.

Proposal 4: For SSB burst periodicity adaptation by DCI 2_9, the UE is not expected to transmit PRACH in a valid PRACH occasion if the occasion precedes a SS/PBCH block in the PRACH slot and does not start at least Ngap symbols after a last SS/PBCH block reception symbol where Ngap is provided in Table 8.1-2 of TS 38.213 and the SS/PBCH block is the SS/PBCH block that is transmitted in response to SSB burst periodicity indication in DCI 2_9.

PDCCH monitoring

From the current specifications, for PDCCH monitoring in a secondary cell, when a UE monitors a PDCCH candidate, if at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon, the UE is not required to monitor the PDCCH candidate.

For the UE supporting SSB burst periodicity adaptation in a secondary cell, similarly, to monitor of a PDCCH candidate by a UE on a secondary cell, if the UE

-	has received ssb-PositionsInBurst in ServingCellConfigCommon for the secondary cell, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by the ssb-PositionsInBurst and associated with a SS/PBCH block that is transmitted according to SS/PBCH block burst periodicity indication in DCI format 2-9,

the UE is not required to monitor the PDCCH candidate. Hence, we make the following proposal:

Proposal 5: Adopt the following update to Clause 10 of TS 38.213 for PDCCH monitoring.

10 UE procedure for receiving control information <omitted text> For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SSB-MTCAdditionalPCI for a serving cell, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SSB-MTCAdditionalPCI with same physical cell identity as the one associated with a RS having same quasi-collocation properties as a CORESET for the PDCCH candidate, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE on a secondary cell, if the UE - has received ssb-PositionsInBurst in ServingCellConfigCommon for the secondary cell, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by the ssb-PositionsInBurst and associated with a SS/PBCH block that is transmitted according to SS/PBCH block burst periodicity indication in DCI format 2-9, the UE is not required to monitor the PDCCH candidate. A UE is not required to monitor PDCCH candidates for a Type0/0A/0B/1/1A /2/2A -PDCCH CSS set when the active TCI state for a corresponding CORESET is not associated with physCellId in ServingCellConfigCommon. If a UE monitors the PDCCH candidate for a Type0-PDCCH CSS set on the serving cell according to the procedure described in clause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell. <omitted text>

PDSCH resource mapping

When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, Multicast MCCH-RNTI or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 of TS 38.214 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted. Similarly, for an UE supporting SSB burst periodicity adaptation, the UE additionally assumes SS/PBCH block transmission according to the SSB burst periodicity indication in DCI 2_9 if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted. Hence, we make the following proposal.

Proposal 6: Adopt the following update to Clause 5.1.4 of TS 38.214 for PDSCH resource mapping.

### 5.1.4 PDSCH resource mapping When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block, after puncturing if applicable, is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MSGB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, Multicast MCCH-RNTI or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted. For an UE supporting SSB burst periodicity adaptation by DCI format 2-9, the UE additionally assumes SS/PBCH block transmission periodicity according to the SSB burst periodicity indicated in DCI format 2-9 if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted. <omitted text>

# Adaptation of PRACH in Time Domain

## Configuration of additional PRACH resources

The following agreement was made during the discussions of RAN1#120:

Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping - Note: the overlapped RO for legacy resource is not impacted - FFS: Clarification on configuration of legacy ROs

Current specs has multiple PRACH configurations including common configurations to provide configuration for various procedures (e.g., 4-step RACH, 2-step RACH, PRACH repetition and for system information request) and dedicated configurations to provide dedicated configuration for procedures such as CFRA or for beam failure recovery. It was agreed that paging DCI and DCI 1_0 with C-RNTI used to trigger PRACH are two methods for PRACH availability indication.

- Paging DCI should be used for indicating availability of the additional ROs associated with the common configuration while DCI 1_0 with C-RNTI used to trigger PRACH should be used for indicating availability of the additional ROs associated with the dedicated configuration if the dedicated configuration is configured.

- It is important to discuss which RACH procedures the adaptation should be applicable. RAN2 #129 made the following agreements which would impact discussion in RAN1:

Agreements - Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed. 2a. RAN2 starts CBRA for RACH adaptation. 2b. RAN2 starts 4-step RACH adaptation. 3. From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request. - From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources. 5a. From R2 perspective, RACH adaptation is not modelled as RA feature(s). 5b. From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain. - Will follow legacy mechanism regarding how to select RACH resource.

From our perspective, the technique should be applicable to 4-step RACH. In particular, PRACH adaptation is applied to common configuration defined by RACH-ConfigCommon associated with 4-step RACH for UEs in all RRC states. Furthermore, the technique should be applicable to PDCCH ordered RACH for the connected mode UEs. More specifically, if the UE is configured with dedicated RACH configuration RACH-ConfigDedicated, the UE shall perform CFRA procedure based on PRACH resources provided by the dedicated configuration and/or the available additional PRACH resources. However, if the UE is not configured with dedicated RACH configuration RACH-ConfigDedicated, the UE shall perform CBRA procedure based on PRACH resources provided by the common configuration associated with 4-step RACH and/or the available additional PRACH resources.

- In addition, it is critical to clarify the relationship between the configuration of additional ROs and the configuration of legacy ROs. In principle, additional PRACH resources are configured with respect to a baseline configuration that could be any of the legacy PRACH configurations. When the network indicates an availability indication for the additional PRACH resources, which one or which group of additional PRACH configuration should be considered to be available? For example, assuming additional PRACH configurations are configured, we see that there are different ways to interpret the configuration of additional PRACH resources as follows:

  - One to one configuration: Each legacy configuration that is subject to adaptation would have its own additional PRACH configuration.

  - One to one configuration: PRACH adaptation is applied according to the same additional PRACH configuration to all legacy configurations that are subject to the adaptation.

Have good understandings of the above issues would give clear guidance on the payload of the PRACH adaptation indication as well as the use cases of the feature. Based on above discussion, we make the following proposals:

Proposal 7: PRACH adaptation is applied to common configuration defined by RACH-ConfigCommon associated with 4-step RACH and to the dedicated configuration defined by RACH-ConfigDedicated associated with a PDCCH order.

- Further discuss whether one configuration of additional ROs is associated with one configuration of legacy ROs or one configuration of additional ROs can be associated with multiple configurations of legacy ROs.

Proposal 8: Discuss the use cases of RACH for which the additional PRACH resources can be applied and whether the adaptation indication indicates availability of additional ROs for all of them or a subset of them.

The above agreement shows that additional ROs that overlap with legacy ROs in time/frequency are not valid however, some of the valid legacy ROs are not used at all because they are not mapped to any SSBs (out of the SSB-RO mapping cycles). The additional ROs that overlap in time/frequency with the legacy ROs that are not mapped to any SSB have no impact to the legacy UE and hence, there is no reason to consider them invalid.

Proposal 9: The additional PRACH resources that overlap, even partially, in both time and frequency with the legacy ROs that are not mapped to SSBs are considered valid.

Further to the above discussion, the following agreement was made in RAN1#120

Agreement For DCI-based adaptation for additional PRACH resources, support optional semi-static signalling of a single PRACH mask to identify the subset of the additional PRACH resources - The mask is applicable at unit of - Alt 1: PRACH association period - Alt 2: PRACH association pattern period - Alt 3: SFN level - The PRACH association period is determined based on valid additional ROs only. - The mask is applied after valid RO determination and SSB-RO mapping. - Note: The existing behaviour in TS 38.213 "An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec." is not impacted due to application of the mask. - This is applicable at least for adaptation for DCI 1_0 with P-RNTI - The DCI does not indicate PRACH mask selection - FFS: how the mask is identified - Option 1: The PRACH mask is from a PRACH mask table - Pre-defined table with N=[4 or 8 or 16] rows - The semi-static signalling indicates a PRACH mask index - Option 2: The PRACH mask is based on configuration parameters e.g. bitmap at SFN-level, periodic time domain window, …

In our view, masking can be performed in one of the following ways:

- Option 1: Define a masking table that is applied to all transmitted SSBs as being provided in Table 1. The association period and association pattern period are defined based on additional ROs only. Limiting to association pattern period can avoid impact to an important spec "An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec." which we don't prefer from UE implementation perspective. Furthermore, the masking should be applied only when the number of association periods per association pattern period is more than one.

Table 1: RO mask table

| Mask index | Association periods with additional valid ROs |
| --- | --- |
| 0 | The first half of the number of association periods in an association pattern period |
| 1 | The second half of the number of association periods in an association pattern period |
| 2 | Every even association period in an association pattern period |
| 3 | Every odd association period in an association pattern period |

- Option 2: Apply a masking technique that is tied to the time domain by defining time windows such that the subset of the additional PRACH resources is defined by the additional ROs overlapping with this time window. This time window can be defined by a start, length and a periodicity parameter similar to how cell DRX is defined. An example is given in the following figure showing that the masking window define a subset that creates a cluster of additional ROs around the legacy ROs.

Figure 4: Masking additional PRACH resources in time domain

Proposal 10: Select one of the following options for semi-statically masking additional ROs:

- Option 1: Define the following masking table in which the association period and association pattern period are defined based on additional ROs only. Furthermore, the masking is only applicable when the number of association periods per association pattern period is more than one.

| Mask index | Association periods with additional valid ROs |
| --- | --- |
| 0 | The first half of the number of association periods in an association pattern period |
| 1 | The second half of the number of association periods in an association pattern period |
| 2 | Every even association period in an association pattern period |
| 3 | Every odd association period in an association pattern period |

- Option 2: Define a periodic time domain window in which valid additional PRACH resources can be used for PRACH transmission.

## Adaptation of additional PRACH resources

For the adaptation indication, it was agreed in RAN1#119 to support paging DCI and in RAN1#120 to support the PDCCH order.

Agreement (RAN1#119) Confirm the following working assumption from RAN1#118bis. Working Assumption For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used Working Assumption (RAN1#119) For DCI-based adaptation for additional PRACH resources, - Select from the following options for carrying the adaptation indication in DCI format 1_0 with P-RNTI - Option 1: Use reserved bits in the DCI format - FFS: relation (if any) to TRS availability bits / short message indicator in the DCI format - Option 2: Use Bits 5-8 within the Short Message (from upper layers) - Note: Availability should be confirmed by checking with RAN2. - Option 3: Use bits available for both Option 1 and Option 2 FFS: Payload size for adaptation for additional PRACH resources Agreement (RAN1#120) For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH. - FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s) FFS: Details on how to reuse existing bit for the 1-bit indication

### Adaptation based on paging DCI

The following working assumption was made on RAN1#119:

Working Assumption (RAN1#119) For DCI-based adaptation for additional PRACH resources, - Select from the following options for carrying the adaptation indication in DCI format 1_0 with P-RNTI - Option 1: Use reserved bits in the DCI format - FFS: relation (if any) to TRS availability bits / short message indicator in the DCI format - Option 2: Use Bits 5-8 within the Short Message (from upper layers) - Note: Availability should be confirmed by checking with RAN2. - Option 3: Use bits available for both Option 1 and Option 2 FFS: Payload size for adaptation for additional PRACH resources

For the adaptation indication in the paging DCI, the reserved bits can be used to support the adaptation of PRACH resources along with the TRS availability indication. In other words, the reserved bits can be split between the TRS availability indication and the PRACH adaptation indication.

Proposal 11: For adaptation indication using the paging DCI, use the reserved bits in the paging DCI (option 1).

- Note: the total number of reserved bits would be split between TRS availability indication and PRACH adaptation indication if both features are supported.

Further to the above, the following agreements related to the timeline of the adaptation indication were made in RAN1#120:

Agreement For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration - FFS: Validity time duration for availability is configured by higher layer signaling or predefined - FFS: Location of the reference point defined in the specification - FFS: Value/granularity of the validity time duration. - FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources Agreement Study the following options for the reference point (for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI in a PF) for RRC idle/inactive mode UE and RRC connected mode UE, - Option 1: SFN of the first PF from the next I-DRX cycle - Option 2: SFN of the first PF from the current I-DRX cycle - Option 3: From the first frame of the first PRACH association period after UE receives the DCI - Option 4: From the first frame of the current SI modification period - Option 5: From the first frame of the next SI modification period

Idle/inactive mode UEs monitor paging every paging cycle specified in Clause 7.1 in TS 38.304. On the other hand, the connected mode UE may monitor paging every system information modification period if it is not capable of ETWS and/or CMAS while the connected UEs capable of ETWS and/or CMAS monitor for indication about PWS notification in every paging cycle according to the configured defaultPagingCycle as being specified in TS 38.331. From our perspective, the reference point should be the SFN of the first PF from the next DRX cycle specified in Clause 7.1 in TS 38.304 for RRC idle/inactive UEs and the SFN of the first PF from the next DRX cycle defined by defaultPagingCycle for RRC connected UEs.

Proposal 12: The reference point for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is,

- The SFN of the first PF from the next DRX cycle specified in Clause 7.1 in TS 38.304 for RRC idle/inactive UEs.

- The SFN of the first PF from the next DRX cycle defined by defaultPagingCycle for RRC connected UEs.

### PRACH adaptation for PDCCH ordered RACH

The following agreement was made in RAN1#120:

Agreement For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH. - FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s) - FFS: Details on how to reuse existing bit for the 1-bit indication

DCI 1_0 with C-RNTI used to trigger PRACH is identified by an FDRA field that is all ones. The preamble index field is either zero (for CBRA) or non-zero for (CFRA). For CFRA, the SSB index and the PRACH mask index fields indicate the corresponding values to be used for during the PRACH process. For CBRA, these fields are reserved. Therefore, for CFRA, only reserved bits can be used to indicate PRACH adaptation while for CBRA, the available bits are the reserved bits which also include the 4 bits of the PRACH mask index and the 6 bits used for the SSB index indication during CFRA.

Proposal 13: When using DCI1_0 with C-RNTI used to trigger PRACH to indicate the availability of additional PRACH resources, the availability indication is carried in the following fields based on whether the PDCCH order triggers CFRA or CBRA:

- For CFRA: use reserved bits in the DCI.

- For CBRA: reuse PRACH mask index field or SSB index field which are reserved under CBRA.

For the PRACH mask index indicated in DCI1_0 for CFRA, given the limited number of reserved bits in DCI1_0, same index is reasonable. To provide flexibility, a new RRC parameter can be introduced to control whether the indicated index is applicable for legacy ROs only or additional ROs only or for both.

Proposal 14: Do not support indication of a separate PRACH mask index in DCI 1_0 triggering CFRA for the additional PRACH resources.

In case the UE is not configured with dedicated configurations or additional PRACH resources associated only to the dedicated configuration, the network may still send a PDCCH order for which the UE would use the common configuration. If additional PRACH resources are configured relative to the common configuration is not indicated as available by the paging DCI, then the PDCCH order availability indication may also indicate the availability indication of the additional PRACH resources with respect to the common configuration.

Proposal 15: The availability indication bit in the PDCCH order may indicate the availability of additional PRACH resources defined with respect to the common configuration if the UE is not configured with dedicated configuration or additional PRACH resources with respect to the dedicated configuration.

### PRACH adaptation based on DCI format 2-7

For a UE that does not support DCI format 2_7, it is required to monitor paging DCI. On the other hand, for a UE supporting DCI format 2_7, it may skip monitoring paging DCI. Therefore, it is reasonable to use both paging DCI and DCI format 2_7 for the adaptation otherwise the UE supporting DCI format 2_7 would have to monitor both PEI and paging DCI to monitor the adaptation indication. Second, it is our view that PDCCH order DCI can also be used to indicate the adaptation of additional PRACH resources for the Rel-19 UEs in RRC connected mode. Such indication can be useful to dedicate activation of additional PRACH resources for a specific UE excluding the idle/inactive mode UEs.

Proposal 16: In addition to paging DCI, consider supporting adaptation based on DCI format 2_7 for UE supporting PEI.

## Considerations on the impact to legacy UE

One of the main objectives of this work item is to minimize the impact on legacy UE. While Rel-19 UE can be indicated with available additional PRACH resources, legacy UEs can only use the legacy PRACH resources.

As the number of UEs transmitting in the same RO increases, the interference among UEs increases even if they use different preambles and the probability of collision also increases. In order to minimize the impact to the legacy UE, when additional PRACH resources are available for transmission, the Rel-19 UE should prioritize the transmission over the additional PRACH resources.

Proposal 17: Rel-19 UE should prioritize the transmission over the additional PRACH resources when they are indicated as available for transmission.

Moreover, if the Rel-19 UE uses the legacy PRACH resources, it may reduce its interference by transmitting with a different Tx power.

Proposal 18: Discuss Rel-19 UE PRACH power control when additional PRACH resources are available for transmission.

## RA-RNTI

When legacy ROs and additional ROs overlap in time, the frequency index of each of the legacy and additional ROs as used in the RA-RNTI calculation starts from 0 in an increasing order in the frequency domain. These ROs share the same RA-RNTI since they share the same frequency index . Therefore, when the UE transmits over, for example the legacy RO, it cannot distinguish whether the network response is in response for its transmission or another transmission over the additional RO. This ambiguity can be resolved by either adding a flag to the RA-RNTI equation or by assigning different frequency index for the additional ROs, for example the frequency domain index assignment of the additional ROs ranges from N1 to N1+N2-1 where N1 is the number of FDMed legacy ROs and N2 is the number of FDMed additional ROs in the same time instance. To avoid RA-RNTI ambiguity when valid legacy and additional ROs overlap in time domain, consider one of the following options:

- Option 1: Enhance RA-RNTI computation formula for the additional ROs.

- Option 2: No change to RA-RNTI computation formula. Interpret the frequency domain index for ROs including both legacy ROs and additional ROs when they are overlapped in time.

Proposal 19: To avoid RA-RNTI ambiguity when valid legacy and additional ROs overlap in time domain, consider one of the following options:

- Option 1: Enhance RA-RNTI computation formula for the additional ROs.

- Option 2: No change to RA-RNTI computation formula. Interpret the frequency domain index for ROs including both legacy ROs and additional ROs when they are overlapped in time.

# Adaptation of Paging

RAN2 made the following agreements related to adaptation of paging

Agreements - Select option-b as baseline for R19 NES paging enhancement. - R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values) - Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration. - Legacy UE is not barred. - Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration. Agreements - Introduce value for Ns=8. FFS on 16. - Introduce value for N= T/32. FFS on T/64, T/128, T/256. Agreement - For N, values smaller than T/32 are not supported. - The maximum possible value for Ns is 8. - Introduce a separate PEI configuration. - Paging adaptations are configured semi-statically and updated via system information update notification. - A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

With additional POs for Rel-19 UEs, the main question that may impact RAN1 spec is on how to indicate the PEI for the additional POs. There are two main options that were briefly discussed in RAN1#120:

- Option 1: PEI bits for legacy POs and additional Rel-19 POs are always multiplexed in the same DCI 2_7.

- Option 2: PEI bits for legacy POs and additional Rel-19 POs are not multiplexed in the same DCI 2_7.

Our understanding of these options is that a single DCI 2_7 can be used to indicate PEI for both legacy POs and additional POs in Option 1. On the other hand, for Option 2, a separate DCI 2_7 should be used to indicate PEI for additional POs. The main issues with Option 2 are as follows:

- Higher NW power consumption in supporting PEI feature: The gNB would need to send two different PDCCHs. Furthermore, the sub-group UE belongs to may be associated with a legacy PO.

- High RAN1 spec impact since we will need to have mechanisms for Rel-19 UEs to identify whether the DCI is for legacy POs or for additional ROs.

With a proper design of the bitmap for PEI bits associated with legacy POs and additional POs, a single DCI can be used to indicate the bitmap without impact to the legacy UEs. More specifically, in current specs, the PEI indication contains a bitmap of size equal to po-NumPerPEI x subgroupsNumPerPO where po-NumPerPEI is the number of paging occasions per PEI and subgroupsNumPerPO is the number of subgroups per PO. The PEI may also carry indication for TRS availability indication.

It is in our view that the PEI can carry indication for both the legacy paging occasions and additional paging occasions. However, the legacy UE should be able to perfectly interpret the PEI for the legacy paging occasions. For this to occur, the bitmap of the PEI has to stay the same and any additional indications (for the additional paging occasions) have to come after the bitmap readable by the legacy UE even if the legacy POs and additional POs are interlaced in time domain. It should be noted that Rel-19 UEs may belong to legacy PO or additional PO. The following figure illustrates how the bitmap carried in the PEI can indicate early indication for both legacy POs and additional POs

Observation 2: The same DCI 2_7 can carry early paging indication for both legacy and additional POs by concatenating a bitmap for the additional POs to the bitmap indicated for the legacy UE including early indication of legacy POs and any TRS availability indication.

Proposal 20: Adopt the following update to TS 38.212:

##### 7.3.1.3.8 Format 2_7 DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field - NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter po-NumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO. - Each bit in the field indicates one UE subgroup of a paging occasion. - The bits associated with po-NumPerPEI are mapped first and then followed up by the bits associated with [po-NumPerPEI-r19].

# Conclusion

The contribution has discussed our views on adaptation of signal/channel transmissions. In particular, we make the following observations and proposals:

SSB adaptation in time domain

Observation 1: The current specifications mean that DCI adaptation of SSB periodicity is only limited to scenario 3B defined in agenda item 9.5.1. No specification change on application scenarios is necessary.

Observation 2: The same DCI 2_7 can carry early paging indication for both legacy and additional POs by concatenating a bitmap for the additional POs to the bitmap indicated for the legacy UE including early indication of legacy POs and any TRS availability indication.

Proposal 1: Support configuration of one additional SSB burst periodicity for an Scell.

Proposal 2: Introduce a new RRC parameter [ssbPeriodicityAdaptation] to indicate whether information of SSB periodicity adaptation will be included in DCI2_9 for a serving cell or not while the position in DCI is determined by one of the following options:

- Option 1: SSB adaptation indication is included at the end of the cell DTX/DRX indication block.

- Option 2: Information block of SSB periodicity adaptation is separate from cell DTX/DRX indication block and is identified by a new parameter [positionInDCI-ssbAdapatation] provided by higher layers.

Proposal 2.1:  If Option 1 is adopted, RAN1 adopts the following update to TS 38.212.

##### 7.3.1.3.10 Format 2_9 DCI format 2_9 is used for activating or de-activating the cell DTX and/or DRX configuration of one or multiple serving cells for one or more UEs, and/or for providing NES-mode indication of the primary cell for one or more UEs. The following information is transmitted by means of the DCI format 2_9 with CRC scrambled by cellDTRX-RNTI: - block number 1, block number 2,…, block number N where the starting position of a block associated with a serving cell is determined by the parameter positionInDCI-cellDTRX provided by higher layers for the UE. If the UE is configured to monitor DCI 2_9 with CRC scrambled by cellDTRX-RNTI, one or more blocks are configured for the UE by higher layers, with the following fields defined for each block: - Cell DTX/DRX indication - number of bits determined by the following: - If higher layer parameter cellDTX-DRX-L1activation is configured - 2 bits as defined in Clause 11.5 of [5, TS38.213] if cellDTX-DRX-ConfigType is configured to dtxdrx for the associated serving cell of the block, with the MSB corresponding to cell DTX configuration and the LSB corresponding to cell DRX configuration; - 1 bit as defined in Clause 11.5 of [5, TS38.213] if cellDTX-DRX-ConfigType is configured to either dtx or drx for the associated serving cell of the block; - 0 bit otherwise. - NES-mode indication – 1 bit indicating NES-specific CHO execution condition as defined in Clause 11.5 of [5, TS38.213], if the higher layer parameter nesEvent is configured and the associated serving cell of the block is primary cell; 0 bit otherwise. - SSB burst periodicity indication – 1 bit as defined in [Table Y] if higher layer parameter [ssbPeriodicityAdaptation] is configured; 0 bit otherwise. [Table Y]: SSB burst periodicity indication for X = 1 SSB burst periodicity indication; SSB burst periodicity. 0; The SSB burst periodicity provided by ssb-periodicityServingCell in ServingCellConfigCommon.. 1; The SSB burst periodicity provided by [ssb-periodicitySetServingCell]. The size of DCI format 2_9 is indicated by the higher layer parameter sizeDCI-2-9.

Proposal 2.2:  If Option 2 is adopted, RAN1 adopts the following update to TS 38.212.

##### 7.3.1.3.10 Format 2_9 DCI format 2_9 is used for activating or de-activating the cell DTX and/or DRX configuration of one or multiple serving cells for one or more UEs, and/or for providing NES-mode indication of the primary cell for one or more UEs. The following information is transmitted by means of the DCI format 2_9 with CRC scrambled by cellDTRX-RNTI for Cell DTX/DRX operation: - block number 1, block number 2,…, block number N where the starting position of a block associated with a serving cell is determined by the parameter positionInDCI-cellDTRX provided by higher layers for the UE. <omitted text> The following information is transmitted by means of the DCI format 2_9 with CRC scrambled by cellDTRX-RNTI for SSB burst periodicity adaptation: - block number N+1, block number N+2, …, block number N + M where the starting position of a block associated with a serving cell is determined by the parameter [positionInDCI-ssbAdaptation] provided by higher layers for the UE and the associated serving cell of the block is secondary cell. If the UE is configured to monitor DCI 2_9 with CRC scrambled by cellDTRX-RNTI, one or more blocks are configured for the UE by higher layers, with the following fields defined for each block: - SSB burst periodicity indication – 1 bit as defined in [Table Y] if higher layer parameter [ssbPeriodicityAdaptation] is configured; 0 bit otherwise. [Table Y]: SSB burst periodicity indication SSB burst periodicity indication; SSB burst periodicity. 0; The SSB burst periodicity provided by ssb-periodicityServingCell in ServingCellConfigCommon.. 1; The SSB burst periodicity provided by [ssb-periodicitySetServingCell]. The size of DCI format 2_9 is indicated by the higher layer parameter sizeDCI-2-9.

Proposal 3: Adopt the following update to Clause 11.5 of TS 38.213 for timeline of SSB burst periodicity adaptation using DCI 2_9.

- If no consensus on a value of ∆ is reached in RAN1, we suggest to send an LS to RAN4 on determining the value.

## 11.5 Adaptation of cell operation <omitted text> When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SS/PBCH burst periodicity of the SSB transmission on a second serving cell, the UE assumes SS/PBCH is transmitted on the second serving cell according to the indicated SS/PBCH burst periodicity from the beginning of the first slot containing the first actually transmitted SSB index within the first SSB burst that does not begin before the beginning of the slot m+d+∆ of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1, ∆ is a number of slots for the SCS of the active DL BWP of the first serving cell corresponding to [4ms]. Table 11.5-1: Minimum time gap value d SCS (kHz); Number of slots. 15; 3. 30; 6. 60; 12. 120; 24. 480; 96. 960; 192.

Proposal 4: For SSB burst periodicity adaptation by DCI 2_9, the UE is not expected to transmit PRACH in a valid PRACH occasion if the occasion precedes a SS/PBCH block in the PRACH slot and does not start at least Ngap symbols after a last SS/PBCH block reception symbol where Ngap is provided in Table 8.1-2 of TS 38.213 and the SS/PBCH block is the SS/PBCH block that is transmitted in response to SSB burst periodicity indication in DCI 2_9.

Proposal 5: Adopt the following update to Clause 10 of TS 38.213 for PDCCH monitoring.

10 UE procedure for receiving control information <omitted text> For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SSB-MTCAdditionalPCI for a serving cell, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SSB-MTCAdditionalPCI with same physical cell identity as the one associated with a RS having same quasi-collocation properties as a CORESET for the PDCCH candidate, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE on a secondary cell, if the UE - has received ssb-PositionsInBurst in ServingCellConfigCommon for the secondary cell, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by the ssb-PositionsInBurst and associated with a SS/PBCH block that is transmitted according to SS/PBCH block burst periodicity indication in DCI format 2-9, the UE is not required to monitor the PDCCH candidate. A UE is not required to monitor PDCCH candidates for a Type0/0A/0B/1/1A /2/2A -PDCCH CSS set when the active TCI state for a corresponding CORESET is not associated with physCellId in ServingCellConfigCommon. If a UE monitors the PDCCH candidate for a Type0-PDCCH CSS set on the serving cell according to the procedure described in clause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell. <omitted text>

Proposal 6: Adopt the following update to Clause 5.1.4 of TS 38.214 for PDSCH resource mapping.

### 5.1.4 PDSCH resource mapping When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block, after puncturing if applicable, is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MSGB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, Multicast MCCH-RNTI or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted. For an UE supporting SSB burst periodicity adaptation by DCI format 2-9, the UE additionally assumes SS/PBCH block transmission periodicity according to the SSB burst periodicity indicated in DCI format 2-9 if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted. <omitted text>

PRACH adaptation in time domain

Proposal 7: PRACH adaptation is applied to common configuration defined by RACH-ConfigCommon associated with 4-step RACH and to the dedicated configuration defined by RACH-ConfigDedicated associated with a PDCCH order.

- Further discuss whether one configuration of additional ROs is associated with one configuration of legacy ROs or one configuration of additional ROs can be associated with multiple configurations of legacy ROs.

Proposal 8: Discuss the use cases of RACH for which the additional PRACH resources can be applied and whether the adaptation indication indicates availability of additional ROs for all of them or a subset of them.

Proposal 9: The additional PRACH resources that overlap, even partially, in both time and frequency with the legacy ROs that are not mapped to SSBs are considered valid.

Proposal 10: Select one of the following options for semi-statically masking additional ROs:

- Option 1: Define the following masking table in which the association period and association pattern period are defined based on additional ROs only. Furthermore, the masking is only applicable when the number of association periods per association pattern period is more than one.

| Mask index | Association periods with additional valid ROs |
| --- | --- |
| 0 | The first half of the number of association periods in an association pattern period |
| 1 | The second half of the number of association periods in an association pattern period |
| 2 | Every even association period in an association pattern period |
| 3 | Every odd association period in an association pattern period |

- Option 2: Define a periodic time domain window in which valid additional PRACH resources can be used for PRACH transmission.

Proposal 11: For adaptation indication using the paging DCI, use the reserved bits in the paging DCI (option 1).

- Note: the total number of reserved bits would be split between TRS availability indication and PRACH adaptation indication if both features are supported.

Proposal 12: The reference point for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is,

- The SFN of the first PF from the next DRX cycle specified in Clause 7.1 in TS 38.304 for RRC idle/inactive UEs.

- The SFN of the first PF from the next DRX cycle defined by defaultPagingCycle for RRC connected UEs.

Proposal 13: When using DCI1_0 with C-RNTI used to trigger PRACH to indicate the availability of additional PRACH resources, the availability indication is carried in the following fields based on whether the PDCCH order triggers CFRA or CBRA:

- For CFRA: use reserved bits in the DCI.

- For CBRA: reuse PRACH mask index field or SSB index field which are reserved under CBRA.

Proposal 14: Do not support indication of a separate PRACH mask index in DCI 1_0 triggering CFRA for the additional PRACH resources.

Proposal 15: The availability indication bit in the PDCCH order may indicate the availability of additional PRACH resources defined with respect to the common configuration if the UE is not configured with dedicated configuration or additional PRACH resources with respect to the dedicated configuration.

Proposal 16: In addition to paging DCI, consider supporting adaptation based on DCI format 2_7 for UE supporting PEI.

Proposal 17: Rel-19 UE should prioritize the transmission over the additional PRACH resources when they are indicated as available for transmission.

Proposal 18: Discuss Rel-19 UE PRACH power control when additional PRACH resources are available for transmission.

Proposal 19: To avoid RA-RNTI ambiguity when valid legacy and additional ROs overlap in time domain, consider one of the following options:

- Option 1: Enhance RA-RNTI computation formula for the additional ROs.

- Option 2: No change to RA-RNTI computation formula. Interpret the frequency domain index for ROs including both legacy ROs and additional ROs when they are overlapped in time.

Paging adaptation in time domain

Proposal 20: Adopt the following update to TS 38.212:

##### 7.3.1.3.8 Format 2_7 DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field - NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter po-NumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO. - Each bit in the field indicates one UE subgroup of a paging occasion. - The bits associated with po-NumPerPEI are mapped first and then followed up by the bits associated with [po-NumPerPEI-r19].

# Reference

RP-242354, Enhancements of network energy savings for NR