3GPP TSG-RAN WG1#120bis	R1-2503124

Wuhan, China, April 7-11 2025

Agenda Item:	9.5.3

Source:	Moderator (Ericsson)

Title:	Summary#4 of AI 9.5.3 for R19 NES

Document for:	Discussion

# Introduction

This is the summary for AI 9.5.3 on the adaptation of common signalling for NES based on the views expressed by companies in the contributions listed in the Appendix A and providing topics and proposals for discussion/agreement.

# Adaptation of SSB in time domain

Several companies provided their views on the design for DCI 2_9 for SSB burst periodicity adaptation as well as other aspects.

RNTI

Several companies provided their views on the RNTI used for DCI 2_9 based signalling for SSB burst periodicity switching.

- New RNTI

  - Huawei, Apple, Mediatek, Xiaomi, ZTE, Panasonic, Ericsson, DoCoMo, ETRI, Xiaomi, Google, Cewit

- Cell DTRX-RNTI

  - Nokia, Samsung, Qualcomm, Interdigital, vivo, LG, CMCC, OPPO, Fujitsu, NEC, Cewit

Some of the Pros and cons listed below.

- New RNTI

  - Pros: independent of cell DTX/DRX functionality/UE capabilities, unnecessary processing of a DCI for a UE, who only supports cell DTX/DRX …

  - Cons: new search space for different RNTI within the same DCI format, …

- Cell DTRX-RNTI

  - Pros: can allow independent operation (separate starting position within DCI for SSB periodicity switching), simplify the specification impact,…

  - Cons: larger DCI size, unnecessary linkage with cell DTX/DRX functionality/UE capabilities, …

## Proposal 2.1.1 (RNTI)

For adaptation of SSB in time-domain, for DCI 2_9-based SSB burst periodicity adaptation for an SCell,

- The DCI is scrambled with following RNTI, select from

  - Alt 1: cellDTRX-RNTI

  - Alt 2: New RNTI

| Company | Comment |
| --- | --- |
| CMCC | We support Alt 1. gNB can configure different starting position for different information bit so there will be no impact between different feature. |
| ZTE, Sanechips | We are open to discuss the RNTI is legacy RNTI or new RNTI. But for the perspective of payload, we think new RNTI is more preferable. |
| OPPO | OK， but we wonder for Alt-2, why do we need to limit to DCI 2-9 if new RNTI is introduced? |
| CATT | We prefer to support Alt 1. Share same view with OPPO that the motivation to support Alt 2 is unclear. The introduction of new RNTI is similar to the introduction a new DCI format. |
| Xiaomi | Support Alt. 1. |
| Panasonic | In our opinion, using a new configurable RNTI with DCI format 2_9 for SSB adaptation is the cleanest solution. When adaptation of SSB and Cell DTX/DRX are to be indicated in the same timing , then the configured value is same as the existing cellDTRX-RNTI. It means the DCI is shared with cellDTRX-RNTI but the different blocks can be used. When adaptation of SSB and Cell DTX/DRX are to be indicated in different timings, then the configured value can be differentiated to the existing cellDTRX-RNTI. Then the block/field design should be Scell/Scell group based for SSB adaptation. |
| NEC | Support Alt-1 |
| Apple | We support Alt 2. If Alt 1 is supported, a UE supporting both DCI-based activation/deactivation of cell DTX/DRX and SSB adaptation will be configured with two positions in DCI format 2_9 and UE needs to check for the two positions whenever it receives the DCI. From NW perspective, at each time NW sends a DCI to switch the SSB periodicity, it needs to send the bit block corresponding to each function which causes larger signaling overhead. Considering the different use cases and scenarios for SSB adaptation and cell DTX, our preference is to use a separate RNTI for SSB adaptation. |
| Nokia, NSB | Alt 1. or cellDTRX-RNTI can be used if location of SSB adaptation bits in the DCI is configured independently from the location of cell DTRX bits. |
| Fujitsu | Our preference is Alt 1. From the perspective of signalling overhead, Alt 1 has lower overhead than Alt 2 since the gNB only need to send a single DCI to indicate both Rel.18 cell DTX/DRX and SSB adaptation. |
| China Telecom | Support Alt 1. Same view as OPPO. |
| LG | We support Alt 2. As ZTE mentioned, the payload size of DCI should be considered to select RNTI. Since the bit-width for SSB periodicity adaptation can be determined depending on the value of X (i.e., number of additional SSB burst periodicities) in Proposal 2.1.3, RNTI can be selected after the value of X is agreed. |
| DCM | We support Alt. 2. we support using new-RNTI to reduce specification complexity by not associating different functions into the same RNTI (i.e., cellDTRX-RNTI). We also support that new-RNTI can be configurable same as the all other RNTIs of group-common DCIs (e.g., cellDTRX-RNTI, tpc-SRS/PUCCH/PUSCH-RNTI) |
| Moderator | Updated proposal from the offline discussion in 2.1.1-rev1 below. |

## Proposal 2.1.1-rev1 (RNTI)

For adaptation of SSB in time-domain, for DCI 2_9-based SSB burst periodicity adaptation for an SCell,

- The DCI is scrambled with following RNTI,

  - New RNTI

    - Same search space as the search space for cell DTX/DRX (if configured)

  - [Detailed design for PDCCH monitoring follows cell DTX/DRX]

| Company | Comment |
| --- | --- |
| Moderator | Closed. |

DCI field

Several companies provided further details on the DCI field and location for DCI 2_9 based signalling for SSB burst periodicity switching.

- Location in the DCI configurable (e.g. independent of cell DTX/DRX block for the Scell (if configured))

  - Huawei, Samsung, Ericsson, Qualcomm, Fujitsu, Nokia?, Cewit?

- Location in the DCI at end of cell DTX/DRX block for the Scell

  - Qualcomm, Fujitsu, Oppo, Fraunhofer

## Proposal 2.1.2 (DCI field)

For DCI 2_9-based SSB burst periodicity adaptation for an SCell

  - the starting location of the information block for SSB burst periodicity indication for a SCell within the DCI format 2_9 is configured using a new RRC parameter

  - the length of the information block is given by ceil(log2(1+X)), where UE is configured with X additional SSB burst periodicities for the SCell

| Company | Comment |
| --- | --- |
| CMCC | Okay |
| ZTE, Sanechips | Fine with FL’s proposal. |
| OPPO | The first bullet is related to 2.1.1, thus can be decided later. |
| CATT | We think the starting location of the block for SSB adaptation indication can reuse the current RRC parameter, i.e., positionInDCI-cellDTRX. This can avoid introducing a new RRC parameter and achieve the same function as the new parameter of the proposal. |
| xiaomi | Support. |
| NEC | Support |
| Apple | Support. |
| Nokia, NSB | Support |
| Fujitsu | We are fine with the proposal. |
| China Telecom | Support. |
| LG | Support |
| DCM | We support that the starting location of the IB for SSB burst periodicity indication for SCell within the DCI format 2_9 is configured using a new RRC parameter. First, we support on allowing SSB adaptation using new-RNTI. In such situation, it is definitely required where in DCI should UE need to look at. Introducing new starting point is necessary. |
| CEWiT | Support |
| Moderator | Closed. |

Number of additional SSB burst periodicities (X)

Several companies provided views on the maximum number of additional SSB burst periodicities for an SCell (Xmax). While there are up to six total SSB burst periodicities possible (5ms/10ms/20ms/40ms/80ms/160ms), different companies suggest different maximum number based on flexibility, network energy savings potential, DCI payload size impacts, etc.

- Xmax=1

  - Qualcomm, CATT, Lenovo, Oppo, FW (sufficient for NES), ETRI, DoCoMo, Interdigital

- Xmax=2

  - Samsung, NEC

- Xmax=3

  - Huawei, Ericsson, CT

- Xmax=4

  - Honor

- Xmax=5

  - Nokia, Mediatek (to cover up to six SSB periodicities that are in spec today), vivo, ZTE, KDDI

## Proposal 2.1.3 (Value of X)

For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell.

- Xmax =3

| Company | Comment |
| --- | --- |
| CMCC | We think 1 additional period is enough but okay with multiple values. |
| ZTE, Sanechips | OK with Xmax = 3. |
| OPPO | we don’t support this proposal. The motivation of introducing more than 2 periodicity is not clear. A short period + a long period would be enough to obtain NES gain. Thus, we support Xmax = 1. |
| CATT | Since SSB burst periodicity adaptation is a cell-level operation, using two SSB burst periodicities for switching is sufficient for the network. Moreover, considering the spec impact and DCI overhead, option 1 is the simplest method with minimal overhead to achieve SSB burst periodicity adaptation. |
| Xiaomi | We are fine with the proposal. |
| NEC | Support |
| Apple | Prefer Xmax=1. |
| Nokia, NSB | Ok. We think that one additional value is not enough considering that sometimes there can be legacy UEs in the cell and sometimes only R19 UEs. When there are legacy UEs, adaptation is only possible to shorter periodicities. |
| Fujitsu | We are fine with the proposal. |
| China Telecom | Xmax =3 can provide more flexibility for SSB adaptation compared with on-demand SSB with case #2. But we can comprise if majority view is Xmax =1. |
| Qualcomm | Xmax = 1. It can be flexible for NW to have Xmax >1. However, from NES perspective, Xmax = 1 should be sufficient. In addition, having switching among X > 1 and the legacy periodicity value potentially introduce more switching points, which complicates UE implementation while not providing clear performance benefits. |
| LG | Prefer Xmax=3 |
| DCM | We are fine with the proposal. |
| Moderator | Closed. |

Application time

Several companies provided views on the delay for applying the SSB burst periodicity switching indication in the DCI. It seems some application delay needs to defined, and companies have different views on those, ranging from 3ms (similar as existing DCI 2_9 application time) to ~7ms (similar delay value as for applying OD-SSB indication using MAC CE).

- Minimum of SSB periodicities before and after adaptation

  - Samsung (with a time-gap), CATT (with a time-gap)

- Same delay value as for applying OD-SSB indication using MAC-CE

  - Qualcomm (3ms+4ms), LG, ZTE (the same with OD-SSB using MAC-CE)

- With a time-gap

  - Apple, Samsung, CATT

- Reuse DCI 2_9 time-gap/timeline

  - Ericsson, Huawei (as baseline)

- First candidate SSB burst after a time-gap

  - vivo

- Maintain the same offset for uniform SSB in time domain. Does not affect SMTC

  - Mediatek

- Discuss separate values for adaptation with and without cell DTX/DRX

  - Nokia

## Proposal 2.1.4 (Application time)

For an indicated SSB burst periodicity switch for an SCell that is received in a DCI 2_9 on a first serving cell,

- The SSB transmission on the SCell according to the indicated SSB burst periodicity starts from the first candidate SSB burst that is Y slots (corresponding to [3ms - 7ms]) after the reception of DCI format 2-9,

  - Y is a number of slots for the SCS of the active DL BWP of the first serving cell.

| Company | Comment |
| --- | --- |
| CMCC | In R18, we introduced a definition on application delay for DCI format 2_9. Such design could be easily reused in SSB adaptation. |
| ZTE, Sanechips | In our opinion, we think application delay and time instance should be also considered for DCI format 2_9 as the discussion in 9.5.1. And our position is on reusing current DCI 2_9 timeline. |
| OPPO | the timeline should be aligned with existing DCI 2-9. What is the reason to align the timeline with OD-SSB with MAC-CE. It is quite puzzling. |
| CATT | We can reuse the applicable time of DCI format 2_9 in Rel-18. From the perspective of applicable timeline, we don’t see any difference between DCI format 2_9 in Rel-18 and in Rel-19. |
| Xiaomi | Not sure where 7 ms comes from. More clarification is needed. |
| Apple | We are supportive of a value larger than 3ms, and we think the exact value when UE is ready to process 3ms is the value that is considered for DCI format 2_9. ‘When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in activation or deactivation of a current cell DTX operation or cell DRX operation for a second serving cell, the UE operates on the second serving cell according to the indicated cell DTX operation or cell DRX operation starting from a slot on the active DL BWP or on the active UL BWP of the second serving cell, respectively, that is not before the beginning of the slot m+d on the active DL BWP of the first serving cell where is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1.’ Table 11.5-1: Minimum time gap value d SCS (kHz); Number of slots. 15; 3. 30; 6. 60; 12. 120; 24. 480; 96. 960; 192. However, we think a similar discussion as OD-SSB, is that whether UE needs additional time to prepare for monitoring the SSB after adaptation. Therefore, a similar procedure can be considered: this 3ms gap is from NW transmission perspective and then regarding when UE assumes to be ready to process SSB after adaptation, send an LS to ask RAN4 whether additional UE processing/preparation time for reception of SSB after adaptation should be considered for RAN4 requirement. |
| Nokia, NSB | It should be discussed if the current timeline of DCI 2_9 can be reused or not. |
| Fujitsu | We think that reusing the existing timeline of DCI format 2_9 is enough. |
| LG | It seems our view is not correctly captured. We support reusing the time-gap/timeline of DCI format 2_9. Regarding the first candidate SSB burst that is Y slots after the reception of DCI format 2-9, UE can expect that SSB with changed periodicity is transmitted from time instance A which is determined as in 9.5.1. |
| DCM | We are fine with proposal. |
| CEWIT | Fine with the proposal |
| Moderator | Below is the status from Wednesday online discussion. Proposal 2.1.4 (Application time) For an indicated SSB burst periodicity switch for an SCell that is received in a DCI 2_9 on a first serving cell, - The SSB transmission on the SCell according to the indicated SSB burst periodicity starts from the first candidate SSB burst that is Y slots (corresponding to [3ms - 7ms]) after the reception of DCI format 2-9, - Y is a number of slots for the SCS of the active DL BWP of the first serving cell. Proposal 2.1.4 (Application time) For an indicated SSB burst periodicity switch for an SCell that is received in a DCI 2_9 on a first serving cell, - The SSB transmission on the SCell according to the indicated SSB burst periodicity starts from the first candidate SSB burst that is time duration Y after the reception of DCI format 2-9, - Send an LS to RAN4 to decide on time duration Y - FFS: How to determine the first candidate SSB burst |
| Qualcomm | We suggest more details should be provided to make the timeline of SSB transmission clear. In particular, we suggest the following proposal in which the 1st bullet is identical to cell DTX/DRX timeline in current spec for SSB transmission. Suggested proposal - When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first actually transmitted SSB index within the first SSB burst that does not begin before the beginning of the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1 of TS 38.213. - Note: Above does not prevent RAN4 from discussing SSB-related processing timeline. |
| Tejas | Support |
| Moderator | Companies can check the suggested proposal from Qualcomm. |
| Moderator | Below is proposal based on Qualcomm. |

## Proposal 2.1.4-rev1 (Application time)

When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first actually transmitted SSB index within the first SSB burst that does not begin before the beginning of the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1 of TS 38.213.

[Note: Above does not prevent RAN4 from discussing SSB-related processing timeline.]

| Company | Comment |
| --- | --- |
| Moderator | Please indicate your view on the above proposal. |
| Huawei, HiSilicon | Suggest update on the wording: …does not begin before the beginning of … is no earlier than … |
| Apple | We think the note should be kept. Also similar to OD-SSB, we prefer to consider it as working assumption. Also considering the our comment in Discussion point 2.2.1, we prefer to have further discussion on how to determine the first SSB burst after the time gap, whether it is the actual first SSB after the time gap, or the first SSB according to a determined pattern. Proposed updates: - Working assumption: When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first actually transmitted SSB index within the [first SSB burst] that does not begin before the beginning of the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1 of TS 38.213. - FFS: how to determine the first SSB burst - [Note: Above does not prevent RAN4 from discussing SSB-related processing timeline. ] |
| Samsung | The cell is not deactivated, the timeline should be similar as PDCCH scheduling PDSCH receptions. We suggest the following update ## Proposal 2.1.4-rev1 (Application time) When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH reception providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, down-select one from - Option 1: the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first actually transmitted SSB index within the first SSB burst that does not begin before the beginning of the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1 of TS 38.213. - Option 2: the UE assumes SSB is transmitted on the second serving cell not before X symbols after the start of the PDCCH reception, where X is based on the SCS of the active DL BWP of the first serving cell [Note: Above does not prevent RAN4 from discussing SSB-related processing timeline.] |

## --->Proposal 2.1.4-rev1 (Application time)

When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB index within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1 of TS 38.213.

  - FFS: how to determine the first [possible] SSB burst

| Moderator | Updated proposal in 2.1.4-rev2. |
| --- | --- |

## Proposal 2.1.4-rev2 (Application time)

When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1 of TS 38.213.

  - FFS: how to determine the first [possible] SSB burst

Search space

Several companies provided views on the search space and other configuration aspects for DCI 2_9.

- Same search space for DCI format 2_9 for adapting the SSB burst periodicity and cell DTX/DRX operation

  - Interdigital, Fujitsu, Samsung, ..

- Configured by higher layer signaling

  - Ericsson

## Discussion point 2.1.5 (Search space)

Please provide your view on the search space details for DCI 2_9 used for SSB burst periodicity adaptation for an Scell.

| Company | Comment |
| --- | --- |
| CMCC | Same search space is enough. |
| ZTE, Sanechips | Same search space is simple and with less specification impact. |
| OPPO | According to the current specification, the search space is DCI format based. Thus, the first bullet is in line with our understanding |
| Xiaomi | We don’t see necessity to put restriction on search space configuration. The search space associated with Cell DTX/DRX DCI and SSB adaptation DCI can be either same or different. |
| NEC | Same search space can be used |
| Apple | Same understanding as OPPO. |
| Nokia, NSB | Same search space can be used. |
| China Telecom | Same search space is enough. |
| LG | Support same search space |
| DCM | We support to have same search space for DCI format 2_9 for SSB adaptation. Search space configuration is associated with DCI format 2_9. Allowing to have different search space based on RNTI will require significant specification impact. Considering such concern, we do not support to be configured with higher layer signaling. |
| Tejas | Same search space is enough |
| Moderator | Search space addressed in Proposal 2.1.1-rev1. |

Additional aspects

Some companies discussed whether additional RRC signaling for offset of SSB for additional SSB periodicity is needed.

- Offset of SSB within the periodicity should be also configured with the adapted SSB periodicity

  - Apple (there are still multiple patterns for the larger periodicity value)

- No need to introduce new RRC signaling for offset to SSB along with adapted SSB periodicity

  - ZTE, CATT (The offset between the latest SSB occasion before the switch and the first SSB occasion after the switch should follow the shorter SSB burst periodicity both before and after the switching.)

## Discussion point 2.2.1 (Additional RRC signaling - offset of SSB)

Please provide your view on whether new RRC signaling is needed for indication of offset of SSB within the periodicity for the additional SSB periodicity.

| Company | Comment |
| --- | --- |
| CMCC | Okay for introduce an offset. The UE does not need to blind detect SSB with such indication so that the energy from UE side can be saved. |
| ZTE, Saenchips | We do not support to introduce a new RRC signalling for indication of offset of SSB within the periodicity. We think the same configuration parameters can configured for SSB before adaptation and after adaptation. |
| OPPO | if only the SSB periodicity is different while the rest of the SSB configuration remain the same. There is no need to configure offset. |
| CATT | No need to introduce new RRC parameter. We can define a rule for the offset of SSB for both of NW and UE to align the understanding. |
| Xiaomi | More clarification is needed. We don’t see the use case for introducing a new RRC singaling to provide additional offset valut. |
| NEC | No need for specifying any additional offset |
| Apple | The intention of introducing an offset, is to limit the pattern and provide more flexibility for NW for sending the DCI signaling in a longer time duration. For example, when NW indicates switching from 20ms to 40ms, there are 2 alternatives for the SSB offset of 40ms can be resulted, while when NW indicates switching from 40ms to 80ms, there are also 2 alternatives for the SSB offset of 80ms can be resulted. In RAN2 LS to RAN4 (RAN1 cced in R1-2501693 in this meeting), it was mentioned that - For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC. Our understanding is that to make sure that neighbor cell could also measure the adapted SSBs, the SSB pattern should fall in the smtc window, which limits the time that NW could send the DCI signaling. For example, if the offset is not configured, the resulted patterns may not fall into the smtc window if the the DCI is sent in T1 or in T2. |
| Fujitsu | Additional configuration of offset is not needed given that only periodicity is adapted. |
| China Telecom | No need to define a offset, but maybe a rule can be defined to align the time position of SSB. |
| Qualcomm | We don’t think the offset indication is needed. Based on explanation from Apple, such offset can be achieved by clearly defining the application timeline. |
| LG | We support the introducing the offset for the additional SSB burst periodicity. SFN offset can be used to determine the candidate SFN position of adapted SSB as in AI 9.5.1. UE can expect the adapted SSB is transmitted at the closest SFN after the processing time of DCI format 2_9. |
| CEWiT | Ok to introduce offset. This reduces blind decoding at UE |
| Moderator | It seems many companies do not see the need for new RRC signalling. |
| Tejas | Not required. |
| DCM | Maybe we need to specify that the SFN-offset before and after should not change to prevent an ambiguity, or we can fine with that UE should monitor based on the place of the SMTC configured for OD-SSB. Which way to go we are fine, but need to be captured as an agreement. |

Some companies discussed RACH interaction with SSB burst periodicity adaptation.

- Interaction with PRACH

  - Qualcomm, LG, Xiaomi (prioritize SSB), Huawei (PRACH occasion validation)

## Discussion point 2.2.2 (Interaction with RACH)

Please provide your view on whether clarifications such as below are needed or whether existing specification text already reflects the UE behaviour.

- For SSB burst periodicity adaptation by DCI 2_9, the UE is not expected to transmit PRACH in a valid PRACH occasion if the occasion precedes a SS/PBCH block in the PRACH slot and does not start at least Ngap symbols after a last SS/PBCH block reception symbol where Ngap is provided in Table 8.1-2 of TS 38.213 and the SS/PBCH block is the SS/PBCH block that is transmitted in response to SSB burst periodicity indication in DCI 2_9.

| Company | Comment |
| --- | --- |
| CMCC | Support |
| OPPO | The current spec may not provide clear UE behavior. The RO validation is preferred to be semi-static instead of dynamic. Thus, we prefer that it is not depending on DCI2-9 to avoid the case that a RO may be valid or invalid based on the reception of DCI2-9 or indication from DCI2-9. This will greatly increase the UE complexity. |
| Xiaomi | Support. It is better to clarify that the ‘valid PRACH’ is from legacy UE perspective. |
| NEC | We have same opinion as Oppo that RO validation should not be dependent on DCI2-9 reception |
| Apple | Similar preference as OPPO. |
| Qualcomm | The RO validation and SSB-RO mapping should be based on the periodicity provided by the legacy signaling (as being agreed “UE is configured with SSB burst periodicity using legacy signalling for the SCell”) – No change is needed for RO validation and SSB-RO mapping. The text under discussion is related to whether UE can transmit PRACH in a valid occasion if the occasion precedes a SSB in the PRACH slot and does not start at least Ngap symbols after a last SSB reception symbol, where the SSB is transmitted according to the periodicity indicated by DCI 2_9. In the legacy spec., the UE uses SSB-positionInBurst and SSB burst periodicity configured in SIB1/RRC to determine whether a RO is valid or not since UE needs to know all time locations of the transmitted SSB. Then the UE can transmit in any valid RO. Now with SSB transmission with the periodicity indicated by DCI, the UE may not be able to transmit in some valid ROs since those ROs that are valid according to the SSB burst periodicity configured by legacy signalling are not qualified for PRACH transmission due to SSB burst periodicity indicated by DCI. Hence, the UE behavior needs to be clarified. |
| LG | Support the above clarifications. |
| DCM | Support FL proposal |
| CEWiT | Support the proposal |
| Tejas | Support |

Some companies discussed PDCCH monitoring interaction with SSB burst periodicity adaptation.

- Interaction with PDCCH monitoring

  - Qualcomm, LG, Xiaomi (UE not required to monitor PDCCH. Need update the specs)

## Discussion point 2.2.3 (Interaction with PDCCH monitoring)

Please provide your view on whether clarifications such as below are needed or whether existing specification text already reflects the UE behaviour.

- UE does not monitor a PDCCH candidate that has at least one RE overlapping with an RE of an SSB burst transmission with periodicity indicated by DCI format 2_9.

| Company | Comment |
| --- | --- |
| CMCC | Support |
| OPPO | from our understanding the current specification is enough |
| Xiaomi | We think the description related to overlapping between SSB and PDCCH candidate is something generic. |
| Nokia, NSB | Support |
| Qualcomm | The current spec does not have SSB burst periodicity switching. Now with periodicity before and after switching, which periodicity is used for determining the overlap should be clarified. Please see the below illustration: |
| LG | Support the above clarifications. |
| DCM | Support FL proposal |
| CEWiT | Support |
| Tejas | Support |

Some companies discussed PDCCH monitoring interaction with PDSCH resource mapping.

- Interaction with PDSCH resource mapping

  - Qualcomm, LG, Xiaomi (prioritize SSB, PDSCH is rate-matched)

## Discussion point 2.2.4 (Interaction with PDSCH resource mapping)

Please provide your view on whether clarifications such as below are needed or whether existing specification text already reflects the UE behaviour.

- The REs of SSB burst transmission with periodicity indicated by DCI format 2_9 are considered as not available for PDSCH resource mapping.

| Company | Comment |
| --- | --- |
| CMCC | Support |
| OPPO | from our understanding the current specification is enough |
| CATT | In our understanding, it can be up to gNB implementation to design the rate matching pattern for the adapted SSB. No spec impact. |
| Xiaomi | Support. |
| Nokia, NSB | Support. We think that clarification is needed because current specification may be interpreted to apply only to legacy SSB configuration. |
| Qualcomm | Similar discussion as being discussed under PDCCH monitoring |
| LG | Support the above clarifications. |
| DCM | Support FL proposal |
| Tejas | Support |

Scenarios

Some companies further discussed scenarios for adaptation of SSB burst periodicity, including referencing scenarios from AI 9.5.1.

- No additional restriction on scenarios

  - Huawei, Ericsson, Samsung, CATT, vivo (Scenario 2/2A/3A/3B)

- At least Scenario 2/2A/3B

  - Nokia

- Scenario 3A,3B

  - CT

- Applicable only for activated SCell (Scenario 3B)

  - Qualcomm, CMCC, ZTE, Xiaomi

## Discussion point 2.2.5 (Scenarios)

Please provide your view on whether there should be any additional restrictions (if any, which ones) on applicable scenarios for DCI-based SSB burst periodicity adaptation for SCell.

| Company | Comment |
| --- | --- |
| CMCC | At least support Scenario 3B. For other scenario, we are also open for it. |
| ZTE, Sanechips | In our views, Scenario #3B should be firstly considered for DCI-based SSB burst periodicity adaptation for SCell. But we also fine with consideration of DCI-based SSB adaptation in agreed scenarios in 9.5.1, i.e., Scenario #2, 2A. |
| OPPO | the SSB burst periodicity adaptation can be applied in any scenarios. No limitation is needed for this agenda item 9.5.3 |
| CATT | We don’t think any additional restriction is needed since it a cell-level operation. |
| Xiaomi | We are wondering how DCI-based indication works considering UE will not monitor any PDCCH associated with a deactivated SCell? |
| Apple | First to clarify that this is only discussing about scenarios for when DCI format 2-9 can be sent instead of combined with OD-SSB, i.e. this is an SCell without OD-SSB configuration. If this is the common understanding, we tend to think there could be no restrictions for the scenario. |
| Nokia, NSB | At least support Scenario #3B. If MAC CE based indication is supported, then Scenarios #2 and #2A can also be considered |
| Fujitsu | Restriction on scenarios is not needed. |
| China Telecom | At least support #3B. |
| Qualcomm | 38.321 has the following: For the applicable scenario, the current TS38.321 specifies that if the Scell is deactivated, the UE doesn’t monitor PDCCH on the Scell nor for the Scell. Therefore, DCI indication of adapted SSB periodicity can only occur on activated Scell for the same cell and/or other activated scells (i.e., Scenario 3B). From our perspective, no specification change is necessary. |
| LG | Support for no additional restrictions on scenarios. |
| DCM | The applicable scenario can be all. |
| CEWiT | Support all scenarios, no restrictions are needed |
| Tejas | Support 4th bullet. |

Interaction with on-demand SSB operation

At least one company brought up interaction with on-demand SSB operation on the SCell.

- SSB periodicity switching on an SCell can be used at least when OD-SSB is not configured for the SCell

  - Apple

- SSB periodicity switching can be used at least when OD-SSB is configured for the SCell but OD-SSB is not activated/transmitted.

  - Apple

## Other aspects

Companies can raise any other aspects related to adaptation of SSB here.

| Company | Comment |
| --- | --- |
| OPPO | we agree to limit the usage of 9.5.3 when OD-SSB is configured. |
| Apple | We prefer to decouple the SSB adaptation with OD-SSB, as in our comment to Discussion point 2.2.5 |
| CEWiT | Change in continous active SSBs within a burst before and after Adaptation can be considered |
| Tejas | Support 1st bullet. |

# Adaptation of PRACH

Legacy PO

Several companies discussed which ROs can be considered as legacy ROs. Some companies seem to suggest a 1-1 associated of one configuration of additional ROs with one configuration of legacy ROs.

- rach-ConfigCommon-r15

  - Qualcomm, vivo, CATT, Ericsson

- msgA-ConfigCommon-r16

  - Ericsson

- rach-ConfigCommon-r17 within additionalRACH-ConfigList-r17

  - vivo, Apple, Ericsson

- Discuss whether one configuration of additional ROs is associated with one configuration of legacy ROs or one configuration of additional ROs can be associated with multiple configurations of legacy ROs

  - Qualcomm

- Pairwise association of an additional RO configuration with legacy RO configuration for a feature for the additional RO invalidation.

  - Apple, vivo, Huawei

    - Apple

      - Legacy PRACH resource is the one that is configured in the same RACH-ConfigCommon as the additional PRACH resources.

      - In case of shared RO used for 4-step and 2-step RACH, 2-step RACH can also be performed on the additional PRACH resource if configured and indicated as available.

      - At least for the feature combinations configured by featureCombinationPreambleList-r17, the additional PRACH resources configured in the same RACH-ConfigCommon can be also used for those feature combinations.

    - Huawei

      - the legacy RO is the RO configured by a legacy prach-ConfigurationIndex where this legacy prach-ConfigurationIndex and R19 additional prach-ConfigurationIndex are associated with the same feature combination.).

    - vivo

      - The additional RO is invalid by a legacy RO in which the additional RO is within the same configuration of the legacy RO.)

        - When UE is not provided with featurecombination for RA partitioning, validation for the additional ROs is based on the ROs in legacy RACH-configCommon.

        - When UE is provided with featurecombination for RA partitioning, validation for the additional ROs for a certain featurecombination is based on the ROs in AdditionalRACH-Config with the same featurecombiantion.

- Invalidate additional ROs overlapping with all the legacy ROs supported by the UE

  - DoCoMo (do not support on adapting PRACH resources except for common PRACH resources for 4-step RACH.)

- Clarify that the legacy RO for valid RO determination on Rel-19 additional PRACH resource is all PRACH resources in SIB1/RRC configuration, regardless of whether a UE support the corresponding feature.

  - CMCC

Our understanding is that the additional PRACH resource can be provided in the RACH-ConfigCommon (as illustrated by the green box in Fig. 3 ). Two configuration alternatives can be considered, with details can be up to RAN2 decision.

Alt 1: The additional PRACH resource is configured by a new IE in parallel with RACH-ConfigGeneric

Alt 2: The additional PRACH resource is configured by additional parameter inside RACH-ConfigGeneric IE

In both alternatives, the legacy PRACH resource is the one that is configured in the same RACH-ConfigCommon as the additional PRACH resources. This is actually the PRACH resource configured in SIB1 for all UEs since Rel-15, which is a simplest solution.

Fig. 3 Additional PRACH resources configured in RACH-ConfigCommon

From Apple (R1-2502614)

## Proposal 3.1.1 (Legacy RO)

For adaptation of PRACH in time-domain, at least for 4-step RACH,

- Support configuration of the additional PRACH resources within RACH-ConfigCommon

- For ‘additional RO invalidation before the SSB-RO mapping’

  - When the additional PRACH resources are configured without feature combination for RA partitioning

    - the legacy PRACH resources used are the ones configured within the same RACH-ConfigCommon as the additional PRACH resources.

  - When additional PRACH resources are configured for a feature combination for RA partitioning,

    - the legacy PRACH resources used are the ones configured within the same RACH-ConfigCommon for the same feature combination.

| Company | Comment |
| --- | --- |
| Xiaomi | For the first sub-bullet, we are fine. For the second sub-bullet and the following sub sub-bullet, we are confused why we need to couple legacy PRACH resources and additional PRACH resources? They are separated configured anyway. |
| Moderator | Updated proposal from the offline discussion in 3.1.1-rev1 below. |

## Proposal 3.1.1-rev1 (Legacy RO)

For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI,

- Support configuration of the additional PRACH resources within [same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources

  - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are the ones configured at least for Rel-15.

| Company | Comment |
| --- | --- |
| Sharp | Support |
| NEC | Support |
| Apple | We think the [] in the main can be deleted. After some offline discussions, and according to RAN2 agreement in last meeting, additional PRACH resources can be also supported for RACH partitioning with all the features, i.e., can be configured also in AdditionalRACH-ConfigList-r17 which also contains RACH-ConfigCommon(shown in red font in Fig.3 in above). In this case, the legacy PRACH resource is the one configured in the same RACH-ConfigCommon in AdditionalRACH-ConfigList-r17 which is different from Rel-15 PRACH resources. From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain. ## Proposal 3.1.1-rev1 (Legacy RO) updated For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI, - Support configuration of the additional PRACH resources within [same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are the ones configured in the same RACH-ConfigCommon at least for Rel-15. ## Proposal 3.1.1-rev1 (Legacy RO) updated v2 For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI, - Support configuration of the additional PRACH resources within [same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are the ones configured in the same RACH-ConfigCommon as the additional PRACH resources at least for Rel-15. |

Additional parameters that can be configured for additional PRACH resource

Some companies discussed additional parameters that can be configured for additional PRACH resources. One company (vivo) also provided some further inputs related to RRC parameters.

- CB-PreamblesPerSSB

  - Huawei (CB-PreamblesPerSSB is dependent on ssb-perRACH-Occasion in legacy), vivo, Ericsson(legacy IE structure is ssb-perRACH-OccasionAndCB-PreamblesPerSSB)

- totalNumberOfRA-Preambles

  - Huawei (should be consistent with ssb-perRACH-OccasionAndCB-PreamblesPerSSB in legacy), vivo

- numberOfRA-PreamblesGroupA

  - Huawei (numberOfRA-PreamblesGroupA should be consistent with ssb-perRACH-OccasionAndCB-PreamblesPerSSB in legacy)

## Proposal 3.1.2 (Additional parameters)

Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported

  - CB-PreamblesPerSSB

  - totalNumberOfRA-Preambles

  - numberOfRA-PreamblesGroupA

| Company | Comment |
| --- | --- |
| Xiaomi | Is it too early to dive into the configuration details? We are still discussing how to provide additional PRACH configuration at high level. |
| Apple | Open to discuss and tend to think at least CB-PreamblesPerSSB is at least needed. |
| Fujitsu | Support |
| CEWiT | Support |
| Moderator | Below is agreement from Wednesday online session. Agreement Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported - CB-PreamblesPerSSB Companies can provide their view on the other two parameters. |
| Tejas | Support separate configuration totalNumberOfRA-Preambles and numberOfRA-PreamblesGroupA |

RAN1 made below agreement in RAN1#120 meeting.

Agreement

- Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported

  - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs)

  - FFS: When there is no configuration of Msg1-FDM

- Separate configuration of number of SSB per RO is supported

Some companies provided views on the FFS.

- default value (=1?)

  - ZTE, Tejas, DoCoMo

- Use the corresponding value from legacy RO configuration

  - Ericsson

## Discussion point 3.1.3 (Msg1-FDM)

Please provide your view on the FFS related to the Msg1-FDM.

| Company | Comment |
| --- | --- |
| ZTE, Sanechips | We are open to discuss the FFS related to the Msg1-FDM. Setting a default value equal to 1 is much simpler for us. |
| CATT | We prefer the Msg1-FDM always configure for the additional RACH resources which is same as legacy configuration. |
| Sharp | Either option is OK though we slightly prefer to use the same value with legacy RO configuration as a default value to have SSB mapping alignment. |
| Apple | Using the same value with legacy seems more reasonable. |
| Fujitsu | We think either option is workable, and we slightly prefer using the same value for legacy RO configuration. |
| LG | Use the corresponding value from legacy RO configuration when there is no configuration of Msg1-FDM for additional PRACH resources. |
| CEWiT | We think both options are workable, and we slightly prefer using the default value. |

Bit-field in DCI 1_0 with P-RNTI

Several companies provided theirs view on the bit-field to use and the size of the bit field for adaptation indication in DCI format 1_0 with P-RNTI.

- Opt1 - Reserved bits

  - Qualcomm (1 bit), Panasonic, Xiaomi (1 bit, no need to involve RAN2), LG, MTK (1 bit), NTT, Honor, Ericsson (sum this bit field + TRS bits <=6)

- Opt2 – Bit 5-8 of Short messsage

  - Huawei, Nokia, Vivo, Apple (1 or 2 bits), ZTE, Google, Fujitsu (1 bit), Panasonic (>1bit, no dependence on TRS bits), InterDigital, ETRI (1 bit), Transsion (1 or 2 bits), Ericsson, CATT (1 bit)

- Opt3 – both opt1 and opt2

  - Futurewei, Spreadtrum, Ericsson

## Proposal 3.2.1 (Bit in DCI P-RNTI)

For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI

- Use one bit from the Bits 5-8 within the Short Message (from upper layers)

- Send LS to RAN2 to confirm the use of this bit.

| Company | Comment |
| --- | --- |
| ZTE, Sanechips | Fine with FL’s proposal. |
| OPPO | support |
| CATT | Support. |
| Panasonic | Support use of option 1 or option 2 and an indication (i.e., short message indicator) that at least PRACH adaptation is being carried in the DCI but should not include TRS availability indication |
| Sharp | Support |
| NEC | Support |
| Apple | Support |
| DCM | Support |

Reference point

Several companies provided theirs view on the reference point for applying the availability information in DCI format 1_0 with P-RNTI.

- Option 1: SFN of the first PF from the next I-DRX cycle

  - Qualcomm, Spreadtrum/Unisoc, Fujitsu, Panasonic, CATT, Apple

- Option 2: SFN of the first PF from the current I-DRX cycle

  - ZTE, Google, Lenovo, Apple

- Option 3: From the first frame of the first PRACH association period after UE receives the DCI

  - Vivo, Google, CT, InterDigital, Xiaomi, LG, ETRI, Transsion, Honor, DoCoMo, Nokia, Ericsson

- Option 4: From the first frame of the current SI modification period

  - Huawei/HiSi, ZTE, KDDI

- Option 5: From the first frame of the next SI modification period

  - FW, Nokia, Samsung, Fujitsu, CMCC, OPPO, MTK, KDDI

Considerations: Latency/delay in using the available additional PRACH resources, alignment of available additional PRACH resources across different UEs (in both idle/inactive and connected modes), fairness to the UE (in terms of usability of resources), ..

It was mentioned that “default paging cycle” is more appropriate than “I-DRX cycle” since connected mode UE is not aware of I-DRX cycles (Ericsson, R1-2502804). So perhaps the Options 1 and 2 are more suited to be formulated as “from the …default paging cycle”.

## Discussion point 3.2.2 (Reference point)

Please provide your view on the above options for reference point for applying the availability information in DCI format 1_0 with P-RNTI. Please indicate all options that you would be OK with, including any further modifications.

| Company | Comment |
| --- | --- |
| CMCC | Support option 5, also ok with option 4 but this might be unfair for the UE that monitor the last PO in the corresponding Si modification period. |
| ZTE, Sanechips | We support Option 2, or Option 4. From the perspective of delay for using additional PRACH resources, we think Option 2 or Option 4 can provide less delay for UE to use additional PRACH resources once UE receives P-RNTI DCI |
| CATT | We are ok with Option1 and Option5. |
| Xiaomi | There are two points from our understanding: - The reference point has to be somewhere after UE receive the DCI, otherwise UE doesn’t know the validity information. - The specific reference point after UE obtains validity information. The first point is the fundamental step of define reference point of validity duration. We notice only option 3 mentioned reception of DCI. For the second point, the only difference is granularity. As we are talking about PRACH adaptation, the one associated with PRACH is the most straightforward. Based on the above analyses, we prefer option 3. |
| Panasonic | In our opinion, using SI modification period to align the assumptions for two RRC mode UEs is too restrictive as the SI modification period can be as long as 16 (modification period coefficient) * 2560 (paging cycle) ms and merit of option 4 or 5 would be very marginal in such case. We propose that the reference point for RRC idle/inactive mode UE and RRC connected mode UE, is the first frame of the next N default paging cycles, where N can be configurable and if not configured, N is equal to the number of paging cycles of SI modification period. |
| NEC | We are okay with Option-3 or Option-4 |
| Apple | Open for the discussion. We think what is more import is how UE assumes the additional PRACH resources become available and what is the ending time. UE assumes the additional PRACH resources are available from the first PRACH association period a time gap after UE receives the DCI. For the reference time, our preference is Option 1 or Option 2. |
| Fujitsu | We support option 1 or option 5. For option 1, the connected mode UEs need to follow the same UE behaviour as the ETWS/CMAS capable connected mode UEs that is monitoring any PO at every default paging cycle. |
| China Telecom | Support Option 3. |
| LG | We support Option 3. The UE can quickly utilize additional ROs to transmit PRACH as soon as the UE is indicated the availability of additional PRACH resources by DCI. It is also considered extending the end point of the validity time duration to be aligned for all UEs (idle/inactive and connected mode UEs). |
| DCM | Support option 3 with some modification; from the first frame of the first PRACH association period after UE finishes decoding DCI upon reception of the DCI. Considering that NW allocates additional PRACH resources based on the traffic, it should be better for UEs to start using the resources upon receiving the DCI instead of waiting other UEs who also receives the DCI to align the starting time. It can be understandable from NW energy saving perspectives to allow additional PRACH resources for short period of time by aligning when additional PRACH resources to be used. However, with this method, it takes long time to start using additional PRACH resources, so by the time when UE can use the resources, legacy PRACH resources are likely to be used by the UE. After the discussion on mask determination, option 1 or option 2 will be fine. Even though it is up to the mask configuration, we are slightly prefer option 1. Based on the current mask design, the available ROs will be appeared around the beginning in the K-APPs. Given that UEs will receive the indication within the K-APP, it is mostly likely that UE has to wait next K-APP to get additional ROs. Waiting extra time until next default paging cycle might not be so different. |
| CEWiT | Support option 3 |
| Moderator | Proposal is made in 3.2.2 |
| CMCC2 | In current deployment, for RRC IDLE/INACTIVE UE, it monitors the Paging DCI on its own PO per I-DRX cycle. Meanwhile, for RRC CONNECTED UE, it monitors the Paging DCI once on any PO per SI modification period for SI update acquisition. Therefore, when additional PRACH resource is about to deactivated/disabled, consider that a CONNECTED UE may monitor Paging DCI at the end of one SI modification period. Therefore, from gNB side, it can only deactivate/disable the additional PRACH resource at the end of the SI modification period to ensure the UE performance, as is shown in the figure below. Given consideration on the above cases, it is recommended the application time for UEs in all RRC modes are aligned to save the energy from gNB side, or to give the UE more chance to use the additional PRACH resource. Note that currently a UE supports ETWS/CMAS indication in CONNECTED mode may monitor the Paging DCI per I-DRX cycle. We are open for Option 1 if there can be some enhancement for R19 NES UE in CONNECTED UE, which is similar as the current ETWS/CMAS feature. |

## Proposal 3.2.2 (Reference point)

For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is:

- Option 1: SFN of the first PF from the next I-DRX default paging cycle where UE receives the DCI

- Option 2: SFN of the first PF from the current I-DRX default paging cycle where UE receives the DCI

- Option 3: From the first frame of the first PRACH association period after UE receives the DCI

- Option 4: From the first frame of the current SI modification period where UE receives the DCI

- Option 5: From the first frame of the next SI modification period where UE receives the DCI

| Company | Comment |
| --- | --- |
| Moderator | Given the inputs and discussion, please indicate whether Option 3 would be OK? |
| LG | We support with Option 3 |
| CMCC | We do not support Option 3, we think Option 5 and Option 1 (with enhancement for CONNECTED mode) should be supported. Please see our comment in discussion point 3.2.2 above. |
| Qualcomm | Look like our comments provided in v029 were missing. We C&P the comments below: From our perspectives, Options 1/2 do not work for idle UEs according to TS 38.331 e.g., “ETWS or CMAS capable UEs in RRC_IDLE or in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for indications about PWS notification in its own paging occasion(s) that the UE monitors as specified in TS 38.304 [20]. ETWS or CMAS capable UEs in RRC_CONNECTED shall monitor for indication about PWS notification in any paging occasion at least once every defaultPagingCycle if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging” ## Suggested update to Proposal 3.2.2 (Reference point) For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is: - Option 1: SFN of the first PF from the next I-DRX default paging cycle where UE receives the DCI - Option 1A : - The SFN of the first PF from the next DRX cycle specified in Clause 7.1 in TS 38.304 for RRC idle/inactive UEs. - The SFN of the first PF from the next DRX cycle defined by default paging cycle (defaultPagingCycle) for RRC connected UEs. - Option 2: SFN of the first PF from the current I-DRX default paging cycle where UE receives the DCI - Option 3: From the first frame of the first PRACH association period after UE receives the DCI - Option 4: From the first frame of the current SI modification period where UE receives the DCI - Option 5: From the first frame of the next SI modification period where after UE receives the DCI |

## --->Proposal 3.2.2 (Reference point)

For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is:

- Option 1: start of SFN of the first PF from the next I-DRX default paging cycle where UE receives the DCI

- Option 1A:

  - For RRC idle/inactive UE, start of SFN of the first PF from the next DRX cycle specified in Clause 7.1 in TS 38.304.

  - For RRC connected UE, start of SFN of the first PF from the next DRX cycle defined by default paging cycle (defaultPagingCycle).

- Option 2: start of SFN of the first PF from the current I-DRX default paging cycle where UE receives the DCI

- Option 4: From the start of first frame of the current SI modification period where UE receives the DCI

- Option 5: From the start of first frame of the next SI modification period where after UE receives the DCI

| Moderator | Proposal is update in 3.2.2-rev1. |
| --- | --- |

## Proposal 3.2.2-rev1 (Reference point)

For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is:

- the start of first frame of the current SI modification period where UE receives the DCI

Validity duration

Several companies provided theirs view on the validity duration for the availability information in DCI format 1_0 with P-RNTI.

Higher layer signaling on validity duration

- Huawei, vivo, Spreadtrum, Google, CT, Fujitsu, ETRI, Honor, MTK, Nokia, Ericsson

## Proposal 3.2.3 (Validity duration)

For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI

- the validity duration is configured via higher layer signalling.

| Company | Comment |
| --- | --- |
| CMCC | We think the duration can be predefined. |
| ZTE, Sanechips | Fine with FL’s proposal. |
| CATT | Support. |
| Sharp | Support |
| Apple | Support. |
| LG | Support |
| DCM | We are fine with the proposal. |
| CEWiT | Support |

Several companies provided theirs view on the validity duration granularity.

- Slot level

  - FW

- PRACH association period

  - Nokia, Vivo, Fujitsu, ETRI, Ericsson

- PRACH association pattern period

  - Nokia

- (Default) paging cycles

  - Apple, Spreadtrum, Google, Fujitsu, CATT, Panasonic

- SI modification period

  - Fujitsu, OPPO, CMCC, MTK

## Discussion point 3.2.4 (Validity duration value)

Please provide your view on the possible value/granularity of validity duration. Please indicate all options that you would be OK with, including any further modifications.

  - (Option 1) Multiple of PRACH association period

  - (Option 2) Multiple of PRACH association pattern period

  - (Option 3) Multiple of default paging cycle

  - (Option 4) Multiple of SI modification period

| Company | Comment |
| --- | --- |
| CMCC | To take care of UEs in all RRC mode, we support option 4. |
| ZTE, Sanechips | For the validity duration value, we are open to discuss. |
| CATT | We are open to discuss Option 3 and Option 4. If the granularity of the validity time duration is shorter than one default paging cycle, it will result in the end of current the validity time duration not aligning with the start time of the next validity time duration. This is because the network requires at least one default paging cycle to page all the UEs within a cell. |
| Xiaomi | Our preference is option 1. |
| NEC | Supportive of Option-1 or Option-2 |
| Apple | Our preference is Option 3. Same understanding as CATT if the granularity of the validity time duration is shorter than one default paging cycle, |
| Fujitsu | We are open to discuss the options. |
| China Telecom | Considering that the PRACH mask can be PRACH association period, we prefer Option 1. |
| LG | We support Option 1. |
| DCM | We are fine with option 2, option 3, or option 4, but our preference is option 3. The default paging cycle has the balanced length between PRACH association period level ~ SI modification period. |
| Cewit | We support Option 4 |
| Samsung | All works. But option 1 and option 2 have better alignment on the K association pattern periods for the mask. So we support option 2. Actually, the best way is multiple of K association pattern periods. |
| CMCC2 | This is strongly related with proposal 3.2.2. We may discussion with it together, or we can wait the progress on such issue first and then look into this proposal. |

## Proposal 3.2.4 (Validity duration value)

The validity duration configured by higher layer signalling for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is

- Select one option from the following for granularity of validity duration.

  - (Option 1) Multiple of PRACH association period

  - (Option 2) Multiple of PRACH association pattern period

  - (Option 3) Multiple of default paging cycle

  - (Option 4) Multiple of SI modification period

| Company | Comment |
| --- | --- |
| Moderator | Given the inputs and discussion, please indicate whether Option 3 would be OK? |
| LG | To align the granularity of validity duration with the reference point in 3.2.2, we support Option 1. |
| CMCC | We think this is strongly related with proposal 3.2.2. We may discuss with it together, or we can wait the progress on such issue first and then look into this proposal. Our preference is Option 4 and Option 3 (with enhancement on CONNECTED mode). |

## -->Proposal 3.2.4 (Validity duration value)

The validity duration configured by higher layer signalling for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is

- Select one option from the following for granularity of validity duration.

  - (Option 1) Multiple of PRACH association period

  - (Option 2) Multiple of PRACH association pattern period

  - (Option 3) Multiple of default paging cycle

  - (Option 4) Multiple of SI modification period (1x, 2x, 4x [8x],… )

| Moderator | Proposal is update in 3.2.4-rev1. |
| --- | --- |

## Proposal 3.2.4-rev1 (Validity duration value)

The validity duration configured by higher layer signalling for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is

- (Option 4) Multiple of SI modification period ({1,2,[3],4,8,[],[],..})

DCI deactivation

Several companies provided theirs view on whether DCI can also be used to explicitly deactivate the additional PRACH resources.

  - Yes

    - CT, Tejas, Transsion, MTK, KDDI

  - No

    - ETRI, Honor, Nokia, Ericsson, Spreadtrum/Unisoc

## Discussion point 3.2.5 (Explicit deactivation)

Please provide your view on whether you see a need for explicit deactivation indication of additional PRACH resources in DCI.

| Company | Comment |
| --- | --- |
| CMCC | 1 bit indication is enough. |
| ZTE, Sanechips | We do not support to consider explicit deactivation indication. As we have already supported validity time duration for additional PRACH resources, it means that additional PRACH resources would be invalid out of the time duration, which achieves the same goal for considering explicit deactivation indication. |
| CATT | We don’t see the motivation to introduce deactivation indication. |
| Xiaomi | We don’t quite understand the proposal. We already support DCI-based indication for PRACH adaptation, i.e., which PRACH configuration is applied. If additional PRACH is not indicated, it is equal to deactivation of additional PRACH. From this perspective, we don’t see the point of the proposal. |
| NEC | There is no need to have deactivation indication |
| Apple | According to previous discussion, the validity duration is introduced and no explicit deactivation is needed. |
| China Telecom | Actually, we have the similar as Xiaomi. It depends on how the 1 bit is used for indicating the PRACH adaptation, if the 1 bit should be always be indicated when the additional PRACH is adopted, it is naturally to deactivate the additional PRACH when it is not indicated; and if the 1 bit is just to indicate the PRACH adaptation is adopted when the legacy PRACH is adopted currently, the 1 bit can also be used to indicate the legacy RPACH should be adopted when the additional PRACH is adopted. |
| DCM | We are open to discuss DCI based deactivation for additional PRACH resources. If validity duration of additional PRACH resources are configured as very long time, it may be beneficial to have explicit deactivation mechanism to turn off PRACH resources. |
| Moderator | It seems several companies do not see need for this. |

Applicable unit of PRACH subset mask

Several companies provided theirs view on applicable unit of PRACH subset mask.

- Alt 1: PRACH association period

  - Huawei, FW, Nokia, Qualcomm, vivo, Apple, Google, Panasonic, InterDigital, LG, Transsion, CATT, Sharp, Ericsson

- Alt 2: PRACH association pattern period

  - Samsung (because in some cases there is only one association period in one association pattern period), Fujitsu, OPPO, LG, DoCoMo

- Alt 3: SFN level

  - Ericsson (more flexible)

## Proposal 3.2.6 (Unit of PRACH subset mask)

For DCI-based adaptation for additional PRACH resources, the single PRACH mask that identifies the subset of the additional PRACH resources is applicable at unit of

  - PRACH association period

| Company | Comment |
| --- | --- |
| ZTE, Sanechips | Fine with FL’s proposal. |
| CATT | Support |
| Sharp | We basically support the FL’s proposal, but the exact granularity of masking can be configurable by signalling the number of association periods applied for the masking table. |
| Apple | Support. |
| LG | Support |
| DCM | We prefer that the unit of PRACH subset mask can be PRACH association pattern period level. The reason is that for association period level, the length of the period can be different within the PRACH association pattern period (it can be 10 ms, 40 ms, or other values within the PRACH association pattern period). In addition, since SSB-RO mapping differs between each UE (due to different interpretation of “legacy” ROs and then invalid additional ROs), activating/deactivating an association period might result in totally different RO patterns for multiple UEs. Considering such difficulty of NW management exists, we prefer the rougher on/off pattern, PRACH association pattern period level. |
| CEWiT | We support SFN level to get more flexibility at gNB |

Identification of PRACH subset mask

For Option 1, some companies provided PRACH mask tables with 4,8 and 16 rows (Annex C has the proposals from companies with detailed entries of the table). Some companies also discussed whether this is applicable within one PRACH association pattern period or across multiple PRACH association pattern periods. Some companies suggested introducing additional signalling parameter on top of PRACH mask index for determining the PRACH mask (e.g. Apple, Nokia). For Option 2, several companies proposed using a bitmap with 1-bit per applicable unit.

- Option 1 (Pre-defined table, semi-static signaling indicates a PRACH mask index, with N rows)

  - Nokia, Samsung, Apple, CT, InterDigital, Qualcomm,Honor, CATT, LG, KDDI

    - N=4

      - Qualcomm (within an association pattern period)

      - Nokia (within one and across multiple association pattern periods)

      - Samsung (across association pattern periods)

    - N=8

      - Apple/Nokia (within one and across multiple association pattern periods)

      - Samsung (across association pattern periods)

    - N=16

      - Nokia (within one and across multiple association pattern periods)

      - Samsung (across association pattern periods)

- Option 2 (The PRACH mask is based on configuration parameters e.g. bitmap at SFN-level, periodic time domain window, …)

  - ZTE, vivo, Fujitsu, Qualcomm (periodic time-domain window), Ericsson, DoCoMo, Cewit, LG, Sharp

    - Bitmap

      - ZTE, Fujitsu (at association pattern period level), Ericsson, DoCoMo, Cewit, LG, Sharp

    - periodic time-domain window

      - Qualcomm

Based on company inputs, the options seem to be as follows.

- Option 1: The PRACH mask is from a PRACH mask table

  - Pre-defined table with N=[4 or 8 or 16] rows

  - The semi-static signalling indicates a PRACH mask index

- Option 2: The PRACH mask is based on configuration parameters

  - Option 2-1: configured bitmap of N2= [4 or 16] bits (one bit per applicable unit)

  - Option 2-2: periodic time-domain window

## Discussion point 3.2.7 (Identification of the PRACH subset mask)

Please provide your view on the above options for identifying the PRACH subset mask, including e.g. size of the PRACH mask index or size of the bitmap. Please also provide any further details, including additional signalling parameters (if applicable/needed).

| Company | Comment |
| --- | --- |
| ZTE, Sanechips | Support Option 2-1. Configured bitmap can provides flexibility for different PRACH mask patterns for different scenarios. |
| OPPO | support option 1 |
| CATT | We slightly prefer option1. |
| Panasonic | Support option 2 |
| Sharp | Our preference is Option 1 with additional configuration. We prefer to configure the masking period for the predefined table by signalling the number of association periods (X) (X equals M for Apple’s table or N for Nokia’s table in Annex C). Such configurable masking period allows the masking with long sleeping opportunity such as association pattern period-level masking. |
| NEC | Support Option-2 |
| Apple | Option 1. Also as mentioned by Sharp, the masking period also needs siganaling. |
| Fujitsu | Support option 2-1. It requires less specification efforts and can offer more flexibility in terms of configuration than option 1. |
| China Telecom | Prefer option 1. |
| LG | We slightly prefer Option 2-1. |
| Nokia | After Tuesday’s agreement on the unit for the application on the mask, i.e., association period, and the fact that the PRACH mask applies to every [configured] K SSB RO association pattern period(s), we think that only viable option in terms of specification impact, flexibility and scalability is Option 1. There are a number of advantages given by this approach. For instance, this allows to accommodate any value of K, irrespective of whether it is determined or configured or given by spec. Additionally, we would also like to note that each codepoint to indicate one row of the PRACH mask table is technically a bitmap, with either 2, 3 or 4 bits depending on what RAN1 will agree. More importantly, it would be a more powerful and flexible bitmap since it would not need one bit per association period to be indicated…but it would have constant size (differently from Option 2-1). |
| CEWiT | Support option 2, specifically bitmap at SFN level |
| Moderator | A proposal is made in 3.2.7. |

## Proposal 3.2.7 (Identification of the PRACH subset mask)

For DCI-based adaptation for additional PRACH resources, select one from the following options:

- Option 1: The PRACH mask is from a PRACH mask table

  - Pre-defined table with N rows, select one alternative.

    - Alt 1-1: N=4

    - Alt 1-2: N=8

    - Alt 1-3: N=16

  - The semi-static signalling indicates a PRACH mask index

  - K is configured via higher layer signalling

- Option 2: The PRACH mask is based on configuration parameters

  - Option 2-1: configured bitmap of N2 bits, select one alternative.

    - Alt 2-1a: N2=4 bits

    - Alt 2-1b: N2=16 bits

  - Option 2-2: periodic time-domain window

| Company | Comment |
| --- | --- |
| Moderator | Companies can provide their view on above proposal, also considering the following agreement from this week. Agreement For DCI-based adaptation for additional PRACH resources, PRACH mask that identifies the subset of the additional PRACH resources is applicable at unit of - PRACH association period - This PRACH mask applies to every [configured] K SSB RO association pattern period(s) |
| Qualcomm | Based on offline discussion with a few companies (e.g., Nokia, Samsung, etc.), it is suggested to consider the following proposal for masking. Example in understanding the above agreement Suggested proposal - Support the following masking table - Alt. 1: Mask index; Indication of masked association periods (AP) within every K association pattern periods (APP). 0; First N APs of K APPs. 1; Last N APs of K APPs. 2; Every odd AP in K APPs. 3; Every even AP in K APPs. - The values of K and N are configured - Alt. 2: One-bit indication to indicate whether first N APs or last N APs within every K APPs are masked - The values of K and N are configured - Alt. 2A: First N APs within every K APPs are masked - The values of K and N are configured - Alt. 3: Mask index; Indication of masked association periods (AP) within every K association pattern periods (APP). 0; 1st half of the APs in K APPs. 1; 2nd half of the APs in K K APPs. 2; 1st quarter of the APs in K APPs. 3; 2nd quarter of the APs in K APPs. 4; 3rd quarter of the APs in K APPs. 5; 4th quarter of the APs in K APPs. 6; Every odd AP in K APPs. 7; Every even AP in K APPs. - The value of K is configured - If a mask index is not configured, PRACH transmission can be on all configured additional valid ROs (if indicated as available) |
| Samsung | We think the proposals from QC can be a way forward on this issue. To us, we are open to all 3 alternatives (2A works but looks too restrictive). But I think we did not need to support all 3, maybe some wording change on the main bullets: Support the following masking table down select from the following masking table/indication: |
| Moderator | Discussion from offline is as below. |

Proposal 3.2.7 (Identification of the PRACH subset mask)-offline

- Support the following masking table

- Option. 1-1: The PRACH mask is from a PRACH mask table

  - Pre-defined table

  - PRACH mask index, K and N are configured

    - For K: one from up to four candidate values

    - For N: one from up to four candidate values

| Mask index | Indication of masked association periods (AP) within every K association pattern periods (APP) |
| --- | --- |
| 0 | First N APs of K APPs |
| 1 | Last N APs of K APPs |
| 2 | [Every odd AP in K APPs |
| 3 | [Every even AP in K APPs] |

    - Example : Value range for K: [1,2,4,8]

    - Example : Value range of N:  [1 … 128]

- Option 1-2:

  - Pre-defined table

  - PRACH mask index, K are configured

    - For K: one from up to four candidate values

    - For N: one from up to four candidate values

| Mask index | Indication of masked association periods (AP) within every K association pattern periods (APP) |
| --- | --- |
| 0 | 1st half of the APs in K APPs |
| 1 | 2nd half of the APs in K APPs |
| 2 | 1st quarter of the APs in K APPs |
| 3 | 2nd quarter of the APs in K APPs |
| 4 | 3rd quarter of the APs in K APPs |
| 5 | 4th quarter of the APs in K APPs |
| 6 | [Every odd AP in K APPs] |
| 7 | [Every even AP in K APPs] |

- Option 2: The PRACH mask is based on configuration parameters

  - Option 2-1: configured bitmap of N2 bits, select one alternative.

    - N2 is based on K and N3,

    - N3 is number of APs corresponding to a bit in the bitmap

    - K is configured

    - If the number of APs in K APP is greater than the size of the bitmap

      - Then the ROs in the AP not covered by the bitmap are not in the subset

    - Alt 2-1a: N2=4 bits

    - Alt 2-1b: N2=16 bits

Example :

      - Maximum 16 APs in APP

      - AP in APPs > 4 bits?

| Moderator | Latest proposal from online is in 3.2.7-rev1. |
| --- | --- |

## Proposal 3.2.7-rev1 (Identification of the PRACH subset mask)--fromOnline

For DCI-based adaptation for additional PRACH resources, the PRACH mask to identify the subset of the additional PRACH resources is select from the following options:

- Option 1-2:  Semi-static signalling of a PRACH mask index and a value of K

    - For K: one from up to four candidate values

    - Samsung, Apple, Huawei, China Telecom, Nokia, CATT, Sharp, OPPO, Lenovo (seriously unhappily), QC (conditionally), IDC, Xiaomi

| Mask index | Indication of association periods (AP) for subset of additional PRACH resources within every K association pattern periods (APP) |
| --- | --- |
| 0 | 1st half of the APs in K APPs |
| 1 | 2nd half of the APs in K APPs |
| 2 | 1st quarter of the APs in K APPs |
| 3 | 2nd quarter of the APs in K APPs |
| 4 | 3rd quarter of the APs in K APPs |
| 5 | 4th quarter of the APs in K APPs |
| 6 | [Every odd AP in K APPs] |
| 7 | [Every even AP in K APPs] |

- Option 2: Semi-static signalling of a bitmap of N2 bits and a value of K

  - For K: one from up to four candidate values

  - Option 2-2: Each bit in the bitmap corresponds to ceil((number of APs in K APP)/N2) APs

    - N2=4

  - LGE, Panasonic, Ericsson, MTK, ZTE, Google, Huawei, Fujitsu, DOCOMO, FW

| Company | Comment |
| --- | --- |
| Moderator | Latest proposal from online is in 3.2.7-rev1 above. Considering the online discussion and chair’s guidance, companies are invited to provide inputs to stabilize the contents of the table in 3.2.7-rev1. |
| LG | We believe that mask indices 0 and 2 in the table are sufficient to identify a subset of the additional PRACH resources. |
| Huawei, HiSilicon | From NES perspective, we also think one of 0 and 1, and one of 2-5 would be ok. |
| Apple2 | After offline discussions among companies, a potential wayforward is proposed as follows: Suggested Proposal: For DCI-based adaptation for additional PRACH resources, the PRACH mask to identify the subset of the additional PRACH resources is : - Option 1-2: Semi-static signalling of a PRACH mask index and a value of K - For K: one from up to four candidate values Mask index; Indication of association periods (AP) for subset of additional PRACH resources within every K association pattern periods (APP). 0; 1st half of the APs in K APPs. 1; 1st quarter of the APs in K APPs. 2; 1st eighth of the APs in K APPs. 3; 1st sixteenth of the APs in K APPs. [4; Every odd AP in K APPs]. [5; Every even AP in K APPs]. [6; Reserved]. [7; Reserved]. |

## --->Proposal 3.2.7-rev1 (Identification of the PRACH subset mask)-

For DCI-based adaptation for additional PRACH resources, the PRACH mask to identify the subset of the additional PRACH resources is selected from the following options:

- Option 1-2:  Semi-static signalling of a PRACH mask index and a value of K (number of association pattern periods)

    - For K: one from up to four candidate values {2,4,8,[1 or 16]}

| Mask index | Indication of association periods (AP) for subset of additional PRACH resources within every K association pattern periods (APP) |
| --- | --- |
| 0 | 1st half of the APs in K APPs |
| 1 | 1st quarter of the APs in K APPs |
| 2 | 1st eighth of the APs in K APPs |
| 3 | 1st sixteenth of the APs in K APPs |

- Option 2: Semi-static signalling of a bitmap of N2 bits and a value of K

  - For K: one from up to four candidate values

  - Option 2-2: Each bit in the bitmap corresponds to ceil((number of APs in K APP)/N2) APs

    - N2=4

| Moderator | Proposal is updated in 3.2.7-rev2. |
| --- | --- |

## Proposal 3.2.7-rev2 (Identification of the PRACH subset mask)

For DCI-based adaptation for additional PRACH resources, the PRACH mask to identify the subset of the additional PRACH resources is given by:

- Option 1-2:  Semi-static signalling of a PRACH mask index and a value of K (number of association pattern periods)

    - For K: one from up to four candidate values {2,4,8, [1 or 16]}

| Mask index | Indication of association periods (AP) for subset of additional PRACH resources within every K association pattern periods (APP) |
| --- | --- |
| 0 | 1st half of the APs in K APPs |
| 1 | 1st quarter of the APs in K APPs |
| 2 | 1st eighth of the APs in K APPs |
| 3 | 1st sixteenth of the APs in K APPs |

PDCCH order

Several companies provided view on which bit to use with the PDCCH order DCI for the new bit field in PDCCH order. Companies indicated that there are enough ‘reserved bits’ in the PDCCH order DCI 1_0 scrambled with C_RNTI that are not repurposed for other functionalities.

- One of the remaining ‘reserved bits’ in DCI 1_0 scrambled by C-RNTI

  - Panasonic, Lenovo, Honor, ZTE, Qualcomm (CBRA), Ericsson, Mediatek, NEC

- Reuse PRACH mask index field or SSB index field which are reserved under CBRA.

  - Qualcomm (for CBRA)

- Configured by higher layers

  - Ericsson

## Proposal 3.3.1 (Bit-field in PDCCH order)

For adaptation of PRACH in time-domain, for a connected mode UE,

- One of the reserved bits of PDCCH order (DCI 1_0 with C-RNTI) is used for the new DCI field that indicates the availability of additional PRACH resources.

- Presence of the DCI field is configured by new RRC parameter

| Company | Comment |
| --- | --- |
| ZTE, Sanechips | Fine with FL’s proposal. |
| OPPO | it is not clear why new RRC parameter is needed. The presence of the DCI field can be associated with the configuration of additional PRACH configuration. This way is more aligned with proposal 3.3.2 |
| CATT | OK |
| Panasonic | Same impression as OPPO |
| NEC | Support |
| Fujitsu | Support |
| LG | Support |
| DCM | Support |
| Qualcomm | The PDCCH ordered RACH can be CFRA if RACH-ConfigDedicated is configured or CBRA (if cfra field in RACH-ConfigDedicated is not configured). If it is CBRA, the PRACH mask index field or SSB index field are reserved. We believe that the proposal should cover both CBRA and CFRA. Suggested update For adaptation of PRACH in time-domain, for a connected mode UE, - One bit of the reserved bits (for CFRA)/fields (for CBRA) of PDCCH order (DCI 1_0 with C-RNTI) is used for the new DCI field that indicates the availability of additional PRACH resources. - Presence of the DCI field is configured by new RRC parameter |
| Tejas | Support 1st bullet. |
| Moderator | Closed. |

Some companies (ETRI, NEC, etc) had proposals on UE behaviour with respect to the new bit field.

## Proposal 3.3.2 (UE behavior for PDCCH order)

For adaptation of PRACH in time-domain, for a connected mode UE, for the new DCI field (in DCI 1_0 with C-RNTI) that indicates the availability of additional PRACH resources

- when the new DCI field indicates that the additional PRACH resources are available,

  - the UE selects the additional PRACH resources for PRACH transmission according to the PDCCH order.

- otherwise,

  - the UE selects the legacy PRACH resources for PRACH transmission according to the PDCCH order.

| Company | Comment |
| --- | --- |
| ZTE, Sanechips | For the PDCCH order DCI, we think the first thing need to clarify is that which type of additional PRACH resources the PDCCH order DCI can adapt. |
| OPPO | support |
| CATT | Support |
| Xiaomi | It is obvious that UE will use legacy RO if gNB doesn’t indicate additional RO. |
| Sharp | Support. In PDCCH order case, 1-bit indication in DCI should indicate which of the legacy PRACH resources and the additional PRACH resource are allocated to the UE. |
| NEC | Support |
| Fujitsu | According to the RAN2 agreement reached in the last RAN2 meeting shown in below, the legacy mechanism is reused for selecting PRACH resources. Agreements on RACH adaptation @RAN2#129 6. Will follow legacy mechanism regarding how to select RACH resource. Similarly, for the PDCCH order indication, how the UE selects PRACH resources can be left to RAN2 discussion. |
| LG | We think that the UE may select the closest RO (legacy RO or additional RO) to transmit PRACH after receiving the DCI. In another way, the UE may select the RO to transmit PRACH depending on the PRACH configuration index for configuring additional PRACH resources. For example, the UE can be configured to select the closest RO if the additional ROs has the same PRACH configuration index as the legacy RO, and otherwise (i.e., different PRACH configuration index) the UE will prioritize select the additional RO |
| CEWIT | Support |
| Qualcomm | We don’t support the proposal. It should be up to UE on select additional RO or legacy RO (similar principle as using paging DCI) as long as the timeline is satisfied. In other word, we do not prefer to have two separate UE behaviours on selection RO for paging DCI and PDCCH-order. |
| Tejas | Similar view as LG. Suggested Proposal: For adaptation of PRACH in time-domain, for a connected mode UE, for the new DCI field (in DCI 1_0 with C-RNTI) that indicates the availability of additional PRACH resources - when the new DCI field indicates that the additional PRACH resources are available, - the UE selects the valid additional or legacy PRACH resources for PRACH transmission according to the PDCCH order. - otherwise, - the UE selects the legacy PRACH resources for PRACH transmission according to the PDCCH order. |
| Samsung | So it seems the proposals says the 1 bit is there, UE can only use the additional RACH. We think that’s not the purpose of the design, I think the UE should be able to use the next available ROs no matter it’s legacy or additional. We already have separate mapping, it means, UE can find the corresponding CFRA RO in each RACH configuration. |
| Moderator | It seems several companies do not see need for this proposal. |

Additional PRACH resources for RACH-ConfigDedicated

Some companies provided inputs on whether additional PRACH resources for RACH-ConfigDedicated is supported or not.

- Yes

  - Qualcomm, Fujitsu, Samsung, ?

- No

  - DoCoMo, KT, Ericsson, ?

## Discussion point 3.3.3 (Additional PRACH resources for RACH-ConfigDedicated)

Please provide your view on whether additional PRACH resources can also be configured via UE-dedicated configuration defined by RACH-ConfigDedicated associated with a PDCCH order DCI. Please also provide any additional details such as whether or not a separate PRACH mask index is introduced for DCI 1_0 triggering CFRA (if supported) for the additional PRACH resources.

| Company | Comment |
| --- | --- |
| ZTE, Sanechips | In our views, PDCCH order DCI is only applicable for the PRACH resources configured by RACH-ConfigDeicated. For the separate PRACH mask index, we think it is not necessary to introduce a separate PRACH mask index. The current PRACH mask index can be also workable for additional PRACH resources. |
| OPPO | For DCI 1-0 triggering CFRA, do not support separate PRACH mask index. The PDCCH order DCI can be limited to additional RO only if the UE supports additional RO. |
| CATT | Since PDCCH order can be used to initial a CBRA or a CFRA, and the RACH-ConfigDedicated can be only used for CFRA. I think the additional RA resource of PDCCH order should be only configured with rach-ConfigCommon. |
| Xiaomi | We are not the proponent to have UE dedicated configuration for PRACH resources. I remove xiaomi under Yes. |
| Sharp | A separate PRACH mask index is not needed. The combination of the additional 1-bit indication and legacy PRACH mask index (and SSB index) indicates a specific RO among legacy PRACH resources and additional PRACH resources. |
| Apple | Prefer not to support additional PRACH resources for RACH-ConfigDedicated. |
| LG | A separate PRACH mask index for CFRA for the additional PRACH resources can be considered for semi-static signaling. |
| DCM | We do not support on providing additional PRACH resources for dedicated resources. For dedicated resources, UE is configured with dedicated ROs and dedicated RACH preamble. No resource shortage will occur. Allowing additional PRACH resource for CFRA will cause additional behaviors of how to deal with resource overlapping between legacy resources and additional PRACH resources for CFRA and resource overlapping between additional PRACH resources, which increase specification complexity. |
| Samsung | We did not support RACH dedicated, which is used in the handover. I removed our name for that. |
| Moderator | It seems several companies do not support additional PRACH resources for RACH-ConfigDedicated. |

RA-RNTI issue

Companies provided input on how to handle RA-RNTI ambiguity issue when the legacy RO and additional RO overlap in time domain. It seems most companies that provided input seem to like Option 2.

- Option 1: Modify RA-RNTI determination equation RACH based on additional RO

  - Qualcomm, Apple, ETRI, Ericsson

- Option 2: Reindex f_id in the RA-RNTI calculation for RACH based on additional RO when it is overlapped with legacy RO

  - Qualcomm, Samsung, Apple, ETRI, Ericsson

- Option 3: Leave it to implementation

  - Ericsson (since it occurs only for Case 2)

- Option 4: Leave it up to RAN2

  - Fujitsu

## Discussion point 3.4.1 (RA-RNTI issue)

Please provide your view on whether you would be OK with the direction of Option 2 above (and please include any further details) to handle the RA-RNTI ambiguity issue.

| Company | Comment |
| --- | --- |
| CMCC | During Rel-15~Rel-18, RAN1 introduced a dozen of RACH features that can configure separate RO with the basic RO. For all this features, RA-RNTI ambiguity issue may occur. Meanwhile, there is no enhancement for such issue which means it can be avoided/solved via implementation. For Rel-19 new RACH feature, we do not see the obvious benefit on top of reuse the legacy. We prefer to handle this issue by implementation. |
| OPPO | R2 is also discussing this issue, R1 can wait for R2 decision. |
| CATT | It’s better to leave to RAN2. |
| Xiaomi | Share same view as CMCC. |
| NEC | Leave this to RAN2 discussion |
| Apple | Open to discuss. Since the additional PRACH resource is configured in the same RACH-ConfigCommon as legacy which is different from legacy configuration. In this case, although a legacy UE and a new UE are sending Msg 1 on different ROs, they use the same RA-RNTI to monitor RAR which increase the collision probability and also increase UE power for detecting RAR not intended for itself. |
| Fujitsu | Although we think this seems as a RAN2 issue, we are also open to the direction of option 2 if the majority view is that it should be addressed in RAN1. |
| LG | Support Option 2, which resolves RNTI ambiguity with minimal specification impact. |
| Samsung | Response to CMCC, indeed the separate RACH configuration happens a lots in past release, but in the past we did not ask UE to select RACH from two different RACH configuration. So gNB have lots of tools to distinguish, e.g., different RO location, different search space, different preamble at least. But now for NES additional RACH, UE can select from both during one attempt. Then the FDMed RO will be merged to contribute the calculation of RA-RNTI. If we don’t do anything, UE will have problem to choose the f_id for RA-RNTI calculation. Last meeting, we already limited the total FDMed RO is 8. So only thing we need to do, is to ask UE to indexing f_id from legacy RO to additional RO. To the proposal, I think the option 2 is not accurately captured the solution at least for us, we are not pursing new indexing, just continuous indexing with no impact to legacy RO. Option2: continuous indexing f_id in the RA-RNTI calculation for RACH from legacy RO to additional RO when they are overlapped in time domain. |
| Moderator | Companies can continue to provide further input given the comments above. |

2-step RACH

- 2-Step RA

  - Study/support

    - Nokia, Samsung (minimize spec impact), Xiaomi

  - Deprioritize

    - Fujitsu

## Discussion point 3.4.2 (2-step RACH)

Please provide your view on the following.

- Whether to configure additional PRACH resources for 2-step RACH or whether the additional RACH resources should be limited for 4-step RACH.

| Company | Comment |
| --- | --- |
| ZTE, Sanechips | We think it is not reasonable to exclude 2-step RACH. Similar as 4-step RACH, the additional 2-step RACH resources can be configured separately with legacy 2-step RACH resources. |
| OPPO | 2-step RACH can be supported |
| CATT | Open to discuss. |
| Apple | Prefer to limit for 4-step, or wait for RAN2. |
| LG | Prefer to focus on 4-step considering the remaining time. |

## Other aspects

Companies can raise any other aspects related to adaptation of PRACH here.

| Company | Comment |
| --- | --- |
| OPPO | We bring up a new issue in our Tdoc R1-2502263 for discussion. when network has triggered PRACH adaptation via DCI, the network normally does not signal the paging DCI again to trigger another PRACH adaptation during the validity time duration.Otherwise, it would result in an endless adaptation. If the UE camps during the validity time duration, the UE is not able to know the additional PRACH occasions are made available. |
| DCM | No additional PRACH resources should be allowed for 2-step RACH/CFRA/etc. |
| Nokia | With reference to the LS draft shared by FL after Tuesday’s online session. We suggest the following modification for the sentence added in Section 2 – Actions: RAN1 respectfully asks RAN2 to confirm the feasibility of the use of above bit and indicate whether any restriction (if any) exists on which bit can be used. |

# Adaptation of Paging

In last RAN1 meeting, below proposal was discussed.

Proposal 4.1.1 From RAN1 perspective, study based on following options for PEI bits (in DCI 2_7) handling in case of legacy and additional Rel-19 POs - Option 1: PEI bits for legacy POs and additional Rel-19 POs are always multiplexed in the same DCI 2_7. - Option 2: PEI bits for legacy POs and additional Rel-19 POs are not multiplexed in the same DCI 2_7. - Option 3: Option 1 and Option 2 are supported via gNB configuration - [Option 4: No additional handling is required]

RAN2 agreements related to PEI/PO as follows.

Agreements - Select option-b as baseline for R19 NES paging enhancement. - R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values) - Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration. - Legacy UE is not barred. - Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration. Agreements - Introduce value for Ns=8. FFS on 16. - Introduce value for N= T/32. FFS on T/64, T/128, T/256. Agreement - For N, values smaller than T/32 are not supported. - The maximum possible value for Ns is 8. - Introduce a separate PEI configuration. - Paging adaptations are configured semi-statically and updated via system information update notification. - A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

RAN2 has agreed to introduce a separate PEI configuration. From RAN1 perspective, one potential aspect is whether and how to handle PEI bits in DCI 2_7 in case of legacy and Rel-19 POs.

Note that according to RAN2 agreement “Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration”, and there would also be a separate PEI configuration. RAN2 has also agreed “allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration”. Given RAN2 has already agreed to separate configuration, it seems remaining aspect is whether anything special needs to be done from RAN1 perspective.

Some companies provided input on DCI aspects of Rel-19 PEI, including whether and how the PEI for legacy POs and PEI for Rel-19 POs are multiplexed (e.g. in same DCI or in different DCI). Below are some options proposed by companies.

- Are always multiplexed into same DCI 2_7 (for Rel-19 PEI, use both legacy paging field and spare bits in DCI 2_7)

  - Nokia (enough spare bits in DCI 2_7,..), Qualcomm (better for NES,..)

- Configurable i.e can be multiplexed into same or different DCI 2_7

  - ZTE, Ericsson (configure additional parameters for Rel-19 PEI to facilitate effficient multiplexing– position of Rel-19 paging field and position of TRS field in DCI 2_7)

- Are not multiplexed in same DCI 2_7

  - Vivo (not enough bits in DCI 2_7), LG (given separate Rel-19 PEI configuration per RAN2,..)

## Discussion Point 4.1.1

Please provide your view on above options proposed by companies for DCI aspects of Rel-19 PEI.

| Company | Comment |
| --- | --- |
| CMCC | RAN2 had already agreed to introduce separate PEI configuration for R19 NES UE. We see not multiplex in same DCI 2_7 is already adopted and no need to further discussed in RAN1. |
| OPPO | we think the simplest way is to invalidate the R19 PO or PEI when overlapping with legacy PO or PEI. |

## Other aspects

Companies can raise any other aspects related to adaptation of paging here.

| Company | Comment |
| --- | --- |
| CMCC | We suggest RAN1 to further discuss the followings: - The indication method for Paging adaptation, i.e., whether the existing procedure (e.g. SI update and RRC re-configuration) is enough or enhancement is needed, should to be considered and discussed in RAN1. - During the previous discussion on SSB adaptation and PRACH adaptation, companies reach a consensus that the adaptation should be triggered by gNB instead of UE. From this point of view, instead of designing separate indication signaling for each common signal, whether a unified design that can reduce the overhead for indication signaling can be discussed. |
| Nokia/NSB | It is more efficient from network energy saving perspective by utilizing a single PEI, where a single PEI with DCI format 2_7 can address both legacy POs and additional release 19 POs using a mixture of multiplexing and separate signalling. If two PEIs agreed to be applied, to minimize the network energy consumption, the second PEI for additional release 19 POs shall only be transmitted when needed. |

# Conclusion

Below is the list of agreements and conclusion from RAN1#120bis.

# Appendix A (Contributions)

| 1 | R1-2501715 | Discussion of the adaptation of common signal/channel transmissions | FUTUREWEI |
| --- | --- | --- | --- |
| 2 | R1-2501766 | Discussion on common signal channel for NES | ZTE Corporation, Sanechips |
| 3 | R1-2501774 | Adaptation of common signal/channel transmissions | Nokia, Nokia Shanghai Bell |
| 4 | R1-2501812 | Discussions on adaptation of common signal/channel transmissions | vivo |
| 5 | R1-2501874 | Discussion on adaptation of common signal/channel transmissions | Spreadtrum, UNISOC |
| 6 | R1-2501954 | Adaptation of Common Signals | Google |
| 7 | R1-2501998 | Discussion on adaptation of common signal/channel transmissions | CATT |
| 8 | R1-2502028 | Discussion on common signal/channel adaptation | China Telecom |
| 9 | R1-2502093 | Discussion on adaptation of common signal/channel transmissions | KT Corp. |
| 10 | R1-2502129 | Discussion on adaptation of common signal channel transmissions | Fujitsu |
| 11 | R1-2502166 | Discussion on adaptation of common signal/channel transmissions | CMCC |
| 12 | R1-2502192 | Discussion on adaptation of common signal/channel transmission | Panasonic |
| 13 | R1-2502200 | Discussion on adaptation of common signal/channel transmissions | NEC |
| 14 | R1-2502231 | On common channel/signal adaptation for eNES | Huawei, HiSilicon |
| 15 | R1-2502254 | Adaptation of common signals and channels | Lenovo |
| 16 | R1-2502263 | Discussion on adaptation of common signal/channel transmission | OPPO |
| 17 | R1-2502336 | Discussion on adaptation of common signal/channel transmissions | InterDigital, Inc. |
| 18 | R1-2502374 | Adaptation of common signal/channel transmissions | Samsung |
| 19 | R1-2502447 | Discussion on adaptation of common signal and channel transmissions | Xiaomi |
| 20 | R1-2502470 | Adaptation of common signal/channels | Tejas Network Limited |
| 21 | R1-2502480 | Adaptation of common signal/channel transmissions | LG Electronics |
| 22 | R1-2502515 | Adaptation of common signal/channel transmissions | ETRI |
| 23 | R1-2502544 | Discussion on adaptive transmission of common signal or common channel | Transsion Holdings |
| 24 | R1-2502614 | On adaptation of common signal/channel for NES enhancements | Apple |
| 25 | R1-2502659 | Adaptation of Common Signals and Channels for NES | Fraunhofer IIS, Fraunhofer HHI |
| 26 | R1-2502682 | Discussion on adaptation of common signal/channel transmissions | Sharp |
| 27 | R1-2502693 | Discussion on adaptation of common signal channel transmissions | HONOR |
| 28 | R1-2502710 | Adaptation of common signal/channel transmissions | MediaTek Inc. |
| 29 | R1-2502772 | Discussion on adaptation of common signal/channel transmissions | NTT DOCOMO, INC. |
| 30 | R1-2502797 | Discussion on common signal adaptation | KDDI Corporation |
| 31 | R1-2502804 | Adaptation of common signal/channel transmissions for NES | Ericsson |
| 32 | R1-2502845 | Adaptation of common channel transmissions | Qualcomm Incorporated |
| 33 | R1-2502919 | Discussion on adaptation of common signal and channel transmissions | CEWiT |

# Appendix B (WI objectives from WID in RP-250343)

# Appendix C (PRACH mask index proposals)

## Qualcomm (R1-2502845)

Proposal 10: Select one of the following options for semi-statically masking additional ROs:

- Option 1: Define the following masking table in which the association period and association pattern period are defined based on additional ROs only. Furthermore, the masking is only applicable when the number of association periods per association pattern period is more than one.

| Mask index | Association periods with additional valid ROs |
| --- | --- |
| 0 | The first half of the number of association periods in an association pattern period |
| 1 | The second half of the number of association periods in an association pattern period |
| 2 | Every even association period in an association pattern period |
| 3 | Every odd association period in an association pattern period |

- Option 2: Define a periodic time domain window in which valid additional PRACH resources can be used for PRACH transmission.

## Apple (R1-2502614)

Proposal 11: Consider using Table 3 to configure the PRACH mask in SIB1, where the single PRACH mask can be indicated from the chosen table, and value M can be the number of association periods within a pattern period, or can be explicitly signaled by the NW.

Table3 PRACH association period mask Example 3 with 3bits Mask index

| Mask index | PRACH association period mask |
| --- | --- |
| 0 | 1st half of M PRACH association periods |
| 1 | 2nd half of M PRACH association periods |
| 2 | Every odd PRACH association periods |
| 3 | Every even PRACH association periods |
| 4 | 1st quarter of M PRACH association periods |
| 5 | 2nd quarter of M PRACH association periods |
| 6 | 3rd quarter of M PRACH association periods |
| 7 | 4th quarter of M PRACH association periods |

## Nokia (R1-2501774)

Proposal-16: If the semi-static configuration of a 4-bit mask is supported, the following Rel-19 NES PRACH mask is supported:

| PRACH Mask Index/ msgA-SSB-SharedRO-MaskIndex/ ssb-SharedRO-MaskIndex | Association periods with additional valid ROs in a sequence of N association periods or in an association pattern period |
| --- | --- |
| 0 | All |
| 1 | mod(association period index, N)=0 |
| 2 | mod(association period index, N)=1 |
| 3 | mod(association period index, N)=2 |
| 4 | mod(association period index, N)=3 |
| 5 | mod(association period index, N)=4 |
| 6 | mod(association period index, N)=5 |
| 7 | mod(association period index, N)=6 |
| 8 | mod(association period index, N)=7 |
| 9 | Every even association period |
| 10 | Every odd association period |
| 11 | Reserved |
| 12 | Reserved |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

Proposal-17: If the semi-static configuration of a 3-bit mask is supported, the following Rel-19 NES PRACH mask is supported:

| PRACH Mask Index/ msgA-SSB-SharedRO-MaskIndex/ ssb-SharedRO-MaskIndex | Association periods with additional valid ROs in a sequence of N association periods or in an association pattern period |
| --- | --- |
| 0 | The first half of N association periods |
| 1 | The second half of N association periods |
| 2 | The first quarter of N association periods |
| 3 | The second quarter of N association periods |
| 4 | The third quarter of N association periods |
| 5 | The fourth quarter of N association periods |
| 6 | Every even association period |
| 7 | Every odd association period |

Proposal-18: If the semi-static configuration of a 2-bit mask is supported, the following Rel-19 NES PRACH mask is supported:

| PRACH Mask Index/ msgA-SSB-SharedRO-MaskIndex/ ssb-SharedRO-MaskIndex | Association periods with additional valid ROs in a sequence of N association periods or in an association pattern period |
| --- | --- |
| 0 | The first half association periods |
| 1 | The second half association periods |
| 2 | Every even association period |
| 3 | Every odd association period |

## Samsung(R1-2502374)

NES_PRACH resource mask indication:     choice

{

2bits   first {1, 2, 3, 4} NES association pattern periods after the starting of every legacy PRACH configuration period

3bits   first {1,2, … ,8} NES association pattern periods after the starting of every legacy PRACH configuration period

4bits   first {1,2, … ,16} NES association pattern periods after the starting of every legacy PRACH configuration period

}

# List of RAN1 agreements

## RAN1#116

Agreement

For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study

- Adaptation of SSB burst periodicity

- Adaptation based on two SSB configurations where up to two configurations can be active

- Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration

- Adapting the transmitted number of SSBs within a SSB burst

- Cell DTX for SSB adaptation

- Whether to support new SSB burst periodicity value(s)

- Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst)

  - New compact SSB burst(s)

  - Adapting the position of SSBs within a SSB burst

- Other mechanisms/combinations are not precluded

Agreement

For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study

- Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

- For the additional PRACH resources,

  - Adaptation of PRACH resource periodicity/PRACH occasion

  - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle

  - Adaptation based on extending cell DRX operation for PRACH

  - Concentrating ROs in time domain

- Other options are not precluded

Agreement

For adaptation of paging,

- Study further from RAN1 perspective, techniques for adaptation of paging occasions in time-domain and achievable network energy savings

- Note: Specification details for PO/PF determination and paging-related configuration/procedures to be handled by RAN2

Agreement

For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following:

- Applicability to UE in idle/inactive and/or connected mode

- Applicability to PCell and/or SCell(s)

Agreement

For the adaptation mechanisms of SSB in time-domain, study further following mechanisms:

- Adaptation mechanism indicated or configured by gNB without UE trigger

- Adaptation triggered by UE (if any)

FFS: Details of associated signaling/indication/configuration

Agreement

For the adaptation mechanisms of PRACH in time-domain

- Support at least PRACH adaptation provided by gNB without UE trigger

  - FFS: PRACH adaptation with UE trigger

  - Note: UE trigger means UE requests adaptation of PRACH

- Study at least the following,

  - Dynamic signaling and/or semi-static signaling of PRACH adaptation

  - Adaptation of PRACH transmission according to certain condition

  - Applicability to idle/inactive and/or connected mode UEs

  - Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 UEs)

## RAN1#116bis

Agreement

For indication of adaptation of SSB in time-domain,

- Support at least SSB adaptation provided by gNB without UE trigger

Agreement

For adaptation of PRACH in time-domain, support at least the following:

- Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

  - Configuration of additional PRACH resources is provided by semi-static signalling

    - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs

  - FFS: adaptation mechanism for additional PRACH resources

  - Note: No change to the existing PRACH configuration tables in 38.211

Agreement

For adaptation of PRACH in time-domain, support the following:

- SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any)

  - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs

  - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources

  - FFS: SSB-RO mapping for the additional PRACH resources

Agreement

Support adaptation mechanisms of PRACH in time-domain for following:

- UE in idle/inactive mode

- UE in connected mode

Agreement

Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s):

- For cell with both legacy UEs and Rel-19 NES-capable UEs

  - Rel-19 NES-capable UE’s PCell (Connected mode)

    - Study from the following options:

      - Option A1: adaptation for CD-SSB

      - Option A2: adaptation for SSB that is not CD-SSB

      - Option A3: adaptation for SSB not on sync raster

  - Rel-19 NES-capable UE’s SCell

    - Study from the following options:

      - Option B1: adaptation for CD-SSB

      - Option B2: adaptation for SSB that is not CD-SSB

      - Option B3: adaptation for SSB not on sync raster

  - FFS: Rel-19 NES-capable UE in idle/inactive mode

- Note: Impact to idle/inactive UEs shall be minimized

Agreement

For adaptation of PRACH in spatial domain,

- Study possibility of scenarios with non-uniform distribution of UEs in different beams

    - Note 6: Companies are encouraged to provide details on how they map UEs to different beams

- Study network energy savings gain achieved by non-uniform PRACH resource allocation across SSBs for scenarios with non-uniform distribution of UEs in different beams (if any),

    - Assume the following framework for network energy evaluation in FR1 and companies to report at least the below settings used in the evaluation/simulation

      - 20ms SSB period

      - 30kHz SCS, DDDSU TDD pattern

      - Setting A: SIB1 period (20ms/40ms/160ms)

      - Setting B1: Cell load (Empty/low/medium)

      - Setting B2: Traffic model

      - Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2

      - Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11

      - Setting E1: PRACH configurations

        - (legacy) PRACH resources according to the following PRACH configuration for all transmitted SSBs

      - Case A1-1: PRACH configuration #5 (20ms)

      - Case A1-2: PRACH configuration #17 (10ms)

      - Case A2-1: PRACH configuration #0 (160ms)

        - (time-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration for all transmitted SSBs

      - Case B1: PRACH configuration #17 (10ms)

      - Case B2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed time domain adaptation mechanism

        - (spatial-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration

      - Case C1: PRACH configuration #17 (10ms)

      - Case C2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed spatial domain adaptation mechanism, including details of non-uniform PRACH resource allocation across SSBs

      - Setting F: Cat 1/Cat 2 BS as defined in TR38.864

      - Setting G1: Number of SSB beams: 4,8 SSBs in a SSB burst with SSB pattern case C

      - Note 1: Baseline to compare is Case C1 vs Case B1/A1-1/A1-2, Case C2 vs Case B2/A2-1

      - Note 2: It is up to company to report the SSB-RO mapping ratio and FDMed RO number, etc

      - Note 3: Other PRACH configuration index with different PRACH format other than format 0 is not precluded

      - Note 4: Other SSB/SIB1/RACH periodicity/PRACH resource/configuration assumptions are not precluded (up to companies to report)

    - Other frameworks for network energy evaluation are not precluded, e.g. including for FR2

## RAN1#117

Agreement

For the study of adaptation of PRACH in spatial domain, following network energy savings gains were reported by sources based on the evaluation framework agreed in RAN1#116bis:

- Two sources showed following NES gain for TDD, CAT1 BS power model, case C1 vs A1-1, zero load [R1-2404409, R1-2405107]

  - -4% ~ -45%

- Seven sources showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1/A1-2, zero load [R1-2404225, R1-2404185, R1-2404334, R1-2404123, R1-2404562, R1-2405107, R1-2405163]

  - 0% ~ 31%

  - Note: Five sources assumed that case B1 has same PRACH resources as case A1-2. Remaining two sources evaluated only A1-2.

  - Note: Three sources showed NES gains 0% ~ 10% [R1-2404225, R1-2404185, R1-2404334]

- One source showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1, zero load [R1-2404464]

  - 1.0%~8.8%

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes.

- One source showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1, zero load [R1-2404626]

  - -48.41%~0%

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed following NES gain for TDD, CAT1 BS power model, for case C1 vs A1-2, zero load [R1-2404626]

  - 4.59%~38.04%

  - Note: For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- Four sources showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1/A1-2, zero load [R1-2404562, R1-2404225, R1-2403943, R1-2404626]

  - 0% ~ 3.5%

  - Note: Three sources assumed that case B1 has same PRACH resources as case A1-2. One source evaluated only A1-2.

- One source showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1, zero load [R1-2404464]

  - 0%~0.2%

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes

- One source showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1, zero load [R1-2404626]

  - -1.19%~0%

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- Two sources showed following NES gain for TDD, CAT1 or CAT2 BS power model, case C2 vs B2, zero load [R1-2403943, R1-2405107]

  - Less than 0.2%

- One source showed following NES gain for TDD, CAT1 BS power model, (C1 vs A1-2 with changed PRACH format), PRACH format A, 10ms PRACH periodicity, different loads [R1-2403980]

  - 13.7%/8.7%/4.9%/2.6% for zero/low/light/medium cell load

- One source showed following NES gain for TDD, CAT1 BS power model, (C1 vs B1 with changed PRACH format), PRACH format A, 10ms PRACH periodicity, different loads [R1-2403980]

  - 8.03%/5.1%/3.06%/1.74% for zero/low/light/medium cell load

- One source showed following NES gain for TDD, C1 vs B1/A1-2, different loads [R1-2404562]

  - 16%/4.78% for light/medium cell load for CAT1 BS power model

  - 0.65%/0.29% for light/medium cell load for CAT2 BS power model

- One source showed following NES gain for TDD, C1 vs B1, different loads [R1-2404626]

  - -18.57%~0%/-2.52%~0% for low /medium cell load for CAT1 BS power model

  - -0.81%~0%/-0.42%~0% for low /medium cell load for CAT2 BS power model

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed following NES gain for TDD, C1 vs A1-2, different loads [R1-2404626]

  - 3.67%~19.88%/2.29%~5.22% for low /medium cell load for CAT1 BS power model

  - 0.67%~1.75%/0.39%~0.91% for low /medium cell load for CAT2 BS power model

  - Note: For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed NES gain for FDD, C1 vs B1, zero load [R1-2404464]

  - 1.4%~7% for CAT1 BS power model

  - 0%~0.3% for CAT2 BS power model

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes

- One source showed NES gain for FR2, CAT1 BS power model, spatial domain adaptation of PRACH configuration index 75 vs a time domain adaptation of PRACH configuration index 75, zero load [R1-2405163]

  - 4%~7%

- Note 1: About possibility of scenarios with non-uniform distribution of UEs in different beams

  - Several companies indicated (and three companies showed data/analysis) that there can be scenarios with non-uniform distribution of UEs in different beams.

  - Several companies mentioned that for non-uniform UE distribution, it can be addressed by gNB implementation e.g. by adjusting SSB beamwidth, etc. Several companies also mentioned that it is not clear how gNB can predict the distribution of UEs in different beams, especially for Idle/Inactive UEs.

- Note 2: Most sources that showed the NES gains (if any) for adaptation of PRACH in spatial domain compared to A1-2/B1 observed that the gain would be due to reduction in the number of overall ROs in time domain in their evaluations. Most of these companies only accounted for ROs in time domain.

- Note 3: The evaluation results assumed the non-uniform distribution of UE is static during the evaluation time period.

Conclusion

There is no consensus in RAN1 on the support of PRACH adaptation in spatial domain

Agreement

For adaptation of PRACH in time-domain, support at least the following case(s)

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains

- FFS: whether additional conditions are needed to support the above cases

- FFS: Additional case whether full/partial overlap in both time and frequency is allowed

- Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources

Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Agreement

For adaptation of SSB in time-domain, Option 1 is supported

- Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s)

- Note: Using Option 2 to realize Option 1 is not precluded

  - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active]

    - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities

- FFS: Details including applicable scenarios

- FFS: Support of Cell DTX for connected mode UEs for SSB

Agreement

For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least:

- a PRACH configuration index

- FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources

Study further the following

- When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource,

  - Additional parameter(s) for determining the additional PRACH resources e.g.

    - Scaled/adjusted PRACH configuration period

    - Additional timing offset

    - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration

    - Muting/masking ROs

- When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource

  - Additional mechanisms (if any) for determining the additional PRACH resources e.g.

    - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources)

- Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

Agreement

For the adaptation mechanism for additional PRACH resources, study further the following:

- Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation

- Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

  - Strive to re-use existing DCI format(s)

- Option 3: Adaptation of PRACH transmission according to predefined condition(s)

  - FFS: details

- Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation

  - Strive to re-use existing DCI format(s)

- Option 5: Enhanced cell DRX

## RAN1#118

Agreement

For adaptation of PRACH in time-domain, select at least one from the following alternatives for configuration of the additional PRACH resources

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources and

  - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g.

    - Opt 1-1: Scaled/adjusted PRACH configuration period

    - Opt 1-2: Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration

    - Opt 1-3: Muting/masking ROs

    - Opt 1-4: additional timing offset(s)

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources,

  - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g.

    - Opt 2-1: Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources)

    - Opt 2-2: Additional timing offset(s)

- FFS: Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

- FFS: Additional frequency domain parameter(s) (e.g., freq. starting offset)

Agreement

Extend the RAN1#117 agreement on SSB-RO mapping rule for additional PRACH resources to Case 1

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

RAN1#117 Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Agreement

For SSB-RO mapping rule for additional PRACH resources for Case 2.

- Extend the RAN1#117 and RAN1#118 agreements on SSB-RO mapping

Agreement

For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE),

- At least DCI based adaptation is supported. No introduction of new DCI format.

Agreement

For adaptation mechanism(s) of SSB in time-domain,

- For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported

  - FFS: Adaptation for SSB that is not CD-SSB is supported (A2)

  - FFS: Adaptation for SSB not on sync raster is supported (A3)

- For Rel-19 NES-capable UE’s SCell

  - Adaptation of SSB configured for the SCell is supported for the following cases

    - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell

    - Adaptation for SSB that is not CD-SSB on sync raster (B2’)

    - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Agreement

For DCI-based adaptation for additional PRACH resources,

- Select from the following DCI format(s) to carry the adaptation indication.

  - DCI format 1_0

  - DCI format 2_7

  - DCI format 2_9

- FFS: existing (P-RNTI, SI-RNTI, CellDTRX-RNTI, PEI-RNTI, C-RNTI) or new RNTI used for detecting the DCI format

Agreement

For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options

- Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period

- Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period

- Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

- Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated.

  - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available.

  - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period

  - FFS: details

## RAN1#118bis

Agreement

For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources.

Agreement

For adaptation of PRACH in time-domain, support both of the following

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources

- FFS: Additional details

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

Agreement

For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different

- FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement).

- Discuss further following options for signaling

- Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources.

  - msg1-FrequencyStart at least for 4-step RACH

  - FFS: other applicable legacy frequency domain parameter(s)

- Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources

  - Note: Offset to legacy frequency domain parameter(s) is a new parameter

  - FFS: applicable legacy frequency domain parameter(s)

Conclusion

There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

Agreement

For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior).

- No spec impact

Agreement

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

- For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

## RAN1#119

Agreement

Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?):

- RAN1 agreed that at least SSB burst periodicity is adapted.

- There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options.

Agreement

Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?):

- The frequency location is same before and after SSB adaptation.

Agreement

Reply to Q3(What is the spatial relation before and after SSB adaptation?):

- There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation.

- At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported.

Further update to be made based on RAN1#119 progress if any.

Agreement

At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH.

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives:

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: Maximum number of subsets of the additional PRACH resources= [2 or 3 or 4 or 16]

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

    - Alt 2-6: Network configured time period

Conclusion

There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

Working Assumption

For DCI-based adaptation for additional PRACH resources,

- Select from the following options for carrying the adaptation indication in DCI format 1_0 with P-RNTI

- Option 1: Use reserved bits in the DCI format

  - FFS: relation (if any) to TRS availability bits / short message indicator in the DCI format

- Option 2: Use Bits 5-8 within the Short Message (from upper layers)

  - Note: Availability should be confirmed by checking with RAN2.

- Option 3: Use bits available for both Option 1 and Option 2

FFS: Payload size for adaptation for additional PRACH resources

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives:

- Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - [DCI payload size = 1 bit]

  - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources

  - FFS: details

Agreement

Confirm the following working assumption from RAN1#118bis.

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

## RAN1#120

Agreement

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping

  - Note: the overlapped RO for legacy resource is not impacted

  - FFS: Clarification on configuration of legacy ROs

Conclusion

There is no RAN1 consensus to support the following in Rel-19

- New SSB burst periodicity values other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

- New UE trigger

- Adapting the transmitted number of SSBs within a SSB burst

- Adaptation of the time domain positions of SSBs within a burst

Conclusion

There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for SCell

Agreement

For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell

- UE is configured with SSB burst periodicity using legacy signalling for the SCell

- UE is configured with X additional SSB burst periodicity for the SCell

  - FFS: Value of X

- SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity

  - FFS: Whether there is specification impact

  - Note: This does not impact the discussion on OD-SSB

- For switching the periodicity, down-select between

  - (MAC-CE)

    - MAC-CE details for SSB burst periodicity adaptation is up to RAN2.

  - (DCI)

    - Alt 1-3: DCI based signalling is used

Agreement

For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH.

- FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s)

- FFS: Details on how to reuse existing bit for the 1-bit indication

Agreement

For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration

- FFS: Validity time duration for availability is configured by higher layer signaling or predefined

- FFS: Location of the reference point defined in the specification

- FFS: Value/granularity of the validity time duration.

- FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources

Agreement

For DCI-based adaptation for additional PRACH resources, support optional semi-static signalling of a single PRACH mask to identify the subset of the additional PRACH resources

- The mask is applicable at unit of

  - Alt 1: PRACH association period

  - Alt 2: PRACH association pattern period

  - Alt 3: SFN level

- The PRACH association period is determined based on valid additional ROs only.

- The mask is applied after valid RO determination and SSB-RO mapping.

- Note: The existing behaviour in TS 38.213 "An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec." is not impacted due to application of the mask.

- This is applicable at least for adaptation for DCI 1_0 with P-RNTI

- The DCI does not indicate PRACH mask selection

- FFS: how the mask is identified

  - Option 1: The PRACH mask is from a PRACH mask table

    - Pre-defined table with N=[4 or 8 or 16] rows

    - The semi-static signalling indicates a PRACH mask index

  - Option 2: The PRACH mask is based on configuration parameters e.g. bitmap at SFN-level, periodic time domain window, …

Agreement

- Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported

  - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs)

  - FFS: When there is no configuration of Msg1-FDM

- Separate configuration of number of SSB per RO is supported

Agreement

Study the following options for the reference point (for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI in a PF) for RRC idle/inactive mode UE and RRC connected mode UE,

- Option 1: SFN of the first PF from the next I-DRX cycle

- Option 2: SFN of the first PF from the current I-DRX cycle

- Option 3: From the first frame of the first PRACH association period after UE receives the DCI

- Option 4: From the first frame of the current SI modification period

- Option 5: From the first frame of the next SI modification period

Agreement

For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell

- Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI]

- FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1)

Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation