3GPP TSG RAN WG1 #120bis			R1-250XXXX

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.13

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 9.13 LTE-based 5G Broadcast Phase 2

Document for:	Discussion, Decision

## LTE-based 5G Broadcast Phase 2

Please refer to RP-250794 for detailed scope of the WI for LTE-based 5G Broadcast Phase 2.

Rapporteur to provide initial input on higher layer signalling under agenda item 9.13. For input on higher layer signalling from any other source, please include it as part of your tdoc to relevant sub-agenda items.

[120bis-R19-Broadcast] Email discussion on Rel-19 LTE-bad 5G Broadcast Phase 2 – Jordi (EBU)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2502548	Higher layer parameters for Rel-19 LTE-based 5G Broadcast after RAN1#120	EBU

### Time-frequency interleaver design for MBMS dedicated cells

R1-2501912	Discussion on time-frequency interleaver design for LTE-based 5G Broadcast	ZTE Corporation, Sanechips

R1-2502209	On time-frequency interleavers for LTE-based 5G Broadcast	Huawei, HiSilicon

R1-2502390	Views on LTE-based 5G broadcast	Samsung

R1-2502546	Further details on open issues regarding TFI for LTE-based 5G Broadcast	EBU

R1-2502582	Design of Time-frequency interleaver for 5G Broadcast	Shanghai Jiao Tong University

R1-2502861	Time and Frequency Interleaving for 5G Broadcast	Qualcomm Incorporated

R1-2503001	FL summary#1 on Rel-19 LTE-based 5G Broadcast Phase 2	Moderator (EBU)

Agreement

Support at least the following values of time interleaver depth:

- Values M={4,8,16,32} and N={1,2,4,8,16} are supported.

  - FFS: UE behaviour in case values of M and N exceed UE capabilities in terms of soft buffer size and maximum TBS supportable in a TTI

- FFS: larger values, including M=64, N={32,64}.

Proposal

UE to discard the interleaved MTCH when the number of the transmitted TBs of the interleaved MTCH exceed UE capability.

- Note: no RAN1 specification impact

Proposal

RAN1 to minimize resource wastage due to the potential mismatch between MSI periodicity and the (MxN) pattern:

- Adopt an extended set of configurable periodicities for MCH scheduling period for R19 PMCHs that includes the intermediate values among the periodicities defined for the legacy configurations.

- FFS: other solutions

Proposal

In order to reduce the resource wastage due to the potential mismatch between MSI periodicity and the (MxN) pattern:

- Adopt an extended set of configurable periodicities for MCH scheduling period for R19 PMCHs that includes the intermediate values among the periodicities defined for the legacy configurations.

R1-2503002	FL summary#2 on Rel-19 LTE-based 5G Broadcast Phase 2	Moderator (EBU)

R1-2503003	FL summary#3 on Rel-19 LTE-based 5G Broadcast Phase 2	Moderator (EBU)