3GPP TSG RAN WG1 #120bis	                                   R1-2502959

Wuhan, China, April 7th – 11th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.1

Agenda Item:		9.15.1

Document for: 	Endorsement

- 9.15.1	UE features for AI/ML for NR Air Interface

For RAN1#120bis, in addition to individual company/organization/university tdocs, the rapporteurs are to provide initial input.

R1-2502738	Summary of UE features for AI/ML for NR Air Interface	Moderator (AT&T)

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agree

| 58. NR_AIML_air | 58-1-1 | Increased number of reported RSs for beam management [for NW-sided model inference] | 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4 FFS: whether/how to have separate FG for some of above components or how to merge components | FFS | yes | n/a | Increased number of reported beams for beam management [for NW-sided model inference] is not supported | FFS | FFS | FFS | FFS | Component 3 candidate values: {FFS} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model [2. Supported mapping pattern between set B and set A] [3. Maximum number of inference report(s) configured for BM-Case1] [4. Maximum number of inference report(s) activated for BM-Case1] [5. Maximum number of inference report(s) triggered for BM-Case1] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B FFS: RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] FFS: component 7 or component 7a+7b or 7+7a+7b [8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 1 with UE side model] [11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both}] [12. Supported maximum number of predicted beams in each reporting instance] FFS: whether/how to report the supported maximum total number of CSI reports across different AI/ML based use-cases FFS: whether some of components will be reported dynamically instead of as capability FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection | FFS | yes | n/a | UE-sided beam prediction for BM Case 1 [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU/AIMLPU related information FFS: candidate values for component 12 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-3 | UE-side beam prediction for BM Case1 with predicted RSRP [for inference] | - Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 [for inference] [2. Supported maximum number of predicted beams in each reporting instance] FFS: other components | 58-1-2 | yes | n/a | UE-side beam prediction for BM Case 1 with predicted RSRP [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: candidate values for component 2 | Optional with capability signalling |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 | 1. Support of beam prediction for BM-Case2 with UE-side model 2. The Support of associated ID(s) 2. Supported mapping pattern between set B and set A 3. Maximum number of resources in Set A for beam prediction Case 2 4. Maximum number of resources in Set B for measurement for Case 2 3. RS type for Set B 5. The supported combinations of the number of resources for Set B and the number of resources for Set A [6. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS] [7. Supported inference report types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS] [8. Supported options for performance monitoring for beam case 2 with UE side model] 9. Maximum number of inference report(s) configured for BM-Case2 10. Maximum number of inference report(s) activated for BM-Case2 11. Maximum number of inference report(s) triggered for BM-Case2 12. Maximum number of past time instance(s) as model input 13. Maximum value of K for reporting beam information on predicted Top K beams 14. Maximum number of supported values of predicted time instance(s) N 15. Supported values of the time gap between two consecutive future time instances for prediction. 16. Supported values of the time gap between two consecutive measurement time instances for measurement. 17. Maximum number of measurements for prediction (or measurement window). | FFS | yes | n/a | UE-side beam prediction for BM-Case2 is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU related information | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-5 | UE-side beam prediction for BM-Case2 with predicted L1-RSRP | 1. Support of beam prediction, reporting of predicted beams and predicted L1-RSRP, for BM-Case2 (spatial and time domain beam prediction) 2. Maximum number of future time instance (or prediction window). 3. Maximum number of reported predicted beams and predicted L1-RSRP (K) | 58-1-4 | yes | n/a | UE-side beam prediction for BM-Case2 is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_air | 58-1-6 | UE assisted performance monitoring for UE-sided model beam prediction | 1. Support of [at least option 1] performance monitoring with [FFS: metric] of AI/ML model for beam prediction. 2. Maximum total number of the configured CSI-RS resources for monitoring RS resource set 3. Maximum number of periodic CSI report setting per BWP for monitoring reporting 4. Maximum number of aperiodic CSI report setting per BWP for monitoring reporting 5. Maximum number of semi-persistent CSI report setting per BWP for monitoring reporting 6. Supported values for monitoring window (number of monitoring instances) | FFS | yes | n/a | UE assisted performance monitoring for UE-sided model beam prediction is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements FFS: separate rows/FGs for case 1 and case 2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_air | 58-1-7 | Data collection for UE-sided model in beam prediction for case 1 | 1. Support of data collection for UE-sided model for case 1 2. Supported BM-Case1 sub-use cases: {‘Set B subset of Set A’,’Set B not a subset of Set A’} 2. Support of associated ID(s) 3. Support of SS/PBCH block and CSI-RS based RSRP measurements for measurement RS resource set (Set B) for data collection 4. Support of CSI-RS based RSRP measurements for prediction RS resource set (Set A) for data collection 5. Maximum total number of the configured CSI-RS resources for measurement RS resource set (Set B) 6. Maximum total number of the configured CSI-RS resources or SS/PBCH blocks for measurement RS resource set (Set B) 7. Maximum total number of the configured CSI-RS resources for Prediction RS resource set (Set A) 8. Maximum number of CSI report setting per BWP for data collection | FFS | yes | n/a | Data collection for UE-sided model in beam prediction is not supported for case 1 | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements FFS: separate rows/FGs for case 1 and case 2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-7-1 | Data collection for UE-sided model in beam prediction for case 2 | 1. Support of data collection for UE-sided model for case 2 2. Supported BM-Case2 sub-use case: {‘Set B equal to Set A’,‘Set B subset of Set A’,’Set B not a subset of Set A’} 3. Support of SS/PBCH block and CSI-RS based RSRP measurements for measurement RS resource set (Set B) for data collection | FFS | yes | n/a | Data collection for UE-sided model in beam prediction is not supported for case 2 | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements FFS: separate rows/FGs for case 1 and case 2 | Optional with capability signalling |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-1 | AI/ML Fingerprint based positioning for UE-side model | 1. Support of AI/ML Fingerprint based positioning Case 1 2. Support of inference input based on DL PRS - Supported PRS resource for Case 1 - DL PRS Resources for Case-1 - DL PRS Resources for Case-1 on a band - DL PRS Resources for Case-1 on a band combination - Supported PRS resource processing for Case 1 - Supported PRS QCL processing for Case 1 [3. Support consistency checking between training and inference by using explicit assistance data] [4. Support consistency checking between training and inference by Associated ID(s)] [5. Support consistency checking between training and inference by new semi-explicit assistance data] 6. Support of UE-based performance monitoring for AI/ML based positioning Case 1 6. Support reporting of UE capability to perform AI/ML positioning Case 1 [7. Support for applicability determination of the AI/ML capability based on the network side conditions] [9. Maximum number of supported TRPs] [10. Maximum number of inference configurations that UE can be configured - Support periodical reporting for Case 1 inference - Support ten-milliseconds response time for Case 1 inference - Support scheduled location requests for Case 1 inference - supported minimum millisecond periodic reporting interval for location information in Case 1 inference - supported multiple location estimate instances in a single measurement report for Case 1 inference - supported geographical location coordinate types for reporting in Case 1 inference] 11. Support of applicable functionality reporting | FFS | N/A | N/A | AI/ML Fingerprint based positioning Case 1 is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-2-2 | Performance monitoring for AI/ML based positioning for UE-side model Case 1 | 1. Support of performance monitoring for AI/ML based positioning Case 1 2. Support of receiving data collection assistance from the LMF 3. Supported number of monitoring instances | FFS | N/A | N/A | Performance monitoring for AI/ML based positioning for UE-side model Case 1 is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 58. NR_AIML_Air | 58-2-3 | Data collection for AI/ML based positioning for UE-side model Case 1 | 1. Support of data collection for AI/ML based positioning for UE-side model 2. Support of Associated ID 3. Support of reporting UTC time with SFN time info as time stamp of ground truth label received from LMF 4. Support of collect explicit assistance data for consistency checking 5. Support of semi-explicit assistance data for consistency checking 6. Support of label-based performance monitoring of AI/ML positioning Case 1 7. Support of label-free performance monitoring of AI/ML positioning Case 1 | FFS | N/A | N/A | Data collection of AI/ML based positioning Case 1 is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-3-1 | AI/ML based CSI prediction for UE-sided model when N4=1 | 1. Support of AI/ML based CSI prediction 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 5. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 6. Support parameter combinations with L=2,4 7. Support for rank = 1,2 [8. Value for CPU occupation, when P/SP-CSI-RS is configured for CMR] [9. Value for CPU occupation, when A-CSI-RS is configured for CMR] 10. Support for the size of DD-basis, N4=1 [11. Scaling factor for active resource counting Kp] [12. Supported types of performance monitoring] 12. Support of CSI prediction at UE-side 13. Supported values of the maximum number of resources for measurement 14. Supported values of the maximum number of future time instance | FFS | yes | n/a | CSI prediction for N4=1 is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-3-2 | AI/ML based CSI prediction for UE-sided model when N4>1 | 1. Support of CSI prediction 2. Supported values of the number of future time instance. 3. Supported values of the maximum number of observation number 4. Support for reporting predicted PMI with N4>1 5. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 6. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 7. Support for the size of DD-basis, N4>1 8. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously 9. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 10. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 11. Supported values of the maximum number of resources for measurement 12. Supported values of the maximum number of future time instance | FFS | yes | n/a | CSI prediction for N4>1 is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| X. NR_AIML_Air | X-1-7 | Active CSI-RS resources and ports for mixed R16 based doppler codebook via AIML-based CSI prediction with other codebooks in any slot | 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-9-1, 16-3a, 2-36, 2-40, 2-41, 23-9-2, 23-9-4, 40-3-2-1, 40-3-2-1a, 40-3-2-2, X-1-1, X-1-1a, X-1-2 |  |  |  | Per band and Per BC |  |  |  | Component 1 candidate values: Codebook 1 = {Rel-16 based doppler codebook via AIML-based CSI prediction N4=1 R=1, Rel-16 based doppler codebook via AIML-based CSI prediction N4=1, R=2, Rel-16 based doppler codebook via AIML-based CSI prediction N4>1, R=1} Codebook 2 = {Type I SP, R16 eType II R=1, R16 eType II R=2} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} NOTE 1: if a UE reports one or more codebook combinations in X-1-7, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations. NOTE 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-9-5, X-1-7 and per-codebook capability 2-36/40/41, 16-3a, and 23-9-1/23-9-2/23-9-4. NOTE 3: Up to 4 combinations for component 1. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2501829	Discussion on UE features for AI/ML for NR Air Interface	vivo

R1-2501920	Discussion on UE features for AI/ML for NR Air Interface	ZTE Corporation, Sanechips

R1-2501979	Discussion on UE features for AI/ML for NR Air Interface	CATT, CICTCI

R1-2502047	Views on UE features for AI/ML for NR Air Interface	Ofinno

R1-2502102	Discussions on UE features for AI/ML for NR Air Interface	LG Electronics

R1-2502133	Views on UE features for AI/ML for NR Air Interface	Nokia

R1-2502178	Discussion on UE features for AI/ML for NR air Interface	CMCC

R1-2502246	UE features for AI/ML for NR air interface	Huawei, HiSilicon

R1-2502292	UE features for AIML for NR air interface	OPPO

R1-2502391	UE features for AI/ML for NR Air Interface	Samsung

R1-2502459	Discussion on UE features for AI/ML for NR Air Interface	Xiaomi

R1-2502639	View on UE features for R19 AI/ML for NR Air Interface	Apple

R1-2502661	UE Features for Rel-19 AI/ML for NR Air Interface	Ericsson

R1-2502785	Discussion on UE features for AI/ML for NR Air Interface	NTT DOCOMO, INC.

R1-2502863	Rapporteur's view on Rel-19 UE features for AI/ML air interface	Qualcomm Incorporated

R1-2502864	UE features for AI/ML for NR air interface	Qualcomm Incorporated