3GPP TSG RAN WG1 #121		                                                      R1-2503383

St Julian’s, Malta, May 19th – 23rd, 2025

Source: 	vivo

Title:	Remaining issues on enhancement of multi-cell PUSCH/PDSCH scheduling with a single DCI

Agenda Item:	9.12.1

Document for:	Discussion and Decision

- Introduction

In the RAN#105 meeting, a new WID for multi-carrier enhancements for NR Phase 3 is approved [1]. The objectives of the Rel-19 MC (multi-carrier) work item are shown as follows. In this contribution, the remaining issues on the multi-cell PUSCH/PDSCH scheduling with a single DCI are provided.

Specify the support of the following for multi-cell PUSCH/PDSCH scheduling with a single DCI [RAN1] - Different SCS/carrier type among co-scheduled cells by the single DCI. - One or multiple PUSCHs/PDSCHs per scheduled cell by the single DCI. - The maximum number of PUSCHs/PDSCHs per scheduled cell is [4 or 8]. - Note: Type-1 HARQ-ACK codebook is not enhanced for Rel-19 multi-cell scheduling. - Note: The maximum number of sub-codebooks for Type-2 HARQ-ACK codebook is not increased for Rel-19 multi-cell scheduling. - Note: UE does not expect to be configured with both single-cell multi-PUSCH/PDSCH scheduling and multi-cell multi-PUSCH/PDSCH scheduling on the same or different cells within a same PUCCH group. - Note: No new DCI format is introduced.

- Discussion

  - Type 2 HARQ-ACK codebook

In RAN1 120bis meeting, the following agreement is agreed.

Agreement (RAN1#120bis) - For a DCI format 1_3 with fields repurposed for SCell dormancy indication and scheduling one or more PDSCHs, if TDRA field indicates multiple SLIVs for the serving cell with smallest serving cell index with invalid FDRA and nrofHARQ-BundlingGroups is not provided for the serving cell, the HARQ-ACK information bit for SCell dormancy indication is ACK for the first SLIV and followed by NACK bits for the remaining SLIVs. - For a DCI format 1_3 with fields repurposed for SCell dormancy indication and scheduling one or more PDSCHs, if TDRA field indicates multiple SLIVs for the serving cell with smallest serving cell index with invalid FDRA and nrofHARQ-BundlingGroups is provided for the serving cell, the HARQ-ACK information bit for SCell dormancy indication is ACK for the first TBG, and followed by NACK bits for the remaining TBGs, if any. - Note: Related working assumption made in RAN1#120 meeting does not need to be confirmed.

The RAN1’s common understanding of this agreement is that the HARQ-ACK codebook for SCell dormancy indication with a row of N SLIVs is constructed by 1-bit ACK and followed by (N-1) NACK bits for the remaining (N-1) SLIVs. However, during the CR drafting, it was observed an issue that the first SLIV may not the SLIV with the earliest starting time.

Draft CR for 38213 - the UE assumes that the UE receives one PDSCH, with first SLIV in the row of pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 for DCI format 1_3, on the serving cell associated with fields in DCI format 1_3 for SCell dormancy indication, as described in Clause 10.3, and that the PDSCH provides one transport block that the UE correctly decodes, if the UE is not provided nrofHARQ-BundlingGroups for the serving cell

Consequently, according to the current draft CR, in the above case, the HARQ-ACK feedback for SCell dormancy indication may be 1-bit ACK in the middle of the (N-1) NACK bits which is not aligned with the RAN1’s agreement part highlight above. Moreover, the current draft CR would result in different behaviors between TB-based and TBG-based CB, which would unnecessarily increase the maintenance effort and implementation complexity. So, we propose to clarify that “the first SLIV” in the agreement should be “the SLIV with the earliest starting time”. The corresponding draft CR is updated.

Proposal 1: “the first SLIV” in the RAN1#120bis agreement should be clarified as “the SLIV with the earliest starting time”.

Proposal 2: The TP1 for TS 38213 is adopted.

----------------------------------------------------Start of TP1 of TS 38213-------------------------------------------------

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

*** Unchanged parts are omitted ***

-	the second HARQ-ACK sub-codebook is associated with DCI format 1_3 that

-	schedules PDSCH receptions on more than one serving cells from the set of serving cells, or

-	schedules more than one PDSCH receptions on a serving cell c, from the set of serving cells, for which the UE is not provided nrofHARQ-BundlingGroups or is provided nrofHARQ-BundlingGroups with value NHARQ-ACK,cTBG,max>1, or

-	does not include a SCell dormancy indication field or the SCell dormancy indication field is reserved, indicates SCell dormancy, and schedules PDSCH receptions on one or more serving cells from the set of serving cells

-	in the above, and for the purpose of providing HARQ-ACK information corresponding to SCell dormancy indication by the DCI format 1_3

-	the UE assumes that the UE receives one PDSCH, with earliest starting time among one or more PDSCH receptions scheduled by first SLIV in the row of pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 for DCI format 1_3, on the serving cell associated with fields in DCI format 1_3 for SCell dormancy indication, as described in Clause 10.3, and that the PDSCH provides one transport block that the UE correctly decodes, if the UE is not provided nrofHARQ-BundlingGroups for the serving cell

-	the UE assumes that the UE receives one or more PDSCHs associated with a TBG with smallest index, or a PDSCH reception group with smallest index, among one or more PDSCH receptions scheduled by the DCI format 1_3, on the serving cell associated with fields in DCI format 1_3 for SCell dormancy indication, as described in Clause 10.3, and that each of the one or more PDSCHs provides one transport block that the UE correctly decodes if the UE is provided nrofHARQ-BundlingGroups for the serving cell

-	the UE assumes incorrect decoding for transport blocks in each of the remaining PDSCH receptions scheduled by the DCI format 1_3 on the serving cell associated with fields in DCI format 1_3 for SCell dormancy indication

----------------------------------------------------End of TP1 of TS 38213-----------------------------------------------------

  - RRC parameter

A list of RRC parameters is agreed in the RAN1 120bis meeting, while the value range for TDRA-FieldIndexListDCI-1-3-r19 and TDRA-FieldIndexListDCI-0-3-r19 are still FFS.

| New | TDRA-FieldIndexListDCI-1-3-r19 | Joint TDRA table for DL scheduling one or multiple PDSCHs per scheduled cell via DCI format 1_3 | SEQUENCE (SIZE (1..FFS)) OF TDRA-FieldIndexDCI-1-3-r19 |
| --- | --- | --- | --- |
| New | TDRA-FieldIndexDCI-1-3-r19 | Configure each row of the joint TDRA field table for DL scheduling via DCI format 1_3 containing the applicable TDRA field indexes for multiple BWPs/cells, where the TDRA index for a BWP of a cell points to a corresponding TDRA in the TDRA table applicable for DCI format 1-3, the order of TDRA index in each row refers the BWP-Id for a cell and the order of cells in scheduledCellListDCI-1-3 (i.e., first TDRA index in a row is for the smallest BWP-Id that can be scheduled by the DCI format 1-3, as specified in 38.212, of the first cell in scheduledCellListDCI-1-3, second TDRA index in a row is for the second smallest BWP-Id that can be scheduled by the DCI format 1-3, as specified in 38.212, of the first cell and so on), and the number of TDRA indices in a row of TDRA-FieldIndexDCI-1-3-r19 should be the same as the total number of BWPs that can be scheduled by the DCI format 1-3, as specified in 38.212, across cells included in scheduledCellListDCI-1-3. | SEQUENCE SIZE (2.. maxNrofBWPsInSetOfCells-r18)) OF INTEGER (0..maxNrofDL-AllocationsExt-r19-1) Note: maxNrofDL-AllocationsExt-r19-1 equals 63 |
| New | pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 | Configure the BWP specific TDRA table for multi-PDSCH scheduling by DCI format 1_3. | Note: This TDRA table has the same structure and size as the existing configurable TDRA table pdsch-TimeDomainAllocationListForMultiPDSCH-r17 for multi-PDSCH scheduling using DCI format 1_1. |
| New | TDRA-FieldIndexListDCI-0-3-r19 | Joint TDRA table for UL scheduling one or multiple PUSCHs per scheduled cell via DCI format 0_3 | SEQUENCE (SIZE (1..FFS)) OF TDRA-FieldIndexDCI-0-3-r19 |
| New | TDRA-FieldIndexDCI-0-3-r19 | Configure each row of the joint TDRA field table for UL scheduling via DCI format 0_3 containing the applicable TDRA field indexes for multiple BWPs/cells, where the TDRA index for a BWP of a cell points to a corresponding TDRA in the TDRA table applicable for DCI format 0-3, the order of TDRA index in each row refers the BWP-Id for a cell and the order of cells in scheduledCellListDCI-0-3 (i.e., first TDRA index in a row is for the smallest BWP-Id that can be scheduled by the DCI format 0-3, as specified in 38.212, of the first cell in scheduledCellListDCI-0-3, second TDRA index in a row is for the second smallest BWP-Id that can be scheduled by the DCI format 0-3, as specified in 38.212, of the first cell and so on), and the number of TDRA indices in a row of TDRA-FieldIndexDCI-0-3-r19 should be the same as the total number of BWPs that can be scheduled by the DCI format 0-3, as specified in 38.212, across cells included in scheduledCellListDCI-0-3. | SEQUENCE SIZE (2.. maxNrofBWPsInSetOfCells-r18)) OF INTEGER (0..maxNrofUL-Allocations-1-r18) |
| New | pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3 | Configure the BWP specific TDRA table for multi-PUSCH scheduling by DCI format 0_3. | Note:This TDRA table has the same structure and size as the existing configurable TDRA table pusch-TimeDomainAllocationListForMultiPUSCH-r16 for multi-PUSCH scheduling using DCI format 0_1. |

In Rel-18, the maximum size of TDRA-FieldIndexListDCI-1-3-r18 is 16, and the maximum size of PDSCH time domain resource allocations is 32. Similarly, the maximum number of TDRA-FieldIndexListDCI-0-3-r18 is 64, and the maximum number of PUSCH time domain resource allocations is 64. The maximum size of the joint TDRA table is at least not smaller than the size of BWP specific TDRA table so that the flexible resource allocation indication and RRC signaling overhead can be traded off. Thus, this principle can be reused to determine the maximum number of TDRA-FieldIndexListDCI-1-3-r19.

tdra-FieldIndexListDCI-1-3-r18 SEQUENCE (SIZE (1..32)) OF TDRA-FieldIndexDCI-1-3-r18 OPTIONAL, -- Need R tdra-FieldIndexListDCI-0-3-r18 SEQUENCE (SIZE (1..64)) OF TDRA-FieldIndexDCI-0-3-r18 OPTIONAL, -- Need R TDRA-FieldIndexDCI-1-3-r18 ::= SEQUENCE (SIZE (2.. maxNrofBWPsInSetOfCells-r18)) OF INTEGER (0..maxNrofDL-Allocations-1-r18) TDRA-FieldIndexDCI-0-3-r18 ::= SEQUENCE (SIZE (2.. maxNrofBWPsInSetOfCells-r18)) OF INTEGER (0..maxNrofUL-Allocations-1-r18) maxNrofDL-Allocations-1-r18 INTEGER ::= 15 -- Maximum number of PDSCH time domain resource allocations minus 1 maxNrofUL-Allocations-1-r18 INTEGER ::= 63 -- Maximum number of PUSCH time domain resource allocations minus 1

Since it is noted in the RRC parameter table that  pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3 has the same structure and size as the existing configurable TDRA table pusch-TimeDomainAllocationListForMultiPUSCH-r16 for multi-PUSCH scheduling using DCI format 0_1, and pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 has the same structure and size as the existing configurable TDRA table pdsch-TimeDomainAllocationListForMultiPDSCH-r17 for multi-PDSCH scheduling using DCI format 1_1. Moreover, pusch-TimeDomainAllocationListForMultiPUSCH-r16 has the maximum number of PUSCH time domain resource allocations 64, pdsch-TimeDomainAllocationListForMultiPDSCH-r17 has the maximum number of PDSCH time domain resource allocations 64. Therefore, the maximum number of rows of TDRA-FieldIndexListDCI-1-3-r19 and TDRA-FieldIndexListDCI-0-3-r19 is 64.

Proposal 3: The maximum number of rows of both TDRA-FieldIndexListDCI-1-3-r19 and TDRA-FieldIndexListDCI-0-3-r19 are 64.

- Conclusion

In this contribution, the following proposal is as follows:

Proposal 1: “the first SLIV” in the RAN1#120bis agreement should be clarified as “the SLIV with the earliest starting time”.

Proposal 2: The TP1 for TS 38213 is adopted.

----------------------------------------------------Start of TP1 of TS 38213-------------------------------------------------

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

*** Unchanged parts are omitted ***

-	the second HARQ-ACK sub-codebook is associated with DCI format 1_3 that

-	schedules PDSCH receptions on more than one serving cells from the set of serving cells, or

-	schedules more than one PDSCH receptions on a serving cell c, from the set of serving cells, for which the UE is not provided nrofHARQ-BundlingGroups or is provided nrofHARQ-BundlingGroups with value NHARQ-ACK,cTBG,max>1, or

-	does not include a SCell dormancy indication field or the SCell dormancy indication field is reserved, indicates SCell dormancy, and schedules PDSCH receptions on one or more serving cells from the set of serving cells

-	in the above, and for the purpose of providing HARQ-ACK information corresponding to SCell dormancy indication by the DCI format 1_3

-	the UE assumes that the UE receives one PDSCH, with earliest starting time among one or more PDSCH receptions scheduled by first SLIV in the row of pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 for DCI format 1_3, on the serving cell associated with fields in DCI format 1_3 for SCell dormancy indication, as described in Clause 10.3, and that the PDSCH provides one transport block that the UE correctly decodes, if the UE is not provided nrofHARQ-BundlingGroups for the serving cell

-	the UE assumes that the UE receives one or more PDSCHs associated with a TBG with smallest index, or a PDSCH reception group with smallest index, among one or more PDSCH receptions scheduled by the DCI format 1_3, on the serving cell associated with fields in DCI format 1_3 for SCell dormancy indication, as described in Clause 10.3, and that each of the one or more PDSCHs provides one transport block that the UE correctly decodes if the UE is provided nrofHARQ-BundlingGroups for the serving cell

-	the UE assumes incorrect decoding for transport blocks in each of the remaining PDSCH receptions scheduled by the DCI format 1_3 on the serving cell associated with fields in DCI format 1_3 for SCell dormancy indication

----------------------------------------------------End of TP1 of TS 38213-----------------------------------------------------

Proposal 3: The maximum number of rows of both TDRA-FieldIndexListDCI-1-3-r19 and TDRA-FieldIndexListDCI-0-3-r19 are 64.

# Reference

- 3GPP TSG RAN Meeting #105, Melbourne, Australia, Sep 9-12 2024, New WID: Multi-carrier enhancements for NR Phase 2, RP-242408.

- R1-2503155 Consolidated Rel-19 higher layers parameter list Post RAN1#120bis

- R1-2503173 Introduction of multi-carrier enhancements for NR Phase 3