3GPP TSG RAN WG1 #121			R1- 2503385

St Julian’s, Malta, May 19th – 23th, 2025

Source: 	vivo

Title:	Discussion on UE features for AIML Air interface

Agenda Item:	9.15.1

Document for:	Discussion and Decision

Introduction

In this contribution we provide our views on UE features for AIML for NR Air interface.

.

Discussion

AI/ML based beam management

Introduce the following Rel. 19 UE FGs for AI/ML based beam management.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X. NR_AIML_Air | X-1-1 | AI/ML based beam management for NW-sided model | 1. Support of M reported RS, M>4 2. Support values of M | FFS | yes | n/a | AI/ML based beam management for NW-sided model is not supported | FFS | FFS | FFS | FFS | [Component 2 candidate values: {5, … , 64}] FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| X. NR_AIML_Air | X-1-2 | AI/ML based beam prediction for UE-sided model | 1. Support of beam prediction for BM-Case1 2. The Support of associated ID(s) [3. The supported value of the maximum number of inference report configuration] 4. Supported values of the maximum number of resources in Set A for prediction [5. Supported values of the maximum number of resources in Set B for measurement] 6.The support quantity of prediction | FFS | yes | n/a | AI/ML based beam management for UE-sided model is not supported | FFS | FFS | FFS | FFS | [Component 4 candidate values: {1, … , 48}] FFS: Further partitioning of this FG based on existing and future agreements [Component 6 candidate values: {predicted-CRI/SSBRI, predicted-CRI/SSBRI and predicted RSRP}] | Optional with capability signalling |
| X. NR_AIML_Air | X-1-3 | AI/ML based beam prediction for UE-sided model | 1. Support of beam prediction for BM-Case2 2. Supported values of the number of future time instance. 3. Supported values of the time gap between two consecutive future time instances for prediction. 4. The Support of associated ID(s) 5. Supported values of the maximum number of resources in Set A for prediction 6. Supported values of the maximum number of resources in Set B for measurement 7.The support quantity of prediction |  | yes | n/a | Beam prediction for BM-Case2 is not supported | FFS | FFS | FFS | FFS | [Component 2 candidate values: {1, 2, 4, 8}] [Component 3 candidate values: {10ms, 20ms, 40ms, 80ms, 160ms}] FFS: Further partitioning of this FG based on existing and future agreements [Component 7 candidate values: {predicted-CRI/SSBRI, predicted-CRI/SSBRI and predicted RSRP}] | Optional with capability signalling |
| X. NR_AIML_Air | X-1-4 | Performance monitoring for UE-sided model | 1. Support of performance monitoring of AI/ML model for beam prediction. | X-1-2, [X-1-3] | yes | n/a | Performance monitoring for UE-sided model is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements (e.g., separate FGs for Mode-A and Mode-B) | Optional with capability signalling |
| X. NR_AIML_Air | X-1-5 | Data collection for UE-sided model | 1. Support of data collection for UE-sided model 2. The Support of associated ID(s) 3. Supported values of the maximum number of resources in Set A [4. Supported values of the maximum number of resources in Set B] | FFS | yes | n/a | Data collection for UE-sided model is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |

AI/ML based positioning

Introduce the following Rel. 19 UE FGs for AI/ML based positioning.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X. NR_AIML_Air | X-2-1 | AI/ML based positioning for UE-side model | 1. Support of AI/ML based positioning Case 1 | FFS | N/A | N/A | AI/ML based positioning Case 1 is not supported | FFS | FFS | FFS | FFS | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported | Optional with capability signalling |
| X. NR_AIML_Air | X-2-2 | Support of model monitoring for AI/ML based positioning for UE-side model | 1. Support of performance monitoring for AI/ML based positioning Case 1 | FFS | N/A | N/A | Performance monitoring of AI/ML based positioning Case 1 is not supported | FFS | FFS | FFS | FFS | FFS: The necessity and further partitioning of this FG depend on the future RAN1/2 agreements on the format of model monitoring outcome. | Optional with capability signalling |
| X. NR_AIML_Air | X-2-3 | Support of data collection for AI/ML based positioning for UE-side model | 1. Support of Data collection for AI/ML based positioning Case 1 2. Support of Associated ID | FFS | N/A | N/A | Data collection of AI/ML based positioning Case 1 is not supported | FFS | FFS | FFS | FFS | FFS: The necessity and further partitioning of this FG depends on the future RAN1 agreements on Associated ID | Optional with capability signalling |

AI/ML CSI prediction

Introduce the following Rel. 19 UE FGs for AI/ML based CSI prediction.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X. NR_AIML_Air | X-3-1 | AI/ML based CSI prediction for UE-sided model when N4=1 | 1. Support of CSI prediction 2. Supported maximum number of observation number | FFS | yes | n/a | 1.CSI prediction for N4=1 is not supported 2. | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| X. NR_AIML_Air | X-3-2 | AI/ML based CSI prediction for UE-sided model when N4>1 | 1. Support of CSI prediction 2. Supported values of the number of future time instance. 3. Supported values of the maximum number of observation number | FFS | yes | n/a | 1.CSI prediction for N4>1 is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| X. NR_AIML_Air | X-3-3 | Performance monitoring for UE-sided model | 1. Support of performance monitoring of AI/ML model for CSI prediction. | X-1-2, [X-1-3] | yes | n/a | - Performance monitoring for UE-sided model is not supported - Cant not determine the level of fitness of the activated functionality | FFS | FFS | FFS | FFS |  | Optional with capability signalling |

Conclusion

In this contribution we provide updates on UE features for AI/ML based beam management, positioning and CSI prediction.

- Introduce the following Rel. 19 UE FGs for AI/ML based beam management in section 2.

- Introduce the following Rel. 19 UE FGs for AI/ML based positioning in section 2.

- Introduce the following Rel. 19 UE FGs for AI/ML based CSI prediction in section 2.