3GPP TSG-RAN WG1 Meeting #121         						 	R1-2503518

St Julian's, Malta, 19 - 23 May, 2025

Agenda Item:     9.4.4

Source:	Spreadtrum, UNISOC

Title:	Discussion on other aspects for Ambient IoT

Document for:	Discussion and decision

Introduction

In recent years, IoT has attracted much attention in the wireless communication world. More ‘things’ are expected to be interconnected for improving productivity efficiency and increasing comforts of life. In order to meet extreme demands of target use cases, e.g., batteryless, extremely low power consumption, etc, an Ambient-IoT(A-IoT) work item [1] was approved  to open new markets within 3GPP systems.

The RAN1 scope for A-IoT WI is as follows [1]:

General Scope - … The following objectives are set, within the General Scope: - RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission. …

In this contribution, we discuss other aspects, e.g., multiplexing/multiple access, scheduling information, and timing relationships for A-IoT.

Multiplexing/multiple access

According to the WID, multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. In this section, we discuss multiplexing/multiple access for R2D and D2R.

D2R multiple access

For D2R multiple access, we understand the main point is to specify resource allocation method for Msg1 and Msg3 in TDMA and FDMA mode.

Resource allocation and selection for Msg1

For Msg1 transmission in TDMA mode, one general solution is that different Msg1s are transmitted in different “slots”. In addition to that, X time domain resource(s) for D2R transmission(s) for Msg1 in one “slot” was studied.  Form our perspective, multiple devices transmit Msg1 within the same ‘slot’ triggered by one R2D message would be beneficial to the latency and efficiency, as the gaps between R2D and D2R can be reduced. Additionally, it cannot guarantee that Msg1 FDMA will always be available. If FDMA is unavailable, TDMed Msg1 transmission is another optional approach to improve inventory efficiency.

In the last RAN1 meeting, agreements related to X=2 time domain resource(s) were achieved as following.

Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission Conclusion For any timing requirement that needs to be specified from device perspective by RAN1, the timing(s) do not include the impacts of SFO from the RAN1 perspective. - Whether 3GPP needs to specify some requirement on device’s SFO is up to RAN4 discussion. - Note: RAN1 will not specify TR2D_min or TR2D_max in Rel-19 - Note: SFO may still be considered when discussing time-domain resources for Msg1 when X=2 Agreement Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s). Agreement For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access. - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact - FFS: how to define the end of the R2D transmission in 9.4.1 Working assumption For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource: - Derived based on the starting time of the first Msg1 time domain resource plus Toffset2 - FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access.

- The starting time for Msg1 time domain resource

Figure 2.1.1-1. An example of Toffset1 and Toffset2

For the starting time for the first Msg1 time domain resource, as discussed in the last meeting, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access regardless of X=1 or X=2. The other issue needs to be discussed is how to indicate/determine Toffset1. From our perspective, predefine Toffset1 in the spec is the simplest way to determine the starting time for the first Msg1 time domain resource and reduce the overhead of R2D scheduling information. It is up to RAN4 to determine the exact value considering the processing time and TX/RX switching time.

Proposal 1: Support predefining Toffset1 in the spec and up to RAN4 to determine the exact value.

For the starting time for the second Msg1 time domain resource, a working assumption was agreed in the last meeting, we suggest to confirm this working assumption. Device determine the starting time for the second Msg1 time domain resource based on the starting time of the first Msg1 time domain resource plus Toffset2. Toffset2 can be determined by the some specified rule, e.g., based on the length of first Msg1 time domain resource add the gap between these two time domain resource considering SFO. In this way, signaling overhead can also be reduced. The Msg1 time domain resource can be derived by the Msg1 payload size or indicated by reader. As for the gap, we has an conclusion that whether 3GPP needs to specify some requirement on device’s SFO is up to RAN4 discussion.

Proposal 2: Confirm the working assumption to determine the starting time for the second Msg1 time domain resource.

- Toffset2 is derived by a rule, i.e., the length of first Msg1 time domain resource plus the gap between these two time domain resource considering SFO.

- Multiple access cases for Msg1

As mentioned before, multiplexing/multiple access in D2R Msg1 is by only TDMA and FDMA. In general, there are three possible cases for a certain Msg1 transmission.

  - Case 1: TDMA only, i.e., only multiple time-domain resources have been allocated, with only one frequency point

  - Case 2: FDMA only, i.e., only multiple frequency domain resources have been allocated, with only one time domain resource

  - Case 3: TDMA+FDMA, i.e., multiple resources have been allocated in both the time and frequency domains

The above three cases, can be illustrated by the following figures. If all three Cases are supported, a unified resource allocation scheme is needed to simplify the access design.

Figure 2.1.1-2. Multiple access for Msg1

Observation 1: There are three possible multiple access cases (i.e., TDMA only, FDMA only and TDMA+FDMA) for a certain Msg1 transmission. If all three cases are supported, a unified resource allocation scheme is beneficial for simplifying design.

- Data rate for Msg1

In the last meeting, we have an agreement related to the data rate for the FDMed Msg1 transmissions, list as follows.

Agreement At least for CBRA, for FDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access, support only the same data rate for the FDMed Msg1 transmissions.

For FDMed Msg1 transmissions, it is assumed that all FDMed devices transmit with the same Msg1 data rate with the same bandwidth.

Additionally, the length of Msg1 transmission depends on factors like TBS, data rate, chip length, modulation and number of repetitions. Considering X=2, as different Msg1 transmissions has same payload size in CBRA, i.e., 16bits. Other factor is also indicated by the reader, there is no clear motivation to support different length of Msg1 transmission as well as different data rates between X=1 and X=2 time domain resources. Therefore, we support the same D2R data rate for Msg1 for CBRA.

Proposal 3: For CBRA, always support the same D2R data rate for Msg1 transmission.

- How a device determines resource for Msg1 transmission

Regarding the issue on how a device determines the time/frequency domain resource for the D2R transmissions for Msg1, we understand that device 1 can randomly select one of the time and frequency domain resource among multiple resources for the Msg1

Observation 2: Device can randomly select one of the time and frequency domain resource among multiple resources for the Msg1.

Resource allocation and selection for Msg3

- Multiple access cases for Msg3

In the last RAN1 meeting, the following multiple access cases for Msg3 were discussed [2].

Table 2.1.2-1 multiple access cases for Msg3

|  | FDMA of Msg3s within one time-domain access occasion | TDMA of Msg3s |  |
| --- | --- | --- | --- |
|  |  | Interleaving | Non-interleaving |
| One Common Msg2 (e.g., One PRDCH for Msg2 Tx in response to Two Msg1s) | Case 1: Support Common Msg2; Msg3. Msg3. | -- | Case 4 Common Msg2; Msg3; Msg3. |
| Separate Msg2s (e.g., Two PRDCHs for Msg2 Tx in response to Two Msg1s) | Case 2 Msg2; Msg2; Msg3. Msg3. | Case 3 Msg2; Msg3; Msg2; Msg3. | Case 5 Msg2; Msg2; Msg3; Msg3. |

For the above cases, Case 1 is supported and Case 4 is excluded based on the agreement list as follows.

Agreement-RAN1#120 Support FDMA of D2R transmissions for Msg3 from multiple devices in response to a PRDCH for Msg2 transmission corresponding to multiple Msg1 during access procedure. Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission.

However, whether support Case 2, Case 3 or Case 4 is FFS.

- Case 2(Msg2, Msg2, FDMed Msg3 and Msg3): it is more complex and no benefit compared with case 1. On the one hand, this case is highly dependent on Y>1 for Msg1, which is already reserved multiple frequency resources. For Y=1, case 2 is not applicable. On the other hand, device with long-time counting ability is required for multiple consecutive Msg2s. This case needs to consider the gap issue caused by receiving multiple Msg2 (e.g., gap is needed between different Msg2), this will cause a longer monitoring time. Therefore, case 2 can be excluded.

- Case 3(Msg2, Msg3, Msg2, Msg3): if X=2 and Y=1 for Msg1, then FDMed is not support for Msg3s. case 3 is a suitable way correspond to multiple Msg1. In addition, case 3 does not require device with long-time counting ability, as device can transmit Msg3 right after it receives Msg2, meeting the processing time requirement.

- Case 5 (Msg2, Msg2, TDMed Msg3 and Msg3): it is also not applicable for device without long-time counting ability, so we prefer to deprioritize case 5. This case also needs to consider the gap issue caused by receiving multiple Msg2 (e.g., gap is needed between different Msg2), this will cause a longer monitoring time.

Proposal 4: Support Case 3 that the PRDCH for Msg2 transmission and the corresponding Msg3 are sent one by one, e.g., Msg2-Msg3-Msg2-Msg3.

- Resource allocation for Msg3

As only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission, the starting time of Msg3 does not need to indicate by reader. For both case 1 and case 3, device can transmit Msg3 right after it receives Msg2, meeting the processing time requirement, which can be predefined in the spec. As we have a conclusion that RAN1 will not specify TR2D_min or TR2D_max in Rel-19, the processing time requirement can be discussed in RAN4.

Proposal 5: The starting time of Msg3 should meet the processing time requirement after the end of the corresponding Msg2, while the processing time requirement can be discussed in RAN4.

In addition, for Msg3 frequency resource allocation, it was agreed that the frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. Whether option 1 should be introduced as default option is FFS [3].

Agreement- RAN1#120 Support the following option(s) for frequency domain resource for Msg3 transmission: - Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. - FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule - Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device

We support option 1 as a default if PRDCH does not provide the indication on the frequency domain resource, as it would be beneficial for payload saving for Msg2 in some cases. As we have an agreement that “any scheduling information related to resource allocation that needs to be signaled is indicated by higher-layer signaling via the corresponding PRDCH.”, the frequency domain resource indication will be included in the higher layer, and device cannot know the presence of it without decoding high layer information. We can support Option1 as a default option when device does not decode the explicit indication from the corresponding Msg2.

Proposal 6: Support option 1 as a default if the corresponding PRDCH does not provide the frequency domain resource for Msg3 transmission.

- Data rate for Msg3

It is still open whether support same or different/scalable D2R data rate for FDMed Msg3. We support different/scalable D2R data rate for FDMed Msg3. Msg3 size may be variable due to the variable size of device ID and/or different use case (e.g., Inventory only or Inventory + Command), it is beneficial for device using higher data rate to improve the transmission efficiency. Reader can indicate the D2R chip durations corresponding to the minimum and maximum R value, device can choose any value within this interval.

Proposal 7: Support different D2R data rate for FDMed Msg3 in response to a PRDCH for Msg2 transmission.

Msg2 releated

In the RAN1#120 meeting [3], it was agreed that both option 1 and option 2 can be supported for Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access..

Agreement Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2.

Regarding the FFS (i.e., the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2), Msg2 needs to carry device ID (e.g., RN16) and Msg3 scheduling information for one or more device. From RAN1 perspective, the maximum number of the Msg1 responses that can be carried within a PRDCH for Msg2 should not be larger than Xmax*Ymax, where Xmax and Ymax is the maximum value of TDMA and FDMA of D2R transmissions for Msg1 in response to a R2D transmission triggering random access.

In general, we support two cases to indicate multiple Msg3 as analyzed in 2.1.2.

- Case 1: Common Msg2, FDMed Msg3 and Msg3

- Case 3: Msg2, Msg3, Msg2, Msg3

Specifically, we have the following proposal:

Proposal 8: For the number of Msg1 responses in a PRDCH for Msg2, support following:

- For case 1, the number of the Msg1 responses that can be carried within a PRDCH for Msg2 should be Xmax*Ymax.

- For case 3, the number of the Msg1 responses that can be carried within a PRDCH for Msg2 should be 1.

In addition, for the monitoring time of Msg2, we have an agreement as follows:

Agreement A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device.

The starting time for Msg2 monitoring is TD2R_min after the end time of Msg1 transmission.

For the end time for Msg2 monitoring, it depends on whether to define a time window for Msg2 monitoring. From our perspective, it is unnecessary to define a window. Device can continue to monitor the Msg2 until it received the a Msg2 corresponding to its RN16, or a new Paging, or its energy exhausted, or other behavior identified by RAN2 as failure of the Msg1.

Proposal 9: It is unnecessary to define a Msg2 monitoring window.

Scheduling information

R2D control information for R2D reception

In the last meeting, some companies have discussed early identification/termination for R2D reception. Device ID or other information in L1's control is used for the device to identify the receiver of R2D transmission. If it is not for the device, the reception will be terminated in advance. However, for device 1, R19 does not support multiple energy states. It should be assumed that as long as it doesn’t power off, R2D reception is attempted at all times except during the D2R transmission. Therefore, early identification/termination for R2D reception does not bring significant power saving to the device. Moreover, the device ID is not visible on the reader side, so whether the device ID can be carried in L1 control is also a question. Therefore, is it not essential for device 1 to achieve early identification/termination for R2D reception.

Proposal 10: Don’t support early identification/termination for R2D reception for device 1.

The R2D chip duration for a PRDCH is determined based on CAP. The R2D chip duration for a PRDCH should be the same as the CAP preceding the PRDCH. There is no clear motivation to support different chip durations between CAP and PRDCH which will increase device complexity.

Proposal 11: The R2D chip duration for a PRDCH is the same as the clock-acquisition part of the R2D time acquisition signal immediately preceding the PRDCH.

R2D control information for D2R scheduling

RAN1 studied R2D control information contents for R2D reception and D2R scheduling, and the followings was achieved in the last meeting.

Agreement For scheduling D2R transmission, any scheduling information related to resource allocation that needs to be signaled is indicated by higher-layer signaling via the corresponding PRDCH. Working assumption For indicating the payload size (i.e. TBS-like) for PDRCH transmission with variable size: - 7 bits for byte-level D2R payload size indication

Payload size indication

The maximum payload size is 1000bits. Therefor, the working assumption should be conformed that 7 bits for byte-level D2R payload size indication is supported.

Proposal 12: Conform the working assumption that 7 bits for byte-level D2R payload size indication is supported.

FEC including the coding rate

FEC can be supported in D2R transmission, and the code rate of FEC was discussed in section 9.4.2. According to the analysis in the contribution of 9.4.2 [4], only 1/3 code rate of FEC is supported. Therefore, only 1 bit is needed to indicate that whether FEC is applied for the D2R transmission.

Proposal 13: 1 bit is used to indicate that whether FEC is applied for the D2R transmission.

Repetition

According to the discussion in the previous meeting, D2R transmission supports block-level repetition to improve reception reliability. And the latest proposal in section 9.4.2 is as following. According to the analysis in the contribution of 9.4.2 [4], the maximum number of block-level repetition should be 2. Therefore, 1 bit is needed to indicate the number of block-level repetitions.

Proposal 14: 1bit is used to indicate the number of block-level repetitions for D2R.

Conclusion

In this contribution, we discuss on frame structure and timing aspects for A-IoT. The following observations and proposals are provided:

- Observations:

Observation 1: There are three possible multiple access cases (i.e., TDMA only, FDMA only and TDMA+FDMA) for a certain Msg1 transmission. If all three cases are supported, a unified resource allocation scheme is beneficial for simplifying design.

Observation 2: Device can randomly select one of the time and frequency domain resource among multiple resources for the Msg1.

- Proposals:

Proposal 1: Support predefining Toffset1 in the spec and up to RAN4 to determine the exact value.

Proposal 2: Confirm the working assumption to determine the starting time for the second Msg1 time domain resource.

- Toffset2 is derived by a rule, i.e., the length of first Msg1 time domain resource plus the gap between these two time domain resource considering SFO.

Proposal 3: For CBRA, always support the same D2R data rate for Msg1 transmission.

Proposal 4: Support Case 3 that the PRDCH for Msg2 transmission and the corresponding Msg3 are sent one by one, e.g., Msg2-Msg3-Msg2-Msg3.

Proposal 5: The starting time of Msg3 should meet the processing time requirement after the end of the corresponding Msg2, while the processing time requirement can be discussed in RAN4.

Proposal 6: Support option 1 as a default if the corresponding PRDCH does not provide the frequency domain resource for Msg3 transmission.

Proposal 7: Support different D2R data rate for FDMed Msg3 in response to a PRDCH for Msg2 transmission.

Proposal 8: For the number of Msg1 responses in a PRDCH for Msg2, support following:

- For case 1, the number of the Msg1 responses that can be carried within a PRDCH for Msg2 should be Xmax*Ymax.

- For case 3, the number of the Msg1 responses that can be carried within a PRDCH for Msg2 should be 1.

Proposal 9: It is unnecessary to define a Msg2 monitoring window.

Proposal 10: Don’t support early identification/termination for R2D reception for device 1.

Proposal 11: The R2D chip duration for a PRDCH is the same as the clock-acquisition part of the R2D time acquisition signal immediately preceding the PRDCH.

Proposal 12: Conform the working assumption that 7 bits for byte-level D2R payload size indication is supported.

Proposal 13: 1 bit is used to indicate that whether FEC is applied for the D2R transmission.

Proposal 14: 1bit is used to indicate the number of block-level repetitions for D2R.

Reference

- RP-250796, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#107, Incheon, Korea, March 12-14, 2025.

- R1-2503104, “FL summary#4 on other aspects for Rel-19 Ambient IoT”, RAN1#120b, Wuhan, China, April 7th – 11th, 2025.

- R1-2501625, “Final FL summary on other aspects for Rel-19 Ambient IoT”, RAN1#120, Athens, Greece, February 17th – 21st, 2025.

- R1-2503516, “Discussion on line coding, FEC, CRC, repetition aspects for Ambient IoT”, RAN1#121, St Julian's, Malta, 19 - 23 May, 2025.