3GPP TSG RAN WG1 #121			R1-2503528

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.11.2

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

Document for:	Discussion and decision

Introduction

In RAN#106, revised WI on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 was approved [1]. One objective is to specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB for HD-FDD RedCap UEs and eRedCap UEs.

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

This contribution provides our views for enhancements for the HD collisions case 4.

Discussion

During last RAN1 meetings, the following agreement of collision handling for case 4 was agreed.

Agreement Update the working assumption in RAN1 #120 with the following updates: For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported: - Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2. - Handling of collision with PDSCH scheduled by Type-1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2. - FFS: handling of PDCCH ordered PRACH transmission - For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all cases - This is signaled by one UE specific RRC parameter - Note1: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized. Note2: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

Possible restrictions for Type0/0A

For PDSCH scheduled by Type-0/0A PDCCH CSS, it can aligned with PDSCH scheduled by Type-1/2-PDCCH CSS. No matter of PDSCH scrambled by C-RNTI or SI-RNTI, it is all up to UE implementation. Therefore, there is no needs to have additional restrictions of system information.

- Handling of collision with all types of PDSCH scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2.

Handling of PDCCH ordered PRACH transmission

For PDCCH ordered PRACH transmission, some suggest to set it as an exception case. Since PDCCH ordered PRACH always happens when downlink data arrival while UE is out of sync. Thus, there is low possibility that gNB scheduled a unicast PDSCH and PDCCH ordered PRACH. For PDSCH carrying SI, it has already covered by the exception case, which is up to UE implementation. Therefore, PDCCH ordered PRACH is not an exception case in case 4.

- PDCCH ordered PRACH is not an exception case in case 4

Cancellation timeline

When DL reception is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized. There are two DCIs for PDSCH/CSI-RS vs PUSCH in case 4, one for DL and the other for UL. While One DCI and another DCI+PDSCH relate with cancelation timeline for PDSCH/CSI-RS vs PUCCH. Therefore, corresponding description in draft CR should change.

As shown in the following figure, PDSCH/CSI-RS scheduled by DCI1 is overlapping with PUSCH scheduled by DCI2. The gap between DCI2 and PUSCH can be equal to or larger than legacy  Tproc,2. Then, only the gap between DCI1 of PDSCH/CSI-RS and PUSCH needs extra considered for case4. Similarly, when PDSCH/CSI-RS vs. PUCCH, the gap between DCI1 and PUCCH matters with UL cancellation timeline. Thus, UL cancellation timeline is satisfied if the first symbol of PUCCH or PUSCH is after Tproc,2  relative to a last symbol of the PDCCH scheduling CSI-RS or PDSCH, or if a symbol of SRS is after Tproc,2  relative to a last symbol of the PDCCH scheduling CSI-RS or PDSCH.

Figure 1: UL cancellation timeline

If a HD-UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH in a set of symbols, or if a HD-UE operating on an NTN serving cell is not provided ntnRedCapCollisionCase4UlPriority and detects a DCI format indicating to the HD-UE to transmit SRS, or PUCCH, or PUSCH in a set of symbols, and the HD-UE detects a DCI format indicating to the HD-UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the HD-UE does not expect to cancel the transmission of the PUCCH or PUSCH in the set of symbols if the first symbol in the set occurs within Tproc,2 relative to a last symbol of a PDCCH reception where the HD-UE detects the DCI format; otherwise, the HD-UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from clauses 9 and 9.2.5 or clause 6.1 of [6, TS 38.214]. - the HD-UE does not expect to cancel the transmission of SRS in symbols from the subset of symbols that occur within Tproc,2 relative to a last symbol of a PDCCH reception where the HD-UE detects the DCI format. The HD-UE cancels the SRS transmission in remaining symbols from the subset of symbols. Tproc,2 is the PUSCH preparation time for HD-UE processing capability 1 [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH.

- UL cancellation timeline is satisfied

- If the first symbol of PUCCH or PUSCH is after Tproc,2  relative to a last symbol of the PDCCH scheduling CSI-RS or PDSCH.

- If a symbol of SRS is after Tproc,2  relative to a last symbol of the PDCCH scheduling CSI-RS or PDSCH.

Type 0/0A/1/2-PDCCH CSS and ntnRedCapCollisionCase4UlPriority

The intention of agreement is ntnRedCapCollisionCase4UlPriority is only allowed to indicate UL overriding DL for cases exclude PDSCH scheduled by Type 0/0A/1/2-PDCCH CSS in case 4. Cases of PDSCH scheduled by Type 0/0A/1/2-PDCCH CSS are up to UE implementation.

- ntnRedCapCollisionCase4UlPriority is only allowed to indicate UL overriding DL other than PDSCH scheduled by PDCCH reception according to a Type0/0A/1/2-PDCCH CSS set.

TA report enhancements

It was agreed to further study TA report enhancements as the following.

Agreement To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied: - Finer TA report granularity - Smaller TA offset threshold - Trigger a TA report when the collision is detected at the UE - TA reporting with a new triggering mechanism from gNB - TA drifting rate reporting Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated. Note: other solutions can also be studied

Our view is no enhancement of TA report is introduced in Rel-19 NTN.

One reason is the enhancements of TA reporting cannot solve TA mismatch issue. It can only mitigate the TA mismatch partially, so it is not a complete solution.

- For first two bullets, finer TA report granularity and smaller TA offset threshold, it is uncertain time for the actual PUSCH with TAR MAC-CE. So the practical effects is limited.

- For triggering a TA report by UE, it is too late to detect a collision of case 4 and then report TA. The collision has already happened. Furthermore, trigger a TA report by UE requires a lot of discussion and specification impacts, e.g. the definition of event, container of TAR, resource allocation, etc. It is impossible to finish it within the current TU allocation.

- For TA reporting with a new triggering mechanism from gNB, it is hard to understand how gNB know when to trigger a TA. If the trigger is gNB find a UL is missed, a collision has already there and it is too late to trigger TAR.

- For TA drifting rate reporting, gNB can obtain TA drift rate according to the previous TA reports, again TA drift rate at UE side is also acquired at same way. Therefore, it really does not necessary to report TA drifting rate.

- No enhancement of TA report is introduced in Rel-19 NTN.

Conclusion

In this contribution, we made the following observations and proposals.

- Handling of collision with all types of PDSCH scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2.

- PDCCH ordered PRACH is not an exception case in case 4

- UL cancellation timeline is satisfied

- If the first symbol of PUCCH or PUSCH is after Tproc,2  relative to a last symbol of the PDCCH scheduling CSI-RS or PDSCH.

- If a symbol of SRS is after Tproc,2  relative to a last symbol of the PDCCH scheduling CSI-RS or PDSCH.

- ntnRedCapCollisionCase4UlPriority is only allowed to indicate UL overriding DL other than PDSCH scheduled by PDCCH reception according to a Type0/0A/1/2-PDCCH CSS set.

- No enhancement of TA report is introduced in Rel-19 NTN.

References

RP-243300  Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

R1-2503051	Summary #3 for Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	Moderator (CATT)