3GPP TSG RAN WG1 #121                                            R1-2503536

St Julian's, Malta, May 19th-23rd, 2025

Source:	TCL

Title:	Discussion on modulation aspects for Ambient IoT physical design

Agenda Item:	9.4.1

Document for:	Discussion and Decision

- Introduction

In RAN #106, detailed scope of newWID has been given for Rel-19 further study. Here, we conclude here general scope and RAN 1 scope [1].

- RAN1 scope - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

Actually, the deployment scenario focus on below from RAN #106 discussion, and it can be seen gNB sends R2D signaling and receives corresponding D2R signal.

In this contribution, based on these agreements and FL’s suggestion, we will discuss R2D modulation related topics and CP handling.

- R2D modulation

## 2.1 R2D waveform and modulation

RAN 1 #118b has agreed CP-OFDM and DFT-s-OFDM can be used for R2D signal generation. The description is as below.

Agreement Capture in the TR: - For R2D OFDM-based OOK waveform generation, CP-OFDM and DFT-s-OFDM are both feasible - For M=1, i.e. use of OOK-1 or OOK-4 for single-chip per OFDM symbol transmission, CP-OFDM and DFT-s-OFDM are both feasible - For M>1, i.e. use of OOK-4 for M-chip per OFDM symbol transmission, DFT-s-OFDM is feasible.

Meanwhile, RAN #106 gives one limited scope as ‘R2D supports only OOK-4 modulation, one solution for CP handling’. As shown figure below based on TR 38.869[2], OOK-4 can be generated by DFT/least square and IFFT transformer. RAN 1#118b also agreed that CP-OFDM could be used for OOK-4 generation. Thus, whatever CP-OFDM or DFT-OFDM should be discussed for OOK-4 generation.

Observation 1: For M=1, OOK-4 can be generated by CP-OFDM and DFT-OFDM, and for M>1, OOK-4 can be generated by DFT-OFDM.

### 2.1.1 OOK-4 waveform

- Two agreements are listed as following for further discussing OOK-4 modulation[3].

Agreement For R2D, for the OOK-4 modulation for M-chip per OFDM symbol transmission, the maximum M value is 24. - RAN1 will further determine the set of M values up to the maximum M value. - The maximum M value applicable to the PRDCH is 24 - The maximum M value applicable to the R-TAS is not larger than 24 Agreement For R2D, at least for PRDCH, the set of M values is {2, 6, 12, 24} - FFS: whether/how CAP use same M values set as PRDCH

One problem is whether/how CAP in R-TAS use same M values {2, 6, 12, 24}. Firstly, we think M values with set {2, 6, 12, 24} is enough for PRDCH according to companies’ evaluation results and requirement for R2D data rate. In last meeting, for 9.4.3 discussing, option 1 for CAP of R-TAS from TR 38.769 is adopted where the CAP duration becomes proportionally shorter with increasing value of M. It seems like that there is no any motivation to add other M values used for CAP even if CP insertion is considered in the length of CAP duration.To reduce specification efforts, we think the set of M values {2, 6, 12, 24} is used for total R2D transmission including CAP and PRDCH.

Proposal 1: For R2D, including CAP of R-TAS and PRDCH, the set of M values is {2, 6, 12, 24} in Rel-19.

### 2.1.2 DFT-s-OFDM based R2D modulation

- In last meeting, specification of DFT-s-OFDM waveform generation for OOK-4 modulation is agreed in RAN 1[3].

Agreement For R2D transmission, from reader perspective, DFT-s-OFDM waveform is supported for OOK-4 modulation. For the details of DFT-s-OFDM waveform generation of OOK-4 modulation: - Certain specification of DFT-s-OFDM waveform generation for OOK-4 modulation is needed in RAN1. - FFS: identify what potential details of 5 steps in section 4.4 of TR 38.769 needs to be specified. - This does not preclude RAN4 to discuss R2D waveform generation themselves when defining requirement(s), if needed.

In last meeting, following one agreement has been given for DFT-s-OFDM specification

Agreement From reader perspective, for the needed certain specification of DFT-s-OFDM waveform generation for OOK-4 modulation: - An example is provided below, which does not presume any specific reader implementation: - Step 1: The time domain OOK signal is the M chips of one OFDM symbol - The specification only needs to reflect that one OFDM symbol contains M chips - Step 2: A chip is represented (e.g. upsampled) by L samples - The specification only needs to reflect that one chip contains L samples as the input to N’-points DFT - Step 3: An N’-points DFT is performed on the samples of one OFDM symbol to obtain the frequency domain signal. - The specification only needs to reflect that there is an N’-points DFT operation, where N’ = M*L - Step 4: Map the frequency domain signal obtained by N’-points DFT to the X subcarriers of Btx,R2D - The specification only needs to reflect that N’ >= X, where X is corresponding to the Btx,R2D - Step 5: An N-points IDFT is performed to obtain the time domain signal. - The specification only needs to reflect that there is an N-points IDFT operation - Note: other examples were provided in contributions to RAN1#120bis, e.g. in annex 2 of R1-2502160 - From the example above, some normative specification related to at least step 1 and step 5 are needed. - Note: RAN1 to consider whether an information annex could describe other steps - Note: some normative RAN1 specification text about waveform is assumed to be needed for RAN4 requirements definition - Note: the specification also needs to reflect the timing of the CP insertion operation.

In this subsection, we would to further discuss two problems as following

- How to handle concentrated frequency spectrum related problems

- Truncation operation after DFT operation

#### 2.1.2.1 Concentrated frequency spectrum related problems

DFT-s-OFDM has been agreed used for OOK-4 generation. Frequency spectrum energy may be concentrated at part of frequency, which will be influenced by multi-path effect. Fortunately, Rel-19 focus on the indoor study, which has less multipath effects. Thus, this issue could be discussed in later version. However, before N’-points DFT performed, concentrated frequency spectrum could impact time domain ON/OFF chips with M*L samples , e.g., no flattened waveform or with higher PAPR, which is not suitable edge detection at device side. Each ON/OFF chip with randomized amplitude or phase in baseband (after step 1) could be considered for handling concentrated frequency spectrum related problems.

Observation 2: Limited by device 1’s detection capability, before N’-points DFT performed, concentrated frequency spectrum could impact time domain ON/OFF chips with M*L samples , e.g., no flattened waveform or with higher PAPR, which is not suitable edge detection at device side.

Proposal 2: Each ON/OFF chip with randomized amplitude or phase after step 1 could be considered for handling concentrated frequency spectrum related problems.

Several methods could be considered for each ON/OFF chip with randomized amplitude or phase before DFT operation.

- If QAM, pi/2-BPSK, BPSK or other modulation are used for ON/OFF chip in baseband, randomized amplitude or phase could be achieved. For example, transmitting ON/OFF chips as 111001010100 is transformed as De{jθ4}，Ce{jθ3}，Be{jθ2}，Be{jθ2}，Bee{jθ2}，Ae{jθ1} if QAM used.

- Scrambling of ON/OFF chips or sampling points, e.g, scrambling for ON chip and no scrambling for OFF chip to randomize phase or amplitude. In our understanding, scrambling sequence could be as CAZAC sequence, ZC sequence, M/Gold sequence, (pi/2-)BPSK sequence or NR/LTE related sequence.

- Sequence types or length might be used for randomized amplitude or phase, for example, different sequence type or length could be considered for SIP and CAP of R2D preamble part. Or the relationship between adjacent OOK chips might be used for considering sequence types or length, for example, if the result of XOR operation between adjacent OOK chips is 1, sequence types or length of second chip is same as first chip.

- Predefined matrix or vector or sequence with the functionality of randomized phase or amplitude is used for multiplying ON/OFF chips.

Observation 3: Modulation like QAM/pi/2-BPSK/BPSK, scrambling of ON/OFF chips or sampling points, different sequence types or length, and predefined matrix or vector or sequence could be considered for handling concentrated frequency spectrum related problems.

Proposal 3: Consider potential methods for randomizing amplitude or phase of ON/OFF chip, e.g., modulation like QAM/pi/2-BPSK/BPSK, scrambling of ON/OFF chips or sampling points, different sequence types or length, and predefined matrix or vector or sequence, etc.

#### 2.1.2.2 Truncation operation

As truncation operation is used after DFT/least square in frequency domain, signal distortion will be generated, which impacts on the detection performance at device side, e.g., not flat ON/OFF time domain chip or with higher PAPR. To ensure the detection performance at device side, how to solve signal distortion because of truncation operation should be considered in WI phase.

Proposal 4: How to solve signal distortion because of truncation operation for DFT-s-OFDM operation should be considered in WI phase.

- As shown in figure above, one method to avoid no flattened waveform is considering two step operations for M OOK baseband chip. Assuming that L(sampling number)=1, using regulated sequence with length of Q scrambles each M ON/OFF chip (or M bit) as first step to get one new sequence with length of M*Q. Then another new sequence could be achieved if adding redundant sequence to resist on the impact of truncation. In our understanding, how to design scrambling sequence and redundant sequence and corresponding length should be carefully considered.

Proposal 5: Consider combination of scrambling and adding redundant sequence to resist on the impact of truncation.

## 2.2 R2D numerology

Last meeting agreed R2D numerology related conclusion about time domain or frequency domain aspects.

Agreement When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used. - Padding is to be down-selected among the following alternatives: - Alt 1a: The content of padding is up to reader implementation and transparent to device. - Note: the timeline determination of any timing relationship refers to the end of padding. - Note: it implies the device should be aware of the duration of padding or the last OFDM symbol boundary by implementation. FFS how accurately the device can be aware. - Alt 1b: The content of padding is up to reader implementation and transparent to device. - Note: the timeline determination of any timing relationship refers to the start of the padding. - Alt 2: Specify the detailed content of padding - Note: the timeline determination of any timing relationship refers to the end of padding. - Note: the end chip(s) of the padding content shall follow the CP handling solution determined in RAN1, and may be affected by other agreements. Agreement Proposal: For Ambient IoT, RAN1 clarify that the definition of PRB is same as NR in TS 38.211.

In 943, one agreement has given to support that SIP duration is one OFDM symbol, but two patterns

of ON-OFF or ON-OFF-ON-OFF will be down-select in RAN 1 #121.

Regardless of CAP length, padding filling may be needed if CAP is also needed to align with OFDM symbol n+1 and ensure SIP duration with one OFDM symbol. Also, padding filling may be needed for CAP if PRDCH transmission needs to align OFDM symbol boundary. To conveniently promote specification in WI phase, we prefer padding filling is up to implementation at reader side.

Additionally, we think padding duration is up to gNB implementation based on duration regulation of SIP of R-TAS. Thus, there is no motivation to consider how device accurately aware the padding duration or last OFDM symbol boundary.

Observation 4: Regardless of CAP length, padding filling may be needed if CAP is also needed to align with OFDM symbol n+1 and ensure SIP duration with one OFDM symbol.

Observation 5: Padding filling may be needed for CAP if PRDCH transmission needs to align OFDM symbol boundary.

Proposal 6: For R2D padding filling, only Alt 1b should be supported in Rel-19.

## 2.3 R2D CP handling

In RAN 1#120 meeting, candidate methods for CP handling have been agreed and listed as below

Agreement For CP handling which retains subcarrier orthogonality, the candidate methods are clarified as follows: - For Method Type 1 (as per TR 38.769) - For purposes of evaluating Method Type 1, it is assumed the device is aware of or determines the boundary of an OFDM symbol using the R2D timing acquisition signal (R-TAS), at least for Alt M1-1 - Detail is up to R2D preamble design - Using Alt M1-1 and/or Alt M1-2 is up to device implementation as per TR 38.769 - FFS: applicable M values - For Method Type 2 Alt M2-1 (as per TR 38.769) - Detail design of Alt M2-1-1 or Alt M2-1-2 from the followings: - Alt M2-1-1 - Candidate 1: - For M less than 6: Use Alt M1-1 - For M >=6: circular shift of the M chips’ samples before CP insertion at the reader side in addition to bit/chip skipping/dropping at the device side after bit decoding - Candidate 2: - Where a known signal shape in the tail-portion of the R2D signal is copied to the CP. - Candidate 3: - Insert padding chips at both start and end OOK chips or only at end OOK chips - Alt M2-1-2 - Candidate 4: - Change a=b≠c case to a=b=c case by changing OOK-4 M value to adjacent M +/- 1 for OFDM symbol n. - where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1. - Candidate 5: - Odd number of chips of PRDCH is transmitted in the first OFDM symbols, while M value is kept unchanged during the remaining OFDM symbols. - FFS: applicable M values with or without combination of Method Type 1 For CP handling which does not retain subcarrier orthogonality, the candidate methods are clarified as follows: - For Method Type 2 Alt M2-2 (as per TR 38.769) - FFS detail design of Alt M2-2 from the following: - Candidate 1: - A duration of OFDM symbol with CP has integer number of OOK symbols. CP is not copied from the end of OFDM symbol. Total 14 OFDM symbols fit in a slot. For further down-selection among candidate methods which retains subcarrier orthogonality, companies are encouraged to provide evaluation (including overhead) among the followings to RAN1#120bis: - Method Type 1 - Method Type 2 Alt M2-1-1 with or without combination of Method Type 1 - Method Type 2 Alt M2-1-2 with or without combination of Method Type 1 - Note: The evaluation for CP handling at least includes a M value from {2,4,6,8}, a M value from {12, 16}, and a M value from {24, 32} - Note: In this evaluation, the transmit power of the ON chips remains constant

In last meeting, some methods agreed in RAN 1#120 meeting has been down-selected and listed as following

Agreement For the below agreement, further update on the followings Agreement For further down-selection among CP handing which retains subcarrier orthogonality, at least for PRDCH, at least Method Type 1 is supported - For supported M values <= 12 - RAN1 will not further pursue additional CP handling design - For supported M values > 12 - RAN1 will further down-select one from the followings - Option 1: Candidate 3 of M2-1-1 (as per agreements from RAN1#120) - Insert padding chips only at the end OOK chips of OFDM symbol - The last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’ - Option 3: RAN1 will not further pursue additional CP handling design

For supported M values>12, like M=24, two options (option 1 and option 3) has been listed for down selection in RAN 1#121. For option 1, padding chips filling at both start and end OOK chips or only at end OOK chips has been proposed in RAN 1 #120. As mentioned in R2D numerology discussion, we prefer padding operation is up to gNB implementation. However, option 1 requires the last 2 out of M OOK chip at the end of OFDM symbol are always ON, which may add other efforts in Rel-19 specification phase.

For option 3, we think it is reasonable to support CP handling when M>12. Thus, we prefer supporting option 3 in Rel-19.

Observation 6: For option 1, it requires the last 2 out of M OOK chip at the end of OFDM symbol are always ON, which may add other efforts in Rel-19 specification phase.

Proposal 7: Support option 3 when M values>12:

- Option 3: RAN1 will not further pursue additional CP handling design

## 2.4 R2D chip duration

- 2.4.1 How to derive R2D chip duration

- In RAN 1#120 meeting and #120b meeting, FL has concluded some potential methods to derive R2D chip duration (copied as below)

[Closed][M] Proposal 6.2.1a-v1: At least from reader perspective, depending on CP handling and/or M values (if supported), the length of a R2D chip duration (noted as ‘C’) can be derived from the followings: - Option 1: C = (1/SCS) / M - Option 2: C = ceil {(1/SCS) / M} or C = floor {(1/SCS) / M} - Option 3: C = (1/SCS + CP) / M - Option 4: C = (1/SCS) / M + CP - Option 5: Dedicated chip duration C is introduced to ensure no additional transition edge introduced by CP - FFS: from device perspective, whether to define the following on top of above options - C − delta_C<= Cd <= C + delta_C - Where, delta_C is a floating chip duration and FFS: delta_C/C #### [Closed][H] Proposal 4.2.1a-v1: Proposal: For R2D chip duration, at least for chips non-adjacent to a CP, the length of R2D chip duration (denoted as ‘C’) is defined as ‘C = (1/SCS)/M’ - FFS: In addition, from device perspective, RAN1 further discuss whether needs to define a floating R2D chip duration from the followings - Option 1: ‘C+delta_C’ or ‘C-delta_C - FFS: the ratio of ‘delta_C/C' - Option 2: ‘ceil(C/Ts)*Ts’ or ‘floor(C/Ts)*Ts’ - Where ‘Ts’ is from the same concept defined in 38.211

- Obviously, regardless of CP handling, R2D chip duration equals to one OFDM symbol duration divided by M. Thus, option 1 is fine for us to drive R2D chip duration.

- Additionally, considering the impact of SFO, we prefer the definition of C − delta_C<= Cd <= C + delta_C and FFS delta_C/C.

- Proposal 8: Support following methods for R2D chip calculation,

- C = (1/SCS) / M , where C represents a R2D chip duration

- C − delta_C<= Cd <= C + delta_C, where, delta_C is a floating chip duration and FFS: delta_C/C

- Conclusion

In this contribution, we provide our views on the feasibility and required functionalities of physical modulation for AIoT. The observations and proposals are listed as below:

Observation 1: For M=1, OOK-4 can be generated by CP-OFDM and DFT-OFDM, and for M>1, OOK-4 can be generated by DFT-OFDM.

Observation 2: Limited by device 1’s detection capability, before N’-points DFT performed, concentrated frequency spectrum could impact time domain ON/OFF chips with M*L samples , e.g., no flattened waveform or with higher PAPR, which is not suitable edge detection at device side.

Observation 3: Modulation like QAM/pi/2-BPSK/BPSK, scrambling of ON/OFF chips or sampling points, different sequence types or length, and predefined matrix or vector or sequence could be considered for handling concentrated frequency spectrum related problems.

Observation 4: Regardless of CAP length, padding filling may be needed if CAP is also needed to align with OFDM symbol n+1 and ensure SIP duration with one OFDM symbol.

Observation 5: Padding filling may be needed for CAP if PRDCH transmission needs to align OFDM symbol boundary.

Observation 6: For option 1, it requires the last 2 out of M OOK chip at the end of OFDM symbol are always ON, which may add other efforts in Rel-19 specification phase.

Proposal 1: For R2D, including CAP of R-TAS and PRDCH, the set of M values is {2, 6, 12, 24} in Rel-19.

Proposal 2: Each ON/OFF chip with randomized amplitude or phase after step 1 could be considered for handling concentrated frequency spectrum related problems.

Proposal 3: Consider potential methods for randomizing amplitude or phase of ON/OFF chip, e.g., modulation like QAM/pi/2-BPSK/BPSK, scrambling of ON/OFF chips or sampling points, different sequence types or length, and predefined matrix or vector or sequence, etc.

Proposal 4: How to solve signal distortion because of truncation operation for DFT-s-OFDM operation should be considered in WI phase.

Proposal 5: Consider combination of scrambling and adding redundant sequence to resist on the impact of truncation.

Proposal 6: For R2D padding filling, only Alt 1b should be supported in Rel-19.

Proposal 7: Support option 3 when M values>12:

- Option 3: RAN1 will not further pursue additional CP handling design

- Proposal 8: Support following methods for R2D chip calculation,

- C = (1/SCS) / M , where C represents a R2D chip duration

- C − delta_C<= Cd <= C + delta_C, where, delta_C is a floating chip duration and FFS: delta_C/C

# References

- 3GPP RAN#106. RP-243312, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR. Madrid, Spain, Dec 9-12, 2024.

- 3GPP TR38.769 V19.0.0, “Study on solutions for Ambient IoT (Internet of Things) in NR”

- 3GPP Session notes for 9.4 Solutions for Ambient IoT in NR, RAN 1#120b.