3GPP TSG RAN WG1 #121                                            R1-2503538

St Julian's, Malta, May 19th-23rd, 2025

Source:	TCL

Title:	Discussion on timing acquisition and synchronization functionalities for Ambient IoT

Agenda Item:	9.4.3

Document for:	Discussion and Decision

- Introduction

In RAN #106, detailed scope of newWID has been given for Rel-19 further study. Here, we conclude here general scope and RAN 1 scope [1].

- RAN1 scope - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

Actually, the deployment scenario focus on below from RAN #106 discussion, and it can be seen gNB sends R2D signaling and receives corresponding D2R signal.

In this contribution, we will discuss R2D and D2R frame structures to consider timing acquisition/sync/channel and interference estimation related topics.

- R2D aspects

## 2.1 Start-indicator part (SIP)

In RAN 1#120b, ON/OFF patterns and length for start-indicator part have been discussed[2].

Agreement For the pattern of SIP of R-TAS, only the following 2 alternatives are considered for further down-selection: - Alt 1-2: ON-OFF with a ratio of 1:3 and with following total SIP duration to be further down-selected: - Option 1: 0.5 OFDM symbol duration - Option 2: 1 OFDM symbol duration - Alt 2-4: ON-OFF-ON-OFF with a ratio of 1:1:1:3 and with following total SIP duration to be further down-selected: - Option 1: 0.5 OFDM symbol duration - Option 2: 1 OFDM symbol duration Agreement For R-TAS, SIP duration of 1 OFDM symbol is adopted with CAP pattern ON-OFF-ON-OFF for all values of M corresponding to PRDCH - Note: device cannot assume the presence/absence of RF transmission prior to the SIP.

- Start-indicator with ON/OFF pattern can be considered for waking-up device from OFF state to ON state and indicating the start position of clock-acquisition. It can be observed from agreement of RAN 1#120b meeting that two options including single ON-OFF or two ON-OFF could be considered for further design SIP of the R2D time acquisition signal, and the length of SIP pattern has been listed as potential selection for RAN 1#121 to down select.

- Next, we further analyze agreed two options of single ON-OFF or two ON-OFF in RAN 1#120b. It is up to the M value during one OFDM symbol, and whether SIP is aligned with the boundary of OFDM symbol. Without Manchester coding for SIP pattern, it could be more flexible to design SIP pattern whatever single ON-OFF or multiple ON-OFF. For example, if the start of SIP is not aligned with that of OFDM symbol, and M value is small (e.g., 2), we prefer single ON-OFF pattern for SIP. If the start of SIP aligned with that of OFDM symbol, and M value is relatively large (e.g., 8), we prefer multiple ON-OFF pattern for SIP.

- Observation 1: If the start of SIP is not aligned with that of OFDM symbol, and M value is small (e.g., 4), single ON-OFF pattern for SIP could be considered.

- Observation 2: If the start of SIP aligned with that of OFDM symbol, and M value is relatively large (e.g., 16), ON-OFF-ON-OFF pattern for SIP could be considered.

- Proposal 1: Support Alt 1-2 (Single ON-OFF transmission) or Alt 2-4 (ON-OFF-ON-OFF) for SIP pattern design.

- For entire length for SIP, two options have been given in last meeting, including 1/2 OFDM symbol duration or 1 OFDM symbol duration or additional duration. Device detects one fixed length of SIP (e.g., one OFDM symbol) with different R2D data rate, which might be able to reduce the detection complexity and power consumption. In our understanding, the entire length of SIP should be set at most one NR symbol length for device 1. In last meeting, it has agreed that the maximum value of M do not greater than 24 and some value may be down selected in 941 of this meeting. The length of start of indicator could be set to flexibility length related with the different number of M with one NR symbol. In our understanding, this is up the detection capability of device 1. Fortunately, another agreement has regulated SIP duration of 1 OFDM symbol in last meeting, which could be supported for specification. Here, we repeatedly support this proposal.

Observation 3: In last meeting, SIP duration with one OFDM symbol has been supported.

Proposal 2: Support the entire length of SIP set is one OFDM symbol length for device 1.

## 2.2 End of PRDCH transmission

In last online progress, end of PRDCH trnasmission has been discussed. Here, we copy this proposal as follow

(High Priority) Proposal 4-1d Manchester coding rule violation corresponding to M value for PRDCH is applied by reader’s implementation that can be used by the device to determine the end of PRDCH transmission - There is no consensus to specify R2D postamble

- Considering limited detection capability for device, we do not think it is good way for implicit indication or violation of Manchester coding rule for ensuring end of PDRCH transmission. In our understanding, explicit indication by R2D control message is fine. However, considering there is no more time to discuss this issue and there is no consensus to specify R2D postamble, so we could accept Manchester coding rule violation applied by reader’s implementation .

- Proposal 3: Support Manchester coding rule violation applied by reader’s implementation for end of PRDCH transmission.

- D2R x-ambles

RAN 1#118 has agreed D2R preamble can be considered for SFO or CFO estimation/channel estimation and interference estimation.

Agreement For D2R transmission, preamble preceding the PDRCH is studied also for the potential additional functionalities: - SFO estimation - CFO estimation - Channel estimation - Interference estimation Note: this does not preclude studying the above functionalities by using a midamble and/or postamble, if supported FFS: Other functionalities, if any

## 3.1 M sequence

As we know, M sequence has good self-correlation performance with higher main lobe and fixed peak side-lobes, and it is better method for device 1 due to its low generation complexity. NR SSB uses M sequence to generate PSS with good capability for resisting on frequency shift.

M sequence has been supported in last meeting and long or short sequence is also supported in Rel-19.

Agreement - For D2R preamble/midamble, base sequence is generated from m-sequence, where the length of the sequence is 2n−1 - Value(s) of n - Long preamble/midamble is generated based on n = 5 - Working assumption: Short preamble/midamble is generated based on n=3 - Only 1-part preamble/midamble are supported for D2R - Preamble immediately precedes the PDRCH without any gap - Both long and short preamble and midamble are supported based on the working assumption on n - when midamble is present at least the following cases are supported and reader explicitly indicates one of the following cases for PDRCH: - Short preamble and short midamble - Long preamble and long midamble Note: the case of short preamble and long midamble will not be supported - When midamble is not present the reader explicitly indicates short or long preamble for PDRCH

- order polynomial for M sequence corresponds with maximum 2^n-1 length, which is associated with D2R preamble/Midamble/Postamble (if supported), e.g., the length of one M sequence should be greater than or equal to the length of one D2R preamble/midamble.

- Like initial sequence in NR, [1,1,1,.....1] might be used for initial M sequence for device 1 with limited capability, but not precluded other potential initial M sequence. Similarly, same shift scheme for register could be used for device 1 to get feedback value.

Device only transmits RN16 in Msg 1, and in this case we think D2R preamble and one Midamble is enough to avoid in-consecutive transmission of RN16, where D2R Midamble used for indicating end of PDRCH transmission. In this way, we think M sequence length is up to the length of D2R preamble and Midamble.

For other cases like data transmission carried by PDRCH, M sequence length is up to data size and the interval between consecutive midambles, and between the preamble and the first midamble.

Observation 4: Device only transmits RN16 in Msg 1, and in this case one D2R preamble and one Midamble is enough to avoid discontinuous transmission of RN16, where D2R Midamble used for indicating end of PDRCH transmission.

Proposal 4: M sequence length like n=3 or n=5 is up to the length of D2R preamble and Midamble.

## 3.2 D2R midamble

Midamble could be used for SFO/interference estimation, and used for indicating end of PDRCH transmission as well. Two agreements have given to guide further D2R Midamble discussion including functionalities and signaling design aspects. We copy related agreements below.

Agreement For D2R midamble, for determining the presence and location of midamble(s) at the device: - Reader explicitly indicates the same interval between consecutive midambles, and between the preamble and the first midamble, via R2D control information - FFS: details of signalling - FFS: whether the reader can explicitly indicate with one bit whether a midamble is additionally present at the end - Note: This does not preclude the support of having no midamble present in the D2R transmission Agreement For indicating the interval between consecutive midambles, and between the preamble and the first midamble, via R2D control information, following is adopted: - Unit of interval is number of bits after FEC (if FEC is applied) and repetition (if repetition is applied) - FFS: the candidate values in terms of the unit of interval

In this meeting, we will further discuss 1) D2R midamble location and signaling consideration, 2) End of PDRCH transmission.

Firstly, consider D2R midamble location and signaling consideration. As shown figure below, D2R Midmable indicated by R2D control information, including the start of D2R midamble and other information like interval or the number of D2R Midamble in one Msg 1/Msg 3. Potential methods include

- For indicating the interval between consecutive midambles, we think it is better to indicate the start of D2R Midamble and Midamble number rather than indicating interval between consecutive midambles, which is beneficial to avoid discuss interval values for Msg 1 and Msg 3, e.g., only transmitting RN 16 or re-transmission with only part data (discussed in RAN 2)

- For indicating the interval between the preamble and the first midamble, i.e., the start of D2R midamble, we need to clarify the reference point is the start of preamble or the end of preamble. If start of preamble as reference point, SFO (δ) could be considered for this interval based on the compensation from reader side, e.g., the interval (△T) between the preamble and the first midamble is △T+δ or △T-δ.

- Observation 5: If start of preamble as reference point, SFO (δ) could be considered for this interval based on the compensation from reader side, e.g., the interval (△T) between the preamble and the first midamble is △T+δ or △T-δ.

- Proposal 5: For indicating the interval between consecutive midambles, consider the start of midamble and midamble number as indication messages to avoid diverse interval values used for Msg 1 or Msg 3.

- Proposal 6: For indicating the interval between the preamble and the first midamble, if start of preamble as reference point, SFO impact should be considered for this interval.

- Next, discuss the end of PDRCH transmission. To avoid other signaling overhead, we think one bit to indicate whether a midamble is additionally present at the end is not necessary. Other methods could be used for implicitly indicating the end of PDRCH transmission. For example, using special M-sequence including sequence value or length to implicitly indicate the end of PDRCH transmission.

- Observation 6: To avoid other signaling overhead, we think one bit to indicate whether a midamble is additionally present at the end is not necessary.

- Proposal 7: Do not support one bit to indicate whether a midamble is additionally present at the end.

- Proposal 8: Using M-sequence special design (e.g., sequence length) to implicitly indicate the end of PDRCH transmission.

- Another example is using predefined to indicate the end of PDRCH transmission. Like mentioned before, if Msg 1 only transmits RN 16 or one bit energy status/one bit other message (if supported), one fix structure could be considered like D2R preamble + RN16/one bit energy status/ one bit other message+D2R midamble, where miamble used for indicating the end of PDRCH transmission.

- Proposal 9: Consider using predefined method to indicate the end of PDRCH transmission, e.g., if Msg 1 only transmits RN 16 or one bit message, one fix structure could be considered like D2R preamble + RN16/one bit message+D2R midamble, where miamble used for indicating the end of PDRCH transmission.

- Conclusion

In this contribution, we provide our views on the frame structure and required functionalities of R2D/D2R signal for AIoT. The observations and proposals are listed as below:

- Observation 1: If the start of SIP is not aligned with that of OFDM symbol, and M value is small (e.g., 4), single ON-OFF pattern for SIP could be considered.

- Observation 2: If the start of SIP aligned with that of OFDM symbol, and M value is relatively large (e.g., 16), ON-OFF-ON-OFF pattern for SIP could be considered.

Observation 3: In last meeting, SIP duration with one OFDM symbol has been supported.

Observation 4: Device only transmits RN16 in Msg 1, and in this case one D2R preamble and one Midamble is enough to avoid discontinuous transmission of RN16, where D2R Midamble used for indicating end of PDRCH transmission.

- Observation 5: If start of preamble as reference point, SFO (δ) could be considered for this interval based on the compensation from reader side, e.g., the interval (△T) between the preamble and the first midamble is △T+δ or △T-δ.

- Observation 6: To avoid other signaling overhead, we think one bit to indicate whether a midamble is additionally present at the end is not necessary.

- Proposal 1: Support Alt 1-2 (Single ON-OFF transmission) or Alt 2-4 (ON-OFF-ON-OFF) for SIP pattern design.

Proposal 2: Support the entire length of SIP set is one OFDM symbol length for device 1.

- Proposal 3: Support Manchester coding rule violation applied by reader’s implementation for end of PRDCH transmission.

Proposal 4: M sequence length like n=3 or n=5 is up to the length of D2R preamble and Midamble.

- Proposal 5: For indicating the interval between consecutive midambles, consider the start of midamble and midamble number as indication messages to avoid diverse interval values used for Msg 1 or Msg 3.

- Proposal 6: For indicating the interval between the preamble and the first midamble, if start of preamble as reference point, SFO impact should be considered for this interval.

- Proposal 7: Do not support one bit to indicate whether a midamble is additionally present at the end.

- Proposal 8: Using M-sequence special design (e.g., sequence length) to implicitly indicate the end of PDRCH transmission.

- Proposal 9: Consider using predefined method to indicate the end of PDRCH transmission, e.g., if Msg 1 only transmits RN 16 or one bit message, one fix structure could be considered like D2R preamble + RN16/one bit message+D2R midamble, where miamble used for indicating the end of PDRCH transmission.

# References

- 3GPP RAN#106. RP-243312, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR. Madrid, Spain, Dec 9-12, 2024.

- R1-2503123, Final FL Summary on timing acquisition & synchronization for Ambient IoT (9.4.3), RAN 1 #120b, Wuhan, China.