3GPP TSG RAN WG1 #121		R1-2503542

St Julian's, Malta, 19 - 23 May, 2025

Agenda item:	5

Source:	Samsung

Title:	Discussion on the startSFN parameter for positioning SRS with TX frequency hopping

Document for:	Discussion and decision

# Introduction

In RAN1#121, RAN2 sends following LS to RAN1 on the the startSFN parameter for positioning SRS with TX frequency hopping. This tdoc is to discuss such issue and try to get reply to RAN2.

# startSFN parameter for positioning SRS with TX FH

The original content of the LS is as following

1. Overall Description: According to the RAN1 higher layers parameter list, RAN2 implemented the parameters startSFN, periodicityAndOffset and duration in IE SRS-PosUplinkTransmissionWindowConfig in TS 38.331. However, there are different understandings in RAN2 on the need and interpretation of the parameter startSFN, which leads to different understanding on the actual time start of the UTW. Therefore, RAN2 would like to ask RAN1 the following questions with regards to the configuration of the parameter startSFN for the UTW: Q1: Is the parameter startSFN needed for the UTW? Q2: If answer to Q1 is ‘yes’, how this parameter should be interpreted? 2. Actions: To RAN WG1: ACTION: RAN2 kindly asks RAN WG1 to answer the above questions and provide feedback.

Interestingly, when review the past agreement from RAN1, there were two agreements related

Agreement from RAN1#114 The UL time window for UL SRS for positioning with Tx hopping can be configured to be periodic with configurable starting SFN, slot and symbol number, periodicity, duration - FFS values for starting SFN, slot and symbol number, periodicity and duration Agreement from RAN1#114bis With regards to the configuration of the UTW: - the window parameters for periodicity and starting slot offset have the same candidate values as the periodicity and starting slot offset parameters for the SRS for positioning in the IE PeriodicityAndOffset - the duration of the window in slot is {1,2,4,6} slots

So by procedure wise, since RAN1 already agreed to support “configurable starting SFN”, so the answer to Q1 is Yes.

Regarding the usage of such parameter, it should be straightforward that it indicates the which SFN the window is started. One potential ambiguity could be that if someone follows the SRS configuration and its usage of PeriodicityAndOffset , it will get the SFN 0 as the natural start. But we see this is the different from exact SRS resource, the candidate value of SRS configuration is reused but it did not mean the start of UTW should be the same.

So the answers to the two question are in following proposal:

Proposal 1: RAN1 adopts following answers to RAN2 LS questions and feedback to RAN2:

Q1: Is the parameter startSFN needed for the UTW?

A: yes.

Q2: If answer to Q1 is ‘yes’, how this parameter should be interpreted?

A: such parameter indicates that starting from which SFN the determination/derivation of the UTW will conducted.

# Conclusion

The contribution discusses the time period determination, and the proposal is summarized below.

Proposal 1: RAN1 adopts following answers to RAN2 LS questions and feedback to RAN2:

Q1: Is the parameter startSFN needed for the UTW?

A: yes.

Q2: If answer to Q1 is ‘yes’, how this parameter should be interpreted?

A: such parameter indicates that starting from which SFN the determination/derivation of the UTW will conducted.

# References

- R1-2503206, “LS on the startSFN parameter for positioning SRS with TX frequency hopping”, 3GPP TSG RAN WG1 Meeting #122-bis, Malta, May 19th – 23rd, 2025