3GPP TSG RAN WG1 Meeting #121	R1-2503565

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.3.3

Source:	Samsung

Title:	Cross-link interference handling for SBFD

Document for:	Discussion

# Introduction

The Rel-19 WID Evolution of Duplex operation introduces NR specification support for gNB-side SBFD within a TDD carrier, enhancements to inter-gNB and inter-UE CLI handling, and BS RF and RRM requirements [1].

In the Rel-19 WID, it is assumed that SBFD is deployed at the gNB side and that the TDD UE operates in half-duplex. Subband full-duplex (SBFD) operation on a time-domain resource is limited to FDM in non-overlapping DL and UL subbands.

SBFD enhancements for SBFD-aware UEs assume SBFD operation for NR single carrier in FR1 or FR2-1. From the perspective of the SBFD-aware UE, SBFD operation is assumed within a single configured DL and UL BWP pair with aligned center frequencies. For the semi-static indication of frequency/time locations of the SBFD subbands to SBFD-aware UEs on a legacy DL or F symbol/slot, it is assumed that one UL subband is configured for SBFD operation in an SBFD symbol.

The revised Rel-19 WID objectives include introduction of specification support for cross-link interference handling [1].

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

  - Information exchange among gNBs, including [RAN3]

    - (…)

  - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

    - (…)

  - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4]

    - (…)

  - Note: Without dedicated optimization for dynamic/flexible TDD.

In Section 2 we discuss some remaining details for UL muting. In Section 3 we discuss some remaining details for L1-based UE-to-UE CLI measurements and reporting.

# UL resource muting for PUSCH

## Configuration and indication

The following FL proposal was discussed in RAN1#120bis

#### Proposal 1-1

UL resource muting can be applied in non-SBFD symbols.

Note: No additional specification impact is expected to support above.

In our view, there is no need to restrict the possible configuration of UL muting for PUSCH to SBFD symbols in the UL subband. UL muting as CLI mitigation feature is equally beneficial for d/f-TDD operation where an UL reception in an UL slot can be interfered by the DL transmissions from the neighbour cells.

We do not see any additional specification impact to support this case. If UL muting for PUSCH in non-SBFD/normal UL slot was excluded, this would result in additional specification effort. For example, the corresponding DCI reception behavior would then need to be described, i.e., excluding applicability of any indication of presence of the 1 or UL muted symbols in the non-SBFD symbols/slots, or adding additional provisions for Rel-19 SBFD parameter ul-MutingIndicator.

Proposal 1: UL resource muting can be applied in non-SBFD symbols.

## PTRS

With respect to UL muting and PTRS, the following earlier RAN1 agreements were made.

Agreement (RAN1#119)

If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS.

Agreement (RAN1#119)

3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting.

- FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

Include the above in the LS to RAN4. In addition, mention that RAN1 is still discussing the boosting in symbols containing PTRS.

Agreement (RAN1#120)

For the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled, 3 dB power boosting for both PUSCH and PTRS REs if present, regardless of the PTRS density K and number of PUSCH layers L.

PTRS, if present, are separately RRC configured for the UE by the network. A potential collision between a muted RE in an RB of a muted UL symbol and a PTRS RE can occur depending on the PTRS time-domain and frequency domain configuration.

In the CP-OFDM case, such a potential frequency-domain collision case can be considered from the RE-level multiplexing perspective. In the DFTS-OFDM case, presence of the transform precoder requires a different consideration, i.e., because the PTRS are “spread” across frequency-domain.

For the CP-OFDM case, the revised WID [1] already assumed that the PTRS RE is transmitted and the UE doesn’t apply the UL resource muting on such an RE. When transform precoding is not present, i.e., for CP-OFDM, a collision of PTRS REs with the muted REs of a muted UL symbol can be avoided for many (but not all) PTRS configurations by the gNB. For example, the gNB can set the time-domain density of the PTRS configuration for the UE such that muted UL symbols and PTRS symbols don’t overlap. Also, the RRC configurable comb offset {0, 1} for the muted UL symbol can be used to set the even/odd subcarrier muting of muted RE such that depending on the PUSCH DMRS port assignment which determines the PTRS RE offset value, no collision occurs. Such an approach is however burdened with a certain complexity due to its impact of RE level mapping the PUSCH processing chain. The RAN1#119 agreement was made that “the UE does not expect the muted REs overlap the REs occupied by PT-RS”.

For the DFTS-OFDM case, difficulties result to apply this principle. When transform precoding is present, all subcarriers after the DFT “carry” PTRS information and accordingly, all subcarriers should be transmitted “as is”. If some muted RE subcarriers then need to be dropped, PTRS (de-)mapping is then necessarily affected. For example, MscPUSCH in table 6.4.1.2.2.2-1 of TS38.211 would need to be changed to MscPUSCH/2.

For handling of the DFT-size when transform precoding is enabled, two options were provided in RAN1#120 for the purpose of specification drafting.

Agreement (RAN1#120)

The following agreement replaces the earlier agreement from RAN1#119

If transform precoding is enabled for a PUSCH, the following options are provided for the purpose of specification drafting (how UE implementation is done is left to each company).

- Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting

  - The MscPUSCH2 modulated symbols are transformed to the frequency domain with a DFT of size MscPUSCH2, and the result is mapped to the MscPUSCH2 unmuted scheduled subcarriers.

- Option 2: The DFT size is MscPUSCH for symbol(s) with UL resource muting

  - Option 2A: The block of MscPUSCH2 modulated symbols is repeated before DFT. After DFT, the MscPUSCH2 non-zero even-indexed subcarriers are extracted and then mapped to the unmuted scheduled subcarriers.

  - Option 2B: The block of MscPUSCH2 modulated symbols is extended to  MscPUSCH modulated symbols by inserting ‘0’ after every modulated symbol. After DFT, the first MscPUSCH2 subcarriers are extracted and then mapped to the unmuted scheduled subcarriers.

Note: MscPUSCH is the total number of subcarriers of the PUSCH.

Note: Above options are mathematically equivalent.

Note: It is up to the spec editor how to capture the above as long as the specification captures the above in a mathematically identical manner.

On symbols with PTRS, for example, using ½ DFT size would then also impact the PTRS sample positions and different PUSCH symbols, i.e., normal symbols and muted UL symbol of the PUSCH allocation can then have different PTRS sample positions. In our view, such an approach would be associated with significant implementation complexity and specification effort. Reducing the DFT size on some symbols where PTRS occurs together with UL resource muting in the PUSCH would have very large implementation impacts and should be avoided.

The following agreement was then made in RAN1#120bis for down-selection in RAN1#121.

Agreement (RAN1#120bis)

When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, down-select from the following alternatives in RAN1#121

- Alt-3

  - For PT-RS insertion in a block of MscPUSCH2 samples prior to transform precoding

    - NgroupPT-RS and Nsampgroup are determined as legacy from 38.214 Table 6.2.3.2-1

    - PT-RS sample positions are determined as legacy, but only for the first NgroupPT-RS2 groups

    - Note: After PT-RS insertion, all of the mathematically equivalent DFT Options 1, 2A, and 2B from the RAN1#120 agreement are applicable, where the choice is up to UE implementation

- Alt-4

  - UL resource muting is not applied in the PUSCH symbol containing PT-RS

Alt-3 introduces a whole new PTRS pattern (Figure 1) where (NgroupPT-RS,NsampPT-RS)=2,2,2,4, 4,2, 4,4,8,4 and NscPUSCH=120  (10RBs) by puncturing. Most specifically, PT-RS pattern is first derived by using MscPUSCH and only half of PT-RS samples are used. Thus, the result is the punctured PT-RS pattern. The signal with comb-2 muting can be represented as the block of MscPUSCH2 modulated symbols repeated before DFT. In Alt-3 as by the example in Figure 1, the PT-RS pattern within the first 60 samples is repeated and thus a PT-RS pattern within 120 samples results.

Figure 1: PT-RS pattern in Alt-3

We observe that

- If (NgroupPT-RS,NsampPT-RS)=2,2 or 4,2, the PTRS pattern of Alt-3 is exactly same as the legacy PTRS pattern.

- However, if (NgroupPT-RS,NsampPT-RS)=2,4 or 4,4 or 8,4, the PTRS pattern of Alt-3 is different from the possible legacy PTRS patterns. The PTRS pattern of Alt-3 is therefore new and has not been used before. Actually, the legacy PTRS pattern is mirrored at the middle of samples and the first samples and the last samples are used for PTRS and the remaining PTRS groups are equally spread over all samples. However, for the new resulting PTRS pattern in Alt-3, these characteristics do not hold any more, i.e., they are no longer equally spread over all samples.

From the approved Rel-19 WID objectives [1], we also note that there is no objective to support any new PT-RS pattern, especially by means of puncturing.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

  - (…)

  - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

    - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

    - PUSCH resource mapping, i.e., rate-matching around the muted REs

    - UCI resource determination in symbols with muted REs

  - (…)

In the last RAN1#120bis meeting, a company argued “PUSCH resource mapping, i.e., rate-matching around the muted REs” would include the new PT-RS pattern. However, it is clearly stated in the WID [1] that rate-matching is assumed, not puncturing.

Therefore, we think Alt-3 which would result in new/additional non-legacy PTRS patterns for cases such as if (NgroupPT-RS,NsampPT-RS)=2,4 or 4,4 or 8,4 is out-of-scope for the Rel-19 Duplex WI.

- Observation 1: Alt-3 can result in new/additional non-legacy PT-RS patterns due to puncturing half of the PT-RS samples. This is out-of-scope as by approved Rel-19 WID objectives.

- Proposal 2: Do not introduce any new/additional PT-RS patterns in Rel-19.

It also needs to be considered that for the case where PTRS is most relevant, i.e., FR2, gNB-gNB CLI is not significant based on the Rel-18 evaluation results. It is then not meaningful to design for a complex solution with HW impact for PTRS handling with DFT-S-OFDM and muted UL symbol collisions when an alternative is simply to rely on other CLI handling schemes. For FR2, sharing strongest beam information is a very viable and effective CLI mitigation solution. The focus for CLI equalization design should therefore be on FR1.

Another consideration for FR2 is that a 1-symbol gap is usually available between PUSCH transmissions with the different (analog) beams to provide sufficient beam switching time. If CLI equalization needs to be implemented by the network, gNB-gNB CLI can still be estimated on transparent muted UL symbol positions and using 13-symbol PUSCH allocations, or using the REs corresponding to REs of a CDM group without data in the DMRS symbols.

Therefore, we do not agree with the assumption that UL muting on a PTRS symbol with transform precoding necessarily needs to be supported, in particular when considering UE implementation challenges for the Alt-1 to Alt-3 (RAN1#120).

- Proposal 3: When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, support:

- Alt-4: UL resource muting is not applied in the PUSCH symbol containing PT-RS.

## Additional restrictions and UE features

In order to keep the expected specification effort in Rel-19 SBFD commensurate when introducing UL resource muting for PUSCH, a number of restrictions and conditions were already captured by the revised WID [1].

In our view, additional restrictions or conditions of UL resource muting for PUSCH are motivated by operational assumptions and should be captured as part of the UE features/components for Rel-19 UL muting.

The following FL proposal was made in RAN1#118bis.

FL proposal 1-7 (RAN1#118bis)

Study the necessity of following conditions to apply UL resource muting for a PUSCH

- Condition 1: When the number of PUSCH symbols is less than threshold, UL resource muting is not applied

- Condition 2: If the scheduled PUSCH uses a MCS lower than certain reference MCS index, UL resource muting is not applied

- Condition 3: When the number of scheduled RBs is less than threshold, UL resource muting is not applied

- Condition 4: When the number of DMRS symbols is larger than threshold, UL resource muting is not applied

- Condition 5: UL resource muting is not applied according to the PHY priority of PUSCH

- Condition 6: For PUSCH that is a retransmission, UL resource muting is not applied

- Condition 7: For PUSCH that is scheduled by DCI format 0_0, UL resource muting is not applied

- Condition 8: for PUSCH with repetition type B, UL resource muting is not applied

- Condition 9: For PUSCH carrying certain UCI type with high PHY priority, UL resource muting is not applied.

- Other conditions are not precluded.

Condition 1: UL resource muting is not motivated for short PUSCH time-domain allocations, e.g., <7 symbols.

The gNB can create a scheduling gap, i.e., insert an un-scheduled symbol between back-to-back PUSCH allocations in the slot (Figure 1). The DMRS symbol and the un-scheduled/transparent muting symbol are then separated only by at most 4 symbols using PUSCH mapping type A. For PUSCH mapping type B, even more flexibility exists and the un-scheduled/transparent muting symbol of the short PUSCH allocation can occur before the first symbol of the PUSCH TDRA in the slot.

The resulting computational delay between CLI estimation and PUSCH equalization is deemed acceptable. It is not necessary to defer CLI equalization during PUSCH Rx processing until the end of the NR slot compared to the case when the un-scheduled/transparent symbol is allocated as last symbol of the slot.

Figure 2: No support for UL resource muting for PUSCH <7 symbols

Another reason to only support UL resource muting for long PUSCH time-domain allocations is that more flexibility exists to avoid impacts to the UCI mapping and in particular the A/N carrying REs following the FL DMRS symbol(s). It becomes more easily possible to configure the comb-2 muted symbol after the UCI carrying symbols in the PUSCH allocation. MCS selection during gNB scheduling and the TBS determination for the PUSCH can more easily absorb the presence of comb-2 muted REs on a symbol in the long PUSCH allocation, i.e., the number of muted REs is small when compared to the total number of REs allocated for PUSCH NRE.

- Proposal 4: UL resource muting is not applied to PUSCH allocations <7 symbols.

(Modified) Condition 9: For PUSCH carrying certain UCI type with high PHY priority, UL resource muting is not applied.

With respect to UCI handling and UL muting, the following latest RAN1 agreements were made.

Agreement (RAN1#118bis)

For UCI rate matching, to determine the number of coded modulation symbols per layer for UCI transmission on PUSCH, the following option is adopted

- Option 2: The number of coded modulation symbols per layer for UCI on PUSCH is calculated based on MscUCIl after excluding the UL muting resource.

Agreement (RAN1#119)

Confirm the following working assumption

Working Assumption

UL resource muting is allowed on PUSCH symbols that carry UCI, i.e., UCI is not mapped on muted REs

Note: As mentioned in the WID, no specification changes are allowed on data and control multiplexing in section 6.2.7, TS 38.212.

It is assumed by the revised WID that no impact on data and control multiplexing as defined in section 6.2.7 “Data and control multiplexing” in TS 38.212 occurs. One reason is to keep the Rel-19 specification effort commensurate.

As by the RAN1#119 agreement, UL resource muting is allowed on PUSCH symbols that carry UCI, i.e., UCI is not mapped on muted REs.

However, while a direct specification impact and corresponding change to implementation for the UCI coding in 38.212 Section 6.2.7 can be avoided, there are indirect impacts on existing specifications.

Any configuration of a comb-2 muted PUSCH symbol on a symbol containing the CSI or A/N still requires to address the UCI resource determination. This is because several loop variables such as the set of REs available for transmission of UCI in the OFDM symbol or the computation of the MscUCI(l) still need to account for absence of every 2nd RE. Some impact, i.e., new adjustment factor, therefore result in section 6.2.7 of 38.212. For example, in the Rel-15 NR specifications, loop variables are set to 0 for symbols carrying the DMRS.

It should also be considered that for the mapping of the UL muting symbol after the FL DMRS (for PUSCH mapping type A and B), configuration of the muted symbol to A/N-carrying symbols can be avoided in principle. The size of the A/N payload in a PUSCH is typically small even if A/N is frequently present.

Depending on the semi-static rate-matching factor settings, i.e., beta value, it is not typical that the channel coded A/N bits map to more than one data symbol after the FL DMRS in the PUSCH allocation. Configuration and indication of the UL muting symbol on the next following data symbol where no A/N UCI is present is then a viable option.

If the UL muting symbol occurs before the FL DMRS (for PUSCH mapping type A), its symbol location would necessarily overlap with a CSI Part 1 or 2, if present. However, it is not a reasonable assumption that CSI is present in every PUSCH allocation. Aperiodic CSI reporting is usually only triggered at reasonably large reporting intervals, e.g., every several tens of msec’s. Periodic CSI reporting due to overhead is often configured at larger intervals. Therefore, even if the CSI payload can potentially become large, i.e. up to 2000 bits or more, depending on the configured CSI reporting mode, the CSI reporting in the PUSCH allocation is infrequent.

The need to design for UL muting before the FL DMRS on CSI carrying symbols in the PUSCH allocation for mapping type A can simply be avoided by not applying the UL muting in the occasional PUSCH where a CSI is present. Note that the muted symbol after FL DMRS can still be used, i.e., UL muting is always available to equalize for PDSCH-PUSCH CLI even if the additional ability to equalize for PDCCH-PUSCH CLI is then lost for a few intermittent PUSCH transmissions where CSI reporting needs to occur.

In summary, we think that mandatory support of UL muting on PUSCH symbols that can carry UCI is not required. For Rel-19 UEs not supporting such a functionality, the gNB can configure time-domain locations of UL muting symbols based on the max. UCI payload assumption and accounting for the beta offsets. When the UE not supporting UL muting on PUSCH symbols that can carry UCI determines presence of UCI on such a symbol resulting from the control/data multiplexing for PUSCH, the UE does not apply the UL muting the in the symbol.

- Proposal 5: Support of UL muting on PUSCH symbols that can carry UCI is an optional UE capability in Rel-19 SBFD.

- Proposal 6: When the UE does not indicate support for UL muting on PUSCH symbols that can carry UCI, and the UE determines presence of UCI on a configured/indicated UL muting symbol, the UE does not apply the UL resource muting in the symbol.

(New) Condition 10: UL resource muting should not be supported for PUSCH with UCI-only payload.

Even with a combined CSI and A/N payload size of ~2000 bits, the resulting PUSCH allocation would typically not exceed a few symbols over the PUSCH allocation BW. In absence of data payload in the PUSCH, the resulting specification effort to support a comb-2 muted UL symbol increases. For UCI-only PUSCH, a transparent muting symbol can be created by the gNB as scheduling gap following the UCI-only PUSCH similar to the case of short PUSCH allocations.

- Proposal 7: UL resource muting is not applied to PUSCH carrying UCI-only payload.

# L1 based UE-to-UE measurement and reporting

The following latest RAN1 agreements were made.

Agreement (RAN1#118)

For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- FFS: Method#3: UE measures RSSI within UL subband

Agreement (RAN1#119)

For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported

- Method #3: UE measures RSSI within UL subband

- A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig.

Agreement (RAN1#120)

Update the following agreement in RAN1#118 (in red)

For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported

- Measurement resource type#1: OneThe CLI-RSSI measurement resource is configured within a DL subband or a UL subband

- Measurement resource type#2: OneThe CLI-RSSI measurement resource is configured across two DL subbands

- FFS: Number of resources that can be configured

FFS: UE behavior for measurement

Agreement (RAN1#120bis)

Configuration of Method 1 or Method 3 are not explicitly provided for L1 CLI-RSSI measurement and reporting. Configuration of Measurement resource type #1 or Measurement resource type #2 are not explicitly provided for L1 CLI-RSSI measurement.

- CLI-RSSI measurement resource is configured in one UL subband only, in one DL subband only, or across two DL subbands only.

  - If CLI-RSSI measurement resource is configured across two DL subbands, the frequency resources of CLI-RSSI measurement resource are derived by excluding the frequency resources outside DL usable PRBs.

One open issue is how to account for the number of supported L1 CLI-RSSI measurement resources by the UE in terms of resource counting on the SRS-RSRP and CLI-RSSI measurement resources.

In particular, for the case of measurement resource type#2 where the CLI-RSSI measurement resource is configured across two DL subbands, it should be avoided to increase the UE complexity when compared to Rel-16 CLI-RSSI reporting.

Another open issue is the maximum number of L1-CLI-RSSI or the L1-SRS-RRSP measurement resources that can be configured.

If a UE supports Rel-16 CLI-RSSI reporting, the UE indicates the maximum number of measurement resources configurable for CLI-RSSI measurement using maxNumberCLI-RSSI-r16. If a UE supports Rel-16 SRS-RSRP reporting, the UE indicates a maximum number of measurement resources across all CCs configurable for the SRS-RSRP measurement using maxNumberCLI-SRS-RSRP-r16, and a maximum number of measurement resources across all CCs configured for SRS-RSRP measurement within a slot using maxNumberPerSlotCLI-SRS-RSRP-r16.

Above were discussed in RAN1#119 and RAN1#120 but without reaching a conclusion/agreement.

FL proposal 2-5 Round 3 (RAN1#119)

The maximum configured number of L1 CLI-RSSI measurement resources in CLI-RSSI-MeasurementResourceSet is same as maxNrofCLI-RSSI-Resources-r16.

- The supported number of CLI-RSSI measurement resources is up to UE capability

The maximum configured number of L1 SRS-RSRP measurement resources in SRS-RSRP-MeasurementResourceSet is same as value of maxNrofSRS-RSRP-Resources-r16.

- The supported number of SRS-RSRP measurement resources is up to UE capability

#### FL proposal 2-3 Round 1 (RAN1#120)

For L1-CLI-RSSI measurement and reporting, a UE reports the maximum number of supported CLI-RSSI measurement resources of all three measurement types across all CCs (sum of measurement resource type#1, measurement resource type#2 and measurement resource type#3)

From the signaling perspective, the same maxNrofCLI-RSSI-Resources can be supported as in Rel-16, i.e., 64.

For Method#1 (RSSI in DL subband) and measurement resource type#1 where the CLI-RSSI measurement resource is configured within a DL subband, the existing Rel-16 CLI-RSSI measurement resource is reused, i.e., configured as a number of contiguous RBs with start PRB and a number of PRBs as integer multiple of 4.The UE can measure and report L1-CLI-RSSI for as long as the CLI-RSSI measurement resource is configured by the gNB within the SBFD DL subband and within the active DL BWP of the UE.

For Method#1 (RSSI in DL subband) and measurement resource type#2 where the CLI-RSSI measurement resource can be configured across two DL subbands, the Rel-16 CLI-RSSI measurement resource is configured as a number of contiguous RBs with start PRB and a number of PRBs as integer multiple of 4. The UE discards, i.e., does not consider the RBs falling into the SBFD UL subband and/or guard band(s) if present for the L1-CLI-RSSI measurement. The SBFD-aware UE should then follow the same approach in terms of resource counting as for the CSI-RS resources and CSI reporting in the general case. The UE implementation can implement a measurement resource type #2 as two separate CLI-RSSI measurement processes, i.e., one for each DL subband, therefore separate counting is required.

For Method#2 (RSRP in UL subband) or Method#2 (RSSI in UL subband), the configured SRS or RSSI measurement resource is contiguous, and the existing Rel-16 behavior can be followed.

- Proposal 8: The CLI-RSSI measurement resource type#2 is counted as two L1 CLI-RSSI measurement resource type#1.

- Proposal 9: The maximum configurable number of L1 CLI-RSSI measurement resources in CLI-RSSI-MeasurementResourceSet is same as maxNrofCLI-RSSI-Resources-r16.

- Proposal 10: The supported number of CLI-RSSI measurement resources is subject to UE capability.

- Proposal 11: The maximum configurable number of L1 SRS-RSRP measurement resources in SRS-RSRP-MeasurementResourceSet is same as value of maxNrofSRS-RSRP-Resources-r16.

- Proposal 12: The supported number of SRS-RSRP measurement resources is subject to UE capability.

Another aspect to consider is measurement resource configuration for the possible case that Rel-16 L3 CLI reporting and the Rel-19 SBFD L1 CLI reporting are both/concurrently configured for the UE. For example, from the L1 perspective, a configured CLI-RSSI measurement resource may then be used to generate a sample for the periodic L3 CLI report or for aperiodic L1 reporting. A limitation to one/same CLI-RSSI resource instead of two concurrent ones is needed.

- Proposal 13: Down-select from the following 2 options

- Option 1: the UE doesn’t expect to be configured for Rel-16 L3 CLI reporting and Rel-19 L1 reporting.

- Option 2: when the UE is configured for both Rel-16 L3 CLI reporting and Rel-19 L1 reporting, the UE expects to be configured with a maximum number of CLI-RSSI or SRS-RSRP measurement resources according to the reported Rel-19 UE capability.

The following RAN1 agreement was made.

Agreement (RAN1#120)

If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for aperiodic CLI reporting and the first symbol of the aperiodic SRS-RSRP measurement resource or CLI-RSSI measurement resources is smaller than the UE reported threshold beamSwitchTiming, down-select from the following alternatives:

- Alt.1: UE does not expect to be configured with a scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming.

- Alt.2: Same default beam rules as AP CSI-RS resource defined in 38.214 can be applied to AP CLI SRS-RSRP or CLI-RSSI resource.

FFS: whether Alt.1 or Alt.2 has any specification impact.

In our view, the default rule according to Alt.2 can be applied.

- Proposal 14: If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for aperiodic CLI reporting and the first symbol of the aperiodic SRS-RSRP measurement resource or CLI-RSSI measurement resources is smaller than the UE reported threshold beamSwitchTiming, support Alt.2.

The following agreements were made in RAN1#120.

Agreement (RAN1#120

For L1 CLI measurement and reporting, the number of occupied CPU(s) is determined as follows

- OCPU=1 for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’, ‘cli-SRS-RSRP’.

Agreement (RAN1#120bis)

The existing CSI timeline for L1 RSRP reporting is reused for the aperiodic CLI measurement and reporting in Rel-19 SBFD.

- FFS: whether the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured shall not be smaller than 1.

In our view, a slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured shall not be smaller than 1. One reason is that switching and preparation time from the measuring victim UE perspective is not sufficient for same slot reception of the SRS transmitted by the aggressor UE in the UL subband.

- Proposal 15: The slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured shall not be smaller than 1.

# Conclusion

In this contribution, we made the following observations and proposals for UL resource muting for PUSCH:

Proposal 1: UL resource muting can be applied in non-SBFD symbols.

- Observation 1: Alt-3 can result in new/additional non-legacy PT-RS patterns due to puncturing half of the PT-RS samples. This is out-of-scope as by approved Rel-19 WID objectives.

- Proposal 2: Do not introduce any new/additional PT-RS patterns in Rel-19.

- Proposal 3: When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, support:

- Alt-4: UL resource muting is not applied in the PUSCH symbol containing PT-RS.

- Proposal 4: UL resource muting is not applied to PUSCH allocations <7 symbols.

- Proposal 5: Support of UL muting on PUSCH symbols that can carry UCI is an optional UE capability in Rel-19 SBFD.

- Proposal 6: When the UE does not indicate support for UL muting on PUSCH symbols that can carry UCI, and the UE determines presence of UCI on a configured/indicated UL muting symbol, the UE does not apply the UL resource muting in the symbol.

- Proposal 7: UL resource muting is not applied to PUSCH carrying UCI-only payload.

In this contribution, we made the following proposals for L1 based UE CLI measurement/reporting:

- Proposal 8: The CLI-RSSI measurement resource type#2 is counted as two L1 CLI-RSSI measurement resource type#1.

- Proposal 9: The maximum configurable number of L1 CLI-RSSI measurement resources in CLI-RSSI-MeasurementResourceSet is same as maxNrofCLI-RSSI-Resources-r16.

- Proposal 10: The supported number of CLI-RSSI measurement resources is subject to UE capability.

- Proposal 11: The maximum configurable number of L1 SRS-RSRP measurement resources in SRS-RSRP-MeasurementResourceSet is same as value of maxNrofSRS-RSRP-Resources-r16.

- Proposal 12: The supported number of SRS-RSRP measurement resources is subject to UE capability.

- Proposal 13: Down-select from the following 2 options

- Option 1: the UE doesn’t expect to be configured for Rel-16 L3 CLI reporting and Rel-19 L1 reporting.

- Option 2: when the UE is configured for both Rel-16 L3 CLI reporting and Rel-19 L1 reporting, the UE expects to be configured with a maximum number of CLI-RSSI or SRS-RSRP measurement resources according to the reported Rel-19 UE capability.

- Proposal 14: If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for aperiodic CLI reporting and the first symbol of the aperiodic SRS-RSRP measurement resource or CLI-RSSI measurement resources is smaller than the UE reported threshold beamSwitchTiming, support Alt.2.

- Proposal 15: The slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured shall not be smaller than 1.

# References

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”; RAN#104

- TR38.858 v2.0.0 “Study on Evolution of NR Duplex Operation (Rel-18)”; RAN#102

- R1-2503543 “Discussion on RAN4 LS on the transient period for SBFD operation”; Samsung

- R1-2504003 “Discussion on RAN2 LS on simultaneous configuration of SBFD and DC”; Samsung

- R1-2503563 “SBFD operation and procedures”; Samsung

- R1-2503564 “Random access on SBFD resources”; Samsung

- R1-2502367 “Cross-link interference handling for SBFD”; Samsung

- R1-2503593 “UE features for Rel-19 Duplex Evolution for NR”; Samsung