3GPP TSG RAN WG1 #121                                  			               R1-2503815

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.11.4

Source:	InterDigital, Inc.

Title:	IoT-NTN uplink capacity/throughput enhancement

Document for:	Discussion

# Introduction

In RAN #102 meeting, the WI for R19 NR NTN for Phase 3 [1] is approved, revised in RAN #106 meeting as [2] to change the study to normative phase, and further revised in RAN #107 as [3].

For UL capacity enhancement, IoT NTN WID has the following objective to improve the UL capacity:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction [RAN2]: - Msg3 transmission without msg1/RAR - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

The following agreements were reached in RAN1 #120Bis [4] meeting:

Agreement For the 3.75kHz SCS symbol-based OCC scheme, the granularity of spreading for data is one symbol. Agreement Dynamic activation / deactivation of OCC is supported by DCI. - FFS: details of signalling by DCI Conclusion RAN1 assumes no specification change to support pairing UEs with different modulation orders. Agreement For 3.75kHz SCS OCC for NPUSCH format 1, RAN1 down selects between the following mappings between DMRS sequence samples and active TDM DMRS slots: - Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots - Option 2: Dropping of samples of the original DMRS sequence in blanked slots Agreement For NPUSCH Format 1 single-tone 15kHz SCS, for CDM DMRS with legacy pattern: - DMRS symbols are spread before the OCC is applied - Option 1_1: according to the formula: ru,OCCMn+m=runqm, 0≤n<X/M, m=0, …, M-1 Where: M is the OCC length, q is the assigned OCC codeword for the UE, run is the reference signal sequence defined in TS36.211 section 10.1.4.1.1 and X is the total number of slots in the NPUSCH transmission after OCC is applied Agreement The OCC sequence index is signalled using DCI format N0. Agreement The RRC parameters for IoT-NTN UL capacity enhancements are: RAN2 Parent IE; Parameter name in the spec; New or existing?; Description; Value range; UE-specific or Cell-specific. NPUSCH-ConfigDedicated-NB; npusch-OCC-Enabled; new; The parameter is used to enable OCC for NPUSCH format 1 single tone.; ENUMERATED {‘true’}; UE-specific. Agreement For indicating OCC sequence index and activation / deactivation, the following fields may be repurposed or constrained to be not present in the DCI: - Modulation and coding scheme - Repetition number - Redundancy version - Number of scheduled TB for Unicast - Subcarrier indication - Resource reservation

In this contribution, we discuss some aspects of UL capacity enhancement for IoT NTN devices.

# Uplink Capacity Enhancement for IoT NTN

Rel-19 WID for IoT NTN [3] has specified the objective of uplink capacity enhancement through the use of orthogonal cover codes if found beneficial.

Wide satellite beams, large number of UEs, IoT support and spectrum scarcity all lead to very limited uplink resource availability per UE for uplink transmission. Due to limited Tx power of the IoT devices and very large propagation distance, the uplink channels have always been considered a bottleneck for NTN networks. To ensure sufficient uplink coverage, for devices based on NB-IoT and eMTC, several PUSCH repetitions are performed typically to achieve successful detection at the network nodes even for terrestrial networks. Large number of PUSCH repetitions inherent to NB-IoT and eMTC UL transmissions lead to very poor uplink throughput and capacity.

We discuss the capacity enhancement for NPUSCH and NPRACH in the following sub-sections.

## OCC for NPUSCH Format 1

The NPUSCH is dedicated to the transmission of data and control information from the UE side. In this respect, two formats are supported: Format 1 for data and Format 2 for control information (i.e., ACK/NACK). Rel-19 scope for IoT NTN UL capacity enhancement is limited to NPUSCH Format 1. NPUSCH Format 1 supports both single tone and multi-tone based transmissions. Single tone transmissions can use 3.75 kHz and 15 kHz SCS whereas multi-tone transmissions can only use 15 kHz SCS.

3GPP introduces a concept of “Resource Unit (RU)” for both single tone and multi-tone transmissions in NB-IoT. A “resource unit (RU)” represents the smallest unit that can be transmitted in uplink regardless of the subcarrrier spacing being used and depends only on the transmission option (i.e., single-tone or multi-tone). For the single-tone transmissions using 15 kHz or 3.75 kHz spacing, one RU will have exactly the same number of REs (112 REs). For multi-tone transmissions, one RU will have 168 REs.

In the case of single-tone transmission, using a subcarrier spacing of 3.75 kHz or 15 kHz, the repetition process is the same for both SCSs. Thus, if NRU x NUL slots of NPUSCH data are to be transmitted where NRU is the number of resource units and NUL is the number of slots in the resource unit, and NPUSCH is scheduled to be repeated NRep times, the process is carried out as follows: One repetition comprising of NRU x NUL slots of data are first transmitted, then another NRU x NUL slots are transmitted and so on up to the NRep repetitions.

As the current NB-IoT design does not support multiple orthogonal DMRS ports for NPUSCH Format 1, to enable UE multiplexing for OCC based transmissions orthogonal DMRS ports need to be assigned to the UEs. RAN1 is considering DMRS patterns for UE multiplexing, e.g., TDM patterns with muting and CDM patterns.

RAN1 #120 made the following agreement to support TDM DMRS in 3.75 kHz SCS:

Working assumption

For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.

Send LS to RAN4 asking whether there would be any issue (e.g. phase continuity) for supporting such TDM DMRS for IoT NTN.

RAN1 #120Bis made the following agreement for the TDM DMRS design in 3.75 kHz SCS on how to map the DMRS samples and how to blank the DMRS in the blank slots:

Agreement

For 3.75kHz SCS OCC for NPUSCH format 1, RAN1 down selects between the following mappings between DMRS sequence samples and active TDM DMRS slots:

- Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots

- Option 2: Dropping of samples of the original DMRS sequence in blanked slots

During the offline and online discussions in RAN1 #120Bis, many companies expressed concerns with option 1, that it may generate interference for neighbouring cells while option 2 results in DMRS reception at the base station similar to the legacy design. For this reason, we support to adopt option 2 for 3.75 kHz SCS OCC for NPUSCH format 1.

Proposal 1: For TDM DMRS in 3.75kHz SCS OCC for NPUSCH format 1, support option 2, i.e., dropping of samples of the original DMRS sequence in blanked slots.

# Conclusions

In this contribution, the following proposals are made for IoT NTN UL capacity enhancements:

Proposal 1: For TDM DMRS in 3.75kHz SCS OCC for NPUSCH format 1, support option 2, i.e., dropping of samples of the original DMRS sequence in blanked slots.

# Reference

- RP-234077, “New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, RAN #102, December 2023.

- RP-243278, “Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, RAN #106, December 2024

- RP-250472, “Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, RAN #107, March 2025

- RAN1 Chairman’s note, RAN1 #120Bis, April 2025.

- RAN1 Chairman’s note, RAN1 #119, November 2024.