[3GPP TSG-RAN WG1 Meeting #121	 R1-2503978

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.2.4

Source:	Ericsson

Title:	Remaining issues on asymmetric DL sTRP UL mTRP scenarios

Document for:	Discussion, Decision

# 1	Introduction

Asymmetric DL sTRP UL mTRP with its justification and objective has been endorsed as one of the features for NR MIMO Phase 5, see also the revised WID from the RAN#105 meeting [1].

Justification:

Heterogeneous Network can be deployed to improve UL throughput. Since the macro gNB and micro nodes differ in power rating, a UE may receive DL transmission from the macro gNB, but transmit UL to either the macro gNB or non-co-located micro nodes in order to maximize UL throughput. As an option to further reduce energy consumption, the micro nodes can, for instance, reduce or even turn off DL transmissions. To support such deployment scenario, enhancements on UL power control (PC) are needed at least. First, when pathloss RS is transmitted from the macro gNB and the UE transmits UL to the micro nodes, the pathloss measured from the pathloss RS from the macro gNB is not accurate. Therefore, it is necessary to configure the UE with pathloss offset to facilitate accurate calculation of the pathloss associated with the micro nodes. Second, an additional SRS closed-loop PC for DL CSI acquisition to the macro gNB (for DL transmission), separate from that for the SRS to the micro nodes (for UL mTRP reception) should be introduced. Therefore, there is a need for supporting two closed-loop PC adjustment states for SRS, both separate from PUSCH.

Objective:

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this contribution we share our view on the remaining issues for asymmetric DL sTRP and UL mTRP scenarios.

# 2	Discussion

Figure 1 illustrates an asymmetric DL sTRP UL mTRP deployment. It consists of an Anchor TRP (providing both DL and UL service), and a TRP which will be called an UL-only TRP (for ease of reference) to provide UL service, where the downlink reference signals, the downlink control channels, and the downlink data transmissions for a UE are only provided from the Anchor TRP.

Figure 1: UL-only TRP in an asymmetric DL sTRP UL mTRP deployment

The main motivation for asymmetric DL sTRP UL mTRP deployment is to improve UL throughput by UE transmitting UL data to the UL-only TRP. For example, the Anchor TRP in Figure 1 provides coverage with both DL and UL transmissions, and the UL-only TRP is deployed to improve UL performance. Other motivations would be to reduce network energy consumption, by avoid sending downlink data from the UL-only TRP. It is envisioned that a UE in the cell may first perform initial access via the Anchor TRP, and then, if the UE is closer to the UL-only TRP than to the Anchor TRP (i.e. better UL link budget), or if the NW decides to move the UE to an UL-only TRP, e.g., to reduce the UL interference for the Anchor TRP, or to improve resource usage efficiency, the NW may direct the UE to transmit towards the UL-only TRP.

Enhancement on power control is needed to support the asymmetric deployment. Apart from the power control aspect, how to set the UL transmission timing towards the Anchor TRP and UL-only TRP respectively is another important aspect. In a practical deployment the propagation delay differences and synchronization timing error between the Anchor TRP and the non-collocated UL-only TRP can be significantly large that will impact the performance of the asymmetric TRP deployment.

## 2.1	Unified TCI framework for asymmetric mTRP deployment

According to the WID [1], the Rel-19 asymmetric DL sTRP and UL mTRP deployment assumes the Rel-17/18 unified TCI framework and fully re-uses the legacy QCL/UL spatial relation rules, targeting both FR1 and FR2. The following agreements on the TCI framework related to pathloss offset of open-loop power control for the UL-only TRP were achieved in the RAN1#116-RAN1#117 meetings [2]-[4].

Agreement (RAN1#116)

For the asymmetric DL sTRP/UL mTRP deployment scenarios, support to associate a UL TCI state with a PL offset:

- When a UL TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this UL TCI state.

  - Reuse the legacy uplink power control formulation by replacing legacy PL with UL PL which is derived from the DL PL RS and the PL offset.

  - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW.

- Note: it does not intend to increase the number of maintained PLs per cell.

- FFS: whether to support associating joint TCI state (if supported) with a PL offset.

Further study whether/how to apply a PL offset on PDCCH-order PRACH transmission too.

- FFS: how to determine the Tx beam of PRACH towards UL TRP

- Note: this does not imply to support 2 TA for single-DCI based system.

Agreement(RAN1#116)

For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate DL/UL TCI state mode of Rel-17/18 unified TCI framework can be configured for both FR1 and FR2.

- Joint TCI state mode can be configured at least for FR1

Agreement(RAN1#116bis)

For FR1, a joint TCI state can be associated with a PL offset.

- When a joint TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this joint TCI state.

  - Reuse the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset.

  - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW.

Conclusion (RAN1#117)

For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework:

- When rel-17 unified TCI/ICBM is configured:

  - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied.

  - For FR2: {one DL TCI state + one UL TCI state} can be applied.

- When rel-18 unified TCI is configured:

  - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied.

  - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

### 2.1.1 Application of Rel-18 unified TCI framework to asymmetric DL sTRP/ UL mTRP deployment

According to the agreements from the RAN1#116 and RAN1#116bis meetings, associating PL offset with a TCI state for the asymmetric DL sTRP/ UL mTRP deployment can be applied to both FR1 and FR2 for the ‘separate’ DL/UL TCI state mode, but only to FR1 for the “joint” TCI state mode. In the RAN1#117 meeting, RAN1 concludes that when the Rel-18 unified TCI is configured, {one DL TCI state + up to two UL TCI states} can be applied. The wording “apply” in conclusion is however ambiguous for us to interpret the expected UE behaviour. In TS 38.214 [5] the wording for describing UE behaviour with respect to TCI states uses “configured”, “indicated”, “update”. With this said, we think, without further clarification, the wording “can be applied” in the conclusion (RAN1#117) may lead to the following two different understanding alternatives:

- Alt 1: {one DL TCI state + up to two UL TCI states} can be configured and indicated, or

- Alt 2: 2 separate DL/UL TCI state pairs can be configured and indicated, but only {one DL TCI state + up to two UL TCI states} are used

In our view, both alternatives can work, and both need further clarifications in the specification.

In Rel-18 unified TCI framework, a TCI codepoint associated with one DL TCI state is only used to updating one of the two indicated DL TCI states for mTRP operation, where the prerequisite for the updating behaviour is that UE “having two” indicated DL or joint TCI states. Text from TS 38.214 [5] is quoted below as reference.

“When a UE is configured with dl-OrJointTCI-StateList and is having two indicated TCI-states, if the UE receives a TCI codepoint mapped with a sub-set of first and second TCI-State(s) and/or a sub-set of first and second TCI-UL-State(s), the UE shall update the first/second TCI-State(s) and/or first/second TCI-UL-State(s) mapped to the TCI codepoint, when applicable, and keep the previously indicated first/second TCI-State(s) and/or first/second TCI-UL-State(s) that is/are not updated by the TCI codepoint.”

In our understanding, Alt 1 is not supported by the Rel-18 unified TCI framework, since “having two” DL TCI states is the prerequisite of updating indicated TCI codepoint. Recall the discussion in Rel-18 unified TCI framework, only pairs of UL/DL TCI states are supported by Rel-18 ,i.e., either there is one indicated DL TCI state and one indicated UL TCI state, or there are two indicated DL TCI state and two indicated UL TCI states. Although via MAC-CE (namely, Enhanced unified TCI states activation MAC CE for Separate TCI states, see TS.38.321) a codepoint of the Transmission Configuration Indication (TCI) bitfield in DCI formt 1_1 can be assocaited with different number of UL TCI states and DL TCI states, such codepoint is used to only update the TCI states that are associated with the codepoint, and leave the remaining indicated TCI states unchanged.Therefore Alt 1 is not supported by current specification. Additionally for Alt 1, clarification on MAC CE updating TCI state(s) for {one DL TCI state + up to two UL TCI states} need to be discussed.

Meanwhile, Alt 2 requires a clarification on whether or not the asymmetric DL sTRP/ UL mTRP deployment can be indicated with two different DL TCI states. Allowing only same DL TCI states would reduce the UE implementation cost for supporting asymmetric mTRP.

Based on the above observations, we propose

RAN1 to clarify the conclusion when Rel-18 unified TCI is configured and {one DL TCI state + up to two UL TCI states} can be applied, which of the two understanding alternatives is meant by “applied”:

- Alt1: {one DL TCI state + up to two UL TCI states} can be configured and indicated

- Alt2: separate DL/UL TCI state pairs can be configured and indicated, but only {one DL TCI state + up to two UL TCI states} are used

If Alt1 is correct, support configuration and indication of {one DL TCI state + up to two UL TCI states}

If Alt2 is correct, clarify whether or not the asymmetric DL sTRP/UL mTRP deployment can be indicated with two different DL TCI states

In the RAN1#120bis meeting, how to indicate TCI states for asymmetric DL sTRP/UL mTRP was discussed offline [6]. According to the feature leader summary, in the asymmetric DL sTRP/UL mTRP scenario, the NW should be able to indicate one DL TCI state and two UL TCI states, and a text proposal to TS 38.214, see below, has been discussed during offline.

5.1.5 Antenna ports quasi co-location … When a UE is configured with dl-OrJointTCI-StateList or TCI-UL-State and is having two indicated TCI-states or TCI-UL-States, if the UE receives a TCI codepoint mapped with a sub-set of first and second TCI-State(s) and/or a sub-set of first and second TCI-UL-State(s), the UE shall update the first/second TCI-State(s) and/or first/second TCI-UL-State(s) mapped to the TCI codepoint, when applicable, and keep the previously indicated first/second TCI-State(s) and/or first/second TCI-UL-State(s) that is/are not updated by the TCI codepoint. …

In our view, the feature leader proposal in [6] is aligned to Alt1 in Proposal 1, hence we propose:

RAN1 agree to support Alt1, i.e., configuration and indication of {one DL TCI state + up to two UL TCI states} and capture the following TP in TS 38.214

5.1.5 Antenna ports quasi co-location … When a UE is configured with dl-OrJointTCI-StateList or TCI-UL-StateList and is having two indicated TCI-states or two indicated TCI-UL-States, if the UE receives a TCI codepoint mapped with a sub-set of first and second TCI-State(s) and/or a sub-set of first and second TCI-UL-State(s), the UE shall update the first/second TCI-State(s) and/or first/second TCI-UL-State(s) mapped to the TCI codepoint, when applicable, and keep the previously indicated first/second TCI-State(s) and/or first/second TCI-UL-State(s) that is/are not updated by the TCI codepoint. …

## 2.2	Power Control Enhancement

In this section we discuss the remaining issues related to power control enhancement for asymmetric mTRP deployment.

### 2.2.1 Configuration of Pathloss (PL) Offset (for PUSCH/PUCCH/SRS)

In the legacy unified TCI state framework, a PL-RS can be associated with an UL TCI state through RRC configuration, and the UE is expected to use the PL-RS which is configured to the indicated UL TCI state when transmitting UL signals/channels according to the indicated UL TCI state (and the PL-RS configured in a UL TCI state cannot be update with a MAC-CE). In UL-only TRP operation, since the UE can only measure PL-RS transmitted from the Anchor TRP, and thereby estimate pathloss towards the Anchor TRP, in order to determine proper output power targeting the UL-only TRP, it is efficient that the pathloss estimation towards the Anchor TRP reflects the actual pathloss. When the UE moves around in a cell in FR2, the downlink beam and the associated DL TCI state from the Anchor TRP may need update frequently. In a similar way, the pathloss reference signal, that the UE uses from the Anchor TRP also needs to be updated frequently, since pathloss reference signals are expected to be beamformed in FR2 (for example the pathloss reference signal can be one of the beamformed SSBs or TRSs). However, since the pathloss reference signal associated with the UL TCI state targeting the UL-only TRP is RRC configured (and can currently not be updated with a MAC-CE), we would need RRC signaling to update the pathloss reference signal every time the UE change e.g. serving SSB/TRS/DL TCI state towards the Anchor TRP, which would generate a significant amount of overhead signaling and latency.

For FR2, when a UE moves around in the cell and need to update the serving Anchor TRP beam, the pathloss reference signal configured in an UL-TCI state used for UL transmission towards the UL-only TRP need to be updated with RRC signaling, which generates a significant amount of overhead signaling and latency.

To solve this issue, it would be preferred if the PL-RS associated with an UL-TCI state targeting UL-only TRP could be linked with a DL-RS associated to an indicated DL TCI state, thus the pathloss reference signal used for UL-only TRP will get updated automatically and follow the change of indicated DL TCI. With this approach, the output power towards UL-only TRP will be automatically updated when the UE moves around and changes serving Anchor TRP beams without the significant overhead and latency introduced by higher layer signaling to update the PL-RS for the UL-TCI state to follow the rapid DL beam change.

Linking the PL-RS of UL-TCI state with a DL-RS associated with an indicated DL TCI state enables automatic update of the PL-RS of UL transmission when UE changes DL TCI states and thus significantly reduces signaling overhead.

We therefore propose:

Support that PL-RS of an UL TCI state to be linked with a DL-RS associated with an indicated DL TCI state.

A candidate option to the same issue mentioned above is to introduce MAC-CE-based updates of a path loss reference signal associated with an UL TCI state used for an UL-only TRP. If the indicated DL TCI state from the Anchor TRP is changed for a UE (due to e.g. UE movement, blocking etc), it is likly that the path loss between the Anchor TRP and the UE also has changed. Hence, in this case, both the path loss offset and the path loss reference signal for the indicate UL TCI state used by the UE towards the UL-only TRP needs to be updated (since the path loss reference signal typically also is beamformed to the UE, at least in FR2). Therefor, one overhead efficient way to solve this could be to introduce the possibility to optionally update the path loss reference signal of an UL TCI state with the same MAC-CE that is used to update the path loss offset for the UL TCI state.

Support that PL-RS of an UL TCI state associated with a UL-only TRP can be update with the same MAC-CE that updates the path loss offset.

### 2.2.3 Application time for PL offset

As per the previous agreement, PL offset can be updated by MAC CE, hence, RAN1 shall define the time required for computing the updated pathloss after receiving MAC CE updated pathloss offset. If the MAC CE does not simultaneously change the PL-RS, UE should be able to estimate the updated pathloss based on the PL-RS and PL offset indicated, within the MAC CE processing delay. Typically, MAC CE processing delay is assumed as 3Nslotsubframe,µ in RAN1. In other words, when a UE receives a MAC CE containing PL offset at slot n, UE shall be able to apply the updated pathloss at slot n+3Nslotsubframe,µ.

When a MAC CE containing PL offset is received at slot n, UE shall be able to apply updated pathloss at slot n+3Nslotsubframe,µ.

## 2.3	Two-TAs

In the RAN#105 meeting, the scope of Rel-19 asymmetric mTRP is extended to include “2TA without CoresetPoolIdx association for asymmetric DL sTRP/UL mTRP”, see below the updated WID[1]:

Objective:

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

The following agreement related to two-TA for the UL-only TRP were achieved in the RAN1#118bis,  RAN1#120 and RAN1#120bis meetings [7]-[9] .

Agreement(RAN1#118bis)

Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios:

- Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

- One downlink reference timing is supported and applied to both TAGs.

  - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG

- One single n-TimingAdvanceoffset is configured and applied to both TAGs.

- Any of the TCI states can be associated with any one of the two TAGs.

- The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA

- The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA.

- Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA

  - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped.

- FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

Agreement(RAN1#120)

Study whether/how to reset the CLPC of PUSCH/PUCCH/SRS based on the following alternatives (including if any enhancement is necessary and other alternatives are not precluded)

- Alt1: For a UE configured with two TAGs, when the UE receives a RAR of a PDCCH-order PRACH, according to the new 1-bit DCI field PL offset indication in DCI format 1_0:

  - When there is one indicated joint/UL TCI state in Rel-17 Unified TCI:

    - If the 1-bit DCI field is 0, the UE resets the CLPC that is different from the CLPC associated with the indicated joint/UL TCI state.

    - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the indicated joint/UL TCI state.

  - When there are two indicated joint/UL TCI states in Rel-18 Unified TCI:

    - If the 1-bit DCI field is 0, the UE resets the CLPC associated with the first indicated joint/UL TCI state;

    - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the second indicated joint/UL TCI state

- Alt 2: For a UE configured with two TAGs, one TAG ID is indicated in the RAR

  - The UE resets the closed loop power control adjustment state associated with a joint/UL TCI state associated with the indicated TAG.

Agreement(RAN1#120bis)

Answer the question of RAN4 as follows:

- The RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are NOT applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s).

Agreement(RAN1#120bis)

- For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing

- For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs:

  - The UE maintains two downlink reference timings;

  - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length.

  - The reference point for PRACH transmission is indicated as follows:

    - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

    - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG

  - Above applies for the case UE is configured with SSB-MTC-additionalPCI

Agreement(RAN1#120bis)

Reply to RAN4 LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP/ UL mTRP is agreed in R1-2503091

### 2.3.1 Practical synchronization error

To enable two TAs with respect to two TRPs, UE need to obtain DL reference timing and UL TA command towards UL-only TRP. Next, we further discuss the aspects of DL reference timing and UL TA towards the UL-only TRP. The two-TA feature in Rel-18 assumes non-ideal synchronization (i.e., TRPs slot boundaries are not perfectly time aligned and there can be synchronization error of up to e.g., 3µs) between the TRPs. As per the WID, asymmetric and non-collocated deployment is supported for UL-only TRP. That means the synchronization assumptions in Rel-18 are applied for Rel-19 UL-only TRP also as operators may not deploy separate TRPs for using the UL-only TRP feature (e.g., operators may use same deployment for Rel-18 and Rel-19).

As UL-only TRP do not transmit any SSB, UE may have to rely on Anchor TRP SSB to derive the DL reference timing. When slot boundaries of Anchor TRP and UL-only TRP are perfectly aligned in time (i.e., synchronization error is 0µs), UE may use the DL reference timing of Anchor TRP to perform random access procedure and obtain TA command to handle the asymmetric propagation delay difference between Anchor TRP to UE and UL-only TRP to UE.

However, in the practical deployments (such asymmetric deployments considered in this WI) it may be sometimes impossible to maintain perfect synchronization between different TRPs even for intra-DU deployments, since most of the NW deployments use switched fronthaul technic between non-collocated TRPs. Figure 2 illustrates the synchronization scenario where the timing of UL-only TRP is ahead of Anchor TRP. When UL-only TRP is ahead of Anchor TRP slot timing, when UE acquires DL timing from Anchor TRP and transmits PUSCH using the same DL reference timing, it may result in inter-slot interference at UL-only TRP.

Figure 2: Synchronization scenario UL-only TRP ahead of Anchor TRP slot timing

As per TS 38.133, initial transmit timing for RACH transmission, UE needs to acquire frame boundary within ±Te error (Te is much smaller compared to possible synchronization error between TRP), and transmits RACH after acquiring DL synchronization, RACH preamble is always expected to receive after the start of the RACH slot at the NW.

To analyze the issue with practical synchronization error for UL-only TRP, we consider three possible synchronization scenarios as shown in Figure 3. When the UL slot boundaries of anchor TRP and UL- only TRP are not perfectly synchronized (scenario 2 and 3) as shown in Figure 3, using DL timing of anchor TRP to transmit RACH preamble to UL TRP may result in early arrival of RACH preamble or later arrival of the RACH preamble at the UL- only TRP.

Figure 3: Different synchronization scenarios at UL- only TRP and anchor TRP

When UL- only TRP is behind anchor TRP slot timing, when UE acquires DL timing from anchor TRP and transmit RACH using the same DL reference timing, it may result in early arrival of RACH preamble at UL- only TRP (i.e., before the start of RACH occasion or RACH slot at the reception window at the UL TRP as shown in Figure 4). This results in inter-slot interference at the UL- only TRP and UL TRP may not be able to determine the timing advance correctly for the UE.

Figure 4: RACH arrival at UL- only TRP and Anchor TRP with anchor TRP DL reference timing

When UL- only TRP is ahead of anchor TRP slot timing, when UE acquires DL timing from anchor TRP and transmit RACH using the same DL reference timing, it may result in late arrival of RACH preamble at UL- only TRP. Based on the below table showing guard period (GP) for different PRACH preamble formats, we think late arrival may not be an issue for the UL- only TRP deployment scenario where the UE is expected to be close to UL- only TRP and GP is large compared to synchronization error.

Table 1: Guard period for different PRACH preamble formats

| Preamble format | 0 | 1 | 2 | 3 | B1 | B2 | B3 | B4 | C0 | C2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GP in µs | 97 | 716 | 953 | 97 | 2.3 | 7 | 11.7 | 25.8 | 37 | 94 |
| Propagation delay (in km) that GP can cover | 29km | 215km | 285km | 29km | 0.7700m | 2.1km | 3.7km | 7.5km | 11km | 28km |

Based on the above discussion, we have the following proposal to consider practical synchronization error between Anchor TRP and UL-only TRP for UL and DL timing control in an asymmetric deployment, which for example could be handled by introducing a timing offset between the Anchor TRP and the UL-only node, which the network signals to the UE such that the UE can determining one reference timing for the Anchor TRP and a second reference timing for the UL-only node.

Study whether and how to address practical synchronization error between Anchor TRP and UL-only TRP for UL and DL timing control in asymmetric DL sTRP UL mTRP deployment.

- FFS: UE can be provided with a DL reference timing offset. DL reference timing for communication with UL-only TRP for the UE is obtained from the DL reference timing of the anchor TRP and DL reference timing offset.

## 2.4 SRS resource set not configured with followUnifiedTCI-StateSRS

In the recent RAN1#120bis meeting [9], the following agreement is achieved regarding how to determine the PL offset, the CPLC adjustment state and how to apply TAG ID for an SRS resource set not configured with followUnifiedTCI-StateSRS.

Agreement(RAN1#120bis) For an SRS resource set not configured with followUnifiedTCI-StateSRS, regarding how to determine the PL offset and the CLPC adjustment state - Alt1: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) and no additional enhancement. When the UE is configured with Rel-19 2 TA, the above design is also applied to TAG.

Text proposals for 38.213 related to the above agreement are provided below:

TP1:

## 4.2	Transmission timing adjustments

A UE can be provided a value NTA,offset of a timing advance offset for a serving cell by n-TimingAdvanceOffset for the serving cell. If for a serving cell the UE is provided two coresetPoolIndex values 0 and 1 for first and second CORESETs, or is not provided coresetPoolIndex value for first CORESETs and is provided coresetPoolIndex value of 1 for second CORESETs, the UE can be provided first and second NTA,offset values by n-TimingAdvanceOffset and n-TimingAdvanceOffset2 for transmissions with first and second spatial filters associated with first and second TCI statesfor the first and second CORESETs, respectively.

If a UE operates with two TAGs on a serving cell and is not provided coresetPoolIndex, the UE can be provided a value NTA,offset by n-TimingAdvanceOffset for transmissions with first and second spatial filters associated with first and second joint TCI states provided by dl-OrJointTCI-StateList or with first and second UL TCI states provided by ul-TCI-State-List, respectively.

If a UE operates with two TAGs and is not configured with coresetPoolIndex for the active BWP of a serving cell, and if followUnifiedTCI-StateSRS is not provided for a SRS resource set, tag-Id-ptr for determining timing adjustment is provided by the TCI-state configured to the SRS resource with lowest SRS-ResourceId in the SRS resource set.

## ---------------------------------------<Unchanged text omitted>-----------------------------

# 7	Uplink Power control

---------------------------------------<Unchanged text omitted>-----------------------------

In the remaining of this clause, except for clause 7.7.3, if in a PUSCH, PUCCH, or SRS transmission occasion i a UE applies a TCI-State or a TCI-UL-State that includes pl-Offset with value PLb,f,c,offset in dB [11, TS 38.321], the UE sets PLb,f,cqd=PLb,f,cqd-PLb,f,c,offset.

In the remaining of this clause, if a PDCCH reception by a UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1

-	a PDCCH monitoring occasion is the union of the PDCCH monitoring occasions for the two PDCCH candidates

-	the end of the PDCCH reception is the end of the PDCCH candidate that ends later

The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10 (except clause 10.4), 11.1, 11.1.1 and 17.2.

If followUnifiedTCI-StateSRS is not provided for a SRS resource set, PLb,f,c,offset is provided by the TCI-State configured to the SRS resource with lowest SRS-ResourceId in the SRS resource set.

---------------------------------------<Unchanged text omitted>-----------------------------

### 7.3.1	UE behaviour

## ---------------------------------------<Unchanged text omitted>-----------------------------

-	l=0 if the UE is not provided enableTwoSeparatePowerControlAdjustmentStatesForSRS; otherwise, l is provided by closedLoopIndex in p0AlphaSetforSRS associated with a TCI-State in dl-OrJointTCI-StateList or a TCI-UL-State. If followUnifiedTCI-StateSRS is not provided for a SRS resource set, l is provided by the TCI-State configured to the SRS resource with lowest SRS-ResourceId in the SRS resource set.

-	δSRS,b,f,c(m,l) is jointly coded with other TPC commands in a PDCCH with DCI format 2_3, as described in clause 11.4, or in a PDCCH with DCI format 1_1 [5, TS 38.212]

---------------------------------------<Unchanged text omitted>-----------------------------

TP2:

Agree to the text proposals TP1 and TP2 for TS 38.213.

# Conclusion

In the previous sections we made the following observations:

Observation 1	For FR2, when a UE moves around in the cell and need to update the serving Anchor TRP beam, the pathloss reference signal configured in an UL-TCI state used for UL transmission towards the UL-only TRP need to be updated with RRC signaling, which generates a significant amount of overhead signaling and latency.

Observation 2	Linking the PL-RS of UL-TCI state with a DL-RS associated with an indicated DL TCI state enables automatic update of the PL-RS of UL transmission when UE changes DL TCI states and thus significantly reduces signaling overhead.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 to clarify the conclusion when Rel-18 unified TCI is configured and {one DL TCI state + up to two UL TCI states} can be applied, which of the two understanding alternatives is meant by “applied”:

Alt1: {one DL TCI state + up to two UL TCI states} can be configured and indicated

Alt2: separate DL/UL TCI state pairs can be configured and indicated, but only {one DL TCI state + up to two UL TCI states} are used

Proposal 2	If Alt1 is correct, support configuration and indication of {one DL TCI state + up to two UL TCI states}

Proposal 3	If Alt2 is correct, clarify whether or not the asymmetric DL sTRP/UL mTRP deployment can be indicated with two different DL TCI states

Proposal 4	RAN1 agree to support Alt1, i.e., configuration and indication of {one DL TCI state + up to two UL TCI states} and capture the following TP in TS 38.214

Proposal 5	Support that PL-RS of an UL TCI state to be linked with a DL-RS associated with an indicated DL TCI state.

Proposal 6	Support that PL-RS of an UL TCI state associated with a UL-only TRP can be update with the same MAC-CE that updates the path loss offset.

Proposal 7	When a MAC CE containing PL offset is received at slot n, UE shall be able to apply updated pathloss at slot n+3Nslotsubframe,µ.

Proposal 8	Study whether and how to address practical synchronization error between Anchor TRP and UL-only TRP for UL and DL timing control in asymmetric DL sTRP UL mTRP deployment.

FFS: UE can be provided with a DL reference timing offset. DL reference timing for communication with UL-only TRP for the UE is obtained from the DL reference timing of the anchor TRP and DL reference timing offset.

Proposal 9	Agree to the text proposals TP1 and TP2 for 38.213.

# References

RP-242394, WID revision: NR MIMO Phase 5, Samsung (Moderator), RAN meeting #105 Melbourne, Australia, September 9-12, 2024

“Chairman’s Notes”, 3GPP TSG RAN1 #116, Feb 2024.

“Chairman’s Notes”, 3GPP TSG RAN1 #116bis, Apr 2024.

“Chairman’s Notes”, 3GPP TSG RAN1 #117, Apr 2024.

TS 38.214, “NR; Physical layer procedures for data”, 3GPP, V18.5.0, March 2025.

R1-2502954, Summary #2 on Rel-19 asymmetric DL sTRP/UL mTRP, Moderator (OPPO), April 2025

“Chairman’s Notes”, 3GPP TSG RAN1 #118bis, Oct 2024.

“Chairman’s Notes”, 3GPP TSG RAN1 #120, Feb 2025.

“Chairman’s Notes”, 3GPP TSG RAN1 #120bis, April 2025.