3GPP TSG RAN WG1 Meeting #121	R1-2504085

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	Fujitsu

Title:	Remaining issues on Rel-19 CSI enhancements

Agenda item:	9.2.2

Document for:	Discussion and Decision

- Introduction

In RAN plenary #102 meeting, the WI on NR MIMO Phase 5 was approved [1]. In the WI, there are five objectives. As shown in below table, objective 2 is for CSI enhancement on up to 128 CSI-RS ports, and objective 3 is for CSI enhancement on CJT.

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

In this contribution, we provide discussions on remaining issues for the CSI enhancements for up to 128 CSI-RS ports and CJT in Rel-19.

- Codebook refinement on up to 128 CSI-RS ports

2.1 IMR configuration for Rel-19 Type II Doppler codebook

In RAN1#116bis meeting as shown below, it was agreed that for up to 128 ports, only one CSI-IM or only one NZP CSI-RS for IM can be associated with all the K CMRs in the resource set.

Agreement in RAN1#116bis For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, the legacy resource configuration for interference measurement is reused, i.e. only one NZP CSI-RS resource for interference measurement or only one CSI-IM resource can be configured where the one IM resource is associated with all the K CSI-RS resources in the CSI-RS resource set for channel measurement

Based on the above agreement, it is clear for non-Doppler-based codebook since there are only K CMRs in the resource set. However, for Rel-19 Type II Doppler codebook, the number of CMR in one resource set is KDOPP*K, instead of K. Thus, based on this agreement, the IMR configuration and the association between CMR and IMR for Rel-19 Type II Doppler codebook is still unclear. To address this issue, there are two following alternatives.

- Alt 1: Only one IMR (CSI-IM or NZP CSI-RS for IM) can be configured in one resource set where one IMR is associated with all KDOPP*K CMRs.

- Alt 2: KDOPP IMRs (CSI-IM or NZP CSI-RS for IM) can be configured in one resource set, where each CSI-IM is associated with K CMRs in each CMR group among all the KDOPP CMR groups.

Fig. 1 Two alternatives on IMR configuration for Rel-19 Type II Doppler codebook

As shown in Fig. 1, compared with Alt 1, the number of IMRs for Alt 2 is extended to KDOPP rather than 1. For Type II Doppler codebook, different CMR groups represents the different time occasions for channel prediction. Thus, as the multiple IMRs are extended to be associated with each of time occasions, the accuracy of interference measurement will be increased. However, in Rel-18 Type II Doppler, only one IMR is sufficient and it is associated with all the AP CMRs within all the time occasions. Therefore, considering the alignment with the Rel-18 Type II Doppler and the overhead of IMR, Alt 1 is more preferred as the simplest method.

- For Rel-19 Type II Doppler codebook on 48, 64, 128 ports, further clarification is needed as below.

  - Only one AP CSI-IM or AP NZP CSI-RS for IM can be configured in one resource set, where one IMR is associated with all KDOPP*K CMRs.

2.2 The slot offset configuration of IMR for Rel-19 Type II Doppler codebook

For the Rel-19 regular Type-I and Type-II (except for Type-II Doppler) codebook, since one CSI-IM should be associated with K CMRs, the triggering offset of AP CSI-IM should be clarified to follow the associated CMR in the first slot.

Agreement in RAN1#120bis For the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48, 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources for CMR are within two consecutive slots, the triggering offset of the associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) for CMR in the first slot. - FFS: How to resolve this for Rel-19 Type-II Doppler

Based on the discussion on IMR configuration for Rel-19 Type II Doppler codebook in section 2.1, single IMR (CSI-IM or NZP CSI-RS for IM) can also be configured in one resource set, where one IMR is associated with all KDOPP*K CMRs. Thus, similar to Rel-19 regular Type I and Type II codebook, the triggering offset of AP CSI-IM and NZP CSI-RS for IM can also follow the associated CMR(s) in the first slot in the resource set.

- For Rel-19 Type II Doppler codebook on 48, 64, 128 ports, the triggering offset of AP CSI-IM and AP NZP CSI-RS for IM can also follow the associated CMR(s) in the first slot.

- UE reporting enhancement for CJT deployments

Regarding the linkage between Rel-19 CJTC Dd reporting and Rel-18 eType II CJT CSI reporting, one issue is about the correspondence between TRS resource set IDs and t CSI-RS resources. For this issue, there are two agreements based on previous discussions in #118bis meeting and #119 meeting.

Agreement in RAN1#118bis For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, support linking the CMRs in the two CSI Report Settings so that UE knows which CMRs in the two report settings correspond to the same TRP. - Based on a fixed correspondence between the set of TRS resource set IDs in sequential order and the set of CSI-RS resource IDs in sequential order of configuration in their respective Resource Setting FFS: linking, when the number of resources configured for CJT CSI is < number of resource sets configured for CJT Dd, in case of separate triggers Agreement in RAN1#119 For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, regarding linking the CMRs in the two CSI Report Settings, for separate triggering, the UE expects that the number of configured CSI-RS resources associated with the Rel-18 eType-II CJT CSI is always the same as the number of TRS resource sets associated with the Rel-19 CJTC Dd report.

Based on the agreements above, it is only specified that there is the fixed correspondence between the same number of TRS sets and CSI-RSs. However, there are no detailed correspondence rules addressed in the current specs. The simplest rule is just one-to-one mapping between TRS sets and CSI-RSs in the sequential order of configuration in their respective Resource Setting. As shown below in Fig. 2, 3 TRS sets for Rel-19 CJTC Dd reporting can be one-to-one mapped to 3 CSI-RSs for Rel-18 eType II CJT CSI reporting. In that case, for the future Rel-18 eType II CJT CSI measurement, UE can know and perform the delay offset compensation for each CSI-RS.

Fig. 2 One-to-one mapping between TRS sets and CSI-RSs

Based on the discussion above, beyond the fixed correspondence in current CR version, we propose that the one-to-one mapping between TRS sets and CSI-RSs should also be explicitly specified in TS 38.214. The text proposal can be considered as shown below.

--------------------------------------- Start of the TP for TS38.214 -------------------------------------------

5.2.1.4.2	Report quantity configurations

<Unrelated part omitted>

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cjtc-Dd',

-	the UE selects and reports a reference CSI-RS resource set, nref, from the NTRP configured CSI-RS resource sets for tracking.

-	The UE reports an indication of the delay offset, Dn,offset, associated with each configured CSI-RS resource set n=0,1,…NTRP-1, relative to the reference CSI-RS resource set, nref, by indicating the interval [δin,δin+1) the delay offset falls into, as described in Clause 5.2.1.4.6. For the reference CSI-RS resource set, nref, the value of Dnref,offset is assumed zero and it is not reported.

-	The UE reports a one-bit indication, dn, for each configured CSI-RS resource set n=0,1,…,NTRP-1, where dn is equal to '1' if the corresponding delay offset, relative to the reference CSI-RS resource set, nref, plus delay spread, is not larger than the length of the cyclic prefix for symbols other than 0 and 7⋅2μ, or '0' otherwise. For the reference CSI-RS resource set, nref, the value of dnref is assumed one and it is not reported.

-	Subject to UE capability, the CSI-ReportConfig with reportQuantity set to 'cjtc-Dd' can be linked, by the higher layer parameter linkedCJTCReport, to an aperiodic CSI-ReportConfig with reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-CJT-r18' and with the corresponding CSI-RS resources for channel measurement that are aperiodic and not QCLed with the same reference signal with respect to {average delay}. The CSI-RS resource sets associated with the CJTC-Dd report are linked to the CSI-RS resources associated with the 'typeII-CJT-r18' report by a fixed one-to-one correspondence between the resource set IDs and resource IDs, respectively, in sequential order of configuration in their respective Resource Setting.

--------------------------------------- End of the TP for TS38.214----------------------------------------

- In Rel-19, regarding the linkage between CJTC Dd and Rel-18 eType-II CJT CSI report, the fixed correspondence rule should be clearly defined, e.g., defined as a one-to-one mapping between TRS sets and CSI-RSs.

- Conclusions

In conclusion, we have the following proposals on Rel-19 CSI enhancements.

- For Rel-19 Type II Doppler codebook on 48, 64, 128 ports, further clarification is needed as below.

  - Only one AP CSI-IM or AP NZP CSI-RS for IM can be configured in one resource set, where one IMR is associated with all KDOPP*K CMRs.

- For Rel-19 Type II Doppler codebook on 48, 64, 128 ports, the triggering offset of AP CSI-IM and AP NZP CSI-RS for IM can also follow the associated CMR(s) in the first slot.

- In Rel-19, regarding the linkage between CJTC Dd and Rel-18 eType-II CJT CSI report, the fixed correspondence rule should be clearly defined, e.g., defined as a one-to-one mapping between TRS sets and CSI-RSs.

- References

- RP-234007, New WID: NR MIMO Phase 5, 3GPP TSG RAN Meeting #102, December 11th-15th, 2023.