3GPP TSG-RAN WG1 Meeting #121	R1-2504293

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on LS on UL Tx Switching for TEI19

Document for:	Discussion and Decision

Introduction

In the RAN4 LS [1], RAN4 provides three scenarios of UL Tx switching and asks RAN1 to introduce functionalities for scenario 1 in Rel-19 RAN1 specification and whether to support scenario 1a and/or scenario 2 in Rel-19. The three scenarios are listed as below.

- Scenario 1. Switching between the following 2 cases for 3Tx UEs configured with uplinkTxSwitchingOption set to 'dualUL’

  - Case 1-1: Two antenna ports on Band A and one antenna port on Band B

  - Case 1-2: One antenna port on Band A and two antenna ports on Band B

- Scenario 1a. Switching between the following 2 cases for 3Tx UEs configured with uplinkTxSwitchingOption set to ‘switchedUL’

  - Case 2-1: Three antenna ports on Band A and zero antenna port on Band B

  - Case 2-2: Zero antenna port on Band A and three antenna ports on Band B

- Scenario 2. Switching between the following 2 cases for 4Tx UEs configured with uplinkTxSwitchingOption set to ‘switchedUL’

  - Case 3-1: Four antenna ports on Band A and zero antenna port on Band B

  - Case 3-2: Zero antenna port on Band A and Four antenna ports on Band B

In this contribution, the necessity and benefits to introduce these three scenarios are briefly discussed, especially the scenario 1a and 2. In addition, the impacts on RAN1 specifications to introduce the scenarios are also analysed.

Discussion

Scenario 1a and Scenario 2 consider devices with demand of large uplink throughput. Especially scenario 2, 4Tx FWA devices, for example, are expected to widely used to meet the demand of growing uplink transmission. Inheriting the merit of 2Tx switching, enabling 4Tx switching between SUL/FDD/TDD bands could make the most of UL slots to reach a larger UL data rate.  For the band combinations in Table 1, the potential gain and detailed analysis can be found in [2].

Table 1. 4Tx Switching Band combination {TDD+TDD, TDD+FDD, TDD+SUL}

| switchingOptionConfigForBandPair set to 'switchedUL' | Combination #1: (TDD+TDD) | Combination #2: (TDD+FDD) | Combination #3: (TDD+SUL) |
| --- | --- | --- | --- |
| Case1 | 4Tx+0Tx | 4Tx+0Tx | 4Tx+0Tx |
| Case2 | 0Tx+4Tx | 0Tx+4Tx | 0Tx+2/4Tx |

Observation 1: UL Tx switching of scenario 2 could bring 31.7%-77.7% gain of UL peak throughput [2].

In subclause 6.1.6.3 of TS 38.214 [3], as excerpts in Appendix A, neither NUL nor SUL band is limited to 2 ports nor 2 Tx. Therefore, for scenario 1a and 2 with UL Tx switching between SUL and NUL, no RAN1 specification change is needed.

Observation 2: No spec change is needed to support scenario 1a and scenario 2 when one of the bands is SUL band.

For two-band CA combinations, in subclause 6.1.6.2.0 and 6.1.6.2.2 of TS 38.214 [3], as excerpts in Appendix B, up to 2Tx is supported. Therefore, for scenario 1a and 2 with UL Tx switching between NUL and NUL, RAN1 specification changes are needed. For scenario 1, on top of the changes of scenario 1a, more RAN1 specification changes are needed for both subclause 6.1.6.2.0 and 6.1.6.2.2.

Observation 3: RAN1 spec changes are needed to support scenario 1a and scenario 2 when switching band combination configured with uplink carrier aggregation.

Observation 4: In addition to specification changes required by scenario 1a/2, more RAN1 spec changes are needed to support scenario 1.

Based on the above analysis, we have the following proposal

Proposal: Reply the questions of RAN4 LS as following

- For scenario 1a and 2 with UL Tx switching between SUL and NUL, no RAN1 specification change is needed according to subclause 6.1.6.3 of TS 38.214.

- For scenario 1a and 2 with UL Tx switching between NUL and NUL, RAN1 specification changes are needed according to subclause 6.1.6.2 of TS 38.214.

- For scenario 1, in addition to specification changes required by scenario 1a/2, more RAN1 specification changes are needed according to subclause 6.1.6 of TS 38.214.

Conclusions

In this contribution, the following observations and proposals are made:

Observation 1: UL Tx switching of scenario 2 could bring 31.7%-77.7% gain of UL peak throughput [2].

Observation 2: No spec change is needed to support scenario 1a and scenario 2 when one of the bands is SUL band.

Observation 3: RAN1 spec changes are needed to support scenario 1a and scenario 2 when switching band combination configured with uplink carrier aggregation.

Observation 4: In addition to specification changes required by scenario 1a/2, more RAN1 spec changes are needed to support scenario 1.

Proposal: Reply the questions of RAN4 LS as following

- For scenario 1a and 2 with UL Tx switching between SUL and NUL, no RAN1 specification change is needed according to subclause 6.1.6.3 of TS 38.214.

- For scenario 1a and 2 with UL Tx switching between NUL and NUL, RAN1 specification changes are needed according to subclause 6.1.6.2 of TS 38.214

- For scenario 1, in addition to specification changes required by scenario 1a/2, more RAN1 specification changes are needed according to subclause 6.1.6 of TS 38.214.

# References

- R4-2505221, LS on UL Tx switching for TEI19, RAN4.

- R4-2504180, Discussion on introducing Tx switching for 4Tx FWA, Huawei, Hisilicon

- 3GPP TS 38.214: "NR; Physical layer procedures for data"

- Appendix A

#### 6.1.6.3 Uplink switching with two uplink bands for supplementary uplink For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, - If the UE is to transmit any uplink channel or signal on a different uplink on a different band from the preceding transmission occasion based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2, where T0 is the start time of the first symbol of the transmission occasion of the uplink channel or signal and Toffset is the preparation procedure time of the transmission occasion of the uplink channel or signal given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213], respectively. During the switching gap NTx1-Tx2, the UE is not expected to transmit on any of the two uplinks. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

- Appendix B

##### 6.1.6.2.0 Uplink switching with two uplink bands For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch or uplinkTxSwitchingPeriod2T2T for a band combination, and if it is for that band combination configured with uplink carrier aggregation: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): - When the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 1-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - When the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 2-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - For the UE configured with uplinkTxSwitchingOption set to 'switchedUL', when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. <--texts omitted--> - If uplinkTxSwitching-2T-Mode is configured, when the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 2-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier on one band, and any transmission on another uplink carrier on another band. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. ##### 6.1.6.2.1 void ##### 6.1.6.2.2 Uplink switching with 3 or 4 uplink bands For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured with uplink carrier aggregation with 2, 3 or 4 bands, the behaviour in subclause 6.1.6.2.0 applies when the two bands involved in the uplink switching belong to different uplink serving cells with the parameters uplinkTxSwitching, uplinkTxSwitchingOption and uplinkTxSwitching-2T-Mode being replaced by UplinkTxSwitchingMoreBands, switchingOptionConfigForBandPair and switching2T-Mode, respectively, and the behaviour in subclause 6.1.6.3 with the parameter uplinkTxSwitching being replaced by UplinkTxSwitchingMoreBands applies when the two bands involved in the uplink switching belong to one uplink serving cell, with the following exceptions: <-- texts omitted-->