3GPP TSG RAN WG1 #121	                                   					       							                          R1-2504372

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.15.6

Source:	Moderator (AT&T)

Title:	Summary of UE features for NR mobility enhancements Phase 4

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion [121-R19-UE_features] during RAN1 #121. According to the Chair’s Notes:

[121-R19-UE_features] Email discussion on Rel-19 UE features – Ralf (AT&T), Naoya (DOCOMO), Ralf (AT&T) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

The following was discussed during RAN1 #121 within the scope of [121-R19-UE_features]. All proposals are based on the latest RAN1 UE features list for Rel. 19 in [1].

# Summary of Contributions Submitted to RAN1 #121

The following is the moderator’s summary of contributions submitted to RAN1 #121 in this agenda item.

| 63. NR_Mob_Ph4 | 63-1 | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For NW triggered L1-RSRP measurement based on CSI-RS, we think the maximum number of CSI reports should be a component of FG 63-1 and this component should be independent from CSI acquisition and SSB based L1-RSRP measurement considering the position of configuration and target of reporting. Thus, those CSI reports should be associated with CSI-RS as L1-RSRP measurement resource. According to RAN4 agreement, before measuring the CSI-RS(s) in a candidate cell, a FR2-1 UE should accomplish P2 procedure based on the SSB in the corresponding cells. Skipping the SSB measurement before CSI-RS measurement is up to UE capability. Thus, we think FG45-1 should be prerequisite of this FG at least for FR2-1. Considering the similar report mechanism between R18 LTM and R19 LTM, the components and the corresponding value ranges of FG45-1 can be inherited in this new FG. Proposal 1: For FG 63-1, support the following revised component 4. - Revised component 4: Maximum number of LTM CSI report configs using CSI-RS as measurement resource for L1-RSRP 63. NR_Mob_Ph4; 63-1; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource for L1-RSRP; 45-1 at least for FR2-1; Yes; NO; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; Per BC; No; No; n/a; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Component 4 candidate values: Aperiodic: {0,1,2,3,4} Periodic: {1,2,3,4} Semi-persistent: {0,1,2,3,4}; Optional with capability signaling. |
| Vivo [3] | 63. NR_Mob_Ph4; 63-1; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource; 45-1; Yes; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; FFS; FFS; FFS; FFS; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Component 4 candidate values: Aperiodic: {0,1,2,3,4} Periodic: {1,2,3,4} Semi-persistent: {0,1,2,3,4}; Optional with capability signaling. |
| Nokia [4] | - The component values for components 2, 3, and 4 in FG 63-1 should align with those defined for FG 45-1 (SSB-based measurements), as follows: - Component 2 candidate values: {1,2,3,4,5,6,7,8} - Component 3 candidate values: - L: {1, 2,3,4} - M: {1, 2,3,4} - M L: {1,2,3,4, 6, 8, 9, 12, 16} - Component 4 candidate values: - Aperiodic: {0,1,2,3,4} - Periodic: {1,2,3,4} - Semi-persistent: {0,1,2,3,4} - The pre-requisite FGs for FG 63-1 should be aligned with those of FG 45-1, i.e., 2-21 or 2-22 or 2-23 or 2-23a - The FG type should be the same as for FG 45-1, i.e., Per BC. 63. NR_Mob_Ph4; 63-1; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource; 2-21 or 2-22 or 2-23 or 2-23a FFS; Yes; No; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; FFS Per BC; FFS No; FFS No; FFS n/a; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Component 4 candidate values: Aperiodic: {0,1,2,3,4} Periodic: {1,2,3,4} Semi-persistent: {0,1,2,3,4}; Optional with capability signaling. |
| Samsung [5] | For gNB scheduled measurement/reporting for LTM, it was agreed in RAN1 that periodic, semi-persistent and aperiodic reporting are supported, where are missing from the current UE features list. Hence, we propose to reflect this reporting behaviors in both 63-1 and 63-2 with corresponding modifications (highlighted in red) as in Proposal 1. Proposal 1. Introduce support of periodic, semi-persistent and aperiodic reporting for CSI-RS based NW scheduled measurement/reporting in Rel-19 LTM. 63. NR_Mob_Ph4; 63-1; NW triggered intra-frequency L1-RSRP measurement/reporting based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource 5. Support of periodic, semi-persistent and aperiodic reporting; FFS; Yes; NW triggered intra-frequency L1-RSRP measurement/reporting based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; FFS; FFS; FFS; FFS; Optional with capability signaling. |
| ZTE Corporation/Sanechips [6] | - Whether to introduce inter-frequency L1-RSRP measurement - According to the following agreement in RAN4#114bis meeting, we can vaguely observe that RAN4 does not tend to define inter-frequency L1-RSRP measurement for CSI-RS in Rel-19 LTM. But whether to introduce it ultimately, we think that the decision should be made by RAN4, because RAN4 will define the relevant requirements and test indicators. Agreement: - From RAN4 perspective, - Not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell. - Not to define RAN4 requirements for RTD > CP.. - Regarding FG 63-1, - A basic principle is that it should follow the principle of FG 45-1 definition in Rel-18 LTM as much as possible because we have agreed in RAN1#118bis meeting that Rel-18 LTM CSI reporting framework will be made as a baseline for CSI-RS based L1 measurement reporting. With this principle, we provide the following opinions: - Component 4 “Maximum number of LTM CSI report configs using CSI-RS as measurement resource” should be kept in FG 63-1. - The prerequisite FG 2-21 or 2-22 or 2-23 or 2-23a need to be added. - The granularity should be per BC. - The need of FDD/TDD differentiation should be “No”. - The need of FR1/FR2 differentiation should be “No”. - The capability interpretation for mixture of FDD/TDD and/or FR1/FR2 should be “n/a”. Proposal 1: For FG 63-1 and FG 63-2 of ‘gNB triggered intra-frequency L1-RSRP measurement’, the following updates highlighted in red are proposed. 63. NR_Mob_Ph4; 63-1; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource; FFS 2-21 or 2-22 or 2-23 or 2-23a; Yes; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; FFS Per BC; FFS No; FFS No; FFS n/a; Optional with capability signaling. |
| CATT [7] |  |
| CMCC [8] | 63. NR_Mob_Ph4; 63-1; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource. |
| Xiaomi [9] |  |
| Ericsson [10] | - The feature groups 63-1 and 63-2 are for intra-frequency measurements. Corresponding feature-groups should be added for the support of inter-frequency L1-RSRP measurements, but the exact definition of intra-frequency measurements on CSI-RS is still being discussed in RAN4 (inter-frequency measurements is defined as everything which is not intra-frequency). Moreover, RAN4 also need to settle on when measurement gaps are needed. - Regarding component 4 for the maximum number of LTM CSI Report Configurations using CSI-RS as measurement resource, the UE capability should be for the total number of LTM CSI Report Configurations, counting both those using SSB as well as CSI-RS as measurement resource together. 63-1; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using either CSI-RS as measurement resource or SSB as measurement resource; FFS; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; FFS; FFS; FFS. |
| OPPO [11] | For FG 63-1, the maximum number of CSI-RS resource for L1 measurement of each candidate cell shall be UE capability. And the maximum number of all CSI-RS resources for all candidate cell for L1 measurement shall also be a UE capability. However, the maximum number of CSI report configuration of CSI-RS resource of candidate cell is not a UE capability. The is applicable to FG 63-2 of semi-persistent CSI-RS resource. Therefore, the FGs 63-1 and 63-2 shall be updated as follows: Proposal 1: Update the FGs 63-1 and 63-2 as follows: 63. NR_Mob_Ph4; 63-1; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of CSI-RS resources per candidate cell configured for intra-frequency L1-RSRP measurement 5. Maximum of the total number of CSI-RS resources of candidate cells configured for intra-frequency L1-RSRP measurement. 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource; FFS; Yes; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported. |
| Qualcomm Incorporated [12] | 63. NR_Mob_Ph4; 63-1; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource; FFS 2-21 or 2-22 or 2-23 or 2-23a; Yes; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; FFS Per BC; FFS No; FFS No; FFS; Optional with capability signaling. |
| NTT DOCOMO, INC. [13] | 63-1 - For component 4, this should be updated as the following since there would be no need to limit the number of LTM CSI report configs using CSI-RS. - “Maximum number of LTM CSI report configs using SSB or CSI-RS as measurement resource” - FG45-1 should be prerequisite FG. - For the type, “per BC” should be used the same as FG45-1. |

| 63. NR_Mob_Ph4 | 63-2 | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For FG 63-2, component 3 and 4 can be inherited from FG 63-1. Thus, we suggest to remove component 3 and 4 of FG 63-2. Proposal 2: For FG 63-2, remove component 3 ad 4. 63. NR_Mob_Ph4; 63-2; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource; 63-1; Yes; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; Per BC; No; No; n/a; Component 2 candidate values: [TBD]; Optional with capability signalling. |
| Vivo [3] | 63. NR_Mob_Ph4; 63-2; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource; 63-1; Yes; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; FFS; FFS; FFS; FFS; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Component 4 candidate values: Aperiodic: {0,1,2,3,4} Semi-persistent: {1,2,3,4}; Optional with capability signaling. |
| Nokia [4] | - FG 63-1 should be the pre-requisite for FG 63-2. Accordingly, there is no need to re-define components 2 and 3 in FG 63-2. - The FG type for FG 63-2 should be the same as FG 63-1, i.e., Per BC. - For reporting based on semi-persistent CSI-RSs, at least one semi-persistent or aperiodic CSI reporting should be supported. Therefore, for component 4 (which will be renumbered as component 2 after removing the current components 2 and 3), the candidate values should be: - Aperiodic: {1,2,3,4} - Semi-persistent: {0,1,2,3,4} 63. NR_Mob_Ph4; 63-2; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource; 63-1 FFS; Yes; No; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; FFS Per BC; FFS No; FFS No; FFS n/a; Component 2 candidate values: Aperiodic: {1,2,3,4} Semi-persistent: {0,1,2,3,4}; Optional with capability signaling. |
| Samsung [5] | For gNB scheduled measurement/reporting for LTM, it was agreed in RAN1 that periodic, semi-persistent and aperiodic reporting are supported, where are missing from the current UE features list. Hence, we propose to reflect this reporting behaviors in both 63-1 and 63-2 with corresponding modifications (highlighted in red) as in Proposal 1. Proposal 1. Introduce support of periodic, semi-persistent and aperiodic reporting for CSI-RS based NW scheduled measurement/reporting in Rel-19 LTM. 63. NR_Mob_Ph4; 63-2; NW triggered intra-frequency L1-RSRP measurement/reporting based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource 5. Support of periodic, semi-persistent and aperiodic reporting; FFS; Yes; NW triggered intra-frequency L1-RSRP measurement/reporting based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; FFS; FFS; FFS; FFS; Optional with capability signaling. |
| ZTE Corporation/Sanechips [6] | - Whether to introduce inter-frequency L1-RSRP measurement - According to the following agreement in RAN4#114bis meeting, we can vaguely observe that RAN4 does not tend to define inter-frequency L1-RSRP measurement for CSI-RS in Rel-19 LTM. But whether to introduce it ultimately, we think that the decision should be made by RAN4, because RAN4 will define the relevant requirements and test indicators. Agreement: - From RAN4 perspective, - Not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell. - Not to define RAN4 requirements for RTD > CP.. - Regarding FG 63-2, - Other components are not needed except component 1 if the prerequisite of FG 63-2 is set to FG 63-1. - In order to avoid ambiguity, “semi-persistent” should be added before “CSI-RS(s) of candidate cell(s)” in component 1. - The granularity should be per BC. - The need of FDD/TDD differentiation should be “No”. - The need of FR1/FR2 differentiation should be “No”. - The capability interpretation for mixture of FDD/TDD and/or FR1/FR2 should be “n/a”. Proposal 1: For FG 63-1 and FG 63-2 of ‘gNB triggered intra-frequency L1-RSRP measurement’, the following updates highlighted in red are proposed. 63. NR_Mob_Ph4; 63-2; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource; FFS 63-1; Yes; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; FFS Per BC; FFS No; FFS No; FFS n/a; Optional with capability signaling. |
| CATT [7] | The report of L cells x M beams follows that specified in Rel-18 LTM CSI reporting. In our opinion, the agreement applies to both periodic CSI-RS based L1-measurement and semi-persistent CSI-RS based L1-measurement. Regarding UE features on L1-measurement, our views are given below: ### 63-2: - Component 3, i.e. support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement, should be supported for FG 63-2. Proposal 1: Adopt the following changes marked in red for FG 63-2: 63. NR_Mob_Ph4; 63-2; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource; FFS; Yes; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; FFS; FFS; FFS; FFS; Optional with capability signaling. |
| CMCC [8] | 63. NR_Mob_Ph4; 63-2; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource. |
| Xiaomi [9] |  |
| Ericsson [10] | - The feature groups 63-1 and 63-2 are for intra-frequency measurements. Corresponding feature-groups should be added for the support of inter-frequency L1-RSRP measurements, but the exact definition of intra-frequency measurements on CSI-RS is still being discussed in RAN4 (inter-frequency measurements is defined as everything which is not intra-frequency). Moreover, RAN4 also need to settle on when measurement gaps are needed. - Regarding component 4 for the maximum number of LTM CSI Report Configurations using CSI-RS as measurement resource, the UE capability should be for the total number of LTM CSI Report Configurations, counting both those using SSB as well as CSI-RS as measurement resource together. - A UE supporting this feature can receive MAC CE for activation/deactivation of semi-persistent CSI-RS for LTM. A corresponding component should be added. 63-2; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource 5. Receiving MAC CE for activation/deactivation of semi-persistent CSI-RS.; FFS; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; FFS; FFS; FFS. |
| OPPO [11] | For FG 63-1, the maximum number of CSI-RS resource for L1 measurement of each candidate cell shall be UE capability. And the maximum number of all CSI-RS resources for all candidate cell for L1 measurement shall also be a UE capability. However, the maximum number of CSI report configuration of CSI-RS resource of candidate cell is not a UE capability. The is applicable to FG 63-2 of semi-persistent CSI-RS resource. Therefore, the FGs 63-1 and 63-2 shall be updated as follows: Proposal 1: Update the FGs 63-1 and 63-2 as follows: 63. NR_Mob_Ph4; 63-2; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource 4. Maximum number of CSI-RS resources per candidate cell configured for intra-frequency L1-RSRP measurement 5. Maximum of the total number of CSI-RS resources of candidate cells configured for intra-frequency L1-RSRP measurement.; FFS; Yes; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported. |
| Qualcomm Incorporated [12] | 63. NR_Mob_Ph4; 63-2; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource; FFS 2-21 or 2-22 or 2-23 or 2-23a; Yes; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; FFS Per BC; FFS No; FFS No; FFS; Optional with capability signaling. |
| NTT DOCOMO, INC. [13] | 63-2 - For component 3/4, they should be removed since there would be no need to split them for periodic CSI-RS and semi-persistent CSI-RS. - FG63-1 should be prerequisite FG. - For the type, “per BC” should be used same as FG63-1. |

| 63. NR_Mob_Ph4 | 63-3 | CSI-RS as Type-D QCL source RS in the indicated joint LTM TCI state | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states FFS: potential other component(s) | FFS | Yes |  | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 63. NR_Mob_Ph4; 63-3; CSI-RS as Type-D QCL source RS in the indicated joint LTM TCI state; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states; 23-1-1; Yes; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint DL/UL LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Vivo [3] | 63. NR_Mob_Ph4; 63-3; CSI-RS as Type-D QCL source RS in the indicated joint LTM TCI state; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states FFS: potential other component(s); 63-1, 45-3; Yes; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Nokia [4] | For each of these FGs (63-3/3a, 63-4/4a), the corresponding Rel-18 FG should serve as the prerequisite FG. Specifically, FGs 45-3, 45-3a, 45-4, and 45-4a should be defined as the prerequisites for FGs 63-3, 63-3a, 63-4, and 63-4a, respectively. With this prerequisite relationship established, there is no need to define any additional components for each of the FGs. Proposal 3: FGs 45-3, 45-3a, 45-4, and 45-4a should be the pre-requisite for FGs 63-3, 63-3a, 63-4, and 63-4a, respectively. 63. NR_Mob_Ph4; 63-3; CSI-RS as Type-D QCL source RS in the indicated joint LTM TCI state; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states FFS: potential other component(s); FFS 45-3; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Samsung [5] |  |
| ZTE Corporation/Sanechips [6] | - Regarding FG 63-3, - The prerequisite FG 45-3, 23-1-1, RAN FG for LTM need to be added. Based on this, “FFS: potential other component(s)” should be removed. Proposal 2: For FG 63-3/3a/4/4a of ‘CSI-RS as QCL source RS in LTM TCI state’, the following updates highlighted in red are proposed. 63. NR_Mob_Ph4; 63-3; CSI-RS as Type-D QCL source RS in the indicated joint LTM TCI state; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states FFS: potential other component(s); FFS 45-3, 23-1-1, RAN FG for LTM; Yes; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| CATT [7] |  |
| CMCC [8] |  |
| Xiaomi [9] |  |
| Ericsson [10] | - There is an FFS in each of these FGs on additional components. We do not see any need for an additional component. - When comparing these FGs to the Rel-18 FG Error! Reference source not found., we note that the type of QCL RS is mentioned only in FG 45-3 and FG 45-4, but not in FG 45-3a or FG 45-4a. The support of a certain QCL RS is thus independent from the support of MAC CE activation. Following this principle, FG 63-3a and FG 63-4a can be removed. - Remove “indicated” from the description in FG 63-3 and FG 63-4, in line with the principle of LTM Rel-18. 63-3; CSI-RS as Type-D QCL source RS in the joint LTM TCI state; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states; FFS; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states; Per band; n/a; n/a. |
| OPPO [11] |  |
| Qualcomm Incorporated [12] | 63. NR_Mob_Ph4; 63-3; CSI-RS as Type-D QCL source RS in the indicated joint LTM TCI state; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states FFS: potential other component(s); FFS 45-3; Yes; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 63-3 - FG45-3 and FG63-1 should be prerequisite FG. |

| 63. NR_Mob_Ph4 | 63-3a | CSI-RS as Type-D QCL source RS for MAC-CE activated joint LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states FFS: potential other component(s) | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 63. NR_Mob_Ph4; 63-3a; CSI-RS as Type-D QCL source RS for MAC-CE activated joint LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states; 63-3; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Vivo [3] | 63. NR_Mob_Ph4; 63-3a; CSI-RS as Type-D QCL source RS for MAC-CE activated joint LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states FFS: potential other component(s); 63-1, 45-3a; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Nokia [4] | For each of these FGs (63-3/3a, 63-4/4a), the corresponding Rel-18 FG should serve as the prerequisite FG. Specifically, FGs 45-3, 45-3a, 45-4, and 45-4a should be defined as the prerequisites for FGs 63-3, 63-3a, 63-4, and 63-4a, respectively. With this prerequisite relationship established, there is no need to define any additional components for each of the FGs. Proposal 3: FGs 45-3, 45-3a, 45-4, and 45-4a should be the pre-requisite for FGs 63-3, 63-3a, 63-4, and 63-4a, respectively. 63. NR_Mob_Ph4; 63-3a; CSI-RS as Type-D QCL source RS for MAC-CE activated joint LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states FFS: potential other component(s); FFS 45-3a; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Samsung [5] |  |
| ZTE Corporation/Sanechips [6] | - Regarding FG 63-3a, - The prerequisite FG 45-3, 63-3 need to be added. Based on this, “FFS: potential other component(s)” should be removed. Proposal 2: For FG 63-3/3a/4/4a of ‘CSI-RS as QCL source RS in LTM TCI state’, the following updates highlighted in red are proposed. 63. NR_Mob_Ph4; 63-3a; CSI-RS as Type-D QCL source RS for MAC-CE activated joint LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states FFS: potential other component(s); FFS FG 45-3, 63-3; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| CATT [7] |  |
| CMCC [8] |  |
| Xiaomi [9] |  |
| Ericsson [10] | - There is an FFS in each of these FGs on additional components. We do not see any need for an additional component. - When comparing these FGs to the Rel-18 FG Error! Reference source not found., we note that the type of QCL RS is mentioned only in FG 45-3 and FG 45-4, but not in FG 45-3a or FG 45-4a. The support of a certain QCL RS is thus independent from the support of MAC CE activation. Following this principle, FG 63-3a and FG 63-4a can be removed. - Remove “indicated” from the description in FG 63-3 and FG 63-4, in line with the principle of LTM Rel-18. |
| OPPO [11] | UE FGs of supporting CSI-RS in TCI state were agreed. For FG-3a, the maximum number of activated joint LTM TCI states with CSI-RS as QCL source shall be UE capability and the maximum number of all activated LTM TCI states of all candidate cells shall also be a component. The same components shall be included in FG-4a for the case of separate TCI states. Therefore, the FGs 63-3a and 63-4a shall be updated as follows: Proposal 2: Update the FGs 63-3a and 63-4a as follows: 63. NR_Mob_Ph4; 63-3a; CSI-RS as Type-D QCL source RS for MAC-CE activated joint LTM TCI states; 1. Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states 2. Maximum number of activated joint LTM TCI states with CSI-RS for BM as Type-D QCL source per candidate cell. 3. Maximum number of activated joint LTM TCI states with CSI-RS for BM as Type-D QCL source of all candidate cell.; FFS; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states. |
| Qualcomm Incorporated [12] | 63. NR_Mob_Ph4; 63-3a; CSI-RS as Type-D QCL source RS for MAC-CE activated joint LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states FFS: potential other component(s); FFS 45-3a, 63-3; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 63-3a - FG45-3a and FG63-1 should be prerequisite FG. |

| 63. NR_Mob_Ph4 | 63-4 | CSI-RS as Type-D QCL source RS in the indicated separate DL/UL LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states FFS: potential other component(s) | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 63. NR_Mob_Ph4; 63-4; CSI-RS as Type-D QCL source RS in the indicated separate DL/UL LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states; 23-10-1; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Vivo [3] | 63. NR_Mob_Ph4; 63-4; CSI-RS as Type-D QCL source RS in the indicated separate DL/UL LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states FFS: potential other component(s); 63-1, 45-4; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Nokia [4] | For each of these FGs (63-3/3a, 63-4/4a), the corresponding Rel-18 FG should serve as the prerequisite FG. Specifically, FGs 45-3, 45-3a, 45-4, and 45-4a should be defined as the prerequisites for FGs 63-3, 63-3a, 63-4, and 63-4a, respectively. With this prerequisite relationship established, there is no need to define any additional components for each of the FGs. Proposal 3: FGs 45-3, 45-3a, 45-4, and 45-4a should be the pre-requisite for FGs 63-3, 63-3a, 63-4, and 63-4a, respectively. 63. NR_Mob_Ph4; 63-4; CSI-RS as Type-D QCL source RS in the indicated separate DL/UL LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states FFS: potential other component(s); FFS 45-4; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Samsung [5] |  |
| ZTE Corporation/Sanechips [6] | - Regarding FG 63-4, - The prerequisite FG 45-4, 23-10-1, RAN FG for LTM need to be added. Based on this, “FFS: potential other component(s)” should be removed. Proposal 2: For FG 63-3/3a/4/4a of ‘CSI-RS as QCL source RS in LTM TCI state’, the following updates highlighted in red are proposed. 63. NR_Mob_Ph4; 63-4; CSI-RS as Type-D QCL source RS in the indicated separate DL/UL LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states FFS: potential other component(s); FFS FG 45-4, 23-10-1, RAN FG for LTM; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| CATT [7] |  |
| CMCC [8] |  |
| Xiaomi [9] |  |
| Ericsson [10] | - There is an FFS in each of these FGs on additional components. We do not see any need for an additional component. - When comparing these FGs to the Rel-18 FG Error! Reference source not found., we note that the type of QCL RS is mentioned only in FG 45-3 and FG 45-4, but not in FG 45-3a or FG 45-4a. The support of a certain QCL RS is thus independent from the support of MAC CE activation. Following this principle, FG 63-3a and FG 63-4a can be removed. - Remove “indicated” from the description in FG 63-3 and FG 63-4, in line with the principle of LTM Rel-18. 63-4; CSI-RS as Type-D QCL source RS in the separate DL/UL LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states; FFS; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states; Per band; n/a; n/a. |
| OPPO [11] | 63. NR_Mob_Ph4; 63-4; CSI-RS as Type-D QCL source RS in the indicated separate DL/UL LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states FFS: potential other component(s); FFS 45-4; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Qualcomm Incorporated [12] |  |
| NTT DOCOMO, INC. [13] | 63-4 - FG45-4 and FG63-1 should be prerequisite FG. |

| 63. NR_Mob_Ph4 | 63-4a | CSI-RS as Type-D QCL source RS for MAC-CE activated separate DL/UL LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states FFS: potential other component(s) | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 63. NR_Mob_Ph4; 63-4a; CSI-RS as Type-D QCL source RS for MAC-CE activated separate DL/UL LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states; 63-4; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Vivo [3] | 63. NR_Mob_Ph4; 63-4a; CSI-RS as Type-D QCL source RS for MAC-CE activated separate DL/UL LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states FFS: potential other component(s); 63-1, 45-4a; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Nokia [4] | For each of these FGs (63-3/3a, 63-4/4a), the corresponding Rel-18 FG should serve as the prerequisite FG. Specifically, FGs 45-3, 45-3a, 45-4, and 45-4a should be defined as the prerequisites for FGs 63-3, 63-3a, 63-4, and 63-4a, respectively. With this prerequisite relationship established, there is no need to define any additional components for each of the FGs. Proposal 3: FGs 45-3, 45-3a, 45-4, and 45-4a should be the pre-requisite for FGs 63-3, 63-3a, 63-4, and 63-4a, respectively. 63. NR_Mob_Ph4; 63-4a; CSI-RS as Type-D QCL source RS for MAC-CE activated separate DL/UL LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states FFS: potential other component(s); FFS 45-4a; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Samsung [5] |  |
| ZTE Corporation/Sanechips [6] | - Regarding FG 63-4a, - The prerequisite FG 45-4, 63-4 need to be added. Based on this, “FFS: potential other component(s)” should be removed. Proposal 2: For FG 63-3/3a/4/4a of ‘CSI-RS as QCL source RS in LTM TCI state’, the following updates highlighted in red are proposed. 63. NR_Mob_Ph4; 63-4a; CSI-RS as Type-D QCL source RS for MAC-CE activated separate DL/UL LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states FFS: potential other component(s); FFS FG 45-4, 63-4; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| CATT [7] |  |
| CMCC [8] |  |
| Xiaomi [9] |  |
| Ericsson [10] | - There is an FFS in each of these FGs on additional components. We do not see any need for an additional component. - When comparing these FGs to the Rel-18 FG Error! Reference source not found., we note that the type of QCL RS is mentioned only in FG 45-3 and FG 45-4, but not in FG 45-3a or FG 45-4a. The support of a certain QCL RS is thus independent from the support of MAC CE activation. Following this principle, FG 63-3a and FG 63-4a can be removed. - Remove “indicated” from the description in FG 63-3 and FG 63-4, in line with the principle of LTM Rel-18. |
| OPPO [11] | UE FGs of supporting CSI-RS in TCI state were agreed. For FG-3a, the maximum number of activated joint LTM TCI states with CSI-RS as QCL source shall be UE capability and the maximum number of all activated LTM TCI states of all candidate cells shall also be a component. The same components shall be included in FG-4a for the case of separate TCI states. Therefore, the FGs 63-3a and 63-4a shall be updated as follows: Proposal 2: Update the FGs 63-3a and 63-4a as follows: 63. NR_Mob_Ph4; 63-4a; CSI-RS as Type-D QCL source RS for MAC-CE activated separate DL/UL LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states 2. Maximum number of activated separate LTM DL/UL TCI states with CSI-RS for BM as Type-D QCL source per candidate cell. 3. Maximum number of activated separated LTM DL/UL TCI states with CSI-RS for BM as Type-D QCL source of all candidate cell.; FFS; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states. |
| Qualcomm Incorporated [12] | 63. NR_Mob_Ph4; 63-4a; CSI-RS as Type-D QCL source RS for MAC-CE activated separate DL/UL LTM TCI states; Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states FFS: potential other component(s); FFS 45-4a, 63-4; Yes; No; UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 63-4a - FG45-4a and FG63-1 should be prerequisite FG. |

| 63. NR_Mob_Ph4 | 63-6 | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE | 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells and CSI-RS resources 3. Maximum number of CSI report configs | FFS | Yes |  | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | According to the following agreements, a single CSI report for CSI acquisition is configured for a candidate cell. Hence, the maximum number of configurable CSI report (s) for CSI acquisition can be implicitly determined by the maximum number of configurable candidate cell(s) for CSI acquisition. Thus, we support to only keep the maximum number of configurable candidate cell(s) for CSI acquisition in component 2 of FG 63-6. Proposal 3: For FG 63-6, support the following revised component 2 and remove original component 3. - Revised component 2: Maximum number of the RRC configured candidate cells Furthermore, as the agreement described, the following components should be included in FG 63-6. Proposal 4: For FG 63-6, support the following component: - Maximum number of RRC configured CSI-RS resources for CSI acquisition per candidate cell - Max number of ranks per NZP CSI-RS resource - Maximum number of Tx ports in one NZP CSI-RS resource - Max total number of Tx ports of NZP CSI-RS resources per candidate cell Agreement Regarding CSI acquisition, for a candidate cell, - A single CSI report configuration is configured - Multiple CSI-RS resources for CMR can be associated with the CSI report configuration - The number of CSI-RS resources for CMR is subject to UE capability Agreement Following restrictions are introduced - For the codebook configurations in report configuration, typeI-SinglePanel is supported for LTM CSI acquisition - For report frequency configuration in report configuration, wideband CQI and wideband PMI are supported for LTM CSI acquisition - For the report quantity in report configuration, cri-RI-PMI-CQI is supported for LTM CSI acquisition - The supported max rank is up to separate UE capability - For the number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for a candidate cell for LTM CSI acquisition - Up to 128 ports is supported - The supported max number of CSI-RS ports is up to separate UE capability. 63. NR_Mob_Ph4; 63-6; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells and CSI-RS resources 3. Maximum number of RRC configured CSI-RS resources for CSI acquisition per candidate cell 4. Max number of ranks per NZP CSI-RS resource 5. Maximum number of Tx ports in one NZP CSI-RS resource 6. Max total number of Tx ports of NZP CSI-RS resources per candidate cell 3. Maximum number of CSI report configs; FFS; Yes; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; Per BC; n/a; n/a; n/a; Component 2 candidate values: [TBD] Component 3 candidate values: [TBD] Component 4 candidate values: [1,2, …, 8] Component 5 candidate values: {2, 4, 8, 12, 16, 24, 32} Component 6 candidate values: {2 to 128}; Optional with capability signaling. |
| Vivo [3] | 63. NR_Mob_Ph4; 63-6; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells and CSI-RS resources 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max rank for CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE 5. Max number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE 3. Maximum number of CSI report configs; FFS; Yes; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; FFS; FFS; FFS; FFS; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,[3,4]} Component 5 candidate values: {1,2,4,8,12,16,24,32,48,64,128}; Optional with capability signaling. |
| Nokia [4] | - There is no need for any prerequisite FG for this FG. - Based on the below agreement reached in RAN1#120bis, only one CSI report configuration can be provided per candidate cell. Therefore, only one of the components, either "number of report configurations" or "number of candidate cells" is necessary. However, for this FG, since the UE performs measurements only for the target cell (indicated in the Cell Switch Command), there is no need for a component related to the number of RRC-configured candidate cells. Based on this, we propose to revise the components as follows: - Component 2: Maximum number of CSI-RS resources associated with the cell indicated in the CSC. - Delete component 3. Agreement Regarding CSI acquisition, for a candidate cell, - A single CSI report configuration is configured - Multiple CSI-RS resources for CMR can be associated with the CSI report configuration - The number of CSI-RS resources for CMR is subject to UE capability - In addition, based on the following agreement in RAN1#120bis, two more components need to be added on the maximum rank and number of CSI-RS ports. Agreement Following restrictions are introduced - For the codebook configurations in report configuration, typeI-SinglePanel is supported for LTM CSI acquisition - For report frequency configuration in report configuration, wideband CQI and wideband PMI are supported for LTM CSI acquisition - For the report quantity in report configuration, cri-RI-PMI-CQI is supported for LTM CSI acquisition - The supported max rank is up to separate UE capability - For the number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for a candidate cell for LTM CSI acquisition - Up to 128 ports is supported - The supported max number of CSI-RS ports is up to separate UE capability - The FG type should be set to: Per Band. Additionally, based on the following agreement in RAN1#120bis, a UE capability on the support for semi-persistent CSI-RS resources needs to be added. This can be added to both FGs as a new component. Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 and RAN3 to inform this agreement Proposal 4: Consider the following updates to FG 63-6: - No pre-requisite FG - Component 2 is updated to Maximum number of the CSI-RS resources associated with the cell indicated in the CSC - Delete component 3 - Add new components: - Component 3: Maximum Rank - Component 4: Maximum number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration - Component 5: Support SP CSI-RSs - FG type: Per band 63. NR_Mob_Ph4; 63-6; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells and CSI-RS resources for the cell indicated in CSC MAC CE 3. Maximum number of CSI report configs 3. Maximum Rank 4. Maximum number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration 5. Support SP CSI-RSs; FFS n/a; Yes; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; Per band FFS; FFS No; FFS No; FFS n/a; Component 2 candidate values: {1,2,…64} Component 3 candidate values: {1,2,..Y} Component 4 candidate values: {1,2,4,8,12,16,24,32,64,128} Component 5 candidate values: {yes, no}; Optional with capability signaling. |
| Samsung [5] | As 63-6 focuses on the UE to perform measurement on a (target) cell after receiving the CSC MAC CE, where the (target) cell is indicated by the CSC MAC CE, the total number of RRC configured candidate cells are irrelevant here. Furthermore, after receiving the CSC MAC CE, only the number of CSI-RS resource(s) and CSI report configuration(s) of the indicated (target) cell would be of UE’s concerns. In addition, RAN1 has agreed to introduce the following two UE capabilities for CSI acquisition: - number of CSI-RS resources for CMR (for a candidate cell configured with one CSI report configuration) - supported max number of CSI-RS ports Based on the above, we present the following proposal with modifications to the corresponding components in 63-6 in red. Proposal 2. Support the following updates in red for 63-6. 63. NR_Mob_Ph4; 63-6; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells and CSI-RS resources for the indicated cell 3. Maximum number of CSI report configs for the indicated cell 4. Maximum number of CSI-RS resources for CMR for the indicated cell 5. Maximum number of CSI-RS ports; FFS; Yes; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; FFS; FFS; FFS; FFS; Optional with capability signaling. |
| ZTE Corporation/Sanechips [6] | - Regarding FG 63-6, - For component 2, “candidate cells” should be removed because UE only needs to perform early CSI acquisition function in a candidate cell that corresponds to target cell provided by Target Configuration ID field in LTM CSC MAC CE. - For component 3, it should be removed since RAN1 has agreed that only one CSI reporting configuration is provided for a candidate cell in RAN1#120bis meeting, as below: Agreement Regarding CSI acquisition, for a candidate cell, - A single CSI report configuration is configured - Multiple CSI-RS resources for CMR can be associated with the CSI report configuration - The number of CSI-RS resources for CMR is subject to UE capability. - The prerequisite FG 2-36 (Type I single panel codebook) need to be added. - The granularity should be per BC. - The need of FDD/TDD differentiation should be “No”. - The need of FR1/FR2 differentiation should be “No”. - The capability interpretation for mixture of FDD/TDD and/or FR1/FR2 should be “n/a”. 63. NR_Mob_Ph4; 63-6; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells and CSI-RS resources 3. Maximum number of CSI report configs; FFS 2-36; Yes; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; FFS Per BC; FFS No; FFS No; FFS n/a; Optional with capability signaling. |
| CATT [7] | Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 and RAN3 to inform this agreement. Therefore, separate UE capability should be introduced correpsondingly. Regarding UE features on CSI Acquisition, our views are given below: 63-6: - FG 63-6 should be used for periodic CSI-RS resources based CSI acquisition. 63. NR_Mob_Ph4; 63-6; Intra-frequency periodic CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells and CSI-RS resources 3. Maximum number of CSI report configs; FFS; Yes; Intra-frequency periodic CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; FFS; FFS; FFS; FFS; Optional with capability signaling. |
| CMCC [8] |  |
| Xiaomi [9] | 63. NR_Mob_Ph4; 63-6; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells 4. Maximum number of CSI-RS resources for CMR in a CSI report configuration. 5. Supported maximum number of CSI-RS ports 6. Supported maximum rank; FFS; Yes; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; FFS; FFS; FFS; FFS; Optional with capability signaling. 63. NR_Mob_Ph4; 63-6a; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on semi-persistent CSI-RS resource; 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on semi-persistent CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells 4. Maximum number of CSI-RS resources for CMR in a CSI report configuration. 5. Supported maximum number of activated CSI-RS resource 6. Supported maximum number of CSI-RS ports 7. Supported maximum rank; 63-6; Yes; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on semi-persistent CSI-RS resource is not supported; FFS; FFS; FFS; FFS; Optional with capability signaling. |
| Ericsson [10] | - A better name for theses FG is needed, CSI-RS measurement is not accurate. We need to have something on what the UE measures. - Support for CSI acquisition based on semi-persistent CSI-RS was agreed at RAN1#120-bis, feature-groups for support of that should be added. One component is to receive activation/deactivation MAC CE. - Regarding inter-frequency measurements we have the same comment as for FGs 63-1, 63-2; when RAN4 has come further in their discussions and definitions about intra- resp. inter-frequency measurements and measurement gaps, then we can come back to this question. - The maximum number of RRC configured candidate cells is given by prerequisite feature-groups, i.e., if the UE supports early CSI acquisition, it supports early CSI acquisition for all candidate cells. In RAN#120bis, the following agreement was made: Agreement Following restrictions are introduced - For the codebook configurations in report configuration, typeI-SinglePanel is supported for LTM CSI acquisition - For report frequency configuration in report configuration, wideband CQI and wideband PMI are supported for LTM CSI acquisition - For the report quantity in report configuration, cri-RI-PMI-CQI is supported for LTM CSI acquisition - The supported max rank is up to separate UE capability - For the number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for a candidate cell for LTM CSI acquisition - Up to 128 ports is supported - The supported max number of CSI-RS ports is up to separate UE capability - In legacy, we have the following UE capability for the number of ports (FG 2-36): 1) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. We believe it would make sense to reuse the same structure for LTM. There is no need to mention ‘across all CC’, since the UE only performs the measurements on one CC. - For the maximum reported rank, there is no legacy counterpart: the UE reports the maximum number of PDSCH layers (FG 2-3), and the UE must then be able to report CSI for that rank. The idea here is to decouple these two properties and introduce a UE capability to describe the maximum reported rank. 63-6; Intra-frequency CSI-RS measurement and CSI reporting based on periodic CSI-RS for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} simultaneously. 3. Maximum reported rank 4. Maximum number of CSI report configs; FFS; Intra-frequency CSI-RS measurement and CSI reporting based on periodic CSI-RS for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; FFS; FFS; FFS. 63-6a; Intra-frequency CSI-RS measurement and CSI reporting based on semi-persistent CSI-RS for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} simultaneously. 3. Maximum reported rank 4. Maximum number of CSI report configs 5. Receiving MAC CE for activation/deactivation of semi-persistent CSI-RS.; FFS; Intra-frequency CSI-RS measurement and CSI reporting based on semi-persistent CSI-RS for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; FFS; FFS; FFS. |
| OPPO [11] | Re the FGs 63-6 and 63-7 for CSI acquisition of candidate cell, the maximum number of CSI report configs is not needed because in RAN1, we have agreed one CSI report configuration for one candidate cell. The maximum number of candidate cells for CSI acquisition is sufficient. Proposal 3: Update the FGs 63-6 and 63-7 as follows: 63. NR_Mob_Ph4; 63-6; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of candidate cells configured for CSI measurement and reporting. 3. Maximum number of CSI-RS resource for CSI measurement and reporting per candidate cell 4. Maximum number of CSI-RS resources for CSI measurement and reporting across all candidate cells. 2. Maximum number of the RRC configured candidate cells and CSI-RS resources 3. Maximum number of CSI report configs; FFS; Yes; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported. |
| Qualcomm Incorporated [12] | 63. NR_Mob_Ph4; 63-6; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells and CSI-RS resources 3. Maximum number of CSI report configs 4. Maximum number of configured NZP-CSI-RS resources across all candidate cells 5. Maximum number of configured NZP-CSI-RS resources per candidate cell 6. Maximum number of ports across all configured NZP-CSI-RS resources per candidate cell 7. Maximum number of CSI-IM resource per candidate cell; FFS 2-33, RAN2 FG for LTM; Yes; Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; FFS Per BC; FFS No; FFS No; FFS; Optional with capability signaling. |
| NTT DOCOMO, INC. [13] | 63-6 - Based on the latest agreement on support of semi-persistent CSI-RS resource with separate UE capability, suggest introducing two separate UE FGs for CSI reporting after reception of LTM CSC MAC CE, one is for periodic CSI-RS resource, and the other is for semi-persistent CSI-RS resource. - For component 3, this should be kept since CSI report configuration for this function is newly added per candidate cell. - Support to add a new component of ‘maximum number of CSI-RS reports of CSI-RS resource(s) associated with a CSI report configuration for a candidate cell’. - For the type, “per band” should be used. |

| 63. NR_Mob_Ph4 | 63-7 | Intra-frequency CSI-RS measurement for candidate cell [after the RRC configuration of configured CSI-RS resource(s) and] before reception of LTM CSC MAC CE | 1. Support of CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources of candidate cell(s) for CSI measurement before LTM CSC MAC CE | 63-6 | Yes |  | Intra-frequency CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For CSI measurement for CSI acquisition before CSC MAC CE, we think CSI measurement cannot be performed without RRC configuration, thus, the bracket part ‘[after the RRC configuration of configured CSI-RS resource(s) and]’ in FG 63-7 is redundant. Considering the UE capability, the maximum number of candidate cell(s) on which UE can perform CSI measurement before LTM Cell Switch command should be reported. Considering multiple CSI-RS resources can be configured for each candidate cell, we support component 3. Proposal 5: For FG 63-7, remove the bracket part ‘[after the RRC configuration of configured CSI-RS resource(s) and]’ in the third column. Proposal 6: For FG 63-7, support component 2 and 3. 63. NR_Mob_Ph4; 63-7; Intra-frequency CSI-RS measurement for candidate cell [after the RRC configuration of configured CSI-RS resource(s) and] before reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources of candidate cell(s) for CSI measurement before LTM CSC MAC CE; 63-6; Yes; Intra-frequency CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE is not supported; Per BC; n/a; n/a; n/a; Component 2 candidate values: [TBD] Component 3 candidate values: [TBD]; Optional with capability signaling. |
| Vivo [3] | 63. NR_Mob_Ph4; 63-7; Intra-frequency CSI-RS measurement for candidate cell [after the RRC configuration of configured CSI-RS resource(s) and] before reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Total number of CSI-RS resources for CMR associated with the CSI report 3. Maximum number of CSI-RS resources of candidate cell(s) for CSI measurement before LTM CSC MAC CE; 63-6; Yes; Intra-frequency CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE is not supported; FFS; FFS; FFS; FFS; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,[2,3,4,5,6,7,8]} Component 4 candidate values: {1,2,3,4..x}; Optional with capability signaling. |
| Nokia [4] | - For this FG, based on the discussion above, component 2 and component 3 can be modified to: - Component 2: Maximum number of the RRC configured candidate cells for CSI measurement before LTM CSC MAC CE - Component 3: Maximum number of the CSI-RS resources per candidate cell for CSI measurement before LTM CSC MAC CE - The FG type should be set to: Per Band. Additionally, based on the following agreement in RAN1#120bis, a UE capability on the support for semi-persistent CSI-RS resources needs to be added. This can be added to both FGs as a new component. Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 and RAN3 to inform this agreement Proposal 5: Consider the following updates to FG 63-7: - Component 2 is updated to Maximum number of the RRC configured candidate cells for CSI measurement before LTM CSC MAC CE - Component 3 is updated to Maximum number of the CSI-RS resources per candidate cell for CSI measurement before LTM CSC MAC CE - Add a new component: - Component 4: Support SP CSI-RSs - FG type: Per band 63. NR_Mob_Ph4; 63-7; Intra-frequency CSI-RS measurement for candidate cell [after the RRC configuration of configured CSI-RS resource(s) and] before reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of the RRC configured candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources of candidate cell(s)per candidate cell for CSI measurement before LTM CSC MAC CE 4. Support SP CSI-RSs; 63-6; Yes; Intra-frequency CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE is not supported; Per band FFS; FFS No; FFS No; FFS n/a; Component 2 candidate values: {1,2,…8} Component 3 candidate values: {1,2,...64} Component 4 candidate values: {yes, no}; Optional with capability signaling. |
| Samsung [5] |  |
| ZTE Corporation/Sanechips [6] | - Regarding FG 63-7, - For the name of FG 36-7, the description of “[after the RRC configuration of configured CSI-RS resource(s) and] ” should be removed since it is a natural way that UE can start CSI-RS measurement after RRC configuration on CSI-RS resource is completed. Thus, it is unnecessary to mention it in here. - For component1, “based on periodic CSI-RS(s) of candidate cells” needs to be added to the end of component 1. - For component 2 and 3, they need to be added in this FG since not all UEs can support early CSI acquisition function on all the configured cells and performing measurement on all configured CSI-RS sources of candidate cells. - The granularity should be per BC. - The need of FDD/TDD differentiation should be “No”. - The need of FR1/FR2 differentiation should be “No”. - The capability interpretation for mixture of FDD/TDD and/or FR1/FR2 should be “n/a”. 63. NR_Mob_Ph4; 63-7; Intra-frequency CSI-RS measurement for candidate cell [after the RRC configuration of configured CSI-RS resource(s) and] before reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement before reception of CSC MAC CE based on periodic CSI-RS(s) of candidate cells 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources of candidate cell(s) for CSI measurement before LTM CSC MAC CE; 63-6; Yes; Intra-frequency CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE is not supported; FFS Per BC; FFS No; FFS No; FFS n/a; Optional with capability signaling. |
| CATT [7] | - Timing to start the CSI measurement before CSC has not been agreed yet, and the corresponding mechanism should be captured in RAN1 spec. It is not necessary to mention the measurement behavior. - FG 63-7 should be used for periodic CSI-RS resources based CSI acquisition. 63. NR_Mob_Ph4; 63-7; Intra-frequency periodic CSI-RS measurement for candidate cell [after the RRC configuration of configured CSI-RS resource(s) and] before reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources of candidate cell(s) for CSI measurement before LTM CSC MAC CE; 63-6; Yes; Intra-frequency periodic CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE is not supported; FFS; FFS; FFS; FFS; Optional with capability signaling. |
| CMCC [8] |  |
| Xiaomi [9] | 63. NR_Mob_Ph4; 63-7; Intra-frequency CSI-RS measurement for candidate cell before reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources of each candidate cell for CSI measurement before LTM CSC MAC CE 4. Supported maximum number of CSI-RS ports 5. Supported maximum rank; 63-6; Yes; Intra-frequency CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE is not supported; FFS; FFS; FFS; FFS; Optional with capability signaling. 63. NR_Mob_Ph4; 63-7a; Intra-frequency CSI-RS measurement for candidate cell before reception of LTM CSC MAC CE based on semi-persistent CSI-RS resource; 1. Support of semi-persistent CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources of each candidate cell for CSI measurement before LTM CSC MAC CE 4. Supported maximum number of activated CSI-RS resource 5. Supported maximum number of CSI-RS ports 6. Supported maximum rank; 63-7; Yes; Intra-frequency CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE based on semi-persistent CSI-RS resource is not supported; FFS; FFS; FFS; FFS; Optional with capability signaling. |
| Ericsson [10] | - A better name for theses FG is needed, CSI-RS measurement is not accurate. We need to have something on what the UE measures. - Support for CSI acquisition based on semi-persistent CSI-RS was agreed at RAN1#120-bis, feature-groups for support of that should be added. One component is to receive activation/deactivation MAC CE. - Regarding inter-frequency measurements we have the same comment as for FGs 63-1, 63-2; when RAN4 has come further in their discussions and definitions about intra- resp. inter-frequency measurements and measurement gaps, then we can come back to this question. - The formulation [after the RRC configuration of configured CSI-RS resource(s)] can be removed from the description since all LTM Candidates are RRC configured. In RAN#120bis, the following agreement was made: Agreement Following restrictions are introduced - For the codebook configurations in report configuration, typeI-SinglePanel is supported for LTM CSI acquisition - For report frequency configuration in report configuration, wideband CQI and wideband PMI are supported for LTM CSI acquisition - For the report quantity in report configuration, cri-RI-PMI-CQI is supported for LTM CSI acquisition - The supported max rank is up to separate UE capability - For the number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for a candidate cell for LTM CSI acquisition - Up to 128 ports is supported - The supported max number of CSI-RS ports is up to separate UE capability - In legacy, we have the following UE capability for the number of ports (FG 2-36): 1) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. We believe it would make sense to reuse the same structure for LTM. There is no need to mention ‘across all CC’, since the UE only performs the measurements on one CC. - For the maximum reported rank, there is no legacy counterpart: the UE reports the maximum number of PDSCH layers (FG 2-3), and the UE must then be able to report CSI for that rank. The idea here is to decouple these two properties and introduce a UE capability to describe the maximum reported rank. 63-7; Intra-frequency CSI-RS measurement based on periodic CSI-RS for candidate cell before reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} simultaneously, before LTM CSC MAC CE; 63-6; Intra-frequency CSI-RS measurement based on periodic CSI-RS for candidate cell before reception of LTM CSC MAC CE is not supported; FFS; FFS; FFS. 63-7a; Intra-frequency CSI-RS measurement based on semi-persistent CSI-RS for candidate cell before reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} simultaneously, before LTM CSC MAC CE 4. Receiving MAC CE for activation/deactivation of semi-persistent CSI-RS.; FFS; Intra-frequency CSI-RS measurement based on semi-persistent CSI-RS for candidate cell before reception of LTM CSC MAC CE is not supported; FFS; FFS; FFS. |
| OPPO [11] | Re the FGs 63-6 and 63-7 for CSI acquisition of candidate cell, the maximum number of CSI report configs is not needed because in RAN1, we have agreed one CSI report configuration for one candidate cell. The maximum number of candidate cells for CSI acquisition is sufficient. Proposal 3: Update the FGs 63-6 and 63-7 as follows: 63. NR_Mob_Ph4; 63-7; Intra-frequency CSI-RS measurement for candidate cell [after the RRC configuration of configured CSI-RS resource(s) and] before reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources for CSI measurement before LTM CSC MAC CE per candidate cell 4. Maximum number of CSI-RS resources across all candidate cells for CSI measurement before LTM CSC MAC CE; 63-6; Yes; Intra-frequency CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE is not supported. |
| Qualcomm Incorporated [12] | 63. NR_Mob_Ph4; 63-7; Intra-frequency CSI-RS measurement for candidate cell [after the RRC configuration of configured CSI-RS resource(s) and] before reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of configured NZP-CSI-RS resources of candidate cell(s) for CSI measurement before LTM CSC MAC CE 4. Maximum total number of configured NZP-CSI-RS resources per candidate cell for CSI measurement before LTM CSC MAC CE 5. Maximum total number of ports across all configured NZP-CSI-RS resources per candidate cell for CSI measurement before LTM CSC MAC CE 6. Max number of CSI-IM resource per candidate cell for CSI measurement before LTM CSC MAC CE; 63-6; Yes; Intra-frequency CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE is not supported; FFS Per BC; FFS No; FFS No; FFS; Optional with capability signaling. |
| NTT DOCOMO, INC. [13] | 63-7 - Similar as FG63-6, we suggest introducing two separate UE FGs for CSI-RS measurement before reception of LTM CSC MAC CE, one is for periodic CSI-RS resource, and the other is for semi-persistent CSI-RS resource. - For the type, “per band” should be used. |

Other

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | In addition to periodic CSI-RS for CSI acquisition, semi-persistent CSI-RS is supported subject to UE capability according to the following agreement. Thus, a separate FG should be added to let UE report the capability. Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 and RAN3 to inform this agreement. Proposal 7: Add a new FG (FG 63-8) to support CSI-RS measurement based on semi-persistent CSI-RS resource (s) for candidate cell. FG 63-6 or 63-7 should be the prerequisite. 63. NR_Mob_Ph4; 63-8; Intra-frequency CSI-RS measurement for candidate cell based on semi-persistent CSI-RS resource (s); 1. Support of CSI-RS measurement based on semi-persistent CSI-RS(s); 63-6 or 63-7; Yes; No; CSI-RS measurement based on semi-persistent CSI-RS(s) is not supported.; Per BC; n/a; n/a; n/a; Optional with capability signalling. Considering the similar report mechanism between R18 LTM and R19 LTM, a separate FG to allow configuring the L1-RSRP(s) of serving cell is always included can be added, similar as FG45-2. Proposal 8: Add a new FG (FG 63-9) for inclusion of current SpCell in the L1 measurement report based on CSI-RS (s). FG 63-1 or 63-2 should be the prerequisite. 63. NR_Mob_Ph4; 63-9; Inclusion of current SpCell in the L1 measurement report based on CSI-RS (s); 1. Support of always including the current SpCell in the L1 measurement report based on CSI-RS (s); 63-1 or 63-2; Yes; No; UE does not always include measurement report for SpCell in the L1 measurement report based on CSI-RS (s); Per BC; No; No; n/a; Optional with capability signalling. For the granularity of UE features, there was a long debate between per band and per BC in R18 LTM. We propose to follow R18 LTM where all L1 measurement and reporting related UE capabilities are reported per band combination to allow sharing the capability of CA. Proposal 9: The reporting granularity of all L1 measurement and measurement related FG in R19 LTM should be per band combination. |
| Vivo [3] | Xx_NR_Mobility_PH4; 63-8; CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on semi-persistent CSI-RS; Support of candidate cell semi-persistent CSI-RS for CSI acquisition for reporting after reception of LTM CSC MAC CE; 63-6; candidate cell semi-persistent CSI-RS for CSI acquisition for reporting after reception of LTM CSC MAC CE is not supported; Optional with capability signaling. Xx_NR_Mobility_PH4; 63-9; Event triggered L1-RSRP for event LTM2; 1. Support of event triggered L1-RSRP reporting measured on candidate cell(s) SSB or periodic CSI-RS for event LTM2; 45-1; Event triggered L1-RSRP reporting measured on candidate cell(s) SSB or periodic CSI-RS for event LTM2 is not supported; Optional with capability signaling. Xx_NR_Mobility_PH4; 63-10; Event triggered L1-RSRP event LTM3; 1. Support of event triggered L1-RSRP reporting measured on candidate cell(s) SSB or periodic CSI-RS for event LTM3 2. Maximum number of RRC configured candidate cells for event triggered L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP or CRI-RSRP pair is used for each beam report L1-RSRP measurement; 45-1; Event triggered L1-RSRP reporting measured on candidate cell(s) SSB or periodic CSI-RS for event LTM3 is not supported; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16}; Optional with capability signaling. Xx_NR_Mobility_PH4; 63-11; Event triggered L1-RSRP event LTM4; 1. Support of event triggered L1-RSRP reporting measured on candidate cell(s) SSB or periodic CSI-RS for event LTM4 2. Maximum number of RRC configured candidate cells for even triggered L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP or CRI-RSRP pair is used for each beam report L1-RSRP measurement; 45-1; Event triggered L1-RSRP reporting measured on candidate cell(s) SSB or periodic CSI-RS for event LTM4 is not supported; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16}; Optional with capability signaling. Xx_NR_Mobility_PH4; 63-12; Event triggered L1-RSRP for event LTM5; 1.Support of event triggered L1-RSRP reporting measured on candidate cell(s) SSB or CSI-RS for event LTM5 2. Maximum number of RRC configured candidate cells for event triggered L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP or CRI-RSRP pair is used for each beam report L1-RSRP measurement; 45-1; Event triggered L1-RSRP reporting measured on candidate cell(s) SSB or periodic CSI-RS for event LTM5 is not supported; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16}; Optional with capability signaling. |
| Nokia [4] |  |
| Samsung [5] | For event-triggered measurement/reporting for Rel-19, support to introduce the following FG based on RAN1’s agreements: Proposal 3. Support to introduce the following FG for event-triggered L1 measurement/reporting for Rel-19 LTM 63. NR_Mob_Ph4; 63-x; Event-triggered L1-RSRP measurement/reporting for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of SSB and periodic CSI-RS based L1-RSRP measurement 2. Support of serving cell RS by using QCL RS in the indicated TCI state or the corresponding QCL SSB 3. Maximum number of beams in a single event triggered reporting; Yes; Event-triggered L1-RSRP measurement/reporting for L1-L2 Triggered Mobility (LTM) procedure is not supported; Component 1: the periodic CSI-RS is configured in an NZP CSI-RS resource set configured with repetition; Optional with capability signaling. |
| ZTE Corporation/Sanechips [6] | - New FG 63-X, - RAN1 has agreed that semi-persistent CSI-RS is supported for early CSI acquisition in the RAN1#120bis meeting, corresponding agreement is copied as follows. Thus, a new FG needs to be added. Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 and RAN3 to inform this agreement.. Proposal 3: For FG 63-6/7 of ‘early CSI acquisition’, the following updates highlighted in red are proposed. 63. NR_Mob_Ph4; 63-X; Intra-frequency CSI-RS measurement before or after reception of CSC MAC CE based on semi-persistent CSI-RS (s); - Support of CSI-RS measurement before reception of CSC MAC CE based on semi-persistent CSI-RS(s) of candidate cells - Support of CSI-RS measurement after reception of LTM CSC MAC CE based on semi-persistent CSI-RS(s) of cell indicated in CSC MAC CE; 63-6, or 63-7; Yes; Intra-frequency CSI-RS measurement before or after reception of CSC MAC CE based on semi-persistent CSI-RS (s) is not supported; Per BC; No; No; n/a; Optional with capability signaling. |
| CATT [7] | - According to the agreement of RAN1 #118 meeting, periodic CSI-RS is supported for L1-RSRP measurement for candidate cell for event triggered reporting. The bracket on periodic CSI-RS(s) in component 1 of FG 63-5 should be removed. Proposal 2: Adopt the following changes marked in red for FG 63-5: 63. NR_Mob_Ph4; 63-5; Event triggered L1-RSRP reporting measured on SSB for event LTM2; 1. Support of even triggered L1-RSRP reporting for LTM based on [periodic CSI-RS(s) and] SSB(s) 2. Support of [LTM2, LTM3, LTM4, LTM5] based measurement and report 3. Support of MAC CE based event triggered beam report 4. Maximum number of the RRC configured candidate cells and beams 5. Support of serving cell beam measurement for event evaluation by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively; FFS; yes; Event triggered L1-RSRP reporting measured on [periodic CSI-RS(s) and] SSB(s) for event [LTM2, LTM3, LTM4, LTM5] is not supported; FFS; FFS; FFS; FFS; FFS: separate row/FG for periodic CSI-RS FFS: separate rows/FGs for LTM2, LTM3, LTM4, LTM5; Optional with capability signalling. Proposal 4: Introduce additional FGs for semi-persistent CSI-RS based CSI acquisition: 63. NR_Mob_Ph4; 63-8; Intra-frequency semi-persistent CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells and CSI-RS resources 3. Maximum number of CSI report configs; FFS; Yes; Intra-frequency semi-persistent CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; FFS; FFS; FFS; FFS; Optional with capability signaling. 63. NR_Mob_Ph4; 63-9; Intra-frequency semi-persistent CSI-RS measurement for candidate cell before reception of LTM CSC MAC CE; 1. Support of CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources of candidate cell(s) for CSI measurement before LTM CSC MAC CE; 63-8; Yes; Intra-frequency semi-persistent CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE is not supported; FFS; FFS; FFS; FFS; Optional with capability signaling. |
| CMCC [8] | 63. NR_Mob_Ph4-Core; 63-1-1a; NW triggered Inter-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure based on periodic CSI-RS(s); 1. Support of inter-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for inter-frequency L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for inter-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource. 63. NR_Mob_Ph4-Core; 63-2-1a; NW triggered Inter-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure based on semi-persistent CSI-RS(s); 1. Support of inter-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for inter-frequency L1-RSRP measurement 4. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for inter-frequency L1-RSRP measurement. 63. NR_Mob_Ph4-Core; 63-3; Event triggered reporting for L1-L2 Triggered Mobility (LTM); 1. Support of event triggered reporting for LTM based on periodic CSI-RS(s) and SSB(s) 2. Support of LTM2, LTM3, LTM4, LTM5 based measurement and report 3. Support of MAC CE based event triggered beam report 4. Maximum number of the RRC configured candidate cells and beams 5. Support of serving cell beam measurement for event evaluation by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively. |
| Xiaomi [9] |  |
| Ericsson [10] | For event-driven reporting for LTM, it is not clear to us if there is a need for a RAN1 feature. As RAN2 has specified most of the critical functionality for event-driven reporting, it would be up to RAN2 to define the corresponding features. |
| OPPO [11] |  |
| Qualcomm Incorporated [12] |  |
| NTT DOCOMO, INC. [13] |  |

# Discussion Items during RAN1 #121 – First Round

After review of contributions submitted to RAN1 #121 in this agenda item, the following topics were identified by the moderator for discussion during RAN1 #121.

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

## FG 63-1

After review of contributions submitted to RAN1 #121 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-1 | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement [4. Maximum number of LTM CSI report configs using either CSI-RS as measurement resource or SSB as measurement resource] | FFS | Yes | No | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS Per BC | FFS n/a | FFS n/a | FFS n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Component 4 candidate values: Aperiodic: {0,1,2,3,4} Periodic: {1,2,3,4} Semi-persistent: {0,1,2,3,4} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | Support. |
| NTT DOCOMO | Support. |
| ZTE | We have a general question on why we need to add additional RS type, like SSB. If adding it in here, it means that UE will perform measurement using SSB, such UE behavior is not consistent with component 1 to 3. |
| Huawei, HiSilicon | We think the SSB is not needed. it is already reported with R18 LTM capbility |

## FG 63-2

After review of contributions submitted to RAN1 #121 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-2 | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce [3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement] [4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource] | FFS 63-1 | Yes | No | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS Per BC | FFS n/a | FFS n/a | FFS n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Component 4 candidate values: Aperiodic: {0,1,2,3,4} Semi-persistent: {0,1,2,3,4} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | For Component 4, support for at least one SP or aperiodic report is required. Since 63-1 is the prerequisite FG, we may not need Components 2 and 3. |
| NTT DOCOMO | Component2 and 3 would not be needed since FG63-1 is the prerequisite FG. |
| ZTE | We have the same view with Nokia and DCM. Besides, we also don’t see the need to keep component 2 in this FG if FG63-1 is the prerequisite FG. Finally, if possible, we can add “semi-persistent” befor CSI-RS in component 1. |
| Huawei, HiSilicon | We think the component 3 and 4 is not necessary. It can be shared with 63-1. |

## FG 63-3

After review of contributions submitted to RAN1 #121 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-3 | CSI-RS as Type-D QCL source RS in the indicated joint LTM TCI state | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states FFS: potential other component(s) | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | Support. |
| NTT DOCOMO | Support. |
| ZTE | Support. |
| Huawei, HiSilicon | support |

## FG 63-3a

After review of contributions submitted to RAN1 #121 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-3a | CSI-RS as Type-D QCL source RS for MAC-CE activated joint LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states FFS: potential other component(s) | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | Support. |
| NTT DOCOMO | Support. |
| ZTE | Support. |
| Ericsson | We can merge this capability with 63-3 to have one capability for both MAC-CE activated and indicated (in LTM CSC MAC CE) joint TCI state. The support for CSI-RS for BM as Type-D QCL source is independent of if it is indicated in CSC or activated in TCI state activation MAC CE. |
| Huawei, HiSilicon | support |

## FG 63-4

After review of contributions submitted to RAN1 #121 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-4 | CSI-RS as Type-D QCL source RS in the indicated separate DL/UL LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states FFS: potential other component(s) | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | Support. |
| NTT DOCOMO | Support. |
| ZTE | Support. |
| Huawei, HiSilicon | support |

## FG 63-4a

After review of contributions submitted to RAN1 #121 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-4a | CSI-RS as Type-D QCL source RS for MAC-CE activated separate DL/UL LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states FFS: potential other component(s) | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | Support. |
| NTT DOCOMO | Support. |
| ZTE | Support. |
| Ericsson | We can merge this capability with 63-4 to have one capability for both MAC-CE activated and indicated (in LTM CSC MAC CE) joint TCI state. The support for CSI-RS for BM as Type-D QCL source is independent of if it is indicated in CSC or activated in TCI state activation MAC CE. |
| Huawei, HiSilicon | support |

## FG 63-6

After review of contributions submitted to RAN1 #121 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-6 | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on periodic CSI-RS resource | 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE [2. Maximum number of the RRC configured candidate cells and CSI-RS resources] [Maximum number of CSI report configs] 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE 5. Maximum number of Tx ports in one NZP CSI-RS resource | FFS | Yes | No | Intra-frequency periodic CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | FFS Per BC | FFS n/a | FFS n/a | FFS n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {2, 4, 8, 12, 16, 24, 32} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-6a | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on semi-persistent CSI-RS resource | 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE 5. Maximum number of Tx ports in one NZP CSI-RS resource | FFS | Yes | No | Intra-frequency semi-persistent CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {2, 4, 8, 12, 16, 24, 32} | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | - For component 3 in both 63-6 and 63-6a: the candidate cell should be the one indicated in the CSC MAC CE; therefore, the component 3 should be updated to: - Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a the candidate cell indicated in the CSC MAC CE. - For consistency, the component 4 in both 63-6 and 63-6a can be modified to: - Max number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for the candidate cell indicated in CSC MAC CE after reception of LTM CSC MAC CE - The minimum value for components 4 and 5 in both 63-6 and 63-6a should be the same. - A component needs to be added for the maximum rank, i.e., Max rank for CSI reporting for the candidate cell indicated in CSC MAC CE after reception of LTM CSC MAC CE |
| NTT DOCOMO | We also support Nokia’s updates. |
| ZTE | We have a general question for FG 63-6/6a: for the case that measurement after reception of LTM CSC MAC CE, only one candidate cell (target cell) to switch is indicated by the LTM CSC MAC CE, why do we need to care about the component 2 “Maximum number of the RRC configured candidate cells”. |
| Huawei, HiSilicon | As for the component 3 of 63-6, we prefer the original version. When CSI RS is configured, it is not known to UE whether it is target cell or candidate cell. similar change should be performed for component 4 as following Max number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell A new component about the rank should be added as commented by Nokia. Similar change should be applied to 63-3a. |

## FG 63-7

After review of contributions submitted to RAN1 #121 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-7 | Intra-frequency CSI-RS measurement for candidate cell [after the RRC configuration of configured CSI-RS resource(s) and] before reception of LTM CSC MAC CE based on periodic CSI-RS(s) of candidate cells | 1. Support of CSI-RS measurement before reception of CSC MAC CE based on periodic CSI-RS(s) of candidate cells [2. Maximum number of RRC configured candidate cells for CSI measurement before LTM CSC MAC CE] [3. Maximum number of CSI-RS resources of per candidate cell(s) for CSI measurement before LTM CSC MAC CE] 4. Total number of CSI-RS resources for CMR associated with the CSI report | 63-6 | Yes | No | Intra-frequency periodic CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE is not supported | FFS Per BC | FFS n/a | FFS n/a | FFS n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,...64} Component 4 candidate values: FFS | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-7 | Intra-frequency CSI-RS measurement for candidate celbefore reception of LTM CSC MAC CE based on semi-persistent CSI-RS(s) of candidate cells | 1. Support of CSI-RS measurement before reception of CSC MAC CE based on semi-persistent CSI-RS(s) of candidate cells 2. Maximum number of RRC configured candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources per candidate cell(s) for CSI measurement before LTM CSC MAC CE 4. Total number of CSI-RS resources for CMR associated with the CSI report | 63-6 | Yes | No | Intra-frequency semi-persistent CSI-RS measurement for candidate cell before reception of LTM CSC MAC CE is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,...64} Component 4 candidate values: FFS | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | - The newsly added 63-7 should be 63-7a. - For component 3 in both 63-7, and 63-7a: - per candidate cell(s) per candidate cell - The candidate values for component 2 should the same as the component 3 in 63-6/63-6a: {1,2,…8} - Component 4 is not needed. - We need to add new components here for the ports, similar to component 4 and 5 in 63-6/63-6a: - Max number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell - Maximum number of Tx ports in one NZP CSI-RS resource associated with a CSI report configuration for CSI reporting for a candidate cell |
| NTT DOCOMO | We also support Nokia’s updates. |
| ZTE | Agree with Nokia’s comment. |
| Huawei, HiSilicon | We think component 3 of 63-7should be changed as following . Maximum number of CSI-RS resources across candidate cell(s) for CSI measurement before LTM CSC MAC CE The component 4 is not need because UE may measure CSI-RS reousrce associated with different CSI reporting configure for multiple candidate cell. the corresponding value of component 3 of 63-6 can be assumed by UE. Similar change should be applied to 63-7a. |

## New FG: Inclusion of current SpCell in the L1 measurement report based on CSI-RS (s)

After review of contributions submitted to RAN1 #121 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 63. NR_Mob_Ph4 | 63-8 | Inclusion of current SpCell in the L1 measurement report based on CSI-RS (s) | 1. Support of always including the current SpCell in the L1 measurement report based on CSI-RS (s) | 63-1 or 63-2 | Yes | No | UE does not always include measurement report for SpCell in the L1 measurement report based on CSI-RS (s) | Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | Support. |
| NTT DOCOMO | We are fine in principle. |
| ZTE | Fine |
| Huawei, HiSlicon | support |

## New FG: Event-driven reporting for LTM

After review of contributions submitted to RAN1 #121 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 63. NR_Mob_Ph4 | 63-9 | Event triggered L1-RSRP for event LTM2 | 1. Support of event triggered L1-RSRP reporting measured on candidate cell(s) SSB or periodic CSI-RS for event LTM2 | 45-1 | Yes | No | Event triggered L1-RSRP reporting measured on candidate cell(s) SSB or periodic CSI-RS for event LTM2 is not supported | Per BC | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-10 | Event triggered L1-RSRP event LTM3 | 1. Support of event triggered L1-RSRP reporting measured on candidate cell(s) SSB or periodic CSI-RS for event LTM3 2. Maximum number of RRC configured candidate cells for event triggered L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP or CRI-RSRP pair is used for each beam report L1-RSRP measurement | 45-1 | Yes | No | Event triggered L1-RSRP reporting measured on candidate cell(s) SSB or periodic CSI-RS for event LTM3 is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} | Optional with capability signaling |
| 63. NR_Mob_Ph4 | 63-11 | Event triggered L1-RSRP event LTM4 | 1. Support of event triggered L1-RSRP reporting measured on candidate cell(s) SSB or periodic CSI-RS for event LTM4 2. Maximum number of RRC configured candidate cells for even triggered L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP or CRI-RSRP pair is used for each beam report L1-RSRP measurement | 45-1 | Yes | No | Event triggered L1-RSRP reporting measured on candidate cell(s) SSB or periodic CSI-RS for event LTM4 is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} | Optional with capability signaling |
| 63. NR_Mob_Ph4 | 63-12 | Event triggered L1-RSRP for event LTM5 | 1.Support of event triggered L1-RSRP reporting measured on candidate cell(s) SSB or CSI-RS for event LTM5 2. Maximum number of RRC configured candidate cells for event triggered L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP or CRI-RSRP pair is used for each beam report L1-RSRP measurement | 45-1 | Yes | No | Event triggered L1-RSRP reporting measured on candidate cell(s) SSB or periodic CSI-RS for event LTM5 is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | We should let RAN2 start to define UE features for event-triggered reporting since it has been driven by them. |
| Huawei, HiSilicon | We think these FGs should be up to RAN2. |

# Discussion Items during RAN1 #121 - Second Round

After review of contributions submitted to RAN1 #121 in this agenda item, the following additional topics were identified by the moderator for discussion during RAN1 #121.

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

## FG 63-6/6a

Companies submitted the following views on the open issues highlighted in yellow.

| 63. NR_Mob_Ph4 | 63-6 | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on periodic CSI-RS resource | 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE [2. Maximum number of the RRC configured candidate cells] 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of Tx ports in one NZP CSI-RS resource 6. Max rank for CSI reporting for a candidate cell | FFS | Yes | No | Intra-frequency periodic CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | FFS | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 6 candidate values: FFS | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-6a | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on semi-persistent CSI-RS resource | 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE [2. Maximum number of the RRC configured candidate cells] 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of Tx ports in one NZP CSI-RS resource 6. Max rank for CSI reporting for a candidate cell | FFS | Yes | No | Intra-frequency semi-persistent CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | FFS | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 6 candidate values: FFS | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | Component 2 is not needed as this FG is related to the measurements only for one particular candidate cell. The candidate values for component 6: {1,2,4,8}. |

## FG 63-7/7a

Companies submitted the following views on the open issues highlighted in yellow.

| 63. NR_Mob_Ph4 | 63-7 | Intra-frequency CSI-RS measurement for candidate cell before reception of LTM CSC MAC CE based on periodic CSI-RS(s) of candidate cells | 1. Support of CSI-RS measurement before reception of CSC MAC CE based on periodic CSI-RS(s) of candidate cells 2. Maximum number of RRC configured candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources across candidate cells 4. Max number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of Tx ports in one NZP CSI-RS resource associated with a CSI report configuration for CSI reporting for a candidate cell [6. Max rank for CSI reporting for a candidate cell] | 63-6 | Yes | No | Intra-frequency periodic CSI-RS measurement for candidate cell before reception of LTM CSC MAC CE is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,...64} Component 4 candidate values: FFS Component 5 candidate values: FFS | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-7a | Intra-frequency CSI-RS measurement for candidate celbefore reception of LTM CSC MAC CE based on semi-persistent CSI-RS(s) of candidate cells | 1. Support of CSI-RS measurement before reception of CSC MAC CE based on semi-persistent CSI-RS(s) of candidate cells 2. Maximum number of RRC configured candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources across candidate cells 4. Max number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of Tx ports in one NZP CSI-RS resource associated with a CSI report configuration for CSI reporting for a candidate cell [6. Max rank for CSI reporting for a candidate cell] | 63-6a | Yes | No | Intra-frequency semi-persistent CSI-RS measurement for candidate cell before reception of LTM CSC MAC CE is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,...64} Component 4 candidate values: FFS Component 5 candidate values: FFS | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | Components 4 & 5: Since 63-6/63-6a is the pre-requite FG, either these components can be removed or should have the same candidate values added for component 4 & 5 in FG 63-6/63-6a. Component 6 is not needed as its already included in the pre-requite FG(s). |

# Conclusion

Agreements reached during RAN1 #121 as part of this agenda item are summarized in [14].

# References

- R1-2502977, Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2503260, UE features for NR mobility enhancements phase 4, Huawei/HiSilicon

- R1-2503390, Discussion on UE features for NR mobility enhancements Phase 4, vivo

- R1-2503404, NR mobility enhancements Phase 4 UE features, Nokia

- R1-2503596, UE features for NR mobility enhancements Phase 4, Samsung

- R1-2503674, Discussion on UE features for NR mobility enhancements Phase 4, ZTE Corporation/Sanechips

- R1-2503810, Discussion on UE features for NR mobility enhancements Phase 4, CATT

- R1-2503855, Discussion on UE features for NR mobility enhancements Phase 4, CMCC

- R1-2503905, Discussion on UE features for NR mobility enhancements Phase 4, Xiaomi

- R1-2504185, UE features for NR mobility enhancements phase 4, Ericsson

- R1-2504234, Discussion on UE features for NR mobility enhancements, OPPO

- R1-2504424, UE features for NR mobility enhancement Phase 4, Qualcomm Incorporated

- R1-2504528, Discussion on UE features for NR mobility enhancemens Phase4, NTT DOCOMO, INC.

- R1-2504928, Session Notes of AI 9.15.6, Ad-Hoc Chair (AT&T)