3GPP TSG RAN WG1 #121	R1-2504476

St Julian’s, Malta, May 19th – 23rd, 2025

Source:		KDDI Corporation

Title:		Remaining issues on enhancements for UE-initiated/event-driven beam management

Agenda Item:	9.2.1	Enhancements for UE-initiated/event-driven beam management

Document for:	Discussion and Decision

Introduction

In the RAN #102 meeting, a new Work Item entitled “NR MIMO Phase 5” with the following scope regarding UE-initiated/event-driven beam management was endorsed [1].

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In this contribution, we provide our views on the remaining issues of enhancements for UE-initiated/event-driven beam management.

Discussion

Left-over on first PUCCH for both Mode-A and Mode-B

Whether/how to (re-)transmit the first PUCCH channel

In UE-initiated/event-driven beam reporting (UEIBR), the UE sends first PUCCH to notify the measurement results satisfy the event condition. From previous meetings, to reduce unnecessary (re-)transmissions, introducing prohibit timer or time interval has been discussed, and the following was agreed [2].

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, down-select at least one of the following candidates in RAN1#121: - Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted. - Candidate#2: To introduce a time interval for mode-A and/or mode-B - Option-2A: For a first PUCCH transmission occasion, if there is a transmission of another second PUSCH [corresponding to the CSI report determined by the same triggering beam(s) as the first PUCCH] within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Option-2B: For a first PUCCH transmission occasion, if there is a transmission of another first PUCCH(s) within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Candidate#3: No further enhancement.

In this section, we show our views on this issue. Firstly, since there are two modes in UEIBR (i.e., mode-A and mode-B), introducing timer for first PUCCH has been discussed for each mode separately. However, if different solutions are supported in each mode, this would result in doubled workload of specification. Moreover, from the perspective of simplifying the specifications, it is desirable to adopt a unified solution over both mode-A and mode-B. Therefore, we have the following proposal:

Proposal 1:

On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, if introducing prohibit timer or time interval is supported, a unified solution should be supported for both mode-A and mode-B.

Next, from the above agreement, there are two candidates, prohibit timer and time interval. Both of candidates prevents (re-)transmission of first PUCCH for a certain duration following the previous UEIBR-related activity. Therefore, either one can be used to reduce unnecessary first PUCCH transmissions. On the other hand, in terms of UE implementation, in the case of the prohibit timer, it simply needs to be activated after the transmission of PUCCH/PUSCH related to UEIBR. However, with the time interval, it is necessary to determine at each transmission occasion whether there was a transmission in the previous period, which could complicate UE implementation. Therefore, we propose the following:

Proposal 2:

On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, the following candidate #1 (i.e., prohibit timer) should be supported.

- Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B

  - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

  - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer

Finally, comparing two options in candidate #1, each has its own pros and cons. A comparison table and examples are provided below.

Table-1: Pros and cons between Option-1A and 1B.

| Options | Pros | Cons |
| --- | --- | --- |
| Option-1A: At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer | Low latency in re-transmission of first PUCCH when the reception of first PUCCH is failed. | Large overhead in re-transmission of first PUCCH, because UE can transmit first PUCCH until the second PUSCH is transmitted. |
| Option-1B: At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer | Small overhead in re-transmission of first PUCCH by preventing frequent re-transmission. | High latency in re-transmission of first PUCCH when the reception of first PUCCH is failed. |

| Case-1: the reception of first PUCCH is failed | Case-2: the first PUCCH is received |
| --- | --- |
| Figure 1. An example of pros and cons between Option-1A and Option-1B. |  |

From the above discussion, we have the following observation and proposals:

Observation 1:

On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, there is trade-off between latency of first PUCCH and overhead of unnecessary first PUCCH.

Proposal 3:

On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, in candidate # 1 (i.e., prohibit timer), the following Option-1A should be supported.

- Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B

  - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

Proposal 4:

On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, in candidate # 1 (i.e., prohibit timer), the following Option-1B should be supported.

- Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B

  - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer

Multiplexing/dropping rule(s) of 1-bit first PUCCH, e.g., Case-2, (collided with PUSCH).

Since the past meetings, multiplexing/dropping rules of first PUCCH has been discussed, and the following is agreed in the last meeting [2].

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rules for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH, in RAN1#121 - Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-4: Reuse the SR dropping rules. - FFS: whether/how to handle the case of different PHY priorities.

A comparison table between 3 options is provided below.

Table-2: Pros and cons between Option-1, 3 and 4.

| Options | Pros | Cons |
| --- | --- | --- |
| Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped. | Low latency in transmission of first PUCCH. | High latency in PUSCH, i.e., UL data transmission. |
| Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. | Low latency in transmission of both first PUCCH and PUSCH. | Higer complexity in NW side (blind decoding of first PUCCH is needed). |
| Option-4: Reuse the SR dropping rules, i.e., the first PUCCH is dropped if the PUSCH is with UL-SCH, if the PUSCH is without UL-SCH, PUSCH is dropped. [3] | Low latency in PUSCH, i.e., UL data transmission. | High latency in transmission of first PUCCH. |

From TS 38.213, 9. UE procedure for reporting control information [3]

---unnecessary parts are omitted--- If a UE would transmit on a serving cell a PUSCH without UL-SCH that overlaps with a PUCCH transmission on a serving cell that includes positive SR information, the UE does not transmit the PUSCH. ---unnecessary parts are omitted---

Considering the WID, Option-4 is not preferable because it may cause delay in UEIBR. On the other hand, taking into account the UL data transmission, delaying the PUSCH could also lead to a deterioration in user experience. Therefore, we propose the following:

Proposal 5:

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, the following rule should be supported for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH

- Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH.

  - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered.

  - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report

As described in Table-2, The one of the disadvantages of Option-3 is high complexity in NW side. However, since only 1-bit first PUCCH is supported, it can always be multiplexed with the PUSCH to avoid blind decoding.

Observation 2:

On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, multiplexing/dropping rule(s) of 1-bit first PUCCH, for the Case-2, always piggyback 1-bit indication of first PUCCH into the PUSCH can avoid blind decoding in the gNB.

Left-over on Step-2&3 in Mode-A

Whether/how to handle the case if UE has not sent any first PUCCH associated for UE -initiated/event-driven beam reporting

From the previous meetings, the behavior when the network side mistakenly detects the first PUCCH and resources for beam reporting are scheduled, has been discussed. There are at least three possible options: 1) Sending Nothing, 2) Sending the latest UEIBR or 3) Sending BR based on the most recent measurement results, ignoring the event conditions. On the other hand, regarding UEIBR format, we already have the following agreement [4]

Agreement On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - FFS: candidate value of ‘N’. - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams - FFS: Option-1/1a/1b/2. - Above applies at least for the single CC case

Therefore, even when UE has not sent any first PUCCH associated for UE -initiated/event-driven beam reporting, the NW expects the report should follow the format, i.e., the new (or current) beam should satisfy the condition. If the UE send 2) the latest UEIBR or 3) BR based on the most recent measurement results, it may cause beam misalignment. Therefore, we propose the following:

Proposal 6:

On UE-initiated/event-driven beam reporting, on the case that UE has not sent any first PUCCH while UE detects the DCI format to indicate a resource for second PUSCH to carry UE-initiated/event-driven beam reporting, for Mode-A, the UE does not send anything.

Left-over on Step-2 in Mode-B

Whether/how to introduce UE capability of ‘X’ and clarify the definition of ‘available’ transmission occasion of the second UL channel

In the case of mode-B (beam report transmission using pre-configured resources), it has been agreed that the report will be transmitted using pre-configured resources after ‘X’ symbols following the transmission of the first PUCCH. The candidates for this number of symbols are currently under discussion [5].

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel.

The X symbols are introduced to provide preparation time for the UE/NW to send and receive UEIBR. However, there may be cases where the UE/NW can send and receive the beam report immediately after receiving the first PUCCH. In such cases, having a setting that allows transmission immediately (i.e., X=0) would be desirable to enhance the timeliness of the beam report.

Proposal 7:

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the value of X symbols for determining available transmission occasion of the second UL channel on Mode-B, at least X=0 should be supported.

Regarding the definition of “the available transmission occasion of the second UL channel”, in our understanding, it means to the resources configured for UEIBR that can be used to transmit the second channel and does not collide with a channel with a higher priority. However, as discussed in section 2.1, the multiplexing/dropping rules for the first PUCCH are still under discussion, so these results also need to be taken into account for the second channel.

Observation 3:

Regarding the definition of “the available transmission occasion of the second UL channel”, the multiplexing/dropping rules for the first PUCCH are still under discussion, so these results also need to be taken into account.

On the other hand, channels that clearly have a higher priority than the second UL channel, such as SSB and PRACH, can be excluded from the definition.

Proposal 8:

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the value of X symbols for determining available transmission occasion of the second UL channel on Mode-B, the ‘available’ transmission occasion of the second UL channel is clarified as ‘an available transmission occasion corresponds to a configured PUSCH resource for UEIBR is within uplink symbols, and does not collide with a channel with a higher priority (at least SSB and PRACH)’.

Note: RAN1 can revisit this clarification when the multiplexing/dropping rules for the first PUCCH are determined.

Critical issues of Event-1 and Event-7

In the previous meetings, the working assumption regarding to introduce other events was confirmed [6].

Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signaling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

In this section, we show our views on issues on supporting the additional events.

Details on second beam report of Event-1

In Event-1, the UE sends beam report when the RSRP of the current beam falls below a specified threshold, with the objective of monitoring the quality of the beam in use. Consequently, Event-1 can be use with frequent monitoring the current beam (and a few other beams). Figure 2 shows one of the use-cases of Event-1. In such scenarios, it is more crucial to convey the occurrence of events to the NW rather than the RSRP of each beam (especially low RSRP of current beam).

Figure 2. An example of use-case of Event-1, monitoring current beam and triggering AP CSI-RS.

The occurrence of an event can be communicated to the NW via the first PUCCH. Therefore, particularly for Event-1, introducing an option to notify only the occurrence of the event via the first PUCCH could potentially reduce the overhead associated with beam reporting. From the above discussion, we propose the following:

Proposal 9:

On UE-initiated/event-driven beam reporting with Event-1, the second beam report can be skipped.

Definition of the activated TCI state with the Q-th best quality in Event-7

The clarification of 'RS derived from the activated TCI state with the Q-th best quality' has become one of the potential issues. In Event 2, if the indicated TCI state is unchanged, the ‘current beam’ represents the same beam. On the other hand, in Event-7 ‘activated TCI state with the Q-th best quality' could differ in each measurement even if the list of activated TCI states is stable. Considering the usage of Event-7, one of its uses is to update TCI states with lower quality among the activated ones. Therefore, it is more reasonable to base it on the Q-th TCI state's beam for each measurement rather than on a fixed beam. Thus, we propose the following:

Proposal 10:

On UE-initiated/event-driven beam reporting with Event-7, 'the activated TCI state with the Q-th best quality' should be determined for each measurement occasion.

Conclusion

In this contribution, we showed our views on the enhancements for UE-initiated/event-driven beam management. Based on above discussions, we have the following observations and proposals:

Observation 1:

On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, if introducing prohibit timer or time interval is supported, a unified solution should be supported for both mode-A and mode-B.

Observation 2:

On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, multiplexing/dropping rule(s) of 1-bit first PUCCH, for the Case-2, always piggyback 1-bit indication of first PUCCH into the PUSCH can avoid blind decoding in the gNB.

Observation 3:

Regarding the definition of “the available transmission occasion of the second UL channel”, the multiplexing/dropping rules for the first PUCCH are still under discussion, so these results also need to be taken into account.

Proposal 1:

On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, if introducing prohibit timer or time interval is supported, a unified solution should be supported for both mode-A and mode-B.

Proposal 2:

On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, the following candidate #1 (i.e., prohibit timer) should be supported.

- Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B

  - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

  - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer

Proposal 3:

On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, in candidate # 1 (i.e., prohibit timer), the following Option-1A should be supported.

- Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B

  - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

Proposal 4:

On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, in candidate # 1 (i.e., prohibit timer), the following Option-1B should be supported.

- Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B

  - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer

Proposal 5:

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, the following rule should be supported for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH

- Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH.

  - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered.

  - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report

Proposal 6:

On UE-initiated/event-driven beam reporting, there is no further enhancement on the case that UE has not sent any first PUCCH while UE detects the DCI format to indicate a resource for second PUSCH to carry UE-initiated/event-driven beam reporting, for Mode-A.

Proposal 6:

On UE-initiated/event-driven beam reporting, on the case that UE has not sent any first PUCCH while UE detects the DCI format to indicate a resource for second PUSCH to carry UE-initiated/event-driven beam reporting, for Mode-A, the UE does not send anything.

Proposal 7:

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the value of X symbols for determining available transmission occasion of the second UL channel on Mode-B, at least X=0 should be supported.

Proposal 8:

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the value of X symbols for determining available transmission occasion of the second UL channel on Mode-B, the ‘available’ transmission occasion of the second UL channel is clarified as ‘an available transmission occasion corresponds to a configured PUSCH resource for UEIBR is within uplink symbols, and does not collide with a channel with a higher priority (at least SSB and PRACH)’.

Note: RAN1 can revisit this clarification when the multiplexing/dropping rules for the first PUCCH are determined.

Proposal 9:

On UE-initiated/event-driven beam reporting with Event-1, the second beam report can be skipped.

Proposal 10:

On UE-initiated/event-driven beam reporting with Event-7, 'the activated TCI state with the Q-th best quality' should be determined for each measurement occasion.

- Reference

- RP-234007, “New WID: NR MIMO Phase 5”, RAN Plenary #102.

- RAN1 Chair’s Notes, RAN1 #120-bis.

- 3GPP TS 38.213, v18.6.0.

- RAN1 Chair’s Notes, RAN1 #117.

- RAN1 Chair’s Notes, RAN1 #118.

- RAN1 Chair’s Notes, RAN1 #119.