3GPP TSG RAN WG1 #121			R1-2504506

St Julian’s, Malta, May 19th – 23th, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on on-demand SIB1 for idle/inactive mode UEs

Agenda Item:	9.5.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1#120bis meeting, following agreements and conclusions on the agenda item of on-demand SIB1 for idle/inactive mode UEs were achieved [1]. In this contribution, we provide our views on issues of on-demand SIB1 for idle/inactive mode UEs.

Conclusion There is no RAN1 consensus to support SSB with on-demand SIB1 not on sync raster. Agreement The following agreement is updated as follows: The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to at least the SSB associated with the transmitted PRACH for UL-WUS if this is indicated via UL WUS configuration by a 1-bit indication - If the 1-bit indication is included in UL WUS config and indicate as TRUE, the UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the PRACH for UL-WUS. - If the 1-bit indication is not included in UL WUS config or the 1-bit indication is FALSE, the UE assumes the same as legacy, i.e., in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB. - Further study possible down selection from the following options to indicate additional information related to SSBs associated with PDCCH monitoring occasions - Alt1: The information is provided in UL-WUS config - Alt2: The information is provided in RAR in response to UL-WUS transmission - Alt3: No additional information is provided Agreement Indication of K_SSB value in the UL-WUS configuration should be 5 bits for FR1 and 4 bits for FR2. - Note: there is no change to current PBCH structure. - UE does not expect K_SSB to be larger than 23 for FR1 and larger than 11 for FR2 Agreement The value of the time offset between the reference time point and the starting time for the time window of OD-SIB1 can be either 0 or positive value. Agreement After transmitting a UL-WUS, UE is not required to monitor Type0-PDCCH occasion (for OD-SIB1) before UE successfully received its RAR in response to the UL WUS. Agreement If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=12 for FR2, select the following: - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission Agreement In the UL WUS configuration, how to support multiple NES-CellId(s) is up to RAN2 discussion. Agreement From RAN1 perspective, for agreed UL WUS parameters, regarding their mandatory or optional presence and applicability to TDD and/or FDD, adopt the followings: - PhysCellId and ARFCN-ValueNR are mandatory - frequencyBandList and absoluteFrequencyPointA are present in IE FrequencyInfoUL for FDD (as in the legacy specification) - K_SSB is mandatory - searchSpaceZero and controlResourceSetZero are mandatory - ra-PreambleStartIndex, od-sib1-duration, offsetToTimeWindow are mandatory Agreement The parameter ra-SearchSpace in the UL-WUS configuration is of type SearchSpace. Agreement Replace the current parameter name offsetToTimeWindow with od-sib1-windowStartOffset (to make the naming more comprehensive). Agreement Include CarrierBandwidth and locationAndBandwidth in the UL-WUS configuration. - Previous RAN1 #120 conclusion is reverted Agreement The following previous RAN1 #118-bis agreement is updated as: - Msg1-FrequencyStart is with respect to the first RB of the UE carrier InitialUplinkBWP determined by locationAndBandwidth offsetToCarrier and absoluteFrequencyPointA (same as legacy) Agreement RAN1 to discuss the value range for od-sib1-duration. For example, - Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point. - Option 2: The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity). - Option 3: The duration of the time window can be a multiple of the on-demand SIB1 repetition periodicity (if supported) - Other options are not precluded. Agreement RAN1 clarifies frequencyBandList in UL WUS configuration refers to the IE within FrequencyInfoUL-SIB.

- Discussion

For discussion purpose, we reuse the definition of following in this contribution.

- Cell A: A cell that is periodically transmitting at least its own SIB1.

- NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE.

2.1	Association between OD-SIB1 and SSB

At RAN1#120bis meeting, followings were agreed and discussed.

Agreement (RAN1-120bis) The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the transmitted PRACH for UL-WUS if this is indicated via UL WUS configuration by a 1-bit indication - If the 1-bit indication is included in UL WUS config and indicate as TRUE, the UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the PRACH for UL-WUS. - If the 1-bit indication is not included in UL WUS config or the 1-bit indication is FALSE, the UE assumes the same as legacy, i.e., in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB. - Further study possible down selection from the following options to indicate additional information related to SSBs associated with PDCCH monitoring occasions - Alt1: The information is provided in UL-WUS config - Alt2: The information is provided in RAR in response to UL-WUS transmission - Alt3: No additional information is provided

It is agreed that 1-bit indication in UL WUS config can be used to differentiate the case of one PRACH to trigger OD-SIB1(s) associated with all SSBs and the case of one PRACH to trigger OD-SIB1 associated with one SSB corresponding to the PRACH. The 1-bit indication can already achieve enough flexibility for OD-SIB1 operation. So for the FFS point of above, we prefer Alt3 that no additional information is provided.

Proposal 1:

- For the FFS issue on additional information related to SSBs associated with PDCCH monitoring occasions, we prefer Alt3 (No additional information is provided).

2.2 SSB selection before sending WUS

At RAN1#120bis meeting, following FL proposal was discussed.

FL Proposal 12-2 (RAN1-120bis) For selecting SSB from NES cell for OD-SIB1 request, UE can select any SSB if none of the measured SSBs are above the RSRP threshold (per legacy).

For the issue of SSB selection for PRACH, it should be a MAC procedure as it is captured in RAN2 spec. in legacy. For OD-SIB1, this issue should be left for RAN2 discussion.

Proposal 2:

- It is up to RAN2 to decide the SSB and PRACH resource selection criterion for UE sending a WUS.

2.3	Monitoring of on-demand SIB1

At RAN1#120bis meeting, followings were agreed.

Agreement (RAN1-120bis) RAN1 to discuss the value range for od-sib1-duration. For example, - Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point. - Option 2: The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity). - Option 3: The duration of the time window can be a multiple of the on-demand SIB1 repetition periodicity (if supported) - Other options are not precluded.

In legacy, the periodicity of SIB1 is 160ms, and the repetition periodicity of SIB1 within 160ms is determined by gNB implementation. For OD-SIB1 triggered by WUS sent by a UE, it should be ensured that OD-SIB1 is transmitted at least once within the duration of OD-SIB1 window. So, it is preferred that the duration of OD-SIB1 is at least 160ms. Furthermore, it is better that the duration of OD-SIB1 can be multiple of 160ms to ensure more OD-SIB1 transmission opportunities.

Proposal 3:

- The duration of OD-SIB1 window could be multiple of 160ms, e.g., [160ms, 320ms, 640ms, 1280ms].

At RAN1#120bis meeting, followings were agreed.

Agreement (RAN1-120bis) After transmitting a UL-WUS, UE is not required to monitor Type0-PDCCH occasion (for OD-SIB1) before UE successfully received its RAR in response to the UL WUS.

In the purpose of avoiding misalignment between UE and gNB on the starting of monitoring time and then achieving energy saving, it is better to define a more precision time point on the starting time of Type0-PDCCH monitoring. The definition of the starting timing of PDCCH monitoring could refer to the related discussion of OD-SSB. Fig.1 shows an example procedure of OD-SIB1 monitoring.

Proposal 4:

- UE monitors from the first type 0 PDCCH monitoring occasion associated with one or all SSB(s) according to 1-bit indication in UL-WUS from the starting of the slot which is n slot after the slot of PDSCH of RAR reception.

  - where, n=3Nslotsubframe,μ

Fig.1 Starting time of PDCCH monitoring occasion

At RAN1#120bis meeting, following FL proposal was discussed.

FL Proposal 10-2 UE stops monitoring Type 0 PDCCH at the end of time window for on-demand SIB1. - UE should receive the PDSCH for on-demand SIB1 outside the time window if its corresponding Type 0 PDCCH is located inside the time window for on-demand SIB1.

According to legacy, there is no cross-slot scheduling for the PDSCH of SI including SIB1. Then PDSCH of SIB1 is within the same slot as PDCCH scheduling the SIB1. So above proposal is not necessary.

Proposal 5:

- There is no cross-slot scheduling for SI, then PDSCH of SIB1 is within the same slot as PDCCH scheduling the SIB1. So, following proposal is NOT necessary.

  - FL Proposal 10-2: UE should receive the PDSCH for on-demand SIB1 outside the time window if its corresponding Type 0 PDCCH is located inside the time window for on-demand SIB1.

- Conclusion

In this contribution, we provided the following observations and proposals of on-demand SIB1 transmission for network energy saving.

Proposal 1:

- For the FFS issue on additional information related to SSBs associated with PDCCH monitoring occasions, we prefer Alt3 (No additional information is provided).

Proposal 2:

- It is up to RAN2 to decide the SSB and PRACH resource selection criterion for UE sending a WUS.

Proposal 3:

- The duration of OD-SIB1 window could be multiple of 160ms, e.g., [160ms, 320ms, 640ms, 1280ms].

Proposal 4:

- UE monitors from the first type 0 PDCCH monitoring occasion associated with one or all SSB(s) according to 1-bit indication in UL-WUS from the starting of the slot which is n slot after the slot of PDSCH of RAR reception.

  - where, n=3Nslotsubframe,μ

Proposal 5:

- There is no cross-slot scheduling for SI, then PDSCH of SIB1 is within the same slot as PDCCH scheduling the SIB1. So, following proposal is NOT necessary.

  - FL Proposal 10-2: UE should receive the PDSCH for on-demand SIB1 outside the time window if its corresponding Type 0 PDCCH is located inside the time window for on-demand SIB1.

References

- 3GPP, RAN1#120bis meeting, Chairman’s notes