3GPP TSG RAN WG1 #121	 R1-2504635

St Julian’s, Malta, May 19th – May 23rd, 2025

Agenda item:       9.4.3

Source:	IIT Kanpur

Title:	Discussion on timing acquisition and synchronization aspects for AIoT

Document for:     Discussion and Decision

# Introduction

With 3GPP Rel-18, the RAN plenary initiated a study on a new class of battery-less User Equipment (UE) designed for low complexity and ultra-low power consumption. The study was further extended to Rel-19, where RAN working groups assessed the technical feasibility of such devices. In the recent RAN#107 meeting, a Work Item Description (WID) was defined to support AIoT Device 1 in Deployment Scenario 1 with Topology 1. The agreed spectrum deployment includes in-band and standalone FDD operation in FR1.

In this contribution, we discuss our views on timing acquisition and synchronization for A-IoT.

# Discussion

Here, we discuss the R2D-related aspects, such as the design of the R-TAS signal and the determination of the end of R2D transmissions.

## 2.1 R2D

### 2.1.1 R-TAS related aspects

The R2D time acquisition signal (R-TAS) is a two-part signal consisting of a start-indicator part (SIP) and a clock-acquisition part (CAP). We further discuss the detailed design aspects of each in the subsequent section.

R2D start-indicator part (SIP)

The following agreements has been made in RAN1#120bis[1] meetings related to SIP of R-TAS.

Agreement For the pattern of SIP of R-TAS, only the following 2 alternatives are considered for further down-selection: - Alt 1-2: ON-OFF with a ratio of 1:3 and with following total SIP duration to be further down-selected: - Option 1: 0.5 OFDM symbol duration - Option 2: 1 OFDM symbol duration - Alt 2-4: ON-OFF-ON-OFF with a ratio of 1:1:1:3 and with following total SIP duration to be further down-selected: - Option 1: 0.5 OFDM symbol duration - Option 2: 1 OFDM symbol duration Agreement For R-TAS, SIP duration of 1 OFDM symbol is adopted with CAP pattern ON-OFF-ON-OFF for all values of M corresponding to PRDCH - Note: device cannot assume the presence/absence of RF transmission prior to the SIP.

The R2D transmission is preceded by the R-TAS, in which the SIP (Start Indication Pattern) serves to indicate the beginning of the PRDCH transmission. Given the critical role of the SIP in enabling reliable PRDCH detection, it is essential that the SIP be designed to maximize detection probability and remain agnostic to all signal transmission scenarios.

As per the agreements reached in the previous RAN1 meeting, the SIP duration is limited to one OFDM symbol, and the pattern must follow one of the two approved formats:

- Alt 1-2: ON-OFF pattern with a 1:3 ratio

- Alt 2-4: ON-OFF-ON-OFF pattern with a 1:1:1:3 ratio

These SIP configurations are illustrated in the figures below.

Figure 1: Alt 1-2: ON-OFF with a ratio of 1:3

Figure 2: Alt 2-4: ON-OFF-ON-OFF with a ratio of 1:1:1:3

Further down-selection between the two proposed SIP patterns is necessary. Under the assumption that no RF energy harvesting (RF-EH) occurs before the SIP, Alt 1-2, which follows a simple ON-OFF pattern with a 1:3 ratio, offers a straightforward and easily detectable structure. However, as illustrated in Figure 1, this simplicity may not be sufficient to meet the Minimum Detection Rate (MDR) and False Alarm Rate (FAR) requirements, particularly when the RF-EH scenario is taken into account.

In contrast, Alt 2-4, which includes two ON-OFF segments with a 1:1:1:3 ratio, offers a more robust design. The first ON-OFF segment can act as a guard period, enhancing edge detection by differentiating the transition from the RF energy harvesting signal to the SIP. The second ON-OFF segment can then be effectively decoded as the SIP indicating the incoming CAP and PRDCH signal.

Considering the design objectives and performance requirements, we believe Alt 2-4 better satisfies the criteria for SIP pattern design and is the more suitable option among the two alternatives discussed in the last meeting.

Proposal 1: Alt 2-4 with pattern of ON-OFF-ON-OFF with a ratio of 1:1:1:3, for 1 OFDM symbol duration should be considered for SIP of the R-TAS.

R2D clock-acquisition part (CAP)

For the clock-acquisition part (CAP) of R-TAS signal the following agreement was made in RAN1#120bis[1]:

Agreement For CAP of R-TAS, following is adopted: - Option 1 for CAP of R-TAS from TR 38.769 is adopted where the CAP duration becomes proportionally shorter with increasing value of M, i.e. if for , duration is OFDM symbol long, then for , duration is OFDM symbol long - Note: Duration without CP insertion is considered above, with CP insertion, the total duration may not be exactly proportional - Only following two alternatives for CAP pattern are considered for further down-selection to one alternative: - Alt 1: ON-OFF-ON-OFF - Alt 2: ON-OFF-ON Agreement For R-TAS, SIP duration of 1 OFDM symbol is adopted with CAP pattern ON-OFF-ON-OFF for all values of M corresponding to PRDCH - Note: device cannot assume the presence/absence of RF transmission prior to the SIP.

and based on the earlier agreements, the same pattern is used for increasing M values without any repetitions, and as the M value increases, the duration of the CAP becomes shorter. Also, there is an FFS in 9.4.1 on whether/how CAP uses the same M values set as PRDCH. Since only four M values were agreed upon for PRDCH (2, 6, 12, 24), and their chip durations are not very close, CAP can handle all the M values in the set.

For the CAP, a pattern of ON-OFF-ON-OFF has been agreed in the last RAN1 meeting. Based on the discussion regarding the M value during the last RAN1 meeting, a proposal was raised to consider separate M value sets for PRDCH and R-TAS. However, if different M value sets are used for PRDCH and CAP, it would require a corresponding mapping relationship or predefined rule, introducing additional complexity without a clear benefit for the Rel-19 A-IoT system.

Therefore, we support using the same M value set for CAP as that of PRDCH to minimize implementation complexity. With this alignment, the M value for PRDCH can be derived from CAP without requiring additional signaling. Since only four M values were agreed upon for PRDCH (2, 6, 12, 24), all of them can be used for CAP

Proposal 2: For R2D, support the use of the same M value set of (2, 6, 12, 24) for CAP as is defined for PRDCH.

R2D postamble

One possible approach for determining the end of the PRDCH transmission is for the device to exploit the self-clocking property of the Manchester-coded PRDCH signal. In this method, the reader pads the remaining duration of the final OFDM symbol with always-ON chips. This padding introduces a discontinuity in the recovered clock signal at the device receiver, which can be leveraged to identify the symbol boundary and thus determine the end of the PRDCH transmission.

An alternative approach is to use the TBS (Transport Block Size) indication provided in the control information to determine the transmission boundary.

Both approaches were explored during the study phase, and we believe that a combined solution may provide a more robust mechanism for end-of-transmission detection. Specifically, the TBS can be explicitly indicated in the control information, and, if necessary, the PRDCH can be terminated with a predefined all-ones chip pattern to reinforce end-of-transmission detection at the device.

Proposal 3: The Transport Block Size (TBS) should be explicitly indicated in the R2D control information to enable the device to determine the end of the PRDCH transmission.

Proposal 4: A postamble that violates the self-clocking property of Manchester coding, such as a sequence of always-ON chips, should be appended at the end of the PRDCH transmission to assist in end-of-transmission detection.

# Conclusion

This paper provides the following observations and proposals:

Proposal 1: Alt 2-4 with pattern of ON-OFF-ON-OFF with a ratio of 1:1:1:3, for 1 OFDM symbol duration should be considered for SIP of the R-TAS.

Proposal 2: For R2D, support the use of the same M value set of (2, 6, 12, 24) for CAP as is defined for PRDCH.

Proposal 3: The Transport Block Size (TBS) should be explicitly indicated in the R2D control information to enable the device to determine the end of the PRDCH transmission.

Proposal 4: A postamble that violates the self-clocking property of Manchester coding, such as a sequence of always-ON chips, should be appended at the end of the PRDCH transmission to assist in end-of-transmission detection.

# 4. References

- 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #120bis, Wuhan, China, Apr 7th – 11th, 2025.