3GPP TSG RAN WG1 #122	R1- 2505131

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:		11.5

Source:	Nokia

Title:	Energy Efficiency in 6G Radio

WI code:	FS_6G_Radio

Release:	Rel-20

Document for:		Discussion and Decision

# Introduction

The 6G study item has been approved in [1]. In this contribution, we provide our view to energy efficiency and energy saving for network and devices.

--- Text Omitted – The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion. --- Text Omitted –

# General principles for Joint Network and UE Energy Saving

Energy efficiency is one of the important aspects for the design of 6G networks. However, independent design of network energy saving (NES) and UE energy saving (ES) mechanisms may limit the targeted benefits from both. In some cases, UE energy saving mechanisms may increase power consumption on the network side. Similarly, designing NES operations without taking the UE ES into account may lead to UE energy consumption issues. In previous 5G NR studies, the alignment between network and UE energy-saving functionalities in different RRC states has been deemed beneficial. For 6G, the network energy saving impact on UE energy consumption and vice versa need to be evaluated and taken into account when designing 6G NES operations and UE power saving mechanisms. Moreover, the NES role to overall 6G energy savings is significant, consideration of the role of devices in ensuring network energy saving is crucial.

Proposal 1: 3GPP to study the trade-offs between network and UE energy savings, and to consider joint network and UE optimizations where feasible when designing 6G energy-saving mechanisms.

Furthermore, the design of joint network and UE energy efficiency (EE)/ES mechanisms should be applicable to any use case, including verticals such as low-power wide are (LPWA) device types, sensing, and NTN as well. If additional energy saving schemes on top of the 6G baseline design are deemed beneficial for certain verticals, there are to be studied in the corresponding A.I.

# Views on 6G Network Energy Saving (NES)

## Review of Network Energy Saving schemes in LTE and NR

4G LTE focused on enhanced eMBB capacity and coverage, while maintaining a reasonable trade-off with UE complexity. Network Energy Saving (NES) was not a primary design criterion. The LTE design includes always-on signals and channels, where Cell Specific Reference Signals (CRS) are transmitted in every DL subframe and enable UE cell search, initial acquisition, and downlink channel estimation, and measurements, and the Primary and Secondary Synchronization Signals (PSS/SSS) are transmitted every 5ms and the Physical Broadcast Channel (PBCH), carrying the Master Information Block (MIB), is transmitted every 10ms.

Unlike 4G LTE, the 5G NR design offers significantly improved network energy saving from the beginning. Unlike LTE’s CRSs in every DL slot, NR avoids this always-on signaling. In addition, the NR design integrates PSS/SSS and PBCH in one single SS/PBCH block, optimizing beam sweeping. The default SS/PBCH block transmission periodicity in NR is 20 ms, and this is the setting used by virtually all deployments.

In 5G-A, further work on NES has been conducted in Rel-18 and Rel-19. The Rel-18 study item phase on NES identified several beneficial NES adaptation schemes categorized by time, frequency, spatial and power domains, as summarized in [2]. RAN1 addressed spatial and power domain techniques primarily in the Rel-18 NES WI [3]. Rel-18 NES also specified a time domain technique, such as Cell DTX/DRX for RRC CONNECTED mode UEs. Rel-19 NES WI [4], further considers time and frequency domain techniques. The following lists the 5G-A Rel-18 and Rel-19 NES features defined by RAN1 per radio domain.

### Spatial and Power domain

Multi-CSI with sub-configurations

In 5G-A Rel-18, the CSI report framework was enhanced to support spatial and power adaptation for PDSCH. The network can adjust the active antenna ports/elements and/or power level based on factors such as cell load and UE radio conditions. This is facilitated by the enhanced UE-assisted CSI report, which includes multi-CSI reports for various sub-configurations corresponding to different antenna ports/elements or power levels.

### Time and Frequency domain

Cell DTX/DRX

Introduced in Rel-18, Cell DTX/DRX is a NES feature for RRC CONNECTED mode UEs. It allows the gNB to turn off, partially or completely, transmission and reception of user data traffic, control signaling, and reference signals during cell DTX/DRX non-active periods. The gNB is then able to enter a sleep state and thereby save energy during these non-active periods. In addition, the UEs can also avoid certain transmissions/receptions when the network is non-active and sleep, resulting in better UE energy savings. To avoid non-backward compatible issues, RRC IDLE/INACTIVE UEs are not impacted by cell DTX/DRX operations.

On-demand SIB1

SIB1 transmissions triggered by UE requests are specified in Rel-19 for UEs in RRC IDLE/INACTIVE mode, eliminating the need for regular SIB1 broadcast. The feature is referred to as on-demand SIB1 and can be applied only in multi-carrier scenarios in Rel-19, in which the cell using on-demand SIB1 mode (referred to as ‘NES cell’) is assisted by an anchor cell (referred to as ‘Cell A’) for the provisioning of the WUS resource configuration to the UEs. The UE requests an OD-SIB1 transmission by transmitting a Physical Random-Access CHannel (PRACH) preamble to the NES Cell, which then transmits the SIB1 in response.

On-demand SS/PBCH and SS/PBCH adaptation

In Rel-19, on-demand SS/PBCH (OD-SS/PBCH) is defined for SCell operation for RRC CONNECTED UEs configured with intra-/inter-band Carrier Aggregation. It is supported for both FR1 and FR2 in non-shared spectrum. The network triggers OD-SS/PBCH transmissions, which can be used by the UE for SCell activation, time/frequency synchronization, and L1/L3 measurements. Furthermore, in Rel-19, the dynamic adaptation of SS/PBCH periodicity has also been specified limited to non-cell defining SS/PBCH (NCD-SS/PBCH).

SS/PBCH-less SCell operation

In legacy NR CA, a long periodicity of SS/PBCH transmissions in the SCell is configurable, i.e. via UE-dedicated RRC-signaling. Furthermore, from network energy saving perspective, the SS/PBCH-less SCell operation is a very important feature that is supported since NR CA Rel-15 in intra-band contiguous scenarios, where the UE measurement, i.e. AGC tuning, T/F sync, L1-RSRP, etc. of SS/PBCH-less SCell may rely on the SS/PBCH signal transmissions from the primary or reference cell. In R18 NES WI, co-located FR1 inter-band CA scenario was further specified with requirements defined mainly led by RAN4 WG.

Paging adaptation

In Rel-19, paging adaptation in time domain is defined by extending the paging frame interval, spacing out paging frames more than in legacy systems, while each paging frame may contain more paging occasions than in legacy to maintain overall paging capacity if needed. This approach enables longer cell sleeping periods, reduces overall paging transmissions during low-load conditions, and facilitates more clustered RACH responses to paging. A limitation of Rel-19 is reduced network flexibility in accommodating temporary increases in paging load.

PRACH adaptation

In Rel-19, PRACH adaptation in time domain is defined. It allows the gNB to activate additional PRACH resources in addition to legacy PRACH resources. The additional PRACH resources are semi-statically configured and activated via DCI. They remain active until the expiration of their validity duration.

5G has been adding NES features on top of the NR baseline (Rel-15), which has inherent limitations on applicability of such techniques given the presence of legacy UEs in the field. For 6G, the intent should be to have NES by design from the first release, so that the entire network energy-saving toolbox, incorporating our learnings from LTE and NR, can be applied across all cells and all 6G devices from day-one.

Observation 1: 5G has been adding NES features on top of the NR baseline (Rel-15), which has inherent limitations on applicability of such techniques given the presence of legacy UEs in the field.

Proposal 2: 6GR to consider NES-native design from the first release, with mandatory UE support of the corresponding energy-saving design and features.

## Design goals and key focus areas for NES in 6G

There is a general consensus among the tele-communication industry partners that network energy saving (NES) features are essential for future 6G network developments, and NES is a core requirement for 6G [1]. To our view, leveraging the NES features developed and specified in 5G NR provides a solid foundation for 6G’s first release. Further enhancements specifically for 6G can be also considered, as we are no longer constrained by 5G legacy deployments. Moreover, based on the learnings from 5G NR NES, certain NES features may benefit from assistance from the UE side to maximize overall network energy savings. In the following, we describe key considerations for the NES feature design in 6G.

For initial 6G deployments, typically characterized by low or no-load conditions, the default operational mode of the 6G cells should be NES. The default NES operation mode means for example infrequent transmission of always-on signals and control channels, and reduced PRACH and paging occasions to increase the NES gain. A 6G cell should be able to switch from the NES-native default operation mode (6G day-1 is NES by design), to a less energy conserving mode when the load increases, for example by adapting the periodicity of the broadcasting signals and channels. It may be beneficial to enable a more aggressive default energy-saving operational mode for capacity layer cells than for coverage layer cells. This could involve more aggressive relaxation or omission of broadcast channels and signals in the capacity cells.

Proposal 3: The first 6G release should support a leaner carrier and signaling reduction (of SS/PBCH synchronization signals, system information, PRACH and paging occasions) including on-demand provisioning mechanisms for NES operations of cells operating under low/no load.

### Spatial and power domain aspects

The coming 6G system is expected to be deployed with more antenna elements and a larger number of antenna ports. This certainly opens up the possibility and necessity of applying spatial adaptations to deactivate these power-hungry RF components where feasible, for example based on the NW load and UE radio quality. Moreover, different spatial configurations, e.g. using a different number of active Tx antenna ports, can be associated with different spatial NES strategies to be applied. Similarly, for power domain adaptation, different power NES strategies can be also associated with different power domain configurations, e.g. different TX power levels.

Spatial domain adaptations of Tx antenna ports or TRX chains and power domain adaptations of Tx power level e.g. for PDSCH transmissions are key features allowing to obtain NES. The 6G CSI framework shall support those adaptations for NES operations from the first release. Dynamic adaptation of spatial pattern/elements (e.g. antenna ports, active TRX chains) and dynamic adaptation of power level should be considered in the design of 6G CSI framework. Furthermore, since 6G MIMO operation is envisioned to support both SRS-based and codebook-based approaches, these adaptation techniques require a flexible and efficient CSI report framework as well as a more efficient mechanism than in 5G to deactivate codebook reporting when SRS-based MIMO operation is in use.

- Proposal 4: The first 6GR release should support spatial adaptation (of base station Tx antenna ports/chains) and power adaptation, with enhanced CSI / SRS frameworks for NES operations.

### Cell DTX/DRX

For 6G day-1 design with no restrictions on backward compatibility solutions, cell DTX/DRX operations can be considered for both RRC CONNECTED and IDLE/INACTIVE UEs. Moreover, to further improve the network energy saving performance, the operation of cell DTX/DRX could be jointly considered with the designs of 6G signals/channels and other features. During the non-active period of cell DTX/DRX, further “relaxation” of transmission/reception (e.g. longer periodicity of NW transmission and UE monitoring/measuring/reporting) could be specifically considered for 6G signals/channels compared with active period of cell DTX/DRX. Moreover, the interaction of feature designs, i.e. between cell DTX/DRX and spatial/power-domain adaptation, could also bring further network energy saving gains, e.g. with less antenna elements/ports to be enabled during the non-active period compared with the active period of cell DTX/DRX.

Furthermore, the joint optimization between cell DTX/DRX and UE C-DRX should be carefully considered, to ensure energy efficiency to be achieved at both network and UE side.

Proposal 5: The first 6G release should support Cell DTX/DRX for all RRC states, including enhancements during Cell DTX/DRX inactive time and interactions with other features (such as spatial/power-domain adaptation).

### SS/PBCH

SS/PBCH is one of the key signals/broadcasting channels, and it is assumed that 6G radio will have an equivalent signal for similar purposes as earlier mobile network generations.

The transmission periodicity of SS/PBCH transmission has the great impact to both network and UE energy efficiency. With the shorter SS/PBCH transmission periodicity, the better UE performance can be guaranteed with also optimized UE power consumption, e.g. the UE can perform measurement, tracking, and/or cell searching in a faster manner with less power consumption. But on the other hand, it requires higher network energy consumption at network side when more frequent SS/PBCH transmissions have to be made. The longer SS/PBCH transmission periodicity, the better network energy saving can be achieved. But on the other hand, it may require longer time and even higher complexity at the UE side, i.e. for measurement, tracking, cell searching, etc. It may also result in higher UE power consumption. Thus, the SS/PBCH design for 6G should consider a trade-off between UE complexity and NES at network side.

Proposal 6: 6G studies to consider trade-off between network energy saving and UE complexity for initial access, including relaxing the default SS/PBCH periodicity.

The on-demand reference signal can be considered together with the periodically transmitted SS/PBCH signals, so as to achieve the balance between NES gain and UE complexity.

The network triggered OD-SS/PBCH operation was mainly considered for SCell operation of RRC CONNECTED mode UEs in Rel-19 NES, where it is identified to have the benefits at least for the use case of fast SCell activation. For 6G design with no restriction on backward compatibility, the OD-SS/PBCH operation can be expanded to cover more scenarios, i.e. OD-SS/PBCH operation in PCell. And the RRC IDLE/INACTIVE mode UEs can be further investigated together with RRC CONNECTED mode UEs for OD-SS/PBCH operation. The UE-triggered OD-SS/PBCH mechanism shall also be investigated on top of network triggered OD-SS/PBCH operation.

Proposal 7: On-demand reference signal, e.g. SS/PBCH, operation shall be studied in 6G.

### SS/PBCH-less SCell operation

SS/PBCH-less SCell operation is one of the frequency domain NES techniques introduced in 5G NR since Rel-15. In Rel-19 NES, the network triggered OD-SS/PBCH mechanism have been specified that provides a more effective solution to the (de-)activation of SCell operation. For 6G design, it is proposed to consider flexible and scalable solutions that can be beneficial in different deployment scenarios.

Proposal 8: For 6G design with SS/PBCH-less SCell operation, it is proposed to consider more flexible and scalable solutions that can fit in with different deployment scenarios.

### SIB transmission

SIB1 and other SIBs have a significant impact on NES, therefore their design needs to be carefully considered. Based on NR field feedback:

- For SIBs other than SIB1, the current SI-periodicity is higher than 80ms, based on 38.331. This relatively long periodicity results in a small impact to NW power consumption. In addition, on-demand Other System Information (OSI) has been specified from Rel-15 and potentially enables NES gain compared to SI broadcasting.

- For SIB1, the current periodicity is 160ms. Repetition of SIB1 within the 160ms window is allowed and it is up to NW implementation. However, in typical deployments in NR a repetition every 20ms is used for SIB1 to match SS/PBCH periodicity. Hence from NES and UE perspective, in NR the SIB1 is transmitted every 20ms in most practical deployments. Due to SIB1 transmissions, every 20ms and in each beam, the SIB1 network energy consumption is relatively high. This was addressed in Rel-19 by specifying OD-SIB1, which achieves a high NES gain if the SIB1 requested on-demand is transmitted only in the SS/PBCH beam where the UE transmits the request. However, on-demand SIB1 was limited to multi-cell scenarios in Rel-19, requiring a coverage cell, e.g. anchor cell, to provide the resource configuration for the OD-SIB1 request and based on the provisioning, UE can request SIB1 from a NES cell, capacity cell/non-anchor cell with the OD-SIB1 feature enabled. It is essential that 6G design supports a SIB1 transmission that is more NES friendly from the first release, without adding latency to initial access.

Observation 2: NR on-demand based design for SI and OSI can be considered as a starting point for 6GR NES design.

Proposal 9: Consider extending the Rel-19 OD-SIB1 for different deployment scenarios, including the single cell scenario.

### Paging adaptations

Rel-19 NES WI introduces the clustering of paging occasions in time domain for low-load scenarios and shows that potential NES gain can be achieved. The 6G radio needs to enable paging adaptations to scale paging capacity down during low load to attain more energy efficient operation while enabling swiftly increasing the paging capacity during temporary paging load increase.

Clustering of broadcasting channels and signals, e.g. SS/PBCH, SIB allows the NW to have longer duration of sleep, which can be reflected by higher NES gain. 6G design to consider clustered broadcasting signals and channels as NES-native design.

Observation 3: Clustering based design for network broadcasting and paging may provide longer duration of network sleeping, which can be reflected by higher NES gain.

Proposal 10: 6G design shall consider clustering based design for network broadcasting (signals/channels) and paging operations as NES-native design.

### PRACH adaptation

Rel-19 PRACH time adaptation allows NW (RX chain) to have longer deep/sleep mode which increase the NES gain. However, Rel-19 introduced solutions considering major limitations due to the presence of legacy NR UEs. 6G day-1 PRACH design should consider the adaptation by design, e.g. when a cell is working with low/empty load, sparse PRACH is activated by default.

Proposal 11: 6GR to consider time domain adaptation for PRACH by design.

# Views on 6G UE energy saving

## Review of the key UE energy saving schemes in NR

In 5G NR, the following UE energy saving schemes were specified:

Efficient PDCCH monitoring

In the long run, PDCCH monitoring can be a large contributor to power consumption in the UE, and therefore, reducing (unnecessary) PDCCH monitoring is essential. In NR, some schemes have been defined particularly for RRC CONNECTED UEs to achieve this objective, such as C-DRX, DCI-based search space set group switching, ​PDCCH monitoring skipping, wake-up signal (e.g. DCP, DCI for power saving) for triggering PDCCH monitoring during C-DRX on duration , and DCI-based SCell dormancy (indication).

PDSCH processing reduction

Minimum scheduling slot offset restriction (K0min/K2min) was introduced in Rel-16 to avoid unnecessary activity and enable UE to preserve power. By configuring the minimum scheduling offset restriction, it implies that the PDSCH (or PUSCH) needs to be scheduled at least by certain minimum scheduling slots in advance. This way the UE can be enabled to omit, for example, PDSCH buffering and reception in a slot after decoding the DCI indicating the allocation and instead can microsleep in the rest of any slot after PDCCH monitoring unless a DCI with DL allocation was previously received. ​

Furthermore, as a part of Bandwidth Part (BWP) based power saving operation, adaptation of number of MIMO layers was extended in Rel-16 also to DL PDSCH. In Rel-15 the BWP specific configuration of maximum number of applied MIMO layers was already supported for PUSCH.

Efficient paging operations

During Rel-17, DCI-based PEI (paging early indication) was specified to reduce the UE’s energy consumption by providing the UE with an indication on whether the UE should monitor for paging during Paging Occasion (PO) [8]. If the UE detects PEI that indicates UE to monitor its associated PO, the UE monitors the associated PO. If the UE does not detect PEI on the monitored PEI occasion or the PEI does not indicate the UE to monitor its associated PO, the UE is not required to monitor the associated PO.

Subgrouping has also been specified to allow the UE to only monitor the PO of the subgroup it belongs to, thus, it reduces the UE’s energy consumption by decoding fewer POs. If PEI and subgrouping are configured, UEs monitoring the same PO can be divided into one or more subgroups. The UE monitors the associated PO (to its subgroup) only if the corresponding bit for subgroup the UE belongs to is indicated as 1 by PEI corresponding to its PO.

UE measurement relaxation

Additional UE energy saving techniques were specified by relaxing the UE measurements for RLM/BFD when in RRC CONNECTED. RLM/BFD measurements can be relaxed based on low mobility or good serving cell radio link quality and can enabled/disabled separately outside the C-DRX configurations. Similar techniques have been specified to relax neighbor cell RRM measurements in RRC IDLE/INACTIVE when the UE meets the criteria of low mobility and/or not at cell edge. RRC IDLE/INACTIVE UE can also achieve additional power saving if it is configured with TRS which allows the UE to sleep longer (by skipping some SS/PBCH measurements) before waking up for its PO.

RRC INACTIVE and SDT

To enable efficient and reduced transitions to RRC CONNECTED, 5G NR allows the UE to be in RRC INACTIVE where the UE context is kept at the network while idle-mode-like power saving is achieved at the UE side [2]. RRC INACTIVE UE is also allowed to perform DL/UL small data transmissions (SDT) without needing to transition to RRC CONNECTED or to receive the DCI required for PUSCH scheduling. SDT is specified with and without UE context relocation via RA-based SDT or CG-based SDT. Upon completion of the SDT procedure, the UE is released to RRC IDLE or can continue in RRC INACTIVE.

LP-WUS (low power wake-up signal)

During Rel-18, low power wake-up signal (LP-WUS) was studied to consider a separate low power wake-up receiver (LP-WUR) at the UE, to further reduce the UE power consumption [6]. LP-WUS targets to power saving by enabling to put the UE main radio (MR) in a sleep mode (or turn it off), and to activate it only when needed in a triggered manner. The UE LP-WUR operates with very low power consumption to monitor for LP-WUS. Upon the reception of LP-WUS, the LP-WUR triggers the MR to wake up and communication can start.

Rel-19 WI specified activation of PDCCH monitoring triggered by LP-WUS for RRC CONNECTED [7] in connection with C-DRX, where LP-WUS can be used to trigger PDCCH monitoring during C-DRX on duration timer, or alternatively, during a separate timer. For RRC IDLE/INACTIVE, the procedure of LP-WUS indicating paging monitoring triggered by LP-WUS, LP-SS for synchronization and RRM for serving cell, UE serving cell RRM measurement offloading from MR to LP-WUR, and RRM relaxation of UE MR for both serving and neighbor cell measurements have also been specified in Rel-19.

While the above schemes represent potential candidates to be taken as baseline for the device energy saving during the 6G SI, it is essential to take them as baseline with a view on reducing the number of the overlapping functionalities between the device energy saving schemes. A quite substantial number of overlapping features is one of the shortcomings in the 5G design. To remedy this, pruning is needed, and a principle to adopt while pruning is to prefer static schemes compared to dynamic ones, unless significant benefits are proved to be achieved by employing the dynamic techniques.

Proposal 12: 6G SI to consider NR device power saving mechanisms as baseline, target to avoid overlapping functionalities, and adopt pruning with a preference of static schemes.

## Design goals and key focus areas for UE power saving in 6G

In 6G, the UE energy saving should be among high priority topics for 6G design. The 6G devices should have long(er) battery life for user experience and sustainability compared to legacy devices. This opportunity would be the most effective if we target for the enhancements and sustainable specification from the beginning of 6G. It is especially important to focus on the main mechanisms for UE power saving are all related to minimizing unnecessary operation, monitoring and processing when there is no active data to be received/transmitted by the UE. 5G showed that introducing these mechanisms in later releases leads to backward compatibility problems and inconsistent support across different networks/regions. The main focus of 6G device energy saving features could be set to optimize the energy consumption of smartphones and other consumer devices. While these devices are anticipated to be key device category to optimize in first release of 6G, those also provide a good platform for other kinds of devices to benefit from the energy saving methodology.

Proposal 13: 3GPP specifies mandatory support for UE energy saving features from the first release of 6G.

### Energy-efficient PDCCH monitoring

Typically, the largest UE modem related contributor to unnecessary power consumption is Physical Downlink Control Channel (PDCCH) monitoring. While frequent PDCCH monitoring in 5G New Radio (NR) is a critical component for timely managing resource allocations and signaling for UEs in CONNECTED mode, it may also lead to undesired power consumption with aperiodic and sparse traffic types. As previously discussed, reducing PDCCH monitoring in both CONNECTED and IDLE/INACTIVE modes has been a significant focus in 5G, resulting in the introduction of several related enhancements. Studying mechanisms to enable minimizing the unnecessary PDCCH monitoring for UE power saving while accounting for different types of traffic scenarios and without penalizing packet latency, should be also in focus for 6G. Well established methods such as C-DRX, PDCCH monitoring adaptation together with WUS-based operation should be studied to enable energy efficient UE operation in 6G.

Observation 4: It is important to avoid defining multiple features with overlapping effect for PDCCH monitoring reduction in 6G.

Proposal 14: RAN1 to study streamlined mechanisms to enable PDCCH monitoring in 6G only when needed, leveraging 5G techniques for limiting unnecessary PDCCH monitoring while avoiding overlapping features.

### 6G Low power wake-up signal and receiver

Rel-19 specified LP-WUS/SS for paging monitoring reduction and enabled serving cell RRM measurement offloading from MR to WUR in RRC IDLE/INACTIVE. However, in order to optimize the WUR power consumption the receiver sensitivity was relaxed, and that together with the low complexity signal design (to enable low complexity WUR) makes it not practical to achieve full coverage reliably with WUS. Hence, LP-WUS monitoring operation is essentially disabled at cell edges, and normal MR based operations are resumed. In context of 6G WUS, enabling better coverage with smaller footprint of the signal (i.e. a shorter duration signal requiring fewer REs) could help to extend the power saving benefits of WUS-based paging monitoring and PDCCH monitoring to the whole cell. Furthermore, with extended WUS/WUR capabilities in 6G, WUS/WUR could be capable of carrying out additional neighbour cell search/measurements in IDLE/INACTIVE states and relaxing/offloading L1/CSI measurements in CONNECTED state (e.g., during periods of low data activities). This allows the UE to operate in more power efficient manner more of the time. Evidently WUR complexity is a trade-off with power consumption, but the overall benefit is expected to larger, and the monitoring/measurement procedures could be simplified.

Furthermore, given the power saving gains shown in NR, evaluating the applicability WUS to a wider range of 6G operations could be beneficial. This includes the indications to dynamic changes of the configurations (in different UE states), measurements relaxation/offloading, and sending paging messages and control information via LP signals.

Proposal 15: 6G SI to study feasibility of WUS/WUR design with improved coverage, reduced footprint and extended applicability to other operations and measurements.

### Energy-efficient RRC states and state transitions

The control plane latency in IMT-2030 requirements refers to the transition time from a most “battery efficient” state (e.g. Idle/Inactive state) to the start of continuous data transfer (e.g. Active state). In IMT-2020 the minimum requirement for control plane latency was 20 ms, and proponents were encouraged to consider lower value as well, e.g. 10 ms. For IMT-2030 it was proposed to set the requirement for CP latency as 10 ms.

In NR, three RRC states were designed to balance network efficiency, user experience, and device battery life. Transitions between these states are governed by data activity, mobility patterns, and network policies. The UE may be released from the CONNECTED state to a low activity state (i.e., INACTIVE) when data transmission ceases but rapid resume is anticipated. When there is incoming data (e.g., MO traffic) or paging (e.g., MT traffic), the UE is triggered to transition from INACTIVE to CONNECTED via a fast resume. In NR, INACTIVE state is introduced to be optional which led to inconsistent support across different networks/regions. For energy efficient procedures in 6GR, e.g. UE reachability and mobility, the introduction of RRC states should assume that all the designed states are mandatory to be supported and the study of energy saving methods (e.g., SDT for efficient support of background, small and sporadic traffic) takes this into account.

Observation 5: It is preferable to ensure that energy-efficient procedures such as SDT are mandatory to be supported at both the network and UE from the first release of 6G.

### Efficient measurements and measurement reporting

In 5G, L1/CSI measurements and reporting is controlled by various network configurations and UE requirements, which may be relaxed considering DRX configurations and mobility states (e.g. for UEs not-at-cell-center and UEs with low mobility). In 6G, the L1/ CSI measurements and reporting design should be simplified and optimized for efficiency. This involves a design in which measurement activity and reporting levels should match the actual conditions, ensuring sufficient information for the network while minimizing UE power consumption. For example, a design that adjusts CSI reporting and SRS transmission’s frequency, accuracy, and timing based on data activity would be beneficial. This could involve means for triggering CSI reporting outside DRX active time, ensuring availability at the start of ON duration, and likewise for triggering SRS transmission as close as possible to the SR/BSR, ensuring immediate availability at the network. Furthermore, relaxing CSI reporting during periods of low data activities can be considered to conserve UE power.

Another example design that takes the impacts (on UE power consumption) of increased CSI and SRS measurements and reporting because of the likely increase in transmit antennas in 6G would be worth study. An energy-aware mechanism that leverages joint network-UE coordination and dynamically manages the CSI and SRS transmissions/reporting based on real-time conditions could be beneficial. In addition, data-driven techniques could be further used to reduce the number of required measurements and associated CSI and SRS transmissions without compromising the performance.

Furthermore, the study of the L1/ CSI measurements could start with the focus on what measurements are needed, what/when measurements are needed less often and how to balance the needed monitoring with power consumption to achieve the best trade-off between system performance and energy efficiency in both network and devices. The deployment scenarios for L1/ CSI power saving can consider different mobility profiles of UEs, e.g. stationary, pedestrian, and high speed. Considering the mobility profile, the UE could do only the measurements that are accounting to the UE power consumption while enabling the expected system performance.

Methods to enable UE energy efficient operation with L3 mobility measurements and procedures should also be considered in 6G SI. These belong more to the jurisdiction of RAN2 and RAN4, and it is felt that these should hence be discussed in these working groups.

Proposal 16: RAN1 to study methods for relaxing L1/CSI measurements and measurement reporting based on the needs of the actual conditions.

# Power consumption models

In [1], Physical Layer structure for 6GR (objective 2), there is the objective of evaluating the energy efficiency of the identified physical layer features.

- Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1].

Further, in [9], the target is to discuss the evaluation assumptions for 6GR air interface objectives.

11.2 Evaluation assumptions for 6GR air interface Discussions on models, scenarios, parameters, and methodology, metrics/criteria that can be used for evaluating technology proposals, energy efficiency, sensing performance (including potential extension of channel model).

## 6G Base Station power consumption model

A base station (BS) power consumption model is required for the evaluation of 6G energy efficiency compared to 5G as well as for evaluating NES gains of selected 6G design and feature options.

The 5G BS energy consumption model, defined in TR38.864, includes the reference configurations for FR1 TDD/FDD and FR2, the deep/light/micro sleep power states with corresponding relative power and transition time/energy, and scaling rules for scaling the active DL/UL power in time, frequency, power and spatial domains to reflect the corresponding adaptations.

The 5G BS power consumption model could be used as the reference model in evaluating the BS power consumption in 6G, where potential value adjustments may be considered, e.g. according to the 6G numerology and parameters, and expected hardware advances.

TR38.864 defines the above parameters for two BS types, so-called Category 1 and 2 BSs. However, different models may lead to different NES evaluations and conclusions, and therefore a single model should be preferred for 6G evaluations. For example, a model with slow transition to the deep sleep mode can show limited NES gain for infrequent always-ON signals, while a faster transition can lead to higher NES gain.

Observation 6: Using a single, consistent BS power model is recommended to maintain consistency and avoid complexities associated with multiple models.

Proposal 17: 6G SI to use the 5G BS power model Cat.2 in TR 38.864 as a starting point for network energy saving evaluations.

## 6G UE power consumption model

Similarly, a UE power consumption model is required for the evaluation of 6G improvements compared to 5G as well as for evaluating UE ES gains of selected 6G design and feature options.

In 5G, as part of the Rel-16 UE power saving study, a UE power consumption model and methodology [10] were defined for the evaluation of UE power saving schemes. This model and framework were utilized and extended in latter 5G releases, e.g. for LP-WUS/WUR [6]. These studies defined models for different UE power states (e.g., deep sleep, light micro sleep, PDCCH-only, etc.) with corresponding characteristics and relative power consumption, transition time, transition energy, and scaling rules for the different power states. The studies also defined models for UE power consumption for RRM measurements for intra/inter-frequency measurements and cell search.

The UE power consumption models developed in 5G could be used as the reference model in evaluating the UE power consumption in 6G, where potential value adjustments may be considered, e.g. according to the 6G numerology and parameters and expected hardware advances.

Proposal 18: 6G SI to use the 5G UE power consumption model (e.g. in TR 38.840 and in TS38.869) as a starting point for UE energy saving evaluations.

## Energy efficiency with AI/ML

Several use-cases utilizing AI/ML techniques have been studied and are under normative specification phase in NR. In [1], one objective is to study the AI/ML for 6GR and RAN, leveraging 5G AI/ML framework. In general, for all these use-cases in NR and use cases to be identified for 6GR, there will be a trade-off between accuracy and complexity. However, it is also important to evaluate the impacts on energy consumption (at both UE and network side) when AI/ML use cases are studied, so that we have a more complete understanding of the overall relative energy efficiency of different AI/ML-enabled solutions.

Observation 7: 6G SI to discuss how to evaluate energy efficiency at both the UE and Network when AI/ML techniques are applied.

# Conclusion

This document has made the following observations and proposals:

Observation 1: 5G has been adding NES features on top of the NR baseline (Rel-15), which has inherent limitations on applicability of such techniques given the presence of legacy UEs in the field.

Observation 2: NR design for SI and OSI is NES by design and can be a starting point for 6GR design.

Observation 3: Clustering based design for network broadcasting and paging may provide longer duration of network sleeping, which can be reflected by higher NES gain.

Observation 4: Unlike 5G, it is important to avoid defining multiple features with overlapping effect for PDCCH monitoring reduction in 6G.

Observation 5: It is preferable to ensure that energy-efficient procedures such as SDT are mandatory to be supported at both the network and UE from the first release of 6G.

Observation 6: Using a single, consistent BS power model is recommended to maintain consistency and avoid complexities associated with multiple models.

Observation 7: 6G SI to discuss how to evaluate energy efficiency at both the UE and Network when AI/ML techniques are applied.

Proposal 1: 3GPP to study the trade-offs between network and UE energy savings, and to consider joint network and UE optimizations where feasible when designing 6G energy-saving mechanisms.

Proposal 2: 6GR to consider NES-native design from the first release, with mandatory UE support of the corresponding energy-saving design and features.

Proposal 3: The first 6G release should support a leaner carrier and signaling reduction (of SS/PBCH synchronization signals, system information, PRACH and paging occasions) including on-demand provisioning mechanisms for NES operations of cells operating under low/no load.

Proposal 4: The first 6GR release should support spatial adaptation (of Tx antenna ports/chains) and power adaptation, with enhanced CSI / SRS frameworks for NES operations.

Proposal 5: The first 6G release should support Cell DTX/DRX for all RRC states, including enhancements during Cell DTX/DRX inactive time and interactions with other features (such as spatial/power-domain adaptation).

Proposal 6: 6G studies to consider trade-off between network energy saving and UE complexity for initial access, including relaxing the default SS/PBCH periodicity.

Proposal 7: On-demand reference signal, e.g. SS/PBCH, operation shall be studied in 6G.

Proposal 8: For 6G design with SS/PBCH-less SCell operation, it is proposed to consider more flexible and scalable solutions that can fit in with different deployment scenarios.

Proposal 9: Consider extending the Rel-19 OD-SIB1 for different deployment scenarios, including the single cell scenario.

Proposal 10: 6G design shall consider clustering based design for network broadcasting (signals/channels) and paging operations as NES-native design.

Proposal 11: 6GR to consider time domain adaptation for PRACH by design.

Proposal 12: 6G SI to consider NR device power saving mechanisms as baseline, target to avoid overlapping functionalities, and adopt pruning with a preference of static schemes.

Proposal 13: 3GPP specifies mandatory support for UE energy saving features from the first release of 6G.

Proposal 14: RAN1 to study streamlined mechanisms to enable PDCCH monitoring in 6G only when needed, leveraging 5G techniques for limiting unnecessary PDCCH monitoring while avoiding overlapping features.

Proposal 15: 6G SI to study feasibility of WUS/WUR design with improved coverage, reduced footprint and extended applicability to other operations and measurements.

Proposal 16: RAN1 to study methods for relaxing L1/CSI measurements and measurement reporting based on the needs of the actual conditions.

Proposal 17: 6G SI to use the 5G BS power model Cat.2 in TR 38.864 as a starting point for network energy saving evaluations.

Proposal 18: 6G SI to use the 5G UE power consumption model (e.g. in TR 38.840 and in TS38.869) as a starting point for UE energy saving evaluations.

# References

[1] RP-251881, “Study on 6G Radio”, NTT Docomo, June 2025.

[2] TR 38.864, “Study on network energy savings for NR”, v.18.1.0, Rel-18, March 30, 2023.

[3] RP-234009, “Revised WID on Network energy savings for NR”, RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023.

[4] RP-250343, “Revised WID on enhancements of network energy savings for NR”, RAN Meeting #107, Incheon, Korea, March 12-14, 2025.

[5] TS 38.300: NR and NG-RAN Overall description; Stage-2.

[6] TR 38.869: Study on low-power Wake-up Signal and Receiver for NR.

[7] RP-241824: Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR).

[8] TS 38.304: NR; User Equipment (UE) procedures in Idle mode and in RRC Inactive state.

[9] R1-2505100: 3GPP TSG RAN WG1 #122 agenda.

[10] TR 38.840: Study on User Equipment (UE) power saving in NR.