3GPP TSG RAN WG1 #122               						R1- 2505635

Bangalore, India, August 25th – August 29th, 2025

Agenda item:	      10.2.1

Source:	      Tejas Networks Ltd

Title:		      SRS Capacity and Coverage Enhancements for NR MIMO Phase 6

Document for:     Discussion and Decision

Introduction

The Rel-20 WID on NR MIMO Phase 6 [1] specifies the following objectives on UL capacity and coverage enhancements.

- On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots

In this contribution, we provide our views on the above objectives that are identified to improve the performance of TDD systems, especially for cell edge users, by enhancing the SRS capacity and coverage. This contribution considers certain issues on this first objective from the Rel 20 5GA WID that specifies SRS repetition enhancements.

SRS capacity and coverage enhancements

According to 5GA Rel 20 MIMO Phase 6 WID [1], two sub-objectives are identified under this agenda item for improving the SRS capacity and coverage. We discuss a few issues on these two items in the following sections.

## RB-level partial frequency sounding enhancement

To improve SRS capability/coverage, RB-level partial frequency sounding (RBPFS) was earlier introduced in Rel-17. Unlike full band sounding, partial band sounding saves UE power and resources. Clearly, this reduces the UE transmit power demand and thereby the extent of PA linearity. It also lowers the resource overhead, while at the same time enabling improved UE multiplexing for the gNB.

Though this feature was introduced in an earlier release, the specification primarily supported a single FD starting position within each SRS frequency hop. This creates certain limitations on the performance of channel interpolation (mostly leading to extrapolation). Hence, enhancing the feature with multiple frequency domain starting positions in the respective time domain symbols within each SRS frequency hop can help to enhance the channel estimation accuracy through channel interpolation.

Regarding RPFS enhancements, support enhanced SRS hopping pattern with multiple frequency starting position of SRS within a frequency hop in partial sounding.

Regarding RPFS enhancements, consider introducing a factor to change the frequency starting position of SRS within a frequency hop by accounting for the channel coherency, to enable efficient interpolation between the measured chunks of REs.

## Cross-slot SRS repetitions

Based on RAN #108 discussions and the Rel 20 WID [1], the scope of configuring the TD resource allocation of an SRS resource is to be extended to beyond the slot boundary as compared to the legacy restriction of configuring it within a slot. This enhancement is particularly relevant for practical TDD deployments, where the SRS is typically configured in an S slot. By efficiently utilizing the limited uplink (UL) symbols available, such configurations ensure non-degraded data transmission and contributes to a cleaner, more robust system design.  Hence, S slot is a realistic slot for practical SRS transmissions.

Having said that, some of the most feasible special slot structures configured by vendors/operators in deployments are DL:6, GP:4, UL:4 and DL:10, GP:2, UL:2 With this assignment in current practical deployments, there is a restriction on the maximum number of SRS symbols for repetition to 4 even though standard allows additional repetitions of 5,6,7,8,10,12 and 14 symbols. It may be noted that even if network wishes to utilise SRS repetitions in subsequent UL slots, the non-contiguous nature of the allocation may make the aggregation futile if a UE fails to maintain phase coherency across aggregated symbols.

To overcome this limitation of legacy, it is now decided to allow cross slot SRS allocation wherein SRS symbols for repetition are configured in the end of S slot and begin of U slot contiguously. This provides a more flexible allocation that helps to improve SRS coverage. As noted, the proponents suggest an overall 30% cell edge user performance with 8 repetitions as compared to 2 repetitions.

The cross-slot SRS repetition solves an issue in the field for DDDSU patterns with antenna switching for massive MIMO reciprocity. In our view, to achieve the full advantage of repetition-based coverage enhancement for the UE configured with the usage as antennaSwitching, the cross-slot repetition should not be limited to within an SRS resource, but rather extended to multiple SRS resource sets.

For cell edge UEs configured with the usage of SRS as antennaSwitching, the cross-slot repetition should not be limited to within an SRS resource, but extended to multiple SRS resource sets.

Support extending the scope of cross-slot repetition to within multiple SRS resource sets.

The main motivation of cross-slot SRS repetition is to enable efficient MIMO transmission and reception with improved coverage in networks with limited number of configured UL symbols in the S slot. Hence, to strictly adhere to the scope of the work item and to not overlap with similar studies in 6G, our view is to limit configuring the resources/resource sets in contiguous resource elements in two consecutive slots of which the first is an S slot and the second is a UL slot.

Bringing in multiple variants of options to configure SRS resources/resource sets for the purpose of configuring cross-slot repetitions can increase scope of 5GA work item and overlap with similar studies in 6G.

Regarding cross-slot SRS repetition, consider limiting the repetition of SRS symbols of the configured SRS resources/resource sets to contiguous REs across one S slot and the consecutive UL slot.

Regarding cross-slot SRS repetition, support removing the legacy restriction that an SRS resource can only be mapped to a single slot not exceeding the slot boundary.

In order to improve the SRS capability and coverage, the maximum number of repetitions of SRS may be extended to support up to 14 consecutive repetitions of symbols. However, increasing the number of repetitions of SRS transmission by a large value is not an efficient way to improve channel estimation accuracy and thereby the UE coverage, as it will also reduce the SRS multiplexing capability. Other techniques like reducing hopping bandwidth or partial sounding, with or without repetition, can also be used to increase coverage without impacting the UE multiplexing capability of SRS. Hence, even though 14 symbol repetitions within a slot is supported in legacy, for the cross-slot SRS repetition, our view is to restrict to a lower value of repetition by also considering the practical requirements of SRS multiplexing capability.

Increasing the number of repetitions of SRS transmission by a large value is not an efficient way to improve channel estimation accuracy and thereby the UE coverage, as it will also reduce the SRS multiplexing capability.

Regarding cross-slot SRS repetition, support restricting the maximum number of SRS repetitions combining the SRS symbols in S and UL slots to a value lower than 14, by studying the practical requirements of SRS multiplexing capability.

Conclusions

In this section we provide a summary of our observations and proposals.

For cell edge UEs configured with the usage of SRS as antennaSwitching, the cross-slot repetition should not be limited to within an SRS resource, but extended to multiple SRS resource sets.

Bringing in multiple variants of options to configure SRS resources/resource sets for the purpose of configuring cross-slot repetitions can increase scope of 5GA work item and overlap with similar studies in 6G.

Increasing the number of repetitions of SRS transmission by a large value is not an efficient way to improve channel estimation accuracy and thereby the UE coverage, as it will also reduce the SRS multiplexing capability.

Regarding RPFS enhancements, support enhanced SRS hopping pattern with multiple frequency starting position of SRS within a frequency hop in partial sounding.

Regarding RPFS enhancements, consider introducing a factor to change the frequency starting position of SRS within a frequency hop by accounting for the channel coherency, to enable efficient interpolation between the measured chunks of REs.

Support extending the scope of cross-slot repetition to within multiple SRS resource sets.

Regarding cross-slot SRS repetition, consider limiting the repetition of SRS symbols of the configured SRS resources/resource sets to contiguous REs across one S slot and the consecutive UL slot.

Regarding cross-slot SRS repetition, support removing the legacy restriction that an SRS resource can only be mapped to a single slot not exceeding the slot boundary.

Regarding cross-slot SRS repetition, support restricting the maximum number of SRS repetitions combining the SRS symbols in S and UL slots to a value lower than 14, by studying the practical requirements of SRS multiplexing capability.

References

- RP-251856, “New WID: NR MIMO Phase 6”, Samsung (Moderator), 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025.