3GPP TSG RAN WG1 #122			R1-2505652

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	10.5.1

Source:	InterDigital, Inc.

Title:	Evaluation assumptions and performance evaluation of NR ISAC

Document for:	Discussion and Decision

# Introduction

The SID on Integrated Sensing and Communication (ISAC) for NR was approved in RAN#108 (FS_Sensing_NR_Ph2, RP-251861) [1]. The main objective of the study is to evaluate the performance of gNB-based monostatic sensing (i.e., single TRP with co-located sensing transmitter and receiver). In this contribution, we provide views on the metrics, measurements, quantization, signals, and assumptions for performance evaluation. Some preliminary performance evaluation results are also provided.

# Sensing KPIs and Methods

NR sensing service is required to fulfil performance requirements based on the characteristics of one or multiple target object(s) and/or the environment to be sensed in a target sensing service area. 3GPP TS 22.137 [2] provides a set of KPIs for NR ISAC use cases.

The corresponding sensing KPIs from 3GPP TS 22.137 may be structured into the following categories:

- Sensing accuracy: defined as the difference between the measured and expected or true value of the sensing metric, e.g., accuracy of target positioning, velocity estimation, detection, etc. It may also include the measurement of the error in the sensing metric, e.g., using RMSE.

- Detection probability: which reflects the ability to detect the presence of a specific target and encompasses the probabilities of target detection, missed detection, and false alarms.

- Latency: corresponds to the time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result.

Observation-1: 3GPP TS 22.137 provides KPIs for NR ISAC, including sensing accuracy, detection probability and latency.

For gNB-based monostatic sensing for UAV detection/tracking, the corresponding sensing service categories are 1,2,3, or 4 from 3GPP TS 22.137 (Table 6.2-1) [2]. For example, for detection of outdoor UAV, the positioning accuracy is expected to be within 2 meters (horizontal) and 5 meters (vertical) at a 95% confidence level.

Table 1: NR sensing requirements (3GPP TS 22.137).

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 1 | 95 | 10 | 10 | N/A | N/A | 10 [3] | 5 [3] | 1000 | 1 | 5 | 2 | Indoor/outdoor (e.g., detection of human, UAV) |
|  | 2 | 95 | 2 | 5 | 1 | N/A | 1 | 1 | 1000 | 0.2 | 0.1 to 5 | 5 | Outdoor (e.g., detection of human, UAV) |
|  | 3 | 95 | 1 | 1 | 1 [3], [4] | 1 | 1 [3], [4] | 1 x 1 [3] | 100 [2], or 1000 (NOTE 3); 5000 for detection in highway | 0.05 to 1 | 2 | 2 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV) |
|  | 4 | 99 for public safety, otherwise, 95 | 0.5 | 0.5 | 1.5 for pedestrian, 15 for vehicle, otherwise, 0.1 | 1.5 for pedestrian | 0.5 | 0.5 x 0.5 for factories | 100 to 5000 | 0.1 | 1 | 3 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV, AGV, vehicle) |
| Environment monitoring | 5 | 95 | 10 | 0.2 (NOTE 4) | N/A | N/A | N/A | N/A | 60000 | 60 to 600 | 0.1 to 5 | 3 | Nature of environments monitored by sensing (e.g. rainfall, flooding monitoring) |
| Motion monitoring | 6 | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60000 | 60 | 5 | 5 | Human motions and activities obtained by sensing (NOTE 5) |
|  | 7 | 95 | 0.2 | 0.2 | 0.1 | 0.1 | 0.375 | 0.3 | 5 to 50 | 0.1 | 5 | 5 | Human hand gestures obtained by sensing (NOTE 6) |
| NOTE 1: For sensing service categories to which UAV, human or vehicle is a sensing target, the typical size (Length x Width x Height) of UAV is 1.6m x 1.5m x 0.7m, the typical size of human is 0.5m x 0.5m x 1.75m, and the typical size of vehicle is 7.5m x 2.5m x 3.5 m. NOTE 2: The safe distance between pedestrian/vehicle and power transmission station/line is 0.7m/0.95m. NOTE 3: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1 s. NOTE 4: This value is derived from the water level where people feel difficulty in walking. NOTE 5: To achieve human motion monitoring, different accuracy KPI is needed to measure different human motions. E.g., respiration rate accuracy (2 times/min) is a KPI used to measure the accuracy of sleep monitoring, sit-up rate accuracy (3 times/min) is a KPI used to measure the accuracy of sports monitoring. NOTE 6: Category 7 has more stringent requirements (e.g., for KPIs such as positioning accuracy and sensing resolution) compared to other categories and typically requires more radio resources. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation-2: The applicable sensing service categories for gNB-based monostatic UAV detection and tracking use cases are 1 – 4 from 3GPP TS 22.137 (Table 6.2-1).

The service categories 1 – 4 cover a wide range of deployment scenarios and targets, and in turn, KPIs, for both detection and tracking sensing use cases. In view of the limited TU for the Rel-20 NR ISAC-UAV, it may not be feasible to rigorously evaluate all these cases.

Observation-3: It may not be feasible to evaluate both target detection and tracking use cases related to sensing service categories 1 – 4 in 3GPP TS 22.137 (Table 6.2-1) in Rel-20.

Based on Observations 1—3, we draw the following proposal.

Proposal-1: Down-select sensing KPIs and values from sensing service categories 1 – 2 in 3GPP TS 22.137 (Table 6.2-1) to prioritize UAV detection use case for Rel-20 NR ISAC-UAV performance evaluation – UAV tracking use case can be considered with lower priority.

In gNB-based monostatic sensing, existing or new (e.g., implementation specific) waveform/RS can be used for sensing of UAV targets. In [2, Table 6.2-1]. The main sensing metrics for NR ISAC-UAV are:

- Target Positioning: This metric encompasses measurement parameters such as ToA, AoA, and AoD. The associated positioning uncertainty and resolution are derived from the estimation accuracy of these underlying parameters, which are critical for precise spatial localization.

- Target Velocity Estimation: Velocity-related metrics are typically inferred from Doppler shift measurements of the reflected RS, enabling dynamic tracking of moving targets such as UAVs.

- Target Detection: This can be defined as the detection of the presence of the target. This is typically obtained from identification of peaks in the PDP or CIR of the reflected RS, e.g., using RSRP per path (RSRPP) measurements.

Observation-4: Positioning accuracy depends on the precision of ToA, AoA, and AoD measurements, while velocity can be derived from Doppler shifts of reflected RS. Target detection relies on analyzing PDP or CIR.

There exist different NR positioning methods such as Multi-cell Round Trip Time (Multi-RTT), Downlink Time Difference of Arrival (DL-TDOA), and Downlink Angle of Departure (DL-AoD). To support UAV target localization in gNB-based monostatic sensing, NR legacy positioning methods can be extended and used for sensing target localization:

- Time-based Methods: Utilize signal propagation delays to estimate target location.

- Angle-based Methods: Rely on directional measurements of signal transmission or reception.

- Hybrid Methods: Combine both time-based and angle-based measurements to improve positioning accuracy and robustness.

Observation-5: There are different methods for measuring the UAV position including delay-based, angle-based, and hybrid delay-angle-based approaches.

Time-dependent and angle-dependent methods often require the involvement of multiple transmitters and/or receivers positioned at different locations relative to the target. However, for gNB-based monostatic sensing with a single TRP, the lack of spatial diversity can impose constraints on the effectiveness of these methods. Combining delay and angular methods (hybrid sensing method) can result in improved accuracy.

Observation-6: Time-based and angle dependent methods may pose limitations for UAV localization using gNB-based monostatic sensing with single TRP, motivating hybrid-based methods to combine delay and angular information.

Proposal-2: Evaluate gNB-based monostatic (with single TRP) sensing performance using at least hybrid delay-angle-based sensing methods.

It is possible to consider ways to enhance sensing by leveraging measurements from multiple monostatic sensing TRP nodes. However, the performance of multi-TRP monostatic sensing relies on coordination of multiple TRPs and is prone to both synchronization and timing errors.

Observation-7: gNB-based monostatic sensing can be performed based on measurements from a single monostatic TRP or using measurements from multiple monostatic TRPs. The latter however requires synchronization across multiple monostatic TRPs.

Proposal-3: Prioritize performance evaluation for gNB-based monostatic sensing using only a single-TRP in Rel-20.

Signal attenuation increases with gNB and the sensing target distance, which degrades the accuracy of ToA and AoA estimations. Hence, targets located near the cell edge experience the highest levels of path loss, making precise sensing more challenging. Also, the separation distance must exceed the system’s maximum unambiguous range, which itself depends on the signal configuration such allocated time slots, signal periodicity, and repetition factor. These parameters determine the temporal resolution and the ability to distinguish between multiple signal reflections. Moreover, the presence or absence of a direct LoS path between the gNB and UAV significantly affects the reliability of sensing measurements.

UAV orientation also impacts the NR ISAC-UAV system performance. For large UAV size, the RCS is sensitive to the incidence and scattering angles of the RS beams. Consequently, the reflected RS power may vary with the UAV’s orientation relative to the gNB. Other important UAV characteristics include UAV mobility conditions (e.g., maximum/minimum velocity).

Reflections from surrounding environmental objects (classified as EO Type 1 and Type 2) may also interfere constructively or destructively with UAV reflections at the gNB receiver, leading to degraded measurement accuracy. Factors such as ground reflections and ambient noise can further distort the received signal.

Observation-8: There are factors impacting the sensing measurements from an NR ISAC-UAV system with single TRP, including TRP position and orientation with respect to the UAV, the LoS/NLoS status of the TRP-UAV link, UAV characteristics, and environmental conditions.

# Measurements and Quantization

In 3GPP TS 38.215 [3], a suite of existing RS measurements is applicable to gNB-based monostatic sensing, including power metrics (e.g., RSRP, RSRPP), quality indicators (e.g., RSRQ, SINR), and angular parameters (e.g., DL-AoD). Some aspects of using these existing RS measurements for sensing can be further studied. For example, a common interpretation of the PDP in identifying distinct peaks corresponding to multiple targets can be discussed. Also, peak detection criteria and the reference path for ToA estimation in gNB-based monostatic scenarios may be discussed further.

Observation-9: Existing NR RS measurements (angle-based, time-based, power-based, quality-based, phase-based) can be reused for gNB-based monostatic sensing.

Proposal-4: Reuse existing NR RS measurements for Rel-20 NR ISAC-UAV performance evaluation.

In addition to existing RS measurements in NR, sensing-specific measurements can be considered. These depend on one or more legacy RS measurements and can be particularly useful in challenging sensing scenarios, e.g., with multiple targets or/and EOs.

Observation-10: There are sensing-specific measurements that are well suited to supporting the UAV sensing in challenging sensing scenarios.

Proposal-5: Sensing-specific measurements should be used in addition to existing NR RS measurements for Rel-20 NR ISAC-UAV performance evaluation.

A key sensing-specific measurement is RCS, which can be applied for a variety of sensing use cases such as object detection and tracking, e.g., when distinguishing a target from others is challenging using legacy measurements. RCS in 3GPP TR 38.901 [4] is defined as a scalar value defined in Local Coordinate System (LCM) of the sensing target and is dependent on both the incident angle and the scattered angle. The RCS values with same incident/scattered angles can be referred to as monostatic RCS values.

The RCS is composed of three main components defined as σRCS=σMσDσS, where the first component is modelled as a large-scale parameter and representing RCS mean denoted by A, the second component can be assigned to 1 or be angular-dependent (represented by B1), and the third component is modelled using log-normal distribution represented by B2. The mean (A) or μσS_dB and standard deviation (B2) σσS_dB used to characterize 10lgσS satisfied a fixed relation. In [4], the RCS of UAVs is modelled in Table 7.9.2.1-1.

Table 2: UAV monostatic RCS Models (3GPP TR 38.901).

| Target Type | RCS Model |  |  | Comments |
| --- | --- | --- | --- | --- |
| UAV | Small size | Monostatic | A: -12.81 dBsm B1: 0 dB B2: modelled using log-normal distribution (standard deviation σ : 3.74 dB) | Upper bound is kσ with k =3 |
|  | Large size | Monostatic | A: -5.85 dBsm B1: have dependency on incident/scattered angles B2: modelled using log-normal distribution |  |

Observation-11: The UAV monostatic RCS models are defined in 3GPP TR 38.901.

The RCS calculation depends on the associated RS measurements (angle measurement, power, time Doppler, spatial)—such as RSRP, RSRP per path (RSRPP), AoD, AoA, and ToA. For example, the RCS can be modelled as:

RCS=RSRPPSTPRST×PL(2LTx-ST)

where RSRPPST is the RS received power per path that is associated with the target path, PRST is the RS transmission power, PL is the estimated path loss based on the distance between gNB and the UAV LTx-ST. The path loss of the link from gNB to the target is estimated based on the UAV position measurements. Moreover, the RCS measurement the UAV is also dependent on accuracy of identifying the peaks that are associated with the power – delay profile of the reflected RS from the target.

Observation-12: The RCS measurement relies on the target path peak identification in received sensing signal power-delay profile. Also, the RCS measurement depends on the calculation of the ToA of the target path.

For EOs, particularly EO Type 1, which are defined as having physical characteristics comparable to those of a sensing target, distinguishing between targets based on RCS can be challenging due to their similar physical attributes. In such cases, the reflected RS may carry information from both the target and surrounding EOs, resulting in a composite RCS.

Figure 1: Received power-delay profile from a target in presence of EO.

An illustrative example of the impact of reflections from an EO is provided in Figure 1, where EO denotes an environmental object and TO represents the target object. The figure depicts the RS power as a function of delay. The blue curve corresponds to the RS power-delay profile in the presence of the target object only, showing a single peak with power level P1 at a time of arrival (ToA) T1. In contrast, the red curve represents the RS power-delay profile when both the target and the environmental object are present. This profile exhibits two distinct peaks: one at power level P2 and ToA T2, and another at P3 and T3, respectively. The additional peaks in the red curve highlight the influence of environmental reflections, which can complicate the interpretation of sensing measurements.

Micro-Doppler (MD) characteristics capture the variations in the phase of the received reference signal due to the object's motion characteristics including vibrations, rotation, and movement. These variations can be unique for different objects considering the unique motion characteristics of the objects.

In the ISAC channel models introduced in TR 38.901 [4], the target velocity can be decomposed into two parts: vt=vmacrot+vmicrot, where vmacrot represents the macro motion target velocity, while vmicrot represents the micromotion velocity of the target.

The placeholder MD motion function in TR 38.901 does not specify the MD functions for different target types. For UAV propeller rotation motion, the MD motion function of the k-th rotating blade on the j-th rotor of UAV can be expressed as [5]:

φmicro,k,p(t)=Rj+ruLcosΩjt+ϕj+k2πNB j=1,⋯,NR;k=0,⋯,NB-1

where NR and NB are the total number of UAV rotors and blades, respectively. Rjis the initial location offset of j-th rotor and ϕjis the initial phase of j-th rotor. ruis the location vector of UAV. L is the distance between the centre of blade and the centre of the rotation. Ωj=2πfj and fj is the frequency of j-th blade rotation.

Observation-13: Example MD equations for UAV (e.g., the propeller rotation function provided in [5]) can be used in the placeholder MD function in TR 38.901 for Rel-20 NR ISAC-UAV performance evaluation.

Measurement quantization is a fundamental aspect for determining the performance of existing NR communications and positioning services. Since Rel-20 NR ISAC is the first study of evaluating ISAC performance, the quantization aspect for sensing measurements should be clarified.

In the context of NR ISAC-UAV performance evaluation, there are different measurements such as delay, Doppler, angle, power, range/Doppler, range/delay, RCS, MD, etc., for which quantization can be considered. The values of such measurements can be absolute or differential (taken with respect to a reference).

Observation-14: Sensing measurement quantization has a critical impact on the sensing performance.

Proposal-6: Define sensing measurement quantization based on the selected measurement types for Rel-20 NR ISAC-UAV performance evaluation. These can be based on:

- Number of quantized bits

- Range (this may include one or more sub ranges)

- Step size

- Quantization type per sub range (linear, non-linear)

Reference value (e.g., for differential measurements)

# Waveform/RS

Signal design is a fundamental aspect of an ISAC system, directly impacting the performance of sensing and communication services. The signal design factors include waveform, numerology, RS, and radio resources, to name a few. The existing signal designs can be broadly divided into sensing-centric design, communication-centric design and multiplexed signal design.

Sensing-centric designs are based on radar waveforms such as LFM and FMCW, which exhibit strong sensing characteristics such as autocorrelation, cross correlation, and passive detection performance. Communication-centric designs, on the other hand, rely on waveforms such as OFDM to maximize data rates. Note that multiplexing techniques can be used to strike trade-offs between communication and sensing performances, though this introduces additional system complexities.

The scope of the NR ISAC Performance Evaluation study [1] requires using existing NR waveforms and RS designs as baseline, and companies can share relevant information for other waveform and RS designs.

To enable sensing capabilities, existing NR waveforms can be reused. OFDM’s inherent properties, such as high time-frequency resolution and robustness to multipath, make it well-suited for extracting sensing measurements from reflected and scattered signals. Additionally, OFDM offers high flexibility in resource allocation across time, frequency, and spatial domains. This flexibility allows for dynamic configuration of sensing resources, allocating more bandwidth, time slots, or spatial beams based on sensing requirements.

Observation-15: OFDM-based NR signals can be effectively leveraged for sensing due to the inherent properties of such signals.

Proposal-7: Use NR waveforms (OFDM-based) and existing NR RS types for Rel-20 NR ISAC-UAV performance evaluation.

Existing RSs in NR may be utilized for sensing in the DL and UL, such as the CSI-RS, TRS variant, PRS, DMRS, SSB, SRS (with SRSp). These RSs have different characteristics in terms of suitability for sensing, as shown in the table below.

Table 3. Existing NR RS suitability for sensing.

| RS | Link | Usage | Characteristics | Sensing Suitability |
| --- | --- | --- | --- | --- |
| CSI-RS | DL | Channel estimation, beam tracking | - Based on QPSK Pseudo-random sequences, e.g. gold sequence - Wideband with flexible time/frequency configuration - Supports a single UE or multiple UEs - Periodic, semi-persistent, aperiodic | High |
| PRS | DL | Positioning | - Pseudo-random sequence Gold or Zadoff-Chu - Good auto-correlation properties - Can achieve dense/sparse time and frequency mapping - Suitable for high precision sensing - Lack of deployments | Very High |
| DMRS | UL/DL | Demodulation | - Can be configured to be dense, e.g. in the frequency domain - Good auto-correlation and cross-correlation properties | Low |
| SSB | DL | Initial access, Synchronisation | - Mandatory deployed - Wide coverage - Periodic - Good for coarse sensing, e.g. low requirement time resolution | Medium |
| SRS | UL | Channel sounding, beamforming | - Pseudo-random sequence - Sent periodically based on configurations - Avoids interference with frequency hopping - Periodic, semi-persistent, aperiodic - Useful for high-resolution range-delay estimation | High |

Observation-16: The existing NR RSs exhibit different characteristics for reuse towards sensing.

PRS in 5G NR presents a compelling candidate for enabling radio sensing due to their inherent configurability and alignment with existing system design. From a system perspective, PRS offers flexibility in the time domain by allowing placement in any OFDM symbol within a slot, thereby avoiding signals and channel such as PSS, SSS, and PDCCH. In the frequency domain, PRS adopts a comb-like structure with configurable offsets and must occupy contiguous resource blocks, which supports structured and scalable sensing deployments.

PRS is conditionally configured, dedicated for positioning and not always active, requiring explicit scheduling and configuration, which allows for controlled sensing overhead. In this context, PRS is optimized for high-accuracy time-based positioning methods and supports multi-cell coordination, making it suitable for distributed sensing scenarios. Its high configurability across time and frequency domains further enhances its adaptability to diverse sensing requirements.

However, practical challenges remain: PRS lacks widespread deployment, demands dedicated resources, and may introduce additional overhead if used frequently. Moreover, its design is primarily time-domain focused, offering limited support for angle-based sensing techniques. Despite these limitations, PRS stands out as a promising signal for integrating sensing into NR systems with minimal disruption to existing architecture. Also, the use of PRS involves the CN (via LMF entity) for positioning procedures, while all other RS types are managed by the gNB, which poses additional considerations.

Observation-17: PRS is the most suitable candidate out of existing RSs for enabling NR ISAC due to its inherent properties – but also comes with challenges from the perspective of involvement of CN (via LMF) for positioning and a lack of deployment in practice.

Proposal-8: Include at least results using PRS for Rel-20 NR ISAC-UAV performance evaluation.

# Evaluation Assumptions

The ISAC channel models introduced in TR 38.901 [4] in Rel-19 include validation results using certain agreed evaluation assumptions for UAV scenarios. The evaluation assumptions from the Rel-19 ISAC validation can be used as baseline; these need to be updated or augmented to facilitate NR ISAC-UAV performance evaluation in Rel-20.

Observation-18: There are existing evaluation assumptions from ISAC channel model validations in 3GPP TR 38.901.

Proposal-9: Use the evaluation assumptions from Rel-19 ISAC channel model validation as starting point, and study which parameters need to be updated or augmented for Rel-20 NR ISAC-UAV performance evaluation.

Accordingly, in Table 4, we propose parameters and evaluation assumptions for Rel-20 NR ISAC-UAV performance evaluation.

Table 4: Proposed evaluation assumptions for Rel-20 NR ISAC-UAV performance evaluation.

|  | UMa-AV | UMa |
| --- | --- | --- |
| Cell layout | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 500m) | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 500m) |
| BS antenna height | 25 m | 25 m |
| Carrier frequency | - 6 GHz (FR1) - 30 GHz (FR2) | - 6 GHz (FR1) - 30 GHz (FR2) |
| System bandwidth | - 100 MHz for FR1 - 400 MHz for FR2 | - 100 MHz for FR1 - 400 MHz for FR2 |
| BS antenna configuration | FR1: (M, N, P, Mg, Ng) = (4,4,2,1,1) FR2: (M, N, P, Mg, Ng) = (4,8,2,1,1) | FR1: (M, N, P, Mg, Ng) = (4,4,2,1,1) FR2: (M, N, P, Mg, Ng) = (4,8,2,1,1) |
| BS Tx power | FR1: 56 dBm FR2: 41 dBm | FR1: 56 dBm FR2: 41 dBm |
| BS self-interference | FFS (companies to provide values) |  |
| BS antenna radiation pattern | Table 9 in Report ITU-R M.2412 (same as 3-sector BS antenna radiation model in Table A.2.1-6 in TR 38.802) |  |
| UE antenna radiation pattern | - Omni-directional with 0 dBi element gain (FR1) - Table 11 in Report ITU-R M.2412 (same as UE antenna radiation pattern model 1 in Table A.2.1-8 in TR 38.802) (FR2) |  |
| Number of sensing targets | N, 1<=N<=x FFS x | N, 1<=N<=x FFS x |
| Sensing target distribution (horizontal plane) | N targets uniformly distributed within the centre cell | N targets uniformly distributed within the centre cell |
| EO Type 1 distribution | M EOs (modelled similar to UAV targets) uniformly distributed within the centre cell 0<=M<=N | M EOs (modelled similar to UAV targets) uniformly distributed within the centre cell 0<=M<=N |
| Minimum 3D distance between Tx/Rx and target | 10 m | 10 m |
| Height of Sensing target | 200 m | 1.5 m |
| LOS condition | LOS and NLOS (as specified in Rel-19 ISAC channel model) | LOS and NLOS (as specified in Rel-19 ISAC channel model) |
| Outdoor/Indoor | Sensing targets and BS are located outdoor | Sensing targets and BS are located outdoor |
| Velocity of sensing target | Uniform distribution between 0 km/h and 30 km/h FFS: time varying velocity between 0 km/h and 30 km/h | Uniform distribution between 0 km/h and 30 km/h FFS: time varying velocity between 0 km/h and 30 km/h |
| Minimum 3D distance between targets | 10 m | 10 m |
| Wrapping method | No wrapping method is used | No wrapping method is used |
| Performance metrics | - Horizontal target localization accuracy for 50%ile, 67%ile, 80%ile and 90%ile of the sensing targets - Vertical target localization accuracy for 50%ile, 67%ile, 80%ile and 90%ile of the sensing targets - Target detection probability - False detection probability - Velocity estimation accuracy for 50%ile, 67%ile, 80%ile and 90%ile of the sensing targets |  |

Proposal-10: Adopt the evaluation assumptions and parameters listed in Table 4 for Rel-20 NR ISAC-UAV performance evaluation.

# Preliminary NR ISAC-UAV Evaluation

In this section, we present preliminary sensing evaluation results for UAV targets and analyse how sensing accuracy performance varies for different PRS bandwidths. System-level simulations are performed with the evaluation parameters specified in Table 4. Here, we utilize a hybrid delay-angle-based sensing method. Each TRP estimates ToA and AoA corresponding to the sensing target. The AoA provides directional insight (azimuth/elevation), while the ToA determines the range (distance) to the target. By fusing both values, the network calculates the target's location relative to the BS location.

Additional evaluation assumptions are as follows:

- Scenario: UMa-AV, FR1

- Number of targets: 3

- Number of EOs: 0

- BS selection: single BS with the strongest sensing link

- ToA estimation errors are modelled as a function of PRS bandwidth

- No angle estimation errors and gNB self-interference were assumed

- Sensing mode: gNB-based monostatic with single TRP

- BW configurations: 5 MHz, 20 MHz, 100 MHz

- gNB measurements using a single TRP are used to estimate a target’s location

Evaluation results for the 3D target localization accuracy are presented in Table 5 and Figure 3.

Table 5: Sensing performance under different PRS bandwidth.

| Bandwidth | 3D target localization accuracy (meter) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50%ile | 67%ile | 80%ile | 90%ile |
| 5 MHz | 3.353 | 4.039 | 4.555 | 4.856 |
| 20 MHz | 0.974 | 1.185 | 1.349 | 1.4413 |
| 100 MHz | 0.1372 | 0.167 | 0.194 | 0.2212 |

Figure 3: CDF of 3D target localization accuracy under different PRS bandwidth.

Based on the results presented in Table 5 and Figure 3, we draw the following observation.

Observation-19: gNB-based monostatic sensing performance improves proportionally with increasing PRS bandwidth.

Based on the discussions and agreements in RAN1#122, the current simulation setup will be further refined, and updated simulation results will be presented in future RAN1 meetings.

# Conclusions

This paper made a number of observations concerning the metrics, measurements, quantization, signals, and assumptions for Rel-20 NR ISAC-UAV performance evaluation. The following proposals were made:

Proposal-1: Down-select sensing KPIs and values from sensing service categories 1 – 2 in 3GPP TS 22.137 (Table 6.2-1) to prioritize UAV detection use case for Rel-20 NR UAV-ISAC performance evaluation – UAV tracking use case can be considered with lower priority.

Proposal-2: Evaluate gNB-based monostatic (with single TRP) sensing performance using at least hybrid delay-angle-based sensing methods.

Proposal-3: Prioritize performance evaluation for gNB-based monostatic sensing using only a single-TRP in Rel-20.

Proposal-4: Reuse existing NR RS measurements for Rel-20 NR ISAC-UAV performance evaluation.

Proposal-5: Sensing-specific measurements should be used in addition to existing NR RS measurements for Rel-20 NR ISAC-UAV performance evaluation.

Proposal-6: Define sensing measurement quantization based on the selected measurement types for Rel-20 NR ISAC-UAV performance evaluation. These can be based on:

- Number of quantized bits

- Range (this may include one or more sub ranges)

- Step size

- Quantization type per sub range (linear, non-linear)

- Reference value (e.g., for differential measurements)

Proposal-7: Use NR waveforms (OFDM-based) and existing NR RS types for Rel-20 NR ISAC-UAV performance evaluation.

Proposal-8: Include at least results using PRS for Rel-20 NR ISAC-UAV performance evaluation.

Proposal-9: Use the evaluation assumptions from Rel-19 ISAC channel model validation as starting point, and study which parameters need to be updated or augmented for Rel-20 NR ISAC-UAV performance evaluation.

Proposal-10: Adopt the evaluation assumptions and parameters listed in Table 4 for Rel-20 NR ISAC-UAV performance evaluation.

# References

[1] RP-251861, “New SID: Study on Integrated Sensing And Communication (ISAC) for NR,” RAN#108, Prague, Czech Republic, June 2025.

[2] 3GPP TS 22.137, "Service Requirements for Integrated Sensing and Communication (Release 19)," V19.1.0, April 2024.

[3] 3GPP TS 38.215, ‘NR; physical layer measurements (Release 19),” V19.0.0, June 2025.

[4] 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz (Release 19)”, V19.0.0, June 2025.

[5] ETSI GR ISC-002, “Integrated Sensing And Communications (ISAC); Channel Modelling, Measurements and Evaluation Methodology” V1.1.1, July 2025.