3GPP TSG RAN WG1 #122	R1-2505674

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:		11.2

Source:				Ofinno

Title:					Discussion on Evaluation assumptions for 6GR air interface

Document for:		Discussion and Decision

# Introduction

The objective on the physical layer structure of 6G radio (6GR) is reproduced below from the SID of the study of 6G radio [1]:

The objectives of the study item are the following: (2) Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1

In this contribution we provide our views on the evaluation assumptions and methodology to evaluate 6G technology in terms of several KPIs including energy efficiency, spectral efficiency, coverage, etc.

# Background: summary of 6G scenarios

Hitherto the TSG-RAN level study on 6G scenarios and requirements SI has identified at least 10 deployment scenarios for the 6G radio. Each of these scenarios includes associated parameters/attributes including carrier frequency, aggregated system bandwidth, network layout/ISD/Orbit, BS and UE antenna elements, UE distribution/speed and service profile [2]. The TSG-RAN level SI is ongoing and therefore some of the parameters and details are still under discussion. These agreed deployment scenarios are reproduced below from the TR 38.914 [2]:

- Indoor hotspot

- Dense urban

- Rural

- Urban macro

- Sub-Urban macro

- High speed

- Extreme long distance coverage in low density area

- Urban coverage for massive connection

- Air-to-Ground Scenario

- Non-Terrestrial Network

# General principles for WG level evaluation

In this section we discuss some general principles for performing the WG level evaluation, i.e., in RAN1.

## Evaluation in typical scenarios

From time and resource perspective, it will be unrealistic to perform evaluations in all the above deployment scenarios and for all the associated attributes. Therefore, RAN1 should focus on and prioritize sub-set of scenarios which could cover most typical cases of 6G deployment. To that end we propose to focus on the following 7 deployment scenarios for the evaluation of the 6G technology:

- Indoor hotspot

- Dense urban

- Urban macro

- Sub-Urban macro

- Rural

- High speed

- Non-Terrestrial Network

The number of above listed sub-set of scenarios is still quite large. To reduce the effort, one or more closely related scenarios with overlapping attributes can be down prioritized. For example, ISD of 1732 m is covered in both sub-urban macro and rural scenarios. The sub-urban macro scenario includes an additional ISD of 1299m. The difference between ISDs of 1732 m and 1299 m is not very large. Therefore, one possibility is to down prioritize sub-urban macro or rural scenario. Alternatively, the evaluation in sub-urban macro scenario can be performed only with ISD=1299 m.

Proposal 1: RAN1 evaluations are performed in sub-set of scenarios which cover most typical cases of 6G deployment.

## Link vs system simulations

In principle, both link level simulation (LLS) and system level simulation (SLS) should be carried out in each scenario to evaluate the proposals. However, it may not be necessary to perform both link level and system level simulations for all the scenarios and for all types of KPIs. Therefore, RAN1 should decide on case-by-case basis whether to carry out link simulation and/or system simulation for a particular deployment scenario and for evaluation of a particular performance target. Secondly, the evaluation should be carried out for selected set of parameters in each deployment scenario. To further limit the effort, the evaluation should be performed in limited frequencies, which is/are most relevant for a particular deployment scenario.

Proposal 2: Decide on case-by-case basis whether to perform link simulation and/or system simulation for a particular deployment scenario used for a particular performance target.

## BS/UE antenna elements at 7 GHz

The 5G mid-band (i.e., around 3.5 GHz) and 7GHz (e.g., 7.125-8.4 GHz) are expected to be co-sited Therefore, according to the SID, the 6G coverage in 7 GHz should be comparable to 5G coverage in 3.5 GHz [1]:

f)	Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

However, the path loss at 7 GHz is typically 6 dB larger compared to the path loss at 3.5 GHz (i.e., 20*LOG10(7/3.5)). To compensate for the path loss, larger number of antenna elements will be employed in 6G at 7 GHz compared to the 5G mid-band. The wavelength at 7 GHz is twice as small compared to that at 3.5 GHz. This decreases the size of antenna by a factor of 2 and creates twice as much space between the antenna elements compared to 3.5 GHz. This in turn will allow enough space in the panel of the 6G BS to accommodate at least 4 times more antenna elements compared to the 5G BS at 3.5 GHz. Therefore, the evaluation assumptions should consider 4 times more antenna elements at 7 GHz compared to 3.5 GHz.

Proposal 3: Evaluation in 7GHz assumes at least 4 times more antenna elements compared to 4 GHz.

## Channel modelling at 7 GHz

As described in section 3.3, the 7GHz coverage loss will be compensated by increasing the number of antenna elements. The antenna diameter/aperture size may also be increased to further accommodate more antenna elements. For example, at 7 GHz and assuming antenna panel diameter (D) of 2 meters, the minimum far field distance (R) will be around 200 m (i.e., R > 2*D2/).

Therefore, there may be considerable fraction of users in the near-field especially in indoor and urban environments (e.g., with ISD up to 500m).

Therefore, the evaluation should consider also near-field channel model for the users in the near field zone at 7 GHz. The near-field channel model defined in TR 38.901 [3] can be used for the evaluation.

Observation 1: Large number of users are expected in near-field in 7GHz deployment scenarios.

Proposal 4: Evaluation in 7GHz considers also near-field channel model for the users in the near-field zone in 7 GHz.

# Proposed 6G scenarios for WG level evaluation

In the following sections we provide a list of attributes/parameters associated with each of the above-mentioned deployment scenarios for evaluation in RAN1. Most of the parameters are based on TR 38.814 [2].

## Indoor hotspot

In TR 38.914 [2], the carrier frequencies include 4, 7, 30 GHz, and combination of 4GHz and 7 GHz for indoor hotspot scenarios. All these frequencies can be considered for RAN1 level studies as they are very relevant for indoor environment. However, in our view if needed 30 GHz can be down prioritized in RAN1 studies.

Table 1: Attributes for indoor hotspot

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4 GHz Around 7 GHz Around 4GHz+Around 7GHz Around 30 GHz |
| Aggregated system bandwidth | Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: 1 GHz (DL+UL) |
| Layout | Single layer: - Indoor floor (Open office) |
| ISD | 20 m for around 30GHz TBD on other carrier frequencies (Equivalent to [12]TRxPs per 120m x 50m) |
| BS antenna elements | 7GHz: 1024 Rx and Tx antenna elements 4 GHz: 256 Rx and Tx antenna elements 30 GHz: 256 Rx and Tx antenna elements |
| UE antenna elements | 7GHz: 32 Rx and Tx antenna elements 4 GHz: 8 Rx and Tx antenna elements 30 GHz: 32 Rx and Tx antenna elements |
| User distribution and UE speed | 100% Indoor, 3km/h, 10 users per TRxP |
| Service profile | Non-full-buffer traffic |

## Dense urban

In TR 38.914 [2], the carrier frequencies include 2 GHz, 4 GHz, 7 GHz, combination of 4 GHz and 7 GHz, combination of 4 GHz and 30 GHz for dense urban scenarios. We suggest all these frequencies except combination of FR1 (4 GHz) and FR2-1 (30 GHz) are considered for RAN1 level studies.

Table 2: Attributes for dense urban

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 2 GHz Around 4 GHz Around 7 GHz Around 30 GHz Around 4 GHz + Around 7 GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| Aggregated system bandwidth | Around 2 GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400] MHz (DL+UL) Around 30 GHz: [1] GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |
| ISD | Macro: 200m Micro: 3 micro TRxPs per macro TRxP Micro: [100] m All micro TRxPs are all outdoor |
| BS antenna elements | 2 GHz: 64 Rx and Tx antenna elements 4 GHz: 256 Rx and Tx antenna elements 7GHz: 1024 Rx and Tx antenna elements 30 GHz: 256 Rx and Tx antenna elements |
| UE antenna elements | 2 GHz: 4 Rx and Tx antenna elements 4 GHz: 8 Rx and Tx antenna elements 7GHz: 32 Rx and Tx antenna elements 30 GHz: 32 Rx and Tx antenna elements |
| User distribution and UE speed | Step1: Uniform/macro TRxP, 10 users per TRxP Step2: Uniform/macro TRxP + Clustered/micro TRxP, 10 users per TRxP [10] users per TRxP with single-layer only 80% indoor (3km/h), 20% outdoor (30km/h) |
| Service profile | Non-full-buffer traffic |

## Urban macro

In TR 38.914 [2], the carrier frequencies include 2, 4, 7 GHz, combination of 4 GHz and 7 GHz, combination of 4 GHz and 30 GHz for urban macro scenarios. Frequencies 2, 4, and 7 GHz, are highly relevant for urban macro environment. We therefore suggest that RAN1 level studies in urban macro environment include all frequencies except combination of 4 GHz and 30 GHz frequencies.

Table 3: Attributes for urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 2GHz Around 4 GHz Around 7 GHz Around 4 GHz + Around 7 GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| Aggregated system bandwidth | Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400] MHz (DL+UL) Around 30 GHz: [1 ]GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |
| ISD | Macro: 500m [TBD on micro layout] |
| BS antenna elements | 2 GHz: 64 Rx and Tx antenna elements 4 GHz: 256 Rx and Tx antenna elements 7GHz: 1024 Rx and Tx antenna elements |
| UE antenna elements | 2 GHz: 4 Rx and Tx antenna elements 4 GHz: 8 Rx and Tx antenna elements 7GHz: 32 Rx and Tx antenna elements |
| User distribution and UE speed | TBD [10] users per TRxP |
| Service profile | Non-full-buffer traffic |

## Sub-urban macro

In TR 38.914 [2], the carrier frequencies include 700 MHz, 2 GHz, 4 GHz, 7 GHz, combination of 4 GHz and 7 GHz, and combination of 2 GHz and 700 GHz for sub-urban macro scenarios. These frequencies are relevant in sub-urban macro scenarios for both ISDs, i.e., 1299m and 1732m. However, to limit the workload, we suggest including all the above frequencies except combination of 2 GHz and 700 GHz for RAN1 level studies in sub-urban macro environment.

Table 4: Attributes for sub-urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz Around 2 GHz Around 4 GHz Around 7GHz Around 7GHz +Around 4GHz Around 2GHz+ Around 700 MHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| Aggregated system bandwidth | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7GHz: Up to [400] MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 1299m ISD 2: 1732m |
| BS antenna elements | 700 GHz: 32 Rx and Tx antenna elements 2 GHz: 64 Rx and Tx antenna elements 4 GHz: 256 Rx and Tx antenna elements 7GHz: 1024 Rx and Tx antenna elements |
| UE antenna elements | 700 GHz: 2 Rx and Tx antenna elements 2 GHz: 4 Rx and Tx antenna elements 4 GHz: 8 Rx and Tx antenna elements 7GHz: 32 Rx and Tx antenna elements |
| User distribution and UE speed | [15% outdoor vehicles (120km/h), 15% outdoor (3 km/h) and 70% indoor (3 km/h), or 50% outdoor vehicles (120km/h), 50% indoor (3 km/h)] [10] users per TRxP |
| Service profile | Non-full-buffer traffic |

## Rural

In TR 38.914 [2], the carrier frequencies include 700 MHz, 4 GHz, and 7 GHz for rural scenario. The TR 38.914 also includes 3 IDs = 1732m, 5000m, and 7500m. To reduce the workload, only ISDs= 1732m and 5000m can be considered. While all these frequencies are relevant in rural scenarios but not all ISDs are needed for all the frequencies. We suggest that RAN1 level studies include 700 MHz using ISD=5000m, and 4 GHz, and 7 GHz frequencies using ISD=1732 m as they are most relevant for rural environment.

Table 5: Attributes for rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7GHz (ISD 1) Around 700MHz + Around 7GHz (for ISD 1) |
| Aggregated system bandwidth | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7GHz: Up to [400] MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 1732m ISD 2: 5000m [ISD 3: 7500m assuming 700MHz] |
| BS antenna elements | 700 GHz: 32 Rx and Tx antenna elements 2 GHz: 64 Rx and Tx antenna elements 4 GHz: 256 Rx and Tx antenna elements 7GHz: 1024 Rx and Tx antenna elements |
| UE antenna elements | 700 GHz: 2 Rx and Tx antenna elements 2 GHz: 4 Rx and Tx antenna elements 4 GHz: 8 Rx and Tx antenna elements 7GHz: 32 Rx and Tx antenna elements |
| User distribution and UE speed | [15% outdoor vehicles (120km/h), 15% outdoor (3 km/h) and 70% indoor (3 km/h), or 50% outdoor vehicles (120km/h), 50% indoor (3 km/h)] [10] users per TRxP |
| Service profile | Non-full-buffer traffic |

## High speed

Currently the TR 38.914 [2] does not include any parameter for high-speed scenario. The most relevant high-speed scenario is the high-speed train scenario. In typical NR high speed trains scenario, the users inside the train are served by wide area base stations deployed along the rail track for speed up to 500 km/hour. However, the high-speed scenarios for 6G are still being discussed at the RAN level studies. We therefore suggest that RAN1 level evaluation assumptions for high-speed scenario are discussed after the corresponding agreement in the RAN level SI on 6G.

Proposal 5: Discuss RAN1 level evaluation assumptions for high-speed scenario after the corresponding agreement in the RAN level SI on 6G.

## Non-Terrestrial Network

Currently the TR 38.914 [2] does not include any parameter also for non-terrestrial network (NTN) scenario. The relevant bands for NR-NTN are around 2 GHz (S band), 12/14 GHz (Ku bands), and around 20/30 GHz (Ka bands). However, for 6G the discussions are still ongoing in RAN level studies. However, the NTN scenarios for 6G are still being discussed at the RAN level studies. We therefore suggest that RAN1 level evaluation assumptions for the NTN are discussed after the corresponding agreement in the RAN level SI on 6G.

Proposal 6: Discuss RAN1 level evaluation assumptions for NTN scenario after the corresponding agreement in the RAN level SI on 6G.

# Conclusion

This contribution has discussed evaluation assumptions and deployment scenarios for evaluating proposals on 6G technology. Based on this discussion our observations and proposals are as follows:

Proposal 1: RAN1 evaluations are performed in sub-set of scenarios which cover most typical cases of 6G deployment, e.g.,

- Indoor hotspot

- Dense urban

- Urban macro

- Sub-Urban macro

- Rural

- High speed

- Non-Terrestrial Network

Proposal 2: Decide on case-by-case basis whether to perform link simulation and/or system simulation for a particular deployment scenario used for a particular performance target.

Proposal 3: Evaluation in 7GHz assumes at least 4 times more antenna elements compared to 4 GHz.

Observation 1: Large number of users are expected in near-field in 7GHz deployment scenarios.

Proposal 4: Evaluation in 7GHz considers also near-field channel model for the users in the near-field zone in 7 GHz.

Proposal 5: Discuss RAN1 level evaluation assumptions for high-speed scenario after the corresponding agreement in the RAN level SI on 6G.

Proposal 6: Discuss RAN1 level evaluation assumptions for NTN scenario after the corresponding agreement in the RAN level SI on 6G.

# References

- RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator).

- TR 38.914, “Study on 6G Scenarios and requirements”, V0.1.0.

- TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”.

- TR 38.811, “Study on New Radio (NR) to support non-terrestrial networks”.

- TR 38.864, “Study on network energy savings for NR”.