3GPP TSG RAN WG1 #122			R1-2505892

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.12.2

Source:	Moderator (Apple)

Title:	FL summary #2 of Low band carrier aggregation via switching

Document for:	Discussion/Decision

# Introduction

The following lists the objectives in Rel-19 WI on Low NR band carrier aggregation via switching (RP-251315) with the highlight of RAN1 related part

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

This contribution summarizes the discussions on maintenance of low NR band carrier aggregation via switching in RAN1#122:

- Section 2 provides contact information for the participating companies

- Section 3 provides the proposals for online sessions.

- Section 4 provides documentation of the open discussions.

- Section 5 provides documentation of the closed discussions.

- Section 6 provides summary of contributions to RAN1#120bis

- Appendix A provides the agreements from the previous and current meetings

- Appendix B provides the existing LS from other WGs

# Contact Information

| Company | Contact information (Name, Email) |
| --- | --- |
| Moderator (Apple) | Haitong Sun (haitong_sun@apple.com) |
| MediaTek | Chiou-Wei Tsai (cw.tsai@mediatek.com) |
| Apple | Ankit Bhamri (a.bhamri@apple.com) |
| Ofinno | Muhammad Kazmi (mkazmi@ofinno.com) |
| Qualcomm | Fred Takeda (ktakeda@qti.qualcomm.com) |
| Samsung | Sa Zhang (sa.zhang@samsung.com) |
| NTT DOCOMO, INC. | Hiroki Harada (hiroki.harada.sv@nttdocomo.com) |
| Spreadtrum | Huan Zhou (Huan.Zhou@unisoc.com ) |
| Ericsson | Stephen Grant (stephen.grant@ericsson.com ) |
| CATT | Min Zhu (zhumin@catt.cn ) |
| Nokia | Karri Ranta-aho (karri.ranta-aho@nokia.com) |
| LG Electronics | Sukchel Yang (suckchel.yang@lge.com) |
| ZTE | Fei Wang (wang.fei11@zte.com.cn) |
| vivo | Na Li (lina5g@vivo.com) |
| Google | Abdellatif Salah (asalah@google.com) |
| OPPO | Wenfeng Zhang (zhangwenfeng@oppo.com) Jing Xu (xj@oppo.com) |
| Xiaomi | Haipeng Lei (leihaipeng@xiaomi.com) |

# Proposals for Online/Offline Sessions

## Proposals for Monday Offline

Proposal 4.1 (Offline consensus)

For Rel-19 low NR band carrier aggregation via switching, UE handles the overlapping of UL transmission with the RRC configured semi-static switching pattern, after the handling of intra-UE multiplexing/prioritization

- Adopt the following TP to Rel-19 TS38.213

Reason for change: The handling order of UCI multiplexing and overlapping of UL transmission with the RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, is not defined. Summary of change: Specify that UE handles the overlapping of UL transmission with the RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, after UCI intra-UE multiplexing/prioritization. Consequences if not approved: UE behavior regarding the order of handling of UCI multiplexing and overlapping of UL transmission with the RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, is not clear.

9 UE procedure for reporting control information ## *** Unchanged parts are omitted *** When a UE determines overlapping for PUCCH and/or PUSCH transmissions of the same priority index other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission due to cell DRX operation [11, TS 38.321] or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24 including repetitions if any, - first, the UE resolves the overlapping for PUCCHs with repetitions as described in clause 9.2.6, if any - second, the UE resolves the overlapping for PUCCHs without repetitions as described in clauses 9.2.5 - third, the UE resolves the overlapping for PUSCHs and PUCCHs with repetitions as described in clause 9.2.6 - fourth, the UE resolves the overlapping for PUSCHs and PUCCHs without repetitions as is subsequently described in this clause. ## *** Unchanged parts are omitted *** When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, other than PUCCH transmissions with SL HARQ-ACK reports, before considering limitations for transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24 including repetitions if any, if the UE is provided uci-MuxWithDiffPrio and the timeline conditions in clause 9.2.5 for multiplexing UCI in a PUCCH or a PUSCH are satisfied - first, the UE resolves overlapping for PUCCH and/or PUSCH transmissions of a same priority index as described in clauses 9.2.5 and 9.2.6 - second, the UE resolves the overlapping for PUCCH transmissions of different priority indexes, and ## *** Unchanged parts are omitted *** the UE - transmits the PUCCH or the PUSCH of the larger priority index subject to the limitations for UE transmissions due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 24 and - does not transmit a PUCCH or a PUSCH of smaller priority index When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, other than PUCCH transmissions with SL HARQ-ACK reports, before considering limitations for transmissions including with repetitions, if any, due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24, if the UE is not provided uci-MuxWithDiffPrio, the UE first resolves overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in clauses 9.2.5 and 9.2.6 - any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24 ## *** Unchanged parts are omitted *** In the remaining of this clause, a UE multiplexes UCIs with same priority index in a PUCCH or a PUSCH before considering limitations for UE transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24. A PUCCH or a PUSCH is assumed to have a same priority index as a priority index of UCIs a UE multiplexes in the PUCCH or the PUSCH. ## *** Unchanged parts are omitted ***

Proposal 4.2 (Offline consensus)

For Rel-19 low NR band carrier aggregation via switching, UE handles the overlapping of SPS PDSCHs with the RRC configured semi-static switching pattern, before the handling of overlapping among SPS PDSCHs

- Adopt the following TP to Rel-19 TS38.214

Reason for change: The handling order of overlapping among SPS PDSCHs within a slot and overlapping between SPS PDSCH and RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, is not defined. Summary of change: Specify that UE handles the overlapping between SPS PDSCH and RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, before the handling of overlapping among SPS PDSCHs within a slot. Consequences if not approved: UE behavior regarding the order of handling overlapping among SPS PDSCHs within a slot and overlapping between SPS PDSCH and RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, is not clear.

5.1 UE procedure for receiving the physical downlink shared channel <Unchanged parts are omitted> If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, or determined as non-active periods of cell DTX, if the serving cell is activated with cell DTX, based on [10, TS 38.321], or not available for SPS PDSCH receptions as described in clause 24 of [6, TS 38.213], a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below. ‒ Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot ‒ Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH. ‒ Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q. ‒ Step 3: Repeat step 1 and 2 until Q is empty or j is equal to the number of unicast/multicast PDSCHs in a slot supported by the UE. <Unchanged parts are omitted>

Proposed Conclusion 4.3

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support the following restrictions for the semi-static switching pattern

- at least one SSB is available every 160ms in PCell

- at least one SSB is available every 160ms in SCell

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, consider the following restrictions for the semi-static switching pattern

- Option 1: maximum 8 switches within 40 slots

  - Support: Spreadtrum, Samsung, Qualcomm, DOCOMO

  - Not support: Ericsson, Nokia, LG

- Option 2: maximum 1 switch(es) within 4 slots

  - Support: Spreadtrum, Qualcomm

  - Not support: Ericsson, Nokia, LG

- Option 3: at least one SSB is available every 160ms in PCell

  - Support: Spreadtrum, DOCOMO, Xiaomi

  - Not support: Ericsson

- Option 4: at least one SSB is available every 160ms in SCell

  - Support: Spreadtrum, DOCOMO, Xiaomi

  - Not support: Ericsson

- Option 5: UE does not stay continuously on SDL carrier for longer than 15 slots

  - Support: vivo, DOCOMO

  - Not support: Ericsson, Nokia, LG

- Option 6: maximum 1 switch(es) within 5 slots

  - Support: vivo

  - Not support: Ericsson, Nokia, LG

- Option 7: maximum 2 switches within 10 slots

  - Support: Xiaomi, OPPO, DOCOMO

  - Not support: Ericsson, Nokia, LG

- Option 8: the maximum of continuous slots on SCell is 7

  - Support: Samsung, DOCOMO

  - Not support: Ericsson, Nokia, LG

- Option 9: the UE does not expect the set of symbols of a slot for reception of SSB on one cell to be indicated as switching gap or another cell

  - Support: ZTE, DOCOMO

  - Not support: Ericsson, LG

- Option 10: at least one PRACH occasion is available every 160ms on PCell

  - Support: DOCOMO

  - Not support: Ericsson, Nokia, LG

Proposal 4.4

- For Rel-19 low NR band carrier aggregation via switching, in terms of introducing UE processing timeline relaxation, i.e., TLB, switch,  consider the following cases for the specification of the UE processing timeline relaxation

  - Case 1: PDSCH reception on SDL carrier, and corresponding PUCCH for HARQ-ACK transmission on FDD carrier

    - Support: Spreadtrum, LG, MediaTek, Google, Samsung

    - Not support: Ericsson, Nokia

  - Case 2: PDCCH  reception on SDL carrier schedules PUSCH transmission on FDD carrier.

    - Support: MediaTek, Samsung

    - Not support: Ericsson, Nokia

  - Case 3: PDCCH  reception on SDL carrier schedules aperiodic SRS transmission on FDD carrier.

    - Support: MediaTek, Samsung

    - Not support: Ericsson, Nokia

  - Case 4: a RAR schedules a PUSCH transmission

    - Support: Samsung

    - Not support:  Ericsson, Nokia

- If UE processing timeline relaxation is introduced, the UE processing timeline relaxation, i.e., TLB, switch is the same as the switching period reported by the UE as defined in RAN4, i.e., among {35us, 70us, 140us}

  - Support: Spreadtrum, MediaTek

  - Not support: Ericsson, Nokia

- If UE processing timeline relaxation is introduced, consider the restriction that a UE does not expect to have more than one SDL->FDD switch or more than one FDD->SDL switch between the received downlink PDCCH/PDSCH and the corresponding uplink transmission (PUCCH/PUSCH/SRS) or DL reception (PDCCH, PDSCH, SSB, CSI-RS)

  - Support: MediaTek, Google

  - Not support: Samsung, Nokia

Proposal 4.5

For Rel-19 low NR band carrier aggregation via switching, when NW configures the semi-static switching pattern to a UE, consider the following for HARQ-ACK enhancement

- Enhancement 1: For Type-I HARQ-ACK codebook generation, a row of the TDRA table overlapping with the invalid symbol(s) is excluded from the determination of the set of occasions for candidate PDSCH receptions.

  - Support:  vivo, Xiaomi, Samsung, LG

  - Not support: Qualcomm, DOCOMO, Ericsson, Apple

- Enhancement 2: SPS PDSCH overlapping with the invalid symbol(s) is excluded from the generation of HARQ-ACK information.

  - Support:  vivo, Samsung, ZTE, LG

  - Not support: Qualcomm, DOCOMO, Ericsson, Apple

- Enhancement 3: DAI signaling via DL DCI for the Type-2 HARQ-ACK codebook considering that only one carrier can be scheduled at a time.

  - Support: LG

  - Not support: Qualcomm, DOCOMO, Ericsson, Apple

- Enhancement 4: For HARQ-ACK feedback timing, support more than 8 values in the K1 set with new values larger than 15.

  - Support: Xiaomi

  - Not support: Qualcomm, DOCOMO, Ericsson, Apple

- Note: Invalid symbol(s) include

  - The symbol(s) overlaps with the configured switching gap

  - The symbol(s) during the semi-static configured PCell (FDD carrier) duration for SCell (SDL carrier)

  - The symbol(s) during the semi-static configured SCell (SDL carrier) duration for PCell (FDD carrier)

Proposal 4.6

For Rel-19 low NR band carrier aggregation via switching, when NW configures a semi-static switching pattern to a UE, considering the following RACH enhancement

- Enhancement 1: if a RACH procedure is triggered by a UE itself or by network signalling, the semi-static switching pattern, if configured, is temporarily suspended until the last DL reception or UL transmission involved in the RACH procedure is successfully completed.

  - Support: Huawei

  - Not support: ZTE, Samsung, DOCOMO, Ericsson, Apple, vivo

- Enhancement 2: consider the following enhancement

  - The UE does not apply the semi-static switching pattern and stays on the PCell (FDD carrier) at least during on-going random-access procedure on the PCell.

  - For a RACH procedure triggered by a PDCCH order, the UE does not apply the low band CA switching pattern starting from a slot in which the UE has received the PDCCH order for the preamble transmission and until the UE transmits HARQ ACK on the PUCCH.

  - In both CFRA and CBRA, the UE starts to re-apply the low band CA switching pattern after a slot in which the UE transmits HARQ-ACK on the PUCCH (i.e., Msg4 in CFRA and Msg5 in CBRA). if a RACH procedure is triggered by a UE itself or by network signalling, the semi-static switching pattern, if configured, is temporarily suspended until the last DL reception or UL transmission involved in the RACH procedure is successfully completed.

  - Support: Ofinno

  - Not support: ZTE, Samsung, DOCOMO, Ericsson, Apple, vivo

- Enhancement 3: For a RACH procedure on FDD carrier,

  - Once a UE transmits PRACH, the UE stays on FDD carrier, i.e., overrides switch pattern configuration, before the RACH procedure completes.

  - After RACH procedure completes, the UE switches to SDL carrier at the next configured FDD-to-SDL switching gap.

where the time of the RACH procedure completion can refer to a given time delay (e.g. 3ms) after Msg-2 for CFRA procedure and Msg-4 for CBRA procedure

  - Support: OPPO

  - Not support: ZTE, Samsung, DOCOMO, Ericsson, Apple, vivo

Proposal 4.6’

Qualcomm proposal: TP to 38.213 Section 24.

A UE that can switch operation between a PCell, that includes a DL carrier and a paired UL carrier, and an SCell, that includes a DL carrier without a paired UL carrier, can be provided by LBCA-SwitchingPattern a bitmap of slots indicating a switching pattern between the PCell and the SCell when the SCell is activated, except during random access procedure after a PRACH transmission according to [38.321]. The switching pattern repeats continuously. The UE can either transmit/receive on the PCell in a slot or receive on the SCell in the slot.

Proposal 4.7

For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling, companies also proposed several other maintenance issue

- Issue 1: CSI-RS and the corresponding CSI dropping

  - Support: vivo (R1-2505393), Ofinno (R1-2505666), ZTE, Qualcomm

  - Not support: Samsung, DOCOMO, Ericsson, Nokia vivo, LG

Proposal 5: For periodic CSI-RS and semi-persistent CSI-RS on a serving cell configured in CSI report configuration in CSI-ReportConfig, - The most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell occurs in valid time of the serving cell - The UE reports a CSI report only if receiving at least one CSI-RS transmission occasion of each periodic CSI-RS resource or semi-persistent CSI-RS resource on a serving cell in valid time of the serving cell, no later than CSI reference resource, and the UE drops the CSI report otherwise

Proposal 5: Clarify that a slot in a serving cell shall be considered to be a valid downlink slot if it does not overlap with any slot assigned to the other serving cell when configured with LBCA-SwitchingPattern. Proposal 6: Include the following text proposal in clause 5.2.2.5 of TS 38.214: A slot in a serving cell shall be considered to be a valid downlink slot if: - it comprises at least one higher layer configured downlink or flexible symbol, - it does not fall within a configured measurement gap for that UE, and - it does not overlap with any slot assigned to the other serving cell when configured with LBCA-SwitchingPattern [12, 38.331].

- Issue 2: Multi-slot PDSCH reception is omitted per PDSCH if overlapping with the unavailable duration of switching gap and another cell. TP proposed to clause 5.1 and 5.1.2.1 in TS38.214.

  - Support: ZTE (R1-2506127), with Proposal 3 and TP,

  - Not support: Qualcomm, Samsung, DOCOMO, Ericsson, Apple, Nokia, vivo, LG

- Issue 3: PUSCH repetition type A and/or TBoMS reception is omitted per slot if overlapping with the unavailable duration of switching gap and another cell. TP proposed to clause 6.1.2.1 in TS38.214

  - Support: ZTE (R1-2506127), with Proposal 4 and TP

  - Not support: Qualcomm, Samsung, DOCOMO, Ericsson, Apple, Nokia, vivo, LG

- Issue 4: For PUSCH repetition type B, invalid symbols shall comprise the symbols overlapped with the unavailable duration of switching gap and another cell.  TP proposed to clause 6.1.2.1 in TS38.214

  - Support: ZTE (R1-2506127), with Proposal 5 and TP

  - Not support: Qualcomm, Samsung, DOCOMO, Ericsson, Apple, Nokia, vivo, LG

- Issue 5: Multi-PUSCH transmission is omitted per PUSCH if overlapping with the unavailable duration of switching gap and another cell. TP proposed to clause 6.1 and 6.1.2.1 in TS38.214

  - Support:  ZTE (R1-2506127), with Proposal 6 and TP

  - Not support: Qualcomm, Samsung, DOCOMO, Ericsson, Apple, Nokia, vivo, LG

## Proposals for Tuesday Online

Proposal 4.1 (Offline consensus)

For Rel-19 low NR band carrier aggregation via switching, UE handles the overlapping of UL transmission with the RRC configured semi-static switching pattern, after the handling of intra-UE multiplexing/prioritization

- Adopt the following TP to Rel-19 TS38.213

Reason for change: The handling order of UCI multiplexing and overlapping of UL transmission with the RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, is not defined. Summary of change: Specify that UE handles the overlapping of UL transmission with the RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, after intra-UE multiplexing/prioritization. Consequences if not approved: UE behavior regarding the order of handling of UCI multiplexing and overlapping of UL transmission with the RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, is not clear.

9 UE procedure for reporting control information ## *** Unchanged parts are omitted *** When a UE determines overlapping for PUCCH and/or PUSCH transmissions of the same priority index other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission due to cell DRX operation [11, TS 38.321] or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24 including repetitions if any, - first, the UE resolves the overlapping for PUCCHs with repetitions as described in clause 9.2.6, if any - second, the UE resolves the overlapping for PUCCHs without repetitions as described in clauses 9.2.5 - third, the UE resolves the overlapping for PUSCHs and PUCCHs with repetitions as described in clause 9.2.6 - fourth, the UE resolves the overlapping for PUSCHs and PUCCHs without repetitions as is subsequently described in this clause. ## *** Unchanged parts are omitted *** When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, other than PUCCH transmissions with SL HARQ-ACK reports, before considering limitations for transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24 including repetitions if any, if the UE is provided uci-MuxWithDiffPrio and the timeline conditions in clause 9.2.5 for multiplexing UCI in a PUCCH or a PUSCH are satisfied - first, the UE resolves overlapping for PUCCH and/or PUSCH transmissions of a same priority index as described in clauses 9.2.5 and 9.2.6 - second, the UE resolves the overlapping for PUCCH transmissions of different priority indexes, and ## *** Unchanged parts are omitted *** the UE - transmits the PUCCH or the PUSCH of the larger priority index subject to the limitations for UE transmissions due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 24 and - does not transmit a PUCCH or a PUSCH of smaller priority index When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, other than PUCCH transmissions with SL HARQ-ACK reports, before considering limitations for transmissions including with repetitions, if any, due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24, if the UE is not provided uci-MuxWithDiffPrio, the UE first resolves overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in clauses 9.2.5 and 9.2.6 - any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24 ## *** Unchanged parts are omitted *** In the remaining of this clause, a UE multiplexes UCIs with same priority index in a PUCCH or a PUSCH before considering limitations for UE transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24. A PUCCH or a PUSCH is assumed to have a same priority index as a priority index of UCIs a UE multiplexes in the PUCCH or the PUSCH. ## *** Unchanged parts are omitted ***

Proposal 4.2 (Offline consensus)

For Rel-19 low NR band carrier aggregation via switching, UE handles the overlapping of SPS PDSCHs with the RRC configured semi-static switching pattern, before the handling of overlapping among SPS PDSCHs

- Adopt the following TP to Rel-19 TS38.214

Reason for change: The handling order of overlapping among SPS PDSCHs within a slot and overlapping between SPS PDSCH and RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, is not defined. Summary of change: Specify that UE handles the overlapping between SPS PDSCH and RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, before the handling of overlapping among SPS PDSCHs within a slot. Consequences if not approved: UE behavior regarding the order of handling overlapping among SPS PDSCHs within a slot and overlapping between SPS PDSCH and RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, is not clear.

5.1 UE procedure for receiving the physical downlink shared channel <Unchanged parts are omitted> If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, or determined as non-active periods of cell DTX, if the serving cell is activated with cell DTX, based on [10, TS 38.321], or not available for SPS PDSCH receptions as described in clause 24 of [6, TS 38.213], a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below. ‒ Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot ‒ Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH. ‒ Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q. ‒ Step 3: Repeat step 1 and 2 until Q is empty or j is equal to the number of unicast/multicast PDSCHs in a slot supported by the UE. <Unchanged parts are omitted>

## Proposals for Wednesday Offline

Proposal 4.7

For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling of SSB, a UE does not receive the SSB that collides with the LB CA switching pattern

- Note: a SSB collides with the LB CA switching pattern if

  - For SSB in FDD carrier, one or multiple SSB symbols overlaps with symbols configured as either SDL carrier or switching gap

  - For SSB in SDL carrier, one or multiple SSB symbols overlaps with symbols configured as either FDD carrier or switching gap

Support: Ericsson, MTK, Samsung, Nokia, Google, LG, vivo

Proposal 4.7

For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling of SSB, a UE does not expect a SSB that collides with the LB CA switching pattern

- Note: a SSB collides with the LB CA switching pattern if

  - For SSB in FDD carrier, one or multiple SSB symbols overlaps with symbols configured as either SDL carrier or switching gap

  - For SSB in SDL carrier, one or multiple SSB symbols overlaps with symbols configured as either FDD carrier or switching gap

Support: ZTE

Strong concern: Samsung, Ericsson, Nokia

Proposed Conclusion 4.3

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support the following restrictions for the semi-static switching pattern

- It is RAN1 understanding that RAN4 requirements are applicable when at least one SSB is available every 160ms in PCell

- at least one SSB is available every 160ms in SCell

## Proposals for Wednesday Online

Proposal 4.7 Alt1

For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling of SSB, a UE does not receive the SSB that collides with the LB CA switching pattern

- Note: a SSB collides with the LB CA switching pattern if

  - For SSB in FDD carrier, one or multiple SSB symbols overlaps with symbols configured as either SDL carrier or switching gap

  - For SSB in SDL carrier, one or multiple SSB symbols overlaps with symbols configured as either FDD carrier or switching gap

Support: Ericsson, MTK, Samsung, Nokia, Google, LG, vivo

Proposal 4.7 Alt2

For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling of SSB, a UE does not expect a SSB that collides with the LB CA switching pattern

- Note: a SSB collides with the LB CA switching pattern if

  - For SSB in FDD carrier, one or multiple SSB symbols overlaps with symbols configured as either SDL carrier or switching gap

  - For SSB in SDL carrier, one or multiple SSB symbols overlaps with symbols configured as either FDD carrier or switching gap

Support: ZTE

Strong concern: Samsung, Ericsson, Nokia

Proposed Conclusion 4.3

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support the following restrictions for the semi-static switching pattern

- It is RAN1 understanding that RAN4 requirements are applicable when at least one SSB in PCell is available every 160ms

- at least one SSB in SCell is available every 160ms

# (Open Discussion) Low band carrier aggregation via switching

## [Closed] UCI multiplexing order

## [Closed] SPS PDSCH

## Switching pattern restriction

[Closed][R2] Proposed Conclusion 4.3

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support the following restrictions for the semi-static switching pattern

- at least one SSB is available every 160ms in PCell

- at least one SSB is available every 160ms in SCell

| Company | Comments |
| --- | --- |
| Moderator | Please share your view on the proposed conclusion and whether RAN1 specification change is needed |
| ZTE | Support as a conclusion and no RAN1 spec impact. |
| MediaTek | Similar view with ZTE, we can live with a conclusion without RAN1 spec impact. From our point of view, this conclusion can provide our product team what to expect when implementing this feature. |
| Nokia | Support, except if the requirements have been defined in RAN 4. |
| DOCOMO | Support the proposed conclusion |
| LGE | Same view as ZTE. Support the proposed conclusion from FL without RAN1 spec impact. |
| vivo | Support. We think the above restriction should be reflected in spec, maybe in RAN2, or RAN1, or the spec. We think the conclusion (if possible, can be an agreement) can included in the description of the parameter of switching pattern. |
| Ofinno | Support the proposal with the understanding that this will not impact RAN4 spec. However, most likely this will (as a condition) be captured in RAN4 requirements. |
| Ericsson | The feedback I have received from RAN4 is the following: - First bullet - The current timing requirements in 38.133 are applicable when at least one SSB is available once every 160ms. Hence the restriction in the first bullet is covered by RAN4 requirements already. # 2nd bullet: - If RAN1 agrees to this bullet, it will preclude operating the SCell as an SSB-less SCell (NES functionality introduced in Rel-18) # In summary, we don’t think Conclusion 4.3 is needed. If companies insist on concluding something, we can be open to delete the 2nd bullet and then modify the 1st bullet as “RAN4 requirements are available when at least one SSB is available every 160ms in PCell” which is in-line with the LS that RAN4 sent us in RAN1#120bis (R1-2502936). |
| Apple | We support the conclusion from FL. And regarding SSB-less SCell operation, already RAN4 agreed in RAN4#115 that SSB-less operation is precluded as they agreed that the baseline assumption is transmitting SSB. Exactly following was agreed in RAN4#115: Issue 1-1-1: SSB assumptions for R19 LB CA via switching Agreement on Monday AH: - The baseline assumption is: - SDL SCell is transmitting SSB - RAN4 to use SSB based requirement as baseline for Rel-19 low band CA via switching - requirement of PCell will still use the existing principle as baseline but the impact of carrier switching in LB CA will be considered |
| Moderator | Discussion closed # Proposal upated for offline discussion |

## [Closed] UE processing timeline relaxation

## [Closed] HARQ-ACK

## RACH

[Closed][R2] Proposal 4.6’

For Rel-19 low NR band carrier aggregation via switching, when NW configures a semi-static switching pattern to a UE, for RACH, adopt the following TP to Rel-19 TS38.213

## *** Unchanged parts are omitted *** 24 Downlink carrier aggregation via switching A UE that can switch operation between a PCell, that includes a DL carrier and a paired UL carrier, and an SCell, that includes a DL carrier without a paired UL carrier, can be provided by LBCA-SwitchingPattern a bitmap of slots indicating a switching pattern between the PCell and the SCell when the SCell is activated, except during random access procedure after a PRACH transmission according to [38.321]. The switching pattern repeats continuously. The UE can either transmit/receive on the PCell in a slot or receive on the SCell in the slot. ## *** Unchanged parts are omitted ***

| Company | Comments |
| --- | --- |
| Moderator | - This TP was proposed by Qualcomm during the round 1 discussion with some support. Please share your view on the TP above. Qualcomm can also provide (1) Reason for change, (2) Summary of change and (3) Consequences if not approved. However, from moderator perspective, this TP conflicts with the following agreement in the last RAN1 meeting, i.e., no violation of the configured pattern Agreement (confirmed in Thursday session as follows) For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling - The following cases for collision handling are specified where UL TA is assumed to be 0 for the determination of overlapping. The UL TA =0 assumption only applies to the UL transmission configured by higher layer - Collision 1: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 2: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 3: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 4: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as FDD carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - Collision 5: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 6: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will lead to above collisions - Except that the PUSCH, PUCCH, PRACH, or SRS transmission that overlapped with the switching gap on SDL carrier when the SDL carrier switches to FDD carrier is not considered as collision |
| ZTE | - Agree with the Moderator’s analysis. |
| MTK | We are in principle supportive for the proposal that UE stays on FDD during a RACH procedure after it transmits PRACH. Again, it is not clear to us how gNB can figure out how to schedule Msg2/Msg3 for a CBRA. On the TP, our reading is that the switching pattern can be discarded/ignore during RACH. However, it is not clear what the UE behavior should be exactly. Also, when/how to resume/reconfigure a switching pattern should be clarified. |
| Nokia | Agree with moderator, therefore, TP is not needed. |
| DOCOMO | We have same understanding with moderator. |
| LGE | Agree with the Moderator’s analysis. |
| vivo | Agree with the Moderator’s analysis. |
| Ofinno | We support the text proposal. Based on the proposed changes, our understanding is that the UE will resume the switching pattern after the RACH procedure is completed. While the proposed TP seems ok. But we are also fine to clarify the ending point/time of the RACH procedure when the switching pattern should resume. Regarding moderator analysis, in our view we should first focus on solving the issue/problem, i.e., RACH transmission. Any conflict/contradiction with the previous agreement can be resolved based on latest agreements. |
| Ericsson | Agree with the moderator’s analysis. We do not support the text proposal. Further, as we said in the 1st round, we think this issue can be handled by gNB implementation, providing an affirmative answer to the following question that Qualcomm posed in the first round: First of all, we would like to check if the issue can be handled by gNB implementation. In our understanding, even if the semi-static pattern is effective during RACH procedure, it is possible for gNB to handle the RACH related transmissions/receptions taking into account the configured pattern/gap. For CBRA, gNB would not know whether and which semi-static pattern/gap is configured to the RACH UE. Nevertheless, if the gNB can configure semi-static pattern/gap such that at least a certain set of slots belong to the PCell for all the UEs in the cell, the UE-common PCell slots can be used for RACH procedure. |
| Apple | Agree with FL |
| Moderator | Discussion closed Not proceed with the proposal due to strong objection |

## Collision handling

[Closed][R2] Proposal 4.7

For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling of SSB, adopt one of the following

- Option 1: UE does not receive the SSB that collides with the LB CA switching pattern

- Option 2: UE does not expect the NW to configure the SSB that collides with the LB CA switching pattern

- Option 3: UE receives the SSB that collides with the LB CA switching pattern

- Note: a SSB collides with the LB CA switching pattern if

  - For SSB in FDD carrier, one or multiple SSB symbols overlaps with symbols configured as either SDL carrier or switching gap

  - For SSB in SDL carrier, one or multiple SSB symbols overlaps with symbols configured as either FDD carrier or switching gap

| Company | Comments |
| --- | --- |
| Moderator | From moderator perspective, Option 1 is the understanding. Please share your view. |
| ZTE | Thanks moderator to list three options to resolve the collision handling of SSB. In our view, Option 3 should be precluded because SSB reception on both PCell and SCell simultaneously may occur, which cannot be allowed based on the WID scope. For Option 1 and Option 2, both options are workable. Since the collision handling operation agreed in RAN1#121 was based on reusing the collision handling rules for a transmission/reception overlapping with semi-static DL/UL symbols for unpaired spectrum. For the same principle as shown below high-lighted by the Moderator, it should be specified based Option 2, that the UE does not expect the set of symbols of a slot for reception of SSB on one cell to be indicated as switching gap or another cell. For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot that are indicated to a UE by ssb- PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.. |
| MediaTek | Our understanding is also Alt 1 which implies the switching pattern has a higher priority than SSBs. |
| Nokia | Agree with Option 1. The SSB periodicity is 20 ms and we would guarantee that at least 1 SSB is received in PCell and SCell within 160 ms. Therefore, it makes sense to discard SSBs colliding with the switching pattern. |
| DOCOMO | Our understanding is also Option 1 |
| LGE | Same understanding as other companies (i.e., Option 1). The switching pattern should be prioritized over any signals/channels including even SSB. |
| vivo | Our understanding is also Option 1 |
| Ofinno | Our understanding is also Option 1. The switching pattern is prioritized over SSB. |
| Ericsson | Our understanding is Option 1 But I have a question, does this require a spec change, or is it a conclusion? If it requires a spec change then it might be good to discuss a TP to Clause 24 that considers both collision with switching pattern and collision with switching gap. Or do we leave it to the editor? |
| Apple | Our understanding is also Option 1 |
| Moderator | Discussion closed # Updated proposal based on option 1 for offline discussion. @Ericsson: we can leave the TP to the editor, e.g., for TS38.213 |

## Others

[Clsoed][R2] Question 4.8

Please provide feedback in the following table regarding what is missing that is essential

| Company | Comments |
| --- | --- |
| Moderator | I may not include all the proposals, please add the missing ones. Please only essential things |
| Qualcomm | We wonder whether we need to fix RRC parameter value ranges for configured gap durations. This impacts ASN.1 and would be more essential. |
| Ericsson | Agree with QC. I think the FL has proposed a value range in the RRC parameter spreadsheet in the drafts folder. I assume the FL plans to discuss this proposal? |
| LGE | It seems necessary to clarify the following cases: - How to handle (e.g. whether pause or not) SR prohibit timer and CG retransmission timer in SDL duration by switching pattern - How to handle (e.g. whether pause or not) BWP inactivity timer and DRX retransmission timer for FDD (or SDL) in SDL duration (or FDD duration) by switching pattern |
| Moderator | @Qualcomm, @Ericsson RRC parameter will be discussed in this meeting. At least we need to resolve the unit and value range of switching gap |
| MediaTek | We agree with FL and other companies that we should resolve RRC parameters at this meeting. |
| Moderator | Discussion closed |

# (Closed Discussion) Low band carrier aggregation via switching

## UCI multiplexing order

Several companies, including Spreadtrum, vivo, Samsung, proposed clarification of UCI multiplexing in TS#38.213

[Closed][R1] Proposal 4.1

For Rel-19 low NR band carrier aggregation via switching, UE handles the overlapping of UL transmission with the RRC configured semi-static switching pattern, before the handling of UCI multiplexing

Adopt the following TP

Reason for change: The handling order of UCI multiplexing and overlapping of UL transmission with the RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, is not defined. Summary of change: Specify that UE handles the overlapping of UL transmission with the RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, before UCI multiplexing Consequences if not approved: UE behavior regarding the order of handling of UCI multiplexing and overlapping of UL transmission with the RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, is not clear.

Proposed TP to Rel-19 TS38.213

9 UE procedure for reporting control information ## *** Unchanged parts are omitted *** When a UE determines overlapping for PUCCH and/or PUSCH transmissions of the same priority index other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission due to downlink carrier aggregation via switching as described in clause 24 or cell DRX operation [11, TS 38.321] or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24 including repetitions if any, - first, the UE resolves the overlapping for PUCCHs with repetitions as described in clause 9.2.6, if any - second, the UE resolves the overlapping for PUCCHs without repetitions as described in clauses 9.2.5 - third, the UE resolves the overlapping for PUSCHs and PUCCHs with repetitions as described in clause 9.2.6 - fourth, the UE resolves the overlapping for PUSCHs and PUCCHs without repetitions as is subsequently described in this clause. ## *** Unchanged parts are omitted *** When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, other than PUCCH transmissions with SL HARQ-ACK reports, before considering limitations for transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24 including repetitions if any, if the UE is provided uci-MuxWithDiffPrio and the timeline conditions in clause 9.2.5 for multiplexing UCI in a PUCCH or a PUSCH are satisfied - first, the UE resolves overlapping for PUCCH and/or PUSCH transmissions of a same priority index as described in clauses 9.2.5 and 9.2.6 - second, the UE resolves the overlapping for PUCCH transmissions of different priority indexes, and ## *** Unchanged parts are omitted *** the UE - transmits the PUCCH or the PUSCH of the larger priority index subject to the limitations for UE transmissions due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 24 and - does not transmit a PUCCH or a PUSCH of smaller priority index When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, other than PUCCH transmissions with SL HARQ-ACK reports, before considering limitations for transmissions including with repetitions, if any, due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24, if the UE is not provided uci-MuxWithDiffPrio, the UE first resolves overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in clauses 9.2.5 and 9.2.6 - any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24 ## *** Unchanged parts are omitted *** In the remaining of this clause, a UE multiplexes UCIs with same priority index in a PUCCH or a PUSCH before considering limitations for UE transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24. A PUCCH or a PUSCH is assumed to have a same priority index as a priority index of UCIs a UE multiplexes in the PUCCH or the PUSCH. ## *** Unchanged parts are omitted ***

| Company | Comments |
| --- | --- |
| Moderator | - Please share your view on the necessity of the proposal, and your comments about the TP |
| ZTE | Support |
| Samsung | Support. The cover page is not aligned with the proposed TP and we suggest the following. Summary of change: Specify that UE handles the overlapping of UL transmission with the RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, before after UCI intra-UE multiplexing/prioritization. |
| DOCOMO | Support |
| Spreadtrum | Same view as Samsung. |
| Ericsson | We acknowledge the issue, and support the TP in general; however, it seems like a correction is needed in the first paragraph: there is some new text that should be highlighted in red “… downlink carrier aggregation via switching as described in clause 24 or …”. Or maybe this text should be deleted, since Clause 24 is referred to twice? |
| Nokia | It is still somewhat unclear when this issue manifests itself. If the UCI transmission is dropped because of the PUCCH/PUSCH timing overlapping with the SCell active phase then it doesn’t matter how the UCI multiplexing took place. If the UCI transmission is not dropped, there is no issue. If the question is about the case when there’s a third carrier that may transmit PUSCH, then the question to answer first is whether such band combination is supported, and only if the answer to that question is YES, is the question here relevant. And in such a case one could still question why the network would schedule a PUSCH that will be dropped. We do not oppose the clarification as such, but doubt whether we are addressing a problem that actually exists. |
| vivo | # Support with Samsung’s suggestion. And same observation as Ericsson. “downlink carrier aggregation via switching as described in clause 24 or” in the first paragraph should not be there. |
| LGE | OK |
| Xiaomi | Generally OK with us. |
| Moderator | Discussion closed # Modified proposal for Monday Offline. |

## SPS PDSCH

Several companies, including Spreadtrum, vivo, ZTE, proposed clarification of SPS PDSCH handling in TS38.214

[Closed][R1] Proposal 4.2

For Rel-19 low NR band carrier aggregation via switching, UE handles the overlapping of SPS PDSCHs with the RRC configured semi-static switching pattern, before the handling of overlapping among SPS PDSCHs

Adopt the following TP

Reason for change: The handling order of overlapping among SPS PDSCHs within a slot and overlapping between SPS PDSCH and RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, is not defined. Summary of change: Specify that UE handles the overlapping between SPS PDSCH and RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, before the handling of overlapping among SPS PDSCHs within a slot. Consequences if not approved: UE behavior regarding the order of handling overlapping among SPS PDSCHs within a slot and overlapping between SPS PDSCH and RRC configured semi-static switching pattern for Rel-19 low NR band CA via switching, is not clear.

Proposed TP to Rel-19 TS38.214

5.1 UE procedure for receiving the physical downlink shared channel <Unchanged parts are omitted> If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, or determined as non-active periods of cell DTX, if the serving cell is activated with cell DTX, based on [10, TS 38.321], or are in a duration or in a slot that a UE does not receive the corresponding PDSCH, as described in clause 24 of [6, TS 38.213], a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below. ‒ Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot ‒ Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH. ‒ Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q. ‒ Step 3: Repeat step 1 and 2 until Q is empty or j is equal to the number of unicast/multicast PDSCHs in a slot supported by the UE. <Unchanged parts are omitted>

| Company | Comments |
| --- | --- |
| Moderator | - Please share your view on the necessity of the proposal, and your comments about the TP |
| ZTE | Support. Otherwise, different results of survivor SPS PDSCHs will be derived. For example, SPS PDSCH 0 overlaps with SPS PDSCH 1 in a slot, and the SPS PDSCH 0 also overlaps with the switching gap. - If the UE first resolves the overlapping with the switching gap, the SPS PDSCH 0 will be dropped and SPS PDSCH 1 will be survived. - If the UE first resolves overlapping between SPS PDSCHs, the SPS 0 will survive initially, but will later be dropped due to its overlap with the switching gap. As a result, both SPS PDSCHs are dropped. Based on the same principle, resolving overlapping with symbols in a slot indicated as the duration of switching gap or another cell should be operated before the Step 0-3. |
| Qualcomm | We are OK with the TP. |
| Samsung | We think the issue is valid. The TP can be simplified, e.g. as below. <Unchanged parts are omitted> If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, or determined as non-active periods of cell DTX, if the serving cell is activated with cell DTX, based on [10, TS 38.321], or not available for SPS PDSCH receptions as described in clause 24 of [6, TS 38.213], a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below. <Unchanged parts are omitted> |
| DCM | Support |
| Spreadtrum | Support |
| Ericsson | We acknowledge the issue, and support the direction of the TP; however, we don’t agree with the wording. It is not necessary to repeat the details of “in a duration or in a slot” since that is described in Clause 24 already. It would be better to simplify and say something like “… or determined as not to be transmitted as described in clause 24 of [6, TS 38.213]…” |
| Apple | In principle, we support and also fine with updated version by Samsung |
| Nokia | Agree in principle that there maybe an issue to be addressed. The proposed wording is somewhat complicated and modification e.g. along the lines suggested by Ericsson might be a better way to go. |
| vivo | The issue is valid. The wording for the TP should be refined. At least “or in a slot” is not needed. If the whole slot is indicated on the other cell, no one PDSCH on the cell needs to receive, and no need such handling. “determined as not to be received as described in clause 24 of [6, TS 38.213]” can be considered. |
| LGE | OK with simpler wording as either Samsung’s or Ericsson’s version. |
| Xiaomi | Generally OK with us. We agree with that the wording of this TP can be refined based on Ericsson’s suggestion. |
| Moderator | Discussion closed Modified proposal for Monday Offline. |

## Switching pattern restriction

[Closed][R1] Question 4.3

- This is a lingering issue. As we are in maintenance phase, please provide your feedback on

  - For proponents of the pattern restriction, please indicate which option(s) that you support that the priority order (only the essential restriction please)

  - For opponents of the pattern restriction, please indicate which option(s) you do not agree with

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, consider the following restrictions for the semi-static switching pattern

- Option 1: maximum 8 switches within 40 slots

  - Proposed by: Spreadtrum, Samsung

- Option 2: maximum 1 switch(es) within 4 slots

  - Proposed by: Spreadtrum

- Option 3: one SSB is available every 160ms PCell

  - Proposed by: Spreadtrum, DoCoMo

- Option 4: one SSB is available every 160ms SCell

  - Proposed by: Spreadtrum, DoCoMo

- Option 5: UE does not stay continuously on SDL carrier for longer than 15 slots

  - Proposed by: vivo

- Option 6: maximum 1 switch(es) within 5 slots

  - Proposed by: vivo

- Option 7: maximum 2 switches within 10 slots

  - Proposed by: Xiaomi, OPPO

- Option 8: the maximum of continuous slots on SCell is 7

  - Proposed by: Samsung

- Option 9: the UE does not expect the set of symbols of a slot for reception of SSB on one cell to be indicated as switching gap or another cell

  - Proposed by: ZTE

- Option 10: at least one PRACH occasion is available every 160ms on PCell

  - Proposed by: DoCoMo

| Company | Comments |
| --- | --- |
| Moderator | - This is a lingering issue. As we are in maintenance phase, please provide your feedback on - For proponents of the pattern restriction, please indicate which option(s) that you support that the priority order (only the essential restriction please) - For opponents of the pattern restriction, please indicate which option(s) you do not agree with Some companies also provided TP, but TP discussion can be left later or to editor. |
| ZTE | Support Option 9. In RAN1#121, collision handling for low NR band carrier aggregation via switching was agreed for most signals/channels. However, it is still not clear how to handle SSB collision with the configured switching pattern. Based on the discussion in RAN1#121 meeting and the corresponding helpful FL summary, the handling operation was agreed based on reusing the collision handling rules for a transmission/reception overlapping with semi-static DL/UL symbols for unpaired spectrum. For the same principle as shown below high-lighted by the Moderator, it should be specified that the UE does not expect the set of symbols of a slot for reception of SSB on one cell to be indicated as switching gap or another cell. Otherwise, an alternative interpretation would be that the UE does not receive the SSB on one cell overlaps, even partially, with the set of symbols of the DL slot indicated as the duration of switching gap or another cell. For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot that are indicated to a UE by ssb- PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.. |
| Qualcomm | We support a joint restriction of 1 + 2. |
| Samsung | If any restriction is to be applied, we support a joint restriction of 1 + 8. |
| DOCOMO | We support following restrictions with priority order. 1st priority: restrictions to ensure SSB availability (Option 3+4 or 9) 2nd priority: restrictions to ensure RO availability (Option 10) 3rd priority: restrictions to avoid too long contiguous duration on SCell (Option 5 or 8) 4th priority: restrictions to avoid too frequent switching (Option 7 or 1) |
| Spreadtrum | Support restriction of 1+2+3+4, or 1/2+3+4 |
| Ericsson | We don’t think restrictions are needed, especially since the pattern is RRC configured. Most of the options feel purely arbitrary. Regarding Options 3, 4, and 10 proposed by DOCOMO, while they are reasonable, we don’t think they require specification, at least in RAN1. It would be a poor configuration indeed if the gNB configured the pattern such that the UE would not be able to receive SSB or transmit RACH. |
| Nokia | In general agree with Ericsson. We do also acknowledge that there may need to be a minimum SSB availability guarantee to the UE, and hence would be fine with 3, 4 and 9. |
| vivo | # We think option 6 is similar as option 1 but with a finer granularity. We are also fine with option 3+4+10. |
| LGE | Similar view as Ericsson that the specification doesn’t seem necessary while Options 3 and 4 are reasonable. |
| Xiaomi | We think some options are not mutually exclusive. We first support Option 7. In addition, Option 3/4 is also fine with us. |
| Moderator | Discussion closed |

## UE processing timeline relaxation

In terms of the UE processing timeline relaxation, it was discussed via email briefly in the last meeting without much progress. In the legacy UL Tx switching, the additional processing time, i.e.,Tswitch, was introduced at least for the following cases, for example

- PUSCH processing time, reference TS38.214 Clause 6.4

- UE CSI computation time, reference TS38.214 Clause 5.4

- DCI triggered SRS transmission, reference TS38.214 Clause 6.2.1

- PRACH transmission, reference TS38.213 Clause 8.1

- Etc.

However, it is worthwhile to note that the UL Tx switching can be triggered by DCI, but Rel-19 low NR band carrier aggregation via switching is configured semi-statically via RRC.

[Closed][R1] Question 4.4

- This is a lingering issue. As we are in maintenance phase, please provide your feedback on

  - Question 4.4.1

    - For proponents, please indicate which cases(s) that you think is essential

    - For opponents, please indicate which option(s) you do not agree with

  - Question 4.4.2

    - If relaxation is needed, do you agree with the proposed relaxation value

  - Question 4.4.3

    - If relaxation is needed, do you agree with the proposed restriction

- Question 4.4.1: For Rel-19 low NR band carrier aggregation via switching, in terms of introducing UE processing timeline relaxation, i.e., TLB, switch,  consider the following cases for the specification of the UE processing timeline relaxation

  - Case 1: PDSCH reception on SDL carrier, and corresponding PUCCH for HARQ-ACK transmission on FDD carrier

    - Proposed by: Spreadtrum, LG, MediaTek, Google, Samsung

  - Case 2: PDCCH  reception on SDL carrier schedules PUSCH transmission on FDD carrier.

    - Proposed by: MediaTek, Samsung

  - Case 3: PDCCH  reception on SDL carrier schedules aperiodic SRS transmission on FDD carrier.

    - Proposed by: MediaTek, Samsung

  - Case 4: a RAR schedules a PUSCH transmission

    - Proposed by: Samsung

- Question 4.4.2: If UE processing timeline relaxation is introduced, the UE processing timeline relaxation, i.e., TLB, switch is the same as the switching period reported by the UE as defined in RAN4 LS R1-2501702, i.e., among {35us, 140us}

  - Proposed by: Spreadtrum, MediaTek

- Question 4.4.3: If UE processing timeline relaxation is introduced, consider the restriction that a UE does not expect to have more than one SDL->FDD switch or more than one FDD->SDL switch between the received downlink PDCCH/PDSCH and the corresponding uplink transmission (PUCCH/PUSCH/SRS) or DL reception (PDCCH, PDSCH, SSB, CSI-RS)

  - Proposed by: MediaTek, Google

| Company | Comments |  |
| --- | --- | --- |
| Moderator | - This is a lingering issue. As we are in maintenance phase, please provide your feedback on - Question 4.4.1 - For proponents, please indicate which cases(s) that you think is essential - For opponents, please indicate which option(s) you do not agree with - Question 4.4.2 - If relaxation is needed, do you agree with the proposed relaxation value - Question 4.4.3 - If relaxation is needed, do you agree with the proposed restriction |  |
| ZTE | - It is preferred to check the Question 4.4.1 first. However, based on the discussion in the last meeting, there is no consensus whether the UE processing timeline relaxation should be introduced based on different views from different UE vendors. |  |
| Qualcomm |  | We are OK to have the relaxation. If introduced, the candidate values of TLB,switch should be {35us, 70us, 140us}, as RAN4 agreed 70us as an additional switching delay. |
| Samsung | Timeline relaxation is necessary. The RAN4 defined switching gap can be considered as the maximum relaxation time. Fine with either to reuse the RAN4 defined switching gap or introduce a separate UE capability to report the value. For Question 4.4.3, we don’t think such restriction is necessary. |  |
| DOCOMO | As semi-static switching pattern is provided and there is no additional switching, we think timeline relaxation would not be so necessary. But we are open to hear answer to Question 4.4.1. |  |
| Spreadtrum | Same view as Qualcomm. |  |
| Ericsson | Question 4.4.1: We don’t think processing time relaxation is needed, especially since the pattern is RRC configured. Question 4.4.2: No Question 4.4.3: No |  |
| Apple | We are generally fine to introduce the proposed restrictions, at least for the cases under 4.4.1 |  |
| Nokia | Same view as Ericsson |  |
| LGE | We think the Case1 under Q4.4.1 requires the processing time relaxation for the HARQ-ACK on Pcell corresponding to the PDSCH on Scell. For other Case 2/3/4, however, it may need to clarify first whether Scell to Pcell UL scheduling is allowed for this low-band CA via switching. |  |
| Moderator | Discussion closed |  |

## HARQ-ACK

[Closed][R1] Question 4.5

- This is a lingering issue. As we are in maintenance phase, please provide your feedback especially if you disagree with any of the following 3 proposed enhancement

For Rel-19 low NR band carrier aggregation via switching, when NW configures the semi-static switching pattern to a UE, consider the following for HARQ-ACK enhancement

- Enhancement 1: For Type-I HARQ-ACK codebook generation, a row of the TDRA table overlapping with the invalid symbol(s) is excluded from the determination of the set of occasions for candidate PDSCH receptions.

  - Proposed by:  vivo, Xiaomi, Samsung, LG

- Enhancement 2: SPS PDSCH overlapping with the invalid symbol(s) is excluded from the generation of HARQ-ACK information.

  - Proposed by:  vivo, Samsung

- Enhancement 3: DAI signaling via DL DCI for the Type-2 HARQ-ACK codebook considering that only one carrier can be scheduled at a time.

  - Proposed by: LG

- Enhancement 4: For HARQ-ACK feedback timing, support more than 8 values in the K1 set with new values larger than 15.

  - Proposed by: Xiaomi

- Note: Invalid symbol(s) include

  - The symbol(s) overlaps with the configured switching gap

  - The symbol(s) during the semi-static configured PCell (FDD carrier) duration for SCell (SDL carrier)

  - The symbol(s) during the semi-static configured SCell (SDL carrier) duration for PCell (FDD carrier)

| Company | Comments |
| --- | --- |
| Moderator | - This is a lingering issue. As we are in maintenance phase, please provide your feedback especially if you disagree with any of the proposed enhancement. - Thanks vivo and Samsung for providing the TP as well. The TP can be discussed later or left to editor. |
| ZTE | - Support Enhancement 2: SPS PDSCH overlapping with the invalid symbol(s) is excluded from the generation of HARQ-ACK information. This is aligned with the principle due to overlapping with the set of symbols determined as non-active period of cell DTX. |
| Qualcomm | Enhancement 1: We are not OK with the proposal. Since semi-static switching pattern is “activated/deactivated” via MAC-CE (per RAN4 agreement), this proposal changes the basic UE procedure of Type-1 HARQ-ACK codebook generation. Enhancement 2: We are not OK with the proposal. Since semi-static switching pattern is “activated/deactivated” via MAC-CE (per RAN4 agreement), this proposal changes the basic UE procedure of HARQ-ACK information generation for SPS PDSCHs. Enhancement 3: It is not clear how exactly this proposal intends to change the spec. Enhancement 4: We do not think this is essential. |
| Samsung | We support Enhancement 1 and 2. Otherwise, the size of the HARQ-ACK codebook would unnecessarily increase, around half of the HARQ-ACK bits are padding NACKs. Type-1 HARQ-ACK codebook is not only determined based on RRC. For SPS PDSCH HARQ-ACK codebook, activation/deactivation would impact the HARQ-ACK codebook. For Type-1 HARQ-ACK codebook described in clause 9.1.2.1, it depends on the activity of a serving cell which can be changed by MAC CE. Therefore, we don’t agree with QC comments for Enhancement 1 and 2. #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel For a serving cell , an active DL BWP, and an active UL BWP, as described in Clause 12, the UE determines a set of occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot . If serving cell is deactivated, the UE uses as the active DL BWP for determining the set of occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based:. |
| DOCOMO | We don’t think any of the listed enhancement is essential. |
| Ericsson | We don’t agree to enhancements of either the semi-static or dynamic HARQ-ACK codebooks. These are optimizations, and without them, nothing is broken. Enhancement 4 could provide some more flexibility; however, if it was agreed, our expectation is that it would become an optional UE feature, which makes it not very useful. |
| Apple | We don’t think any of these enhancements are essential |
| Nokia | We don’t see these optimizations as essential either. |
| vivo | We support Enhancement 1 and 2, same view as Samsung. In addition, for SPS PDSCH HARQ-ACK feedback, non-active period of cell DTX is already considered in legacy spec, where cell DTX can be affected by group common DCI. |
| LGE | We support Enhancement 1/2/3. For clarification on the Enhancement 3, total-DAI field/signaling can be omitted (i.e., only counter-DAI field/signaling is sufficient) since Only one carrier can be scheduled with PDSCH at one time in this low-band CA via switching. |
| Xiaomi | We firstly support Enhancement 1 as this can greatly save HARQ-ACK feedback overhead for Type-1 HARQ-ACK codebook. In addition, we support Enhancement 4 which is also dependent on the switching pattern design. |
| Moderator | Discussion closed |

## RACH

[Closed][R1] Question 4.6

- This is a lingering issue. As we are in maintenance phase, please provide your feedback especially if you disagree with any RACH enhancement

For Rel-19 low NR band carrier aggregation via switching, when NW configures a semi-static switching pattern to a UE, considering the following RACH enhancement

- Enhancement 1: if a RACH procedure is triggered by a UE itself or by network signalling, the semi-static switching pattern, if configured, is temporarily suspended until the last DL reception or UL transmission involved in the RACH procedure is successfully completed.

  - Proposed by: Huawei

- Enhancement 2: consider the following enhancement

  - The UE does not apply the semi-static switching pattern and stays on the PCell (FDD carrier) at least during on-going random-access procedure on the PCell.

  - For a RACH procedure triggered by a PDCCH order, the UE does not apply the low band CA switching pattern starting from a slot in which the UE has received the PDCCH order for the preamble transmission and until the UE transmits HARQ ACK on the PUCCH.

  - In both CFRA and CBRA, the UE starts to re-apply the low band CA switching pattern after a slot in which the UE transmits HARQ-ACK on the PUCCH (i.e., Msg4 in CFRA and Msg5 in CBRA). if a RACH procedure is triggered by a UE itself or by network signalling, the semi-static switching pattern, if configured, is temporarily suspended until the last DL reception or UL transmission involved in the RACH procedure is successfully completed.

  - Proposed by: Ofinno

- Enhancement 3: For a RACH procedure on FDD carrier,

  - Once a UE transmits PRACH, the UE stays on FDD carrier, i.e., overrides switch pattern configuration, before the RACH procedure completes.

  - After RACH procedure completes, the UE switches to SDL carrier at the next configured FDD-to-SDL switching gap.

where the time of the RACH procedure completion can refer to a given time delay (e.g. 3ms) after Msg-2 for CFRA procedure and Msg-4 for CBRA procedure

  - Proposed by: OPPO

| Company | Comments |
| --- | --- |
| Moderator | - This is a lingering issue. As we are in maintenance phase, please provide your feedback especially if you disagree with any RACH enhancement - Even though I separate them in different enhancement, but all proposals follow the similar design that RACH can override the switching pattern, and the switching pattern resumes after RACH procedure completes (different proposals for how to define when the RACH procedure completes) |
| ZTE | - Since potential collisions during the RACH procedure can be addressed by the collision handling mechanism captured in the spec, the enhancement appears unnecessary. |
| Qualcomm | First of all, we would like to check if the issue can be handled by gNB implementation. In our understanding, even if the semi-static pattern is effective during RACH procedure, it is possible for gNB to handle the RACH related transmissions/receptions taking into account the configured pattern/gap. For CBRA, gNB would not know whether and which semi-static pattern/gap is configured to the RACH UE. Nevertheless, if the gNB can configure semi-static pattern/gap such that at least a certain set of slots belong to the PCell for all the UEs in the cell, the UE-common PCell slots can be used for RACH procedure. The latency of random access would be increased, but we think it is not really critical. If the issue is understood as critical and gNB implementation approach does not work, we can accept a clarification that the semi-static switching pattern/gap is not effective during RACH procedure. However, we do not think strict UE behavior shall be captured in the spec. Instead, simple clarification would be sufficient, e.g., the change as follows in 38.213 Section 24. A UE that can switch operation between a PCell, that includes a DL carrier and a paired UL carrier, and an SCell, that includes a DL carrier without a paired UL carrier, can be provided by LBCA-SwitchingPattern a bitmap of slots indicating a switching pattern between the PCell and the SCell when the SCell is activated, except during random access procedure after a PRACH transmission according to [38.321]. The switching pattern repeats continuously. The UE can either transmit/receive on the PCell in a slot or receive on the SCell in the slot.. The above amendment is similar to the measurement gap handling during random access procedure specified in RAN4 spec (38.133). During the per-UE measurement gaps the UE: […] - is not required to conduct reception/transmission from/to the corresponding NR serving cells for SA (with single carrier or CA configured) except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to [7].. |
| Samsung | We think that would make matters worse for the base station as it won’t know what the UE is doing, will unnecessarily complicate the specifications, and will probably require long maintenance. Basically, we don’t even think this is an “enhancement”. The issue can be handled as today for a DL-heavy UL-DL configuration (LTE even supports 9 DL and 1 UL) and the RAR window is long enough. |
| DOCOMO | We don’t think any of the listed enhancement is essential. |
| Ericsson | We do not support Enhancements 1, 2, or 3. We are concerned about any enhancement that is based on autonomous switch back to the FDD carrier, since we think that can lead to ambiguity between gNB and UE as pointed out by Samsung. Furthermore, by implementation the gNB can ensure that the UE is on the PCell when needed as described above by Qualcomm. |
| Ofinno | The switching pattern is semi-static, and gNB is more likely to keep the same pattern once configured. Configuring a pattern which will allow all RACH related transmissions/receptions within the pattern, will be very challenging and limiting. Therefore, in our view collision handling of RACH when semi-static pattern is configured is needed. We are open to simplify the wording in the spec. |
| Apple | With the understanding, that network can/should handle and ensure UE is on PCell when needed, we don’t think these enhancements are essential |
| Nokia | We do have a concern on the CBRA handling, where the gNB would not know when to transmit Msg2 to the UE, when to schedule the UE to transmit Msg3 and when to transmit Msg4 to the UE. In that respect it would seem safer to freeze the switching as suggested in the Qualcomm TP. |
| vivo | In the last meeting, we already agreed that UE does not transmit the PRACH in collision 1/2 and a UE is not expected any scheduling/triggering from network that will lead to above collisions. We think the collision for PRACH and switching pattern is solved by the agreement. - Collision 1: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 2: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - … - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will lead to above collisions - Except that the PUSCH, PUCCH, PRACH, or SRS transmission that overlapped with the switching gap on SDL carrier when the SDL carrier switches to FDD carrier is not considered as collision |
| LGE | Regarding RACH case, we think the ROs in SDL duration by switching pattern should be treated as invalid and then not selected by UE. |
| Moderator | Discussion closed |

## Collision handling

[Closed][R1] Question 4.7

- Please provide your feedback on the following collision handling issues proposed by vivo, Ofinno and ZTE.

For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling, companies also proposed several other maintenance issue

- Issue 1: CSI-RS and the corresponding CSI dropping

  - Proposed by vivo (R1-2505393), with Proposal 5 quoted below and the corresponding TP

Proposal 5: For periodic CSI-RS and semi-persistent CSI-RS on a serving cell configured in CSI report configuration in CSI-ReportConfig, - The most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell occurs in valid time of the serving cell - The UE reports a CSI report only if receiving at least one CSI-RS transmission occasion of each periodic CSI-RS resource or semi-persistent CSI-RS resource on a serving cell in valid time of the serving cell, no later than CSI reference resource, and the UE drops the CSI report otherwise

  - Proposed by Ofinno (R1-2505666) with Proposal 5 and 6 quoted below

Proposal 5: Clarify that a slot in a serving cell shall be considered to be a valid downlink slot if it does not overlap with any slot assigned to the other serving cell when configured with LBCA-SwitchingPattern. Proposal 6: Include the following text proposal in clause 5.2.2.5 of TS 38.214: A slot in a serving cell shall be considered to be a valid downlink slot if: - it comprises at least one higher layer configured downlink or flexible symbol, - it does not fall within a configured measurement gap for that UE, and - it does not overlap with any slot assigned to the other serving cell when configured with LBCA-SwitchingPattern [12, 38.331].

- Issue 2: Multi-slot PDSCH reception is omitted per PDSCH if overlapping with the unavailable duration of switching gap and another cell. TP proposed to clause 5.1 and 5.1.2.1 in TS38.214.

  - Proposed by ZTE (R1-2506127), with Proposal 3 and TP

- Issue 3: PUSCH repetition type A and/or TBoMS reception is omitted per slot if overlapping with the unavailable duration of switching gap and another cell. TP proposed to clause 6.1.2.1 in TS38.214

  - Proposed by ZTE (R1-2506127), with Proposal 4 and TP

- Issue 4: For PUSCH repetition type B, invalid symbols shall comprise the symbols overlapped with the unavailable duration of switching gap and another cell.  TP proposed to clause 6.1.2.1 in TS38.214

  - Proposed by ZTE (R1-2506127), with Proposal 5 and TP

- Issue 5: Multi-PUSCH transmission is omitted per PUSCH if overlapping with the unavailable duration of switching gap and another cell. TP proposed to clause 6.1 and 6.1.2.1 in TS38.214

  - Proposed by ZTE (R1-2506127), with Proposal 6 and TP

| Company | Comments |
| --- | --- |
| Moderator | Thanks vivo, Ofinno and ZTE for raising the issues of collision handling. Before we get into detailed TP discussion, would like to get feedback on whether companies are willing to discuss issue 1-5 listed above |
| ZTE | All above issues should be discussed similarly to proposal 4.1 and proposal 4.2. Otherwise, the UE behavior is not clear. Note Issue 3 above includes some minor updates. |
| Qualcomm | Issue 1: We think Offino’s proposal makes sense. Regarding vivo’s proposal, we are not sure why we need to specify “the most recent one within the valid time”. Such requirement does not exist for legacy TDD case. Issue 2: We are not OK with the proposal. It is against the following agreement: - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will lead to above collisions Issue 3: We are not OK with the proposal. It is against the following agreement: - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will lead to above collisions Issue 4: We are not OK with the proposal. It is against the following agreement: - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will - lead to above collisions Issue 5: We are not OK with the proposal. It is against the following agreement: - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will lead to above collisions |
| Samsung | Issue 1 is similar as the issue in legacy when CSI-RS overlaps with semi-static UL symbols, no special handling is needed following legacy rules. We don’t think Issue 2-5 are valid, this is FR1, we don’t think repetition is typical. For the worst case, network scheduling can avoid the collision. So, no need to add specifications for things that don’t happen/exist. |
| DOCOMO | As QC clarified, we already made agreement that UE does not expect the collision caused by DCI scheduling/triggering. So, we don’t need to discuss those cases. |
| Ericsson | Issue 1: We think that the gNB by implementation can ensure that the UE is on the right carrier to measure and report Issue 2,3,4,5: Agree with QC and DOCOMO that the prior agreement means we don’t need to discuss these issues for channels scheduled by DCI. However, repetition for PDSCH/PUSCH can also be configured by RRC, hence it seems this case can be discussed. |
| Ofinno | To Samsung, on Issue 1, please note semi-static pattern is conceptually similar to a measurement gap. Currently a valid downlink slot is defined (in section 5.2.2.5 of 38.214) as the one which does not overlap with the measurement gap. Therefore, the valid downlink slot in a the serving should be the one that does not overlap with the slot assigned to other serving cell. |
| Apple | On issue 1, although not necessary and we think it could be handled by gNB implementation, but we are open to discuss this. Issue 2,3,4,5 are not needed and agree that they go against the agreement from last meeting on dynamic scheduling |
| Nokia | Same view as Ericsson. Addressing these cases seems overspecification. |
| vivo | For issue 1, we don’t see the need to change the definition on CSI reference resource, one function of CSI reference resource is to define a timeline for UE to process CSI measurement and reporting. We already agreed in the last meeting that UE does not receive a CSI-RS that overlaps with switching gap or slots on the other cell, that means, some CSI-RS occasions are not available for CSI measurement which is similar as Cell DTX or DRX, so we gave TP on the most recent measurement occasion and UE behaviour when UE does not receive any valid CSI-RS. For issue 2-5, agree with Ericsson, we need to solve the cases for CG/SPS PDSCH. In addition, PUCCH repetition should also be considered, PUCCH repetition is similar as PUSCH repetition with available slot counting, i.e., postpone is adopted. |
| LGE | Similar view as Ericsson and Nokia. |

# Summary of contribution

## R1-2505230 Huawei, HiSilicon

Proposal: For R19 low band switching, if a RACH procedure is triggered by a UE itself or by network signalling, the semi-static switching pattern, if configured, is temporarily suspended until the last DL reception or UL transmission involved in the RACH procedure is successfully completed.

## R1-2505249 Spreadtrum

Observation 1: For Rel-19 low NR band carrier aggregation via switching, for the duration of the switching gap configured by RRC, NW ensures that the switching gap is enough to also cover the DL receive timing difference between the FDD carrier and SDL carrier - The maximum downlink receive timing difference between the FDD carrier and SDL carrier is up to RAN4 - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, specify the following restrictions for the semi-static switching pattern - (4) Option 1: maximum 8 switch(es) within 40 slot(s) and/or - (3) Option 2: maximum 1 switch(es) within 4 slot(s) - (1) Option 3: at least one SSB is available every 160ms PCell - (2) Option 4: at least one SSB is available every 160ms SCell - For Rel-19 low NR band carrier aggregation via switching, in terms of introducing UE processing timeline relaxation, i.e., TLB, switch, specify the following case for the UE processing timeline relaxation - Case 1: PDSCH reception on SDL carrier, and corresponding PUCCH for HARQ-ACK transmission on FDD carrier - If UE processing timeline relaxation is introduced, regarding the UE processing timeline relaxation, i.e., TLB, switch, - Option 1: The same as the switching period reported by the UE as defined in RAN4 LS R1-2501702, i.e., among {35us, 70us, 140us} - SPS PDSCH overlapping with a non-operational period or switching gap can be excluded first. - Non-transmitted UL channel due to overlapping with switching gap is excluded after UL multiplexing.

## R1-2505393 vivo

Proposal 1: The configured switching pattern shall guarantee that UE does not stay continuously on SDL carrier for longer than 15 slots. Proposal 2: For the restriction of maximum X switch(es) within Y slot(s), at least support Y=5, X=1. Proposal 3: For low band carrier aggregation via switching, UE handles the overlapping among SPS PDSCHs after resolving overlapping with symbols of the switching gap. Proposal 4: UE performs UL multiplexing of PUCCH/PUSCH before considering limitations for UE transmission due to switching gap. Proposal 5: For periodic CSI-RS and semi-persistent CSI-RS on a serving cell configured in CSI report configuration in CSI-ReportConfig, - The most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell occurs in valid time of the serving cell - The UE reports a CSI report only if receiving at least one CSI-RS transmission occasion of each periodic CSI-RS resource or semi-persistent CSI-RS resource on a serving cell in valid time of the serving cell, no later than CSI reference resource, and the UE drops the CSI report otherwise. where the valid time of the cell includes the symbols in a slot configured on the cell by switching pattern and not overlapping with switching gap. Proposal 6: For HARQ-ACK codebook generation of SPS PDSCHs, UE will not generate HARQ-ACK information for SPS PDSCH overlapping with symbols indicated as invalid periods of the serving cell based on semi-statically configured switching pattern including the switching gap when the switching pattern is applied. Proposal 7: For type 1 HARQ-ACK codebook generation, a row of the TDRA table overlapping with the symbols in invalid periods of the serving cell is excluded when determining the set of occasions of candidate PDSCH receptions when the switching pattern is applied. # Appendix ## TP#1 for TS 38.213 Reason for change: For SDL carrier, the PUCCH transmission can only be performed in FDD UL carrier. The ending slot of PDSCH nD to PUCCH timing is configured by dl-DataToUL-ACK as a value from the set of {0,1…15}. Thus, the duration of SDL carrier should be limited to have HARQ-ACK feedback in time. Summary of change: Add “The configured switching pattern shall guarantee that UE does not stay continuously on SDL carrier for longer than 15 slots”. Consequences if not approved: If the duration of SDL carrier is more than 15 slots, the PDSCH scheduled in the beginning slot may not have PUCCH resource to feedback in time.. ***********************************Start of TP#1******************************************** # 24 Downlink carrier aggregation via switching A UE that can switch operation between a PCell, that includes a DL carrier and a paired UL carrier, and an SCell, that includes a DL carrier without a paired UL carrier, can be provided by LBCA-SwitchingPattern a bitmap of slots indicating a switching pattern between the PCell and the SCell when the SCell is activated. The switching pattern repeats continuously. The UE can either transmit/receive on the PCell in a slot or receive on the SCell in the slot. A bit value of ‘0’ for a first slot in the bitmap indicates that the UE can transmit/receive on the PCell in the first slot, and a bit value of ‘1’ for a second slot in the bitmap indicates that the UE can receive on the SCell in the second slot. The first slot of the bitmap is same as the first slot of a system frame with SFN 0 on the PCell. The SCS for all configured DL BWPs or UL BWPs on both the PCell and the SCell is 15 kHz and the periodicity of the switching pattern is 40 slots. The configured switching pattern shall guarantee that UE does not stay continuously on SDL carrier for longer than 15 slots. <Unchanged part omitted> ***********************************End of TP#1******************************************** ## TP#2 for TS 38.214 Reason for change: The handling order of overlapping among SPS PDSCHs within a slot and overlapping between SPS PDSCH and switching is not defined. Summary of change: Specify that UE handles the overlapping between SPS PDSCH and switching gap before the handling of overlapping among SPS PDSCHs within a slot. Consequences if not approved: UE behavior regarding the order of f overlapping among SPS PDSCHs within a slot and overlapping between SPS PDSCH and switching gap is not clear.. ***********************************Start of TP#2******************************************** ## 5.1 UE procedure for receiving the physical downlink shared channel <Unchanged part omitted> If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, or determined as non-active periods of cell DTX, if the serving cell is activated with cell DTX, based on [10, TS 38.321], or indicated as switching gap by LBCA-SwitchingGap-Duration-PCelltoSCell for Pcell or LBCA-SwitchingGap-SCelltoPCell for Scell, a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below. ‒ Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot ‒ Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH. ‒ Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q. ‒ Step 3: Repeat step 1 and 2 until Q is empty or j is equal to the number of unicast/multicast PDSCHs in a slot supported by the UE. ***********************************End of TP#2******************************************** ## TP#3 for TS 38.213 Reason for change: Capture the handling order of overlapping between PUCCHs or PUCCHs and PUSCHs within a slot and overlapping between PUCCH/PUSCH and switching gap is not defined. Summary of change: Specify that UE handles the overlapping between PUCCH/PUSCH and switching gap before the handling of overlapping between PUCCHs or PUCCHs and PUSCHs within a slot. Consequences if not approved: UE behavior regarding the order of f overlapping between PUCCHs or PUCCHs and PUSCHs within a slot and overlapping between PUCCH/PUSCH and switching gap is not clear.. **********************************Start of TP#3******************************************** # 9 UE procedure for reporting control information <Unchanged part omitted> When a UE determines overlapping for PUCCH and/or PUSCH transmissions of the same priority index other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission due to cell DRX operation [11, TS 38.321] or as described in clauses 11.1, 11.1.1, 11.2A, 15 and 17.2 and 24 including repetitions if any, - first, the UE resolves the overlapping for PUCCHs with repetitions as described in clause 9.2.6, if any - second, the UE resolves the overlapping for PUCCHs without repetitions as described in clauses 9.2.5 - third, the UE resolves the overlapping for PUSCHs and PUCCHs with repetitions as described in clause 9.2.6 - fourth, the UE resolves the overlapping for PUSCHs and PUCCHs without repetitions as is subsequently described in this clause. After resolving the overlapping for PUCCH and/or PUSCH transmissions, and if cell DRX is activated for a serving cell and a PUCCH or PUSCH transmission would overlap with the non-active period of cell DRX of the serving cell, the UE does not transmit the PUCCH if HARQ-ACK information is not multiplexed in the PUCCH, or does not transmit the PUSCH if HARQ-ACK information is not multiplexed in the PUSCH and the PUSCH is not associated with a corresponding PDCCH, respectively. If a UE - is provided simultaneousPUCCH-PUSCH and would transmit a PUCCH with a first priority index and PUSCHs with a second priority index that is different than the first priority index, where the PUCCH and the PUSCHs overlap in time on different respective cells - can simultaneously transmit the PUCCH and the PUSCHs with different priority indexes [18, TS 38.306], the UE excludes the PUSCHs for resolving the time overlapping between the PUCCH and PUSCHs with different priority indexes, where the timeline conditions for resolving the overlapping PUCCH and PUSCHs are not required for the excluded PUSCHs. If a UE - is provided simultaneousPUCCH-PUSCH-SamePriority and would transmit a PUCCH and PUSCHs with same priority index, where the PUCCH and the PUSCHs overlap in time on different respective cells, - can simultaneously transmit the PUCCH and the PUSCHs with same priority index [18, TS 38.306], the UE excludes the PUSCHs for resolving the time overlapping between the PUCCH and PUSCHs with same priority index, where the timeline conditions for resolving the overlapping PUCCH and PUSCHs are not required for the excluded PUSCHs. When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, other than PUCCH transmissions with SL HARQ-ACK reports, before considering limitations for transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15 and 17.2 and 24 including repetitions if any, if the UE is provided uci-MuxWithDiffPrio and the timeline conditions in clause 9.2.5 for multiplexing UCI in a PUCCH or a PUSCH are satisfied - first, the UE resolves overlapping for PUCCH and/or PUSCH transmissions of a same priority index as described in clauses 9.2.5 and 9.2.6 <Unchanged part omitted> the UE - transmits the PUCCH or the PUSCH of the larger priority index subject to the limitations for UE transmissions due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, and 15 and 24 and - does not transmit a PUCCH or a PUSCH of smaller priority index When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, other than PUCCH transmissions with SL HARQ-ACK reports, before considering limitations for transmissions including with repetitions, if any, due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15 and 17.2, and 24, if the UE is not provided uci-MuxWithDiffPrio, the UE first resolves overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in clauses 9.2.5 and 9.2.6 - any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15 and 17.2 and 24 <Unchanged part omitted> In the remaining of this clause, a UE multiplexes UCIs with same priority index in a PUCCH or a PUSCH before considering limitations for UE transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15 and 17.2 and 24. A PUCCH or a PUSCH is assumed to have a same priority index as a priority index of UCIs a UE multiplexes in the PUCCH or the PUSCH. ************************************End of TP#3******************************************** ## TP#4 for TS 38.214 Reason for change: The most recent measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on a serving cell is not defined when UE is configured with a DL carrier switching pattern and UE reporting behavior when UE does not receive any CSI-RS transmission occasion of each periodic CSI-RS resource or semi-persistent CSI-RS resource on a serving cell is not clear. Summary of change: Specify the most recent measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on a serving cell is not defined when UE is configured with a DL carrier switching pattern and UE reporting behavior when UE does not receive any CSI-RS transmission occasion of each periodic CSI-RS resource or semi-persistent CSI-RS resource on a serving cell. Consequences if not approved: The measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on a serving cell is not clear when UE is configured with LBCA-SwitchingPattern.. *********************************Start of TP#4******************************************** #### 5.1.6.1 CSI-RS reception procedure <Unchanged part omitted> If the UE is configured with DRX and, - if the UE is configured to monitor DCI format 2_6 or WUS and configured by higher layer parameter ps-TransmitOtherPeriodicCSI or lpwus-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP', 'cri-RSRP-Index', 'ssb-Index-RSRP' and 'ssb-Index-RSRP-Index' when drx-onDurationTimer in DRX-Config [or lpwus_PDCCHMonitoringTimer in [XYZxxx]] is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer in DRX-Config also outside DRX active time for CSI to be reported; - if the UE is configured to monitor DCI format 2_6 or WUS and configured by higher layer parameter ps-TransmitPeriodicL1-RSRP or lpwus-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP' or 'cri-RSRP-Index' when drx-onDurationTimer in DRX-Config [or lpwus_PDCCHMonitoringTimer [in XYZxxx]] is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer in DRX-Config also outside DRX active time for CSI to be reported; - otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported. During non-active periods of cell DTX if cell DTX is activated for a serving cell, the UE is not expected to receive the periodic CSI-RS and semi-persistent CSI-RS on the serving cell configured in CSI report configuration in CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P'. If the cell DTX is activated for a serving cell [10, TS 38.321], the most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell occurs in active periods of cell DTX for CSI report configured by CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P'. If a UE is provided with LBCA-SwitchingPattern, the most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on a serving cell occurs in symbols in the slots indicated that the UE can receive on the serving cell by the bitmap provided by LBCA-SwitchingPattern and not overlapping with the first or the second duration as described in Clause 24 of [6, TS 38.213]. #### 5.2.2.5 CSI reference resource definition <Unchanged part omitted> For the CSI report configuration in CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI', the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion of each periodic CSI-RS resource or semi-persistent CSI-RS resource on a serving cell with cell DTX activated [10, TS 38.321] for channel measurement and/or interference measurement in active periods of cell DTX of the serving cell no later than CSI reference resource, and the UE drops the CSI report otherwise. When the UE is configured with LBCA-SwitchingPattern, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion of each periodic CSI-RS resource or semi-persistent CSI-RS resource on a serving cell for channel measurement and/or interference measurement in symbols in the slots indicated that the UE can receive on the serving cell by the bitmap provided by LBCA-SwitchingPattern and not overlapping with the first or the second duration as described in Clause 24 of [6, TS 38.213], and no later than CSI reference resource, and the UE drops the CSI report otherwise. When deriving CSI feedback, the UE is not expected that a NZP CSI -RS resource for channel measurement overlaps with CSI-IM resource for interference measurement or NZP CSI -RS resource for interference measurement. *********************************End of TP#4******************************************** ## TP#5 for TS 38.213 Reason for change: In the periods that a UE is configured to perform TX/RX on FDD carrier, the SPS PDSCHs on SDL carrier are not received by the UE. In the periods that a UE is configured to perform RX on SDL carrier, the SPS PDSCHs on FDD carrier are not received by the UE. There is no need to transmit corresponding HARQACK information of these SPS PDSCHs. Summary of change: Specify that SPS PDSCH HARQ-ACK codebook should exclude HARQ-ACK information the SPS PDSCHs on a cell overlapping with switching gap or the slots configured on the other cell. Consequences if not approved: UE transmits the HARQ-ACK information for the SPS PDSCH on a cell overlapping with switching gap or the slots configured on the other cell.. **********************************Start of TP#5******************************************** ### 9.1.2 Type-1 HARQ-ACK codebook determination <Unchanged part omitted> In the following pseudo-code, SPS PDSCH receptions associated with a SPS PDSCH configuration are activated by a DCI format with CRC scrambled by a CS-RNTI or by a DCI format with CRC scrambled by a G-CS-RNTI. Set NcellsDL to the number of serving cells configured to the UE Set NcSPS to the number of SPS PDSCH configurations configured to the UE for serving cell c Set NcDL to the number of DL slots for SPS PDSCH receptions on serving cell c with HARQ-ACK information multiplexed on the PUCCH Set j=0 – HARQ-ACK information bit index Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell while c<NcellsDL Set s=0 – SPS PDSCH configuration index: lower indexes correspond to lower RRC indexes of corresponding SPS configurations while s<NcSPS Set nD=0 – slot index while nD<NcDL if { a UE is configured to receive SPS PDSCHs from slot nD-NPDSCHrepeat+1 to slot nD for SPS PDSCH configuration s on serving cell c, excluding SPS PDSCHs that are not required to be received in any slot among overlapping SPS PDSCHs, if any according to [6, TS 38.214], or based on a UE capability for a number of PDSCH receptions in a slot according to [6, TS 38.214], or due to overlapping with a set of symbols - indicated as uplink by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated, and/or - determined as non-active period of cell DTX [11, TS 38.321] - indicated as switching gap by LBCA-SwitchingGap-Duration-PCelltoSCell for SPS PDSCHs on Pcell or by LBCA-SwitchingGap-SCelltoPCell for SPS PDSCHs on Scell when the switching pattern is applied - in a slot indicated as on Scell for SPS PDSCHs on Pcell, or in a slot indicated as on PCell for SPS PDSCHs on Scell, by LBCA-SwitchingPattern when the switching pattern is applied where, for unicast SPS PDSCHs, NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in SPS-Config or, if pdsch-AggregationFactor-r16 is not included in SPS-Config, by pdsch-AggregationFactor in PDSCH-config and, for multicast SPS PDSCHs, NPDSCHrepeat is provided by repetitionNumber if contained in an entry indicated by the time domain resource assignment field in the DCI format scheduling the PDSCH repetition, or provided by pdsch-AggregationFactor-r16 if included in SPS-Config or, otherwise, NPDSCHrepeat=1, and HARQ-ACK information for the SPS PDSCH is associated with the PUCCH } ojACK = HARQ-ACK information bit for this SPS PDSCH reception j=j+1; end if nD=nD+1; end while s=s+1; end while c=c+1; end while **********************************End of TP#5********************************************* ## TP#6 for TS 38.213 Reason for change: In the periods that a UE is configured to perform TX/RX on FDD carrier, the PDSCHs on SDL carrier are not received by the UE, In the periods that a UE is configured to perform RX on SDL carrier, the PDSCHs on FDD carrier are not received by the UE. There is no need to transmit corresponding HARQACK information of the PDSCHs. Summary of change: Specify to exclude the SLIVs on a cell overlapping with switching gap or the slots configured on the other cell when generating type 1 HARQ-ACK codebook. Consequences if not approved: UE transmits the HARQ-ACK information for the candidate PDSCH reception occasion on a cell overlapping with switching gap or the slots configured on the other cell.. **********************************Start of TP#6******************************************** #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel <Unchanged part omitted> Set j=0 - index of occasion for candidate PDSCH reception or SPS PDSCH release or TCI state update Set B=∅ Set MA,c=∅ Set CK1 to the cardinality of set K1 Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 for serving cell c If a UE is not provided ca-SlotOffset for any serving cell of PDSCH receptions and for the serving cell of corresponding PUCCH transmission with HARQ-ACK information while k<CK1 if modnU-K1,k+1,max2μUL-μDL,1=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook Set nD=0 – index of a DL slot overlapping with an UL slot Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1 while nD<Nk if either pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3, and timeDomainHARQ-BundlingType1 are provided for serving cell c R=RT; R'=RT'; Elseif either pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 is provided and timeDomainHARQ-BundlingType1 is not provided for serving cell c R=RT; else Set R to the set of rows end if Set CR to the cardinality of R Set r=0 – index of row in set R if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn or pucch-sSCellDynDCI-1-2 or pucch-sSCellDynDCI-1-3, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn and pucch-sSCellDynDCI-1-2 and pucch-sSCellDynDCI-1-3, and slot n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the serving cell of PUCCH transmission, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, nD=nD+1; else while r<CR if the UE is not provided timeDomainHARQ-BundlingType1 and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is - configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or - in switching gap by LBCA-SwitchingGap-Duration-PCelltoSCell for a PDSCH on Pcell or by LBCA-SwitchingGap-SCelltoPCell for a PDSCH on Scell, when the switching pattern is applied, or - in a slot indicated as slot on Scell for a PDSCH on Pcell, or in a slot indicated as slot on PCell for a PDSCH on Scell, by LBCA-SwitchingPattern when the switching pattern is applied where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if either pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 is provided, and HARQ-ACK information for PDSCH time resource derived by row r in slot n0,k+nD cannot be provided in slot nU R=R\r; elseif the UE is provided timeDomainHARQ-BundlingType1 and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,k+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is - configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or - in switching gap by LBCA-SwitchingGap-Duration-PCelltoSCell for a PDSCH on Pcell or LBCA-SwitchingGap-SCelltoPCell for a PDSCH on Scell, when the switching pattern is applied, or - in a slot indicated as on Scell for a PDSCH on Pcell, or in a slot indicated as on PCell for a PDSCH on Scell, by LBCA-SwitchingPattern when the switching pattern is applied and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD at least one symbol of the PDSCH time resource derived by row r of set R is - configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or - in switching gap by LBCA-SwitchingGap-Duration-PCelltoSCell for a PDSCH on Pcell or LBCA-SwitchingGap-SCelltoPCell for a PDSCH on Scell, when the switching pattern is applied, or - in a slot indicated as on Scell for a PDSCH on Pcell, or in a slot indicated as on PCell for a PDSCH on Scell, by LBCA-SwitchingPattern when the switching pattern is applied if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r. R=R\r; R'=R'\r; else r=r+1; end if end while if the UE is not provided with multiPDSCH-perSlotType1-CB and the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅, or if the UE is provided with multiPDSCH-perSlotType1-CB = 'disabled' and R≠∅, MA,c=MA,c∪j; j=j+1; else Set CR to the cardinality of R Set m to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of R while R≠∅ Set r=0 while r<CR if S≤m for start OFDM symbol index S for row r br,k,nD=j; - index of occasion for candidate PDSCH reception, or SPS PDSCH release, or TCI state update associated with row r R=R\r; B=B∪br,k,nD; else r=r+1; end if end while MA,c=MA,c∪j j=j+1; Set m to the smallest last OFDM symbol index among all rows of R; end while end if nD=nD+1; end if end while end if k=k+1; end while else while k<CK1 if modnU-K1,k+(Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c)∙2μUL+1,max⁡(2μUL-μDL,1)=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook Set nD=0 – index of a DL slot overlapping with an UL slot Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1 while nD<Nk if either pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3, and timeDomainHARQ-BundlingType1 are provided for serving cell c R=RT; R'=RT'; Elseif either pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 is provided, and timeDomainHARQ-BundlingType1 is not provided for serving cell c R=RT; else Set R to the set of rows end if Set CR to the cardinality of R Set r=0 – index of row in set R if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn or pucch-sSCellDynDCI-1-2 or pucch-sSCellDynDCI-1-3, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn and pucch-sSCellDynDCI-1-2 and pucch-sSCellDynDCI-1-3, and slot n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the serving cell of PUCCH transmission where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0, nD=nD+1; else while r<CR if the UE is not provided timeDomainHARQ-BundlingType1 and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is - configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or - in switching gap by LBCA-SwitchingGap-Duration-PCelltoSCell for a PDSCH on Pcell or LBCA-SwitchingGap-SCelltoPCell for a PDSCH on Scell, when the switching pattern is applied, or - in a slot indicated as on Scell for a PDSCH on Pcell, or in a slot indicated as on PCell for a PDSCH on Scell, by LBCA-SwitchingPattern when the switching pattern is applied where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1 or if either pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 is provided, and HARQ-ACK information for PDSCH time resource derived by row r in slot n0,k+nD cannot be provided in slot nU R=R\r; elseif the UE is provided timeDomainHARQ-BundlingType1 and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,k+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured - configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or - in switching gap by LBCA-SwitchingGap-Duration-PCelltoSCell for a PDSCH on Pcell or LBCA-SwitchingGap-SCelltoPCell for a PDSCH on Scell, when the switching pattern is applied, or - in a slot indicated as on Scell for a PDSCH on Pcell, or in a slot indicated as on PCell for a PDSCH on Scell, by LBCA-SwitchingPattern when the switching pattern is applied and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD at least one symbol of the PDSCH time resource derived by row r of set R is - configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or - in switching gap by LBCA-SwitchingGap-Duration-PCelltoSCell for a PDSCH on Pcell or LBCA-SwitchingGap-SCelltoPCell for a PDSCH on Scell, when the switching pattern is applied, or - in a slot indicated as on Scell for a PDSCH on Pcell, or in a slot indicated as on PCell for a PDSCH on Scell, by LBCA-SwitchingPattern when the switching pattern is applied if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2, where d = 0,1,…,C∆K0,r-1. R=R\r; R'=R'\r; else r=r+1; end if end while if the UE is not provided with multiPDSCH-perSlotType1-CB and the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅, or if the UE is provided with multiPDSCH-perSlotType1-CB = 'disabled' and R≠∅, MA,c=MA,c∪j; j=j+1; else Set CR to the cardinality of R Set m to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of R while R≠∅ Set r=0 while r<CR if S≤m for start OFDM symbol index S for row r br,k,nD=j; - index of occasion for candidate PDSCH reception, or SPS PDSCH release, or TCI state update associated with row r R=R\r; B=B∪br,k,nD; else r=r+1; end if end while MA,c=MA,c∪j; j=j+1; Set m to the smallest last OFDM symbol index among all rows of R; end while end if nD=nD+1; end if end while end if k=k+1; end while end if **********************************End of TP#6********************************************

## R1-2505441 Xiaomi

Observation 1: The carrier switching pattern should be designed to guarantee DL transmission occasions for SSBs and reference signals on one carrier different from those on another carrier to avoid losing the synchronization on two carriers. Observation 2: The carrier switching pattern should be designed to avoid frequent switching as much as possible. Observation 3: The carrier switching pattern should be designed to guarantee the HARQ-ACK feedback corresponding to PDSCHs on DL-only carrier can be transmitted on the paired carrier. Observation 4: Rel-16 HARQ-ACK suspension and one-shot HARQ-ACK request can simplify the carrier switching pattern design to avoid frequent carrier switching for transmitting HARQ-ACK feedback for PDSCHs on Cell 2. Proposal 1: For NR low band CA via switching, the semi-static carrier switching pattern can be designed to have maximum 2 switches within 10 slots (i.e., one radio frame) subject to UE capability. Proposal 2: For Type-1 HARQ-ACK codebook, exclude candidate PDSCH reception occasions from a cell if UE switches to another cell. Proposal 3: For HARQ-ACK feedback timing, support more than 8 values in the K1 set with new values larger than 15.

## R1-2505558 Samsung

Proposal 1: Regarding the restriction of maximum X switch(es) within Y slot(s), support Y = 40 and X = 8. Observation 1: For low band carrier aggregation via switching, if the number of continuous slots on SCell is larger than 7, HARQ-ACK of a PDSCH reception in the last slot on PCell cannot be provided due to the K1 value restriction. Proposal 2: For low band carrier aggregation via switching, the maximum of continuous slots on SCell is 7. Observation 2: Alt 1 (Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier) would impact on UL transmissions on PCell as well as UCI multiplexing procedure. Proposal 3: A UE first resolves the overlapping UL transmission and then resolves the collision due to LB-CA switching. Adopt TP#1 for TS 38.213 clause 9. TP#1 Reason for change: UE behaviour is undefined for overlapping UL transmisions in case of LB-CA switching. Summary of change: Clarify that a UE first resolves the overlapping UL transmission and then resolves the collision due to LB-CA switching. Consequences if not approved: Undefined UE behaviour for intra-UE multiplexing in case of LB-CA switching. # 9 UE procedure for reporting control information ## *** Unchanged parts are omitted *** When a UE determines overlapping for PUCCH and/or PUSCH transmissions of the same priority index other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission due to downlink carrier aggregation via switching as described in clause 24 or cell DRX operation [11, TS 38.321] or as described in clauses 11.1, 11.1.1, 11.2A, , 15, and 17.2 and 24 including repetitions if any, - first, the UE resolves the overlapping for PUCCHs with repetitions as described in clause 9.2.6, if any - second, the UE resolves the overlapping for PUCCHs without repetitions as described in clauses 9.2.5 - third, the UE resolves the overlapping for PUSCHs and PUCCHs with repetitions as described in clause 9.2.6 - fourth, the UE resolves the overlapping for PUSCHs and PUCCHs without repetitions as is subsequently described in this clause. ## *** Unchanged parts are omitted *** When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, other than PUCCH transmissions with SL HARQ-ACK reports, before considering limitations for transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, , 15, and 17.2 and 24 including repetitions if any, if the UE is provided uci-MuxWithDiffPrio and the timeline conditions in clause 9.2.5 for multiplexing UCI in a PUCCH or a PUSCH are satisfied - first, the UE resolves overlapping for PUCCH and/or PUSCH transmissions of a same priority index as described in clauses 9.2.5 and 9.2.6 - second, the UE resolves the overlapping for PUCCH transmissions of different priority indexes, and the UE - transmits the PUCCH or the PUSCH of the larger priority index subject to the limitations for UE transmissions due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 24 and - does not transmit a PUCCH or a PUSCH of smaller priority index ## *** Unchanged parts are omitted *** When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, other than PUCCH transmissions with SL HARQ-ACK reports, before considering limitations for transmissions including with repetitions, if any, due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24, if the UE is not provided uci-MuxWithDiffPrio, the UE first resolves overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in clauses 9.2.5 and 9.2.6 - any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24 In the remaining of this clause, a UE multiplexes UCIs with same priority index in a PUCCH or a PUSCH before considering limitations for UE transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24. A PUCCH or a PUSCH is assumed to have a same priority index as a priority index of UCIs a UE multiplexes in the PUCCH or the PUSCH. ## *** Unchanged parts are omitted ***. Proposal 4: For low band carrier aggregation via switching, additional processing time should be introduced for the following cases if carrier switching happens between a DL reception and a UL transmission, - a PDCCH reception schedules a PUSCH transmission - a PDCCH reception indicates SRS transmission - a RAR schedules a PUSCH transmission - a PUCCH transmission with HARQ-ACK for a PDSCH or PDCCH reception Proposal 5: For low band carrier aggregation via switching, if a UE is provided with Type-1 HARQ-ACK codebook, and a candidate PDSCH reception on a serving cell overlaps with a non-operation period of the serving cell or switching gap, the candidate PDSCH reception is excluded from the set of occasions for candidate PDSCH receptions. Adopt TP#2 for TS 38.213 clause 9.1.2.1. TP#2 Reason for change: Type-1 HARQ-ACK codebook size is unnecessarily increased without considering the unavailable PDSCH SLIVs due to LB-CA switching, as a consequence, the spetrum effiency would unnecessarily decrease and UE power consumption would unnecessarily increase. Summary of change: Exclude unavailable PDSCH SLIVs due to LB-CA switching for Type-1 HARQ-ACK codebook. Consequences if not approved: The size of Type-1 HARQ-ACK codebook and the UE power consumption for transmiting a PUCCH with the Type-1 HARQ-ACK codebook would unnecessarily increase. #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel ## *** Unchanged parts are omitted *** while r<CR if the UE is not provided timeDomainHARQ-BundlingType1 and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or the UE does not receive the PDSCH corresponding to row r as described in clause 24 where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if either pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 is provided, and HARQ-ACK information for PDSCH time resource derived by row r in slot n0,k+nD cannot be provided in slot nU R=R\r; elseif the UE is provided timeDomainHARQ-BundlingType1 and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,k+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or the UE does not receive the PDSCH corresponding to row r as described in clause 24, and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or the UE does not receive the PDSCH corresponding to row r as described in clause 24 if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r. R=R\r; R'=R'\r; else r=r+1; end if end while ## *** Unchanged parts are omitted ***. Proposal 6: For low band carrier aggregation via switching, if a SPS PDSCH reception on a serving cell overlaps with the non-operation period of the serving cell or the switching gap, the UE does not transmit HARQ-ACK for the SPS PDSCH reception. Adopt TP#3 for TS 38.213 clause 9.1.2. TP#3 Reason for change: Type-1 HARQ-ACK codebook size is unnecessarily increased without considering the unavailable SPS PDSCH due to LB-CA switching, as a consequence, the spetrum effiency would unnecessarily decrease and UE power consumption would unnecessarily increase. Summary of change: Exclude unavailable SPS PDSCH due to LB-CA switching for Type-1 HARQ-ACK codebook. Consequences if not approved: The size of Type-1 HARQ-ACK codebook and the UE power consumption for transmiting a PUCCH with the Type-1 HARQ-ACK codebook would unnecessarily increase. ### 9.1.2 Type-1 HARQ-ACK codebook determination ## *** Unchanged parts are omitted *** In the following pseudo-code, SPS PDSCH receptions associated with a SPS PDSCH configuration are activated by a DCI format with CRC scrambled by a CS-RNTI or by a DCI format with CRC scrambled by a G-CS-RNTI. Set NcellsDL to the number of serving cells configured to the UE Set NcSPS to the number of SPS PDSCH configurations configured to the UE for serving cell c Set NcDL to the number of DL slots for SPS PDSCH receptions on serving cell c with HARQ-ACK information multiplexed on the PUCCH Set j=0 – HARQ-ACK information bit index Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell while c<NcellsDL Set s=0 – SPS PDSCH configuration index: lower indexes correspond to lower RRC indexes of corresponding SPS configurations while s<NcSPS Set nD=0 – slot index while nD<NcDL if { a UE is configured to receive SPS PDSCHs from slot nD-NPDSCHrepeat+1 to slot nD for SPS PDSCH configuration s on serving cell c, excluding SPS PDSCHs that are not required to be received in any slot among overlapping SPS PDSCHs, if any according to [6, TS 38.214], or based on a UE capability for a number of PDSCH receptions in a slot according to [6, TS 38.214], or due to overlapping with a set of symbols - indicated as uplink by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated, and/or - determined as non-active period of cell DTX [11, TS 38.321], or - not available for SPS PDSCH receptions as described in clause 24 where, for unicast SPS PDSCHs, NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in SPS-Config or, if pdsch-AggregationFactor-r16 is not included in SPS-Config, by pdsch-AggregationFactor in PDSCH-config and, for multicast SPS PDSCHs, NPDSCHrepeat is provided by repetitionNumber if contained in an entry indicated by the time domain resource assignment field in the DCI format scheduling the PDSCH repetition, or provided by pdsch-AggregationFactor-r16 if included in SPS-Config or, otherwise, NPDSCHrepeat=1, and HARQ-ACK information for the SPS PDSCH is associated with the PUCCH } ojACK = HARQ-ACK information bit for this SPS PDSCH reception j=j+1; end if nD=nD+1; end while s=s+1; end while c=c+1; end while.

## R1-2505614 Ericsson

Proposal 1 The value range is the same for both RRC parameters LBCA-SwitchingGap-Duration-PCelltoSCell-r19 and LBCA-SwitchingGap-SCelltoPCell-r19 and defined as 1 .. 30. Proposal 2 Remove the extra ‘g’ in the switching gap parameter names in the endorsed RRC parameter table in [1] (highlighted in the above table in red).

## R1-2505666 Ofinno

Observation 1: The contention-based random access (CBRA) may be triggered due to a radio link problem (e.g., RLF, BFR etc.), loss uplink synchronization (e.g., expiry of time alignment timer), etc. The contention-free random access (CFRA) may be initiated by the network for critical tasks such as handover, etc. Proposal 1: The UE does not apply the semi-static switching pattern and stays on the PCell (FDD carrier) at least during on-going random-access procedure on the PCell. Proposal 2: For a RACH procedure triggered by a PDCCH order, the UE does not apply the low band CA switching pattern starting from a slot in which the UE has received the PDCCH order for the preamble transmission and until the UE transmits HARQ ACK on the PUCCH. Proposal 3: In both CFRA and CBRA, the UE starts to re-apply the low band CA switching pattern after a slot in which the UE transmits HARQ-ACK on the PUCCH (i.e., Msg4 in CFRA and Msg5 in CBRA). Proposal 4: Include the following text proposal in clause 24 of TS 38.213: If Type-1 random access procedure for a UE is triggered upon request of a PRACH transmission by higher layers, then the UE shall not apply LBCA-SwitchingPattern starting from a slot in which the UE transmits PRACH preamble and until a slot in which the UE transmits HARQ-ACK information in a PUCCH. If Type-1 random access procedure for a UE is triggered by a PDCCH order, then the UE shall not apply LBCA-SwitchingPattern starting from a slot in which the UE receives the PDCCH order and until a slot in which the UE transmits HARQ-ACK information in a PUCCH. Observation 2: When configured with the low band CA switching pattern, a CSI reference resource for CSI reporting in a serving cell (e.g., FDD PCell) may fall in or overlap with a slot, which is assigned to the other serving cell (e.g., SDL SCell). Proposal 5: Clarify that a slot in a serving cell shall be considered to be a valid downlink slot if it does not overlap with any slot assigned to the other serving cell when configured with LBCA-SwitchingPattern. Proposal 6: Include the following text proposal in clause 5.2.2.5 of TS 38.214: A slot in a serving cell shall be considered to be a valid downlink slot if: - it comprises at least one higher layer configured downlink or flexible symbol, - it does not fall within a configured measurement gap for that UE, and - it does not overlap with any slot assigned to the other serving cell when configured with LBCA-SwitchingPattern [12, 38.331]. # Annex A: TP to clause 24 of TS 38.213 ---------------------------------Start of Text Proposal to TS 38.213--------------------------------------- 24 Downlink carrier aggregation via switching A UE that can switch operation between a PCell, that includes a DL carrier and a paired UL carrier, and an SCell, that includes a DL carrier without a paired UL carrier, can be provided by LBCA-SwitchingPattern a bitmap of slots indicating a switching pattern between the PCell and the SCell when the SCell is activated. The switching pattern repeats continuously. The UE can either transmit/receive on the PCell in a slot or receive on the SCell in the slot. A bit value of ‘0’ for a first slot in the bitmap indicates that the UE can transmit/receive on the PCell in the first slot, and a bit value of ‘1’ for a second slot in the bitmap indicates that the UE can receive on the SCell in the second slot. The first slot of the bitmap is same as the first slot of a system frame with SFN 0 on the PCell. The SCS for all configured DL BWPs or UL BWPs on both the PCell and the SCell is 15 kHz and the periodicity of the switching pattern is 40 slots. When the UE is provided LBCA-SwitchingPattern, the UE is provided - a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell to the SCell over the first duration, and - a second duration, by LBCA-SwitchingGap-SCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the SCell to the PCell over the second duration The UE does not transmit a PUSCH, or PUCCH, or SRS, or PRACH configured by higher layers in a set of symbols when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’, assuming TTA=0. The UE does not receive a PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols on the PCell when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’. The UE does not receive a PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. The UE does not expect to detect a DCI format scheduling or triggering a PUSCH, or PUCCH, or SRS, or PRACH transmission in a set of symbols when the set of symbols includes at least one symbol that is in the first duration, or is in a slot with bit value ‘1’ and is not in the second duration. The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the PCell when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value of ‘1’. The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. If Type-1 random access procedure for a UE is triggered upon request of a PRACH transmission by higher layers, then the UE shall not apply LBCA-SwitchingPattern starting from a slot in which the UE transmits PRACH preamble and until a slot in which the UE transmits HARQ-ACK information in a PUCCH. If Type-1 random access procedure for a UE is triggered by a PDCCH order, then the UE shall not apply LBCA-SwitchingPattern starting from a slot in which the UE receives the PDCCH order and until a slot in which the UE transmits HARQ-ACK information in a PUCCH. ---------------------------------End of Text Proposal to TS 38.213--------------------------------------- # Annex B: TP to clause 5.2.2.5 of TS 38.214 ---------------------------------Start of Text Proposal to TS 38.214--------------------------------------- 5.2.2.5 CSI reference resource definition The CSI reference resource for a serving cell is defined as follows: - In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates. - In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6, TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1 and for FR2-NTN, - where +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and and are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211] - where for periodic and semi-persistent CSI reporting - if a single CSI-RS/SSB resource is configured for channel measurement nCSI_ref is the smallest value greater than or equal to 4⋅2µDL, such that it corresponds to a valid downlink slot, or - if multiple CSI-RS/SSB resources are configured for channel measurement nCSI_ref is the smallest value greater than or equal to , such that it corresponds to a valid downlink slot. If a CSI-ReportConfig is configured with the higher layer parameter codebookType set to 'typeI-SinglePanel-r19', with total number of CSI-RS ports P=128, for capability 2, as reported by UE capability indication, nCSI_ref is the smallest value greater than or equal to 10⋅2μDL, such that it corresponds to a valid downlink slot. - where for aperiodic CSI reporting and for CSI reporting with CSI-ReportConfig with eventType and with reportTransmissionMode set to ‘ModeA’, if the UE is indicated by the DCI to report CSI in the same slot as the CSI request, nCSI_ref is such that the reference resource is in the same valid downlink slot as the corresponding CSI request, otherwise nCSI_ref is the smallest value greater than or equal to , such that slot n- nCSI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in Clause 5.4. - where for CSI reporting with CSI-ReportConfig with eventType and with reportTransmissionMode set to ‘ModeB’, nCSI_ref is the smallest value greater than or equal to , such that slot n- nCSI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in Clause 5.4. - when periodic or semi-persistent CSI-RS/CSI-IM or SSB is used for channel/interference measurements, the UE is not expected to measure channel/interference on the CSI-RS/CSI-IM/SSB whose last OFDM symbol is received up to Z' symbols before transmission time of the first OFDM symbol of the aperiodic CSI reporting. A slot in a serving cell shall be considered to be a valid downlink slot if: - it comprises at least one higher layer configured downlink or flexible symbol, - it does not fall within a configured measurement gap for that UE, and - it does not overlap with any slot assigned to the other serving cell when configured with LBCA-SwitchingPattern [12, 38.331]. If there is no valid downlink slot for the CSI reference resource corresponding to a CSI Report Setting in a serving cell, CSI reporting is omitted for the serving cell in uplink slot n'. ….. -----------------------------------End of Text Proposal to TS 38.214-----------------------------------

## R1-2505726 OPPO

Observation 1: The overlapping between carrier unavailability duration and a channel/signal not involving in RACH procedure can be avoided by proper switching pattern configuration and channel/signal resource allocation. Proposal 1: UE does not expect a switching pattern configuration that gives more than two switching within one frame. Proposal 2: Adopt the following TP for Clause 24 in TS38.213. 24 Downlink carrier aggregation via switching ************** Unchanged parts omitted ************** The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. The UE does not expect the total number of first durations and second durations in a radio frame is more than two. ************** Unchanged parts omitted **************. Proposal 3: UE does not expect its transmission or reception in switch-to carrier could occur earlier than end of switching period after the starting of switching gap. - The relation of above restriction to UL TA is not explicitly described in specification. Proposal 4: Adopt the following TP for Clause 24 in TS38.213. 24 Downlink carrier aggregation via switching ************** Unchanged parts omitted************** The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. It is assumed that the switching gap duration is always sufficient for a UE to start an uplink transmission in the switching gap, where the starting of the uplink transmission is no earlier than the end of the switching period as defined in [Clause x.x in RAN4 spec] after the starting of the switching gap. ************** Unchanged parts omitted**************. Proposal 5: A PRACH occasion is invalid if it falls within the duration where FDD-carrier is indicated as unavailable carrier by switching pattern. In other words, UE can transmit PRACH only when FDD carrier is indicated as available carrier by switching pattern. Proposal 6: For a RACH procedure on FDD carrier, - Once a UE transmits PRACH, the UE stays on FDD carrier, i.e., overrides switch pattern configuration, before the RACH procedure completes. - After RACH procedure completes, the UE switches to SDL carrier at the next configured FDD-to-SDL switching gap. where the time of the RACH procedure completion can refer to a given time delay (e.g. 3ms) after Msg-2 for CFRA procedure and Msg-4 for CBRA procedure. Proposal 7: Adopt the following TP for Clause 24 in TS38.213. 24 Downlink carrier aggregation via switching ************** Unchanged parts omitted************** The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. After the UE transmits a PRACH in a RACH procedure on PCell, the UE stays on the PCell, regardless of configuration by LBCA-SwitchingPattern, until the next configured first duration after the ending of Msg-2 for CFRA or Msg-4 for CBRA of the RACH procedure. ************** Unchanged parts omitted**************.

## R1-2505832 Nokia

Observation 1: It is not obvious at which point in time the UE starts the switching operation after receiving the received SDL SCell configuration. Proposal 1: The UE starts applying the switching operation at the point it considers the SDL SCell as active Proposal 2: The UE suspends the switching operation and resumes “normal” FDD operation on the PCell after receiving an SCell deactivation MAC-CE.

## R1-2505849 LG Electronics

Proposal #1: Clarify UE behaviour in the situation that semi-static switching pattern activates Case 2 during periodic UL signal/channel transmission occasion/resource (e.g. RACH occasion, SR PUCCH, CG PUSCH, PUCCH/PUSCH repetition) on FDD carrier 1. - Define how to handle SR transmission counter/prohibit timer or available slot counting for PUSCH repetition on FDD carrier 1 during Case 2 - Consider FDD carrier 1 with discontinuous UL slots as TDD carrier, and the UL occasions/ resources on FDD carrier 1 during Case 2 as invalid Proposal #2: Add the switching time from Case 2 (with PDSCH reception) to Case 1 (with HARQ-ACK transmission) in the determination of UE PDSCH processing time. Proposal #3: Consider the following aspects related to HARQ-ACK codebook for the CA of carrier 1 and 2 via switching. - Exclusion of the slots invalid for PDSCH scheduling on carrier 1 (or 2) during Case 2 (or 1) in the construction of Type-1 HARQ-ACK codebook - DAI signaling via DL DCI for the Type-2 HARQ-ACK codebook considering that only one carrier can be scheduled at a time

## R1-2506032 MediaTek

Observation 1: Some front-end components are shared for uplink transmission path and downlink reception path. Therefore, When SDL->FDD switch is performed, the transmission path for FDD uplink carrier is not ready before the RF retuning is complete. Proposal 1: For low-band carrier aggregation via switching, when there is at least a carrier switch between the received downlink PDCCH/PDSCH and the corresponding uplink transmission PUCCH/PUSCH/SRS, - introduce additional processing time for UE in the following cases: - Case 1: A PUCCH transmission with HARQ-ACK for a PDSCH - Case 2: PDCCH reception schedules a PUSCH transmission - Case 3: A PDCCH reception indicates SRS transmission - The additional processing required for a UE is the switching period, 35us or 140us, indicated by the UE. - UE does not expect to have more than one SDL->FDD switch or more than one FDD->SDL switch between the received downlink PDCCH/PDSCH and the corresponding uplink transmission (PUCCH/PUSCH/SRS).

## R1-2506105 Google

- For the selection of the values X and Y, a sliding window within any continuous number Y of slots (which could span across the pattern periodicities) should be considered, the number of carrier switches in the continuous window of Y slots should not exceed X. - The UE can perform or the network can trigger the UE to perform carrier switching outside the semi-static carrier switching pattern to initiate or respond to specific high-priority procedures. The set of procedures and conditions for such UE deviation needs further study (e.g., RACH initiation on PCell, handover command, RLF, LTM, …). - When a Beam Failure Recovery (BFR) procedure or a handover procedure is initiated while the UE is active on PCell, the UE shall disregard any imminent switch to an SCell as per a configured semi-static carrier switching pattern and shall remain on PCell to complete the ongoing BFR or handover procedure. - When an RRC-configured measurement gap overlaps with a switching gap associated with the semi-static carrier switching pattern, the UE shall prioritize the measurement gap (i.e., performing measurements). Upon completion of the measurement gap, the UE shall resume adherence to the semi-static carrier switching pattern. - When a UE is configured with both DRX and a semi-static carrier switching pattern, the switching pattern shall only be considered during DRX Active Time. - The UE is not expected to do carrier switches during DRX Inactive Time. - If a switching gap, as defined by a semi-static carrier switching pattern, is scheduled to coincide with the expiry of drx-OnDurationTimer or drx-InactivityTimer (thus aligning with the transition to DRX Inactive Time), the UE shall prioritize entering DRX Inactive Time. The scheduled carrier switch shall be deferred, and the UE shall determine the appropriate carrier based on the state of the semi-static switching pattern upon starting the next DRX Active Time. - For Rel-19 low NR band carrier aggregation via switching, in terms of introducing UE processing timeline relaxation, i.e., TLB, switch, specify at least the following case for the UE processing timeline relaxation - Case 1: PDSCH reception on SDL carrier, and corresponding PUCCH for HARQ-ACK transmission on FDD carrier - UE does not expect to have more than one SDL->FDD switch or more than one FDD->SDL switch between the received downlink and the corresponding uplink transmission or downlink reception

## R1-2506127 ZTE

Proposal 1: The UE does not expect the set of symbols of a slot for reception of SSB on one cell to be indicated as switching gap or another cell. The following TP for clause 24 in TS38.213 should be adopted. Reason for change: For low NR band carrier aggregation via switching, collision handling has been resolved for most of signals/channels, while there is no explicit collision handling for SSB reception. Summary of change: Based on same principle of the collision handling rules for a transmission/reception overlapping with semi-static DL/UL symbols for unpaired spectrum, the UE does not expect the set of symbols of a slot for reception of SSB on one cell to be indicated as switching gap or another cell. Consequences if not approved: For low NR band carrier aggregation via switching, how to handle SSB collision is not clear. 24 Downlink carrier aggregation via switching A UE that can switch operation between a PCell, that includes a DL carrier and a paired UL carrier, and an SCell, that includes a DL carrier without a paired UL carrier, can be provided by LBCA-SwitchingPattern a bitmap of slots indicating a switching pattern between the PCell and the SCell when the SCell is activated. The switching pattern repeats continuously. The UE can either transmit/receive on the PCell in a slot or receive on the SCell in the slot. A bit value of ‘0’ for a first slot in the bitmap indicates that the UE can transmit/receive on the PCell in the first slot, and a bit value of ‘1’ for a second slot in the bitmap indicates that the UE can receive on the SCell in the second slot. The first slot of the bitmap is same as the first slot of a system frame with SFN 0 on the PCell. The SCS for all configured DL BWPs or UL BWPs on both the PCell and the SCell is 15 kHz and the periodicity of the switching pattern is 40 slots. When the UE is provided LBCA-SwitchingPattern, the UE is provided - a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell to the SCell over the first duration, and - a second duration, by LBCA-SwitchingGap-SCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the SCell to the PCell over the second duration The UE does not transmit a PUSCH, or PUCCH, or SRS, or PRACH configured by higher layers in a set of symbols when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’, assuming TTA=0. The UE does not receive a PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols on the PCell when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’. The UE does not receive a PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. The UE does not expect to detect a DCI format scheduling or triggering a PUSCH, or PUCCH, or SRS, or PRACH transmission in a set of symbols when the set of symbols includes at least one symbol that is in the first duration, or is in a slot with bit value ‘1’ and is not in the second duration. The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the PCell when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value of ‘1’. The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. The UE does not expect the set of symbols of a slot for reception of SS/PBCH blocks on PCell indicated by ssb- PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon includes at least one symbols that is in the first duration or is in a slot with bit value of ‘1’. The UE does not expect the set of symbols of a slot for reception of SS/PBCH blocks on SCell indicated by ssb-PositionsInBurst in ServingCellConfigCommon includes at least one symbols that is in the second duration or is in a slot with bit value of ‘0’. <Unchanged parts are omitted>. Proposal 2: Resolving overlapping with symbols in a slot indicated as the duration of switching gap or another cell should be operated before the step 0-3 for survivor SPS PDSCHs determination. The following TP for clause 5.1 in TS38.214 should be adopted. Reason for change: When the UE is provided LBCA-SwitchingPattern, resolving overlapping with symbols in a slot indicated as the duration of switching gap or another cell is not clear before or after the step 0-3 for survivor SPS PDSCHs determination. Different survivor SPS PDSCHs may be derived. Summary of change: Based on same principle of the collision handling rules for a transmission/reception overlapping with semi-static DL/UL symbols for unpaired spectrum, resolving overlapping with symbols in a slot indicated as the duration of switching gap or another cell should be operated before the step 0-3. Consequences if not approved: For low NR band carrier aggregation via switching, it is not clear how to determine the survivor SPS PDSCHs when the UE is provided LBCA-SwitchingPattern. 5.1 UE procedure for receiving the physical downlink shared channel <Unchanged parts are omitted> If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, or determined as non-active periods of cell DTX, if the serving cell is activated with cell DTX, based on [10, TS 38.321], or are in a duration or slot that the UE does not receive, as described in clause 24 of [6, TS 38.213], a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below. ‒ Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot ‒ Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH. ‒ Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q. ‒ Step 3: Repeat step 1 and 2 until Q is empty or j is equal to the number of unicast/multicast PDSCHs in a slot supported by the UE. <Unchanged parts are omitted>. Proposal 3: Multi-slot PDSCH reception is omitted per PDSCH if overlapping with the unavailable duration of switching gap and another cell. The following TP for clause 5.1 and 5.1.2.1 in TS38.214 should be adopted. Reason for change: When the UE is provided LBCA-SwitchingPattern, it is not clear that multi-slot PDSCH reception is omitted per PDSCH or not if overlapping with the unavailable duration of switching gap and another cell. Summary of change: For multi-slot PDSCH reception, it is omitted per PDSCH if overlapping with the unavailable duration of switching gap and another cell. Consequences if not approved: For low NR band carrier aggregation via switching, it is not clear how to receive multi-slot PDSCH. 5.1 UE procedure for receiving the physical downlink shared channel <Unchanged parts are omitted> A UE shall upon detection of a PDCCH with a configured DCI format 1_0, 1_1, 1_2, 1_3, 4_0, 4_1, or 4_2 decode the corresponding PDSCHs as indicated by that DCI. When the UE is scheduled with multiple PDSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, HARQ process ID is then incremented by 1 for each subsequent PDSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPDSCH applied if nrofHARQ-ProcessesForPDSCH is provided, or with modulo operation of nrofHARQ-ProcessesForPDSCH-v1700 applied if or nrofHARQ-ProcessesForPDSCH-v1700 is provided, or with modulo operation of 8 applied, otherwise. HARQ process ID is not incremented for PDSCH(s) not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol in a slot indicated for the UE can transmit/receive on another cell by LBCA-SwitchingPattern, or a symbol in a duration indicated by LBCA-SwitchingGap-Duration-PCelltoSCell or LBCA-SwitchingGap-Duration-SCelltoPCell if provided. When a UE is configured by the higher layer parameter repetitionScheme set to 'tdmSchemeA', the PDSCH includes two PDSCH transmission occasions. For each PDSCH, if either PDSCH occasion overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, the PDSCH is not received and HARQ process ID is not increment for the PDSCH. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to receive a PDSCH that overlaps in time with another PDSCH if the UE is not capable of receiving FDMed unicast and multicast PDSCH per slot per carrier. When HARQ feedback for the HARQ process ID is not disabled, or for the HARQ process associated with the first SPS PDSCH when HARQ-feedbackEnablingforSPSactive is provided and enabled, the UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Clause 9.2.3 of [6, TS 38.213]. For HARQ-ACK subject to HARQ-ACK deferral described in Clause 9.2.5.4 of [6 TS 38.213], the expected transmission of HARQ-ACK corresponds to the expected transmission HARQ-ACK in a first slot. When HARQ feedback for the HARQ process ID is disabled, the UE is not expected to receive another PDCCH carrying a DCI scheduling a PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process or to receive another PDSCH without corresponding PDCCH for the given HARQ process that starts until Tproc,1 after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationDL-r16, in a given scheduled cell, the UE is not expected to receive a first PDSCH and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH, where the two resources are in different slots for the associated HARQ-ACK transmissions, each slot is composed of symbols [4] or a number of symbols indicated by subslotLengthForPUCCH if provided, and the HARQ-ACK for the two PDSCHs are associated with the HARQ-ACK codebook of the same priority. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationDL-r16, in a given scheduled cell, the UE is not expected to receive a first PDSCH and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH if the HARQ-ACK for the two PDSCHs are associated with HARQ-ACK codebooks of different priorities. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH ending in symbol i on a scheduling cell, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i of a scheduling cell,. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], the PDCCH ending in symbol i is determined based on the PDCCH candidate that ends later in time. In a given scheduled cell, for any PDSCH corresponding to SI-RNTI, the UE is not expected to decode a re-transmission of an earlier PDSCH with a starting symbol less than N symbols after the last symbol of that PDSCH, where the value of N depends on the PDSCH subcarrier spacing configuration , with N=13 for =0, N=13 for =1, N=20 for =2, N=24 for =3, N=96 for =5, and N=192 for =6. <Unchanged parts are omitted> 5.1.2.1 Resource allocation in time domain <Unchanged parts are omitted> A PDSCH reception in a slot of a multi-slot PDSCH reception is omitted according to the conditions in clause 11.1,and clause 17.2 and clause 24 of [6, TS 38.213]. The UE is not expected to receive a PDSCH with mapping type A in a slot, if the PDCCH scheduling the PDSCH was received in the same slot and was not contained within the first three symbols of the slot. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], if the two PDCCH candidates scheduling the PDSCH with mapping Type A were received in the same slot as the PDSCH, both PDCCH candidates are expected to be contained within the first three symbols of the slot. <Unchanged parts are omitted>. Proposal 4: PUSCH reception is omitted per slot if overlapping with the unavailable duration of switching gap and another cell. The following TP for clause 6.1.2.1 in TS38.214 should be adopted. Reason for change: When the UE is provided LBCA-SwitchingPattern, it is not clear that multi-slot PUSCH transmission is omitted per slot or not if overlapping with the unavailable duration of switching gap and another cell. Summary of change: For multi-slot PUSCH transmission, a slot is not counted if at least one symbol indicated by resource allocation overlaps with unavailable duration of switching gap and another cell when AvailableSlotCounting is enabled, and a PUSCH in a slot is omitted per slot if overlapping with the unavailable duration of switching gap and another cell when AvailableSlotCounting is disabled. Consequences if not approved: For low NR band carrier aggregation via switching, it is not clear how to transmit multi-slot PUSCH. 6.1.2.1 Resource allocation in time domain <Unchanged parts are omitted> For paired spectrum and SUL band: - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or 0_3 for paired spectrum only, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, or 0_3 for paired spectrum only, based on the TDRA information field value in the DCI format 0_1, 0_2 or 0_3. - For the case of a reduced capability half-duplex UE, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3 when AvailableSlotCounting is enabled and K>1, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1, 0_2 or 0_3, based on the TDRA information field value in the DCI format 0_1, 0_2 or 0_3. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot does not start or end at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - For the case of the UE provided LBCA-SwitchingPattern, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3 when AvailableSlotCounting is enabled and K>1, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1, 0_2 or 0_3, based on the TDRA information field value in the DCI format 0_1, 0_2 or 0_3. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol in a slot indicated as value of ‘1’ by LBCA-SwitchingPattern, or a symbol in a duration indicated by LBCA-SwitchingGap-Duration-PCelltoSCell if provided. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on the TDRA information field value in the RAR UL grant. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on the TDRA information field value in the DCI scheduling the PUSCH. If a UE would transmit a PUSCH of PUSCH repetition Type A when AvailableSlotCounting is enabled and K>1 or a TB processing over multiple slots over N∙K slots, and the UE does not transmit the PUSCH of a TB processing over multiple slots or the PUSCH repetition Type A in a slot from the N∙K slots, according to Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213], and Clause 5.34.3 of [10, TS 38.321], the UE counts the slots in the number of N∙K slots. <Unchanged parts are omitted> For PUSCH repetition Type A and TB processing over multiple slots, a PUSCH transmission in a slot of a multi-slot PUSCH transmission is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A, Clause 15,and clause 17.2 and clause 24 of [6, TS 38.213], and Clause 5.34.3 of [10, TS 38.321]. <Unchanged parts are omitted>. Proposal 5: For PUSCH repetition type B, invalid symbols shall comprise the symbols overlapped with the unavailable duration of switching gap and another cell. The following TP for clause 6.1.2.1 in TS38.214 should be adopted. Reason for change: When the UE is provided LBCA-SwitchingPattern, it is not clear whether the invalid symbols comprise the symbols overlapped with the unavailable duration of switching gap and another cell. Summary of change: For PUSCH repetition type B, invalid symbols shall comprise the symbols overlapped with the unavailable duration of switching gap and another cell. Consequences if not approved: For low NR band carrier aggregation via switching, it is not clear how to transmit PUSCH repetition type B. 6.1.2.1 Resource allocation in time domain <Unchanged parts are omitted> For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows: - A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission. - A symbol in a slot indicated as value of ‘1’ by LBCA-SwitchingPattern, or a symbol in a duration indicated by LBCA-SwitchingGap-Duration-PCelltoSCell is considered as an invalid symbol for PUSCH repetition Type B transmission. - For operation in unpaired spectrum, symbols indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission. - For a reduced capability half-duplex UE in paired spectrum, symbols that do not start or end at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol of an SS/PBCH block with index indicated by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon or by NonCellDefiningSSB, or by ssb-PositionsInBurst in SSB-MTC-AdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission. - For operation in unpaired spectrum, symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set are considered as invalid symbol(s) for PUSCH repetition Type B transmission. <Unchanged parts are omitted>. Proposal 6: Multi-PUSCH transmission is omitted per PUSCH if overlapping with the unavailable duration of switching gap and another cell. The following TP for clause 6.1 and 6.1.2.1 in TS38.214 should be adopted. Reason for change: When the UE is provided LBCA-SwitchingPattern, it is not clear that multi-PUSCH transmission is omitted per PUSCH or not if overlapping with the unavailable duration of switching gap and another cell. Summary of change: For multi-PUSCH transmission, it is omitted per PUSCH if overlapping with the unavailable duration of switching gap and another cell. Consequences if not approved: For low NR band carrier aggregation via switching, it is not clear how to transmit multi-PUSCH. 6.1 UE procedure for transmitting the physical uplink shared channel <Unchanged parts are omitted> When the UE is scheduled with multiple PUSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH not overlapping with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPUSCH applied if nrofHARQ-ProcessesForPUSCH is provided, or with modulo operation of 16 applied, otherwise. HARQ process ID is not incremented for PUSCH(s) not transmitted if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, or a symbol in a slot indicated for the UE can transmit/receive on another cell by LBCA-SwitchingPattern, or a symbol in a duration indicated by LBCA-SwitchingGap-Duration-PCelltoSCell if provided. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to transmit a PUSCH that overlaps in time with another PUSCH. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet for the active BWP of a serving cell and PDCCHs that schedule two PUSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationUL-r16 or outOfOrderOperationUL-r18, for any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i on a scheduling cell,, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i of the scheduling cell. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the PDCCH ending in symbol i, the PDCCH candidate that ends later in time is used. The UE is not expected to be scheduled to transmit another PUSCH by a DCI format 0_0 with CRC scrambled by TC-RNTI, for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI format 0_0 with CRC scrambled by TC-RNTI or by an UL grant in RA Response. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1, 0_2 or 0_3 scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI. <Unchanged parts are omitted> 6.1.2.1 Resource allocation in time domain <Unchanged parts are omitted> If a UE is configured with pusch-TimeDomainAllocationListForMultiPUSCH or pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not expect to be scheduled with one or multiple PUSCH transmissions by a single DCI format 0_1 or 0_3, where each PUSCH transmission overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, or a symbol in a slot indicated for the UE can transmit/receive on another cell by LBCA-SwitchingPattern, or a symbol in a duration indicated by LBCA-SwitchingGap-Duration-PCelltoSCell if provided. <Unchanged parts are omitted>. Proposal 7: There is no additional spec impact, and the UE shall apply the same switching pattern in case of SCell dormancy. Proposal 8: Endorse the following updates on RRC parameters for low band carrier aggregation via switching: Parameter name in the spec; New or existing?; Default value aspect Description; Value range; Per (UE, cell, TRP, …); UE-specific or Cell-specific. LBCA-SwitchingPattern-r19; New; A 40-bit bitmap with each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier (PCell), and ‘1’ indicates SDL carrier (SCell).; BIT STRING (SIZE (40)); Per UE; UE-specific. LBCA-SwitchinggGap-Duration-PCelltoSCell-r19; New; Indicates the duration of the switching gap for the switching from PCell to SCell, in the unit of 15KHz SCS symbol.; INTEGER (1..FFS2); Per UE; UE-specific. LBCA-SwitchinggGap-SCelltoPCell-r19; New; Indicates the duration of switching gap for the switching from SCell to PCell, in the unit of 15KHz SCS symbol. NW ensures that the switching gap (“LBCA-SwitchinggGap-SCelltoPCell-r19”) is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA.; INTEGER (1..FFS[4 or 30]); Per UE; UE-specific.

## R1-2506193 Qualcomm

Proposal 1: - The value range of LBCA-SwitchingGap-Duration-PCelltoSCell-r19 is INTEGER (1, 2, 3) Proposal 2: - The value range of LBCA-SwitchingGap- SCelltoPCell-r19 is INTEGER (1,..31)

## R1-2506283 NTT DOCOMO

Proposal 1: RAN1 informs RAN4/2 that following restrictions are necessary from RAN1 perspective. - The semi-static switching pattern ensures that at least one SSB is available every 160ms on PCell - The semi-static switching pattern ensures that at least one SSB is available every 160ms on SCell - The semi-static switching pattern ensures that at least one PRACH occasion is available every 160ms on PCell

# References

- R1-2505230 Maintenance of Rel-19 low band CA via switching Huawei, HiSilicon

- R1-2505249 Remaining issues on low band carrier aggregation via switching Spreadtrum, UNISOC

- R1-2505393 Maintenance on Low band carrier aggregation via switching vivo

- R1-2505441 Remaining issues on low band CA via switching Xiaomi

- R1-2505558 Remaining issues on low band carrier aggregation via switching Samsung

- R1-2505614 Maintenance on Low band carrier aggregation through switching  Ericsson Inc.

- R1-2505666 Discussion on low-band CA with switching Ofinno

- R1-2505726 Maintenance on low-band CA via switching OPPO

- R1-2505832 Maintenance of low-band CA via switching Nokia

- R1-2505849 Remaining issues on low band CA operation via switching for Rel-19 LG Electronics

- R1-2506032 Remaining issues on low band carrier aggregation via switching MediaTek Inc.

- R1-2506105 On remaining issues for Low band Carrier Aggregation via switching Google

- R1-2506127 Maintenance on Low band carrier aggregation via switching ZTE Corporation, Sanechips

- R1-2506193 Low band carrier aggregation via switching Qualcomm Incorporated

- R1-2506283 Maintenance on Low band carrier aggregation via switching NTT DOCOMO, INC.

# Appendix A: Agreements from previous and current meeting

## RAN1#122

## RAN1#121

Agreement

For Rel-19 low NR band carrier aggregation via switching, support configuration of only one switching pattern to a UE based on UE-specific RRC configuration.

Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, only support the switching gap is located at the “switch from” carrier, i.e., FDD carrier.

- Switching gap ends at the end of the last slot on the “switch from” carrier.

Agreement (confirmed in Thursday session as follows)

For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling

- The following cases for collision handling are specified where UL TA is assumed to be 0 for the determination of overlapping. The UL TA =0 assumption only applies to the UL transmission configured by higher layer

  - Collision 1: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS

  - Collision 2: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS

  - Collision 3: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS.

  - Collision 4: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as FDD carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS

  - Collision 5: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS.

  - Collision 6: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS

- For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI

  - A UE is not expected any scheduling/triggering from network that will lead to above collisions

    - Except that the PUSCH, PUCCH, PRACH, or SRS transmission that overlapped with the switching gap on SDL carrier when the SDL carrier switches to FDD carrier is not considered as collision

UE expects that the switching gap configured by the NW for SDL carrier to FDD carrier switching covers at least the switching period as in RAN4 LS (R1-2501702) and UL TA

## RAN1#120bis

Agreement

- For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases

- Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

- Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

- FDD carrier 1 is PCell and SDL carrier 2 is SCell

- SCS 15KHz on both carriers

Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier.

Agreement

For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell.

Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS).

Agreement

- For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design

  - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2

  - The pattern starts from the beginning of SFN 0 of PCell

- Restriction of maximum X switch(es) within Y slot(s),

  - FFS: detailed value and relation of X and Y.

- FFS: other restriction

Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap,

- For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier.

- For switching from FDD carrier to SDL carrier, downselect one of the following solutions

  - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier

  - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier

  - Alt 3: Switching gap is always assumed at the SDL carrier

Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms

Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW,

- Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch.

- Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed)

  - For the UL TA,

    - Switching gap for SDL to FDD switch ends at the end of slot on SDL

      - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA

    - FFS: whether it has specification impact

# Appendix B: LS from other WGs

## R1-2503215 LS of RAN4 RRM agreements on LB CA via switching

RAN4 discussed the applicability of switching pattern for SDL SCell for RRM requirements and would like to share the following information to RAN1. - For the SCell which has been activated (i.e., after UE reporting valid CQI for SCell activation, e.g., in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - RAN4 will also discuss potential scheduling restriction/interruption on PCell during the SCell activation procedure for receiving the above mentioned RS occasions for the SCell activation - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - RAN4 will discuss and decide whether/how UE to perform the deactivated SDL SCell measurement Note: The switching pattern for SDL SCell referred above is the RAN1 defined semi-static switching pattern configured through RRC for the FDD PCell/SDL SCell switching.

## R1-2501700 LS of RAN4 RRM agreements on LB CA via switching RAN4

RAN4 discussed the impact of LB CA via switching on RAN4 RRM requirements and would like to share the following information to RAN1. - In TS38.133, UL transmit timing requirements are applicable provided that at least one SSB is available at the UE during the last 160 ms. - The required availability of SSB on PCell and/or SCell for RRM requirements is still under discussion in RAN4. RAN1 does not need to wait for RAN4 discussion.

## R1-2501702 LS on switching periods for low-low band switching RAN4

RAN4 would like to update RAN1 on the progress of the discussions related to the switching time for low NR band carrier aggregation via switching. To accommodate different UE implementations RAN4 has reached the following agreement: - Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us. Additional value(s) are under discussion. If any additional value(s) are confirmed, RAN4 will provide them in a future LS. RAN4 respectfully requests RAN1 to take this information into account in their work.