3GPP TSG RAN WG1 Meeting #122                       R1-2505940

Bengaluru, India, Aug 25th – Aug 29th, 2025

Agenda Item: 10.6.1

Source:	TCL

Title: Discussion on the GNSS resilient NR-NTN operation

Document for: Discussion and Decision

- Introduction

In the RAN#108 meeting, a new SID on NR-NTN GNSS resilient[1] has been approved, the detailed objectives are as follows:

Study GNSS resilient NR-NTN operation [RAN1, RAN4] - Assess impact on initial access and connected mode procedures for NR-NTN, including potential implications if any on RAN4 RRM specifications - Aim to minimize physical layer procedures impact - Avoid physical layer channel/signal changes - Backward compatible issues with legacy NTN capable UEs will be assessed and should be prevented - Applicable to NGSO and GSO constellations, NTN operating in below and in above 10 GHz frequency bands, transparent and regenerative satellites - Check in RAN#112 (June, 2026) to decide whether to proceed with normative work, extend the study item, or postpone normative work to Release 21

In this contribution, we provide our views on the study of NR-NTN GNSS resilient.

- Discussion

According to RP-251863[1] achieved in RAN#108, the study of NR-NTN GNSS resilient will initially focus on the scenario when GNSS information is temporarily unavailable. Two approaches can be considered to access the network when GNSS information is temporarily unavailable: UE-based positioning assisted access and enhanced close-loop adjustment including TA and frequency correction.

2.1 UE-based positioning assisted access

In the NTN scenario, the UE must rely on GNSS and ephemeris to compensate for the TA and Doppler frequency shift, so the NTN-UE needs to be equipped with GNSS. In scenarios where GNSS is unavailable due to coverage limitations, interference, jamming, or intentional avoidance, NR-NTN systems must maintain robust initial access and connected mode operations. The absence of GNSS can severely impact synchronization and Doppler frequency shift compensation, thereby affecting overall system efficiency and user experience. To address these challenges, the existing RAT-dependent positioning methods are suitable to be employed for NTT position without GNSS using single or multiple satellites.

Observation 1: RAT-dependent positioning can be considered for NTN positioning.

In TN network, the existing RAT-dependent positioning methods include E-CIN, multi-RTT, DL-AoD, UL-AoD, DL-TDOA, and UL-TDOA. In Rel-18, the RAT-dependent positioning methods for the verification of UE UE-reported location have been discussed. Among these position methods, Cell-ID-based and angle-based positioning methods due to low accuracy can be considered for the study on the network-verified UE location. Only multi-RTT and DL/UL-TDOA are considered for the study. Therefore, we think the method used for the verification of UE UE-reported location can be taken as a baseline, and we propose to use time-based positioning methods for GNSS resilient NR-NTN operation. During initial access,

Proposal 1: Time-based positioning methods can be a starting point for UE-based positioning to access the network.

- Single satellite for positioning

In a practical case, due to satellite density and orbital characteristics, UE may frequently encounter scenarios where only a single satellite signal is available, rendering multiple-satellite positioning infeasible. Hence, developing robust single-satellite positioning solutions is crucial to ensure initial access, synchronization, and basic service continuity even under constrained satellite availability. Additionally, compared to multiple-satellite positioning, single-satellite positioning offers advantages such as reduced system complexity, lower resource utilization, and simpler implementation, making it highly suitable for scenarios involving minimal resource availability or streamlined NTN deployments.

Proposal 2: For NTN positioning, the single satellite-based position should be considered.

As discussed in the previous meeting in Rel-18, the scenario of a single satellite in view by the UE at a time can be considered.	In the scenario of single-satellite positioning, positioning is based on measurements between a satellite and the UE at different time instances. During initial access, UE needs to pre-compensate TA and Doppler frequency shift before transmitting PRACH, while the connected mode is not established right now so the TDOA for positioning can be performed. During connected mode, TDOA and multi-RTT can be considered. However, the time interval between time instants must be large enough(tens of seconds) to ensure positioning accuracy with a qualified GDOP. Otherwise, the RTTs at different time instants almost remain unchanged, and it would thus be challenging to determine the position of the UE. As a result, the large time interval will cause a larger delay for UE access to the network.

Proposal 3: For single-satellite positioning, the enhancement of PRS can be considered to improve the position accuracy and reduce access latency.

Figure 1. Illustration of single-satellite based position

- Multiple satellites for positioning

In the TN position, multiple TRPs are deployed to transmit PRS to determine DL measurement for positioning, and the ultimate UE location is calculated based on PRS reception time from these TRPs. In the multiple-satellite case, PRS from multiple satellites/TRPs can be received, and the measurement of the difference of PRS reception time (RSTD) between different satellites/TRPs can be determined. Thus, the satellites can be considered as a TRP in the multiple-satellite case.

Proposal 4: For multiple-satellite positioning, each satellite can be considered as a TRP.

Figure 2. Illustration of multiple-satellite based position

Similar to the single-satellite case, TDOA can be considered for both initial access and connected mode, and multiple-RTT can be considered for connected mode. Taking the TDOA method as an example, at least three RSTD values must be measured for positioning, consequently requiring at least 4 satellites. In theory, the more satellites used for positioning, the higher the accuracy. So, the number of satellites for the NTN position to guarantee the accuracy needs to be evaluated and clarified.

Proposal 5: For multiple-satellite positioning, the number of satellites for positioning should be decided.

In addition, the configuration of PRS from different TRPs can be transmitted in posSIB for the TN position. For the NTN position, the configuration method can be reused, but the main difference could be that the satellites used for position need to make a relationship with the corresponding PRS ID to enable the UE to distinguish the PRS signals from different satellites.

Proposal 6: The mapping between satellites and PRS ID should be clarified under the multiple-satellite case.

2.2 Closed-loop control

For the TA adjustment procedure during initial access, UE will pre-compensate the TA before transmitting PRACH[2]. The common TA can be determined based on the system information and UE-specific TA can be determined via GNSS and ephemeris. UE can not calculate the specific TA without GNSS so the closed-loop timing needs to be enhanced. For example, the range of common TA can be extended for the cell-level UE just like the k_offset. Part of UE will be overcompensated and part of UE will be undercompensated. Thus, the TA command in RAR can indicate a negative value to solve this.

Proposal 7: The enhancement of the closed-loop time control can be studied.

UE needs to pre-compensate the Doppler frequency shift based on the position of the satellite and UE via GNSS. Thus, UE can not calculate the position without GNSS. Similarly, the closed-loop control for frequency correction should be taken just like the closed-loop timing adjustment.

Proposal 8: Frequency closed loop adjustment can be considered for GNSS-resilient NR-NTN.

2.3 Others

As mentioned above, UE can perform the TA and Doppler frequency shift pre-compensation via UE-based positioning and closed-loop control for timing advance or frequency correction with the unavailability of GNSS. If the two approaches are adopted for pre-compensation, the base station can not distinguish the UE with GNSS or without GNSS. This is because even if the discrepancy in compensation values is large, it is not necessarily caused by the UE-based positioning or closed-loop control processes. Issues such as poor GNSS signal quality, fluctuations in ephemeris data, or outdated GNSS measurement data can also cause this problem. Thus, UE needs to report whether the pre-compensation is based on GNSS. The candidate method is to associate with the PRACH resource, such as RO or preamble.

Proposal 9: Whether it is covered by GNNS can be reported implicitly via the PRACH resource, such as RO or preamble.

If the TA closed-loop control can be enhanced or the frequency shift correction can be introduced, the base station will indicate the correction value to UE after receiving PRACH. However, the UE cannot distinguish whether the indication applies to a UE without GNSS capability or a GNSS-capable UE. Consequently, the UE will apply the compensation value incorrectly based on the indication.

Observation 2: If the closed-loop control can be enhanced, UE cannot distinguish whether the indication applies to a UE without GNSS or a GNSS-capable UE.

- Conclusion

In this contribution, we provided our views on GNSS-resilient NR-NTN operation. Our observations and proposals are as follows:

Observation 1: RAT-dependent positioning can be considered for NTN positioning.

Observation 2: If the closed-loop control can be enhanced, UE cannot distinguish whether the indication applies to a UE without GNSS or a GNSS-capable UE.

Proposal 1: Time-based positioning methods can be a starting point for UE-based positioning to access the network.

Proposal 2: For NTN positioning, the single satellite-based position should be considered.

Proposal 3: For single-satellite positioning, the enhancement of PRS can be considered to improve the position accuracy and reduce access latency.

Proposal 4: For multiple-satellite positioning, each satellite can be considered as a TRP.

Proposal 5: For multiple-satellite positioning, the number of satellites for positioning should be decided.

Proposal 6: The mapping between satellites and PRS ID should be clarified under the multiple-satellite case.

Proposal 7: The enhancement of the closed-loop time control can be studied.

Proposal 8: Frequency closed loop adjustment can be considered for GNSS-resilient NR-NTN.

Proposal 9: Whether it is covered by GNNS can be reported implicitly via the PRACH resource, such as RO or preamble.

- References

- RP-251863, New SID: Study on GNSS resilient NR-NTN operation.

- 3GPP TS 38.211, NR; Physical channels and modulation.

- R1-2500667, ‘Final FL summary for IoT-NTN’, Moderator, Sony.