3GPP TSG-RAN WG1 Meeting #122	R1-2506168

Bengaluru, India, August 25th – August 29th, 2025

Agenda Item:	8.1.1

Source:	Ericsson

Title:	Maintenance on Rel-19 AI/ML Beam Management

Document for:	Discussion

# 1	Introduction

In Rel-19, beam prediction has been studied and specified. In this contribution, we share our views and proposals on a few critical maintenance issues.

# 2	Discussion on Remaining Issues

## 2.1	Limitations of multiple resourceSets for AP inference configuration

Agreement For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: whether/how to support 'aperiodic' CSI RS Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model. Agreement For UE-sided model, in CSI-ReportConfig for inference - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: At least BM-Case 1, the applicability for 'aperiodic' CSI RS

During RAN1#120, it was agreed that one or two associated IDs can be configured in CSI-ReportConfig.  When set B is a subset of set A, then only a single associated ID is needed. Since only one associated ID is configured for set A in the CSI report configuration, the resourceConfig for set A needs to be limited to a single resourceSet also for aPeriodic resource type. Otherwise, the UE assumptions of the associated ID and the resourceSets are unclear.

- For UE-sided model, in CSI-ReportConfig for AP inference configuration, the resourceConfig for set A can only include a single resource set.

Regarding the configuration for set B, when set B is a subset of set A, the set B is indicated according to the NZP-CSI-RS-ResourceId/SSB-Index. In this scenario, there could be multiple resourceSets in the set B resource configuration, each set having a different combination of the NZP-CSI-RS-ResourceId/SSB-Index. That is, there is no specification impact for this case and this could be supported already. For the scenario when set B is not a subset of set A, since the CSI-ReportConfig only includes a single set B associated ID, the UE would not know for which resourceSet the associated ID is applicable for. Possible solutions could comprise of limitting the resourceConfig of set B to a single resourceSet or configuring the associated ID in the associatedCSIreportConfig. For the latter solution, it could be complex for UE to support multiple setBs that are triggered in DCI, then the UE would need to quickly load a specific model. Hence, the former solution is preferable.

- For UE-sided model, for aPeriodic CSI inference report configuration, when set B is NOT a subset of set A, the resourceConfig for set B can only include a single resourceSet.

## 2.2 Support for UE report of inaccurate/invalid inference result

Once a UE is configured with an AI/ML model/functionality, then the UE is expected to report the requested radio measurement predictions to the NW via UCI. There are situations where the UE could fail to provide the requested reports, or even would want to request the NW to deactivate the AL/ML model/functionality due to performance issues. RAN2 for example made the following agreement in RAN2#128.

Agreements 1. When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network. FFS detailed design 2. When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

The agreement indicates that the UE may report non-applicability of the AI/ML functionality (equivalent to the UE not being able to generate reliable predictions anymore) using RRC signalling and then expecting the NW to deactivate the functionality in response. Using RRC signalling to deactivate the AI/ML functionality has some drawbacks that should be addressed. First, RRC signalling may introduce delays (for requesting resources to send the request to the NW and signalling to the UE to deactivate the feature) which in turn may degrade the performance, since during these delays the prediction (also called inference) reports received from the UE are not accurate or could not be trusted. Secondly, the UE might temporarily not be able to provide the requested prediction report. In such cases, the UE behaviour should be defined so that the NW can correctly interpret the received inference results, without necessarily deactivating completely the AI/ML functionality. RAN1 should hence introduce support for an indication in the UCI indicating if the inference report (i.e. AI/ML prediction report) is invalid or inaccurate.

For the UE-sided inference result report, support UE indicating in UCI if the inference result report is inaccurate/invalid by setting the CRI or SSB-RI to a fixed “dummy” value.

## 2.3 Active CSI resource count

The UE should not have more active CSI-RS ports or resources in active BWPs than its reported capability. The definition of the active NZP CSI-RS Resource duration is the following:

- Aperiodic CSI-RS: Active from the end of the PDCCH containing the request to the end of the scheduled PUSCH containing the report.

- Semi-persistent CSI-RS: Active from the end of the activation command to the end of the deactivation command.

- Periodic CSI-RS: Active from when it is configured by higher layer signaling to when the configuration is released.

The definition of active CSI-RS resources for the purpose of AI/ML inference operation needs to be addressed, the UE should not be configured with an infinite number of CSI-RS resources for the purpose of inference, however, these CSI-RS resources are not transmitted for inference, so it is counter-intuitive to denote them as active when configured. One solution can comprise that the CSI-RS resources part of set A is not considered as active even when they are triggered/configured. This would enable the NW to configure many non-transmitted CSI-RS resources for the purpose of set A.

Even if the predicted resources are inactive, the UE may need to have a limit on the number of CSI-RS resources it can predict. However, the number of resources is not the main issue, as the model's complexity remains similar whether it predicts 32 or 64 resources. Instead, the associated ID used to inform the UE about the network transmission properties on set A/B may impact the models used by the UE. Therefore, it may be more complex for a UE to manage two associated IDs with 16 CSI-RS resources each, compared to a single associated ID with 32 CSI-RS resources. It would be more reasonable to include a limit on the number of associated IDs that could be configured, since each associated ID may correspond to a separate model.

RAN1 to agree that predicted CSI-Resources that are part of set A for inference are not considered as active.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	For UE-sided model, in CSI-ReportConfig for AP inference configuration, the resourceConfig for set A can only include a single resource set.

Proposal 2	For UE-sided model, for aPeriodic CSI inference report configuration, when set B is NOT a subset of set A, the resourceConfig for set B can only include a single resourceSet.

Proposal 3	For the UE-sided inference result report, support UE indicating in UCI if the inference result report is inaccurate/invalid by setting the CRI or SSB-RI to a fixed “dummy” value.

Proposal 4	RAN1 to agree that predicted CSI-Resources that are part of set A for inference are not considered as active.